| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |
| --- | --- |
| Title | Organization of Section 6.5 |
| DCN | 21-07-0304-00-0000 |
| Date Submitted | 2007-09-14 |
| Source(s) | Y. Alice Cheng (Telcordia), Yoshihiro Ohba (Telcordia Technologies) |
| Re: | IEEE 802.21 Session #22 in Big Island |
| Abstract | Propose to add in the description of using RDF/SPARQL for Information Service Query in section 6.5. |
| Purpose | Improve the readability in section 6.5 for MIIS query methods and operation. |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |

# Introduction

Information query supported by MIIS includes TLV Query (Binary-representation) and SPARQL/RDF Query (RDF-representation).

Following is the current index

6.5 Media-independent information service .................................................................................... 45

6.5.1 Introduction................................................................................................................... 45

6.5.2 Access information service before authentication ......................................................... 46

6.5.3 Information service elements ........................................................................................ 47

6.5.4 Definition of information element name space.............................................................. 50

6.5.5 TLV Representation of information elements ............................................................... 51

6.5.5.1 General............................................................................................................... 51

6.5.5.2 IE containers ...................................................................................................... 52

6.5.6 Information service schema .......................................................................................... 53

6.5.6.1 General............................................................................................................... 53

6.5.6.2 RDF schema representation .............................................................................. 54

6.5.6.3 The MIIS schema............................................................................................... 55

The index and its content do not harmonize the support of both Binary query and SPARQL query that is supported in MIIS. Moreover, there is no high level description on the MIIS behavior without reading into the primitive parameters in section 7.

Following information needs to be clarified:

- How does Binary query and SPARQL query can be used for obtaining Information Elements.

- The expected behaviors and parameters for MIIS.

Section 6.5.5 should include at least some description of the RDF representation.

# Proposed Change

## Index

6.5 Media-independent information service .................................................................................... 45

6.5.1 Introduction................................................................................................................... 45

6.5.2 Access information service before authentication ......................................................... 46

6.5.3 Restricting query response size

6.5.4 Information elements ........................................................................................ 47

6.5.5 Definition of information element name space.............................................................. 50

6.5.6 Information element representation and query methods ............................................................... 51

      - Introductionl............................................................................................................... 52

      - Binary representation and TLV query

      - .......................................................................... 52

      - RDF Representation and SPARQL query

6.5.7 Information service schema .......................................................................................... 53

6.5.7.1 General............................................................................................................... 53

6.5.7.2 RDF schema .............................................................................. 54

6.5.7.3 The MIIS RDF schema............................................................................................... 55

## Section Content

Note to Editor: The following section numbers should be update to the numbers specified above or other appropriate assignments. Contributor has problem updating the section numbers in the following text due the text was imported from RTF file exported by FM.

- Restricting query response size (should be 6.5.3)

When sending a request for information query, the MIIS client may provide a maximum response size to limit the query response message size. A request may contain multiple queries in the order of significance. In case of the query result exceeding the maximum response size, least significant query results shall be removed from the response. The MIIS server may use its own maximum response size smaller than that is specified by the MIIS client.

- Information element representation and query methods (should be 6.5.6)

- Introduction (should be 6.5.6.1)

MIIS defines two methods for representing Information Elements, i.e., binary representation and RDF[ref] representation, for which different query methods are used. For binary representation, the TLV query method defined in Clause 6.5.5.2 is used. For RDF representation, the SPARQL[ref] query method is used.

- Binary representation and TLV query

In binary representation, Information Elements shall be represented and encoded in Type Length Value form as follows.

| Type (4 octets) | Length (variable octets) | Value (variable octets) |
| --- | --- | --- |
| Represents the type or ID of the Information Element | Length of the Value field of the IE | The value of the IE |

- TLV representation of information elements

The Length field may be interpreted as follows.

Case 1: If the number of octets occupied by the Value field is less than 128, the size of the Length field is always 1 octet and the MSB of the octet is set to the value '0'. The values of the other seven bits of this octet indicate the actual length of the Value field.

Case 2: If the number of octets occupied by the Value field is exactly 128, the size of the Length field is one octet. The MSB of the Length octet is set to the value '1' and the other seven bits of this octet are all set to the value '0'.

Case 3: If the number of octets occupied by the Value field is greater than 128, then the Length field is always greater than 1 octet. The MSB of the first octet of the Length field is set to the value '1' and the remaining 7 bits of the first octet indicate the number of octets that are appended further. The number represented by the 2nd and subsequent octets of the Length field, when added to 128, indicates the total size of the Value field, in octets.

TLV Queries

(Move the text from page 197 L38 – p198 L24 here).

(Also page 199 L12 – L14, the text should be slightly modified to ensure the paragraph flow.

e.g. Should use Network Type Inclusion  instead of NETWORK_TYPE_INCLUSION.)

IE containers

To effectively query binary-represented Information Elements, three Information Element Containers are defined, namely the List of Neighboring Access Networks Container, the Access Network Container, and the PoA Container:

- List of Neighboring Access Networks Container - contains a list of heterogeneous neighboring access networks for a given geophysical location. 

A List of Neighboring Access Networks Container shall contain at least one Access Network and may contain one or more Vendor Specific IEs.

| Type= TYPE_IE_CONTAINER_LIST_OF_NETWORKS | Length= Variable |
| --- | --- |
| Access Network Container #1 |  |
| Access Network Container #2 (optional) |  |
| … |  |
| Access Network Container #k (optional) |  |
| Vendor Specific IE (optional) |  |

- Access Network Container - contains all the information depicting an access network. 

An Access Network Container shall contain at least one PoA Container and may contain one or more Vendor Specific Network IEs:

| Type=TYPE_IE_CONTAINER_NETWORK | Length= Variable |
| --- | --- |
| Network Type IE |  |
| Operator Identifier IE |  |
| Service Provider Identifier IE (optional) |  |
| Access Network Identifier IE (optional) |  |
| Network System ID IE (optional) |  |
| Roaming Partners IE (optional) |  |
| Cost IE (optional) |  |
| Network Security IE (optional) |  |
| Network QoS IE (optional) |  |
| Network Data Rate IE (optional) |  |
| Network IP Configuration Methods IE (optional) |  |
| Network Capabilities IE (optional) |  |
| List of Supported LCP IE (optional) |  |
| PoA Container #1 |  |
| PoA Container #2 (optional) |  |
| … |  |
| PoA Container #k (optional) |  |
| Vendor Specific Network IE (optional) |  |

- PoA Container - contains all the information depicting a PoA and may include one or more optional Vendor Specific PoA IEs:

| Type=TYPE_IE_CONTAINER_POA | Length= Variable |
| --- | --- |
| PoA MAC Address IE |  |
| PoA Location IE |  |
| PoA Channel Range IE |  |
| PoA System Information IE |  |
| PoA Subnet Information IE #1 |  |
| PoA Subnet Information IE #2 (optional) |  |
| ... |  |
| PoA Subnet Information IE #k (optional) |  |
| PoA IP Address IE #1 (optional) |  |
| ... |  |
| PoA IP Address IE #k (optional) |  |
| Vendor Specific PoA IE (optional) |  |

TLVs for the component IEs contained in the Access Network Container and PoA Container are defined in Annex B.

- RDF representation and SPARQL query (should be 6.5.6.3)

In RDF representation, Information Elements are represented in XML format. SPARQL is used as the query method..RDF representation and SPARQL query requires an RDF schema as described in Clause 6.5.6.2.

- . The basic MIH schema is defined and ways to extend the schema provided

- Information service schema (should be 6.5.7)

- General

A schema defines the structure of certain information. A schema is used in the IEEE 802.21 Information Service to define the structure of each information element, as well as the relationship among different supported information elements. The IEEE 802.21 Information Service schema needs to be supported by every MIHF that implements the MIIS to support flexible and efficient information queries. The IEEE 802.21 Information Service defines the various information elements and their structure. The various IEs represent information about lower layers of the network protocol stack, as well as information about higher layer services available in different access networks. A schema is defined by a language and may be represented in multiple ways. Examples include Resource Description Framework (RDF) or a simple TLV representation of different information elements.

- RDF schema (informative)

RDF (see W3C Recommendation, “RDF/XML Syntax Specification” and W3C Recommendation, “Resource Description Framework (RDF) – Concepts and Abstract Syntax”) uses SPARQL as a query language for querying information. An RDF schema defines the structure of a set of expressions, where the underlying structure of any expression is a collection of triples, each consisting of a subject, a predicate and an object. The XML syntax for RDF is called RDF/XML. RDF has the following advantages:

- Supports both hierarchical and non-hierarchical information structure

- Allows for flexible data query

- Allows for distributed schema definition

- Allows for easily changeable schema definitions.

In order to strictly define each information element in an RDF schema, the schema is augmented with Web Ontology Language (OWL) (see W3C Recommendation “OWL Web Ontology Language Reference”). OWL uses both URLs for naming and the description framework provided by RDF to add the following capabilities to ontologies:

- Ability to be distributed across many systems

- Scalability

- Compatibility with other Web standards for accessibility and internationalization

- Openness

- Extensibility

OWL builds on RDF and the RDF Schema and adds more vocabulary for describing properties and classes: among others, relations between classes (e.g., disjointness), cardinality (e.g., “exactly one”), equality, richer typing of properties and characteristics of properties (e.g., symmetry), and enumerated classes.

- The MIIS RDF schema (should be 6.5.7.3)

[Notes to Editor: following two paragraphs are moved from previous section where the bold indicates sentence that were modified.]

The RDF schema definition for MIIS consists of two parts; the basic and the extended schema. An MIH entity is pre-provisioned with the basic schema for ease of implementation of schema-based query. In scenarios where the basic schema is not pre-provisioned methods such as DHCP may be used to access the URL of the basic schema.

Unlike the basic schema, the extended schema may be updated, e.g., when a new link-layer technology is introduced. The extended schema may be retrieved from the specified URL via the IEEE 802.21 Information Service using the schema query capability without any pre-provision of such extended schema. The URL of the extended schema may also be obtained via the schema URL query capability. Alternatively other methods such as DHCP may be enabled for finding out the URL of the extended schema. The extended schema is defined as an extension of the basic schema and includes a data structure and relationship of media-specific or higher-layer information. In that sense, the extended schema is the complement of the basic schema.

The basic schema is defined in Annex D. Annex F is a UML representation of the basic schema. The basic schema contains the schema for information elements defined in Table 8. The extended schema is defined by individual vendors and may contain the schema for vendor-specific information elements.

## Additional Updates Required

Page 47  Subclause 6.5.3  Line43

Table 8 represents the list of Information Elements and their sematics. Each Information Element has an abstract data type. The binary and RDF representation of these Information Elements are described in 6.5.6.2 and 6.5.6.3, respectively. The IEs may be retrieved using TLV or SPARQL based query mechanisms. The standard does not recommend or mandate the choice of a particular mechanism. Vendors may need to specify additional IEs beyond the IEs specified in Table 8. Vendors can choose to do that using the Vendor Specific IEs. These IEs will then be available only in vendor specific deployments.

Page 133 Subclause 7.4.27: Change description “binary query” to “TLV query”. No modification required for the data type naming and the parameter naming.