| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH Event Subscription Update |  |
| DCN | 21-07-0305-00-0000-Event-Subscribe-Update.doc |  |
| Date Submitted | 09-29-2007 |  |
| Source(s) | Srinivas Sreemanthula, Mika Forssell, Janne Tervonen |  |
| Re: | Event Subscription parameter addition for link detect |  |
| Abstract | This contribution proposes to add a mechanism to MIH Even Subscribe to limit the networks / PoAs that the link event indications are created for. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

# Problem Statement

In 802.21/D7.1, link event reporting cannot be filtered in any way. Especially when using remote link event reporting, this may result in a vast number of link event indications created and sent across the network. To limit the amount of link event indications, it would be beneficial to have a mechanism for defining a limited set of networks/PoAs that the link event indications are created for.

Summary of the Proposed Solution

During the link event subscription, the subscriber can define a set of networks/PoAs from which the subscriber is interested in getting link event indications. The link event service provider shall create and send link event indications only for the networks/PoAs defined in the corresponding MIH_Event_Subscribe request message.

# Proposed Changes to the Current Specification

7.3.1 Link_Event_Subscribe

7.3.1.1 Link_Event_Subscribe.request

7.3.1.1.1 Function

This primitive is used by MIHF (the subscriber) to subscribe an interest in one or more events from a specific link layer technology. Optionally, the subscriber may indicate a list of specific configuration information applicable for various events being subscribed. The response indicates which of the requested events were successfully subscribed to. Events that were not successfully subscribed to will not be delivered to the subscriber.

7.3.1.1.2 Semantics of service primitive

Link_Event_Subscribe.request (

RequestedLinkEventList,

EventConfigurationInfoList

)

Parameter:

| Name | Type | Description |
| --- | --- | --- |
| Requested Link Event List | LINK_EVENT_ LIST | List of link layer events that the subscriber would like to receive indications for. |
| Event Configuration Info List | EVENT_CONFIGURATION_INFO_LIST | List of additional configuration info for event subscription. |

7.3.1.1.3 When generated

This primitive is generated by a subscriber such as the MIHF that is seeking to receive event indications from different link layer technologies.

7.3.1.1.4 Effect on receipt

The recipient responds immediately with Link_Event_Subscribe.confirm primitive.

8.6.1.7 MIH_Event_Subscribe request

This message is sent by a remote MIHF (the subscriber) to subscribe to one or more event types from a particular event origination point.

MIH Header Fixed Fields (SID=1, Opcode=1, AID=4)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

LinkIdentifier (Link Identifier TLV)

RequestedMihEventList (MIH Event List TLV)

EventConfigurationInfo List (Event Configuration Info List TLV)

Table B-4—Data types for links

| LINK_DETECT_CONFIG | SEQUENCE ( LINK_TYPE, ACCESS_NETWORK_IDENTIFIER, SIGNAL_STRENGTH) | Represents data type used to configure link detect event triggering | Not Applicable |
| --- | --- | --- | --- |
| EVENT_CONFIGURATION_INFO | CHOICE ( SEQUENCE(MIH_LINK_DETECTED, LIST(LINK_DETECT_CONFIG)), NULL) | Represents additional configuration info for event subscription. | Not Applicable |

TABLE ?? – MIH-EVENT-LIST

| MIH_LINK_UP | 1 |
| --- | --- |
| MIH_LINK_DOWN | 2 |
| MIH_LINK_GOING_DOWN | 3 |
| MIH_LINK_EVENT_ROLLBACK | 4 |
| MIH_LINK_DETECTED | 5 |
| MIH_LINK_PARAMETERS_REPORT | 6 |
| MIH_LINK_HANDOVER_IMMINENT | 7 |
| MIH_LINK_HANDOVER_COMPLETE | 8 |