| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH Event Subscription Update |  |
| DCN | 21-07-0305-02-0000-Event-Subscribe-Update.doc |  |
| Date Submitted | 11-15-2007 |  |
| Source(s) | Srinivas Sreemanthula, Mika Forssell, Janne Tervonen |  |
| Re: | Event Subscription parameter addition for link detect |  |
| Abstract | This contribution proposes to add a mechanism to MIH Even Subscribe to limit the networks / PoAs that the link event indications are created for. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

Proposed Changes to the Current Specification

- MIH_Event_Subscribe

- MIH_Event_Subscribe.request

- Function

This primitive is used by an MIH User (the subscriber) to subscribe an interest in one or more MIH event types from the local or a remote MIHF. Optionally, the subscriber may indicate a list of specific configuration information applicable for various events being subscribed. If configured, the event must be triggered only when all the criteria set in the parameters are met.

- Semantics of service primitive

MIH_Event_Subscribe.request			 	(
				DestinationIdentifier,
				LinkIdentifier,
				RequestedMihEventList,

EventConfigurationInfoList

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | This identifies the local MIHF or a remote MIHF which will be the destination of this request. |
| Link Identifier | LINK_TUPLE_ID | Identifier of the link for event subscription. For local event subscription, PoA MAC Address in the Link Identifier need not be present if the link type lacks such a value. |
| Requested MIH Event List | MIH_EVENT_LIST | List of MIH events that the endpoint would like to receive indications for, from the Event Source. |
| Event Configuration Info List | LIST(EVENT_CONFIGURATION_INFO) | List of additional configuration info for event subscription. |

- When generated

This primitive is invoked by an MIH User when it wants to receive indications on a set of specific MIH events from the local MIHF or a remote MIHF.

- Effect on receipt

If the destination of the request is the local MIHF itself, the local MIHF shall respond immediately with an MIH_Event_Subscribe.confirm primitive. If the destination of the request is a remote MIHF, the local MIHF shall generate and send an MIH_Event_Subscribe request message to the remote MIHF.

8.6.1.7 MIH_Event_Subscribe request

This message is sent by a remote MIHF (the subscriber) to subscribe to one or more event types from a particular event origination point.

MIH Header Fixed Fields (SID=1, Opcode=1, AID=4)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

LinkIdentifier (Link Identifier TLV)

RequestedMihEventList (MIH Event List TLV)

EventConfigurationInfo List (Event Configuration Info List TLV)

B.2.18 Datatype for MIH Event Subscription Configuration

Table B-24—Data types for MIH Event Subscription Configuration

| MIH_EVENT_ID | ENUMERATED | MIH Event Identifiers | 1 MIH_LINK_UP 2 MIH_LINK_DOWN 3 MIH_LINK_GOING_DOWN 4 MIH_LINK_EVENT_ROLLBACK 5 MIH_LINK_DETECTED 6 MIH_LINK_PARAMETERS_REPORT 7 MIH_LINK_HANDOVER_IMMINENT 8 MIH_LINK_HANDOVER_COMPLETE |
| --- | --- | --- | --- |
| EVENT_CONFIGURATION_INFO | CHOICE ( SEQUENCE( MIH_EVENT_ID, LIST(LINK_DETECT_CONFIG)), NULL) | Represents additional configuration info for event subscription. | Not Applicable |
| LINK_DETECT_CONFIG | SEQUENCE ( LINK_TYPE, CHOICE ( ACCESS_NETWORK_ID, NULL) CHOICE ( SIGNAL_STRENGTH), NULL) ) | Represents data type used to configure link detect event triggering | Not Applicable |

Table C2

Type			Value			Datatype

MIHEventId List		??			MIH EVENT_ID

EventConfigurationInfo	??			EVENT_CONFIGURATION_INFO

LinkDetectConfig		??			LINK_DETECT_CONFIG