| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH Link Detection Update |  |
| DCN | 21-07-0306-02-0000-Link-Detection-Update.doc |  |
| Date Submitted | 11-15-07 |  |
| Source(s) | Srinivas Sreemanthula, Timo Vuori, Janne Tervonen |  |
| Re: |  |  |
| Abstract | A proposal to redefine MIH_Link_Detected indication. |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

Proposed Changes to the Current Specification

CHANGE #1

- Link_Detected.indication

- Function

Link_Detected indicates that a new type of link has been detected for use.

This may imply that the mobile node is in the coverage area and may listen to a beacon, or that the mobile node may have received a response to a probe. Link_Detected does not guarantee that the mobile node would be able to establish connectivity with the detected link, but just that the mobile node may attempt to gain connectivity. MIH Users and the MIHF itself may evaluate additional properties of the link before attempting to establish a L2 connection with the link.

- Semantics of service primitive

Link_Detected.indication 			(
			LinkIdentifier,

AccessNetworkIdentifier,

SignalStrength,
			MIHCapability
			)

Parameters

| Name | Type | Description |
| --- | --- | --- |
| Link Identifier | LINK_TUPLE_ID | Identifier of the link associated with the event |
| Access Network Identifier | ACCESS_NETWORK_ID | Identifier to uniquely determine the Access Network. |
| Signal Strength | SIGNAL_STRENGTH | Current signal strength on that link |
| MIH Capability | LINK_MIH_CAPABILITY_FLAG | Used to indicate which MIH capabilities are supported on this link. |

:

- When generated

This notification is generated when a new link is detected. The detection may be based on a beacon or response to a probe, in the case of an IEEE 802.11 network.

- Effect on receipt

The MIHF receives this event from the link layer. The MIHF then passes this notification to the MIH User(s) which have subscribed for this notification. The MIH User(s), including the MIHF itself, may discover additional properties of the link before selecting it for establishing connectivity.

CHANGE #2

7.4.9 MIH_Link_Detected

7.4.9.1 MIH_Link_Detected.indication

7.4.9.1.1 Function

The MIH_Link_Detected.indication is sent to local MIHF users to notify them of a local event, or is the result of the receipt of an MIH_Link_Detected indication message to indicate to the remote MIHF users, who have subscribed to this remote event.

7.4.9.1.2 Semantics of the service primitive

MIH_Link_Detected.indication (

SourceIdentifier,

LinkDetectInfoList

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| LinkDetectInfo List | LIST(LINK_DETECT_INFO) | This is the list of link detection information to be reported. |

8.6.2.5 MIH_Link_Detected indication

This message is transmitted to the remote MIHF when a new link has been detected.

MIH Header Fixed Fields (SID=2, Opcode=3, AID=5)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

LinkDetectInfoList (Link DetectInfo List TLV)

B.2.18 Datatype for MIH Link Detected Indication

Table B-24—Data types for MIH Link Detected Indication

| LINK_DETECT_INFO | SEQUENCE ( LINK_TYPE, ACCESS_NETWORK_ID, SIGNAL_STRENGTH, LINK_MIH_CAPABILITY_FLAG) | Represents the network detection with best signal strength information. | Not Applicable |
| --- | --- | --- | --- |

Table C2

Type			Value			Datatype

LinkDetectInfo	??			LINK_DETECT_INFO