| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Net Handover Commit Updates |  |
| DCN | 21-07-0308-01-0000 |  |
| Date Submitted | 11-15-07 |  |
| Source(s) | Mika Forssell, Janne Tervonen, Srinivas Sreemanthula |  |
| Re: | IEEE 802.21 Adhoc Session #NN in Korea |  |
| Abstract | Net-HO-Commit updates |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

- MIH_Net_HO_Commit

- MIH_Net_HO_Commit.request

- Function

This primitive is used by an MIHF on the network to communicate with the remote MIHF on the mobile node. The primitive is used to request the peer MIHF the commitment to perform link handover based on selected choices for candidate networks and PoA.

- Semantics of service primitive

MIH_Net_HO_Commit.request

(

DestinationIdentifier,

LinkType,

TargetNetworkInfoList,

LinkActionSetList,
				)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | Shall contain the MIHF ID of the node that sent the MIH_Net_HO_Commit request message. |
| LinkType | LINK_TYPE | Contains target link type |
| TargetNetworkInfo List | LIST(TARGET_NETWORK_INFO) | Contains target network information for handover |
| Link Action Set List | LIST(LINK_ACTION_REQ) | (Optional) A list of handover actions for the links. |

- When generated

The MIH User generates this primitive to order specific handover actions on one or more links.

- Effect on receipt

Upon receipt of this primitive MIHF shall send a request message to the destination.

- MIH_Net_HO_Commit.indication

- Function

This primitive is generated by an MIHF to indicate that an MIH_Net_HO_Commit request message has been received from a peer MIHF.

- Semantics of service primitive

MIH_Net_HO_Commit.indication (

SourceIdentifier,

LinkType,

TargetNetworkInfoList,

LinkActionSetList
				)

Parameters:

| Name | Type | Description |  |
| --- | --- | --- | --- |
| Source Identifier | MIHF_ID | Shall contain the MIHF ID of the node that sent the MIH_Net_HO_Commit request message. |  |
| LinkType | LINK_TYPE | Contains target link type |  |
| TargetNetworkInfo List | LIST(TARGET_NETWORK_INFO) | Contains target network information for handover |  |
| Link Action Set List | LIST(LINK_ACTION_REQ) | (Optional) A list of handover actions for the links. |  |

- When generated

This primitive is generated by an MIHF on receiving an MIH_Net_HO_Commit request message from a peer MIHF.

- Effect on receipt

The MIH User receiving this primitive replies with an MIH_Net_HO_Commit.response primitive.

3ignaled

- MIH_Net_HO_Commit.response

- Function

This primitive is used by an MIHF to communicate with a peer MIHF from which an MIH_Net_HO_Commit request message is received. The primitive is used to communicate the response of a handover commit request.

- Semantics of service primitive

MIH_Net_HO_Commit.response 	(

DestinationIdentifier,

LinkType,

TargetNetworkInfo,
				LinkActionResultList,
				Status

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | Shall contain the MIHF ID of the node that sent the MIH_Net_HO_Commit response message. |
| LinkType | LINK_TYPE | Contains target link type |
| TargetNetworkInfo | TARGET_NETWORK_INFO | Contains target network information for handover |
| Link Action Result List | LIST(LINK_ACTION_RSP) | (Optional) A list of link actions result. |
| Status | STATUS | Status of operation. |

- When generated

This primitive is generated in response to an MIH_Net_HO_Commit.indication primitive.

- Effect on receipt

Upon receipt, the old Serving PoS is informed about the status of the previously issued command request.

- MIH_Net_HO_Commit.confirm

- Function

This primitive is generated by the MIHF to confirm that an MIH_Net_HO_Commit response message is received from a peer MIHF.

- Semantics of service primitive

MIH_Net_HO_Commit.confirm (
				SourceIdentifier,

LinkType,

TargetNetworkInfo,
				LinkActionResultList,
				Status
				)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | Shall contain the MIHF ID of the node that sent the MIH_Net_HO_Commit response message. |
| LinkType | LINK_TYPE | Contains Target link type |
| TargetNetworkInfo | TARGET_NETWORK_INFO | Contains target network information for handover |
| Link Action Result List | LIST(LINK_ACTION_RSP) | (Optional)A list of link actions result. |
| Status | STATUS | Status of operation. |

- When generated

This primitive is generated by the MIHF on receiving an MIH_Net_HO_Commit response message from a peer MIHF.

- Effect on receipt

Upon receipt, the old serving PoS is informed about the status of the previously issued command request

.

8.6.3.15 MIH_Net_HO_Commit request

This message is used by the MIHF to communicate the intent to commit to a handover request to a specific

link and PoA.

MIH Header Fixed Fields (SID=3, Opcode=1, AID=8)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

LinkType

TargetNetworkInfoList (List of Target_Network Info TLV)

LinkActionSetList (Optional) (Link Action Set List TLV)

8.6.3.16 MIH_Net_HO_Commit response

This message is used by the MIHF to respond to a request to commit to a handover request to a specific link and PoA.

MIH Header Fixed Fields (SID=3, Opcode=2, AID=8)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

LinkType

TargetNetworkInfo (Target_Network Info TLV)

LinkActionsResponse (Optional) (Link Action ResponseTLV)

Status (Status TLV)

B.2.17 Data type for Handover Commit Information

Table B-24—Data type for Handover Commit Information

| TARGET_NETWORK_INFO | SEQUENCE ( CHOICE ( ACCESS_NETWORK_IDENTIFIER, NULL) CHOICE ( ACCESS_NETWORK_AUX_ID, NULL) CHOICE ( LINK_ADDRESS, NULL) ) | Represents the handover commit iinformation. LINK_ADDRESS corresponds to the PoA for handover. | Not Applicable |
| --- | --- | --- | --- |

Table C2

Type			Value			Datatype

TargetNetworkInfo	??			TARGET_NETWORK_INFO