| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Data type for SYSTEM _INFORMATION_IE |  |
| DCN | 21-07-0317-00-0000-SYSTEM_IFNORMATION_IE |  |
| Date Submitted | 2007-09-14 |  |
| Source(s) | Jin Lee, Ronny kim LG Electronics | E-mail : { jins978, ronnykim}@lge.com |
| Re: | IEEE 802.21 Session #22 in Hawaii |  |
| Abstract | Data type for SYSTEM_INFORMATION_IE |  |
| Purpose | This document proposes the data type for SYSTEM_INFORMATION_IE |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

Data type for SYSTEM_INFORMATION_IE

Jin Lee, Ronny Kim

LG Electronics

Proposed Text Changes

[Replace table B-12 with the following table, Page 193, line 1]

| Type | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| PARAMETERS | CHOICE (DCD_UCD, SIB, SYSTEM_PARAMETERS) | A data type to represent system information depending on Network type | DCD_UCD : IEEE 802.16 SIB : UMTS SYSTEM_PARAMETERS : CDMA 2000 |
| DCD_UCD | SEQUENCE (BS ID, BS_EIRP, DL_FREQUENCY, UL_FREQUENCY,TTG , RTG, DOWNLINK_BURST_PROFILE, UPLINK_BURST_PROFILE, RANGING_CDMA_CODE ) | A type to represent DCD _UCD | N/A |
| BS ID | INTEGER (6) | Base Station Identifier |  |
| BS_EIRP | INTEGER (1) | BS’s effective isotropic radiated power level | Signed in units of 1 dBM |
| DL_FREQUENCY | INTERGER(4) | Downlink center frequency |  |
| UL_FREQUENCY | INTERGER (4) | Uplink center frequency |  |
| TTG | INTERGER (2) | Transmit transition gap | Physical slot unit |
| RTG | INTERGER (1) | Receive transaction gap | Physical slot unit |
| DOWNLINK_BURST_PROFILE | BITMAP (255) | A List of FEC Code Type for Downlink burst | Refer to IEEE 802.16Rev2/D0d 11.4.2 |
| UPLINK_BURST_PROFILE | BITMAP (255) | A List of FEC Code Type for Uplink burst | Refer to IEEE 802.16Rev2/D0d 11.3.1.1 |
| RANGING_CDMA_ CODES | SEQUENCE (INITIAL_CODE, HANDOVER_CODE) | A set of CDMA ranging codes |  |
| INITIAL_CODE | INTEGER(1) | INITIAL_RANGING_CODE | Refer to IEEE 802.16Rev2/D0d 11.3.1 |
| HANDOVER_CODE | INTEGER(1) | HANDOVER_RANGING_CODE | Refer to IEEE 802.16Rev2/D0d 11.3.1 |
| SIB | SEQUENCE(CELL_ID, FREQUENCY_CODE_NUMBER) | A type to represent SIB |  |
| CELL_ID | INTEGER(10) | Base Station Cell ID |  |
| FREQUENCY_CODE_NUMBER | INTEGER(2) | UMTS scrambling code, cdma2000 Walsh code) | 0-65535 |
| SYSTEM_PARAMETERS | SEQUENCE (BASE_ID, PILOT_PN, FREQUENCY, BAND_CLASS) | A type to represent SYSTEM_PARAMETERS |  |
| BASE_ID | INTEGER (2) | Base station identifier |  |
| PILOT_PN | INTEGER (2) | Pilot PN sequence offset index |  |
| FREQUENCY | INTEGER (2) | Identifier the carrier frequency |  |
| BAND_CLASS | INTEGER (1) | CDMA band class |  |