IEEE 802.21
Media Independent Handover Services

Resource List Definition

Date:	November 13, 2007

Authors:	Juan Carlos Zuniga, Ulises Olvera-Hernandez

InterDigital Communications LLC

e-Mail:	 juancarlos.zuniga@interdigital.com

ulises.olvera-hernandez@interdigital.com

Abstract

This document presents a solution to the resource definition description that is currently missing in the D7.1 version of the 802.21 specification.

# Background

In typical mobile wireless technologies, an inter-RAT handover would have the following sequence of events:

- Query phase - to find out the status of resources at both source and target networks before taking a handover decision

- Preparation phase - where resources are reserved at the target network once a handover decision has been taken

- Execution phase - where the handover commands are sent and performed

- Completion phase - where the result of the handover is informed and the original resources are released at the source network

Currently, the 802.21 D07.01 specification supports some handover messages that can be used in each one of these phases. However, functionality is still needed inside these messages, particularly for the preparation and execution phases, in order for them to be useful for other wireless technologies. This document proposes some of the missing functionalities for the handover preparation and execution phases.

# Proposed Solution

Following the above mentioned principles, we propose using new resource Information Element definitions in the current 802.21 messages, as described below.

- Requested Resource Set

  - MIH_N2N_HO_Commit request

- Assigned Resource Set

  - MIH_N2N_HO_Commit response

  - MIH_Net_HO_Commit request

Update the information to include, update and/or remove the following Information Elements in the Messages (section 8.6) and Primitives (section 7.4) as follows:

On Page 124, Line 16

MIH_N2N_HO_Commit.request

(

DestinationIdentifier,

MobileNodeIdentifier,

TargetMNLinkIdentifier,

TargetPoA
				PreferredLinkList,

RequestedResourceSet,

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | This identifies a remote MIHF which will be the destination of this request. |
| Mobile Node Identifier | MIHF_ID | This identifies an MIHF of the mobile node which commits to perform handover action. |
| Target Mobile Node Link Identifier | LINK_ID | This is the identifier of the mobile node’s target link for which resources are requested |
| Target PoA | MAC_ADDRESS | This is the MAC address of the target Point of Attachment (AP/BS). |
| Preferred Link List | LIST( LINK_POA_LIST) | A list of PoAs for each link, suggesting the new access networks and PoAs to which handover initiation should be considered. This may be different than the networks that were suggested in the handover query. The list is sorted from most preferred first to least preferred last. |
| Requested Resource Set | REQUESTED_ RESOURCE_SET | This includes the set of parameters required for performing MN admission control and resource reservation at the target network |

On Page 124, Line 62

MIH_N2N_HO_Commit.indication

(

SourceIdentifier,

MobileNodeIdentifier,

TargetMNLinkIdentifier,

TargetPoA
				PreferredLinkList,

RequestedResourceSet,

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | This identifies the invoker of this primitive which can be a remote MIHF. |
| Mobile Node Identifier | MIHF_ID | This identifies an MIHF of the mobile node which commits to perform handover action. |
| Target Mobile Node Link Identifier | LINK_ID | This is the identifier of the mobile node’s target link for which resources are requested |
| Target PoA | MAC_ADDRESS | This is the MAC address of the target Point of Attachment (AP/BS). |
| Preferred Link List | LIST( LINK_POA_LIST) | A list of PoAs for each link, suggesting the new access networks and PoAs to which handover initiation should be considered. This may be different than the networks that were suggested in the handover query. The list is sorted from most preferred first to least preferred last. |
| Requested Resource Set | REQUESTED_ RESOURCE_SET | This includes the set of parameters required for performing MN admission control and resource reservation at the target network |

On Page 125, Line 45

MIH_N2N_HO_Commit.response

(

DestinationIdentifier,

MobileNodeIdentifier,

TargetLinkIdentifier,
				AssignedResourceSet,

Status

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | This identifies a remote MIHF which will be the destination of this request. |
| Mobile Node Identifier | MIHF_ID | This identifies an MIHF of the mobile node which commits to perform handover action. |
| Target Link Identifier | LINK_TUPLE_ID | This contains the identifier of the target Point of Attachment (AP/BS) for the MN. |
| Assigned Resource Set | ASSIGNED_ RESOURCE_SET | This includes the set of resource parameters assigned by the target network to the MN |
| Status | STATUS | Status of operation |

On Page 126, Line 29

MIH_N2N_HO_Commit.confirm

(

DestinationIdentifier,

MobileNodeIdentifier,

TargetLinkIdentifier,
				AssignedResourceSet,

Status

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | This identifies a remote MIHF which will be the destination of this request. |
| Mobile Node Identifier | MIHF_ID | This identifies an MIHF of the mobile node which commits to perform handover action. |
| Target Link Identifier | LINK_TUPLE_ID | This contains the identifier of the target Point of Attachment (AP/BS) for the MN. |
| Assigned Resource Set | ASSIGNED_ RESOURCE_SET | This includes the set of resource parameters assigned by the target network to the MN |
| Status | STATUS | Status of operation |

On Page 118, Line 15

MIH_Net_HO_Commit.request

(

DestinationIdentifier,

TargetPoA
				LinkActionSetList,

AssignedResourceSet

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | This identifies a remote MIHF which will be the destination of this request. |
| Target PoA | LINK_ID | Notify MN about the selected target PoA to handover. |
| Link Action Set List | LIST(LINK_ACTION _REQ) | A list of handover actions for the links. |
| Assigned Resource Set | ASSIGNED_ RESOURCE_SET | This includes the set of resource parameters assigned to the MN for performing the handover |

On Page 118, Line 58

MIH_Net_HO_Commit.indication

(

SourceIdentifier,

TargetPoA
				LinkActionSetList,

AssignedResourceSet

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | Contains the MIHF ID of the node that sent the MIH_Net_HO_Commit request message. |
| Target PoA | LINK_ID | Notify MN about the selected target PoA to handover. |
| Link Action Set List | LIST(LINK_ACTION _REQ) | A list of handover actions for the links. |
| Assigned Resource Set | ASSIGNED_ RESOURCE_SET | This includes the set of resource parameters assigned to the MN for performing the handover |

On Page 107 Line 25, and Page 108 Line 20,

Correct the Net Query response resource definition to make it consistent with the MN Query response resource definition as follows:

| Requested Resource Set | QOS_LIST | Requested resources needed by the mobile terminal |
| --- | --- | --- |
| Available Resource Set | LIST(QOS_LIST) | A list of available resources for each link |

On Page 167, Line 32

8.6.3.19 MIH_N2N_HO_Commit request

This message is used by the MIHF on the serving network to communicate with its peer MIHF on the selected target network. This is used to request a inform the target network to allocate resources to that a mobile node that is about to move toward that network link and PoA.

MIH Header Fixed Fields (SID=3, Opcode=1, AID=10)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

MobileNodeIdentifier (Mobile Node MIHF ID TLV)

Target Mobile Node Link Identifier (MN Link ID TLV)

Target PoA (PoA TLV)

Preferred Link List (Link PoA List TLV)

Requested Resource Set (Requested Resource Set TLV)

8.6.3.20 MIH_N2N_HO_Commit response

This message is used by the MIHF on the selected target network to communicate with its peer MIHF on the serving network. This is used to respond to the MIH_N2N_HO_Commit request message and in case of a successful assignation provide the list of assigned resources.

MIH Header Fixed Fields (SID=3, Opcode=2, AID=10)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

MobileNodeIdentifier (Mobile Node MIHF ID TLV)

Target Link Identifier (Link Identifier TLV)

Assigned Resource Set (Assigned Resource Set TLV)

Status (Status TLV)

On Page 166, Line 1

8.6.3.15 MIH_Net_HO_Commit request

This message is used by the MIHF to communicate the intent to commit to a handover request to a specific link and PoA.

MIH Header Fixed Fields (SID=3, Opcode=1, AID=8)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

Target PoA (PoA TLV)

LinkActionList (Link Action List TLV)

Assigned Resource Set (Assigned Resource Set TLV)

Add the following to Table C-2: Type values for TLV encoding

| TLV Type Name | TLV Type Value | Data Type |
| --- | --- | --- |
| MN Link ID | (assigned by Editor) | LINK_ID |
| Requested Resource Set | (assigned by Editor) | REQUESTED_LINK_RESOURCE_SET |
| Assigned Resource Set | (assigned by Editor) | ASSIGNED_LINK_RESOURCE_SET |

Page 184, line 32.

Add at the end of table B-4 (Data type for link identification and manipulation) the following fields

| Data Type Name | Derived From | Definition | Valid Range |
| --- | --- | --- | --- |
| REQUESTED_RESOURCE_SET | SEQUENCE (QOS_LIST, SOURCE2TARGET_TRANSPARENT_CONTAINER, HANDOVER_CAUSE) | Set of resource parameters required for performing admission control and resource reservation for the MN at the target network | N/A |
| ASSIGNED_RESOURCE_SET | SEQUENCE (QOS_ LIST, TARGET2SOURCE_TRANSPARENT_CONTAINER, LINK_AC_EXEC_TIME) | Set of resource parameters reserved and assigned by the target network to the MN for performing handover to a network PoA | N/A |
| HANDOVER_CAUSE | ENUMERATED | Represents the reason for performing a handover | See Table B-7 |
| SOURCE2TARGET_TRANSPARENT_CONTAINER | SEQUENCE (S2T_CONTAINER_TYPE, S2T_CONTAINER_VALUE) | Source to target transparent container including the required MN configuration for admitting the new connection at the target network and reserving resources | N/A |
| S2T_CONTAINER_TYPE | ENUMERATED | Type of source to target container | 0:Parameter not available 1:Pre-defined configuration 2: Transparent carrier 3-255: Reserved |
| S2T_CONTAINER_VALUE | CHOICE (NULL, PREDEFINED_CONFIG_ID, TRANSPARENT_CARRIER, …) | A value associated to the source to target container type | N/A |
| PREDEFINED_CONFIG_ID | INTEGER(1) | Pre-defined configuration identifier | 0-255 |
| TRANSPARENT_CARRIER | OCTET_STRING | Transparent carrier containing link specific information which content and format is specified by the link-specific SDO | N/A |
| TARGET2SOURCE_TRANSPARENT_CONTAINER | SEQUENCE (T2S_CONTAINER_TYPE, T2S_CONTAINER_VALUE) | Target to source transparent container including the required configuration of the reserved resources at the target network | N/A |
| T2S_CONTAINER_TYPE | ENUMERATED | Type of target to source container | 0:Parameter not available 1:Pre-defined configuration 2: Transparent carrier 3-255: Reserved |
| T2S_CONTAINER_VALUE | CHOICE (NULL, PREDEFINED_CONFIG_ID, TRANSPARENT_CARRIER, …) | A value associated to the source to target container type | N/A |