| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Use case scenarios, available features and potential approach for Pre-authentication |  |
| Date Submitted | November 05, 2007 |  |
| Source(s) | Subir Das (Telcordia), Yoshihiro Ohba (Toshiba) |  |
| Re: | IEEE 802.21 Plenary Meeting in Atlanta, Nov 11-16, 2007 |  |
| Abstract | This document describes a few use case scenarios, requirements and high level approaches that can be added to a Technical Report produced by the 802.21 Security Study Group |  |
| Purpose | Discussion |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Use Case Scenarios, Available features and Potential Approach for Pre-authentication

# Introduction

This document describes a few scenarios and presents several high-level parameters that need to be looked into in support for pre-authentication. It also highlights the potential approach for pre-authentication. These scenarios and parameters can be discussed and improved for the inclusion in the study group technical contribution to be developed in the 802.21 Security Study Group.

# Use Case Scenarios

- Scenario #1:  Figure 2-1and Figure 2-2capture the scenarios whereby handover happens between WIMAX access and WLAN access networks whereby both access networks use EAP-based authentication mechanism. In particular, Figure 2-1 depicts the single administrative domain scenario while Figure 2-2depicts multiple administrative domain scenarios. An administrative domain is a logical network that is administered by a single authority using its own authentication and authorization mechanisms.

Figure 2-1: Handover Scenario between WLAN and WiMAX networks (Single Domain)

Figure 2-2:  Handover Scenario between WLAN and WiMAX networks (Multi-Domain)

- Scenario #2: Figure 2-3 and Figure 2.4　capture the scenarios whereby handover happens from 3GPP/3GPP2 access network to WLAN access network whereby WLAN access network uses EAP-based authentication mechanism and 3GPP/3GPP2 access network allows the mobile to initiate an EAP-based authentication mechanism when the mobile is connected to either 3GPP or 3GPP2 access networks.

Figure 2-3: Handover Scenario From  3GPP/PP2  to WLAN Networks (Single Domain)

In particular, Figure 2-3depicts the single administrative domain scenario while depicts multiple administrative domain scenarios.

Figure 2-4: Handover Scenario From  3GPP/PP2  to WLAN Networks (Single Domain)

- Scenario #3:  Figure 2-5captures a scenario whereby handover happens from 3GPP/3GPP2 access network to WiMAX access network whereby WiMAX access network uses EAP-based authentication mechanism and either 3GPP or 3GPP2 access network allows the mobile to initiate an EAP-based authentication mechanism when the mobile is connected to either 3GPP or 3GPP2 access networks. In this scenario both access networks are under one administrative domain.

Figure 2-5: Handover Scenario From 3GPP/PP2 to WiMAX Networks

- Scenario #4: Figure 2-6captures a scenario whereby handover happens from  3GPP/3GPP2 access network to WiMAX access network whereby WiMAX access network uses EAP-based authentication mechanism and either 3GPP or 3GPP2 access network allows the mobile to initiate an EAP-based authentication mechanism when the mobile is connected to either 3GPP or 3GPP2 access networks. However this scenario is different from Scenario #3 since both access networks are under different administrative domains.

Figure 2-6: Handover Scenario From 3GPP/PP2 to WiMAX

- Additionally, following  scenarios may be possible, however, it requires further investigations:

  - Handover between two WiMAX access networks that are under different administrative domains

  - Handover between two ESSes of WLAN access networks or between two mobility domains of WLAN access networks.

# High Level Requirements

The use cases described above assume that the target network uses an EAP-based authentication mechanism. Thus the approach described here uses EAP protocol and its methods to perform the pre-authentication. Following are the high level principles that may be used to perform the pre-authentication:

- Pre-authentication architecture should follow the current 802.21 functional architecture in that serving and candidate POAs/PoSes are the serving and target authenticators, respectively.  Therefore the reference points R1, R2 and R5  should be involved in authentication based transition

- Mobile should use EAP-based authentication mechanism for pre-authentication

- Mobile node should be able to discover and connect to the target authenticator via the serving authenticator. However, serving authenticator is unaware of the pre-authentication. For example, this can be achieved either via EAP lower or via higher layer support.

- Mobile node should be able to discover and connect to the target authenticator via the serving authenticator and serving authenticator takes part in the pre-authentication process. For example, this can be achieved either via EAP lower or via higher layer support.

- Mobile node and target authenticator are considered as the EAP peer and  EAP authenticator, respectively.

# Potential Approaches

There are two high level approaches that can be derived from the above requirements: i) Direct Pre-authentication and ii) Indirect Pre-authentication. Figure 4-7 gives a mapping of  pre-authentication functional elements to 802.21 functional architecture in which serving POA/PoS is the serving authenticator  and candidate POA/PoS is the target authenticator.

Figure 4-7:  Mapping of  Pre-authentication Functional Elements to 802.21 Functional Architecture

### Direct Pre-Authentication

Figure 4-8 gives an overview of direct pre-authentication whereby MN discovers and connects the target authenticator via the serving network but the serving network does not have any knowledge of authentication that is happening between the MN and the target authenticator. MN performs EAP-based authentication mechanism and methods that are supported by the target network.

Serving PoA

M

I

H

MN

R1

M

I

H

P

oS

MIH PoS

R5

Candidate PoA

Home AAA

Server

MN-CA Signaling

EAP over L2 or

higher layers (HL)

EAP over AAA

R2

Figure 4-8: Direct Pre-authentication

  - Indirect Pre-Authentication

Figure 4-9 gives an overview of direct pre-authentication whereby MN discovers and connects the target authenticator via the serving network and the serving network has the knowledge of authentication that is happening between the MN and the target authenticator. Thus serving network forwards the authentication message to the target authenticator. MN performs EAP-based authentication mechanism and methods that are supported by the target network.

Serving PoA

M

I

H

MN

R1

R2

MIH PoS

R5

Candidate PoA

Home AAA

Server

MN-SA Signaling

EAP over L2/HL

EAP over AAA

SA-CA Signaling

EAP over L2/HL

SA  Serving Access Network

CA  Candidate Access Network

MIH PoS

Figure 4-9: Indirect Pre-authentication

# Additional Considerations

This section presents a snap-shot of several parameters that need to be understood and/or further studied in support for pre-authentication. Table 1 captures the available features of access networks and highlights them with reference to pre-authentication. For example, native pre-authentication means that access layer has support for pre-authentication. Impact on lower layer means either access layer needs to support additional functionalities or need further modifications.

Table 1:  Snap-shot of Access Networks Available Features

| Access Networks | Network Access Authentication Mechanism | Native Pre-authentication Support | Impact on Lower layers |
| --- | --- | --- | --- |
| Wi-Fi | EAP-based | Yes (within the same LAN) | Yes |
| WiMAX | EAP-based | No | Yes |

Table 2 captures the handover scenarios and corresponding available features for pre-authentication. It also highlights the lower layer support that is available and what additional work is needed.

Table 2:  Snap-shot of  Handover Scenarios and  Available features

| Handover Scenarios | EAP-based pre-authentication possible | Lower Layer Support Available | Need Additional Work |
| --- | --- | --- | --- |
| WiFi to WiMAX | Yes | No | May be |
| WiMAX to WiFi | Yes | No | Yes |
| 3GPP2 toWiMAX | Yes | No | Yes |
| 3GPP toWiMAX | Yes | No | Yes |

# Example Call Flows

TBD

# References

- “Technical Requirements document for MIH Security”, http://www.ieee802.org/21/doctree/March2007/802.21_Security_SG_Proposal_march2007.ppt

# Annex A:

  - Broad Market Potential

Pre-authentication is very critical to realize the required user performance during session handover.  The technology can be used by both 802.11 an 802.16 wireless community as well as 3G operators that are willing to interwork with these wireless technologies. Thus it has a broad market potential for multiple vendors and numerous users if developed correctly. Moreover, in most cases it only requires a software upgrade.

  - Compatibility

Since the proposed approach is EAP-based, it is compatible to 802.11 and 802.16 access authentication schemes. It is also compatible with 802.1 and 802.21　architecture.

  - Distinct Identity

This has its own distinct identity since no Standards organization within 802 and outside are addressing the authentication based transition mechanism during handover between heterogeneous access networks.

  - Technical Feasibility

The proposed project is highly feasible since the base authentication protocol (i.e., EAP) is widely deployed and used in each vertical technology for access authentication. Furthermore, the proposed solution is an optimization to the existing proven technology and thus it is expected that it will have at least similar confidence in reliability.

  - Economic Feasibility

This is economically feasible since it mostly requires a software upgrade. So the cost should be reasonable for performance. Since this is an optimization technique to existing technology, not every system may  be required to have this capability if cost of upgrading the software becomes an issue.