| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH Protocol Version Issue |  |
| DCN | 21-07-0395-00-0000 |  |
| Date Submitted | November 11, 2007 |  |
| Source(s) | Yoshihiro Ohba |  |
| Re: | IEEE 802.21 Session 23 in Atlanta |  |
| Abstract | This document addresses SB Comment #205 on MIH Protocol Version field. |  |
| Purpose | Sponsor Ballot comment resolution. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

# Issue on MIH Protocol Version field

Original Comment: “Version field is not useful unless the specification completely specifies how MIHFs can negotiate on a single version.”

The comment was rejected because “This field allows implementations to recognize different versions.”

However, still there is an issue as to what an MIH node should process an MIH message with an unsupported version.

- Currently there is only one version. However, higher versions of this protocol may be defined in a future, which can result an implementation based on the current version receiving an MIH message with a higher version. An appropriate behavior when receiving an MIH message with a different version needs to be described in the current specification.

- Guidance is missing as to which case a new version should be used.

# Suggested Text

Change the Description column of Version field in Table 15 as follows:

“

This field is used to specify the version of MIH protocol used.

0: Not to be used

1: The current version

2 - 15: (Reserved)

The version number will be incremented only when a fundamental incompatibility exists between a new revision and the prior edition of the standard. An MIH node that receives an MIH message with a higher version number than it supports will discard the frame without indication to the sending MIH node.

"