| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Issue 17 |  |
| DCN | 21-07-0396-00-0000 |  |
| Date Submitted | November 13, 2007 |  |
| Source(s) | Yoshihiro Ohba, Yuu-Heng Alice Cheng and Subir Das |  |
| Re: | IEEE 802.21 Session 23 in Atlanta |  |
| Abstract | This document addresses Issue 17 (SB Comment #576,577,578) on MIH Protocol Version field. |  |
| Purpose | Sponsor Ballot comment resolution. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

# Issues

Clarification text is needed in terms of transmission order of bits and octets in data type encoding.

Original Comments and Proposed Resolutions:

Comment 576:

"Each bit of a BITMAP(N) value [N=8*i, i=1, 2, &] is encoded as an N/ 8-octet value in order of significance." Increasing or decreasing order of significance?

Original Proposed Resolution: "A one-octet Selector field, followed by a variable length Value field. The Selector value determines the data type. If Selector==i, (i+1)-th data type in the list of data types DATATYPE1,DATATYPE2,[,&] is selected. The Selector field is encoded in increasing order of significance. The Value field is encoded using the encoding rule for the selected data type."

Comment 577:

"Each character is encoded in order of appearance where each bit of each character is encoded in order of significance." Increasing or decreasing order of significance?

Original Proposed Resolution: "Each character is encoded in order of appearance where each bit of each character is encoded in increasing order of significance."

Comment 578:

"A one-octet Selector field, followed by a variable length Value field. The Selector value determines the data type. If Selector==i, (i+1)-th data type in the list of data types DATATYPE1,DATATYPE2,[,&] is selected. The Value field is encoded using the encoding rule for the selected data type."

What is the binary encoding of the selector octet? Order ov value? Increasing order or decreasing order?

Original Proposed Resolution:

"A one-octet Selector field, followed by a variable length Value field. The Selector value determines the data type. If Selector==i, (i+1)-th data type in the list of data types DATATYPE1,DATATYPE2,[,&] is selected. The Selector field is encoded in increasing order of significance. The Value field is encoded using the encoding rule for the selected data type."

# Proposed Text

# Text can be clarified with less words by using the term “network byte order”.

Comment 576: Modify the Proposed Resolution as follows: "A BITMAP(N) value

[N=8*i, i=1, 2, &] is encoded as an N/8-octet value in network byte order.

Comment 577: Modify the Proposed Resolution as follows: "The N-octet characters are encoded in network byte order."

Comment 578: Modify the Proposed Resolution as follows:

"A one-octet Selector field, followed by a variable length Value

field. The Selector value determines the data type. If Selector==i,

(i+1)-th data type in the list of data types DATATYPE1,DATATYPE2,[,&]

is selected. The Selector value is encoded as UNSIGNED_INT(1) data.

The Value field is encoded using the encoding rule for the selected data type."

Note that encoding rule for UNSIGNED_INT(1) is defined elsewhere.