| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH services for MRPM |  |
| DCN | 21-07-0410-00-0000 |  |
| Date Submitted | Nobember 9, 2007 |  |
| Source(s) | Yoon-Young An (ETRI) Junghoon Jee (ETRI) |  |
| Re: | IEEE 802.21 Session #23 in Atlanta |  |
| Abstract | This contribution proposes the MIH services extension for MRPM, |  |
| Purpose | Adoption |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

Apply the following change to the section 5.1 of contribution, 21-07-0335-01as suggested below: [Previous texts are black and added texts are red]

5.1	Multi-Radio Power Management Enabled Media Independent Handover (MIH) Function

The MIH Function resides just below the IP layer and is uniform across bearer types. The MIH Function can use inputs from layer 2 such as trigger events, hints, access to information about different networks and can help in the handover decision making process. MIH Function would also include inputs based on user policy and configuration that shall affect the handover process.

MRPM-Enabled MIHF will be an extension of MIHF defined in 802.21. 802.21 defines three services: Event, Command and Information services. MRPM will define extensions to these three services. Such an entity will be called MRPM-Enabled MIHF. MRPM Enabled MIHF reference model is shown in Figure.2

Figure 2.  MRPM Reference Model

Generally, a network interface with low power consumption or active state is utilized for power management for the multi-radio terminal. MIHF extensions to support the Multi-Radio Power Management Application are like below:

- MIH Information Service extension

MIH user requests and responds the policy and configuration information for MRPM. Therefore, this service needs new information elements.

- MIH Event Service extension

Event Services provides the change of power state from each interfaces, such as sleep mode, idle mode or active mode and informs paging for another interface.

- MIH Command Service extension

MIH user sends the command for MRPM to selected interface and the command for power mode change to called interface (i.e. idle mode to active mode).