| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |
| --- | --- |
| Title | 802.21 MIB |
| Date Submitted | March 27, 2008 |
| Source(s) | Yoshihiro Ohba (Toshiba), Miriam Tauil (Telcordia) and Yuu-Heng Alice Cheng (Telcordia) |
| Re: | IEEE 802.21 Session #25 in March 2008 |
| Abstract | This document describes 802.21 MIB |
| Purpose | Sponsor Ballot comment resolution |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

# Introduction

A MIB is required to expose internal parameters that govern the operation and internal state/statistics that allows an external management entity to monitor the operation of the protocol entity. This document identifies a list of internal parameters for which a MIB would need to be defined and defines a MIB for such a list.

# Parameters Requiring MIB Definition

In this specification, only parameters that govern the operation of 802.21 are defined.

## MIH capability parameters

Following is a list of parameters that is used in the MIH capability discovery:

- Link MACs (Read-only): A list of network type and MAC address pair that is controlled by this MIHF. Note that not all interfaces an MIH-capable node may be under control of MIHF.

- MIH Event List (Read-only): A list of supported events by this MIHF.

- MIH Command List (Read-only): A list of supported commands by this MIHF.

- MIH IS Query List (Read-only): A list of supported MIH IS query types by this MIHF.

- MIH Transport List (Read-only): A list of supported MIH transport protocols by this MIHF. This is the transport that transmits the MIH protocol messages.

- List of MBB Handover support (Read-only): A list of make-before-break support information for each pair of serving and target network types.

## MIH protocol parameters

Following is a list of parameters that is used for MIH protocol.

- Local MIHF ID (Read-Write) the local MIHF ID.

- List of Peer MIHF IDs (Read-Write)

- Transport Type (L2 or L3) (Read-Write, for each peer MIHF)

- Source Transport Address (link-layer/IPv4/IPv6 address, Read-Write)

- Destination Transport  Address (link-layer/IPv4/IPv6 address, Read-Write)

- Version (Read-only)

- Maximum Transaction Lifetime (Read-Write)

  - Unit: seconds

- Maximum Retransmission Interval (Read-Write)

  - Implementations may dynamically adjust retransmission interval up to this value

  - Unit: seconds

- Maximum Retransmission Counter (Read-Write)

  - Unit: none

- Maximum Average Transmission Rate (Read-Write)

  - Maximum value of average transmission rate

  - Unit: Octets per second

- Maximum Burst Size (Read-Write)

  - The maximum number of octets transmitted in a burst

  - Unit: octets

# 802.21 MIB Definition (revised)

IEEE802dot21-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE, Integer32, IpAddress FROM SNMPv2-SMI

MODULE-COMPLIANCE, OBJECT-GROUP FROM SNMPv2-CONF

TEXTUAL-CONVENTION, MacAddress, TruthValue FROM SNMPv2-TC

Ipv6Address FROM IPV6-TC

ifEntry FROM RFC1213-MIB;

-- **********************************************************************

-- * MODULE IDENTITY

-- **********************************************************************

ieee802dot21 MODULE-IDENTITY

LAST-UPDATED "200803252200Z"

ORGANIZATION "IEEE 802.21"

CONTACT-INFO

"WG E-mail: stds-802-21@ieee.org

Chair: Vivek G. Gupta

Intel Corporation

E-mail: mailto:vivek.g.gupta@intel.com

Editor: Qiaobing Xie

E-mail: Qiaobing.Xie@MOTOROLA.COM"

DESCRIPTION

"The MIB module for IEEE 802.21 entities.

iso(1).std(0).iso8802(8802).ieee802dot21(21)"

REVISION    "200803252200Z"

DESCRIPTION

"The latest version of this MIB module."

::= { iso std(0) iso8802(8802) ieee802dot21(21) }

-- **********************************************************************

-- * Textual Conventions

-- **********************************************************************

Dot21MihfID ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The MIHF ID of an MIH node."

REFERENCE "IEEE Std 802.21, 2008 Edition, B.3.11"

SYNTAX OCTET STRING (SIZE(0..255))

Dot21LinkType ::= TEXTUAL-CONVENTION

DISPLAY-HINT "d"

STATUS	current

DESCRIPTION

"This attribute represents the type of a link."

REFERENCE "IEEE Std 802.21, 2008 Edition, B.3.4"

SYNTAX Integer32 (0..255)

Dot21NetworkRevision ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This attribute represents the network revision of a link."

REFERENCE "IEEE Std 802.21, 2008 Edition, B.3.8"

SYNTAX OCTET STRING (SIZE(0..8))

Dot21NetworkTypeExtension ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This attribute represents a network type extension."

REFERENCE "IEEE Std 802.21, 2008 Edition, B.3.8"

SYNTAX OCTET STRING (SIZE(0..253))

Dot21EventList ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This attribute represents a list of supported events."

REFERENCE "IEEE Std 802.21, 2008 Edition, B.3.12"

SYNTAX BITS

{ mihLinkUp(0),

mihLinkDown(1),

mihLinkGoingDown(2),

mihLinkDetected(3),

mihLinkParametersReport(4),

mihLinkHandoverImminent(5),

mihLinkHandoverComplete(6) }

Dot21CommandList ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This attribute represents a list of supported commands."

REFERENCE "IEEE Std 802.21, 2008 Edition, B.3.12"

SYNTAX BITS

{ mihGetLinkParameters(0),

mihLinkConfigureThresholds(1),

mihLinkActions(2),

mihNetHoCandidateQuery(3),

mihNetHoCommit(4),

mihN2nHoQueryResources(5),

mihN2nHoCommit(6),

mihN2nHoComplete(7),

mihMnHoCandiateQuery(8),

mihMnHoCommit(9),

mihMnHoComplete(10) }

Dot21ISQueryTypeList ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

" This attribute shall be a set of supported MIH IS query types."

REFERENCE "IEEE Std 802.21, 2008 Edition, B.3.12"

SYNTAX BITS

{ binary(0),

rdfData(1),

rdfSchemaUrl(2),

rdfSchema(3),

typeIeNetworkType(4),

typeIeOperatorIdentifier(5),

typeIeServiceProviderIdentifier(6),

typeIeCountryCode(7),

typeIeAccessNetworkIdentifier(8),

typeIeAccessNetworkAuxIdentifier(9),

typeIeRoamingPartners(10),

typeIeCost(11),

typeIeNetworkQos(12),

typeIeNetworkDataRate(13),

typeIeNetworkRegulatoryDomain(14),

typeIeNetworkFrequencyBands(15),

typeIeNetworkIpConfigMethods(16),

typeIeNetworkCapabilities(17),

typeIeListSupportedLcp(18),

typeIeNetworkMobilityManagementProtocol(19),

typeIeNetworkEmergencyServiceProxy(20),

typeIeNetworkImsProxyCscf(21),

typeIeNetworkMobileNetwork(22),

typeIePoaMacAddress(23),

typeIePoaLocation(24),

typeIePoaChannelRange(25),

typeIePoaSystemInformation(26),

typeIePoaSubnetInformation(27),

typeIePoaIpAddress(28) }

Dot21TransportList ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

" This attribute shall be a set of supported MIH transports."

REFERENCE "IEEE Std 802.21, 2008 Edition, B.3.12"

SYNTAX BITS { udp(0), tcp(1) }

-- **********************************************************************

-- * Major sections

-- **********************************************************************

-- MIH Function Management (MIHMT) Attributes

-- DEFINED AS "The MIHMT object class provides the necessary support

-- at the MIHF to manage the processes in the station such that

-- the MIHF may work cooperatively as a part of an IEEE 802.21

-- network."

dot21mihmt OBJECT IDENTIFIER ::= { ieee802dot21 1 }

-- dot21mihmt GROUPS

-- dot21LocalMihfTable ::= { dot21mihmt 1 }

-- dot21PeerMihfTable ::= { dot21mihmt 2 }

-- dot21IfTable ::= { dot21mihmt 3 }

-- dot21MbbHandoverSupportTable ::= { dot21mihmt 4 }

-- **********************************************************************

-- * MIB attribute OBJECT-TYPE definitions follow

-- **********************************************************************

--

-- Local MIHF Table

--

dot21LocalMihfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF Dot21LocalMihfEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"The table of local MIHFs. The MIH MIB allows to have more than one local MIHFs per SNMP engine."

::={ dot21mihmt 1 }

dot21LocalMihfEntry	OBJECT-TYPE

SYNTAX	Dot21LocalMihfEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"The value contains information associated with a particular local MIHF. In most cases, there will be only one local MIHF on a node."

INDEX	{ dot21LocalMihfIndex }

::={ dot21LocalMihfTable 1}

Dot21LocalMihfEntry ::=

SEQUENCE{

dot21LocalMihfIndex Integer32,

dot21LocalMihfID Dot21MihfID,

dot21LocalEventList BITS,

dot21LocalCommandList BITS,

dot21LocalISQueryTypeList BITS,

dot21LocalTransportList BITS,

dot21LocalVersion Integer32,

dot21LocalMaximumTransactionLifetime Integer32,

dot21LocalMaximumRetransmissionInterval Integer32,

dot21LocalMaximumRetransmissionCounter Integer32,

dot21LocalMaximumAverageTransmissionRate Integer32,

dot21LocalMaximumBurstSize Integer32

}

dot21LocalMihfIndex OBJECT-TYPE

SYNTAX	Integer32(0..2147483647)

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Index of local MIHF table."

::= { dot21LocalMihfEntry 1 }

dot21LocalMihfID OBJECT-TYPE

SYNTAX Dot21MihfID

MAX-ACCESS read-write

STATUS	current

DESCRIPTION

"The MIHF ID of this node."

REFERENCE "IEEE Std 802.21, 2008 Edition, B.3.11"

::={ dot21LocalMihfEntry 2 }

dot21LocalEventList OBJECT-TYPE

SYNTAX	Dot21EventList

MAX-ACCESS read-only

STATUS	current

DESCRIPTION

" This attribute shall be a set of all the MIH events supported by this MIH node."

DEFVAL { {} }

::={ dot21LocalMihfEntry 3 }

dot21LocalCommandList OBJECT-TYPE

SYNTAX	Dot21CommandList

MAX-ACCESS read-only

STATUS	current

DESCRIPTION

" This attribute shall be a set of all the MIH commands supported by this MIH node."

DEFVAL { {} }

::={ dot21LocalMihfEntry 4 }

dot21LocalISQueryTypeList OBJECT-TYPE

SYNTAX	Dot21ISQueryTypeList

MAX-ACCESS read-only

STATUS	current

DESCRIPTION

" This attribute shall be a set of MIH IS query types supported by this MIH node."

DEFVAL { {} }

::={ dot21LocalMihfEntry 5 }

dot21LocalTransportList OBJECT-TYPE

SYNTAX	Dot21TransportList

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

" This attribute shall be a set of MIH transports supported by this MIH node."

DEFVAL { {} }

::={ dot21LocalMihfEntry 6 }

dot21LocalVersion OBJECT-TYPE

SYNTAX	Integer32 (1..255)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The MIH protocol version supported by this MIHF."

DEFVAL { 1 }

::={ dot21LocalMihfEntry 7 }

dot21LocalMaximumTransactionLifetime OBJECT-TYPE

SYNTAX	Integer32 (1..255)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The maximum time in seconds for a MIH protocol transaction."

DEFVAL { 30 }

::={ dot21LocalMihfEntry 8 }

dot21LocalMaximumRetransmissionInterval OBJECT-TYPE

SYNTAX	Integer32 (1..10000)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The maximum time in milliseconds for retransmitting a MIH message."

DEFVAL { 10000 }

::={ dot21LocalMihfEntry 9 }

dot21LocalMaximumRetransmissionCounter OBJECT-TYPE

SYNTAX	Integer32 (1..255)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The maximum amount of retransmission retries for MIH messages."

DEFVAL { 2 }

::={ dot21LocalMihfEntry 10 }

dot21LocalMaximumAverageTransmissionRate OBJECT-TYPE

SYNTAX	Integer32 (0..255)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The maximum amount MIH messages can be transmitted per second on this node. If the value is 0, no limitation is set."

DEFVAL { 0 }

::={ dot21LocalMihfEntry 11 }

dot21LocalMaximumBurstSize OBJECT-TYPE

SYNTAX	Integer32 (0..255)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

" The maximum number of octets transmitted in a burst. If the value is 0, no limitation is set."

DEFVAL { 0 }

::={ dot21LocalMihfEntry 12 }

--

-- The Peer MIHF Table

--

dot21PeerMihfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF Dot21PeerMihfEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"The table of MIHF known by this MIHF."

::={ dot21mihmt 2 }

dot21PeerMihfEntry	OBJECT-TYPE

SYNTAX	Dot21PeerMihfEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Details of a specific MIHF peer."

INDEX	{dot21PeerMihfIndex}

::= { dot21PeerMihfTable 1 }

Dot21PeerMihfEntry ::=

SEQUENCE {

dot21PeerMihfIndex Integer32,

dot21PeerMihfID	Dot21MihfID,

dot21PeerLocalMihfID Dot21MihfID,

dot21PeerEventList BITS,

dot21PeerCommandList BITS,

dot21PeerISQueryTypeList BITS,

dot21PeerTransportList BITS,

dot21PeerTransportType	Integer32,

dot21PeerSrcLinkAddress	MacAddress,

dot21PeerDstLinkAddress	MacAddress,

dot21PeerSrcIpv4Address	IpAddress,

dot21PeerDstIpv4Address	IpAddress,

dot21PeerSrcIpv6Address	Ipv6Address,

dot21PeerDstIpv6Address	Ipv6Address,

dot21PeerVersion Integer32,

dot21PeerMaximumTransactionLifetime Integer32,

dot21PeerMaximumRetransmissionInterval Integer32,

dot21PeerMaximumRetransmissionCounter Integer32,

dot21PeerMaximumAverageTransmissionRate Integer32,

dot21PeerMaximumBurstSize Integer32

}

dot21PeerMihfIndex OBJECT-TYPE

SYNTAX	Integer32(0..2147483647)

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Index of peer MIHF table."

::= { dot21PeerMihfEntry 1 }

dot21PeerMihfID OBJECT-TYPE

SYNTAX	Dot21MihfID

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The MIHF ID of a peer MIH node."

::={ dot21PeerMihfEntry 2 }

dot21PeerLocalMihfID OBJECT-TYPE

SYNTAX	Dot21MihfID

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The MIHF ID of the local MIH node for this peer MIH node."

::={ dot21PeerMihfEntry 3 }

dot21PeerEventList OBJECT-TYPE

SYNTAX	Dot21EventList

MAX-ACCESS read-only

STATUS	current

DESCRIPTION

" This attribute shall be a set of all the MIH events supported by peer MIH node."

DEFVAL { {} }

::={ dot21PeerMihfEntry 4 }

dot21PeerCommandList OBJECT-TYPE

SYNTAX	Dot21CommandList

MAX-ACCESS read-only

STATUS	current

DESCRIPTION

" This attribute shall be a set of all the MIH commands supported by peer MIH node."

DEFVAL { {} }

::={ dot21PeerMihfEntry 5 }

dot21PeerISQueryTypeList OBJECT-TYPE

SYNTAX	Dot21ISQueryTypeList

MAX-ACCESS read-only

STATUS	current

DESCRIPTION

" This attribute shall be a set of MIH IS query types supported by peer MIH node."

DEFVAL { {} }

::={ dot21PeerMihfEntry 6 }

dot21PeerTransportList OBJECT-TYPE

SYNTAX	Dot21TransportList

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

" This attribute shall be a set of MIH transports supported by peer MIH  node."

DEFVAL { {} }

::={ dot21PeerMihfEntry 7 }

dot21PeerTransportType OBJECT-TYPE

SYNTAX	INTEGER { layerTwo(1), layerThreeIpv4(2), layerThreeIpv6(3) }

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"This value should be set for the MIH protocol layer used for transmitting MIH messages."

DEFVAL { 1 }

::= {dot21PeerMihfEntry 8 }

dot21PeerSrcLinkAddress OBJECT-TYPE

SYNTAX	MacAddress

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"This value indicates the source link-layer address used for transmitting MIH messages using L2 transport."

::= {dot21PeerMihfEntry 9 }

dot21PeerDstLinkAddress OBJECT-TYPE

SYNTAX	MacAddress

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"This value indicates the destination link-layer address used for transmitting

MIH messages using L2 transport."

::= {dot21PeerMihfEntry 10 }

dot21PeerSrcIpv4Address OBJECT-TYPE

SYNTAX	IpAddress

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"This value indicates the source IPv4 address used for transmitting MIH

messages using IPv4 transport."

::= {dot21PeerMihfEntry 11 }

dot21PeerDstIpv4Address OBJECT-TYPE

SYNTAX	IpAddress

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"This value indicates the destination IPv4 address used for transmitting MIH messages using IPv4 transport."

::= {dot21PeerMihfEntry 12 }

dot21PeerSrcIpv6Address OBJECT-TYPE

SYNTAX	Ipv6Address

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"This value indicates the source IPv6 address used for transmitting MIH

messages using IPv6 transport."

::= {dot21PeerMihfEntry 13 }

dot21PeerDstIpv6Address OBJECT-TYPE

SYNTAX	Ipv6Address

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"This value indicates the destination IPv6 address used for transmitting MIH

messages using IPv6 transport."

::= {dot21PeerMihfEntry 14 }

dot21PeerVersion OBJECT-TYPE

SYNTAX	Integer32 (1..255)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The MIH protocol version supported by peer MIHF. The default version is 1."

DEFVAL { 1 }

::={ dot21PeerMihfEntry 15 }

dot21PeerMaximumTransactionLifetime OBJECT-TYPE

SYNTAX	Integer32 (1..255)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The maximum time in seconds for a MIH protocol transaction used for a  particular peer MIHF."

DEFVAL { 30 }

::={ dot21PeerMihfEntry 16 }

dot21PeerMaximumRetransmissionInterval OBJECT-TYPE

SYNTAX	Integer32 (1..10000)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The maximum time in milliseconds for retransmitting a MIH message used for a particular peer MIHF."

DEFVAL { 10000 }

::={ dot21PeerMihfEntry 17 }

dot21PeerMaximumRetransmissionCounter OBJECT-TYPE

SYNTAX	Integer32 (1..255)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The maximum amount of retransmission retries for MIH messages used for a particular peer MIHF."

DEFVAL { 2 }

::={ dot21PeerMihfEntry 18 }

dot21PeerMaximumAverageTransmissionRate OBJECT-TYPE

SYNTAX	Integer32 (0..255)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The maximum amount MIH messages can be transmitted per second on this node for a particular peer MIHF. If the value is 0, no limitation is set."

DEFVAL { 0 }

::={ dot21PeerMihfEntry 19 }

dot21PeerMaximumBurstSize OBJECT-TYPE

SYNTAX	Integer32 (0..255)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The maximum number of octets transmitted in a burst. If the value is 0, no limitation is set."

DEFVAL { 0 }

::={ dot21PeerMihfEntry 20 }

--

-- MIH Interface Table

--

dot21IfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF Dot21IfEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"The table of Dot21IfEntry."

::={ dot21mihmt 3 }

dot21IfEntry	OBJECT-TYPE

SYNTAX	Dot21IfEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"The value contains information associated with a particular link controlled by

this MIHF."

AUGMENTS { ifEntry }

::={ dot21IfTable 1}

Dot21IfEntry ::=

SEQUENCE{

dot21MihSupported TruthValue

}

dot21MihSupported OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represends whether a particular interface supports MIH."

::= { dot21IfEntry 1 }

--

-- The Make-Before-Break Handover Support Table

--

dot21MbbHandoverSupportTable OBJECT-TYPE

SYNTAX	SEQUENCE OF Dot21MbbHandoverSupportEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"The table of make-before-break handover support entries."

::={ dot21mihmt 4 }

dot21MbbHandoverSupportEntry	OBJECT-TYPE

SYNTAX	Dot21MbbHandoverSupportEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"The value contains information associated with a particular MBB support."

INDEX	{ dot21MbbHandoverSupportIndex }

::={ dot21MbbHandoverSupportTable 1 }

Dot21MbbHandoverSupportEntry ::=

SEQUENCE{

dot21MbbHandoverSupportIndex Integer32,

dot21FromLinkType Dot21LinkType,

dot21FromNetworkRevision Dot21NetworkRevision,

dot21FromNetworkTypeExtension Dot21NetworkTypeExtension,

dot21ToLinkType Dot21LinkType,

dot21ToNetworkRevision Dot21NetworkRevision,

dot21ToNetworkTypeExtension Dot21NetworkTypeExtension,

dot21IsMbbSupported TruthValue

}

dot21MbbHandoverSupportIndex OBJECT-TYPE

SYNTAX	Integer32(0..2147483647)

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Index of make-before-break handover support table."

::= { dot21MbbHandoverSupportEntry 1 }

dot21FromLinkType OBJECT-TYPE

SYNTAX Dot21LinkType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents the link type of serving link."

DEFVAL { 0 }

::={ dot21MbbHandoverSupportEntry 2 }

dot21FromNetworkRevision OBJECT-TYPE

SYNTAX Dot21NetworkRevision

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents the network revision of serving link."

DEFVAL { ''H }

::={ dot21MbbHandoverSupportEntry 3 }

dot21FromNetworkTypeExtension OBJECT-TYPE

SYNTAX Dot21NetworkTypeExtension

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents the network type extension of serving link."

DEFVAL { ''H }

::={ dot21MbbHandoverSupportEntry 4 }

dot21ToLinkType OBJECT-TYPE

SYNTAX Dot21LinkType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents the link type of target link."

DEFVAL { 0 }

::={ dot21MbbHandoverSupportEntry 5 }

dot21ToNetworkRevision OBJECT-TYPE

SYNTAX Dot21NetworkRevision

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents the network revision of target link."

DEFVAL { ''H }

::={ dot21MbbHandoverSupportEntry 6 }

dot21ToNetworkTypeExtension OBJECT-TYPE

SYNTAX Dot21NetworkTypeExtension

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents the network type extension of target link."

DEFVAL { ''H }

::={ dot21MbbHandoverSupportEntry 7 }

dot21IsMbbSupported OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates whether make-before-break handover is supported. A value of true indicates that make-before-break handover is supported. A value of false indicates that make-before-break handover is not supported."

::={ dot21MbbHandoverSupportEntry 8 }

-- **********************************************************************

-- * Conformance Information

-- **********************************************************************

dot21Conformance OBJECT IDENTIFIER ::= { ieee802dot21 2 }

dot21Groups OBJECT IDENTIFIER ::= { dot21Conformance 1 }

dot21Compliances OBJECT IDENTIFIER ::= { dot21Conformance 2 }

-- **********************************************************************

-- * Compliance Statements

-- **********************************************************************

dot21Compliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement

the IEEE 802.21 MIB."

MODULE -- this module

MANDATORY-GROUPS {

dot21MihmtBase1

}

::= { dot21Compliances 1 }

dot21MihmtBase1 OBJECT-GROUP

OBJECTS {

dot21LocalMihfID,

dot21LocalEventList,

dot21LocalCommandList,

dot21LocalISQueryTypeList,

dot21LocalTransportList,

dot21LocalVersion,

dot21LocalMaximumTransactionLifetime,

dot21LocalMaximumRetransmissionInterval,

dot21LocalMaximumRetransmissionCounter,

dot21LocalMaximumAverageTransmissionRate,

dot21LocalMaximumBurstSize,

dot21PeerMihfID,

dot21PeerLocalMihfID,

dot21PeerEventList,

dot21PeerCommandList,

dot21PeerISQueryTypeList,

dot21PeerTransportList,

dot21PeerTransportType,

dot21PeerSrcLinkAddress,

dot21PeerDstLinkAddress,

dot21PeerSrcIpv4Address,

dot21PeerDstIpv4Address,

dot21PeerSrcIpv6Address,

dot21PeerDstIpv6Address,

dot21PeerVersion,

dot21PeerMaximumTransactionLifetime,

dot21PeerMaximumRetransmissionInterval,

dot21PeerMaximumRetransmissionCounter,

dot21PeerMaximumAverageTransmissionRate,

dot21PeerMaximumBurstSize,

dot21MihSupported,

dot21FromLinkType,

dot21FromNetworkRevision,

dot21FromNetworkTypeExtension,

dot21ToLinkType,

dot21ToNetworkRevision,

dot21ToNetworkTypeExtension,

dot21IsMbbSupported

}

STATUS current

DESCRIPTION

"This object class provides the necessary support at the MIH node to manage the processes in the MIH node so that the MIH node may work cooperatively as a part of an IEEE 802.21 network."

::= { dot21Groups 1 }

END

# Acknowledgments

The authors would like to thank Adrian P Stephens and Michael Montemurro for their valuable comments.