| Project | IEEE 802.21 |  |
| --- | --- | --- |
| Title | Proposal for a new IE: Proxy address for unauthenticated ES access |  |
| DCN | 21-08-0036-00-0000 |  |
| Date Submitted | January 13, 2008 |  |
| Source(s) | Gabor Bajko, Ajay Rajkumar Nokia, Alcatel-Lucent |  |
| Abstract | The contribution proposes a new IE, Proxy address for ES access |  |
| Purpose | approval |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.2 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

A new IE provisioned with the address of a proxy which provides access to Emergency Services would provide the STA with useful information in case of unauthenticated handover between available access networks.

# Problem Statement

STAs may move during an emergency call. When they move, location may change, access network may change, IP address may change, the boundary of  PSAPs may be crossed, outbound (SIP) proxies may be changed, etc.

If the outbound (SIP) proxy which provides access to PSAP for a STA does not changes while handing over from one AN to another, the handover procedure will not involve additional application layer procedures in order to keep session continuity. Thus, mobility during the unauthenticated emergency call can be easily preserved.

A STA in unauthenticated state (state 1) does not have any means to discover the address of a proxy providing access to PSAP, other than the 802.11u defined mechanisms.

Similarly, the mobile has no way to determine the P-CSCF or E-CSCF addresses in an un-authenticated state if a VoIP call is to be initiated and supported by the IMS network.

# Suggested Remendy

[Modify the text as suggested below: Page 50, line 49]

Add a new row into Table 8:

- Type: Access Network Specific Information Elements

- Name of Information Element: TYPE_IE_EmServ_Proxy_Address

- Description: address of the proxy providing access to PSAP

- Data type name: EmServ_Proxy_Address

Add a new row into Table B 13:

- Entry 1

  - Type name: EmServ_Proxy_Address

  - Derived from: CHOICE(IPv4ADDRESS, IPv6ADDRESS)

  - Definition: Address of the proxy providing access to PSAP

  - Valid Range: N/A

- Entry 2

  - Type name: IPv4ADDRESS

  - Derived from: Octet(4)

  - Definition: An IP v4 Address

  - Valid Range: as described in RFC791

- Entry 3

  - Type name: IPv6ADDRESS

  - Derived from: OCTET(16)

  - Definition: An IPv6 Address

  - Valid Range: As described in RFC 2373

Add a type Identifier into Table A-1:

Type: TYPE_IE_EmServ_Proxy_Address

Identifier: 0x1000010C

RDF part: Transport_address (ask Kenichi)

Add a new entry into the Access Network Container list in section 6.5.6.2.1 (IE container):

Type: TYPE_IE_EmServ_Proxy_Address

Length: variable

EmServ_Proxy_Address (optional)

Add the following to the Abbreviation section:

PSAP: Public Safety Answering Point

EmServ: Emergency Service

Add a new row into Table 8:

- Type: Access Network Specific Information Elements

- Name of Information Element: TYPE_IE_Proxy_CSCF_Address

- Description: address of the proxy providing access to P-CSCF

- Data type name: Proxy_CSCF_Address

Add a new row into Table B 13:

- Entry 1

  - Type name: Proxy_CSCF_Address

  - Derived from: CHOICE(IPv4ADDRESS, IPv6ADDRESS)

  - Definition: Address of the P-CSCF

  - Valid Range: N/A

Add a type Identifier into Table A-1:

Type: TYPE_IE_Proxy_CSCF_Address

Identifier: 0x1000010D

RDF part: Transport_address (ask Kenichi)

Add a new entry into the Access Network Container list in section 6.5.6.2.1 (IE container):

Type: TYPE_IE_Proxy_CSCF_Address

Length: variable

Proxy_CSCF_Address (optional)

Add the following to the Abbreviation section:

PCSCF: Proxy CSCF

EmServ: Emergency Service

Add a new row into Table 8:

- Type: Access Network Specific Information Elements

- Name of Information Element: TYPE_IE_EmServ_CSCF_Address

- Description: address of the proxy providing access to E-CSCF

- Data type name: EmServ_CSCF_Address

Add a new row into Table B 13:

- Entry 1

  - Type name: EmServ_CSCF_Address

  - Derived from: CHOICE(IPv4ADDRESS, IPv6ADDRESS)

  - Definition: Address of the ECSCF

  - Valid Range: N/A

Add a type Identifier into Table A-1:

Type: TYPE_IE_EmServ_CSCF_Address

Identifier: 0x1000010E

RDF part: Transport_address (ask Kenichi)

Add a new entry into the Access Network Container list in section 6.5.6.2.1 (IE container):

Type: TYPE_IE_EmServ_CSCF_Address

Length: variable

EmServ_CSCF_Address (optional)

Add the following to the Abbreviation section:

ECSCF: Emergency Services CSCF

EmServ: Emergency Service