| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | 802.21 PICS |  |
| DCN | 21-08-0025-00-0000 |  |
| Date Submitted | January, 2008 |  |
| Source(s) | Vivek Gupta |  |
| Re: | IEEE 802.21 Session #24 in Taipei |  |
| Abstract | Defining a PICS for 802.21 |  |
| Purpose | Define conformance criteria for 802.21 |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

Protocol Implementation Conformance Statement (PICS)

proforma

A.1 Introduction

The supplier of a protocol implementation that is claimed to conform to IEEE Std 802.21, shall complete the following protocol implementation conformance statement (PICS) proforma.

A completed PICS proforma is the PICS for the implementation in question. The PICS is a statement of which capabilities and options of the protocol have been implemented. The PICS can have a number of uses, including use

a) By the protocol implementer, as a checklist to reduce the risk of failure to conform to the standard through oversight;

b) By the supplier and acquirer, or potential acquirer, of the implementation, as a detailed indication of the capabilities of the implementation, stated relative to the common basis for understanding provided by the standard PICS proforma;

c) By the user, or potential user, of the implementation, as a basis for initially checking the possibility of interworking with another implementation (note that, while interworking can never be guaranteed, failure to interwork can often be predicted from incompatible PICS proformas);

d) By a protocol tester, as the basis for selecting appropriate tests against which to assess the claim for conformance of the implementation.

A.2 Abbreviations and special symbols

A.2.1 Symbols for Status column

M 	mandatory

O 	optional

O.<n> 	optional, but support of at least one of the group of options labeled by the same numeral <n> is required

pred: 	conditional symbol, including predicate identification

A.2.2 General abbreviations for Item and Support columns

N/A 	not applicable

A.3 Instructions for completing the PICS proforma

A.3.1 General structure of the PICS proforma

The first parts of the PICS proforma, Implementation identification and Protocol summary, are to be completed as indicated with the information necessary to identify fully both the supplier and the implementation.

The main part of the PICS proforma is a fixed questionnaire, divided into sub-clauses, each containing a number of individual items. Answers to the questionnaire items are to be provided in the rightmost column, either by simply marking an answer to indicate a restricted choice (usually Yes or No) or by entering a value or a set or a range of values. (Note that there are some items where two or more choices from a set of possible answers may apply. All relevant choices are to be marked in these cases.)

Each item is identified by an item reference in the first column. The second column contains the question to be answered. The third column contains the reference or references to the material that specifies the item in the main body of this standard. The remaining columns record the status of each item, i.e., whether support is mandatory, optional, or conditional, and provide the space for the answers. Marking an item as supported is to be interpreted as a statement that all relevant requirements of the sub-clauses and normative annexes, cited in the References column for the item, are met by the implementation.

A supplier may also provide, or be required to provide, further information, categorized as either Additional Information or Exception Information. When present, each kind of further information is to be provided in a further sub-clause of items labeled A<I> or X<I>, respectively, for cross-referencing purposes, where <I> is any unambiguous identification for the item (e.g., simply a numeral). There are no other restrictions on its format or presentation.

A completed PICS proforma, including any Additional Information and Exception Information, is the PICS for the implementation in question.

NOTE—Where an implementation is capable of being configured in more than one way, a single PICS may be able to describe all such configurations. However, the supplier has the choice of providing more than one PICS, each covering some subset of the implementation’s capabilities, if this makes for easier and clearer presentation of the information.

A.3.2 Additional information

Items of Additional Information allow a supplier to provide further information intended to assist in the interpretation of the PICS. It is not intended or expected that a large quantity of information will be supplied, and a PICS can be considered complete without any such information. Examples of such Additional Information might be an outline of the ways in which an (single) implementation can be set up to operate in a variety of environments and configurations, or information about aspects of the implementation that are outside the scope of this standard but have a bearing upon the answers to some items.

References to items of Additional Information may be entered next to any answer in the questionnaire, and may be included in items of Exception Information.

A.3.3 Exception information

It may happen occasionally that a supplier will wish to answer an item with mandatory status (after any conditions have been applied) in a way that conflicts with the indicated requirement. No preprinted answer will be found in the Support column for this. Instead, the supplier shall write the missing answer into the Support column, together with an X<I> reference to an item of Exception Information, and shall provide the appropriate rationale in the Exception Information item itself.

An implementation for which an Exception Information item is required in this way does not conform to this standard.

NOTE— A possible reason for the situation described above is that a defect in this standard has been reported, a correction for which is expected to change the requirement not met by the implementation.

A.3.4 Conditional status

The PICS proforma contains a number of conditional items. These are items for which both the applicability of the item itself, and its status if it does apply, mandatory or optional, are dependent upon whether or not certain other items are supported.

Where a group of items is subject to the same condition for applicability, a separate preliminary question about the condition appears at the head of the group, with an instruction to skip to a later point in the questionnaire if the N/A answer is selected. Otherwise, individual conditional items are indicated by a conditional symbol in the Status column.

A conditional symbol is of the form “<pred>:<S>”, where “<pred>” is a predicate as described below, and “<S>” is one of the status symbols M or O.

If the value of the predicate is true, the conditional item is applicable, and its status is given by S: the support column is to be completed in the usual way. Otherwise, the conditional item is not relevant and the N/A answer is to be marked.

A predicate is one of the following:

a) An item-reference for an item in the PICS proforma: the value of the predicate is true if the item is marked as supported, and is false otherwise.

b) A boolean expression constructed by combining item-references using the boolean operator OR: the value of the predicate is true if one or more of the items is marked as supported, and is false otherwise.

Each item referenced in a predicate, or in a preliminary question for grouped conditional items, is indicated by an asterisk in the Item column.

A.4 PICS proforma—IEEE Std 802.21

A.4.1 Implementation identification

| Supplier |  |
| --- | --- |
| Contact point for queries about the PICS |  |
| Implementation Name(s) and Version(s) |  |
| Order information necessary for full identification, e.g. name(s) and version(s) of the machines and/or operating systems(s), system names |  |

NOTES

1—Only the first three items are required for all implementations. Other information may be completed as appropriate in meeting the requirement for full identification.

2—The terms Name and Version should be interpreted appropriately to correspond with a supplier’s terminology (e.g.,Type, Series, Model).

A.4.2 Protocol summary

| Identification of Protocol Standard | IEEE Std 802.21 |
| --- | --- |
| Have any exception items been required? (See A.3.3; the answer Yes means that the implementation does not conform to IEEE Std 802.21.) |  |
| Date of statement (dd/mm/yyyy) |  |

A.4.3 MIH Services configuration

| Item | MIH Services | References | Status | Support |
| --- | --- | --- | --- | --- |
| MIH1 | What are the MIH Services supported? Event Service (ES) | 6.3 | O.1 | Yes o No o |
| MIH2 | Command Service (CS) | 6.4 | O.1 | Yes o No o |
| MIH3 | Information Service (IS) | 6.5 | O.1 | Yes o No o |
| MIH4 | Capability Discovery | 6.2.3 | (MIH1 OR MIH2):M | Yes o No o |
| MIH5 | Registration Service | 6.2.4 | (MIH1 OR MIH2):M | Yes o No o |

A.4.4 Event Services (ES)

| Item | ES Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| ES1 | Event subscription | 6.3.2 | MIH1:M | Yes o No o |
| ES2 | Link_Detected | 7.3.1 | MIH1:M | Yes o No o |
| ES3 | Link_Up | 7.3.2 | MIH1:M | Yes o No o |
| ES4 | Link_Down | 7.3.3 | MIH1:M | Yes o No o |
| ES5 | Link_Going_Down | 7.3.4 | MIH1:M | Yes o No o |
| ES6 | Link_Event_Rollback | 7.3.5 | ES5:M | Yes o No o |
| ES7 | Link_Parameters_Report | 7.3.6 | MIH1:M | Yes o No o |
| ES8 | Link_PDU_Transmit_Status | 7.3.7 | O | Yes o No o |
| ES9 | Link_Handover_Imminent | 7.3.8 | O | Yes o No o |
| ES10 | Link_Handover_Complete | 7.3.9 | O | Yes o No o |
| ES11 | Link_Event_Subscribe | 7.3.10 | MIH1:M | Yes o No o |
| ES12 | Link_Event_Unsubscribe | 7.3.11 | MIH1:M | Yes o No o |
| ES13 | MIH_Link_Detected | 7.3.1, 8.6.2.4 | MIH1:M | Yes o No o |
| ES14 | MIH_Link_Up | 7.3.2, 8.6.2.1 | MIH1:M | Yes o No o |
| ES15 | MIH_Link_Down | 7.3.3, 8.6.2.2 | MIH1:M | Yes o No o |
| ES16 | MIH_Link_Going_Down | 7.3.4, 8.6.2.3 | MIH1:M | Yes o No o |
| ES17 | MIH_Link_Event_Rollback | 7.3.5, 8.6.2.6 | ES16:M | Yes o No o |
| ES18 | MIH_Link_Parameters_Report | 7.3.6, 8.6.2.5 | MIH1:M | Yes o No o |
| ES19 | MIH_Link_PDU_Transmit_Status | 7.3.7 | O | Yes o No o |
| ES20 | MIH_Link_Handover_Imminent | 7.3.8, 8.6.2.7 | O | Yes o No o |
| ES21 | MIH_Link_Handover_Complete | 7.3.9, 8.6.2.8 | O | Yes o No o |
| ES22 | MIH_Event_Subscribe request | 7.3.10, 8.6.1.7 | MIH1:M | Yes o No o |
| ES23 | MIH_Event_Subscribe response | 7.3.11, 8.6.1.8 | MIH1:M | Yes o No o |
| ES24 | MIH_Event_Unsubscribe request | 8.6.1.9 | MIH1:M | Yes o No o |
| ES25 | MIH_Event_Unsubscribe response | 8.6.1.10 | MIH1:M | Yes o No o |

A.4.5 Command Services (CS)

| Item | CS Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| CS1 | Link_Configure_Thresholds | 7.3.12 | MIH2:M | Yes o No o |
| CS2 | Link_Get_Parameters | 7.3.14 | MIH2:M | Yes o No o |
| CS3 | Link_Action | 7.3.15 | MIH2:M | Yes o No o |
| CS4 | MIH_Link_Configure_Thresholds | 8.6.3.3, 8.6.3.4 | MIH2:M | Yes o No o |
| CS5 | MIH_Link_Get_Parameters | 8.6.3.1, 8.6.3.2 | MIH2:M | Yes o No o |
| CS6 | MIH_Link_Action | 8.6.3.5, 8.6.3.6 | MIH2:M | Yes o No o |
| CS7 | MIH_Net_HO_Candidate_Query | 7.4.18, 8.6.3.7, 8.6.3.8 | MIH2:M1 | Yes o No o |
| CS8 | MIH_Net_HO_Commit | 7.4.21, 8.6.3.13, 8.6.3.14 | MIH2:M1 | Yes o No o |
| CS9 | MIH_MN_HO_Candidate_Query | 7.4.19, 8.6.3.9, 8.6.3.10 | MIH2:M2 | Yes o No o |
| CS10 | MIH_MN_HO_Commit | 7.4.22, ??????? | MIH2:M2 | Yes o No o |
| CS11 | MIH_MN_HO_Complete | 7.4.24, 8.6.3.17, 8.6.3.18 | MIH2:M2 | Yes o No o |
| CS12 | MIH_N2N_HO_Query_Resources | 7.4.20, 8.6.3.11, 8.6.3.12 | MIH2:M3 | Yes o No o |
| CS13 | MIH_N2N_HO_Commit | 7.4.23, 8.6.3.15, 8.6.3.16 | MIH2:M3 | Yes o No o |
| CS14 | MIH_N2N_Complete | 7.4.25, 8.6.3.19, 8.6.3.20 | MIH2:M3 | Yes o No o |

A.4.6 Information Services (IS)

| Item | IS Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| IS1 | TYPE_IE_NETWORK_TYPE | 6.5.4 | MIH3:M | Yes o No o |
| IS2 | TYPE_IE_OPERATOR_IDENTIFIER | 6.5.4 | MIH3:M | Yes o No o |
| IS3 | TYPE_IE_SERVICE_PROVIDER_IDENTIFIER | 6.5.4 | MIH3:M | Yes o No o |
| IS4 | TYPE_IE_COUNTRY_CODE | 6.5.4 | MIH3:M | Yes o No o |
| IS5 | TYPE_IE_ACCESS_NETWORK_IDENTIFIER | 6.5.4 | MIH3:M | Yes o No o |
| IS6 | TYPE_IE_ACCESS_NETWORK_AUX_ID | 6.5.4 | MIH3:M | Yes o No o |
| IS7 | TYPE_IE_ROAMING_PARTNERS | 6.5.4 | MIH3:M | Yes o No o |
| IS8 | TYPE_IE_COST | 6.5.4 | MIH3:O | Yes o No o |
| IS9 | TYPE_IE_NETWORK_SECURITY | 6.5.4 | MIH3:O | Yes o No o |
| IS10 | TYPE_IE_NETWORK_QOS | 6.5.4 | MIH3:O | Yes o No o |
| IS11 | TYPE_IE_NETWORK_DATA_RATE | 6.5.4 | MIH3:O | Yes o No o |
| IS12 | TYPE_IE_NETWORK_CHANNEL_RANGE | 6.5.4 | MIH3:O | Yes o No o |
| IS13 | TYPE_IE_NETWORK_IP_CONFIG_METHODS | 6.5.4 | MIH3:O | Yes o No o |
| IS14 | TYPE_IE_NETWORK_CAPABILITIES | 6.5.4 | MIH3:O | Yes o No o |
| IS15 | TYPE_IE_NETWORK_SUPPORTED_LCP | 6.5.4 | MIH3:O | Yes o No o |
| IS16 | TYPE_IE_NETWORK_MOBILITY_MGMT_PROT | 6.5.4 | MIH3:O | Yes o No o |
| IS17 | TYPE_IE_POA_MAC_ADDRESS | 6.5.4 | MIH3:O | Yes o No o |
| IS18 | TYPE_IEPOA_LOCATION | 6.5.4 | MIH3:O | Yes o No o |
| IS19 | TYPE_IE_POA_CHANNEL_RANGE | 6.5.4 | MIH3:O | Yes o No o |
| IS20 | TYPE_IE_POA_SYSTEM_INFORMATION | 6.5.4 | MIH3:O | Yes o No o |
| IS21 | TYPE_IE_POA_SUBNET_INFORMATION | 6.5.4 | MIH3:O | Yes o No o |
| IS22 | TYPE_IE_POA_IP_ADDRESS | 6.5.4 | MIH3:O | Yes o No o |
| IS23 | Vendor Specific IEs | 6.5.4 | MIH3:O | Yes o No o |
| IS24 | TLV Query Method | 6.5.6.2 | MIH3:O2 | Yes o No o |
| IS25 | RDF query Method | 6.5.6.3 | MIH3:O2 | Yes o No o |
| IS26 | MIH_Get_Information | 7.4.26, 8.6.4.1, 8.6.4.2 | MIH3:M | Yes o No o |

A.4.7 MIH Protocol

| Item | MIH Protocol Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| MP1 | MIH Protocol Frame | 8.4 | M | Yes o No o |
| MP2 | MIH Protocol Acknowledgement | 8.2.1 | O | Yes o No o |
| MP3 | MIH Protocol State Machine | 8.2.2 | M | Yes o No o |
| MP4 | MIH Protocol Identifiers – MIHF ID | 8.3.1 | M | Yes o No o |
| MP5 | MIH Protocol Identifiers – Transaction ID | 8.3.2 | M | Yes o No o |