| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH_MN_HO_Commit |  |
| DCN | 21-08-0062-00-0000 |  |
| Date Submitted | Feb 25, 2008 |  |
| Source(s) | Ajay Rajkumar, David Faucher | Alcatel-Lucent |
| Re: | IEEE 802.21 Session # in Orlando |  |
| Abstract | MIH_MN_HO_Commit |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

Add the following to Table 10.

| MIH_MN_HO_Commit | L | Command used by MN to notify the network that a candidate has been committed for handover. | 7.4.X |
| --- | --- | --- | --- |

Add the following to Table 17.

| MIH_MN_HO_Commit | L | Mobile node has committed to handover. | 7.4.X |
| --- | --- | --- | --- |

Change section 7.4.13.1 to:

MIH_Link_Handover_Complete indication is issued by the MIHF to report the completion of a

handover to MIH Users. The event may be associated with either intra-technology or inter-technology handovers.

Change section 7.4.13.1.1 to:

This primitive is issued by the MIHF to report the completion of an intra-technology or inter-technology handover. This MIH event may be local or remote.

Change section 7.4.13.1.2 to:

MIH_Link_Handover_Complete.indication (

SourceIdentifier,

OldLinkIdentifier,

NewLinkIdentifier,

OldAccessRouter,

NewAccessRouter,

LinkHandoverStatus

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | This identifies the invoker of this primitive which can be either the local MIHF or a remote MIHF. |
| Old Link Identifier | LINK_TUPLE_ID | Identifier of the old link |
| New Link Identifier | LINK_TUPLE_ID | Identifier of the new link |
| Old AccessRouter | LINK_ADDR | (Optional) Link address of old Access Router |
| New AccessRouter | LINK_ADDR | (Optional) Link address of new Access Router |
| Link Handover Status | STATUS | Status of the link handover |

Change section 7.4.13.1.3 to:

This notification is generated by the MIHF when a link layer inter-technology or intra-technology handover

is completed  For intra-technology handovers this indication is posted in response to receiving a Link_Handover_Complete indication from the link layer. For inter-technology handovers this indication is posted in response to the command that actually executed the handover (see MIH_MN_HO_Commit).

Add the following sections:

7.4.X MIH_MN_HO_Commit

7.4.X.1 MIH_MN_HO_Commit.request

7.4.X.1.1 Function

This primitive is used by MIH Users on a mobile node to commit to a handover to a specific target link.

7.4.X.1.2 Semantics of service primitive

MIH_MN_HO_Commit.request (

DestinationIdentifier,

SourceLinkIdentifier,

TargetLinkIdentifier,

SourceLinkAction.,

TargetLinkAction

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | This identifies the target of this primitive. |
| Source Link Identifier | LINK_TUPLE_ID | This identifies the source link. |
| Target Link Identifier | LINK_TUPLE_ID | This identifies the target link to which the handover needs to be initiated. |
| Source Link Action | LINK_ACTION | Specifies the action to be performed on the source link during the handover. |
| Target Link Action | LINK_ACTION | Specifies the action on the target link during the handover. |

7.4.X.1.3 When generated

The MIH User generates this primitive to execute a handover.

7.4.X.1.4 Effect on receipt

Upon receipt of this primitive MIHF marks the beginning of the inter-technology handover by posting an MIH_Link_Handover_Imminent event. It then submits the link actions to the appropriate link for execution.

7.4.X.2 MIH_MN_HO_Commit.confirm

7.4.X.2.1 Function

This primitive is issued by an MIHF to report the result of an MIH_MN_HO_Commit request.

MIH_MN_HO_Commit.confirm (

SourceIdentifier,

Status,

SourceLinkIdentifier,

TargetLinkIdentifier,

SourceLinkActionResult,

TargetLinkActionResult,

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| SourceIdentifier | MIHF_ID | This identifies the invoker of this primitive. |
| Status | STATUS | Status of operation. |
| Source Link Identifier | LINK_TUPLE_ID | This identifies the source link. |
| Target Link Identifier | LINK_TUPLE_ID | This identifies the target link to which the handover needs to be initiated. |
| Source Link Action Result | LINK_AC_RESULT_CODE | The result status of the source link action. Note: this parameter is not included if Status does not indicate “Success”. |
| Target Link Action Result | LINK_AC_RESULT_CODE | The result status of the target link action. Note: this parameter is not included if Status does not indicate “Success”. |

7.4.X.2.2 When generated

This primitive is generated by MIHF when it completes the execution of an MIH_MN_HO_Commit.request. Additionally, MIHF marks the end of an inter-technology handover by posting an MIH_Link_Handover_Complete event.

7.4.X.2.3 Effect on receipt

MN may now proceed with the next step in handover.

Change MIH_CMD_LIST (page 194 line 46) bit 4 to:

Bit 4: MIH_MN_HO_Candidate_Query

MIH_MN_HO_Commit

MIH_MN_HO_Complete