| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |
| --- | --- |
| Title | Resolution for Comment #2176 |
| Date Submitted | March 19, 2008 |
| Source(s) | Yoshihiro Ohba (Toshiba) and Qiaobing Xie (Motorola) |
| Re: | IEEE 802.21 Session #25 in March 2008 |
| Abstract | This document addresses SB recirc-2 Comment #2176 on QUERIER_LOC. |
| Purpose | Sponsor Ballot comment resolution |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

# Introduction

Comment #2176:

# MAC_ADDR in QUERIER_LOC should be LINK_TUPLE_ID because (i) it is important to indicate LINK_TYPE and LINK_ADDR that the MN is using, and (ii) PoA's link-layer address may be not only IEEE MAC address but also Cell ID or other L2 address.

# Proposed Resolution

[1] Change Section 6.5.6.2.2 to:

6.5.6.2.2 TLV Queries

A TLV query may include the following optional parameters to refine the query.

QUERIER_LOC parameter (see Table B-15 for definition) can be useful for the Information Server to refine its response. The value field contains either the querier’s current location measurement or, when the querier does not have its current location information, an observed MAC link-layer address (e.g., from an IEEE 802.11 Beacon frame or some broadcast mechanism for other technologies) which the Information Server will be able to use as a hint to establish an estimate of the client's current location. The querier should not use both the MAC_ADDR LINK_ADDR and LOCATION value in the same query. Moreover, the NGHB_RADIUS value, if provided, indicates the radius of the neighborhood, centered at the indicated location, within which all available access networks will be included in the list of neighboring networks. If NGHB_RADIUS value is not present, the Information Server will decide the radius for the search.

[2] Change QUELIER_LOC data type in Annex B to:

SEQUENCE(

CHOICE(NULL, LOCATION),

CHOICE(NULL, MAC_ADDRLINK_ADDR),

CHOICE(NULL, NGHB_RADIUS)

)

[3] Add the following parameter to MIH_Get_Information.request primitive.

Name: Querier network type

Type: NETWORK_TYPE

Description: (optional) The type of the network being used by the querier. This parameter is valid only with Info Query Binary Data List and Info Query RDF Data List.

[4] Add the following parameter to MIH_Get_Information.request primitive.

Queier Network Type (Network Type TLV)

[5] Add the following TLV to Table C-2.

TLV Type Name: Network Type

TLV Type Value: XX

Data Type: NETWORK_TYPE