| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | MIH_MN_HO_Commit |  |
| DCN | 21-08-0110-00-0000 |  |
| Date Submitted | April 18, 2008 |  |
| Source(s) | Junghoon Jee | ETRI jhjee@etri.re.kr +82-42-860-5126 |
| Re: | IEEE 802.21 Session # in Florida |  |
| Abstract | MIH_MN_HO_Commit Wrap-Up |  |
| Purpose | Adoption |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

[Remedy 1]. Add the following to Table 6 in the Page 43.

| MIH_MN_HO_Commit | R | Command used by MN to notify the serving network of the decided target network information. | 7.4.20 |
| --- | --- | --- | --- |
| MIH_Net_HO_Commit | R | In this case the network commits to do the handover and sends the choice of selected network and associated PoA. | 7.4.20 7.4.21 |

[Remedy 2]. Add the following to Table 16 in the Page 59.

| MIH_MN_HO_Commit | R | Notify the serving network of the decided target network information | 7.4.20 |
| --- | --- | --- | --- |

[Remedy 3]. Add the following sections:

7.4.20 MIH_MN_HO_Commit

7.4.20.1 MIH_MN_HO_Commit.request

7.4.20.1.1 Function

This primitive is used by MIH Users on a mobile node to notify the serving network of the decided target network information.

7.4.20.1.2 Semantics of service primitive

MIH_MN_HO_Commit.request (

DestinationIdentifier,

LinkType,

TargetNetworkInfo,

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | This specifies the MIHF ID of the serving network which is the target of this primitive. |
| Link Type | LINK_TYPE | This contains the target link type. |
| Target Network Info | TGT_NET_INFO | This contains the target network information. |

7.4.20.1.3 When generated

The MIH User generates this primitive to notify the serving network of the target network information.

7.4.20.1.4 Effect on receipt

Upon receipt of this primitive, MIHF on the mobile node sends the corresponding MIH_MN_HO_Commit request message to the serving network.

7.4.20.2 MIH_MN_HO_Commit.indication

7.4.20.2.1 Function

This primitive is generated by an MIHF on the serving network to indicate that an MIH_MN_HO_Commit request message has been received from a peer MIHF on the mobile node.

7.4.20.2.2 Semantics of service primitive

MIH_MN_HO_Commit.indication (

SourceIdentifier,

LinkType,

TargetNetworkInfo,

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | This specifies the MIHF ID of the mobile node that sent the MIH_MN_HO_Commit request message. |
| Link Type | LINK_TYPE | This contains the target link type. |
| Target Network Info | TGT_NET_INFO | This contains the target network information. |

7.4.20.2.3 When generated

This primitive is generated by an MIHF on the serving network when receiving an MIH_MN_HO_Commit request message from a peer MIHF on the mobile node.

7.4.20.2.4 Effect on receipt

Upon receipt of this primitive MIH User on the serving network replies with an MIH_MN_HO_Commit.response primitive. MIH User may invoke the MIH_N2N_HO_Commit.request primitive to reserve the resource at the target network.

7.4.20.3 MIH_MN_HO_Commit.response

7.4.20.3.1 Function

This primitive is used by an MIH User on the serving network to communicate with a peer MIH User on the mobile node from which an MIH_MN_HO_Commit request message is received.

MIH_MN_HO_Commit.response (

DestinationIdentifier,

Status,

LinkType,

TargetNetworkInfo

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| DestinationIdentifier | MIHF_ID | This specifies the MIHF ID of the mobile node that sent the MIH_MN_HO_Commit request message. |
| Status | STATUS | Status of operation. |
| Link Type | LINK_TYPE | This contains the target link type. |
| Target Network Info | TGT_NET_INFO | This contains the target network information. |

7.4.20.3.2 When generated

This primitive is generated in response to an MIH_MN_HO_Commit.indication primitive.

7.4.20.3.3 Effect on receipt

When receiving this primitive from the MIH User, the MIHF on the Serving PoS sends the corresponding MIH_MN_HO_Commit response to its peer MIHF on the mobile node.

7.4.20.4 MIH_MN_HO_Commit.confirm

7.4.20.4.1 Function

This primitive is generated by the MIHF on the mobile node to confirm that an MIH_MN_HO_Commit response message is received from a peer MIHF on the serving network.

MIH_MN_HO_Commit.confirm (

SourceIdentifier,

Status,

LinkType,

TargetNetworkInfo

)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Source Identifier | MIHF_ID | This specifies the MIHF ID of the Serving PoS that sent the MIH_MN_HO_Commit response message. |
| Status | STATUS | Status of operation. |
| Link Type | LINK_TYPE | This contains the target link type. |
| Target Network Info | TGT_NET_INFO | This contains the target network information. |

7.4.20.4.2 When generated

This primitive is generated by the MIHF on the mobile node when it receives an MIH_MN_HO_Commit response message from a peer MIHF on the serving network.

7.4.20.4.3 Effect on receipt

Upon receipt, the MIH User on the mobile node is informed about the status of the previously issued target notification request.

[Remedy 4]. Add the following sections and apply changes:

      - MIH_MN_HO_Commit request

The corresponding MIH primitive of this message is defined in 7.4.20.1.

This message is used by the MIHF on the mobile node to notify the Serving PoS of the decided target network information.

MIH Header Fields (SID=3, Opcode=1, AID=7)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

LinkType (Link Type TLV)

TargetNetworkInfo (Target Network Info TLV)

      - MIH_MN_HO_Commit response

The corresponding MIH primitive of this message is defined in 7.4.20.3

This message is used by the MIHF on the Serving PoS to respond to a MIH_MN_HO_Commit request message.

MIH Header Fields (SID=3, Opcode=2, AID=7

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

Status (Status TLV)

LinkType (Link Type TLV)

TargetNetworkInfo (Target Network Info TLV)

      - MIH_Net_HO_Commit request

MIH Header Fields (SID=3, Opcode=1, AID=7 8

      - MIH_Net_HO_Commit response

MIH Header Fields (SID=3, Opcode=2, AID=7 8

      - MIH_N2N_HO_Commit request

MIH Header Fields (SID=3, Opcode=1, AID=7 9

      - MIH_N2N_HO_Commit response

MIH Header Fields (SID=3, Opcode=2, AID=7 9

      - MIH_MN_HO_Complete request

MIH Header Fields (SID=3, Opcode=1, AID=7 10

      - MIH_MN_HO_Complete response

MIH Header Fields (SID=3, Opcode=2, AID=7 10

      - MIH_N2N_HO_Complete request

MIH Header Fields (SID=3, Opcode=1, AID=7 11

      - MIH_N2N_HO_Complete response

MIH Header Fields (SID=3, Opcode=2, AID=7 12

[Remedy 5]. Apply the following changes to Table C-1—AID assignment in the Page 201.

MIH message for Command Service

| MIH_MN_HO_Commit | 7 |
| --- | --- |
| MIH_Net_HO_Commit | 7 8 |
| MIH_N2N_HO_Commit | 8 9 |
| MIH_MN_HO_Complete | 9 10 |
| MIH_N2N_HO_Complete | 10 11 |

[Remedy 6]. Add the following change to Table B-20 in the Page 196.

Table B-20—Data type for MIH capabilities

Change MIH_CMD_LIST bit 4 as:

Bit 4: MIH_MN_HO_Candidate_Query

MIH_MN_HO_Commit

MIH_MN_HO_Complete