| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |
| --- | --- |
| Title | Resolution for Comment #2184 |
| Date Submitted | April, 2008 |
| Source(s) | Vivek Gupta (Intel Corporation) |
| Re: | IEEE 802.21 Session #25 in May 2008 |
| Abstract | This document addresses SB recirc-2 Comment #2184 |
| Purpose | Sponsor Ballot comment resolution |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

Comment 2184:

For Inter-Tech Handover 802.21 needs to define if the handovers would take place with Single radio in operation or dual radio in operation. Solutions needs to be provided for both cases.

Add definitions in the draft. Consensus to keep the definitions crisp and simple.

Resolution

Add the following text:

Dual-Radio operation: In this mode a dual radio device can receive and transmit simultaneously on both the radios. Since both radios can be active simultaneously in these types of devices, the target radio connects with the target network to prepare the target network for handover. The source radio maintains connection with the source network during the handover.

Single-Radio operation: In this mode a dual radio device can receive and transmit on only one radio at a time. This is usually the mode of operation when radio frequencies of the two radios are close to each other (e.g. in IMT 2000 bands). Since only one radio can be active at a time in these types of devices, the source radio uses the back-end connection of the source network with the target network to prepare the target network for handover while maintaining the client side connections. Once the target preparation is complete the device switches from source radio to target radio. Since all the target preparation has been completed a priori, the target radio quickly establishes connectivity with the target network and all the connections are then transferred from source network to target network.