| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Updates due to PICS |  |
| DCN | 21-08-0114-00-0000 |  |
| Date Submitted | May, 2008 |  |
| Source(s) | Vivek Gupta |  |
| Re: | Update due to PICS |  |
| Abstract | Update due to PICS in Annex-1 |  |
| Purpose | Update due to PICS in Annex-1 |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

- Type identifiers for information elements

Type identifiers for information elements lists the Type identifier values for different individual IEs and IE containers.

| - Type identifiers for information elements |  |
| --- | --- |
| Type | Identifier |
| IE_NETWORK_TYPE | 0x10000000 |
| IE_OPERATOR_ID | 0x10000001 |
| IE_SERVICE_PROVIDER_ID | 0x10000002 |
| IE_COUNTRY_CODE | 0x10000003 |
| IE_NETWORK_ID | 0x10000100 |
| IE_NETWORK_AUX_ID | 0x10000101 |
| IE_ROAMING_PARTNERS | 0x10000102 |
| IE_COST | 0x10000103 |
| IE_NETWORK_QOS | 0x10000105 |
| IE_NETWORK_DATA_RATE | 0x10000106 |
| IE_NET_CHANNEL_RANGE | 0x10000107 |
| IE_NET_IP_CFG_METHODS | 0x10000108 |
| IE_NET_CAPABILITIES | 0x10000109 |
| IE_NET_SUPPORTED_LCP | 0x1000010A |
| IE_NET_MOB_MGMT_PROT | 0x1000010B |
| IE_NET_EMSERV_PROXY | 0x1000010C |
| IE_NET_IMS_PROXY_CSCF | 0x1000010D |
| IE_POA_MAC_ADDR | 0x10000200 |
| IE_POA_LOCATION | 0x10000201 |
| IE_POA_CHANNEL_RANGE | 0x10000202 |
| IE_POA_SYSTEM_INFO | 0x10000203 |
| IE_POA_SUBNET_INFO | 0x10000204 |
| IE_POA_IP_ADDR | 0x10000205 |
| IE_CONTAINER_LIST_OF_NETWORKS | 0x10000300 |
| IE_CONTAINER_NETWORK | 0x10000301 |
| IE_CONTAINER_POA | 0x10000302 |

- Data type definition

- General

This Annex defines data types used in the IEEE 802.21 standard. Any variable-length data type in this specification contains information needed for determining the end of data.

- Basic data types

The data types defined in this subclause are used as the basis for defining any other data types. All basic data types are for general purpose. The “Binary Encoding Rule” column in Basic dat describes the encoding rules used when the data types are carried in MIH protocol messages.

| - Basic data types |  |  |
| --- | --- | --- |
| Type Name | Definition | Binary Encoding Rule |
| BITMAP(size) | A bitmap of the specified size. Usually used to represent a list of IDs. Range: Each bit has a value of '0' or '1'. | A BITMAP(N), where N must be a multiple of 8, is made up of an N/8 octet values and encoded in network byte order. |
| CHAR(size) | An array of characters. Each character is encoded as an octet. The size specifies the length. | The characters are encoded in network byte order. |
| CHOICE( DATATYPE1, DATATYPE2[,…]) | A data type that consists of only one of the data types listed: DATATYPE1,DATATYPE2[,…]. | A one-octet Selector field, followed by a variable length Value field. The Selector value determines the data type. If Selector==i, (i+1)-th data type in the list of data types DATATYPE1,DATATYPE2,[,…] is selected. The Selector value is encoded as UNSIGNED_INT(1). The Value field is encoded using the encoding rule for the selected data type. |
| INFO_ELEMENT | A binary encoded structure for Information Elements. | See Subclause 6.5.6 |
| INTEGER(size) | A signed integer of the specified size in number of octets. Range: Each octet has a value of 0x00 to 0xff. | Each octet of an INTEGER(N) value [N=1,2,...] is encoded in network-byte order into an N-octet field. The most significant bit of the first octets is the sign bit. If the sign bit is set, it indicates a negative integer. Otherwise, it indicates a non-negative integer. A negative integer is encoded as 2's complement. |
| LIST(DATATYPE) | A list of values of DATATYPE | See subclause Encoding rule fo for details. |
| NULL | A data type with empty data. | No octet is encoded for this data type. This data type is used to define an optional data type. |
| SEQUENCE( DATATYPE1, DATATYPE2[,...]) | A data type that consists of two or more data types. | DATATYPE1,DATATYPE2,[,…] are encoded in the order of appearance. Each data type is encoded using the encoding rule for the data type. |
| UNSIGNED_INT(size) | An unsigned integer of the specified size in number of octets. Range: Each octet has a value of 0x00 to 0xff. | Each octet of an UNSIGNED_INT(N) value [N=1,2,...] is encoded in network-byte order into an N-octet field. |

- Encoding rule for data type LIST(DATATYPE)

A variable length Length field is followed by a variable length Value field. The Length field shall be interpreted as follows:

Case 1: If the number of list elements in the Value field is less than 128, the size of the Length field is always 1 octet and the MSB of the octet is set to the value ‘0’. The values of the other seven bits of this octet indicate the actual number of list elements in the Value field.

Case 2: If the number of list elements in the Value field is exactly 128, the size of the Length field is one octet. The MSB of the Length octet is set to the value '1' and the other seven bits of this octet are all set to the value ‘0’.

Case 3: If the number of list elements in the Value field is greater than 128, then the Length field is always greater than 1 octet. The MSB of the first octet of the Length field is set to the value ‘1’ and the remaining 7 bits of the first octet indicate the number of octets that are appended further. The number represented by the 2nd and subsequent octets of the Length field, when added to 128, indicates the total number of list elements in the Value field.

For example, an attribute of type LIST(LINK_ID) with two elements is encoded as following (LINK_ID is defined in subclause Data types for link identification and manipulation):

- Encoding example of a LIST with two LINK_ID elements

- Derived data types

- General

Derived data types are those which are derived from other data types or parent data types. A derived data type uses the same encoding as the parent data type.

- General data types

The derived data types defined in this subclause are for general purpose only.

| - General data types |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| ENUMERATED | UNSIGNED_INT(1) | An enumerated attribute. Valid Range: 0..255 |
| BOOLEAN | ENUMERATED | Represents a boolean. 0: FALSE 1: TRUE |
| OCTET_STRING | LIST(CHAR(1)) | An array of octets. |
| PERCENTAGE | UNSIGNED_INT(1) | Represents a percentage. Valid Range: 0..100 |
| STATUS | ENUMERATED | The status of a primitive execution. 0: Success 1: Unspecified Failure 2: Rejected 3: Authorization Failure 4: Network Error |

- Data types for addresses

The data types defined in this subclause are related to addresses of network elements.

| - Data types for address |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| 3GPP_2G_CELL_ID | SEQUENCE( PLMN_ID, LAC, CI ) | A data type to represent 3GPP 2G cell identifier |
| 3GPP_3G_CELL_ID | SEQUENCE( PLMN_ID, CELL_ID ) | A data type to represent 3GPP 3G cell identifier. |
| CELL_ID | UNSIGNED_INT(4) | This data type identifies a cell uniquely within 3GPP UTRAN and consists of RNC-ID and C-ID as defined in 3GPP TS 25.401. Valid Range: 0..268435455 |
| CI | CHAR(2) | The BSS and cell within the BSS are identified by Cell Identity (CI). See 3GPP TS 23.003. |
| IP_ADDR | TRANSPORT_ADDR | Represents an IP address. The Address Type is either 1 (IPv4) or 2 (IPv6). |
| LAC | CHAR(2) | Location Area Code (LAC) is a fixed length code (of 2 octets) identifying a location area within a PLMN. See 3GPP TS 23.003. |
| LINK_ADDR | CHOICE( MAC_ADDR, 3GPP_3G_CELL_ID, 3GPP_2G_CELL_ID, OTHER_L2_ADDR ) | A data type to represent an address of any link-layer. |
| MAC_ADDR | TRANSPORT_ADDR | Represents a MAC address. The Address Type contains the one used for a specific link layer. |
| OTHER_L2_ADDR | OCTET_STRING | A data type to represent a link-layer address other than the address already defined. For example, SSID. |
| PLMN_ID | OCTET_STRING(3) | The Public Land Mobile Network (PLMN) unique identifier. PLMN_ID consists of Mobile Country Code (MCC) and Mobile Network Code (MNC). This is to represent the access network identifier. Coding of PLMN_ID is defined in 3GPP TS 25.413. |
| TRANSPORT_ADDR | OCTET_STRING | A type to represent a transport address using Address base type defined in RFC3588. The first 2-octets contain AddressType. AddressType values are defined in http://www.iana.org/assignments/address-family-numbers. |

- Data types for link identification and manipulation

The data types defined in this subclause are used for representing attributes for identification and manipulation of links.

| - Data types for links |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| BATT_LEVEL | INTEGER(1) | Represents battery level. Valid Range: -1..100. -1 indicates battery level unknown. |
| CHANNEL_ID | UNSIGNED_INT(2) | Channel identifier as defined in the specific link technology (e.g. SDO). Valid Range: 0..65535 |
| CONFIG_STATUS | BOOLEAN | The status of link parameter configuration. TRUE: Success FALSE: Error |
| DEVICE_INFO | OCTET_STRING | A non-NULL terminated string whose length shall not exceed 253 octets, representing information on manufacturer, model number, revision number of the software/firmware and serial number in displayable text. |
| DEV_STATES_REQ | BITMAP(16) | A set of device status request. Bitmap Values: Bit 0: DEVICE_INFO Bit 1: BATT_LEVEL Bit 2-15: (Reserved) |
| DEV_STATES_RSP | CHOICE( DEVICE_INFO, BATT_LEVEL ) | Represents a device status. |
| LINK_AC_EX_TIME | UNSIGNED_INT(2) | Time (in ms) to elapse before an action needs to be taken. A value of 0 indicates that the action will be taken immediately. Time elapsed will be calculated from the instance the command arrives until the time when the execution of the action is carried out. Valid Range: 0..65535 |
| LINK_AC_RESULT | ENUMERATED | Link action result. 0: Success 1: Failure |
| LINK_ACTION | SEQUENCE( LINK_AC_TYPE, LINK_AC_ATTR ) | Link action. |
| LINK_AC_ATTR | BITMAP(8) | Link action attribute that can be executed along with a valid link action. Detail description of each attribute is in Link action attributes. Bitmap Values: Bit 0: LINK_SCAN Bit 1: LINK_RES_RETAIN Bit 2: DATA_FWD_REQ Bit 3-7: (Reserved) |
| LINK_ACTION_REQ | SEQUENCE( LINK_ID, CHOICE(NULL, LINK_ADDR), LINK_ACTION, LINK_AC_EX_TIME ) | A set of handover action request parameters. The choice of LINK_ADDR is to provide PoA address information when the LINK_ACTION contains the attribute for DATA_FWD_REQ. |
| LINK_ACTION_RSP | SEQUENCE( LINK_ID, LINK_AC_RESULT, CHOICE(NULL, LIST(LINK_SCAN_RSP)) ) | A set of link action returned results. |
| LINK_AC_TYPE | UNSIGNED_INT(1) | An action for a link. The meaning of each link action is defined in Link actions. 0: NONE 1: LINK_DISCONNECT 2: LINK_LOW_POWER 3: LINK_POWER_DOWN 4: LINK_POWER_UP 5-255: (Reserved) |
| LINK_CMD_LIST | BITMAP(32) | A set of link commands. Bitmap Values: Bit 0: Link_Capability_Discover Bit 1: Link_Event_Subscribe Bit 2: Link_Event_Unsubscribe Bit 3: Link_Get_Parameters Bit 4: Link_Configure_Thresholds Bit 5: Link_Action Bit 6-31: (Reserved) |
| LINK_CFG_PARAM | SEQUENCE( LINK_PARAM_TYPE, CHOICE(NULL, TIMER_INTERVAL), LIST(THRESHOLD) ) | A link configuration parameter. |
| LINK_CFG_STATUS | SEQUENCE( LINK_PARAM_TYPE, CONFIG_STATUS ) | The status of link parameter configuration. |
| LINK_DESC_REQ | BITMAP(16) | A set of link descriptors. Bitmap Values: Bit 0: Number of Classes of Service Supported Bit 1: Number of Queues Supported Bits 2-15: (Reserved) |
| LINK_DESC_RSP | CHOICE(NUM_COS, NUM_QUEUE) | Descriptors of a link. |
| LINK_DATA_RATE | UNSIGNED_INT(4) | A type to represent the maximum data rate in kbps. Valid Range: 0 - 2^32-1 |
| LINK_DN_REASON | UNSIGNED_INT(1) | Represents the reason of a link down event. See Link Down reason code for the enumaration values. |
| LINK_EVENT_LIST | BITMAP(32) | A list of link events. The specified event is selected if the corresponding bit is set to 1. Bitmap values: Bit 0: Link_Detected Bit 1: Link_Up Bit 2: Link_Down Bit 3: Link_Event_Rollback Bit 4: Link_Parameters_Report Bit 5: Link_Going_Down Bit 6: Link_Handover_Imminent Bit 7: Link_Handover_Complete Bit 8: Link_PDU_Transmit_Status Bit 9-31: (Reserved) |
| LINK_GD_REASON | UNSIGNED_INT(1) | Represents the reason of a link going down. See Link Going Down reason code for the enumeration values. |
| LINK_ID | SEQUENCE( LINK_TYPE LINK_ADDR ) | The identifier of a link that is not associated with the peer node. The LINK_ADDR contains the address of this link. |
| LINK_MIHCAP_FLAG | BITMAP(8) | Represents if MIH capability is supported or not. If the bit is set, it indicates that the capability is supported. Bitmap values: Bit 1: ES Supported Bit 2: CS Supported Bit 3: IS Supported Bit 0, 4-7: (Reserved) |
| LINK_PARAM | SEQUENCE( LINK_PARAM_TYPE, CHOICE(LINK_PARAM_VAL, QOS_PARAM_VAL) ) | Represents a link parameter type and value pair. |
| LINK_PARAM_802_11 | UNSIGNED_INT(1) | A type to represent a link parameter for 802.11. 0: RSSI of the beacon channel 1: Flag indicating no QoS resources available (this applies when the traffic stream to be transmitted is on an access category configured for HCCA operation and the request for bandwidth was denied by the available APs in the access network) 2-255: (Reserved) |
| LINK_PARAM_802_16 | UNSIGNED_INT(1) | A type to represent a link parameter for 802.16. 0-255: (Reserved) |
| LINK_PARAM_802_20 | UNSIGNED_INT(1) | A type to represent a link parameter for 802.20. 0-255: (Reserved) |
| LINK_PARAM_802_22 | UNSIGNED_INT(1) | A type to represent a link parameter for 802.22. 0-255: (Reserved) |
| LINK_PARAM_C2K | UNSIGNED_INT(1) | A type to represent a link parameter for CDMA2000. 0: PILOT_STRENGTH 1-255: (Reserved) |
| LINK_PARAM_HRPD | UNSIGNED_INT(1) | A type to represent a link parameter for CDMA2000 HRPD. 0: PILOT_STRENGTH 1-255: (Reserved) |
| LINK_PARAM_EDGE | UNSIGNED_INT(1) | A type to represent a link parameter for EDGE. 0-255: (Reserved) |
| LINK_PARAM_ETH | UNSIGNED_INT(1) | A type to represent a link parameter for Ethernet. 0-255: (Reserved) |
| LINK_PARAM_GEN | UNSIGNED_INT(1) | A type to represent a link parameter that is applicable to any link type. 0: Speed 1: Signal Strength 2: SINR 3:Throughput 4: Packet Error Rate 5-255: (Reserved) |
| LINK_PARAM_GG | UNSIGNED_INT(1) | A type to represent a link parameter for GSM and GPRS. See 3GPP TS 25.008. 0: RxQual 1: RsLev 2: Mean BEP 3: StDev BEP 4-255: (Reserved) |
| LINK_PARAM_QOS | UNSIGNED_INT(1) | A type to represent QOS_LIST parameters. 0: Maximum number of differentiable classes of service supported. 1: Throughput 2: Packet Error Rate 3: Minimum packet transfer delay for all CoS 4: Average packet transfer delay for all CoS 5: Maximum packet transfer delay for all CoS 6: Packet transfer delay jitter for all CoS 7: Packet loss rate for all Cos 8-255: (Reserved) |
| LINK_PARAM_RPT | SEQUENCE( LINK_PARAM, LINK_REPORT_RN ) | Represents a link parameter report. |
| LINK_PARAM_TYPE | CHOICE( LINK_PARAM_GEN, LINK_PARAM_QOS, LINK_PARAM_GG, LINK_PARAM_EDGE, LINK_PARAM_ETH, LINK_PARAM_802_11, LINK_PARAM_C2K, LINK_PARAM_FDD, LINK_PARAM_HRPD, LINK_PARAM_802_16, LINK_PARAM_802_20, LINK_PARAM_802_22 ) | Measurable link parameter for which thresholds are being set. |
| LINK_PARAM_VAL | UNSIGNED_INT(2) | The current value of the parameter. The format of the media-dependent value is defined in the respective media specification standard and the equivalent number of bits (i.e. first bits) of this data type is used. In case that there are remaining unused bits in the data type, these are marked as all-zeros (‘0’). Valid Range: 0..65535 |
| LINK_PARAM_FDD | UNSIGNED_INT(1) | A type to represent a link parameter for UMTS. See 3GPP TS 25.215. 0: CPICH RSCP 1: PCCPCH RSCP 2: UTRA carrier RSSI 3: GSM carrier RSSI 4: CPICH Ec/No 5: Transport channel BLER 6: UE transmitted power 7-255: (Reserved) |
| LINK_POA_LIST | SEQUENCE( LINK_ID, LIST(LINK_ADDR) ) | A list of PoAs for a particular link. The LIST(LINK_ADDR) is a list of PoA link addresses and is sorted from most preferred first to least preferred last. |
| LINK_REPORT_RN | UNSIGNED_INT(1) | The reason for generating a link parameter report. 0: PERIODIC 1: ABOVE_THRESHOLD 2: BELOW_THRESHOLD 2-255: (Reserved) |
| LINK_RR_STATUS | BOOLEAN | Represent a status of resource. TRUE: Retain resource. FALSE: Release resource. |
| LINK_RES_STATUS | BOOLEAN | Indicates if a resource is available or not. TRUE: Available FALSE: Not available. |
| LINK_SCAN_RSP | SEQUENCE( LINK_ADDR, NETWORK_ID, SIG_STRENGTH ) | Represents a scan response. The LINK_ADDR contains the PoA link address. The PoA belongs to the NETWORK_ID with the given SIG_STRENGTH. |
| LINK_STATES_REQ | BITMAP(16) | Link status to be requested. Bit 0: OP_MODE Bit 1: CHANNEL_ID Bit 2-15: (Reserved) |
| LINK_STATES_RSP | SEQUENCE( CHOICE(NULL, OP_MODE), CHOICE(NULL, CHANNEL_ID) ) | Represents a link state. |
| LINK_STATUS_REQ | SEQUENCE( CHOICE(NULL, LINK_STATES_REQ), CHOICE(NULL, LIST(LINK_PARAM_TYPE)), CHOICE(NULL, LINK_DESC_REQ) ) | Represents the possible information to request from a link. |
| LINK_STATUS_RSP | SEQUENCE( LINK_ID, LIST(LINK_STATES_RSP), LIST(LINK_PARAM), LIST(LINK_DESC_RSP) ) | A set of link status parameters for a given LINK_ID |
| LINK_TUPLE_ID | SEQUENCE( LINK_ID, CHOICE(NULL, LINK_ADDR) ) | The identifier of a link that is associated with a PoA. The optional LINK_ADDR contains a link address of PoA. |
| LINK_TYPE | UNSIGNED_INT(1) | Represents the link type. Note, the values defined are made consistent with RADIUS NAS-Port-Type definitions as specified by Internet Assigned Numbers Authority (IANA). (see RFC 2865) Number assignments: 0: Reserved 1: Wireless - GSM 2: Wireless - GPRS 3: Wireless - EDGE 15: Ethernet 18: Wireless - Other 19: Wireless - IEEE 802.11 22: Wireless - CDMA2000 23: Wireless - UMTS 24: Wireless - cdma2000-HRPD 27: Wireless - IEEE 802.16 28: Wireless - IEEE 802.20 29: Wireless - IEEE 802.22 |
| NUM_COS | UNSIGNED_INT(1) | The maximum number of differentiable classes of service supported. Valid Range: 0..255 |
| NUM_QUEUE | UNSIGNED_INT(1) | The maximum number of differentiable classes of service supported. Valid Range: 0..255 |
| OP_MODE | UNSIGNED_INT(1) | The link power mode. 0: Normal Mode 1: Power Saving Mode 2: Power Down 3-255: (Reserved) |
| OP_MODE_STATUS | UNSIGNED_INT(1) | Operation mode setting status. 0: Success 1: Error 2-255: (Reserved) |
| SIG_STRENGTH | PERCENTAGE | Represents the signal strength. 00 - 24: Poor 25 - 49: Average 50 - 74: Good 75 -100: Excellent |
| THRESHOLD | SEQUENCE( THRESHOLD_VAL, THRESHOLD_X_DIR ) | A link threshold. The threshold is considered crossed when the value of the link parameter passes the threshold in the specified direction. |
| THRESHOLD_VAL | UNSIGNED_INT(2) | Threshold value. The format of the media-dependent value is defined in the respective media specification standard and the equivalent number of bits (i.e. first bits) of this data type is used. In case that there are remaining unused bits in the data type, these are marked as all-zeros (‘0’). Vsalid Range: 0..65535 |
| THRESHOLD_X_DIR | UNSIGNED_INT(1) | The direction the threshold is to be crossed. 0: ABOVE_THRESHOLD 1: BELOW_THRESHOLD 2-255: (Reserved) |
| TIMER_INTERVAL | UNSIGNED_INT(2) | This timer value (ms) is used to set the interval between periodic reports. Valid Range: 0..65535 |

| - Link actions |  |
| --- | --- |
| Action Name | Description |
| LINK_DISCONNECT | Disconnect the link connection directly. |
| LINK_LOW_POWER | Cause the link to adjust its battery power level to be low power consumption. |
| LINK_POWER_DOWN | Cause the link to power down and turn off the radio. |
| LINK_POWER_UP | Cause the link to power up and establish L2 connectivity. For UMTS link type, power up lower layers and establish PDP context. |

| - Link action attributes |  |
| --- | --- |
| Action Name | Description |
| DATA_FWD_REQ | This indication requires the buffered data at the old serving PoA entity to be forwarded to the new target PoA entity in order to avoid data loss. This action can be taken immediately after the old serving PoS receives MIH_N2N_HO_Commit response message from the new target PoS, or the old serving PoS receives MIH_Net_HO_Commit response message from the MN. This is not valid on UMTS link type. |
| LINK_RES_RETAIN | The link will be disconnected but the resource for the link connection still remains so reestablishing the link connection later can be more efficient. |
| LINK_SCAN | Cause the link to perform a scan. |

| - Link Down reason code |  |  |
| --- | --- | --- |
| Reason code | Reason | Description |
| 0 | Explicit Disconnect | The link is down because of explicit disconnect procedures initiated either by MN or network. |
| 1 | Packet Timeout | The link is down because no acknowledgements were received for transmitted packets within the specified time limit. |
| 2 | No Resource | The link is down because there were no resources to maintain the connection. |
| 3 | No Broadcast | The link is down because broadcast messages (control frames such as beacons) could not be received by MN. |
| 4 | Authentication Failure | Authentication failure. |
| 5 | Billing Failure | Billing failure. |
| 6-127 | (Reserved) | Reserved for IEEE 802.21 future use. |
| 128-255 | Vendor Specific Reason Codes | Vendors specify their own specific reason codes in this range. |

| - Link Going Down reason code |  |  |
| --- | --- | --- |
| Reason code | Reason | Description |
| 0 | Explicit Disconnect | The link is going to be down because explicit disconnect procedures will be initiated either by MN or network. For example, when a BS has decided to shutdown for administrative reasons or an operator of the terminal has decided to execute a handover manually, a Link_Going_Down trigger is sent to the MIHF. |
| 1 | Link Parameter Degrading | The link is going to be down because broadcast messages (control frames such as beacons) could not be received by MN. |
| 2 | Low Power | The link is going to be down because the power level of the terminal is low and the current link will not be maintained in such a low power level. Mobile terminals usually have limited battery supply, and when the battery level of the terminal is low, a terminal can choose a link that has lower power consumption for handover according to the received Link_Going_Down triggers with the reason code RC_LOW_POWER. This will lengthen the usable time for the terminal. |
| 3 | No Resource | The link is going to be down because there will be no resources to maintain the current connection. For example, a BS that has too many users can send Link_Going_Down indications to terminals when the links with them can not be kept because of insufficient resources. Another example is that users with higher priority can preempt the ones with lower priority when no more resources can be allocated in 3GPP, and this can also cause a Link_Going_Down indication with RC_NORESOURCE reason code. |
| 4-127 | (Reserved) | Reserved for IEEE 802.21 future use. |
| 128-255 | Vendor Specific Reason Codes | Vendors specify their own specific reason codes in this range. |

- Data types for QoS

The data types defined in this subclause are related to QoS.

| - Data types for QoS |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| QOS_LIST | SEQUENCE( NUM_COS_TYPES, LIST(MIN_PK_TX_DELAY), LIST(AVG_PK_TX_DELAY), LIST(MAX_PK_TX_DELAY), LIST(PK_DELAY_JITTER), LIST(PK_LOSS_RATE) ) | A list of Class of Service (CoS) parameters. |
| NUM_COS_TYPES | UNSIGNED_INT(1) | A type to represent the maximum number of differentiable classes of service supported. Valid Range: 0..255 |
| MIN_PK_TX_DELAY | SEQUENCE( COS_ID, UNSIGNED_INT(2) ) | A type to represent the minimum packet transfer delay in ms for the specific CoS specified by the COS_ID. |
| AVG_PK_TX_DELAY | SEQUENCE( COS_ID, UNSIGNED_INT(2) ) | A type to represent the average packet transfer delay in ms for the specific CoS specified by the COS_ID. |
| MAX_PK_TX_DELAY | SEQUENCE( COS_ID, UNSIGNED_INT(2) ) | A type to represent the maximum packet transfer delay in ms for the specific CoS specified by the COS_ID. |
| PK_DELAY_JITTER | SEQUENCE( COS_ID, UNSIGNED_INT(2) ) | A type to represent the packet transfer delay jitter in ms for the specific CoS specified by the COS_ID. |
| PK_LOSS_RATE | SEQUENCE( COS_ID, UNSIGNED_INT(2) ) | A type to represent the packet loss rate for the specific CoS specified by the COS_ID. The loss rate is equal to the integer part of the result of multiplying -100 times the log10 of the ratio between the number of packets lost and the total number of packets transmitted in the class population of interest. |
| COS_ID | UNSIGNED_INT(1) | A type to represent a class of service identifier. Valid Range: 0 - 255 |
| QOS_PARAM_VAL | CHOICE( NUM_COS_TYPES, LIST(MIN_PK_TX_DELAY), LIST(AVG_PK_TX_DELAY), LIST(MAX_PK_TX_DELAY), LIST(PK_DELAY_JITTER), LIST(PK_LOSS_RATE) ) | A choice of Class of Service (CoS) parameters. |

- Data types for location

| - Data types for location |  |  |
| --- | --- | --- |
| Type name | Derived from | Definition |
| LOCATION | CHOICE( CIVIC_LOC, GEO_LOC, CELL_ID ) | A type to represent the format and value of the location information. The location can be civic location, geospacial location, or a cellular ID value as reference location. |
| CIVIC_LOC | CHOICE( BIN_CIVIC_LOC, XML_CIVIC_LOC ) | A type to represent a civic address. |
| BIN_CIVIC_LOC | SEQUENCE( CNTRY_CODE, CIVIC_ADDR ) | A type to represent a binary-formatted civic address. See CNTRY_CODE and CIVIC_ADDR definitions. |
| XML_CIVIC_LOC | OCTET_STRING | A type to represent an XML-formatted civic location. Civic address elements, as described in RFC4119. |
| CIVIC_ADDR | OCTET_STRING | A type to represent civic address elements in BIN_CIVIC_LOC. Civic address elements, as described in RFC4776. |
| GEO_LOC | CHOICE( BIN_GEO_LOC, XML_GEO_LOC ) | A type to represent a geospatial location. |
| BIN_GEO_LOC | CHAR(16) | A type to represent a binary-formatted geospatial location. See Value field format of PoA location information (geospatial location). |
| XML_GEO_LOC | OCTET_STRING | A type to represent an XML-formatted geospatial location. Geo address elements as described in IETF RFC 4119. For example, <gml:location> <gml:Point gml:id=“point1” srsName=“epsg:4326”> <gml:coordinates>37:46:30N 122:25:10W</gml:coordinates> </gml:Point> </gml:location> |

| - Value field format of PoA location information (geospatial location) |  |  |
| --- | --- | --- |
| Syntax | Length (bits) | Notes (See RFC 3825 for details) |
| LatitudeResolution (LaRes) | 6 | Latitude resolution. 6 bits indicating the number of valid bits in the fixed-point value of Latitude. Any bits entered to the right of this limit should not be considered valid and might be purposely false, or zeroed by the sender. |
| Latitude | 34 | A 34 bit fixed point value consisting of 9 bits of integer and 25 bits of fraction. Latitude should be normalized to within +/- 90 degrees. Positive numbers are north of the equator and negative numbers are south of the equator. |
| LongitudeResolution (LoRes) | 6 | Longitude resolution. 6 bits indicating the number of valid bits in the fixed-point value of Longitude. This value is the number of high-order Longitude bits that should be considered valid. Any bits entered to the right of this limit should not be considered valid and might be purposely false, or zeroed by the sender. |
| Longitude | 34 | A 34 bit fixed point value consisting of 9 bits of integer and 25 bits of fraction. Longitude should be normalized to within +/- 180 degrees. Positive values are East of the prime meridian and negative (2s complement) numbers are West of the prime meridian. |
| AltitudeType (AT) | 4 | Following codes are defined: 1: Meters: in 2s-complement fixed-point 22-bit integer part with 8-bit fraction. If AT = 1, an AltRes value 0.0 would indicate unknown altitude. The most precise Altitude would have an AltRes value of 30. Many values of AltRes would obscure any variation due to vertical datum differences. 2: Floors: in 2s-complement fixed-point 22-bit integer part with 8-bit fraction. AT = 2 for Floors enables representing altitude in a form more relevant in buildings which have different floor-to-floor dimensions. |
| AltitudeResolution (AltRes) | 6 | Altitude resolution. 6 bits indicating the number of valid bits in the altitude. Values above 30 (decimal) are undefined and reserved. |
| Altitude | 30 | A 30 bit value defined by the AT field. |
| Datum | 8 | Following codes are defined: 1: WGS 2: NAD 83 (with associated vertical datum for North American vertical datum for 1998) 3: NAD 83 (with associated vertical datum for Mean Lower Low Water (MLLW)) |

- Data types for IP configuration

| - Data types for IP configuration |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| IP_CFG_MTHDS | BITMAP(32) | A set of IP configuration methods. Bit 0: IPv4 static configuration Bit 1: IPv4 dynamic configuration (DHCPv4) Bit 2: Mobile IPv4 with foreign agent (FA) care of address (CoA) (FA-CoA) Bit 3: Mobile IPv4 without FA (Co-located CoA) Bits 4-10: reserved for IPv4 address configurations Bit 11: IPv6 stateless address configuration Bit 12: IPv6 stateful address configuration (DHCPv6) Bit 13: IPv6 manual configuration Bits 14-31: (Reserved) |
| IP_CFG_STATUS | BITMAP(8) | Status of the IP configuration methods. Bit 0: IP configuration Method is not available Bit 1: DHCP Server address is not available Bit 2: FA address is not available Bit 3: Access Router Address is not available Bit 4: No information is provided due to accessibility of same entity (FA, Access Router, DHCP Server, etc.) Bit 5-7: (Reserved) |
| IP_MOB_MGMT | BITMAP(16) | Indicates the supported mobility management protocols. Bit 0: Mobile IPv4 (RFC3344) Bit 1: Mobile IPv4 Regional Registration (RFC4857) Bit 2: Mobile IPv6 (RFC3375) Bit 3: Hierarchical Mobile IPv6 (RFC4140) Bit 4: Low Latency Handoffs (RFC4881) Bit 5: Fast Handovers for Mobile IPv6 (RFC4068) Bit 6: IKEv2 Mobility and Multihoming Protocol (RFC4555) Bit 7-15: (Reserved) |
| IP_PREFIX_LEN | UNSIGNED_INT(1) | The length of an IP subnet prefix. Valid Range: 0..32 for IPv4 subnet. 0..64, 66..127 for IPv6 subnet. |
| IP_RENEWAL_FLAG | BOOLEAN | Indicates whether MN’s IP address needs to be changed or not. TRUE: Change required. FALSE: Change not required. |
| IP_SUBNET_INFO | SEQUENCE( IP_PREFIX_LEN, IP_ADDR ) | Represent an IP subnet. The IP_PREFIX_LEN contains the bit length of the prefix of the subnet to which the IP_ADDR belongs. |

- Data types for information elements

Data types defined in this subclause are used only by IEs.

| - Data types for information elements |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| NET_AUX_ID | OCTET_STRING | A type to represent an auxiliary access network identifier. This is SSID if network type is IEEE 802.11. |
| NETWORK_ID | OCTET_STRING | A type to represent a network identifier. A non-NULL terminated string whose length shall not exceed 253 octets. |
| BAND_CLASS | INTEGER(1) | CDMA band class. |
| BANDWIDTH | INTEGER(1) | Channel bandwidth. |
| BASE_ID | INTEGER(2) | Base station identifier. |
| BURST_PROF | SEQUENCE( DOWN_BP, UP_BP ) | Burst profile |
| CH_RANGE | SEQUENCE( UNSIGNED_INT(4), UNSIGNED_INT(4) ) | A type contains two numbers. The first unsigned integer is the low range. The second unsigned integer is the high range. Both values are in KHz. The first unsigned integer value should always be less or equal to the second unsigned integer. For example, [2412000, 5000000] would indicate a channel range from 2.4GHz to 5.0GHz. |
| COST | SEQUENCE( COST_UNIT, COST_VALUE, COST_CURR ) | A type to represent a cost. |
| COST_CURR | CHAR(3) | A type to represent the currency of a cost. A three-letter currency code (e.g., “USD”) specified by ISO 4217 [ISO 4217]. |
| COST_UNIT | UNSIGNED_INT(1) | A type to represent the unit of a cost. 0: second 1: minute 2: hours 3: day 4: week 5: month 6: year 7: free 8: flat rate 9-255: (Reserved) |
| COST_VALUE | SEQUENCE( UNSIGNED_INT(4), UNSIGNED_INT(2) ) | A type to represent the value of a cost. The first 4-octet contains the integer part of the cost. The last 2-octet contains the fraction part where it represents a 3-digit fraction. Therefore, the value range of the fraction part is [0,999]. For example, for a value of “0.5”, the integer part is zero and the fraction part is 500. |
| CNTRY_CODE | CHAR(2) | Country code, represented as two letter ISO 3166-1 country code in capital ASCII letters. |
| DATA_RATE | UNSIGNED_INT(4) | A type to represent the maximum data rate in kb/s. Valid Range: 0..2^32-1 |
| DCD_UCD | SEQUENCE( BASE_ID, BANDWIDTH, DU_CTR_FREQ, EIRP, GAP, BURST_PROF, CDMA_CODES ) | A type to represent DCD _UCD. |
| DOWN_BP | BITMAP(256) | A List of FEC Code Type for Downlink burst. Refer to subclause 11.4.1 in IEEE 802.16Rev2/D0d [ref]. |
| EIRP | INTEGER(1) | BS’s effective isotropic radiated power level. Signed in units of 1 dBM. |
| FQDN | OCTET_STRING | The fully qualified domain name of a host as described in RFC 2181. |
| DU_CTR_FREQ | INTEGER(8) | Downlink/Uplink center frequency in KHz. |
| FREQ_ID | INTEGER(2) | Identifier the carrier frequency. Valid Range: 0..65535 |
| FQ_CODE_NUM | INTEGER(2) | UMTS scrambling code, cdma2000 Walsh code. Valid Range: 0..65535 |
| GAP | SEQUENCE( TTG, RTG ) |  |
| HO_CODE | INTEGER(1) | HANDOVER_RANGING_CODE. Refer to subclause 11.3.1 in IEEE 802.16Rev2/D0d [ref]. |
| INIT_CODE | INTEGER(1) | INITIAL_RANGING_CODE. Refer to subclause 11.3.1 in IEEE 802.16Rev2/D0d [ref]. |
| IP4_ADDR | CHAR(4) | An IPv4 address as described in RFC 791. |
| IP6_ADDR | CHAR(16) | An IPv6 address as described in RFC 2373. |
| NET_CAPS | BITMAP(32) | A type to represents a set of network capabilities. Bitmap Values: Bit 0: Security Bit 1: QoS Bit 2: Internet Access Bit 3: Emergency Services Bit 4: MIH Capability Bit 5-31: (Reserved) |
| NETWORK_TYPE | SEQUENCE( CHOICE(NULL, LINK_TYPE), CHOICE(NULL, REVISION), CHOICE(NULL, TYPE_EXT) ) | A type to represent a network type and its revision. See Network type and revision representation for details. |
| OPERATOR_ID | SEQUENCE( OP_NAME, OP_NAMESPACE ) | A type to represent an operator identifier. |
| OP_NAME | OCTET_STRING | A type to represent a operator name. The value uniquely identifies the operator name within the scope of the OP_NAMESPACE. The value is a non NULL terminated string whose length shall not exceed 253 octets. |
| OP_NAMESPACE | UNSIGNED_INT(1) | A type to represent a type of operator name. 0: GSM/UMTS 1: CDMA 2: REALM (as defined in [30]). 3: ITU-T/TSB 4-255: (Reserved) |
| PARAMETERS | CHOICE( DCD_UCD, SIB, SYS_PARAMS ) | A data type to represent system information depending on the network type. DCD_UCD: IEEE 802.16 SIB: UMTS SYS_PARAMS: cdma2000 |
| PILOT_PN | INTEGER(2) | Pilot PN sequence offset index. |
| PROXY_ADDR | CHOICE( IP4_ADDR, IP6_ADDR, FQDN ) | L3 address of a proxy server. |
| CDMA_CODES | SEQUENCE( INIT_CODE, HO_CODE ) | A set of CDMA ranging codes. |
| REVISION | BITMAP(64) | A network type revision. See Network type and revision representation. |
| ROAMING_PTNS | LIST(OPERATOR_ID) | A list of roaming partners. |
| RTG | INTEGER(1) | Receive transition gap. Physical slot unit. |
| SP_ID | OCTET_STRING | A service provider identifier. A non-NULL terminated string whose length shall not exceed 253 octets. |
| SIB | SEQUENCE( CELL_ID, FQ_CODE_NUM ) | A type to represent SIB. [??? Reference to SIB???] |
| SUPPORTED_LCP | UNSIGNED_INT(1) | A type represent supported Location Configuration Protocol. (LCP). LbyR: Location by Reference. Values represent LCPs: 0: NULL 1: LLDP 2: LbyR with LLDP 3-10: (Reserved) 11: LLDP-MED 12: LbyR with LLDP-MED 13-20: (Reserved) 21: U-TDoA 22: D-TDoA 23-30: (Reserved) 31: DHCP 32: LbyR with DHCP 33-40: (Reserved) 41: OMA SUPL 42-50: (Reserved) 51: HELD 52: LbyR with HELD 53-255: (Reserved) |
| SYSTEM_INFO | SEQUENCE( NETWORK_TYPE, MAC_ADDR, PARAMETERS ) | A type to represent system information . |
| SYS_PARAMS | SEQUENCE ( BASE_ID, PILOT_PN, FREQ_ID, BAND_CLASS ) | A type to represent SYS_PARAMS. |
| TTG | INTEGER(2) | Transmit transition gap. Physical slot unit. |
| TYPE_EXT | OCTET_STRING | A generic type extension contained indicating a flexible length and format field. The content is to be defined and filled by the appropriate SDO or service provider consortium, etc. The value is a non-NULL terminated string whose length shall not exceed 253 octets. |
| UP_BP | BITMAP(256) | A List of FEC Code Type for Uplink burst. Refer to subclause 11.3.1 in IEEE 802.16Rev2/D0d [ref]. |

| - Network type and revision representation |  |  |
| --- | --- | --- |
| Network | Link Type | Revision |
| (Reserved) | 0 | N/A |
| Wireless - GSM | 1 | N/A |
| Wireless - GPRS | 2 | N/A |
| Wireless - EDGE | 3 | N/A |
| (Reserved) | 4-14 | N/A |
| Ethernet | 15 | Bit 0: 10Mb Bit 1: 100Mb Bit 2: 1000Mb Bit 3-63: (Reserved) |
| (Reserved) | 16-17 | N/A |
| Wireless - Other | 18 | N/A |
| Wireless - IEEE 802.11 | 19 | (PHY/MAC Features) Bit 0: Access point (AP) Bit 1: Independent station (not an AP) Bit 2: FHSS PHY for 2.4GHz band Bit 3: DSSS PHY for 2.4GHz band Bit 4: IR PHY Bit 5: OFDM PHY Bit 6: High-speed PHY Bit 7: Multi-domain operation capability implemented Bit 8: Extended Rate PHY (ERP) Bit 9: Spectrum management operation supported Bit 10: Regulatory class capability implemented Bit 11: QoS Supported Bit 12: Robust Secure Network (RSN) supported Bit 13-63: (Reserved) |
| (Reserved) | 20-21 | N/A |
| Wireless - CDMA2000 | 22 | N/A |
| Wireless - UMTS | 23 | Bit 0: Rel-99 Bit 1: Rel-4 Bit 2: Rel-5 (w/ HSDPA) Bit 3: Rel-6 (w/ HSUPA) Bit 4: Rel-7 (MIMO/OFDM) Bit 5: Rel-8 Bit 6-63: (Reserved) |
| Wireless - cdma2000-HRPD | 24 | Bit 0: Rev-0 Bit 1: Rev-A Bit 2: Rev-B Bit 3: Rev-C Bit 4-63: (Reserved) |
| (Reserved) | 25-26 | N/A |
| Wireless - IEEE 802.16 | 27 | Bit 0: IEEE 802.16-2001 Bit 1: IEEE 802.16c-2002 Bit 2: IEEE 802.16a-2003 Bit 3: IEEE 802.16-2004 Bit 4: IEEE 802.16e-2005 Bit 5: IEEE 802.16g-2007 Bit 6-63: (Reserved) |
| Wireless - IEEE 802.20 | 28 | N/A |
| Wireless - IEEE 802.22 | 29 | N/A |
| (Reserved) | 30-255 | N/A |

Note, the Link Type values in Network type and revision representation are deliberately made consistent with RADIUS network access server (NAS)-Port-Type definitions as specified by Internet Assigned Numbers Authority (IANA).

- Data types for information service query

- Binary representation

| - Data types for binary query |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| CURR_PREF | COST_CURR | A type to indicate currency preference. |
| IE_TYPE | UNSIGNED_INT(4) | A type to represent an IE type. See Type identifiers for information elements for more information. |
| IQ_BIN_DATA | SEQUENCE( CHOICE(NULL, QUERIER_LOC), CHOICE(NULL, NET_TYPE_INC), CHOICE(NULL, NETWK_INC), CHOICE(NULL, RPT_TEMPL), CHOICE(NULL, RPT_LIMIT), CHOICE(NULL, CURR_PREF) ) | Represents a binary query. There should exist at least one of the query data type QUERIER_LOC, NET_TYPE_INC, and NETWK_INC. One CURR_PREF at most is included in an Info Query Binary TLV. If included, it indicates to the MIIS server the preferred currency the returned cost should be represented in. If the MIIS server cannot return the cost in the specified currency, it can return the cost in other currencies. |
| NGHB_RADIUS | UNSIGNED_INT(4) | The radius in meters from the center point of querier's location. Valid Range: 0..2^32-1 |
| NETWK_INC | LIST(NETWORK_ID) | A type to represent a list of network identifiers. |
| NET_TYPE_INC | BITMAP(32) | A type to represent a set of link types. The value is a 4 octet bitmap: Bit 0: Wireless - GSM Bit 1: Wireless - GPRS Bit 2: Wireless - EDGE Bit 3: IEEE 802.3 (Ethernet) Bit 4: Wireless - Other Bit 5: Wireless - IEEE 802.11 Bit 6: Wireless - CDMA2000 Bit 7: Wireless - UMTS Bit 8: Wireless - cdma2000-HRPD Bit 9: Wireless - IEEE 802.16 Bit 10: Wireless - IEEE 802.20 Bit 11: Wireless - IEEE 802.22 Bit 12-31: (Reserved AND shall be always set to “0”) |
| QUERIER_LOC | SEQUENCE( CHOICE(NULL, LOCATION), CHOICE(NULL, MAC_ADDR), CHOICE(NULL, NGHB_RADIUS) ) | A type to represent a querier's location. It is not valid to use both LOCATION and MAC_ADDR at the same time. |
| RPT_LIMIT | SEQUENCE( UNSIGNED_INT(2), UNSIGNED_INT(2) ) | A type to represent a report limitation. The first UNSIGNED_INT(2) contains the maximum number of IEs in the IR_BIN_DATA. The second UNSIGNED_INT(2) contains the starting entry number (offset = 1 points to the first entry) from which a chunk of entries are to be included in the IQ_BIN_DATA. It is assumed that the IS server generates the same ordered list of entries for queries from the same IS client with the same IR_BIN_DATA content (except for RPT_LIMIT) before the limitation on the RPT_LIMIT is applied. |
| RPT_TEMPL | LIST(IE_TYPE) | A type to represent a list of IE types. Inclusion of any IE type is optional. |

- RDF representation

| - Data type for RDF query |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| IQ_RDF_SCHM | OCTET_STRING | A type to represent the URL of an RDF schema to obtain. |
| IQ_RDF_DATA | SEQUENCE( CHOICE(NULL, MIME_TYPE), OCTET_STRING ) | Represents RDF query. If MIME_TYPE is omitted, MIME type “application/sparql-query” is used. Each OCTET_STRING is formatted with the MIME type. |
| MIME_TYPE | OCTET_STRING | Represents MIME type. |

- Data types for information service response

- Binary representation

| - Data type for binary information query response |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| IR_BIN_DATA | LIST(INFO_ELEMENT) | A type to represent a binary query response data. |

- RDF representation

| - Data type for RDF information query response |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| IR_RDF_DATA | SEQUENCE( CHOICE(NULL, MIME_TYPE), OCTET_STRING ) | Represents RDF data query result. If MIME_TYPE is omitted, MIME type “application/ sparql-results+xml” is used. OCTET_STRING is formatted with the MIME type. |
| IR_SCHM_URL | OCTET_STRING | An URL of an RDF schema. |
| IR_RDF_SCHM | SEQUENCE( CHOICE(NULL, MIME_TYPE), OCTET_STRING ) | Represents an RDF schema. If MIME_TYPE is omitted, MIME type “application/xml” is used. OCTET_STRING is formatted with the MIME type. |

- Data type for MIHF identification

| - Data type for MIHF identification |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| MIHF_ID | OCTET_STRING | The MIHF Identifier. MIHF-ID is an invariant even if the device obtains different interfaces or other components in its registration lifetime. For example, MIHF-ID may be an FQDN or NAI. |

- Data type for MIH capabilities

| - Data type for MIH capabilities |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| EVT_CFG_INFO | SEQUENCE( MIH_EVENT_ID, CHOICE(NULL, LIST(LINK_DET_CFG)) ) | Represents additional configuration information for event subscription. |
| LINK_DET_CFG | SEQUENCE( CHOICE(NULL, NETWORK_ID), CHOICE(NULL, SIG_STRENGTH), CHOICE(NULL, LINK_DATA_RATE) ) | A data type for configuring link detected event trigger. |
| LINK_DET_INFO | SEQUENCE ( LINK_TUPLE_ID, NETWORK_ID, SIG_STRENGTH, LINK_DATA_RATE, LINK_MIHCAP_FLAG ) | Information of a detected link. |
| MBB_HO_SUPP | SEQUENCE( NETWORK_TYPE, NETWORK_TYPE, BOOLEAN ) | Indicates if make before break is supported FROM the first network type TO the second network type. The BOOLEAN value assignment: True: Make before break is supported. False: Make before break is not supported. |
| MIH_CMD_LIST | BITMAP(32) | A set of MIH commands. Bitmap Values: Bit 0: MIH_Link_Get_Parameters Bit 1: MIH_Link_Configure_Thresholds Bit 2: MIH_Link_Actions Bit 3: MIH_Net_HO_Candidate_Query MIH_Net_HO_Commit MIH_N2N_HO_Query_Resources MIH_N2N_HO_Commit MIH_N2N_HO_Complete Bit 4: MIH_MN_HO_Candidate_Query MIH_MN_HO_Complete Bit 5-31: (Reserved) |
| MIH_EVENT_ID | UNSIGNED_INT(1) | MIH event identifiers. 0: MIH_Link_Detected 1: MIH_Link_Up 2: MIH_Link_Down 3: MIH_Link_Event_Rollback 4: MIH_Link_Parameters_Report 5: MIH_Link_Going_Down 6: MIH_Link_Handover_Imminent 7: MIH_Link_Handover_Complete 8: MIH_Link_PDU_Transmit_Status 9-255: (Reserved) |
| MIH_EVT_LIST | BITMAP(32) | A set of MIH events. Bitmap Values: Bit 0: MIH_Link_Detected Bit 1: MIH_Link_Up Bit 2: MIH_Link_Down Bit 3: MIH_Link_Event_Rollback Bit 4: MIH_Link_Parameters_Report Bit 5: MIH_Link_Going_Down Bit 6: MIH_Link_Handover_Imminent Bit 7: MIH_Link_Handover_Complete Bit 8: MIH_Link_PDU_Transmit_Status Bit 9-31: (Reserved) |
| MIH_IQ_TYPE_LST | BITMAP(64) | A set of IS query types. Bitmap Values: Bit 0: Binary data Bit 1: RDF data Bit 2: RDF schema URL Bit 3: RDF schema Bit 4: IE_NETWORK_TYPE Bit 5: IE_OPERATOR_ID Bit 6: IE_SERVICE_PROVIDER_ID Bit 7: IE_COUNTRY_CODE Bit 8: IE_NETWORK_ID Bit 9: IE_NETWORK_AUX_ID Bit 10: IE_ROAMING_PARTNERS Bit 11: IE_COST Bit 12: IE_NETWORK_QOS Bit 13: IE_NETWORK_DATA_RATE Bit 14: IE_NET_CHANNEL_RANGE Bit 15: IE_NET_IP_CFG_METHODS Bit 16: IE_NET_CAPABILITIES Bit 17: IE_NET_SUPPORTED_LCP Bit 18: IE_NET_MOB_MGMT_PROT Bit 19: IE_NET_EMSERV_PROXY Bit 20: IE_NET_IMS_PROXY_CSCF Bit 21: IE_POA_MAC_ADDR Bit 22: IE_POA_LOCATION Bit 23: IE_POA_CHANNEL_RANGE Bit 24: IE_POA_SYSTEM_INFO Bit 25: IE_POA_SUBNET_INFO Bit 26: IE_POA_IP_ADDR Bit 27- 63: (Reserved) |
| MIH_TRANS_LST | BITMAP(16) | List of supported transports. Bitmap Values: Bit 0: UDP Bit 1: TCP Bit 2-15: (Reserved) |
| NET_TYPE_EVTS | SEQUENCE( NETWORK_TYPE, MIH_EVT_LIST ) | Represent the supported events on a given network type. |
| NET_TYPE_MAC | SEQUENCE( NETWORK_TYPE, MAC_ADDR ) | Represent an MAC address of a specific network type. |

- Data type for MIH registration

| - Data type for MIH registration |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| REG_REQUEST_CODE | ENUMERATED | The registration code: 0 - Registration 1 - Re-Registration |

- Data types for handover operation

| - Data type for handover operation |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| ASGN_RES_SET | SEQUENCE ( QOS_ LIST, TSP_CONTAINER, LINK_AC_EX_TIME ) | Set of resource parameters reserved and assigned by the target network to the MN for performing handover to a network PoA. The transparent container is from target to source which includes the required configuration of the reserved resources at the target network. |
| HO_CAUSE | UNSIGNED_INT(1) | Represents the reason for performing a handover. Same enumeration list as link down reason code. See Link Down reason code. |
| HO_RESULT | ENUMERATED | Handover result. 0: Success 1: Failure 2: Rejected |
| HO_STATUS | ENUMERATED | Represents the permission for handover. 0: HandoverPermitted 1: HandoverDeclined |
| PREDEF_CFG_ID | INTEGER(1) | Pre-defined configuration identifier. 0..255 |
| RQ_RESULT | SEQUENCE( LINK_POA_LIST, QOS_LIST, CHOICE(NULL, IP_CFG_MTHDS), CHOICE(NULL, DHCP_SERV), CHOICE(NULL, FN_AGNT), CHOICE(NULL, ACC_RTR) ) | Represents the result of network resource query. The LINK_POA_LIST is a list of potential PoAs for a given link with the same IP configuration information. The list is sorted from most preferred first to least preferred last. The QOS_LIST contains the available resources for this list of PoAs. IP_CFG_MTHDS represents the IP configuration methods applicable. |
| DHCP_SERV | IP_ADDR | IP address of candidate DHCP Server. It is only included when dynamic address configuration is supported. |
| FN_AGNT | IP_ADDR | IP address of candidate Foreign Agent. It is only included when Mobile IPv4 is supported. |
| ACC_RTR | IP_ADDR | IP address of candidate Access Router. It is only included when IPv6 Stateless configuration is supported. |
| REQ_RES_SET | SEQUENCE ( QOS_LIST, TSP_CONTAINER, HO_CAUSE ) | Set of resource parameters required for performing admission control and resource reservation for the MN at the target network. The transparent container is from source to target which includes the required MN configuration for admitting the new connection at the target network and reserving resources. |
| TGT_NET_INFO | SEQUENCE ( CHOICE(NULL, NETWORK_ID), CHOICE(NULL, NET_AUX_ID), CHOICE(NULL, LINK_ADDR) ) | Represents the handover commit information. LINK_ADDR corresponds to the target PoA. |
| TSP_CARRIER | OCTET_STRING | Transparent carrier containing link specific information whose content and format are to be specified by the link specific SDO. |
| TSP_CONTAINER | CHOICE ( NULL, PREDEF_CFG_ID, TSP_CARRIER ) | Transparent container. If the value is null, this parameters is not available. |

- Data types for MIH_NET_SAP primitives

| - Data type for MIH_NET_SAP primitives |  |  |
| --- | --- | --- |
| Type Name | Derived From | Definition |
| TRANSPORT_TYPE | ENUMERATED | The transport type supported: 0: L2 1: L3 or higher layer protocols |

- MIH protocol message code assignments

| - AID assignment |  |
| --- | --- |
| MIH messages | AID |
| MIH messages for Service Management |  |
| MIH_Capability_Discover | 1 |
| MIH_Register | 2 |
| MIH_DeRegister | 3 |
| MIH_Event_Subscribe | 4 |
| MIH_Event_Unsubscribe | 5 |
| MIH messages for Event Service |  |
| MIH_Link_Detected | 1 |
| MIH_Link_Up | 2 |
| MIH_Link_Down | 3 |
| MIH_Link_Event_Rollback | 4 |
| MIH_Link_Parameters_Report | 5 |
| MIH_Link_Going_Down | 6 |
| MIH_Link_Handover_Imminent | 7 |
| MIH_Link_Handover_Complete | 8 |
| MIH messages for Command Service |  |
| MIH_Link_Get_Parameters | 1 |
| MIH_Link_Configure_Thresholds | 2 |
| MIH_Link_Actions | 3 |
| MIH_Net_HO_Candidate_Query | 4 |
| MIH_MN_HO_Candidate_Query | 5 |
| MIH_N2N_HO_Query_Resources | 6 |
| MIH_Net_HO_Commit | 7 |
| MIH_N2N_HO_Commit | 8 |
| MIH_MN_HO_Complete | 9 |
| MIH_N2N_HO_Complete | 10 |
| MIH messages for Information Service |  |
| MIH_Get_Information | 1 |

Type values for TLV encoding provides the TLV type value assignment for MIH messages. The type value can be extracted from the binary encoding method of the corresponding data type. TLV type value 110-127 is reserved for experimental TLVs.

| - Type values for TLV encoding |  |  |
| --- | --- | --- |
| TLV Type Name | TLV Type Value | Data Type |
| Source MIHF ID |  | MIHF_ID |
| Destination MIHF ID |  | MIHF_ID |
| Status |  | STATUS |
| List of Network Type and Events |  | LIST(NET_TYPE_EVTS) |
| MIH Event List |  | MIH_EVT_LIST |
| MIH Command List |  | MIH_CMD_LIST |
| MIIS Query Type List |  | MIH_IQ_TYPE_LST |
| Transport Option List |  | MIH_TRANS_LST |
| Link MACs |  | LIST(NET_TYPE_MAC) |
| MBB Handover Support |  | LIST(MBB_HO_SUPP) |
| Register Request Code |  | REG_REQUEST_CODE |
| Valid Time Interval |  | UNSIGNED_INT(4) |
| Link Identifier |  | LINK_TUPLE_ID |
| New Link Identifier |  | LINK_TUPLE_ID |
| Old Access Router |  | LINK_ADDR |
| New Access Router |  | LINK_ADDR |
| IP Renewal Flag |  | IP_RENEWAL_FLAG |
| Mobility Management Support |  | IP_MOB_MGMT |
| IP Address Configuration Methods |  | IP_CFG_MTHDS |
| Link Down Reason Code |  | LINK_DN_REASON |
| Time Interval |  | UNSIGNED_INT(2) |
| Link Going Down Reason |  | LINK_GD_REASON |
| Unique Event Identifier |  | UNSIGNED_INT(2) |
| MIH Capability Flag |  | LINK_MIHCAP_FLAG |
| Link Parameter Report List |  | LIST(LINK_PARAM_RPT) |
| Device States Request |  | DEV_STATES_REQ |
| Link Identifier List |  | LIST(LINK_ID) |
| Device States Response |  | DEV_STATES_RSP |
| Operation Mode |  | OP_MODE |
| Operation Mode Status |  | OP_MODE_STATUS |
| Get Status Request Set |  | LINK_STATUS_REQ |
| Get Status Response List |  | LIST(LINK_STATUS_RSP) |
| Configure Request List |  | LIST(LINK_CFG_PARAM) |
| Configure Response List |  | LIST(LINK_CFG_STATUS) |
| Scan Response Set |  | LIST(LINK_SCAN_RSP) |
| List of Link PoA List |  | LIST(LINK_POA_LIST) |
| Link Action |  | LINK_ACTION |
| Preferred Link List |  | LIST(RQ_RESULT) |
| Handover Resource Response Set |  | QOS_LIST |
| Handover Resource Query List |  | QOS_LIST |
| Handover Status |  | HO_STATUS |
| Access Router Address |  | IP_ADDR |
| DHCP Server Address |  | IP_ADDR |
| FA Address |  | IP_ADDR |
| Access Router Address List |  | LIST(IP_ADDR) |
| DHCP Server Address List |  | LIST(IP_ADDR) |
| FA Address List |  | LIST(IP_ADDR) |
| IP Address Information Status |  | IP_CFG_STATUS |
| Link Actions List |  | LIST(LINK_ACTION_REQ) |
| Link Actions Result List |  | LIST(LINK_ACTION_RSP) |
| Handover Result |  | HO_RESULT |
| Resource Status |  | LINK_RES_STATUS |
| Resource Retention Status |  | LINK_RR_STATUS |
| Info Query Binary Data List |  | LIST(IQ_BIN_DATA) |
| Info Query RDF Data List |  | LIST(IQ_RDF_DATA) |
| Info Query RDF Schema URL |  | BOOLEAN |
| Info Query RDF Schema List |  | LIST(IQ_RDF_SCHM) |
| Max Response Size |  | UNSIGNED_INT(2) |
| Info Response Binary Data list |  | LIST(IR_BIN_DATA) |
| Info Response RDF Data List |  | LIST(IR_RDF_DATA) |
| Info Response RDF Schema URL List |  | LIST(IR_SCHM_URL) |
| Info Response RDF Schema List |  | LIST(IR_RDF_SCHM) |
| Mobile Node MIHF ID |  | MIHF_ID |
| Query Resource Report Flag |  | BOOLEAN |
| Target Link Action |  | LINK_ACTION |
| Link Action Result |  | LINK_AC_RESULT |
| Target Link Action Result |  | LINK_AC_RESULT |
| Event Configuration Info List |  | LIST(EVT_CFG_INFO) |
| Target Network Info |  | TGT_NET_INFO |
| List of Target Network Info |  | LIST(TGT_NET_INFO) |
| Assigned Resource Set |  | ASGN_RES_SET |
| Link Detected Info List |  | LIST(LINK_DET_INFO) |
| MN Link ID |  | LINK_ID |
| PoA |  | LINK_ADDR |
| (Reserved) | 76-109 | (Reserved |
| (Reserved for experimental TLVs) | 110-127 | ( Used for experimental purposes) |

- MIIS basic schema

The remaining text in this section defines the RDF vocabularies for MIIS.

<?xml version="1.0"?>

<!DOCTYPE rdf:RDF [

<!ENTITY rdf "http://www.w3.org/1999/02/22-rdf-syntax-ns#">

<!ENTITY rdfs "http://www.w3.org/2000/01/rdf-schema#">

<!ENTITY mihbasic "URL_TO_BE_ASSIGNED">

<!ENTITY owl "http://www.w3.org/2002/07/owl#">

<!ENTITY xsd "http://www.w3.org/2001/XMLSchema#">

]>

<rdf:RDF xmlns:rdf="&rdf;" xmlns:rdfs="&rdfs;"

xmlns:mihbasic="&mihbasic;" xml:base="&mihbasic;"

xmlns:owl="&owl;" xmlns:xsd="&xsd;">

<owl:Ontology rdf:about="">

<rdfs:label>

Basic Schema for IEEE 802.21 Information Service

</rdfs:label>

<owl:versionInfo>1.0</owl:versionInfo>

</owl:Ontology>

<owl:DatatypeProperty rdf:ID="ie_type_identifier">

<rdfs:subPropertyOf rdf:resource="&rdfs;label"/>

<rdfs:range rdf:resource="&xsd;hexBinary"/>

<rdfs:comment>

A type identifier values for Information Elements.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="bit_number">

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

This property represents a bit number that has

the value as true.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="ie_container_list_of_networks">

<mihbasic:ie_type_identifier>0x10000300</mihbasic:ie_type_identifier>

<rdfs:range rdf:resource="#LIST_OF_NETWORKS"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="LIST_OF_NETWORKS">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#ie_container_network"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:ObjectProperty rdf:ID="ie_container_network">

<mihbasic:ie_type_identifier>0x10000301</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#LIST_OF_NETWORKS"/>

<rdfs:range rdf:resource="#NETWORK"/>

<rdfs:comment>

This class contains General Information depicting and Access

Network Specific Information.

</rdfs:comment>

</owl:ObjectProperty>

<owl:Class rdf:ID="NETWORK">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#ie_network_type"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#ie_operator_identifier"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:ObjectProperty rdf:ID="ie_network_type">

<mihbasic:ie_type_identifier>0x10000000</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#NETWORK_TYPE"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="NETWORK_TYPE">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#link_type"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="link_type">

<rdfs:domain rdf:resource="#NETWORK_TYPE"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

<rdfs:comment>

Link type of a network. The following values are assigned:

1: Wireless - GSM

2: Wireless - GPRS

3: Wireless - EDGE

15: Ethernet

18: Wireless - Other

19: Wireless - IEEE 802.11

22: Wireless - CDMA2000

23: Wireless - UMTS

24: Wireless - cdma-2000-HRPD

27: Wireless - IEEE 802.16

28: Wireless - IEEE 802.20

29: Wireless - IEEE 802.22

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="revision">

<rdfs:subPropertyOf rdf:resource="#bit_number"/>

<rdfs:domain rdf:resource="#NETWORK_TYPE"/>

<rdfs:comment>

The range of #bit_number is 0-63.

</rdfs:comment>

</owl:ObjectProperty>

<owl:DatatypeProperty rdf:ID="type_extension">

<rdfs:domain rdf:resource="#NETWORK_TYPE"/>

<rdfs:range rdf:resource="&xsd;string"/>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="ie_operator_id">

<mihbasic:ie_type_identifier>0x10000001</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#OPERATOR_ID"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="OPERATOR_ID">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#operator_namespace"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#operator_name"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="operator_namespace">

<rdfs:domain rdf:resource="#OPERATOR_ID"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

A value of Operator Type:

0: GSM/UMTS

1: CDMA

2: REALM

3: ITU-T/TSB

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="operator_name">

<rdfs:domain rdf:resource="#OPERATOR_ID"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

The value is a non NULL terminated

string whose length will not exceed 253 octets.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="ie_service_provider_id>

<mihbasic:ie_type_identifier>0x10000002</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

A non-NULL terminated string whose length will not exceed 253 octets.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="ie_country_code">

<mihbasic:ie_type_identifier>0x10000003</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="&xsd;string"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="ie_network_id">

<mihbasic:ie_type_identifier>0x10000100</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

A non-NULL terminated string whose length will not exceed 253 octets.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="ie_network_aux_id">

<mihbasic:ie_type_identifier>0x10000101</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

It is SSID if network type is IEEE 802.11.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="ie_roaming_partner">

<mihbasic:ie_type_identifier>0x10000102</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#OPERATOR_ID"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="ie_cost">

<mihbasic:ie_type_identifier>0x10000103</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#COST"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="COST">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cost_unit"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cost_value"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cost_currency"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="cost_unit">

<rdfs:domain rdf:resource="#COST"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

The unit of the cost:

0: second

1: minute

2: hours

3: day

4: week

5: month

6: year

7: free

8: flat rate

9-255: Reserved

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="cost_value">

<rdfs:domain rdf:resource="#COST"/>

<rdfs:range rdf:resource="&xsd;double"/>

<rdfs:comment>

The cost value in Currency/Unit

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="cost_currency">

<rdfs:domain rdf:resource="#COST"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

A three-letter currency code(e.g. "USD") specified by

ISO 4217.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="ie_network_qos">

<mihbasic:ie_type_identifier>0x10000105</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#QOS_LIST"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="QOS_LIST">

</owl:Class>

<owl:DatatypeProperty rdf:ID="num_qos_types">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:Class rdf:ID="COS">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cos_id"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cos_value"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="cos_id">

<rdfs:domain rdf:resource="#COS"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

A type to represent a class of service identifier.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="cos_value">

<rdfs:domain rdf:resource="#COS"/>

<rdfs:range rdf:resource="&xsd;unsignedShort"/>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="cos_min_packet_transfer_delay">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="#COS"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="cos_avg_packet_transfer_delay">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="#COS"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="cos_max_packet_transfer_delay">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="#COS"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="cos_packet_transfer_delay_jitter">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="#COS"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="cos_packet_loss_rate">

<rdfs:domain rdf:resource="#QOS_LIST"/>

<rdfs:range rdf:resource="#COS"/>

</owl:ObjectProperty>

<owl:DatatypeProperty rdf:ID="ie_network_data_rate">

<mihbasic:ie_type_identifier>0x10000106</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="ie_net_channel_range">

<mihbasic:ie_type_identifier>0x10000107</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#CH_RANGE"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="ie_net_ip_cfg_methods">

<mihbasic:ie_type_identifier>0x10000108</mihbasic:ie_type_identifier>

<rdfs:subPropertyOf rdf:resource="#bit_number"/>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:comment>

The range of #bit_number is 0-31.

</rdfs:comment>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="ie_net_capabilities">

<mihbasic:ie_type_identifier>0x10000109</mihbasic:ie_type_identifier>

<rdfs:subPropertyOf rdf:resource="#bit_number"/>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:comment>

The range of #bit_number is 0-31.

</rdfs:comment>

</owl:ObjectProperty>

<owl:DatatypeProperty rdf:ID="ie_net_supported_lcp">

<mihbasic:ie_type_identifier>0x1000010A</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="ie_net_mob_mgmt_prot">

<mihbasic:ie_type_identifier>0x1000010B</mihbasic:ie_type_identifier>

<rdfs:subPropertyOf rdf:resource="#bit_number"/>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:comment>

The range of #bit_number is 0-15.

</rdfs:comment>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="ie_net_emserv_proxy">

<mihbasic:ie_type_identifier>0x1000010C</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#EMSERV_PROXY_ADDR"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="EMSERV_PROXY_ADDR">

</owl:Class>

<owl:DatatypeProperty rdf:ID="emserv_proxy_addr_ip">

<rdfs:domain rdf:resource="#EMSERV_PROXY_ADDR"/>

<rdfs:range rdf:resource="#TRANSPORT_ADDR"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="emserv_proxy_addr_fqdn">

<rdfs:domain rdf:resource="#EMSERV_PROXY_ADDR"/>

<rdfs:range rdf:resource="&xsd;String"/>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="ie_net_ims_proxy_cscf">

<mihbasic:ie_type_identifier>0x1000010D</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#IMS_PROXY_CSCF_ADDR"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="IMS_PROXY_CSCF_ADDR">

</owl:Class>

<owl:DatatypeProperty rdf:ID="ims_proxy_cscf_addr_ip">

<rdfs:domain rdf:resource="#IMS_PROXY_CSCF_ADDR"/>

<rdfs:range rdf:resource="#TRANSPORT_ADDR"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="ims_proxy_cscf_addr_fqdn">

<rdfs:domain rdf:resource="#IMS_PROXY__CSCF_ADDR"/>

<rdfs:range rdf:resource="&xsd;String"/>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="ie_container_poa">

<mihbasic:ie_type_identifier>0x10000302</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#NETWORK"/>

<rdfs:range rdf:resource="#POA"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="POA">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#ie_poa_mac_addr"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#ie_poa_location"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#ie_poa_channel_range"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#ie_poa_system_info"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#ie_poa_subnet_info"/>

<owl:minCardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:minCardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:comment>

This class contains all the information depicting a PoA.

</rdfs:comment>

</owl:Class>

<owl:ObjectProperty rdf:ID="ie_poa_mac_addr">

<mihbasic:ie_type_identifier>0x10000200</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#POA"/>

<rdfs:range rdf:resource="#TRANSPORT_ADDR"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="ie_poa_location">

<mihbasic:ie_type_identifier>0x10000201</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#POA"/>

<rdfs:range rdf:resource="#LOCATION"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="LOCATION">

</owl:Class>

<owl:Class rdf:ID="BIN_GEO_LOC">

<rdfs:subClassOf rdf:resource="#LOCATION"/>

<rdfs:comment>

This class has properties that represent geographic coordinate.

The format is based on the Location Configuration Information (LCI)

defined in RFC 3825.

</rdfs:comment>

</owl:Class>

<owl:DatatypeProperty rdf:ID="la_res">

<rdfs:domain rdf:resource="#BIN_GEO_LOC"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="latitude">

<rdfs:domain rdf:resource="#BIN_GEO_LOC"/>

<rdfs:range rdf:resource="&xsd;double"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="lo_res">

<rdfs:domain rdf:resource="#BIN_GEO_LOC"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="longitude">

<rdfs:domain rdf:resource="#BIN_GEO_LOC"/>

<rdfs:range rdf:resource="&xsd;double"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="altitude_type">

<rdfs:domain rdf:resource="#BIN_GEO_LOC"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

Following codes are defined:

1: Meters: in 2s-complement fixed-point 22-bit integer part with

8-bit fraction. If AT = 1, an AltRes value 0.0 would indicate

unknown altitude. The most precise Altitude would have an AltRes

value of 30. Many values of AltRes would obscure any variation

due to vertical datum differences.

2: Floors: in 2s-complement fixed-point 22-bit integer part with

8-bit fraction. AT = 2 for Floors enables representing altitude in

a form more relevant in buildings which have different

floor-to-floor dimensions.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="alt_res">

<rdfs:domain rdf:resource="#BIN_GEO_LOC"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

Altitude resolution. 6 bits indicating the number of valid bits

in the altitude. Values above 30 (decimal) are undefined and

reserved.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="altitude">

<rdfs:domain rdf:resource="#BIN_GEO_LOC"/>

<rdfs:range rdf:resource="&xsd;double"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="datum">

<rdfs:domain rdf:resource="#BIN_GEO_LOC"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

Following codes are defined:

1: WGS

2: NAD 83 (with associated vertical datum for North American

vertical datum for 1998)

3: NAD 83 (with associated vertical datum for Mean Lower Low Water

(MLLW))

</rdfs:comment>

</owl:DatatypeProperty>

<owl:Class rdf:ID="XML_GEO_LOC">

<rdfs:subClassOf rdf:resource="#LOCATION"/>

</owl:Class>

<owl:DatatypeProperty rdf:ID="xml_geo_location">

<rdfs:domain rdf:resource="#XML_GEO_LOC"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

Geo address elements as described in RFC4119.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:Class rdf:ID="BIN_CIVIC_LOC">

<rdfs:subClassOf rdf:resource="#LOCATION"/>

<rdfs:comment>

This class has properties that represent civic address.

The format is defined in IETF RFC 4676.

</rdfs:comment>

</owl:Class>

<owl:DatatypeProperty rdf:ID="civic_country_code">

<rdfs:domain rdf:resource="#BIN_CIVIC_LOC"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

Two-letter ISO 3166 country code in capital ASCII letters.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="civic_addr_elem">

<rdfs:domain rdf:resource="#BIN_CIVIC_LOC"/>

<rdfs:range rdf:resource="#CIVIC_ADDR"/>

<rdfs:comment>

This property contains the civic address elements.

The format of the civic address elements is described

in Section 3.4 of IETF RFC 4676 with a TLV pair

(whereby the Type and Length fields are one octet long).

</rdfs:comment>

</owl:ObjectProperty>

<owl:Class rdf:ID="CIVIC_ADDR">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#catype"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#cavalue"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="catype">

<rdfs:domain rdf:resource="#CIVIC_ADDR"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

A one-octet descriptor of the data civic address value.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="cavalue">

<rdfs:domain rdf:resource="#CIVIC_ADDR"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

The civic address value.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:Class rdf:ID="XML_CIVIC_LOC">

<rdfs:subClassOf rdf:resource="#LOCATION"/>

</owl:Class>

<owl:DatatypeProperty rdf:ID="xml_civic_location">

<rdfs:domain rdf:resource="#XML_CIVIC_LOC"/>

<rdfs:range rdf:resource="&xsd;string"/>

<rdfs:comment>

Geo address elements as described in RFC4119.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:Class rdf:ID="LOCATION_CELL_ID">

<rdfs:subClassOf rdf:resource="#LOCATION"/>

</owl:Class>

<owl:DatatypeProperty rdf:ID="location_cell_id">

<rdfs:domain rdf:resource="#LOCATION_CELL_ID"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="ie_poa_channel_range">

<mihbasic:ie_type_identifier>0x10000202</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#POA"/>

<rdfs:range rdf:resource="#CH_RANGE"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="CH_RANGE">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#low_channel_range"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#high_channel_range"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="low_channel_range">

<rdfs:domain rdf:resource="#CH_RANGE"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

<rdfs:comment>

Lowest channel frequency in MHz

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="high_channel_range">

<rdfs:domain rdf:resource="#CH_RANGE"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

<rdfs:comment>

Highest channel frequency in MHz

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="ie_poa_system_info">

<mihbasic:ie_type_identifier>0x10000203</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#POA"/>

<rdfs:range rdf:resource="#SYSTEM_INFO"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="SYSTEM_INFO">

</owl:Class>

<owl:ObjectProperty rdf:ID="system_info_network_type">

<rdfs:domain rdf:resource="#SYSTEM_INFO"/>

<rdfs:range rdf:resource="#NETWORK_TYPE"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="system_info_mac_address">

<rdfs:domain rdf:resource="#SYSTEM_INFO"/>

<rdfs:range rdf:resource="#TARNSPORT_ADDR"/>

</owl:ObjectProperty>

<owl:ObjectProperty rdf:ID="system_info_parameters">

<rdfs:domain rdf:resource="#SYSTEM_INFO"/>

<rdfs:range rdf:resource="#PARAMETERS"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="PARAMETERS">

</owl:Class>

<owl:Class rdf:ID="DCD_UCD">

<rdfs:subClassOf rdf:resource="#PARAMETERS"/>

</owl:Class>

<owl:DatatypeProperty rdf:ID="bs_id">

<rdfs:domain rdf:resource="#DCD_UCD"/>

<rdfs:range rdf:resource="#TRANSPORT_ADDR"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="bandwidth">

<rdfs:domain rdf:resource="#DCD_UCD"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="dcd_ucd_frequency">

<rdfs:domain rdf:resource="#DCD_UCD"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="eirp">

<rdfs:domain rdf:resource="#DCD_UCD"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="ttg">

<rdfs:domain rdf:resource="#DCD_UCD"/>

<rdfs:range rdf:resource="&xsd;unsignedShort"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="rtg">

<rdfs:domain rdf:resource="#DCD_UCD"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="down_burst_profile">

<rdfs:subPropertyOf rdf:resource="#bit_number"/>

<rdfs:domain rdf:resource="#DCD_UCD"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="up_burst_profile">

<rdfs:subPropertyOf rdf:resource="#bit_number"/>

<rdfs:domain rdf:resource="#DCD_UCD"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="initial_code">

<rdfs:domain rdf:resource="#DCD_UCD"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="handover_code">

<rdfs:domain rdf:resource="#DCD_UCD"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:Class rdf:ID="SIB">

<rdfs:subClassOf rdf:resource="#PARAMETERS"/>

</owl:Class>

<owl:DatatypeProperty rdf:ID="cell_id">

<rdfs:domain rdf:resource="#SIB"/>

<rdfs:range rdf:resource="&xsd;unsignedInt"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="frequency_code_number">

<rdfs:domain rdf:resource="#SIB"/>

<rdfs:range rdf:resource="&xsd;unsignedShort"/>

</owl:DatatypeProperty>

<owl:Class rdf:ID="SYS_PARAMS">

<rdfs:subClassOf rdf:resource="#PARAMETERS"/>

</owl:Class>

<owl:DatatypeProperty rdf:ID="base_id">

<rdfs:domain rdf:resource="#SYS_PARAMS"/>

<rdfs:range rdf:resource="&xsd;unsignedShort"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="pilot_pn">

<rdfs:domain rdf:resource="#SYS_PARAMS"/>

<rdfs:range rdf:resource="&xsd;unsignedShort"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="frequency">

<rdfs:domain rdf:resource="#SYS_PARAMS"/>

<rdfs:range rdf:resource="&xsd;unsignedShort"/>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="band_class">

<rdfs:domain rdf:resource="#SYS_PARAMS"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="ie_poa_subnet_info">

<mihbasic:ie_type_identifier>0x10000204</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#POA"/>

<rdfs:range rdf:resource="#IP_SUBNET_INFO"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="IP_SUBNET_INFO">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#subnet_address"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#ip_prefix_len"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:ObjectProperty rdf:ID="subnet_address">

<rdfs:domain rdf:resource="#IP_SUBNET_INFO"/>

<rdfs:range rdf:resource="#TRANSPORT_ADDR"/>

<rdfs:comment>

An IP address of the PoA encoded as Address base type defined in

RFC3588. The first 2-octet contains AddressType, which can be

either 1 (IPv4) or 2 (IPv6). If AddressType==1, the subnet_address

property contains a 4-octet IPv4 address. If AddressType==2, the

subnet_address property contains a 16-octet IPv6 address.

</rdfs:comment>

</owl:ObjectProperty>

<owl:DatatypeProperty rdf:ID="ip_prefix_len">

<rdfs:domain rdf:resource="#IP_SUBNET_INFO"/>

<rdfs:range rdf:resource="&xsd;unsignedByte"/>

<rdfs:comment>

The bit length of the prefix of the subnet to which subnet_address

property belongs. The prefix_length is less than or equal to 32

for IPv4 subnet and less than or equal to 128 for IPv6 subnet.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:ObjectProperty rdf:ID="ie_poa_ip_addr">

<mihbasic:ie_type_identifier>0x10000205</mihbasic:ie_type_identifier>

<rdfs:domain rdf:resource="#POA"/>

<rdfs:range rdf:resource="#TRANSPORT_ADDR"/>

</owl:ObjectProperty>

<owl:Class rdf:ID="TRANSPORT_ADDR">

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#address_type"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

<rdfs:subClassOf>

<owl:Restriction>

<owl:onProperty rdf:resource="#address_value"/>

<owl:cardinality rdf:datatype="&xsd;nonNegativeInteger">1

</owl:cardinality>

</owl:Restriction>

</rdfs:subClassOf>

</owl:Class>

<owl:DatatypeProperty rdf:ID="address_type">

<rdfs:domain rdf:resource="#TRANSPORT_ADDR"/>

<rdfs:range rdf:resource="&xsd;unsignedShort"/>

<rdfs:comment>

An Address Family defined in

http://www.iana.org/assignments/address-family-numbers.

</rdfs:comment>

</owl:DatatypeProperty>

<owl:DatatypeProperty rdf:ID="address_value">

<rdfs:domain rdf:resource="#TRANSPORT_ADDR"/>

<rdfs:range rdf:resource="&xsd;hexBinary"/>

<rdfs:comment>

An address value specific to address_type.

</rdfs:comment>

</owl:DatatypeProperty>

</rdf:RDF>

- IEEE 802.21 MIB

- Parameters requiring MIB definition

In this specification, only parameters that govern the operation of 802.21 are defined.

- MIH capability parameters

Following is a list of parameters that are used in the MIH capability discovery:

- Link MACs (Read-only): A list of network type and MAC address pairs that are controlled by this MIHF. Note that not all interfaces of an MIH-capable node can be under control of MIHF.

- MIH Event List (Read-only): A list of supported events by this MIHF.

- MIH Command List (Read-only): A list of supported commands by this MIHF.

- MIH IS Query List (Read-only): A list of supported MIH IS query types by this MIHF.

- MIH Transport List (Read-only): A list of supported MIH transport protocols by this MIHF. This is the transport that transmits the MIH protocol messages.

- List of MBB Handover support (Read-only): A list of make-before-break support information for each pair of serving and target network types.

- MIH protocol parameters

Following is a list of parameters that are used in the MIH protocol.

- Local MIHF ID (Read-Only)

- List of Peer MIHF IDs (Read-Write)

- Transport Type (Read-Write)

o L2 or L3

o Defined for each peer MIHF

- Source Transport Address (Read-Write)

o Link/layer/IPv4/IPv6 address

- Destination Transport Address (Read-Write)

o Link/layer/IPv4/IPv6 address

- Version (Read-only)

- Maximum Transaction Lifetime (Read-Write)

o Unit: seconds

- Maximum Retransmission Interval (Read-Write)

o Unit: seconds

- Maximum Retransmission Counter (Read-Write)

o Unit: none

- ACK Delay Time List (Read-Write):

o A list of (Message ID, ACK Delay Time) pairs

o Unit for Message ID: none

o Unit for ACK Delay Time : seconds

o A value of 0 indicates that ACK is not delayed

- Default ACK Delay Time (Read-Write)

o Used for Message IDs that are not specified in ACK Delay Time List

o Unit: seconds

o A value of 0 indicates that ACK is not delayed

- Maximum Average Transmission Rate (Read-Write)

o Unit: octets per second

- Maximum Burst Size (Read-Write)

o Unit: octets

- IEEE 802.21 MIB definition

IEEE802dot21-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE, NOTIFICATION-TYPE,

Integer32, Counter32, Unsigned32, IpAddress

FROM SNMPv2-SMI

TEXTUAL-CONVENTION, PhysAddress, MacAddress, DisplayString, TruthValue

FROM SNMPv2-TC

Ipv6Address

FROM IPV6-TC

ifIndex, ifEntry

FROM RFC1213-MIB

MODULE-COMPLIANCE, OBJECT-GROUP, NOTIFICATION-GROUP

FROM SNMPv2-CONF

InterfaceIndex

FROM IF-MIB;

-- **********************************************************************

-- * MODULE IDENTITY

-- **********************************************************************

ieee802dot21 MODULE-IDENTITY

LAST-UPDATED "0801150000Z"

ORGANIZATION "IEEE 802.21"

CONTACT-INFO

"WG E-mail: stds-802-21@ieee.org

Chair: Vivek G. Gupta

Intel Corporation

E-mail: mailto:vivek.g.gupta@intel.com

Editor: Qiaobing Xie

E-mail: Qiaobing.Xie@MOTOROLA.COM"

DESCRIPTION

"The MIB module for IEEE 802.21 entities.

iso(1).member-body(2).us(840).ieee802dot21(xxxxx)"

::= { us xxxxx }

-- TODO: Assign OBJECT IDENTIFIER for ieee802dot21

member-body OBJECT IDENTIFIER ::= { iso 2 }

us OBJECT IDENTIFIER ::= { member-body 840 }

--

-- Textual Conventions

--

Dot21MihfID ::= OCTET STRING (SIZE(0..255))

Dot21LinkType ::= Integer32

Dot21NetworkRevision ::= OCTET STRING (SIZE(0..8))

Dot21NetworkTypeExtension ::= OCTET STRING (SIZE(0..253))

Dot21EventList ::= BITS

{ mih-link-up(0),

mih-link-down(1),

mih-link-going-down(2),

mih-link-detected(3),

mih-link-parameters-report(4),

mih-link-event-rollback(5),

mih-link-handover-imminent(6),

mih-link-handover-complete(7) }

Dot21CommandList ::= BITS

{ mih-get-link-parameters(0),

mih-link-configure-thresholds(1),

mih-link-actions(2),

mih-net-ho-candidate-query(3),

mih-net-ho-commit(4),

mih-n2n-ho-query-resources(5),

mih-n2n-ho-commit(6),

mih-n2n-ho-complete(7),

mih-mn-ho-candiate-query(8),

mih-mn-ho-commit(9),

mih-mn-ho-complete(10) }

Dot21ISQueryTypeList ::= BITS

{ binary(0),

rdf-data(1),

rdf-schema-url(2),

rdf-schema(3),

type-ie-network-type(4),

type-ie-operator-identifier(5),

type-ie-service-provider-identifier(6),

type-ie-country-code(7),

type-ie-access-network-identifier(8),

type-ie-roaming-partners(9),

type-ie-cost(10),

type-ie-network-security(11),

type-ie-network-qos(12),

type-ie-network-data-rate(13),

type-ie-network-channel-range(14),

type-ie-network-ip-config-methods(15),

type-ie-network-capabilities(16),

type-ie-list-supported-lcp(17),

type-ie-poa-mac-address(18),

type-ie-poa-location(19),

type-ie-poa-channel-range(20),

type-ie-poa-system-information(21),

type-ie-poa-subnet-information(22),

type-ie-poa-ip-address(23) }

Dot21TransportList ::= BITS { udp(0), tcp(1) }

-- **********************************************************************

-- * Major sections

-- **********************************************************************

-- MIH Function Management (MIHMT) Attributes

-- DEFINED AS "The MIHMT object class provides the necessary support

-- at the MIHF to manage the processes in the station such that

-- the MIHF can work cooperatively as a part of an IEEE 802.21

-- network."

dot21mihmt OBJECT IDENTIFIER ::= { ieee802dot21 1 }

-- dot21mihmt GROUPS

-- dot21LocalMihfTable ::= { dot21mihmt 1 }

-- dot21PeerMihfTable ::= { dot21mihmt 2 }

-- dot21IfTable ::= { dot21mihmt 3 }

-- dot21MbbHandoverSupportTable ::= { dot21mihmt 4 }

-- dot21AckDelayTable ::= { dot21mihmt 5 }

-- **********************************************************************

-- * MIB attribute OBJECT-TYPE definitions follow

-- **********************************************************************

--

-- Local MIHF Table

--

dot21LocalMihfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF Dot21LocalMihfEntry

MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"The table of local MIHFs. The MIH MIB allows to have more than one local MIHFs per SNMP engine."

::={ dot21mihmt 1 }

dot21LocalMihfEntry			OBJECT-TYPE

SYNTAX	Dot21LocalMihfEntry

MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"The value contains information associated with a particular local MIHF. In most cases, there will be only one local MIHF on a node."

INDEX	{ dot21LocalMihfID }

::={ dot21LocalMihfTable 1}

Dot21LocalMihfEntry ::=

SEQUENCE{

dot21LocalMihfIndex INTEGER,

dot21LocalMihfID Dot21MihfID,

dot21LocalEventList BITS,

dot21LocalCommandList BITS,

dot21LocalISQueryTypeList BITS,

dot21LocalTransportList BITS,

dot21LocalVersion INTEGER,

dot21LocalMaximumTransactionLifetime INTEGER,

dot21LocalMaximumRetransmissionInterval INTEGER,

dot21LocalMaximumRetransmissionCounter INTEGER,

dot21LocalMaximumAverageTransmissionRate INTEGER,

dot21LocalMaximumBurstSize INTEGER

}

dot21LocalMihfIndex OBJECT-TYPE

SYNTAX	INTEGER(0..2147483647)

MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"Index of local MIHF table."

::= { dot21LocalMihfEntry 1 }

dot21LocalMihfID OBJECT-TYPE

SYNTAX Dot21MihfID

MAX-ACCESS read-write

STATUS	current

DESCRIPTION

"The MIHF ID of this node. The default value is null (i.e., the same as

broadcast MIHFID)"

REFERENCE "IEEE Std 802.21, 2008 Edition, X.X.X"

DEFVAL { ''H }

::={ dot21LocalMihfEntry 2 }

dot21LocalEventList OBJECT-TYPE

SYNTAX	Dot21EventList

MAX-ACCESS read-only

STATUS	current

DESCRIPTION

" This attribute will be a set of all the MIH events supported by this MIH node."

REFERENCE "IEEE Std 802.21, 2008 Edition, X.X.X"

DEFVAL { {} }

::={ dot21LocalMihfEntry 3 }

dot21LocalCommandList OBJECT-TYPE

SYNTAX	Dot21CommandList

MAX-ACCESS read-only

STATUS	current

DESCRIPTION

" This attribute will be a set of all the MIH commands supported by this MIH node."

REFERENCE "IEEE Std 802.21, 2008 Edition, X.X.X"

DEFVAL { {} }

::={ dot21LocalMihfEntry 4 }

dot21LocalISQueryTypeList OBJECT-TYPE

SYNTAX	Dot21ISQueryTypeList

MAX-ACCESS read-only

STATUS	current

DESCRIPTION

" This attribute will be a set of all the MIH IS query types supported by this MIH node."

REFERENCE "IEEE Std 802.21, 2008 Edition, X.X.X"

DEFVAL { {} }

::={ dot21LocalMihfEntry 5 }

dot21LocalTransportList OBJECT-TYPE

SYNTAX	Dot21TransportList

MAX-ACCESS		read-only

STATUS	current

DESCRIPTION

" This attribute will be a set of all the MIH transports supported by this MIH node."

REFERENCE "IEEE Std 802.21, 2008 Edition, X.X.X"

DEFVAL { {} }

::={ dot21LocalMihfEntry 6 }

dot21LocalVersion OBJECT-TYPE

SYNTAX	INTEGER (1..255)

MAX-ACCESS		read-only

STATUS	current

DESCRIPTION

"The MIH protocol version supported by this MIHF."

DEFVAL { 1 }

::={ dot21LocalMihfEntry 7 }

dot21LocalMaximumTransactionLifetime OBJECT-TYPE

SYNTAX	INTEGER (1..255)

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"The maximum time in seconds for a MIH protocol transaction."

DEFVAL { 30 }

::={ dot21LocalMihfEntry 8 }

dot21LocalMaximumRetransmissionInterval OBJECT-TYPE

SYNTAX	INTEGER (1..10000)

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"The maximum time in milliseconds for retransmitting a MIH message."

DEFVAL { 10000 }

::={ dot21LocalMihfEntry 9 }

dot21LocalMaximumRetransmissionCounter OBJECT-TYPE

SYNTAX	INTEGER (1..255)

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"The maximum amount of retransmission retries for MIH messages."

DEFVAL { 2 }

::={ dot21LocalMihfEntry 10 }

dot21LocalMaximumAverageTransmissionRate OBJECT-TYPE

SYNTAX	INTEGER (0..255)

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"The maximum amount MIH messages can be transmitted per second on this node. If the value is 0, no limitation is set."

DEFVAL { 0 }

::={ dot21LocalMihfEntry 11 }

dot21LocalMaximumBurstSize OBJECT-TYPE

SYNTAX	INTEGER (0..255)

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

" The maximum number of octets transmitted in a burst. If the value is 0, no limitation is set."

DEFVAL { 0 }

::={ dot21LocalMihfEntry 12 }

--

-- The Peer MIHF Table

--

dot21PeerMihfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF Dot21PeerMihfEntry

MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"The table of MIHF known by this MIHF."

::={ dot21mihmt 2 }

dot21PeerMihfEntry			OBJECT-TYPE

SYNTAX	Dot21PeerMihfEntry

MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"Details of a specific MIHF peer."

INDEX	{dot21PeerMihfIndex}

::= { dot21PeerMihfTable 1 }

Dot21PeerMihfEntry ::=

SEQUENCE {

dot21PeerMihfIndex INTEGER,

dot21PeerMihfID		Dot21MihfID,

dot21PeerLocalMihfID Dot21MihfID,

dot21PeerEventList BITS,

dot21PeerCommandList BITS,

dot21PeerISQueryTypeList BITS,

dot21PeerTransportList BITS,

dot21PeerTransportType			INTEGER,

dot21PeerSrcLinkAddress			MacAddress,

dot21PeerDstLinkAddress			MacAddress,

dot21PeerSrcIpv4Address			IpAddress,

dot21PeerDstIpv4Address			IpAddress,

dot21PeerSrcIpv6Address			Ipv6Address,

dot21PeerDstIpv6Address			Ipv6Address,

dot21PeerVersion INTEGER,

dot21PeerMaximumTransactionLifetime INTEGER,

dot21PeerMaximumRetransmissionInterval INTEGER,

dot21PeerMaximumRetransmissionCounter INTEGER,

dot21PeerMaximumAverageTransmissionRate INTEGER,

dot21PeerMaximumBurstSize INTEGER

}

dot21PeerMihfIndex OBJECT-TYPE

SYNTAX	INTEGER(0..2147483647)

MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"Index of peer MIHF table."

::= { dot21PeerMihfEntry 1 }

dot21PeerMihfID OBJECT-TYPE

SYNTAX	Dot21MihfID

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"The MIHF ID of a known peer node. The value must be non-null."

REFERENCE "IEEE Std 802.21, 2008 Edition, X.X.X"

::={ dot21PeerMihfEntry 2 }

dot21PeerLocalMihfID OBJECT-TYPE

SYNTAX	Dot21MihfID

MAX-ACCESS		read-only

STATUS	current

DESCRIPTION

"This attribute contains an MIHF ID of local node."

DEFVAL { ''H }

::={ dot21PeerMihfEntry 3 }

dot21PeerEventList OBJECT-TYPE

SYNTAX	Dot21EventList

MAX-ACCESS read-only

STATUS	current

DESCRIPTION

" This attribute will be a set of all the MIH events supported by peer MIH node."

REFERENCE "IEEE Std 802.21, 2008 Edition, X.X.X"

DEFVAL { {} }

::={ dot21PeerMihfEntry 4 }

dot21PeerCommandList OBJECT-TYPE

SYNTAX	Dot21CommandList

MAX-ACCESS read-only

STATUS	current

DESCRIPTION

" This attribute will be a set of all the MIH commands supported by peer MIH node."

REFERENCE "IEEE Std 802.21, 2008 Edition, X.X.X"

DEFVAL { {} }

::={ dot21PeerMihfEntry 5 }

dot21PeerISQueryTypeList OBJECT-TYPE

SYNTAX	Dot21ISQueryTypeList

MAX-ACCESS read-only

STATUS	current

DESCRIPTION

" This attribute will be a set of all the MIH IS query types supported by peer

MIH node."

REFERENCE "IEEE Std 802.21, 2008 Edition, X.X.X"

DEFVAL { {} }

::={ dot21PeerMihfEntry 6 }

dot21PeerTransportList OBJECT-TYPE

SYNTAX	Dot21TransportList

MAX-ACCESS		read-only

STATUS	current

DESCRIPTION

" This attribute will be a set of all the MIH transports supported by peer MIH

node."

REFERENCE "IEEE Std 802.21, 2008 Edition, X.X.X"

DEFVAL { {} }

::={ dot21PeerMihfEntry 7 }

dot21PeerTransportType OBJECT-TYPE

SYNTAX	INTEGER { layer2(1), layer3-ipv4(2), layer3-ipv6(3) }

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"This value should be set for the MIH protocol layer used for transmitting MIH messages."

DEFVAL { 1 }

::= {dot21PeerMihfEntry 8 }

dot21PeerSrcLinkAddress OBJECT-TYPE

SYNTAX	MacAddress

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"This value indicates the source link-layer address used for transmitting MIH messages using L2 transport."

::= {dot21PeerMihfEntry 9 }

dot21PeerDstLinkAddress OBJECT-TYPE

SYNTAX	MacAddress

MAX-ACCESS		read-only

STATUS	current

DESCRIPTION

"This value indicates the destination link-layer address used for transmitting

MIH messages using L2 transport."

::= {dot21PeerMihfEntry 10 }

dot21PeerSrcIpv4Address OBJECT-TYPE

SYNTAX	IpAddress

MAX-ACCESS		read-only

STATUS	current

DESCRIPTION

"This value indicates the source IPv4 address used for transmitting MIH

messages using IPv4 transport."

::= {dot21PeerMihfEntry 11 }

dot21PeerDstIpv4Address OBJECT-TYPE

SYNTAX	IpAddress

MAX-ACCESS		read-only

STATUS	current

DESCRIPTION

"This value indicates the destination IPv4 address used for transmitting MIH messages using IPv4 transport."

::= {dot21PeerMihfEntry 12 }

dot21PeerSrcIpv6Address OBJECT-TYPE

SYNTAX	Ipv6Address

MAX-ACCESS		read-only

STATUS	current

DESCRIPTION

"This value indicates the source IPv6 address used for transmitting MIH

messages using IPv6 transport."

::= {dot21PeerMihfEntry 13 }

dot21PeerDstIpv6Address OBJECT-TYPE

SYNTAX	Ipv6Address

MAX-ACCESS		read-only

STATUS	current

DESCRIPTION

"This value indicates the destination IPv6 address used for transmitting MIH

messages using IPv6 transport."

::= {dot21PeerMihfEntry 14 }

dot21PeerVersion OBJECT-TYPE

SYNTAX	INTEGER (1..255)

MAX-ACCESS		read-only

STATUS	current

DESCRIPTION

"The MIH protocol version supported by peer MIHF. The default version is 1."

DEFVAL { 1 }

::={ dot21PeerMihfEntry 15 }

dot21PeerMaximumTransactionLifetime OBJECT-TYPE

SYNTAX	INTEGER (1..255)

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"The maximum time in seconds for a MIH protocol transaction used for a  particular peer MIHF."

DEFVAL { 30 }

::={ dot21PeerMihfEntry 16 }

dot21PeerMaximumRetransmissionInterval OBJECT-TYPE

SYNTAX	INTEGER (1..10000)

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"The maximum time in milliseconds for retransmitting a MIH message used for a particular peer MIHF."

DEFVAL { 10000 }

::={ dot21PeerMihfEntry 17 }

dot21PeerMaximumRetransmissionCounter OBJECT-TYPE

SYNTAX	INTEGER (1..255)

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"The maximum amount of retransmission retries for MIH messages used for a particular peer MIHF."

DEFVAL { 2 }

::={ dot21PeerMihfEntry 18 }

dot21PeerMaximumAverageTransmissionRate OBJECT-TYPE

SYNTAX	INTEGER (0..255)

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"The maximum amount MIH messages can be transmitted per second on this node for a particular peer MIHF. If the value is 0, no limitation is set."

DEFVAL { 0 }

::={ dot21PeerMihfEntry 19 }

dot21PeerMaximumBurstSize OBJECT-TYPE

SYNTAX	INTEGER (0..255)

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"The maximum number of octets transmitted in a burst. If the value is 0, no limitation is set."

DEFVAL { 0 }

::={ dot21PeerMihfEntry 20 }

--

-- MIH Interface Table

--

dot21IfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF Dot21IfEntry

MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"The table of Dot21IfEntry."

::={ dot21mihmt 3 }

dot21IfEntry		OBJECT-TYPE

SYNTAX	Dot21IfEntry

MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"The value contains information associated with a particular link controlled by

this MIHF."

AUGMENTS { ifEntry }

::={ dot21IfTable 1}

Dot21IfEntry ::=

SEQUENCE{

dot21MihSupported TruthValue

}

dot21MihSupported OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represends whether a particular interface supports MIH."

::= { dot21IfEntry 1 }

--

-- The Make-Before-Break Handover Support Table

--

dot21MbbHandoverSupportTable OBJECT-TYPE

SYNTAX	SEQUENCE OF Dot21MbbHandoverSupportEntry

MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"The table of make-before-break handover support entries."

::={ dot21mihmt 4 }

dot21MbbHandoverSupportEntry				OBJECT-TYPE

SYNTAX	Dot21MbbHandoverSupportEntry

MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"The value contains information associated with a particular MBB support."

INDEX	{ dot21MbbHandoverSupportIndex }

::={ dot21MbbHandoverSupportTable 1 }

Dot21MbbHandoverSupportEntry ::=

SEQUENCE{

dot21MbbHandoverSupportIndex INTEGER,

dot21FromLinkType Dot21LinkType,

dot21FromNetworkRevision Dot21NetworkRevision,

dot21FromNetworkTypeExtension Dot21NetworkTypeExtension,

dot21ToLinkType Dot21LinkType,

dot21ToNetworkRevision Dot21NetworkRevision,

dot21ToNetworkTypeExtension Dot21NetworkTypeExtension,

dot21IsMbbSupported TruthValue

}

dot21MbbHandoverSupportIndex OBJECT-TYPE

SYNTAX	INTEGER(0..2147483647)

MAX-ACCESS		read-only

STATUS	current

DESCRIPTION

"Index of make-before-break handover support table."

::= { dot21MbbHandoverSupportEntry 1 }

dot21FromLinkType OBJECT-TYPE

SYNTAX Dot21LinkType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents the link type of serving link."

DEFVAL { 0 }

::={ dot21MbbHandoverSupportEntry 2 }

dot21FromNetworkRevision OBJECT-TYPE

SYNTAX Dot21NetworkRevision

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents the network revision of serving link."

DEFVAL { ''H }

::={ dot21MbbHandoverSupportEntry 3 }

dot21FromNetworkTypeExtension OBJECT-TYPE

SYNTAX Dot21NetworkTypeExtension

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents the network type extension of serving link."

DEFVAL { ''H }

::={ dot21MbbHandoverSupportEntry 4 }

dot21ToLinkType OBJECT-TYPE

SYNTAX Dot21LinkType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents the link type of target link."

DEFVAL { 0 }

::={ dot21MbbHandoverSupportEntry 5 }

dot21ToNetworkRevision OBJECT-TYPE

SYNTAX Dot21NetworkRevision

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents the network revision of target link."

DEFVAL { ''H }

::={ dot21MbbHandoverSupportEntry 6 }

dot21ToNetworkTypeExtension OBJECT-TYPE

SYNTAX Dot21NetworkTypeExtension

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents the network type extension of target link."

DEFVAL { ''H }

::={ dot21MbbHandoverSupportEntry 7 }

dot21IsMbbSupported OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates whether make-before-break handover is supported. A value of true indicates that make-before-break handover is supported. A value of false indicates that make-before-break handover is not supported."

::={ dot21MbbHandoverSupportEntry 8 }

--

-- ACK Message Delay Table

--

dot21AckDelayTable OBJECT-TYPE

SYNTAX	SEQUENCE OF Dot21AckDelayEntry

MAX-ACCESS not-accessible

STATUS	current

DESCRIPTION

"The table of the acknowledgement delay per MIHF ID and per MIH message ID."

::={ dot21mihmt 5 }

dot21AckDelayEntry OBJECT-TYPE

SYNTAX	Dot21AckDelayEntry

MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"An Entry that reflects the acknowledgement delay."

INDEX	{ dot21AckDelayIndex }

::={ dot21AckDelayTable 1 }

Dot21AckDelayEntry ::=

SEQUENCE {

dot21AckDelayIndex Integer32,

dot21AckPeerMihfID Dot21MihfID,

dot21AckMessageID INTEGER,

dot21AckDelayTime INTEGER

}

dot21AckDelayIndex OBJECT-TYPE

SYNTAX	INTEGER(0..2147483647)

MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"The auxiliary variable used to identify instances of the columnar objects in the Acknowledgement Delay Table."

::={ dot21AckDelayEntry 1 }

dot21AckPeerMihfID OBJECT-TYPE

SYNTAX Dot21MihfID

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute represends the MIHF IF of peer node for which ack delay

parameters are applied. If this value is null, the acknowledgment delay

parameters of this row applies to any peer that does not have an entry in

dot21AckDelayTable."

DEFVAL { ''H }

::={ dot21AckDelayEntry 2 }

dot21AckMessageID OBJECT-TYPE

SYNTAX	INTEGER (0..255)

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"The MIH message ID for determining the acknowledgement delay. If this value is zero, the ack delay parameters of this row applies to any message ID that does not have an entry in dot21AckDelayTable."

REFERENCE "IEEE Std 802.21, 2008 Edition, 8.X.X"

::={ dot21AckDelayEntry 3 }

dot21AckDelayTime OBJECT-TYPE

SYNTAX	INTEGER (0..10000)

MAX-ACCESS		read-write

STATUS	current

DESCRIPTION

"The delay time in milliseconds for sending out the acknowledgement packet. The default value is 0, which indicates that the acknowledgement should be sent immediately after receiving a message."

DEFVAL { 0 }

::={ dot21AckDelayEntry 4 }

END

- Protocol implementation conformance statement (PICS) proforma

- Introduction

The supplier of a protocol implementation that is claimed to conform to IEEE Std 802.21, shall complete the following protocol implementation conformance statement (PICS) proforma.

A completed PICS proforma is the PICS for the implementation in question. The PICS is a statement of which capabilities and options of the protocol have been implemented. The PICS can have a number of uses, including use:

- By the protocol implementer, as a checklist to reduce the risk of failure to conform to the standard through oversight;

- By the supplier and acquirer, or potential acquirer, of the implementation, as a detailed indication of the capabilities of the implementation, stated relative to the common basis for understanding provided by the standard PICS proforma;

- By the user, or potential user, of the implementation, as a basis for initially checking the possibility of interworking with another implementation (note that, while interworking can never be guaranteed, failure to interwork can often be predicted from incompatible PICS proformas);

- By a protocol tester, as the basis for selecting appropriate tests against which to assess the claim for conformance of the implementation.

- Abbreviations and special symbols

- Symbols for status column

M 	mandatory

O 	optional

O.<n> 	optional, but support of at least one of the group of options labeled by the same numeral <n> is required

pred: 	conditional symbol, including predicate identification

- General abbreviations for item and support columns

N/A 	not applicable

- Instructions for completing the PICS proforma

- General structure of the PICS proforma

The first parts of the PICS proforma, Implementation identification and Protocol summary, are to be completed as indicated with the information necessary to identify fully both the supplier and the implementation.

The main part of the PICS proforma is a fixed questionnaire, divided into sub-clauses, each containing a number of individual items. Answers to the questionnaire items are to be provided in the rightmost column, either by simply marking an answer to indicate a restricted choice (usually Yes or No) or by entering a value or a set or a range of values. (Note that there are some items where two or more choices from a set of possible answers can apply. All relevant choices are to be marked in these cases.)

Each item is identified by an item reference in the first column. The second column contains the question to be answered. The third column contains the reference or references to the material that specifies the item in the main body of this standard. The remaining columns record the status of each item, i.e., whether support is mandatory, optional, or conditional, and provide the space for the answers. Marking an item as supported is to be interpreted as a statement that all relevant requirements of the sub-clauses and normative annexes, cited in the References column for the item, are met by the implementation.

A supplier can also provide, or be required to provide, further information, categorized as either Additional Information or Exception Information. When present, each kind of further information is to be provided in a further sub-clause of items labeled A<I> or X<I>, respectively, for cross-referencing purposes, where <I> is any unambiguous identification for the item (e.g., simply a numeral). There are no other restrictions on its format or presentation.

A completed PICS proforma, including any Additional Information and Exception Information, is the PICS for the implementation in question.

NOTE—Where an implementation is capable of being configured in more than one way, a single PICS may be able to describe all such configurations. However, the supplier has the choice of providing more than one PICS, each covering some subset of the implementation’s capabilities, if this makes for easier and clearer presentation of the information.

- Additional information

Items of Additional Information allow a supplier to provide further information intended to assist in the interpretation of the PICS. It is not intended or expected that a large quantity of information will be supplied, and a PICS can be considered complete without any such information. Examples of such Additional Information might be an outline of the ways in which an (single) implementation can be set up to operate in a variety of environments and configurations, or information about aspects of the implementation that are outside the scope of this standard but have a bearing upon the answers to some items.

References to items of Additional Information may be entered next to any answer in the questionnaire, and can be included in items of Exception Information.

- Exception information

It can happen occasionally that a supplier will wish to answer an item with mandatory status (after any conditions have been applied) in a way that conflicts with the indicated requirement. No preprinted answer will be found in the Support column for this. Instead, the supplier shall write the missing answer into the Support column, together with an X<I> reference to an item of Exception Information, and will provide the appropriate rationale in the Exception Information item itself.

An implementation for which an Exception Information item is required in this way does not conform to this standard.

NOTE— A possible reason for the situation described above is that a defect in this standard has been reported, a correction for which is expected to change the requirement not met by the implementation.

- Conditional status

The PICS proforma contains a number of conditional items. These are items for which both the applicability of the item itself, and its status if it does apply, mandatory or optional, are dependent upon whether or not certain other items are supported.

Where a group of items is subject to the same condition for applicability, a separate preliminary question about the condition appears at the head of the group, with an instruction to skip to a later point in the questionnaire if the N/A answer is selected. Otherwise, individual conditional items are indicated by a conditional symbol in the Status column.

A conditional symbol is of the form “<pred>:<S>”, where “<pred>” is a predicate as described below, and “<S>” is one of the status symbols M or O.

If the value of the predicate is true, the conditional item is applicable, and its status is given by S: the support column is to be completed in the usual way. Otherwise, the conditional item is not relevant and the N/A answer is to be marked.

A predicate is one of the following:

- An item-reference for an item in the PICS proforma: the value of the predicate is true if the item is marked as supported, and is false otherwise.

- A boolean expression constructed by combining item-references using the boolean operator OR: the value of the predicate is true if one or more of the items is marked as supported, and is false otherwise.

Each item referenced in a predicate, or in a preliminary question for grouped conditional items, is indicated by an asterisk in the Item column.

- PICS proforma - IEEE Std 802.21

- Implementation identification

| Supplier |  |
| --- | --- |
| Contact point for queries about the PICS |  |
| Implementation Name(s) and Version(s) |  |
| Order information necessary for full identification, e.g. name(s) and version(s) of the machines and/or operating systems(s), system names |  |

NOTES

1—Only the first three items are required for all implementations. Other information can be completed as appropriate in meeting the requirement for full identification.

2—The terms Name and Version should be interpreted appropriately to correspond with a supplier’s terminology (e.g., Type, Series, Model).

- Protocol summary

| Identification of Protocol Standard | IEEE Std 802.21 |
| --- | --- |
| Have any exception items been required? (See A.3.3; the answer Yes means that the implementation does not conform to IEEE Std 802.21.) |  |
| Date of statement (dd/mm/yyyy) |  |

- MIH Services configuration

| Item | MIH Services | References | Status | Support |
| --- | --- | --- | --- | --- |
| *MIH1 | What are the MIH Services supported? Event Service (ES) | 6.3 | O.1 | Yes o No o |
| *MIH2 | Command Service (CS) | 6.4 | O.1 | Yes o No o |
| *MIH3 | Information Service (IS) | 6.5 | O.1 | Yes o No o |
| MIH4 | Capability Discovery | 6.2.3 | M | Yes o No o |
| MIH5 | Registration Service | 6.2.4 | MIH2:M | Yes o No o |
| MIH6* | Mobile Initiated Handover | ????? | O.2 | Yes o No o |
| MIH7* | Network Initiated Handover | ????? | O.2 | Yes o No o |

- Event Services (ES)

| Item | ES Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| ES1 | Event subscription | 6.3.2 | MIH1:M | Yes o No o |
| ES2 | Link_Detected | 7.3.1 | MIH1:M | Yes o No o |
| ES3 | Link_Up | 7.3.2 | MIH1:M | Yes o No o |
| ES4 | Link_Down | 7.3.3 | MIH1:M | Yes o No o |
| ES5 | Link_Going_Down | 7.3.4 | MIH1:M | Yes o No o |
| ES6 | Link_Event_Rollback | 7.3.5 | ES5:M | Yes o No o |
| ES7 | Link_Parameters_Report | 7.3.6 | MIH1:M | Yes o No o |
| ES8 | Link_PDU_Transmit_Status | 7.3.7 | MIH1:O | Yes o No o |
| ES9 | Link_Handover_Imminent | 7.3.8 | MIH1:O | Yes o No o |
| ES10 | Link_Handover_Complete | 7.3.9 | MIH1:O | Yes o No o |
| ES11 | Link_Event_Subscribe | 7.3.10 | MIH1:M | Yes o No o |
| ES12 | Link_Event_Unsubscribe | 7.3.11 | MIH1:M | Yes o No o |
| ES13 | MIH_Link_Detected | 7.3.1, 8.6.2.4 | MIH1:M | Yes o No o |
| ES14 | MIH_Link_Up | 7.3.2, 8.6.2.1 | MIH1:M | Yes o No o |
| ES15 | MIH_Link_Down | 7.3.3, 8.6.2.2 | MIH1:M | Yes o No o |
| *ES16 | MIH_Link_Going_Down | 7.3.4, 8.6.2.3 | MIH1:M | Yes o No o |
| ES17 | MIH_Link_Event_Rollback | 7.3.5, 8.6.2.6 | ES16:M | Yes o No o |
| ES18 | MIH_Link_Parameters_Report | 7.3.6, 8.6.2.5 | MIH1:M | Yes o No o |
| ES19 | MIH_Link_PDU_Transmit_Status | 7.3.7 | MIH1:O | Yes o No o |
| ES20 | MIH_Link_Handover_Imminent | 7.3.8, 8.6.2.7 | MIH1:O | Yes o No o |
| ES21 | MIH_Link_Handover_Complete | 7.3.9, 8.6.2.8 | MIH1:O | Yes o No o |
| ES22 | MIH_Event_Subscribe request | 7.3.10, 8.6.1.7 | MIH1:M | Yes o No o |
| ES23 | MIH_Event_Subscribe response | 7.3.11, 8.6.1.8 | MIH1:M | Yes o No o |
| ES24 | MIH_Event_Unsubscribe request | 8.6.1.9 | MIH1:M | Yes o No o |
| ES25 | MIH_Event_Unsubscribe response | 8.6.1.10 | MIH1:M | Yes o No o |

- Command Services (CS)

| Item | CS Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| CS1 | Link_Configure_Thresholds | 7.3.12 | MIH2:M | Yes o No o |
| CS2 | Link_Get_Parameters | 7.3.14 | MIH2:M | Yes o No o |
| CS3 | Link_Action | 7.3.15 | MIH2:M | Yes o No o |
| CS4 | MIH_Link_Configure_Thresholds | 8.6.3.3, 8.6.3.4 | MIH2:M | Yes o No o |
| CS5 | MIH_Link_Get_Parameters | 8.6.3.1, 8.6.3.2 | MIH2:M | Yes o No o |
| CS6 | MIH_Link_Action | 8.6.3.5, 8.6.3.6 | MIH2:M | Yes o No o |
| CS7 | MIH_Net_HO_Candidate_Query | 7.4.18, 8.6.3.7, 8.6.3.8 | MIH7:M | Yes o No o |
| CS8 | MIH_Net_HO_Commit | 7.4.21, 8.6.3.13, 8.6.3.14 | MIH7:M | Yes o No o |
| CS9 | MIH_MN_HO_Candidate_Query | 7.4.19, 8.6.3.9, 8.6.3.10 | MIH6:M | Yes o No o |
| CS10 | MIH_MN_HO_Commit | 7.4.22, ??????? | MIH6:M | Yes o No o |
| CS11 | MIH_MN_HO_Complete | 7.4.24, 8.6.3.17, 8.6.3.18 | MIH6:M | Yes o No o |
| CS12 | MIH_N2N_HO_Query_Resources | 7.4.20, 8.6.3.11, 8.6.3.12 | MIH2:M | Yes o No o |
| CS13 | MIH_N2N_HO_Commit | 7.4.23, 8.6.3.15, 8.6.3.16 | MIH2:M | Yes o No o |
| CS14 | MIH_N2N_Complete | 7.4.25, 8.6.3.19, 8.6.3.20 | MIH2:M | Yes o No o |

- Information Services (IS)

| Item | IS Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| IS1 | TYPE_IE_NETWORK_TYPE | 6.5.4 | MIH3:M | Yes o No o |
| IS2 | TYPE_IE_OPERATOR_IDENTIFIER | 6.5.4 | MIH3:M | Yes o No o |
| IS3 | TYPE_IE_SERVICE_PROVIDER_IDENTIFIER | 6.5.4 | MIH3:M | Yes o No o |
| IS4 | TYPE_IE_COUNTRY_CODE | 6.5.4 | MIH3:M | Yes o No o |
| IS5 | TYPE_IE_ACCESS_NETWORK_IDENTIFIER | 6.5.4 | MIH3:M | Yes o No o |
| IS6 | TYPE_IE_ACCESS_NETWORK_AUX_ID | 6.5.4 | MIH3:M | Yes o No o |
| IS7 | TYPE_IE_ROAMING_PARTNERS | 6.5.4 | MIH3:M | Yes o No o |
| IS8 | TYPE_IE_COST | 6.5.4 | MIH3:O | Yes o No o |
| IS9 | TYPE_IE_NETWORK_SECURITY | 6.5.4 | MIH3:O | Yes o No o |
| IS10 | TYPE_IE_NETWORK_QOS | 6.5.4 | MIH3:O | Yes o No o |
| IS11 | TYPE_IE_NETWORK_DATA_RATE | 6.5.4 | MIH3:O | Yes o No o |
| IS12 | TYPE_IE_NETWORK_CHANNEL_RANGE | 6.5.4 | MIH3:O | Yes o No o |
| IS13 | TYPE_IE_NETWORK_IP_CONFIG_METHODS | 6.5.4 | MIH3:M | Yes o No o |
| IS14 | TYPE_IE_NETWORK_CAPABILITIES | 6.5.4 | MIH3:M | Yes o No o |
| IS15 | TYPE_IE_NETWORK_SUPPORTED_LCP | 6.5.4 | MIH3:O | Yes o No o |
| IS16 | TYPE_IE_NETWORK_MOBILITY_MGMT_PROT | 6.5.4 | MIH3:O | Yes o No o |
| IS17 | TYPE_IE_POA_MAC_ADDRESS | 6.5.4 | MIH3:O | Yes o No o |
| IS18 | TYPE_IE_POA_LOCATION | 6.5.4 | MIH3:O | Yes o No o |
| IS19 | TYPE_IE_POA_CHANNEL_RANGE | 6.5.4 | MIH3:O | Yes o No o |
| IS20 | TYPE_IE_POA_SYSTEM_INFORMATION | 6.5.4 | MIH3:O | Yes o No o |
| IS21 | TYPE_IE_POA_SUBNET_INFORMATION | 6.5.4 | MIH3:O | Yes o No o |
| IS22 | TYPE_IE_POA_IP_ADDRESS | 6.5.4 | MIH3:O | Yes o No o |
| IS23 | Vendor Specific IEs | 6.5.4 | MIH3:O | Yes o No o |
| IS24 | TLV Query Method | 6.5.6.2 | MIH3:O3 | Yes o No o |
| IS25 | RDF query Method | 6.5.6.3 | MIH3:O3 | Yes o No o |
| IS26 | MIH_Get_Information | 7.4.26, 8.6.4.1, 8.6.4.2 | MIH3:M | Yes o No o |

- MIH Protocol

| Item | MIH Protocol Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| MP1 | MIH Protocol Frame | 8.4 | M | Yes o No o |
| MP2 | MIH Protocol Acknowledgement | 8.2.1 | M | Yes o No o |
| MP3 | MIH Protocol State Machine | 8.2.2 | M | Yes o No o |
| MP4 | MIH Protocol Identifiers – MIHF ID | 8.3.1 | M | Yes o No o |
| MP5 | MIH Protocol Identifiers – Transaction ID | 8.3.2 | M | Yes o No o |