| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |
| --- | --- |
| Title | Resolution for Comment #2004 (Reliability definition) |
| Date Submitted | May 12, 2008 |
| Source(s) | Yoshihiro Ohba (Toshiba) |
| Re: | IEEE 802.21 Session #26 in May 2008 |
| Abstract | This document addresses SB recirc-2 Comment #2004 |
| Purpose | Sponsor Ballot comment resolution |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

# Comment #2004

Comment: “There is no definition of the level of reliability required for proper operation of the MIH protocol. The text says that there should be a "reliable" underlying transport or that the Acknowledgement service be used, but no specification for the level of reliability is given. How reliable must the underlying service be (i.e, what is the allowable undetected message loss rate)?”

Proposed Resolution by the commenter: “Specify a minimum reliability for the underlying transport that avoids the need for the Acknowledgement service.”

# Discussion

Literature “Handover in DVB-H”, pp 105-130, Springer Berlin Heidelberg,  ISSN 1860-4862, provides that a known acceptable handover failure probability for inter-technology handovers is 2%. We can set a smaller value than 2% as the acceptable message loss rate for MIH. The commenter suggested to choose 0.1% or 0.01 %. It may be good to choose smaller number to be on the safer side about reliability.

# Proposed Resolution

[1] Change Section 8.2.4.2 as follows:

8.2.4.2 Reliability

MIH protocol messages are delivered via media dependent transport. The MIH Function determines the transport reliability provided to the MIH user. However, the MIH user can provide its preference to the MIH Function. How the preference is communicated to the MIHF depends on the implementation and is outside the scope of this standard. If the MIHF uses unreliable transport and a reliable message delivery is required, the MIH protocol ACK operation is applied for transmitting the message. If the MIHF uses reliable transport, then the MIH protocol ACK operation should not be required for transmitting the message.

MIH protocol messages are delivered via media dependent transport. To ensure proper operation, a reliable message delivery service is required. If the media dependent transport is unreliable, then the Acknowledgement Service shall be enabled, as specified in Clause 8.2.2. If the media dependent transport is reliable, then the Acknowledgement Service may be implemented.

A reliable media dependent transport is one which exhibits a message loss rate of less than 0.01%,