| - Network type and revision representation |  |  |
| --- | --- | --- |
| Network | Link Type | Revision |
| (Reserved) | 0 | N/A |
| Wireless - GSM | 1 | N/A |
| Wireless - GPRS | 2 | N/A |
| Wireless - EDGE | 3 | N/A |
| (Reserved) | 4-14 | N/A |
| Ethernet | 15 | Bit 0: 10Mb Bit 1: 100Mb Bit 2: 1000Mb Bit 3-16: (Reserved) The above bits represent the link speeds that Ethernet supports. The capability information of twisted pair Ethernet link can be obtained via auto-negotiation as defined in Clause 28 of the 1998 edition of IEEE 802.3. |
| (Reserved) | 16-17 | N/A |
| Wireless - Other | 18 | N/A |
| Wireless - IEEE 802.11 | 19 | Bit 0: 2.4 GHz Bit 1: 5 GHz Bit 2-16 (Reserved) The above bits represent the frequency band that IEEE 802.11 link supports. The capability information of IEEE 802.11 link can further be represented as defined in Section 7.3.1.4 of IEEE 802.11-2007. |
| (Reserved) | 20-21 | N/A |
| Wireless - CDMA2000 | 22 | N/A |
| Wireless - UMTS | 23 | Bit 0: Rel-99 Bit 1: Rel-4 Bit 2: Rel-5 (w/ HSDPA) Bit 3: Rel-6 (w/ HSUPA) Bit 4: Rel-7 (MIMO/OFDM) Bit 5: Rel-8 Bit 6-63: (Reserved) |
| Wireless - cdma2000-HRPD | 24 | Bit 0: Rev-0 Bit 1: Rev-A Bit 2: Rev-B Bit 3: Rev-C Bit 4-63: (Reserved) |
| (Reserved) | 25-26 | N/A |
| Wireless - IEEE 802.16 | 27 | Bit 0: 2.5 GHz Bit 1: 3.5 GHz Bit 2-16: (Reserved) The above bits represent the frequency band that IEEE 802.16 link supports. The system profiles of IEEE 802.16 link can further be represented as defined in SubClause-12 (12.3, and 12.4) of IEEE 802.16-2007 |
| Wireless - IEEE 802.20 | 28 | N/A |
| Wireless - IEEE 802.22 | 29 | N/A |
| (Reserved) | 30-255 | N/A |

Note, the Link Type values in Network type and revision representation are deliberately made consistent with RADIUS network access server (NAS)-Port-Type definitions as specified by Internet Assigned Numbers Authority (IANA).