| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |
| --- | --- |
| Title | An MIH Fragmentation Mechanism |
| Date Submitted | May 23, 2008 |
| Source(s) | Yoshihiro Ohba (Toshiba), Subir Das (Telcordia), Yuu-Heng Alice Cheng (Telcordia), Vivek Gupta (Intel), Dennis Edwards (CoCo Communications), Anthony Chan (Huawei), Nada Golmie (NIST), Junghoon Jee (ETRI) |
| Re: | IEEE 802.21 WG discussion in May 2008 |
| Abstract | This document describes a fragmentation mechanism for the MIH protocol to addresses SB recirc-2 Comment #145. |
| Purpose | Sponsor Ballot comment resolution |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

# Introduction

If a new EtherType is defined to carry MIH protocol over 802.3, a major issue is that 802.3 does not support fragmentation while MIH message size can be larger than link-layer MTU for 802.3.  In general, for the MIH protocol to work over a transport that does not support fragmentation, a fragmentation mechanism is needed within the MIH protocol such that the MIH protocol can work regardless of the MTU size of the link layers.

This document proposes an MIH fragmentation mechanism to address this issue.

# Overview of the proposal

(This section is not part of proposed text.)

## Basic Requirements

- An MIH message is fragmented only when MIH message is sent natively over a L2 medium such as Ethernet. The message is fragmented when the message size exceeds the Maximum Transmission Unit (MTU) size of the link layer. When the MTU size of the link layer is unknown, the maximum MIH fragment size is set to the minimum MTU of 1500 octets. An MTU size may be known via the MIB of the link layer. When MIH message is sent using a L3 or higher layer transport, other layers take care of fragmentation issue and MIH Protocol does not handle fragmentation in such cases.

## Required changes to MIH message format

The following new fields are needed for the MIH protocol header.

- ‘M’ (More Fragment), a bit to indicate that the MIH message is a fragment to be followed by another fragment. This bit is set to ‘0’ for a message that is not fragmented and for the last fragment of a multi-fragment message.

- ‘FN’ (Fragment Number), a six-bit unsigned integer value to indicate the sequence number of a fragment in the original message. ‘FN’ starts from ‘0’ and increases by 1 for each of the corresponding fragment. ‘FN’ is also set to ‘0’ for a message that is not fragmented.

## No impact on MIH State Machines

The proposed fragmentation mechanism does not require any changes to the MIH state machines.  MIH State machine deals with protocol level transactions which are performed on a complete MIH message. Fragmentation and reassembly are outside the scope of MIH protocol level transactions and hence do not affect MIH state machine. Please note:

- Fragmentation is performed within ‘Transmit()’ procedure in the MIH state machine.

- Reassembly is performed before the MIH state machine comes into play for the received message.. ‘MsgIn’ and ‘MsgInAvail’ variables are set only after successful reassembly.

## Error handling

- If the number of fragments for the message exceeds 64, the   message is  discarded by the source MIHF without being transmitted.

- When the first fragment of a multi-fragment message arrives, the destination MIHF starts a timer. Note that the first received fragment may not be the first transmitted fragment. If this timer expires before all the fragments of the message have been received, the destination MIHF discards the fragments it has received. The source MIHF is then expected to retransmit the entire MIHF message when the MIH acknowledgment service is enabled.  Retransmissions of any single fragment of a multi-fragment message are not permitted.

# Proposed Text

[1] Change Figure 27 as follows:

MIH Protocol Frame

MIH Protocol Header

(8 octets)

MIH Protocol Payload

MIH Service Specific TLVs

Source

MIHF Identifier TLV

Destination MIHF Identifier TLV

“

[2] Add the following paragraph to Clause 8.4.1:

“

An MIH protocol payload carries a Source MIHF Identifier TLV and a Destination MIHF Identifier TLV followed by MIH Service Specific TLVs.

“

[3] Change Figure 28 as follows:

(4)

UIR

VER

Ack

MIH Message ID

(16)

Transaction ID

(16)

Variable Payload Length

(16)

SID

(4)

Opcode

(2)

AID

(10)

Rsp

Ack

Req

Octet 1

Octet 2

Octet 3

Octet 4

(1)

(1)

(1)

(

MS

B)

(

LS

B)

FN

(6)

M

(1)

Reserved

(2)

[4] Add the following rows to Table 22:

| Field Name | Size (bits) | Description |
| --- | --- | --- |
| More Fragment (M) | 1 | This field is used for indicating that the message is a fragment to be followed by another fragment. It is set to ‘0’ for a message that is not fragmented and for the last fragment. The two 0 valued conditions are differentiated by the FN field. It is set to ‘1’ for a fragment that is not the last one. |
| Fragment Number (FN) | 6 | This field is used for representing the sequence number of a fragment. The fragment number starts from 0. The maximum fragment number is 63. This field is set to ‘0’ for a message that is not fragmented. |

[5] Change the Size of Reserved field in Table 22 from 9 to 2.

[6] Add the following Clause:

8.4.2 Fragmentation and Reassembly

8.4.2.1 General

The MIH protocol supports a fragmentation mechanism.  The MIH fragmentation mechanism is defined using ‘M’ (More Fragment) and ‘FN’ (Fragment Number) fields of the MIH protocol header.

An MIH message is fragmented only when MIH message is sent natively over a L2 medium such as Ethernet. The message is fragmented when the message size exceeds the Maximum Transmission Unit (MTU) size of the link layer. When the MTU size of the link layer is unknown, the maximum MIH fragment size is set to the minimum MTU of 1500 octets. An MTU size may be known via the MIB of the link layer. When MIH message is sent using a L3 or higher layer transport, other layers take care of fragmentation issue and MIH Protocol does not handle fragmentation in such cases.

Figure X shows the components of the fragmented MIH protocol frame.

When fragmentation is performed, an MIH protocol payload carries a Source MIHF Identifier TLV and a Destination MIHF Identifier TLV followed by a fragment payload. Based on the size of the fragment, a fragment payload may not be aligned on a TLV boundary.

MIH Protocol Frame (with fragmentation)

MIH Protocol Header

(8 octets)

MIH Protocol Payload

Source

MIHF Identifier TLV

Destination MIHF Identifier TLV

Fragment Payload

Figure Y: Fragmented MIH Protocol Frame Format

8.4.2.2 Fragmentation

When an MIH message is fragmented, the fragmentation is performed within ‘Transmit()’ procedure in the MIH transaction protocol state machines. The MIH protocol header, the Source Identifier and Destination Identifier of the original message are copied to each fragment. However the ‘Variable Payload Length’, ‘More Fragment’ and ‘Fragment Number’ fields are updated accordingly for each fragment.

Variable Payload Length of each fragment indicates the number of octets in the MIH protocol payload of that fragment. The length of each of the fragments is the same except the last one, which may be smaller.

‘More Fragment’ and ‘Fragment Number’ fields of each fragment are set according to the description in Table 22.

If the number of fragments for the original message exceeds 64, the original message is discarded by the source MIHF without being transmitted.

No retransmission is performed for any single fragment of a multi-fragment message .

8.4.2.3 Reassembly

The destination MIHF reassembles the received fragments into an original message. Reassembly is performed outside the MIH transaction state machines. ‘MsgIn’ and ‘MsgInAvail’ variables are set only after successful reassembly.

The following fields are used for reassembling fragments:

- MIH Message ID

- Transaction ID

- Source Identifier TLV

- Destination Identifier TLV

- More Fragment

- Fragment Number

When any fragment of a multi-fragment message has arrived first, the destination MIHF starts a timer referred to as ‘ReassemblyTimer'. If this ‘ReassemblyTimer’ expires before all fragments have been received, the destination MIHF discards those fragments that it has received. A duplicate fragment is discarded.

An example of an original MIH message and fragmented MIH messages is shown in Figure Y.

SID(20) DID(30)  MIH Service Specific TLVs(1600)

SID(20) DID(30)  Fragment Payload (158)

SID(20) DID(30)  Fragment Payload (1442)

Header(8)

Original message (M=0, FN=0, size=1658)

iMIH Protocol Payload

ze=1558 octets)

1st fragment message (M=1, FN=0, size=1500 octets)

2nd fragment message (M=0, FN=1, size=216 octets)

SID: Source MIHF Identifier TLV

DID: Destination MIHF Identifier TLV

The integer number within the curved-brackets of each field indicates the length of the field in octets.

Header(8)

Header(8)

Figure Y: MIH Fragmentation Example for MTU of 1500 octets

[7] Add the following MIB object in Annex F.2:

SEQUENCE{

dot21LocalMihfIndex Integer32,

dot21LocalMihfID Dot21MihfID,

dot21LocalEventList Dot21EventList,

dot21LocalCommandList Dot21CommandList,

dot21LocalISQueryTypeList Dot21ISQueryTypeList,

dot21LocalTransportList Dot21TransportList,

dot21LocalVersion Integer32,

dot21LocalMaxTransactionLifetime Integer32,

dot21LocalMaxRetransmissionIntvl Integer32,

dot21LocalMaxRetransmissionCntr Integer32,

dot21LocalMaxAvgTransmissionRate Integer32,

dot21LocalMaxBurstSize Integer32,

dot21LocalReassemblyTimeout Integer32

}

dot21LocalReassemblyTimeout OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION "The timeout value for ReassemblyTimer."

DEFVAL { 5 } ::={ dot21LocalMihfEntry 13 }

[8] Add the following PICS entry to Clause G.8.3:

| Item number | Item description | References | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| F.9.3.9 | Is MIH fragmentation supported? | 8.5 | M | Yes[] No[] | MC9 |

[9] Add the following PICS entry to Clause G.8.6 (Timers):

| Item number | Item description | References | Status | Support | Allowed Values | Supported Values |
| --- | --- | --- | --- | --- | --- | --- |
| F.9.6.3 | ReassemblyTimer | 8.5.2 | M | Yes[] No[] |  |  |