| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |
| --- | --- |
| Title | An MIH Fragmentation Mechanism |
| Date Submitted | May 28, 2008 |
| Source(s) | Yoshihiro Ohba (Toshiba), Subir Das (Telcordia), Yuu-Heng Alice Cheng (Telcordia), Vivek Gupta (Intel), Dennis Edwards (CoCo Communications), Anthony Chan (Huawei), Nada Golmie (NIST), Junghoon Jee (ETRI), Richard Rouil (NIST), Ajay Rajkumar (Alcatel-Lucent) |
| Re: | IEEE 802.21 WG discussion in May 2008 |
| Abstract | This document describes a fragmentation mechanism for the MIH protocol to addresses SB recirc-2 Comment #145. |
| Purpose | Sponsor Ballot comment resolution |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

# Introduction

If a new EtherType is defined to carry MIH protocol over 802.3, a major issue is that 802.3 does not support fragmentation while MIH message size can be larger than link-layer MTU for 802.3.  In general, for the MIH protocol to work over a transport that does not support fragmentation, a fragmentation mechanism is needed within the MIH protocol such that the MIH protocol can work regardless of the MTU size of the link layers.

This document proposes an MIH fragmentation mechanism to address this issue.

# Proposed Text

[1] Change Figure 27 as follows:

MIH Protocol Frame

MIH Protocol Header

(8 octets)

MIH Protocol Payload

MIH Service Specific TLVs

Source

MIHF Identifier TLV

Destination MIHF Identifier TLV

“

[2] Add the following paragraph to Clause 8.4.1:

“

An MIH protocol payload carries a Source MIHF Identifier TLV and a Destination MIHF Identifier TLV followed by MIH Service Specific TLVs.

“

[3] Change Figure 28 as follows:

(4)

UIR

VER

Ack

MIH Message ID

(16)

Transaction ID

(12)

Variable Payload Length

(16)

SID

(4)

Opcode

(2)

AID

(10)

Rsp

Ack

Req

Octet 1

Octet 2

Octet 3

Octet 4

(1)

(1)

(1)

(

MS

B)

(

LS

B)

M

(1)

Rsvd2

(4)

(1)

FN

(7)

Rsvd1

[4] Add the following rows to Table 22:

| Field Name | Size (bits) | Description |
| --- | --- | --- |
| More Fragment (M) | 1 | This field is used for indicating that the message is a fragment to be followed by another fragment. It is set to ‘0’ for a message that is not fragmented and for the last fragment. The two 0 valued conditions are differentiated by the FN field. It is set to ‘1’ for a fragment that is not the last one. |
| Fragment Number (FN) | 7 | This field is used for representing the sequence number of a fragment. The fragment number starts from 0. The maximum fragment number is 127. This field is set to ‘0’ for a message that is not fragmented. |

[5] Change Table 22 as follows. Change the Size of Reserved field from 9 to 1 and change the field name to Rsvd1. Add another 4-bit reserved (Rsvd2) field before Transaction ID. Change the Size of Transaction ID to 12.

[6] Add the following Clause:

8.4.2 Fragmentation and Reassembly

8.4.2.1 General

The MIH fragmentation mechanism is defined using ‘M’ (More Fragment) and ‘FN’ (Fragment Number) fields of the MIH protocol header.

An MIH message is fragmented only when MIH message is sent natively over a L2 medium such as Ethernet. The message is fragmented when the message size exceeds aFragmentationThreshold. The size of each of the fragments is the same except the last one, which may be smaller. The maximum fragment size is defined as the maximum value of aFragmentationThreshold, which shall be equal to the Maximum Transmission Unit (MTU) of the link layer that is on the path between two MIHF nodes. When the MTU of the link layer between two MIHF nodes is known, the maximum fragment size is set to the MTU. The method of determining such an MTU is outside the scope of this specification. When the MTU of the link layer between two MIHF nodes is unknown, the maximum fragment size is set to the minimum MTU of 1500 octets. When MIH message is sent using a L3 or higher layer transport, L3 takes care of fragmentation issue and MIH Protocol does not handle fragmentation in such cases.

Figure X shows the components of the fragmented MIH protocol frame. The MIH protocol payload carries a Source MIHF Identifier TLV and a Destination MIHF Identifier TLV followed by a fragment payload. Based on the fragment size, the fragment payload may not be aligned on a TLV boundary, i.e., TLVs other than the Source MIHF Identifier and Destination Identifier TLVs may not be complete within the fragment payload. The fragment size may be smaller than the maximum fragment size and shall be larger than that can generate more than 128 fragments.

MIH Protocol Frame (with fragmentation)

MIH Protocol Header

(8 octets)

MIH Protocol Payload

Source

MIHF Identifier TLV

Destination MIHF Identifier TLV

Fragment Payload

Figure X: Fragmented MIH Protocol Frame Format

8.4.2.2 Fragmentation

When an MIH message is fragmented, the fragmentation is performed within ‘Transmit()’ procedure in the MIH transaction protocol state machines. The MIH protocol header, the Source Identifier and Destination Identifier of the original message are copied to each fragment. However the ‘Variable Payload Length’, ‘More Fragment’ and ‘Fragment Number’ fields are updated accordingly for each fragment.

Variable Payload Length of each fragment indicates the number of octets in the MIH protocol payload of that fragment.

‘More Fragment’ and ‘Fragment Number’ fields of each fragment are set according to the description in Table 22.

When data are to be transmitted, the number of octets in the fragment shall be determined by the fragment size and the number of octets in the multi-fragment message that have yet to be assigned to a fragment at the instant the fragment is constructed for the first time. Once a fragment is transmitted for the first time, its frame body content and length shall be fixed until it is successfully delivered to the destination MIHF.

No retransmission by the MIH protocol  (defined in 8.2) is performed for any single fragment of a multi-fragment message.

8.4.2.3 Reassembly

The destination MIHF reassembles the received fragments into an original message. Reassembly is performed outside the MIH transaction state machines. ‘MsgIn’ and ‘MsgInAvail’ variables are set only after successful reassembly. An MIHF shall be capable of receiving fragments of arbitrary length.

The following fields are used for reassembling fragments:

- MIH Message ID

- Transaction ID

- Source Identifier TLV

- Destination Identifier TLV

- More Fragment

- Fragment Number

When any fragment of a multi-fragment message has arrived first, the destination MIHF starts a timer referred to as ‘ReassemblyTimer'. If this ‘ReassemblyTimer’ expires before all fragments have been received, the destination MIHF discards those fragments that it has received. A duplicate fragment is discarded.

An example of an original MIH message and fragmented MIH messages is shown in Figure Y.

SID(20) DID(30)  MIH Service Specific TLVs(1600)

SID(20) DID(30)  Fragment Payload (158)

SID(20) DID(30)  Fragment Payload (1442)

Header(8)

Original message (M=0, FN=0, size=1658)

iMIH Protocol Payload

ze=1558 octets)

1st fragment message (M=1, FN=0, size=1500 octets)

2nd fragment message (M=0, FN=1, size=216 octets)

SID: Source MIHF Identifier TLV

DID: Destination MIHF Identifier TLV

The integer number within the curved-brackets of each field indicates the length of the field in octets.

Header(8)

Header(8)

Figure Y: MIH Fragmentation Example for MTU of 1500 octets

[7] Add the following text in Annex F.1.2:

- aFragmentationThreshold (Read-Write)

  - Unit: octets

- ReassemblyTimer (Read-Write)

  - Unit: seconds

[8] Add the following MIB object in Annex F.2:

Dot21LocalMihfEntry ::=

SEQUENCE{

dot21LocalMihfIndex Integer32,

dot21LocalMihfID Dot21MihfID,

dot21LocalEventList Dot21EventList,

dot21LocalCommandList Dot21CommandList,

dot21LocalISQueryTypeList Dot21ISQueryTypeList,

dot21LocalTransportList Dot21TransportList,

dot21LocalVersion Integer32,

dot21LocalMaxTransactionLifetime Integer32,

dot21LocalMaxRetransmissionIntvl Integer32,

dot21LocalMaxRetransmissionCntr Integer32,

dot21LocalMaxAvgTransmissionRate Integer32,

dot21LocalMaxBurstSize Integer32,

dot21LocalFragmentationThreshold Integer32,

dot21LocalReassemblyTimeout Integer32

}

dot21LocalFragmentationThreshold OBJECT-TYPE

SYNTAX Unsigned32 (8..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION "The value for aFragmentationThreshold on this node."

DEFVAL { 1500 } ::={ dot21LocalMihfEntry 13 }

dot21LocalReassemblyTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION "The timeout value for ReassemblyTimer."

DEFVAL { 5 } ::={ dot21LocalMihfEntry 14 }

Dot21PeerMihfEntry ::=

SEQUENCE {

dot21PeerMihfIndex Integer32,

dot21PeerMihfIDDot21MihfID,

dot21PeerLocalMihfID Dot21MihfID,

dot21PeerEventList Dot21EventList,

dot21PeerCommandList Dot21CommandList,

dot21PeerISQueryTypeList Dot21ISQueryTypeList,

dot21PeerTransportList Dot21TransportList,

dot21PeerTransportTypeINTEGER,

dot21PeerVersion Integer32,

dot21PeerMaxTransactionLifetime Integer32,

dot21PeerMaxRetransmissionIntvl Integer32,

dot21PeerMaxRetransmissionCntr Integer32,

dot21PeerMaxAvgTransmissionRate Integer32,

dot21PeerMaxBurstSize Integer32,

dot21PeerFragmentationThreshold Integer32,

dot21PeerReassemblyTimeout Integer32

}

dot21PeerFragmentationThreshold OBJECT-TYPE

SYNTAX Unsigned32 (8..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION "The value for aFragmentationThreshold used for this peer MIH node."

DEFVAL { 1500 } ::={ dot21LocalMihfEntry 15 }

dot21PeerReassemblyTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION "The timeout value for ReassemblyTimer used for this peer MIH node."

DEFVAL { 5 } ::={ dot21LocalMihfEntry 16 }

[9] Add the following PICS entry to Clause G.8.3:

| Item number | Item description | References | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| F.9.3.9 | Is MIH fragmentation supported? | 8.5 | M | Yes[] No[] | MC9 |

[10] Add the following PICS entry to Clause G.8.6 (Timers):

| Item number | Item description | References | Status | Support | Allowed Values | Supported Values |
| --- | --- | --- | --- | --- | --- | --- |
| F.9.6.3 | ReassemblyTimer | 8.5.2 | M | Yes[] No[] |  |  |