| Project | IEEE 802.21 MIHO <http://www.ieee802.org/21/> |
| --- | --- |
| Title | Resolution for SB4 Comment #26 and #27 |
| Date Submitted | June, 2008 |
| Source(s) | Vivek Gupta (Intel Corporation), Subir Das (Telcordia) |
| Re: | IEEE 802.21 WG Teleconference in June 2008 |
| Abstract | This document addresses SB recirc-4 Comments 26 and 27 |
| Purpose | Sponsor Ballot comment resolution |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

Comment 26:

The Information service supports different information elements. The IS should also support an opaque data type (a blob data type) that could be used to represent policies. This would allow operators to push policies to MN using IS as well.

There is no need to define an IE for this purpose. The primitive can include appropriate data type for satisfying this.

Comment 27:

Operators may want to push policies or other IEs to Mobile devices. The IS could easily support that. Add a capability for operators to push information as well, as opposed to UEs just querying this information.

Change 1:----------------------------------------------------------------------------------------------

    - MIH_Push_Information

      - MIH_Push_Information.indication

      - Function

This primitive is used by an MIH User, i.e., MIIS Server, to push information to the.MN.

      - Semantics of service primitive

MIH_Push_Information.indication

(
				DestinationIdentifier,

InfoNetworkPolicy
				InfoBinaryDataList,
				InfoRDFDataList,
				InfoRDFSchemaURLList,
				InfoRDFSchemaList
			 	)

Parameters:

| Name | Type | Description |
| --- | --- | --- |
| Destination Identifier | MIHF_ID | The local MIHF or a remote MIHF which will be the destination of this response. |
| InfoNetworkPolicy | LIST (OCTET) | Network Selection Policy that may be updated by the operator. |
| Info Binary Data List | LIST(IR_BIN_DATA) | (Optional) A list of TLVs for Network Information. The list will be sorted from most preferred first to least preferred last. |
| Info RDF Data List | LIST(IR_RDF_DATA) | (Optional) A list of RDF responses. The list will be sorted from most preferred first to least preferred last. |
| Info RDF Schema URL List | LIST(IR_SCHM_URL) | (Optional) A list of RDF Schema URL. The list will be sorted from most preferred first to least preferred last. |
| Info RDF Schema List | LIST(IR_RDF_SCHM) | (Optional) A list of RDF schema responses. The list will be sorted from most preferred first to least preferred last. |

      - When generated

This message can be generated by the MIIS server to update any policies on the MN or to update any other IEs from MIIS on to the MN.

      - Effect on receipt

Upper layer entities take different actions on this notification.

Change 2:----------------------------------------------------------------------------------------------

Update an entry in Table-17 as follows

MIH_Push_Information	Indication to push operator policies or other information on to MN 	7.4.26

Change 3:-----------------------------------------------------------------------------------------------------------------

8.6.4

MIH Information service uses following  messages:

- MIH_Get_Information request

- MIH_Get_Information response

- MIH_Push_Information indication

These messages are described separately in the following sub-clauses.

Change 4:-----------------------------------------------------------------------------------------------------------------

8.6.4.3 MIH_Push_Information indication

The corresponding MIH primitive of this message is defined in 7.4.26.1. This is an indication to push operator policies or other network information to the MN.

MIH Header Fields (SID=4, Opcode=3, AID=2)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

InfoNetworkPolicy (Info Network Policy TLV)

InfoResponseBinaryDataList (optional) (Info Response Binary Data List TLV)

InfoResponseRDFDataList (optional) (Info Response RDF Data List TLV)

InfoResponseRDFSchemaURLList (optional) (Info Response RDF Schema URL List TLV)

InfoResponseRDFSchemaList (optional) (Info Response RDF Schema List TLV)

Change 5:-----------------------------------------------------------------------------------------------------------------

Update Table D-1

MIH_Push_Information	2

Change 6:-----------------------------------------------------------------------------------------------------------------

Update to PICS., Table after G.8.4 PDUs (page 251)

Add following entry after *G.8.4.31 and update subsequent entries accordingly.

G.8.4.32 MIH_Push_Information indication 8.6.4.3 MC3:M  Yes [ ] No [ ] N/A[ ]  PDU32

Change 7:-----------------------------------------------------------------------------------------------------------------

Update Table-H1

MIH_Push_Information		RP1

Change 8:-----------------------------------------------------------------------------------------------------------------

MIH_Register.request ( DestinationIdentifier, LinkIdentifierList, RequestCode, locationInfo )

locationInfo		LOCATION		Location of MN

Change 9:-----------------------------------------------------------------------------------------------------------------

MIH_Register.indication ( DestinationIdentifier, LinkIdentifierList, RequestCode, locationInfo )

locationInfo		LOCATION		Location of MN

Change 10:-----------------------------------------------------------------------------------------------------------------

6.2.4 MIH Registration

MIH registration provides a mechanism for an MIH entity to make its presence known to a peer MIH entity. For example, in a network controlled inter-technology handover framework, MIH registration can be used by an MN to declare its presence, including current location to a selected MIH PoS. MIH Registration is  mandatory for use with push mode of MIIS.

Change 11:-----------------------------------------------------------------------------------------------------------------

8.6.1.3 MIH_Register request

Add below row after RequestCode

locationInfo location Info TLV

Change 12: -----------------------------------------------------------------------------------------------------------------

5.3.4 Media independent information service

The Media Independent Information Service (MIIS) provides a framework and corresponding mechanisms by which an MIHF entity can discover and obtain network information existing within a geographical area to facilitate the handovers.

The neighboring network information discovered and obtained by this frame-work and mechanisms can also be used in conjunction with user and network operator policies for optimum initial network selection and access (attachment), or network re-selection in idle mode.

MIIS primarily provides a set of information elements (IEs), the information structure and its representation, and a query/response type of mechanism (pull mode) for information transfer. The information can also include inter-technology handover policies. The definition of such policies is outside the scope of this standard. MIIS also supports a push mode wherein the information can be pushed to the MN by the operator. The information can be present in an information server from where the MIHF in the MN accesses it. The definition of the information server is outside the scope of this standard. In other cases information can be present locally in the MN, and can be learned by the MN or pre-provisioned, or both. The definition of and indexing of such a local database, as well as the regime for maintaining it or accessing it, are outside the scope of this standard.

Change 13: -----------------------------------------------------------------------------------------------------------------

Table L2.

Add entry:

Info Network Policy Length 		66

Info Network Policy  		67

Reserved 			68-99

Vendor Specific TLV 		100

Reserved for Experimental TLVs	101-255