| Project | IEEE 802.21 Media Independent Handover Services IEEE 802.21b: Broadcast Services <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Use Cases and Requirements for Handovers with Broadcast Services |  |
| Date Submitted | March 10, 2009 |  |
| Source(s) | Juan Carlos Zuniga (InterDigital Communications, LLC), Burak Simsek (Fraunhofer Institute) |  |
| Re: | IEEE 802.21 Session #31 in Vancouver, BC |  |
| Abstract | This document describes the use cases and technical requirements document on MIH with broadcast technologies. |  |
| Purpose | Task Group acceptance |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

The IEEE 802.21-2008 published standard provides a media independent solution for handovers between bidirectional technologies but it does not address the case when a unidirectional (i.e. downlink only) type of technology is involved in the handover.

Downlink-only technologies such as Digital Video Broadcasting (DVB), Digital Multimedia Broadcasting (DMB) and Media Forward Link Only (MediaFLOTM) are becoming more widespread in use. There is a need to support optimized handovers between these technologies and other technologies already supported by IEEE 802.21-2008.

The 802.21b Task Group will address these issues by providing an amendment to the IEEE 802.21-2008 specification with the missing functionalities to support handovers with broadcast technologies.

The present document describes the use cases that will be addressed by the group, as well as the set of requirements that future technical proposals should address.

## Broadcast Service Network

Broadcast networks are usually comprised of Content Provider(s), Content Aggregator(s) and Service Provider(s). A particular service provider can have more than one radio access technology to provide the service. Also, users can have multi-mode devices supporting two or more radio access technologies.

Since not all radio access technology networks are tightly coupled, there are missing functionalities when a particular session is to be transferred from one network to another.

Although there are many levels involved in transferring the session, 802.21b will address the missing functionalities at the lower layers (i.e. below layer 3) that are required to perform this session transfer in a seamless manner.

The following figure depicts a typical Broadcast Service Network.

In this case, 802.21b can enhance the session handover by providing the missing functionalities in the red and orange blocks.

# Scope

The scope of this document is to describe usage scenarios, requirements and potential high-level approaches for the following 802.21b topics:

  - Broadcast-related signaling for handover optimization

  - MIH-level interface definitions

  - Link-level interface definitions

# Handover Types

There are several ways in which a handover sequence can take place. In general, handovers can be considered as one of the following types: Mobile-controlled Handover (MCH), Network-controlled Handover (NCH), or Network-assisted Handover (NAH).

MCH are the types of handovers where the handover decision is taken locally at the mobile node. In this case there is little or no 802.21 signaling involved with the network and the existence of a Point of Service (PoS) is optional. One example of MCH can be when a mobile node detects that a signal level is rapidly decreasing and it then immediately initiates the attachment with the target network.

NCH are the handovers where the handover decision is taken at the network side. In this case the network will send an explicit handover command to the mobile node, and the mobile node will then continue the handover process. An example of a NCH can be for instance when the decision to perform load-balancing is taken at the network side and then mobile nodes are forced to handover from the high load network to the low load network.

NAH are the handovers where the decision is taken at the mobile node with assistance from the network. In this case there is a PoS providing services to the mobile node, but a handover command is not necessarily sent from the network. An example of this type of handovers is when a mobile node performs an information service request when reaching a new location area, and based on the information service response it realizes that there is a better radio access network in the new location. The mobile node can then locally perform a handover but based on the new information received from the network.

## Mobile controlled handover

### From DO to Bidirectional Technology

#### Scenario

Based on different triggers such as signal quality, power consumption, service unavailability the mobile decides to change to a bidirectional technology.

The determinants for the rest of the handover procedure are:

- New service can be a multicast service or a unicast service.

- If the new service is a multicast service

  - Is the service available at the new access point?

#### Role of IEEE 802.21

- Event services for monitoring DO technology behavior

- Command services for the new link, media independent scan for new links, link up, down…

- MIIS for the availability of the new service at a specific link (OMA BCAST at the 3G side)

- MIIS for multicast stream information (Multicast IP address)

- In case service is not available, multicast join request can be sent prior to IP assignment

### From Bidirectional to DO Technology

#### Scenario

Network triggers at the Bidirectional side as it exists currently in IEEE 802.21 result a decision to handoff from Bidirectional to DO (Signal quality, price, better QoS…).

The determinants for the rest of the handover procedure are:

- Does the service exist in DO technology

- In case the service does not exist:

  - How do we initiate the service at the DO side?

    - Send stream/service relevant information to DO technology?

  - How do we determine the location of the service at the DO technology (IP/Mac address, frequency and other DO specific parameters)

  - How is the synchronization possible?

- In case the service exists at the DO technology

  - How do we get information for successful tuning?

  - How are the security issues handled?

  - How do we decide on synchronization during handover?

#### Role of IEEE 802.21

- MIIS can provide information relevant to services available at the DO technology

  - Frequency, FEC, Cell ID…

- Handover can further be optimized by giving the PID information directly to the mobile

- If the service did not exist before, then network can tell the new address (MAC/IP or even PID) at the DO side

## Network controlled handover

### Handover from DO to a Bidirectional Technology

#### Scenario

## Network assisted handover

# Use Cases

## Coverage extension

In this case coverage extension refers to the case when a coverage are for a service provided by a certain broadcast technology (e.g. DVB / DMB / MediaFLO) is to be extended to a wider area by relying on a different radio access technology (e.g. WiMAX / WLAN / 3GPP)

Use Case 1: Traditional broadcast scenario

Use Case 2: On-demand scenario

Use Case 3: Indoor coverage scenario

## Load Balancing

Use Case 4: Number of unicast subscribers justifies setting up a broadcast service to hand them over

## Bidirectional channel

Use Case 5: Support for interactive services