| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |
| --- | --- |
| Title | IEEE 802.21 MIH Protocol Detail Test Cases |
| DCN | 21-09-0050-00-0000 |
| Date Submitted | March 12, 2009 |
| Source(s) | Y. Alice Cheng (Telcordia) |
| Re: | IEEE 802.21 Session #31 in Vancouver, BC, Canada |
| Abstract | This contribution provides a draft of interoperability test cases for implementing MIH protocol. The very last page is for discussion. |
| Purpose | Specific functional requirements need to be developed for the IEEE 802.21 devices to provide the necessary reliability, availability, and interoperability of communications with different operator networks. In addition, guidelines for using MIH protocol need to be developed so that vendors and operators can better understand the issues, pros, and cons of implementing IEEE 802.21 for supporting various mobility handover scenarios. |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |

# Introduction

## Scope

The primary scope of this document is to develop the functional requirements for interoperability between implementations of IEEE Std 802.21-2008. The potential end points include wireless mobile devices and network entities described in the standard.

The focus is MIH protocol exchange and the functionality of the exchange. This document is not intended to re-iterate the details of the PICS proforma which is already defined in IEEE 802.21-2008.

At its current state, this document provides only a collection of test cases.

## Purpose

IEEE 802.21 standard defines extensible IEEE 802 media access independent mechanisms as a set of tools to incorporate information from mobile device and network operators for supporting a seamless handover between heterogeneous access technologies, in turn, improve user experiences.

Specific functional requirements need to be developed for the IEEE 802.21 devices to provide the necessary reliability, availability, and interoperability of communications with different operator networks. In addition, guidelines for using MIH protocol need to be developed so that vendors and operators can better understand the issues, pros, and cons of implementing different IEEE 802.21 functionalities for improving user experiences over various mobility handover scenarios.

The purpose of this document is to define MIH interoperable features and its test cases for addressing the needs described above.

## Target Environment

Figure 1 depicts the target environment for IEEE 802.21 interoperability use cases. There are two operators: Operator O and Operator P. Operator O owns a WiMAX network and a Wi-Fi network with ESSID “Sandbox.” Operator P owns a 3GPP network and a Wi-Fi network with ESSID “Pandora.” There are three types of mobile node: aPhone (access to 3GPP and Wi-Fi networks), pinkBerry (access to WiMax and Wi-Fi networks), and myTop (access to WiMAX, Wi-Fi, and 3GPP networks).

Figure 1 Network Deployment Example

# Test Suites

The test cases are categorized into the following suites:

- Mobile Initiated Handover (MHO) which are test cases for handover initiated by a mobile device and assisted by the network operator;

- Network-initiated Handover (NHO) which are test cases for handover initiated by the operator and enforced by the mobile node;

- Information Service (IS) identifies test cases which are related to MIH Information Service; and

- Protocol Data Unit (PDU) contains the test cases directly derived from the PDUs (IEEE Std 802.21-2008 Clause M8.4) in PICS perfoma.

A summary of the test cases identified in this document is shown in Table 1.

Table 1 Interoperability use cases summary

|  | Test Case | Summary | PICS Addressed |
| --- | --- | --- | --- |
| Mobile Initiated Handover | MHO-1 Mobile initiated handover w/o network resource reservation | MN switches from one network to the other network. | MC6 PDU16, PDU17 (MIH_MN_HO_Candidate_Query) PDU20, PDU21 (MIH_MN_HO_Commit) PDU26, PDU27 (MIH_MN_HO_Complete) |
|  | MHO-2 Mobile initiated handover w/ network resource reservation | MN switches from one network to the other network based with the support of resources reserved by the network | MC6 PDU16, PDU17 (MIH_MN_HO_Candidate_Query) PDU20, PDU21 (MIH_MN_HO_Commit) PDU18, PDU19 (MIH_N2N_HO_Query_Resources) PDU24, PDU25 (MIH_N2N_HO_Commit) PDU26, PDU27 (MIH_MN_HO_Complete) PDU28, PDU29 (MIH_N2N_HO_Complete) |
| Network Initiated Handover | NHO-1 Network initiated handover w/o network resource reservation | Service Provider/Operator requests the MN to switch from one network to the other without network resource reservation. | MC7 PDU14, PDU15 (MIH_Net_HO_Candidate_Query) PDU22, PDU23 (MIH_Net_HO_Commit) PDU26, PDU27 (MIH_MN_HO_Complete) |
|  | NHO-2 Network initiated handover w/ network resource reservation | Service Provider/Operator requests the MN to switch from one network to the other with reserving resource on the other network. | MC7 PDU14, PDU15 (MIH_Net_HO_Candidate_Query) PDU18, PDU19 (MIH_N2N_HO_Query_Resources) PDU22, PDU23 (MIH_Net_HO_Commit) PDU24, PDU25 (MIH_N2N_HO_Commit) PDU26, PDU27 (MIH_MN_HO_Complete) PDU28, PDU29 (MIH_N2N_HO_Complete) |
| Information Service | IS-1 Query available networks | Discover the available network types based on the MN’s current location (via geo-location or its PoA’s location) using binary/RDF query. | MC3 (MC3.1 or MC3.2) PDU30, PDU31 (MIH_Get_Information) |
|  | IS-2 IS push mode | After MIH registration, the MN receives push information from the network [NOTE: need to determine what information]. | MC3.3 [OPT] PDU32 (MIH_Push_Information) |
| Protocol Data Unit | PDU35-36 MIH Register | MIH_Register request and response message exchange. | PDU35, PDU36 (MIH_Register) |
|  | PDU37-38 MIH Deregister | MIH_Deregister request and response message exchange. | PDU37, PDU38 (MIH_Deregister) |
|  | PDU39-40 MIH Event Subscribe | MIH_Event_Subscribe request and response message exchange [NOTE: How to verify the subscription is successful without receiving a notification?] | PDU39, PDU40 (MIH_Event_Subscribe) |
|  | [Work In Progress] |  |  |

## Abstract Architecture

Following is a general architecture.

MN1

PoS1

PoS2

Link 1

Link 2

Conn 5

MIHF ID=pos1.opero.com

MIHF ID=pos2.operp.com

MIHF ID=mn1.opero.com

IS2

IS1

MIHF ID=is1.opero.com

Conn 6

Conn 7

MIHF ID=is2.operp.com

PoA2

PoA1

Conn 3

Figure 2 Abstract architecture

Conn 4

The architecture in Error: Reference source not found contains several MIH devices: one MN (mobile node), two PoS (Point of Service), and two IS (Information Service Server). These devices understand MIH protocol and each have their own unique MIHF ID. The two PoAs (Point of Attachments, POA1 and PoA2) are used for illustrating the network connectivity status and are not part of the testing units. In this example, the MN is attached to the OperO network (operator O) via Link1 and to the OperP network (operator P) via Link2. Conn#, where # indicates a number, is a UDP connection between the two end points. [NOTE: UDP or UDP over something else?]

The following table describes the potential access network technology for Link1 and Link2. [NOTE: What link types need to be specified? Will the actual test case need to depend on the specific link type?]

Table 2 Access technologies for Link1 and Link2

| Operator | Link1 | Link 2 |
| --- | --- | --- |
| Same Operator | 802.11 | 802.16e |
| Same Operator | 802.11 | EVDO |
| Different Operator | 3GPP | 802.11 |

Following are different test architectures that are applied to different test cases.

MN1

PoS1

Link 1

MIHF ID=pos1.opero.com

MIHF ID=mn1.opero.com

PoA1

Conn 3

Figure 2 Test Architecture 1

Figure 2  contains only two MIH devices, i.e., MN1 and PoS1. The MN1 is attached to PoA1 and communicates with PoS1 via Conn3.

PoS1

MIHF ID=pos1.opero.com

Conn 5

PoS2

MIHF ID=pos2.opero.com

Figure 3 Test Architecture 2

Figure 3  architecture tests the communication between two PoS.

# Mobile-initiated handover test suite

The purpose of this test suite is to verify that the mobile node can initiate and complete a handover-related sequence of operations from one network to the other network using MIH protocol with network entities.

[NOTE: Does it need to be seamless? Do we have performance requirement?]

## MHO-1 Mobile-initiated handover w/o network resource reservation

[Work In Progress]

## MHO-2 Mobile-initiated handover w/ network resource reservation

A mobile node is attached to PoA1 and discovers the network PoS via pre-configuration. The MN then decides to perform a handover with the assistance from the network (to reserve network resources) within the same operator. Figure 4 shows the devices, links, and connections that are used for this test.

MN

PoS1

PoA2

Link 1

Link 2

MIHF ID=pos1.opero.com

MIHF ID=mn.opero.com

Conn4

PoA1

Conn3

PoS2

MIHF ID=pos2.opero.com

Conn5

Figure 4 MHO-1 Test Architecture

### Sequence

MN was registered to PoS1 through Conn3 via Link1. When test starts, MN provides a list of candidate networks and queries PoS1 for recommendation. The PoS1 returns PoA2 as MN’s preferred point of attachment.  MN then establishes Link 2. PoS1 communicates with PoS2 through Conn5 for network resource query and reservations. Once attached to PoA2, the MN re-registers to PoS2 through Conn4 via Link2.

See clause C.1 in IEEE Std 802.21-2008.

### Test Case

| Test: | MHO-2 | Selection Criteria: | Mandatory | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Mobile-initiated handover with network resource reservation. |  |  |  |
| Purpose: |  | To verify that - The mobile node can initiate and complete a handover using MIH protocol message exchange - The network PoS reserve network resources to assist handover using MIH protocol message exchange |  |  |  |
| Pre-Condition: |  | Use Figure 4 MHO-1 Test Architecture - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - PoS1 is aware of the existence of PoS2 and recognizes what are the PoAs that belong to PoS2. - Based on the network preference, PoS1 is configured to return PoA2 if exists in the candidate list for the current serving link that is connected to PoA1. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends MIH_MN_HO_Query request message to PoS1 through Conn3 via Link 1 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: the MN current serving link (Link1), the list of candidate PoAs (including PoA2) and a list of QoS requirements [NOTE: Need to define the exact value for the QoS data type structure.]? |  |  | Yes | No |
|  | PoS1 sends MIH_N2N_HO_Query_Resources request message to PoS2 via Conn5 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: a list of QoS requirements and a list of PoAs that are candidates (including PoA2)? |  |  | Yes | No |
|  | PoS2 responds with MIH_N2N_HO_Query_Resources response message to PoS1 via Conn5 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: status (0, success), resource available(0, available), and the resource query result for the candidates? |  |  | Yes | No |
|  | PoS1 responds with MIH_MN_HO_Query response message to MN through Conn3 |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: a list of candidates (PoA2 is the first item). |  |  | Yes | No |
|  | MN sends MIH_MN_HO_Commit request message to PoS1 through Conn3 via Link 1 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: the target link type (Link2) and the target network information (PoA2)? |  |  | Yes | No |
|  | PoS1 sends MIH_N2N_HO_Commit request message to PoS2 through Conn5 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: MIHF ID of the MN, target MN link identifier (Link2), target PoA (PoA2), and requested resource set? The request resource set contains the following information: QoS requirements, container (null), and HO_Cause (0, explicit disconnect). |  |  | Yes | No |
|  | PoS2 reserves the network resource and responds with MIH_N2N_HO_Commit response message to PoS1 through Conn5 |  |  |  |  |
|  | Is the QoS resource reserved? [NOTE: Is it verifiable?] |  |  | Yes | No |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: status (0, success), the target link type (Link2) and the target network information (PoA2)? |  |  | Yes | No |
| - P | Pos1 responds with MIH_MN_HO_Commit response message to MN through Conn3 |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: status (0, success), link type (Link2), and the target network information (PoA2)? |  |  | Yes | No |
|  | MN attaches to PoA2 and establishes Link2 |  |  |  |  |
|  | Is the MN attached to PoA2? |  |  | Yes | No |
|  | MN establishes Conn4 and registers to PoS2 |  |  |  |  |
|  | Is MN registered to PoS2? (Reference Test PDU35-36) |  |  | Yes | No |
|  | MN sends MIH_MN_HO_Complete request message to PoS2 through Conn4 via Link2 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: the source link identifier (Link1), the target link identifier (Link2), and handover result (0, handover success)? |  |  | Yes | No |
|  | PoS2 sends MIH_N2N_HO_Complete request message to PoS1 via Conn5 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: MN MIHF ID (mn.opero.com), source link identifier (Link1), target link identifier (Link2), and handover result (0, handover success)? |  |  | Yes | No |
|  | PoS1 responds with MIH_N2N_HO_Complete response message to PoS2 via Conn5 [NOTE: Does PoS1 need to release resource?] |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: status (0, success), MN MIHF ID (mn.opero.com), source link identifier (Link1), target link identifier (Link2), and resource retention status (FALSE, release resource)? |  |  | Yes | No |
|  | PoS2 responds with MIH_MN_HO_Complete response message to MN through Conn4 via Link2 |  |  |  |  |
|  | Is the message received by MN |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: status (0, success), source link identifier (Link1), and target link identifier (Link2). |  |  | Yes | No |
| - M | MN disconnects to PoA1 and break Link1 [NOTE: make-before-break] |  |  |  |  |
|  | Is MN deregistered from PoS1? (Reference Test PDU37-38) [NOTE: Is this mandatory?] |  |  | Yes | No |
|  | Is MN disconnected from PoA1? |  |  | Yes | No |

# Network-initiated handover

## NHO-1 Network-initiated handover w/o network resource reservation

[Work In Progress]

## NHO-2 Network-initiated handover w network resource reservation

A mobile node is attached to PoA1 and registered to PoS1. The operator decides to request the MN to perform a handover to PoS2 with the assistance from the network (to reserve network resources) within the same operator. Figure 4 shows the devices, links, and connections that are used for this test.

### Sequence

[Work In Progress]

See clause C.2 in IEEE Std 802.21-2008.

### Test Case

| Test: | NHO-2 | Selection Criteria: | Mandatory | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Network-initiated handover with network resource reservation. |  |  |  |
| Purpose: |  | To verify that - PoS1 can initiate and complete a handover using MIH protocol message exchange - MN successfully completes a handover using MIH protocol message exchange. - The network PoS reserve network resources to assist handover using MIH protocol message exchange |  |  |  |
| Pre-Condition: |  | Use Figure 4 MHO-1 Test Architecture - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - PoS1 is aware of the existence of PoS2 and recognizes what are the PoAs that belong to PoS2. - Based on the network preference, PoS1 is configured to return PoA2. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | PoS1 sends MIH_Net_HO_Query request message to MN through Conn3 via Link 1 |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: [NOTE: Details TBD.]? |  |  | Yes | No |
|  | MN responds with MIH_Net_HO_Query response message to PoS1 through Conn3 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information:[NOTE: Details TBD.] |  |  | Yes | No |
|  | PoS1 sends MIH_N2N_HO_Query_Resources request message to PoS2 via Conn5 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: a list of QoS requirements and a list of PoAs that are candidates (including PoA2)? |  |  | Yes | No |
|  | PoS2 responds with MIH_N2N_HO_Query_Resources response message to PoS1 via Conn5 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: status (0, success), resource available(0, available), and the resource query result for the candidates? |  |  | Yes | No |
|  | PoS1 sends MIH_N2N_HO_Commit request message to PoS2 through Conn5 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: MIHF ID of the MN, target MN link identifier (Link2), target PoA (PoA2), and requested resource set? The request resource set contains the following information: QoS requirements, container (null), and HO_Cause (0, explicit disconnect). |  |  | Yes | No |
|  | PoS2 reserves the network resource and responds with MIH_N2N_HO_Commit response message to PoS1 through Conn5 |  |  |  |  |
|  | Is the QoS resource reserved? [NOTE: Is it verifiable?] |  |  | Yes | No |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: status (0, success), the target link type (Link2) and the target network information (PoA2)? |  |  | Yes | No |
| - P | PoS1 sends MIH_Net_HO_Commit request message to MN through Conn3 |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: [NOTE: Details TBD]? |  |  | Yes | No |
|  | MN attaches to PoA2 and establishes Link2 |  |  |  |  |
|  | Is the MN attached to PoA2? |  |  | Yes | No |
|  | MN establishes Conn4 and registers to PoS2 |  |  |  |  |
|  | Is MN registered to PoS2? (Reference Test PDU35-36) |  |  | Yes | No |
|  | MN responds with MIH_Net_HO_Commit response message to PoS1 through Conn3 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the information and obtain the following information: [NOTE: Details TBD] |  |  |  |  |
|  | MN sends MIH_MN_HO_Complete request message to PoS2 through Conn4 via Link2 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: the source link identifier (Link1), the target link identifier (Link2), and handover result (0, handover success)? |  |  | Yes | No |
|  | PoS2 sends MIH_N2N_HO_Complete request message to PoS1 via Conn5 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: MN MIHF ID (mn.opero.com), source link identifier (Link1), target link identifier (Link2), and handover result (0, handover success)? |  |  | Yes | No |
|  | PoS1 responds with MIH_N2N_HO_Complete response message to PoS2 via Conn5 [NOTE: Does PoS1 need to release resource?] |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: status (0, success), MN MIHF ID (mn.opero.com), source link identifier (Link1), target link identifier (Link2), and resource retention status (FALSE, release resource)? |  |  | Yes | No |
|  | PoS2 responds with MIH_MN_HO_Complete response message to MN through Conn4 via Link2 |  |  |  |  |
|  | Is the message received by MN |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: status (0, success), source link identifier (Link1), and target link identifier (Link2). |  |  | Yes | No |
| - M | MN disconnects to PoA1 and break Link1 [NOTE: make-before-break] |  |  |  |  |
|  | Is MN deregistered from PoS1? (Reference Test PDU37-38) [NOTE: Is this mandatory?] |  |  | Yes | No |
|  | Is MN disconnected from PoA1? |  |  | Yes | No |

# Information Service Use Cases

This section contains the detail description for MIH Information Service related use cases.

[Work In Progress]

# Common Test Cases

## MIH Capability Discovery

[Work In Progress]

## MIH Register

| Test: | PDU35-36 | Selection Criteria: | Mandatory | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH register request and response |  |  |  |
| Purpose: |  | To verify that - MN sends a valid MIH_Register request message to PoS1 - PoS1 receives the message and registers the MN with infinite validity period - After the registration, PoS1 recognizes MN . |  |  |  |
| Pre-Condition: |  | Use Figure 2 Test Architecture 1 - MN is attached to PoA1. - MN has discovered the PoS1 via pre-configuration or the process of MC4.2. (TODO: Reference the MIH Discovery Test case) |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an MIH_Register request message to PoS1 via Conn3 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information from the MN: MIHF_ID, Link ID list (Link1 and Link2 not connected), and registration code is 0? |  |  | Yes | No |
|  | PoS1 responds with a successful MIH_Register response message |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: status of the operation is success and the valid time interval is 0. |  |  | Yes | No |
| Observations: |  |  |  |  |  |

## MIH Deregister

| Test: | PDU37-38 | Selection Criteria: | Mandatory | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH deregister request and response |  |  |  |
| Purpose: |  | To verify that - MN sends a valid MIH_DeRegister request message to PoS1 - PoS1 receives the message and deregisters the MN. |  |  |  |
| Pre-Condition: |  | Use Figure 2 Test Architecture 1 - MIH Register test case has passed. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an MIH_DeRegister request message to PoS1 via Conn3 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information from the MN: MN MIHF_ID? |  |  | Yes | No |
|  | PoS1 responds with a successful MIH_DeRegister response message |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: status of the operation is success. |  |  | Yes | No |

## MIH Event Subscribe

[Work In Progress] (PDU39 PDU40)

[NOTE: How to verify the subscription is successful without receiving a notification?]

## MIH N2N HO Commit Request

[Work In Progress]

| Test: | PDU24 | Selection Criteria: |  | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH N2N HO Commit request |  |  |  |
| Purpose: |  | To verify that when handover from PoS1 to PoS2 - PoS2 receives MIH_N2N_HO_Commit request message from PoS1 - PoS2 can parse MIH_N2N_HO_Commit request message with the following parameters |  |  |  |
| Pre-Condition: |  | Use Figure 3 Test Architecture 2 - PoS1, PoS2 relationship [Work in Progress] - Unless specified, following are the default parameters to verify against - Source MIH Identifier = pos1.opero.com - Destination MIH Identifier = pos2.opero.com - Link Type = Link type of Link 2 - Target Network info = Information of PoA2 |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | PoS1 sends an MIH_N2N_HO_Commit request message to PoS2 via Conn5 |  |  |  |  |

# Bibliography

| [1] | "IEEE Standard for Local and metropolitan area networks— Part 21: Media Independent Handover Services," IEEE 802 IEEE 802.21, Jan. 2009. |
| --- | --- |
| [2] | "Telecommunications and Internet Protocol Harmonization Over Networks (TIPHON) Release 4;Interoperability test methods and approaches;Part 1: Generic approach to interoperability testing," ETSI TS 102 237-1, Dec. 2003. [Online]. HYPERLINK "http://portal.etsi.org/docbox/MTS/MTS/STFs/STF295/MBS/Referenced%20Documents/ts_102_237_01.pdf" http://portal.etsi.org/docbox/MTS/MTS/STFs/STF295/MBS/Referenced%20Documents/ts_102_237_01.pdf |

# Acknowledgement

Thanks to the feedback from the following working group members:

- David Cypher (NIST)

- Subir Das (Telcordia)

- Yoshihiro Ohba (Toshiba)

# Interoperable Functions Statement

This part is a revised section of M8.2 Roles and M8.3 Major Capabilities in the Annex M (PICS).

## Protocol Summary

| Protocol | MIH protocol |
| --- | --- |
| Protocol version | 1 |
| Features | MIH fragmentation |
|  | MIH acknowledgement (optional) |

## MIH entities

Table 3 lists the entities that are involved in MIH protocol exchanges. [NOTE: Figure2 in IEEE Std 802.21-2008 and its description is used. The content is not very clear.]

Table 3 MIH Entities

| Item | MIH entities | Reference | Support |
| --- | --- | --- | --- |
|  | Mobile Node (MN) | Error: Reference source not found5.4.2 |  |
|  | Point of Service (PoS) | Error: Reference source not found5.4.2 |  |
|  | Non-PoS | Error: Reference source not found5.4.2 |  |
| Comments: |  |  |  |

### Roles

Table 4 lists the different roles for a PoS. There is no separation of roles for other MIH entities, i.e., MN and IS. [NOTE: Do we need separate role for home or visiting network?]

Table 4 PoS Roles

| Item | Role | Reference | Support |
| --- | --- | --- | --- |
|  | Source PoS |  |  |
|  | Target PoS |  |  |
|  | Information Server | Error: Reference source not found5.4.3 |  |
| Comments: The role “source” and “target” apply to a PoS role regarding a handover. Since a PoS is going to take each position during its usage, the capabilities are not listed separately in the corresponding clauses. |  |  |  |

### MN and PoS Capabilities

Following are the capabilities that apply to both MN and PoS entities. [NOTE: Item and Mnemonic seems duplicating index referencing, can we just keep one?]

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| *M.8.3.4 | Capability Discovery | Error: Reference source not found6.2.3 | M | Yes [ ] No [ ] | MC4 |
| *M.8.3.5 | Registration | Error: Reference source not found6.2.4 | MC2 OR MC3.3: M | Yes [ ] No [ ] | MC5 |
| *M.8.3.1 | Event Service | Error: Reference source not found6.3 | O.2 | Yes [ ] No [ ] | MC1 |
| *M.8.3.2 | Command Service | Error: Reference source not found6.4 | O.2 | Yes [ ] No [ ] | MC2 |
| *M.8.3.3 | Information Service | Error: Reference source not found6.5 | O.2 | Yes [ ] No [ ] | MC3 |
| Comment: |  |  |  |  |  |

#### MIH discovery

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| M.8.3.4.2 | Solicitated Uni-cast capability discovery | Error: Reference source not found8.2.4.3.4 | M | Yes [ ] No [ ] | MC4.1 |
|  | Soliciated Multi-cast capability discovery | Error: Reference source not found8.2.4.3.4 | O* | Yes [ ] No [ ] | MC4.2 |
| M.8.3.4.1 | Unsolicited capability discovery | Error: Reference source not found8.2.4.3.3 | O | Yes [ ] No [ ] | MC4.3 |
| Comment: |  |  |  |  |  |

Rationale:

- In [1] 8.2.4.3.4 the “Solicitated capability discovery” is describe using either unit-cast or multicast to send an MIH_Capability_Request message. For this reason the “Soliciated capability discovery” capability is separated in to uni-cast and multicast.

- The “Unsolicited capability discovery” is the functionality of using media-specific broadcast control messages for discovering MIH enabled network entities.

#### Registration

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
|  | Registration | Error: Reference source not found6.2.4 | MC5:M | Yes [ ] No [ ] |  |
|  | Re-registration | Error: Reference source not found6.2.4 | MC5:O | Yes [ ] No [ ] N/A [ ] |  |
|  | Deregistration | Error: Reference source not found6.2.4 | MC5:M | Yes [ ] No [ ] |  |
| Comment: [Note: There’s no text describing re-registration. The term appears only in the primitive parameter description and message.] |  |  |  |  |  |

#### Command Service

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
|  | Get the status of link on a MN | Error: Reference source not found6.4.3.2.1 | MC2:O.5 | Yes [ ] No [ ] |  |
|  | Control the behavior of a set of links on a MN | Error: Reference source not found6.4.3.2.1 | MC2:O.5 | Yes [ ] No [ ] |  |
| M.8.3.6 | Mobile initiated handover | Error: Reference source not found6.4.3.2.3 | MC2:O.5 | Yes [ ] No [ ] | MC6 |
| M.8.3.7 | Network initiated handover | Error: Reference source not found6.4.3.2.4 | MC2:O.5 | Yes [ ] No [ ] | MC7 |
|  | Network resource reservation | Error: Reference source not found6.4.3.2.3, 6.4.3.2.4 | MC6 OR MC7:O | Yes [ ] No [ ] |  |
| Comment: M.8.3.10 is only relevant for the functional entity of an target PoS. |  |  |  |  |  |

Rationale:

- Define the different capabilities for command service. The actual PDUs are the implementation of the capability.

#### Event Notification

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
|  | Subscribe to events | Error: Reference source not found6.3.2.3 | MC1:M | Yes [ ] No [ ] |  |
|  | Unsubscribe to events | Error: Reference source not found6.3.2.3 | MC1:M | Yes [ ] No [ ] |  |
|  | Link thresholds report | Error: Reference source not found7.4.15.1 | MC1 and MC2:O | Yes [ ] No [ ] |  |
|  | Link notification |  |  |  |  |
| Comment: |  |  |  |  |  |

Rationale:

- The link thresholds notification capability is implemented by the MIH_Configure_Thresholds, MIH_Link_Parameter_Report in conjunction with the event subscriptions.

- Alternatively, have the MIH_Configure_Threshold, MIH_Link_Parameter_Report as a supporting group.

#### Information Service

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| M.8.3.3.1 | Binary query method | Error: Reference source not found6.5.6.2 | MC3:O.3 | Yes [ ] No [ ] N/A [ ] | MC3.1 |
| M.8.3.3.2 | RDF query method | Error: Reference source not found6.5.6.3 | MC3:O.3 | Yes [ ] No [ ] N/A [ ] | MC3.2 |
| M.8.3.3.3 | Push mode | Error: Reference source not found5.3.4 | MC5 AND MC3:O | Yes [ ] No [ ] N/A [ ] | MC3.3 |
| Comment: |  |  |  |  |  |

## Protocol Data Units

This section remains the same as in IEEE Std 802.21 clause M.8.4 with updates on the Mnemonic predicate identification [Work In Progress].

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| M.8.3.3.1 | MIH_Get_Parameters request | Error: Reference source not found | MCX | Yes [ ] No [ ] N/A [ ] | PDU8 |
| M.8.3.3.2 | MIH_Get_Parameters response | Error: Reference source not found6.5.6.3 | MC3:O.3 | Yes [ ] No [ ] N/A [ ] | PDU |
| M.8.3.3.3 | MIH_Link_Action request | Error: Reference source not found5.3.4 |  | Yes [ ] No [ ] N/A [ ] |  |
|  | MIH_Net_HO_Candidate request |  | MC6:M |  |  |
|  | MIH_Net_HO_Candidate response |  | MC6:M |  |  |