| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |
| --- | --- |
| Title | IEEE 802.21 MIH Protocol Detail Test Cases |
| DCN | 21-09-0050-01-0000 |
| Date Submitted | May 12, 2009 |
| Source(s) | Y. Alice Cheng (Telcordia), Subir Das (Telcordia), Yoshihiro Ohba (Toshiba), David Cypher (NIST) |
| Re: | IEEE 802.21 Session #32 in Montreal, QC, Canada |
| Abstract | This contribution provides a draft of interoperability test cases for implementing MIH protocol. |
| Purpose | Specific functional requirements need to be developed for the IEEE 802.21 devices to provide the necessary reliability, availability, and interoperability of communications with different operator networks. In addition, guidelines for using MIH protocol need to be developed so that vendors and operators can better understand the issues, pros, and cons of implementing IEEE 802.21 for supporting various mobility handover scenarios. |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |

# Introduction

## Scope

The primary scope of this document is to develop test cases for achieving interoperability between two or more systems that implements the MIH (Media-Independent Handover) protocol defined in IEEE Std 802.21-2008. The potential end points include wireless mobile devices and network entities described in the standard.

## Purpose

The purpose of this document is to ensure the ability of two or more MIH protocol entities to exchange information and to use the information that has been exchanged

## Target Environment

Figure 1 depicts an example target environment for IEEE 802.21 interoperability use cases. There are two operators: Operator O and Operator P. Operator O owns a WiMAX network and a Wi-Fi network with ESSID “Sandbox.” Operator P owns a 3GPP network and a Wi-Fi network with ESSID “Pandora.” There are three types of mobile node: aPhone (access to 3GPP and Wi-Fi networks), pinkBerry (access to WiMax and Wi-Fi networks), and myTop (access to WiMAX, Wi-Fi, and 3GPP networks).

Figure 1 Network Deployment Example

# MIH Tests

## Test Suites

The test cases are categorized into the following suites:

- Mobile Initiated Handover (MHO) which are test cases for handover initiated by a mobile device and assisted by the network operator;

- Network-initiated Handover (NHO) which are test cases for handover initiated by the operator and enforced by the mobile node;

- Information Service (IS) identifies test cases which are related to MIH Information Service; and

- Protocol Data Unit (PDU) contains the test cases directly derived from the PDUs (IEEE Std 802.21-2008 Clause M8.4) in PICS perfoma.

A summary of the test cases identified in this document is shown in Table 1.

Table 1 Interoperability test cases summary

|  | Test Case | Summary | PICS Addressed |
| --- | --- | --- | --- |
| Mobile Initiated Handover | MHO-1 Mobile initiated handover w/ network assistance | MN switches from one network to the other network via communication with POS. | MC6 PDU20, PDU21 (MIH_MN_HO_Commit) PDU26, PDU27 (MIH_MN_HO_Complete) |
|  | MHO-2 Mobile initiated handover w/ network resource reservation signal | MN switches from one network to the other network based with the support of resources reservation signals to the network | MC6 PDU16, PDU17 (MIH_MN_HO_Candidate_Query) PDU18, PDU19 (MIH_N2N_HO_Query_Resources) PDU20, PDU21 (MIH_MN_HO_Commit) PDU24, PDU25 (MIH_N2N_HO_Commit) PDU26, PDU27 (MIH_MN_HO_Complete) PDU28, PDU29 (MIH_N2N_HO_Complete) |
| Network Initiated Handover | NHO-1 Network initiated handover w/o network resource reservation | Service Provider/Operator requests the MN to switch from one network to the other without network resource reservation. | MC7 PDU22, PDU23 (MIH_Net_HO_Commit) PDU26, PDU27 (MIH_MN_HO_Complete) |
|  | NHO-2 Network initiated handover w/ network resource reservation signals | Service Provider/Operator requests the MN to switch from one network to the other with reserving resource signals to another PoS. | MC7 PDU14, PDU15 (MIH_Net_HO_Candidate_Query) PDU18, PDU19 (MIH_N2N_HO_Query_Resources) PDU22, PDU23 (MIH_Net_HO_Commit) PDU24, PDU25 (MIH_N2N_HO_Commit) PDU26, PDU27 (MIH_MN_HO_Complete) PDU28, PDU29 (MIH_N2N_HO_Complete) |
|  | NHO-3 Network initiated handover w/ network resource reservation signals within one PoS | Service Provider/Operator requests the MN to switch from one network to the other with reserving resource signals. | MC7 PDU14, PDU15 (MIH_Net_HO_Candidate_Query) PDU22, PDU23 (MIH_Net_HO_Commit) PDU26, PDU27 (MIH_MN_HO_Complete) |
| Information Service | IS-1 Query available networks | Discover the available network types based on the MN’s current location (via geo-location or its PoA’s location) using binary/RDF query. | MC3 (MC3.1 or MC3.2) PDU30, PDU31 (MIH_Get_Information) |
|  | IS-2 IS push mode | After MIH registration, the MN receives push information from the network. | MC3.3 [OPT] PDU32 (MIH_Push_Information) |
| Protocol Data Unit | PDU35-36 MIH Register | MIH_Register request and response message exchange. | PDU35, PDU36 (MIH_Register) |
|  | PDU37-38 MIH Deregister | MIH_Deregister request and response message exchange. | PDU37, PDU38 (MIH_Deregister) |
|  | PDU39-40 MIH Event Subscribe | MIH_Event_Subscribe request and response message exchange [NOTE: this is the preconditions for all the event notification test cases.] | PDU39, PDU40 (MIH_Event_Subscribe) |
|  | PDU |  |  |
|  | [Work In Progress] |  |  |

## Abstract Architecture

Following is a general architecture.

MN1

PoS1

PoS2

Link 1

Link 2

Com 5

MIHF ID=pos1.opero.com

MIHF ID=pos2.operp.com

MIHF ID=mn1.opero.com

IS2

IS1

MIHF ID=is1.opero.com

Com 6

Com 7

MIHF ID=is2.operp.com

PoA2

PoA1

Com 3

Figure 2 Abstract architecture

Com 4

The architecture in Error: Reference source not found contains several MIH devices: one MN (mobile node), two PoS (Point of Service), and two IS (Information Service Server). These devices understand MIH protocol and each have their own unique MIHF ID. The two PoAs (Point of Attachments, POA1 and PoA2) are used for illustrating the network connectivity status and are not part of the testing units. In this example, the MN is attached to the OperO network (operator O) via Link1 and to the OperP network (operator P) via Link2. Com#, where # is a number, indicates that there is a communication between the two end points using UDP as its transport mechanism.

The following table describes the potential access network technology for Link1 and Link2.

Table 2 Access technology examples for Link1 and Link2

| Operator | Link1 | Link 2 |
| --- | --- | --- |
| Same Operator | 802.11 | 802.16e |
| Same Operator | 802.11 | EVDO |
| Different Operator | 3GPP | 802.11 |

Following are different test architectures that are applied to different test cases.

MN1

PoS1

Link 1

MIHF ID=pos1.opero.com

MIHF ID=mn1.opero.com

PoA1

Com 3

Figure 2 Test Architecture 1

Figure 2  contains only two MIH devices, i.e., MN1 and PoS1. The MN1 is attached to PoA1 and communicates with PoS1 via Com3.

PoS1

MIHF ID=pos1.opero.com

Com 5

PoS2

MIHF ID=pos2.operp.com

Figure 3 Test Architecture 2

Figure 3 covers test cases for the communication between two PoS.

# Mobile-initiated handover test suite

The purpose of this test suite is to verify that the mobile node can initiate and complete a handover-related sequence of operations from one network to the other network using MIH protocol with network entities.

[NOTE: Make-before-break/break-before-make? Do we have performance requirement?]

## MHO-1 Mobile-initiated handover w/ network assistance

[Work In Progress]

## MHO-2 Mobile-initiated handover w/ network resource reservation signals

A mobile node is attached to PoA1 and discovers the network PoS via pre-configuration. The MN then decides to perform a handover with the assistance from the network within the same operator. Though there are MIH message exchanges for QoS resource information, the actual network resource reservation is performed by applying the resource reservation techniques for the specific access technology. Figure 4 shows the devices, links, and connections that are used for this test. This test case describes a make-before-break handover scenario.

MN

PoS1

PoA2

Link 1

Link 2

MIHF ID=pos1.opero.com

MIHF ID=mn.opero.com

Com4

PoA1

Com3

PoS2

MIHF ID=pos2.opero.com

Com5

Figure 4 MHO-1 Test Architecture

### Sequence

MN was registered to PoS1 through Com3 via Link1. When test starts, MN provides a list of candidate networks and queries PoS1 for recommendation. The PoS1 returns PoA2 as MN’s preferred point of attachment.  MN then establishes Link 2. PoS1 communicates with PoS2 through Com5 for network resource query and reservation signals. Once attached to PoA2, the MN re-registers to PoS2 through Com4 via Link2.

See clause C.1 in IEEE Std 802.21-2008.

### Test Case

| Test: | MHO-2 | Selection Criteria: | Mandatory | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Mobile-initiated handover with network resource reservation signals. |  |  |  |
| Purpose: |  | To verify that - The mobile node can initiate and complete a handover using MIH protocol message exchange - The network PoS reserve network resources to assist handover using MIH protocol message exchange |  |  |  |
| Pre-Condition: |  | Use Figure 4 MHO-1 Test Architecture - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - PoS1 is aware of the existence of PoS2 and recognizes that handovers to specific the point of attachment (i.e., PoA2) is served by PoS2. - MN should provide PoA2 in its candidate list when connected to PoA1. - PoS1 is configured to return PoA2 (if exists in the MN candidate list) as the best candidate for the MN. Network resource assumptions: - PoS2 is able to reserve the QoS resource specified in the parameter QOS_LIST. - PoS1 is able to release the QoS resource at the end of the test. |  |  |  |
| Parameters |  | - QOS_LIST: a list of QoS requirements for this test case. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends MIH_MN_HO_Query request message to PoS1 through Com3 via Link 1 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: the MN current serving link (Link1), the list of candidate PoAs (including PoA2) and QOS_LIST. |  |  | Yes | No |
|  | PoS1 sends MIH_N2N_HO_Query_Resources request message to PoS2 via Com5 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: a list of QoS requirements and a list of PoAs that are candidates (including PoA2)? |  |  | Yes | No |
|  | PoS2 responds with MIH_N2N_HO_Query_Resources response message to PoS1 via Com5 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: status (0, success), resource available (0, available), and the resource query result for the candidates? |  |  | Yes | No |
|  | PoS1 responds with MIH_MN_HO_Query response message to MN through Com3 |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: a list of candidates (PoA2 is the first item). |  |  | Yes | No |
|  | MN sends MIH_MN_HO_Commit request message to PoS1 through Com3 via Link 1 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: the target link type (Link2) and the target network information (PoA2)? |  |  | Yes | No |
|  | PoS1 sends MIH_N2N_HO_Commit request message to PoS2 through Com5 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: MIHF ID of the MN, target MN link identifier (Link2), target PoA (PoA2), and requested resource set? The request resource set contains the following information: QoS requirements, container (null), and HO_Cause (0, explicit disconnect). |  |  | Yes | No |
|  | PoS2 reserves the network resource and responds with MIH_N2N_HO_Commit response message to PoS1 through Com5 |  |  |  |  |
|  | Is the QoS resource reserved? |  |  | Yes | No |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: status (0, success), the target link type (Link2) and the target network information (PoA2)? |  |  | Yes | No |
| - P | PoS1 responds with MIH_MN_HO_Commit response message to MN through Com3 |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: status (0, success), link type (Link2), and the target network information (PoA2)? |  |  | Yes | No |
|  | MN attaches to PoA2 and establishes Link2 |  |  |  |  |
|  | Is the MN attached to PoA2? |  |  | Yes | No |
|  | MN establishes Com4 and registers to PoS2 |  |  |  |  |
|  | Is MN registered to PoS2? (Reference Test PDU35-36) |  |  | Yes | No |
|  | MN sends MIH_MN_HO_Complete request message to PoS2 through Com4 via Link2 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: the source link identifier (Link1), the target link identifier (Link2), and handover result (0, handover success)? |  |  | Yes | No |
|  | PoS2 sends MIH_N2N_HO_Complete request message to PoS1 via Com5 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: MN MIHF ID (mn.opero.com), source link identifier (Link1), target link identifier (Link2), and handover result (0, handover success)? |  |  | Yes | No |
|  | PoS1 responds with MIH_N2N_HO_Complete response message to PoS2 via Com5. |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: status (0, success), MN MIHF ID (mn.opero.com), source link identifier (Link1), target link identifier (Link2), and resource retention status (FALSE, release resource)? |  |  | Yes | No |
|  | PoS2 responds with MIH_MN_HO_Complete response message to MN through Com4 via Link2 |  |  |  |  |
|  | Is the message received by MN |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: status (0, success), source link identifier (Link1), and target link identifier (Link2). |  |  | Yes | No |
|  | MN deregistered from PoS1. (Reference Test PDU37-38) [NOTE: Is this mandatory?] |  |  | Yes | No |
| - M | MN disconnects from PoA1 and break Link1 [NOTE: make-before-break] |  |  |  |  |
|  | Is MN disconnected from PoA1? |  |  | Yes | No |

# Network-initiated handover

## NHO-1 Network-initiated handover w/o network resource reservation

[Work In Progress]

## NHO-2 Network-initiated handover w network resource reservation signals

A mobile node is attached to PoA1 and registered to PoS1. Operator O decides to request the MN to perform a handover to PoS2 with the assistance from the network (reserve network resources) within the same operator. Figure 4 shows the devices, links, and connections that are used for this test.

### Sequence

[Work In Progress]

See clause C.2 in IEEE Std 802.21-2008.

### Test Case

| Test: | NHO-2 | Selection Criteria: | Mandatory | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Network-initiated handover with network resource reservation signals. |  |  |  |
| Purpose: |  | To verify that - PoS1 can initiate and complete a handover using MIH protocol message exchange - MN successfully completes a handover using MIH protocol message exchange. - The network PoS reserve network resources to assist handover using MIH protocol message exchange |  |  |  |
| Pre-Condition: |  | Use Figure 4 MHO-1 Test Architecture - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - PoS1 is aware of the existence of PoS2 and recognizes that handovers to specific points of attachment (e.g., PoA2) are served by PoS2. - Based on the network preference, PoS1 is configured to return PoA2. Network resource assumptions: - PoS2 is able to reserve the QoS resource specified in the parameter QOS_LIST. - PoS1 is able to release the QoS resource at the end of the test. |  |  |  |
| Parameter |  | - QOS_LIST: a list of QoS requirements for this test case. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | PoS1 sends MIH_Net_HO_Query request message to MN through Com3 via Link 1 |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: [NOTE: Details WIP.]? |  |  | Yes | No |
|  | MN responds with MIH_Net_HO_Query response message to PoS1 through Com3 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: [NOTE: Details WIP.] |  |  | Yes | No |
|  | PoS1 sends MIH_N2N_HO_Query_Resources request message to PoS2 via Com5 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: parameter QOS_LIST and a list of PoAs that are candidates (including PoA2)? |  |  | Yes | No |
|  | PoS2 responds with MIH_N2N_HO_Query_Resources response message to PoS1 via Com5 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: status (0, success), resource available(0, available), and the resource query result for the candidates? |  |  | Yes | No |
|  | PoS1 sends MIH_N2N_HO_Commit request message to PoS2 through Com5 [Note: step 13-19 can be merged to a specific PDU test] |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: MIHF ID of the MN, target MN link identifier (Link2), target PoA (PoA2), and requested resource set? The request resource set contains the following information: QoS requirements, container (null), and HO_Cause (0, explicit disconnect). |  |  | Yes | No |
|  | PoS2 reserves the network resource and responds with MIH_N2N_HO_Commit response message to PoS1 through Com5 |  |  |  |  |
|  | Is the QoS resource reserved? |  |  | Yes | No |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: status (0, success), the target link type (Link2) and the target network information (PoA2)? |  |  | Yes | No |
| - P | PoS1 sends MIH_Net_HO_Commit request message to MN through Com3 |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: [NOTE: Details TBD]? |  |  | Yes | No |
|  | MN attaches to PoA2 and establishes Link2 |  |  |  |  |
|  | Is the MN attached to PoA2? |  |  | Yes | No |
|  | MN establishes Com4 and registers to PoS2 |  |  |  |  |
|  | Is MN registered to PoS2? (Reference Test PDU35-36) |  |  | Yes | No |
|  | MN responds with MIH_Net_HO_Commit response message to PoS1 through Com3 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the information and obtain the following information: [NOTE: Details TBD] |  |  |  |  |
|  | MN sends MIH_MN_HO_Complete request message to PoS2 through Com4 via Link2 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: the source link identifier (Link1), the target link identifier (Link2), and handover result (0, handover success)? |  |  | Yes | No |
|  | PoS2 sends MIH_N2N_HO_Complete request message to PoS1 via Com5 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information: MN MIHF ID (mn.opero.com), source link identifier (Link1), target link identifier (Link2), and handover result (0, handover success)? |  |  | Yes | No |
|  | PoS1 responds with MIH_N2N_HO_Complete response message to PoS2 via Com5. |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Can PoS2 parse the message and obtain the following information: status (0, success), MN MIHF ID (mn.opero.com), source link identifier (Link1), target link identifier (Link2), and resource retention status (FALSE, release resource)? |  |  | Yes | No |
|  | PoS2 responds with MIH_MN_HO_Complete response message to MN through Com4 via Link2 |  |  |  |  |
|  | Is the message received by MN |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: status (0, success), source link identifier (Link1), and target link identifier (Link2). |  |  | Yes | No |
|  | MN deregistered from PoS1 (Reference Test PDU37-38) [NOTE: Is this mandatory?] |  |  | Yes | No |
| - M | MN disconnects from PoA1 and break Link1 [NOTE: make-before-break] |  |  |  |  |
|  | Is MN disconnected from PoA1? |  |  | Yes | No |

# Information Service Test Cases

This section contains the detail description for MIH Information Service related test cases.

[Work In Progress]

## IS-1 Query available networks

MN1

PoS1

Link 1

MIHF ID=pos1.opero.com

MIHF ID=mn1.opero.com

IS1

MIHF ID=is1.opero.com

Conn 6

PoA1

Conn 3

Figure 6 IS-1 Test Architecture

### Sequence

The MN is connected to the network. It queries the IS server for a suggested access network connection by providing a query message.

### Test Case

| Test: | IS-1 | Selection Criteria: |  | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Query available access technologies |  |  |  |
| Purpose: |  | To verify that the Information Server is providing the correct information. - IS is able accept binary query or RDF query. - MN is able to successfully connect to an access technology without scanning. - IS returned the expected value for this test case defined in QUERY_RESPONSE. |  |  |  |
| Pre-Condition: |  | Use Error: Reference source not found - MN has at least two access technology interfaces. - IS contains information of all the access points and its location for the two access technology the MN supports. |  |  |  |
| Parameters: |  | - QUERY_TYPE: The type of query that is supported by the IS - QUERY: The query message which is defined in a separate table. - QUERY_RESPONSE: The expected results of the query. This response should contain enough information for the MN to connect to an access network. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN registers to PoS1 |  |  |  |  |
|  | Is MN registered to PoS2? (Reference Test PDU35-36) |  |  | YES | NO |
|  | MN sends QUERY of the QUERY_TYPE to IS1 |  |  |  |  |
|  | Is the corresponding test passed with the QUERY and QUERY_RESPONSE parameters required for this test? (Reference Test PDU30-B for QUERY_TYPE is binary and Test PDU30-R for QUERY_TYPE is RDF) |  |  | YES | NO |
|  | IS1 responds the QUERY_RESPONSE to MN |  |  |  |  |
|  | Is MIH_GET_INFORMATION response message received by MN? |  |  | YES | NO |
|  | Can MN parse the message and obtain the following information: QUERY_RESPONSE (based on the query type) |  |  | YES | NO |

## IS-2 IS push mode

# Common Test Cases

## MIH Capability Discovery

[Work In Progress]

## MIH Register

| Test: | PDU35-36 | Selection Criteria: | Mandatory | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH register request and response |  |  |  |
| Purpose: |  | To verify that - MN sends a valid MIH_Register request message to PoS1 - PoS1 receives the message and registers the MN with infinite validity period - MN is registered to PoS1. |  |  |  |
| Pre-Condition: |  | Use Figure 2 Test Architecture 1 - MN is attached to PoA1. - MN has discovered the PoS1 via pre-configuration |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an MIH_Register request message to PoS1 via Com3 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information from the MN: MIHF_ID, Link ID list (Link1 and Link2 not connected), and registration code is 0? |  |  | Yes | No |
|  | PoS1 responds with a successful MIH_Register response message |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: status of the operation is success and the valid time interval is 0. |  |  | Yes | No |
| Observations: |  |  |  |  |  |

## MIH Deregister

| Test: | PDU37-38 | Selection Criteria: | Mandatory | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH deregister request and response |  |  |  |
| Purpose: |  | To verify that - MN sends a valid MIH_DeRegister request message to PoS1 - PoS1 receives the message and deregisters the MN. |  |  |  |
| Pre-Condition: |  | Use Figure 2 Test Architecture 1 - MIH Register test case has passed. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an MIH_DeRegister request message to PoS1 via Com3 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Can PoS1 parse the message and obtain the following information from the MN: MN MIHF_ID? |  |  | Yes | No |
|  | PoS1 responds with a successful MIH_DeRegister response message |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Can MN parse the message and obtain the following information: status of the operation is success. |  |  | Yes | No |

## MIH Event Subscribe

[Work In Progress] (PDU39 PDU40)

## MIH N2N HO Commit Request

[Work In Progress]

| Test: | PDU24 | Selection Criteria: |  | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH N2N HO Commit request |  |  |  |
| Purpose: |  | To verify that when handover from PoS1 to PoS2 - PoS2 receives MIH_N2N_HO_Commit request message from PoS1 - PoS2 can parse MIH_N2N_HO_Commit request message with the following parameters |  |  |  |
| Pre-Condition: |  | Use Figure 3 Test Architecture 2 - PoS1, PoS2 relationship [Work in Progress] - Unless specified, following are the default parameters to verify against - Source MIH Identifier = pos1.opero.com - Destination MIH Identifier = pos2.opero.com - Link Type = Link type of Link 2 - Target Network info = Information of PoA2 |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | PoS1 sends an MIH_N2N_HO_Commit request message to PoS2 via Com5 |  |  |  |  |

## MIH Get Information

IEEE 802.21 supports both binary query and RDF query. The corresponding PDU30 tests are split into two types of test cases.

### Binary Query

| Test: | PDU30-31_B | Selection Criteria: | Mandatory | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH Get Information using binary query |  |  |  |
| Purpose: |  | To verify that a MIH user or MIHF can communicate with IS server using RDF query. |  |  |  |
| Pre-Condition: |  | Use Figure 2 Test Architecture 1 - MIH Register test case has passed. |  |  |  |
| Parameters: |  | - QUERY: The query message which is defined in a separate table. - QUERY_RESPONSE: The expected results of the query. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  |  |  |  | Yes | No |
|  |  |  |  | Yes | No |
|  |  |  |  | Yes | No |
|  |  |  |  | Yes | No |

### RDF Query

| Test: | PDU30-31_R | Selection Criteria: | Mandatory | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH Get Information using RDF query |  |  |  |
| Purpose: |  | To verify that a MIH user or MIHF can communicate with IS server using RDF query. |  |  |  |
| Pre-Condition: |  | Use Figure 2 Test Architecture 1 - MIH Register test case has passed. |  |  |  |
| Parameters: |  | - QUERY: The query message which is defined in a separate table. - QUERY_RESPONSE: The expected results of the query. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  |  |  |  | Yes | No |
|  |  |  |  | Yes | No |
|  |  |  |  | Yes | No |
|  |  |  |  | Yes | No |

# Bibliography

| [1] | "IEEE Standard for Local and metropolitan area networks— Part 21: Media Independent Handover Services," IEEE 802 IEEE 802.21, Jan. 2009. |
| --- | --- |
| [2] | "Telecommunications and Internet Protocol Harmonization Over Networks (TIPHON) Release 4;Interoperability test methods and approaches;Part 1: Generic approach to interoperability testing," ETSI TS 102 237-1, Dec. 2003. [Online]. HYPERLINK "http://portal.etsi.org/docbox/MTS/MTS/STFs/STF295/MBS/Referenced%20Documents/ts_102_237_01.pdf" |

- Mapping the FMCA use cases to the test cases

  - Introduction

The test cases can be combined to match to general use cases. This section provides an example of such mappings to the FMCA use cases provided in http://mentor.ieee.org/802.21/dcn/09/21-09-0061-00-0000-fmca-use-cases.ppt

[Note: Add some description of how it may use the Interoperable Function Statements.]

  - Additional case

FMCA use cases requires specific MIH functionalities that does not require any network communictions. In order to address these requirements, Table 3 includes additional conformance test cases.

Table 3 Additional test cases for FMCA

| Test Case | Description | PICS-Addressed |
| --- | --- | --- |
| MN-1 Mobile device handover | MN switches from one network to the other network without communicating with a POS. | N/A |

    - MN-1 Mobile device handover

Mobile device is able to perform a handover from Link 1 to Link 2.

  - Network controlled access technology selection

    - Introduction

The network controls the device to perform a handover.

    - Sequence

- MHO-1 The MN handover to the network based on the IS result.

    - Test Case

| Test: | FMCA1 | Selection Criteria: |  | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Network controlled access technology selection |  |  |  |
| Purpose: |  | To verify that MN switches to the specific access technology based on the PoA provided by the network. - Network is able to provide access network information to MN without MN turning on the specific interface. - MN is able to successfully connect to an access technology without scanning. |  |  |  |
| Pre-Condition: |  | Use - MN has at least two access technology interfaces. - IS contains information of all the access points and its location for the two access technology the MN supports. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |

  - 3G to WLAN Handover

  - Per-flow

MN handover based on application type

- IS-1 The MN queries that IS server for the point of attachment. The network operator provides a PoA  corresponding point of attachment for network handover.

  - QUERY: MN provides the current geo-location and request for any alternative access networks.

  - QUERY_RESPONSE: Network respond the network id (e.g. ESSID for Wi-Fi) or the actual point of attachment (e.g. BSSID for Wi-Fi) that the MN should connect to in the order of the network preference.

  - Resource limited backhaul

  - Seamless handover and service adaptation

  - Adaptive Handover for Application

  - Multiple Simultaneous Sessions (Data, Voice & Mobility)

  - Summary

Following tale provides a summary of the mapping from the use cases to a sequence of test cases provided in this document.

| Use Case | Title | Summary Sequence |
| --- | --- | --- |
| FMCA-1a | Network controlled access technology selection | - IS-1: MN query for access technology network information. - MHO-1: MN handover to the query results provided by the IS. |
| FMCA-1b | Network controlled access technology selection | Precondition: Network contains the location information of the MN - IS-1: MN query for access technology network information. - NHO-1: The PoS commands the MN to handover to connect a specific PoA based on the location provided in the IS query from MN. |
| FMCA-2 | 3G to WLAN Handover (device-controlled) | - IS-1: MN query for WLAN radio via the 3G network link. - MN-1: MN attaches to the WLAN provided by the IS. |
| FMCA-3 | Per-flow | Precondition: PoS and IS are collocated in the same entity. - IS-1: The MN queries IS for the WLAN access points which are reachable from the MN location. - PDU12-13: The PoS commands the MN to turn on WLAN interface - NHO-1: The PoS commands the MN to handover to connect a specific PoA. However, keep the 3G connection. Voice is over the 3G and video is over the WLAN. - When video download completed, MN performs MHO-1 to handover from WLAN to 3G. |
| FMCA-4 | Seamless handover and service adaptation | Precondition: A PoS gets network - NHO-2: Based on the network situation, the PoS request the device to handover to a different network. Notes: Device switching with same session is not supported by 802.21. Application switching from Video conference to Audio conference is a software application and is not supported by 802.21 |
| FMCA-5 | Adaptive Handover for Application | - MN-1: Based on the application type, the device connects to different network. |
| FMCA-6 | Multiple Simultaneous Sessions (Data, Voice & Mobility) | This is session continuity. Requires MIH to work with a mobility protocol or a session continuity protocol. |

- Interoperable Functions Statement

This part is a revised section of M8.2 Roles and M8.3 Major Capabilities in the Annex M (PICS).

  - Protocol Summary

| Protocol | MIH protocol |
| --- | --- |
| Protocol version | 1 |
| Features | MIH fragmentation |
|  | MIH acknowledgement (optional) |

  - MIH entities

Table 4 lists the entities that are involved in MIH protocol exchanges. [NOTE: Figure2 in IEEE Std 802.21-2008 and its description is used. Not sure if we need to have “non-PoS” as one of the MIH entities in this section.]

Table 4 MIH Entities

| Item | MIH entities | Reference | Support |
| --- | --- | --- | --- |
|  | Mobile Node (MN) | Error: Reference source not found5.4.2 |  |
|  | Point of Service (PoS) | Error: Reference source not found5.4.2 |  |
|  | Non-PoS | Error: Reference source not found5.4.2 |  |
| Comments: |  |  |  |

    - Roles

Table 5 lists the different roles for a PoS. There is no separation of roles for other MIH entities, i.e., MN.

Table 5 PoS Roles

| Item | Role | Reference | Support |
| --- | --- | --- | --- |
|  | Source PoS |  |  |
|  | Target PoS |  |  |
|  | Information Server | Error: Reference source not found5.4.3 |  |
| Comments: The role “source” and “target” apply to a PoS role regarding a handover. Since a PoS is going to take each position during its usage, the capabilities are not listed separately in the corresponding clauses. |  |  |  |

    - MN and PoS Capabilities

Following are the capabilities that apply to both MN and PoS entities. [NOTE: Item and Mnemonic seems duplicating index referencing, can we just keep one? Suggest just keep Mnemonic]

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| *M.8.3.4 | Capability Discovery | Error: Reference source not found6.2.3 | M | Yes [ ] No [ ] | MC4 |
| *M.8.3.5 | Registration | Error: Reference source not found6.2.4 | MC2 OR MC3.3: M | Yes [ ] No [ ] | MC5 |
| *M.8.3.1 | Event Service | Error: Reference source not found6.3 | O.2 | Yes [ ] No [ ] | MC1 |
| *M.8.3.2 | Command Service | Error: Reference source not found6.4 | O.2 | Yes [ ] No [ ] | MC2 |
| *M.8.3.3 | Information Service | Error: Reference source not found6.5 | O.2 | Yes [ ] No [ ] | MC3 |
| Comment: |  |  |  |  |  |

    - MIH discovery

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| M.8.3.4.2 | Solicitated Uni-cast capability discovery | Error: Reference source not found8.2.4.3.4 | M | Yes [ ] No [ ] | MC4.1 |
|  | Soliciated Multi-cast capability discovery | Error: Reference source not found8.2.4.3.4 | O* | Yes [ ] No [ ] | MC4.2 |
| M.8.3.4.1 | Unsolicited capability discovery | Error: Reference source not found8.2.4.3.3 | O | Yes [ ] No [ ] | MC4.3 |
| Comment: |  |  |  |  |  |

Rationale:

- In [1] 8.2.4.3.4 the “Solicitated capability discovery” is described using either unicast or multicast to send an MIH_Capability_Request message. For this reason the “Soliciated capability discovery” capability is separated in to uni-cast and multicast.

- The “Unsolicited capability discovery” is the functionality of using media-specific broadcast control messages for discovering MIH enabled network entities.

    - Registration

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
|  | Registration | Error: Reference source not found6.2.4 | MC5:M | Yes [ ] No [ ] |  |
|  | Re-registration | Error: Reference source not found6.2.4 | MC5:O | Yes [ ] No [ ] N/A [ ] |  |
|  | Deregistration | Error: Reference source not found6.2.4 | MC5:M | Yes [ ] No [ ] |  |
| Comment: [Note: There’s no text describing re-registration. The term appears only in the primitive parameter description and message.] |  |  |  |  |  |

    - Command Service

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
|  | Get the status of link on a MN | Error: Reference source not found6.4.3.2.1 | MC2:O.5 | Yes [ ] No [ ] |  |
|  | Control the behavior of a set of links on a MN | Error: Reference source not found6.4.3.2.1 | MC2:O.5 | Yes [ ] No [ ] |  |
| M.8.3.6 | Mobile initiated handover | Error: Reference source not found6.4.3.2.3 | MC2:O.5 | Yes [ ] No [ ] | MC6 |
| M.8.3.7 | Network initiated handover | Error: Reference source not found6.4.3.2.4 | MC2:O.5 | Yes [ ] No [ ] | MC7 |
|  | Network resource reservation | Error: Reference source not found6.4.3.2.3, 6.4.3.2.4 | MC6 OR MC7:O | Yes [ ] No [ ] |  |
| Comment: M.8.3.10 is only relevant for the functional entity of an target PoS. |  |  |  |  |  |

Rationale:

- Define the different capabilities for command service. The actual PDUs are the implementation of the capability.

    - Event Notification

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
|  | Subscribe to events | Error: Reference source not found6.3.2.3 | MC1:M | Yes [ ] No [ ] |  |
|  | Unsubscribe to events | Error: Reference source not found6.3.2.3 | MC1:M | Yes [ ] No [ ] |  |
|  | Link thresholds report | Error: Reference source not found7.4.15.1 | MC1 and MC2:O | Yes [ ] No [ ] |  |
|  | Link notification |  |  |  |  |
| Comment: |  |  |  |  |  |

Rationale:

- The link thresholds notification capability is implemented by the MIH_Configure_Thresholds, MIH_Link_Parameter_Report in conjunction with the event subscriptions.

- Alternatively, have the MIH_Configure_Threshold, MIH_Link_Parameter_Report as a supporting group.

    - Information Service

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| M.8.3.3.1 | Binary query method | Error: Reference source not found6.5.6.2 | MC3:O.3 | Yes [ ] No [ ] N/A [ ] | MC3.1 |
| M.8.3.3.2 | RDF query method | Error: Reference source not found6.5.6.3 | MC3:O.3 | Yes [ ] No [ ] N/A [ ] | MC3.2 |
| M.8.3.3.3 | Push mode | Error: Reference source not found5.3.4 | MC5 AND MC3:O | Yes [ ] No [ ] N/A [ ] | MC3.3 |
| Comment: |  |  |  |  |  |

  - Protocol Data Units

This section remains the same as in IEEE Std 802.21 clause M.8.4 with updates on the Mnemonic predicate identification [Work In Progress].

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| M.8.3.3.1 | MIH_Get_Parameters request | Error: Reference source not found | MCX | Yes [ ] No [ ] N/A [ ] | PDU8 |
| M.8.3.3.2 | MIH_Get_Parameters response | Error: Reference source not found6.5.6.3 | MC3:O.3 | Yes [ ] No [ ] N/A [ ] | PDU |
| M.8.3.3.3 | MIH_Link_Action request | Error: Reference source not found5.3.4 |  | Yes [ ] No [ ] N/A [ ] |  |
|  | MIH_Net_HO_Candidate request |  | MC6:M |  |  |
|  | MIH_Net_HO_Candidate response |  | MC6:M |  |  |