| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |
| --- | --- |
| Title | IEEE 802.21 MIH Protocol Test Cases |
| DCN | 21-09-0050-05-0000 |
| Date Submitted | August 16, 2009 |
| Source(s) | Y. Alice Cheng (Telcordia), Subir Das (Telcordia), Yoshihiro Ohba (Toshiba), Juan Carlos Zuniga (InterDigital) |
| Re: | IEEE 802.21 Session #33 in San Francisco, CA |
| Abstract | This contribution provides a draft of interoperability test cases for implementing MIH protocol. |
| Purpose | Specific functional requirements need to be developed for the IEEE 802.21 devices to provide the necessary reliability, availability, and interoperability of communications with different operator networks. In addition, guidelines for using MIH protocol need to be developed so that vendors and operators can better understand the issues, pros, and cons of implementing IEEE 802.21 for supporting various mobility handover scenarios. |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |

# Introduction

## Scope

The primary scope of this document is to develop test cases for achieving interoperability between two or more systems that implement the MIH (Media-Independent Handover) protocol defined in IEEE Std 802.21-2008. The potential end points include wireless mobile devices and network entities described in the standard. Performance requirements on the test cases are out of the scope of this document.

## Purpose

The purpose of this document is to ensure the ability of two or more MIH protocol entities to exchange information and to use the information that has been exchanged.

# MIH Tests

## Test Suites

The test cases are categorized into the following suites:

- Mobile Initiated Handover (MHO) which are test cases for handover initiated by a mobile device and assisted by the network operator;

- Network-initiated Handover (NHO) which are test cases for handover initiated by the operator and enforced by the mobile node;

- Information Service (IS) identifies test cases which are related to MIH Information Service; and

- Protocol Data Unit (PDU) contains the test cases directly derived from the PDUs (IEEE Std 802.21-2008 Clause M8.4) in PICS perfoma.

A summary of the test cases identified in this document is shown in Table 1.

Table 1 Interoperability test cases summary

|  | Test Case | Summary | PICS Addressed |
| --- | --- | --- | --- |

| Mobile Initiated Handover | MHO-1a Mobile initiated handover w/ network assistance and retaining the connection to the old network after handover | MN switches from one network to the other network via communication with POS. After a handover, MN retains the connection to the old network. |  | MC6 PDU20, PDU21 (MIH_MN_HO_Commit) PDU35,PDU36 (MIH_Register) |
| --- | --- | --- | --- | --- |
|  | MHO-1b Mobile initiated handover w/ network assistance and deleting the connection to the old network after handover | MN switches from one network to the other network via communication with POS. After a handover, MN deletes the connection to the old network. |  | MC6 PDU20, PDU21 (MIH_MN_HO_Commit) PDU35,PDU36 (MIH_Register) |
|  | MHO-2a Mobile initiated handover w/ network resource reservation signal and retaining the connection to the old network after handover | MN switches from one network to the other network with the support of resources reservation signals to the network. After a handover, MN retains the connection to the old network. |  | MC6 PDU16, PDU17 (MIH_MN_HO_Candidate_Query) PDU18, PDU19 (MIH_N2N_HO_Query_Resources) PDU20, PDU21 (MIH_MN_HO_Commit) PDU24, PDU25 (MIH_N2N_HO_Commit) PDU35,PDU36 (MIH_Register) |
|  | MHO-2b Mobile initiated handover w/ network resource reservation signal and deleting the connection to the old network after handover | MN switches from one network to the other network with the support of resources reservation signals to the network. After a handover, MN deletes the connection to the old network. |  | MC6 PDU16, PDU17 (MIH_MN_HO_Candidate_Query) PDU18, PDU19 (MIH_N2N_HO_Query_Resources) PDU20, PDU21 (MIH_MN_HO_Commit) PDU24, PDU25 (MIH_N2N_HO_Commit) PDU35,PDU36 (MIH_Register) PDU37, PDU-38 (MIH_DeRegister) |
| Network Initiated Handover | NHO-1a Network initiated handover w/o network resource reservation and retaining the connection to the old network after handover | Service Provider/Operator requests the MN to switch from one network to the other without network resource query or reservation. After a handover, MN retains the connection to the old network. |  | MC7 PDU22, PDU23 (MIH_Net_HO_Commit) PDU35,PDU36 (MIH_Register) |
|  | NHO-1b Network initiated handover w/o network resource reservation and deleting the connection to the old network after handover | Service Provider/Operator requests the MN to switch from one network to the other without network resource query or reservation. After a handover, MN deletes the connection to the old network. |  | MC7 PDU22, PDU23 (MIH_Net_HO_Commit) PDU35,PDU36 (MIH_Register) |
|  | NHO-2a Network initiated handover w/ network resource reservation signals and retaining the connection to the old network after handover | Service Provider/Operator requests the MN to switch from one network to the other with reserving resource signals to another PoS. After a handover, MN retains the connection to the old network. |  | MC7 PDU14, PDU15 (MIH_Net_HO_Candidate_Query) PDU18, PDU19 (MIH_N2N_HO_Query_Resources) PDU22, PDU23 (MIH_Net_HO_Commit) PDU24, PDU25 (MIH_N2N_HO_Commit) PDU35,PDU36 (MIH_Register) |
|  | NHO-2b Network initiated handover w/ network resource reservation signals and deleting the connection to the old network after handover | Service Provider/Operator requests the MN to switch from one network to the other with reserving resource signals to another PoS. After a handover, MN deletes the connection to the old network. |  | MC7 PDU14, PDU15 (MIH_Net_HO_Candidate_Query) PDU18, PDU19 (MIH_N2N_HO_Query_Resources) PDU22, PDU23 (MIH_Net_HO_Commit) PDU24, PDU25 (MIH_N2N_HO_Commit) PDU35,PDU36 (MIH_Register) PDU37, PDU-38 (MIH_DeRegister) |
|  | NHO-3a Network initiated handover w/ candidate query w/o source reservation and retaining the connection to the old network after handover | Service Provider/Operator requests the MN to switch from one network to the other with candidate query to MN and without reserving resources. After a handover, MN retains the connection to the old network. |  | MC7 PDU14, PDU15 (MIH_Net_HO_Candidate_Query) PDU22, PDU23 (MIH_Net_HO_Commit) PDU35,PDU36 (MIH_Register) |
|  | NHO-3b Network initiated handover w/ candidate query w/o source reservation and deleting the connection to the old network after handover | Service Provider/Operator requests the MN to switch from one network to the other with candidate query to MN and without reserving resources. After a handover, MN deletes the connection to the old network. |  | MC7 PDU14, PDU15 (MIH_Net_HO_Candidate_Query) PDU22, PDU23 (MIH_Net_HO_Commit) PDU35,PDU36 (MIH_Register) |
| Information Service | IS-1a Query available networks without registration | Discover the available network types based on the MN’s current location (via geo-location or its PoA’s location) using binary/RDF query. |  | MC3 (MC3.1 or MC3.2) PDU30, PDU31 (MIH_Get_Information) |
|  | IS-1b Query available networks with registration | Discover the available network types based on the MN’s current location (via geo-location or its PoA’s location) using binary/RDF query. MIH registration is performed before sending a query. |  | MC3 (MC3.1 or MC3.2) PDU30, PDU31 (MIH_Get_Information) PDU35, PDU-36(MIH_Register) |
|  | IS-2 IS push mode | After MIH registration, the MN receives push information from the network. |  | MC3.3 [OPT] PDU32 (MIH_Push_Information) PDU35, PDU-36(MIH_Register) |
| Protocol Data Unit | PDU35-36 MIH Register |  | MIH_Register request and response message exchange. | PDU35, PDU36 (MIH_Register) |
|  | PDU37-38 MIH Deregister |  | MIH_Deregister request and response message exchange. | PDU37, PDU38 (MIH_Deregister) |
|  | PDU39-40 MIH Event Subscribe |  | MIH_Event_Subscribe request and response message exchange | PDU39, PDU40 (MIH_Event_Subscribe) |
|  | PDU5 MIH Link Parameter Report |  | MIH_Link_Parameter_Report indication message | PDU5 (MIH_Link_Parameter_Report) |
|  | PDU33-34 MIH Capability Discovery |  | MIH_Capability_Discover request and response message exchange | PDU33, PDU34 (MIH_Capability_Discover) |
|  | PDU20-21 MIH MN HO Commit |  | MIH_MN_HO_Commit request and response message exchange | PDU20, PDU21 (MIH_MN_HO_Commit) |
|  | PDU26-27 MIH MN HO Complete |  | MIH_MN_HO_Complete request and response message exchange | PDU26, PDU27 (MIH_MN_HO_Complete) |
|  | PDU16-17 MIH MN HO Candidate Query |  | MIH_MN_HO_Candidate_Query request and response message exchange | PDU16, PDU17 (MIH_MN_HO_Candidate_Query) |
|  | PDU22-23 MIH Net HO Commit |  | MIH_Net_HO_Commit request and response message exchange | PDU22, PDU23 (MIH_Net_HO_Commit) |
|  | PDU14-15 MIH Net HO Candidate Query |  | MIH_Net_HO_Candidate_Query request and response message exchange | PDU14, PDU15 (MIH_Net_HO_Candidate_Query) |
|  | PDU18-19 MIH N2N HO Query Resources |  | MIH_N2N_HO_Query_Resources request and response message exchange | PDU18, PDU19 (MIH N2N HO Query Resources) |
|  | PDU24-25 MIH N2N HO Commit |  | MIH_N2N_HO_Commit request and response message exchange | PDU24, PDU25 (MIH_N2N_HO_Commit) |
|  | PDU28-29 MIH N2N HO Complete |  | MIH_N2N_HO_Complete request and response message exchange | PDU28, PDU29 (MIH_N2N_HO_Complete) |
|  | PDU30-31 MIH Get Information |  | MIH_Get_Information request and response message exchange | PDU30, PDU31 (MIH_Get_Information) |
|  | PDU32 MIH Push Information |  | MIH_Push_Information indication message | PDU32 (MIH_Push_Information) |

## Abstract Architecture

The architecture in Error: Reference source not foundError: Reference source not foundcontains several MIH devices: one MN (mobile node), two PoS (Point of Service), and two IS (Information Service Server).

MN1

PoS1

PoS2

Link 1

Link 2

Com 5

MIHF ID=pos1.opero.com

MIHF ID=pos2.operp.com

MIHF ID=mn1.opero.com

PoA2

PoA1

Com 3

Figure 1 Abstract architecture

Com 4

These devices understand MIH protocol and each have their own unique MIHF ID. The two PoAs (Point of Attachments, POA1 and PoA2) are used for illustrating the network connectivity status and are not part of the testing units. In this example, the MN is attached to the OperO network (operator O) via Link1 and to the OperP network (operator P) via Link2. Com#, where # is a number, indicates that there is a communication between the two end points using UDP as its transport mechanism.

The following table describes the potential access network technology for Link1 and Link2.

Table 2 Access technology examples for Link1 and Link2

| Operator | Link1 | Link 2 |
| --- | --- | --- |
| Same Operator | 802.11 | 802.16e |
| Same Operator | 802.11 | EVDO |
| Different Operator | 3GPP | 802.11 |

Following are different test architectures that are applied to different test cases.

MN1

PoS1

Link 1

MIHF ID=pos1.opero.com

MIHF ID=mn1.opero.com

PoA1

Com 3

Figure 1: Test Architecture 1

Figure 1 contains only two MIH devices, i.e., MN1 and PoS1. The MN1 is attached to PoA1 and communicates with PoS1 via Com3.

PoS1

MIHF ID=pos1.opero.com

Com 5

PoS2

MIHF ID=pos2.operp.com

Figure 2: Test Architecture 2

Figure 2covers test cases for the communication between two PoSes.

The test suites described in the subsequent sections cover only handovers within the same operator’s networks.

# Mobile-initiated handover test suite

The purpose of this test suite is to verify that the mobile node can initiate and complete a handover-related sequence of operations from one network to the other network using MIH protocol with network entities. Figure 3 or Figure 4 is used for this test suite depending on whether resource allocation for the target network is performed.  This test suite describes make-before-break handover scenarios.

MN

PoS1

PoA2

Link 1

Link 2

MIHF ID=pos1.opero.com

MIHF ID=mn.opero.com

PoA1

Com3

Com3’

Figure 3 Test Architecture (without resource allocation)

MN

PoS1

PoA2

Link 1

Link 2

MIHF ID=pos1.opero.com

MIHF ID=mn.opero.com

Com4

PoA1

Com3

PoS2

MIHF ID=pos2.opero.com

Com5

Figure 4: Test Architecture (with resource allocation)

## MHO-1a Mobile-initiated handover w/ network assistance and retaining the connection to the old network after handover

A mobile node is attached to PoA1 and discovers PoS1 via pre-configuration. The MN then decides to perform a handover with the assistance from the network within the same operator. The MN first execute a remote MIH_MN_HO_Commit command with PoS1 to indicate that the MN is going to perform a handover to the target network. After the handover, the MN performs MIH re-registration to PoS1. The connection to PoA1is retained after the handover.

### Test Sequence

MN was registered to PoS1 through Com3 via Link1. When test starts, MN indicates PoS1 about its target point of attachment.  MN then establishes Link 2. Once attached to PoA2, the MN re-registers to PoS1 through Com3’ via Link2.

### Test Case

| Test: | MHO-1a | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Mobile initiated handover w/ network assistance and retaining the connection to the old network after handover |  |  |  |
| Purpose: |  | To verify that - The mobile node can initiate and complete a handover using MIH protocol message exchange |  |  |  |
| Pre-Condition: |  | Use Figure 3Error: Reference source not found - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - MN recognizes that handovers to specific the point of attachment (i.e., PoA2) is served by PoS1. - MN should provide PoA2 in its candidate list when connected to PoA1. - PoS1 is configured to return PoA2 (if exists in the MN candidate list) as the best candidate for the MN. Network resource assumptions: - It is assumed that there is no resource reservation required. |  |  |  |
| Parameters |  | None. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN performs an MIH_MN_HO_Commit command with PoS1 through Com3 via Link 1 (Reference: Test PDU20-21) |  |  |  |  |
|  | Is Test PDU20-21 successful? |  |  | Yes | No |
|  | MN attaches to PoA2 and establishes Link2 |  |  |  |  |
|  | Is the MN attached to PoA2? |  |  | Yes | No |
|  | MN establishes Com3’ and re-registers to PoS1 (Reference: Test PDU35-36) where requester_code parameter in Test PDU35-36 is set to 1 (Re-Registration). |  |  |  |  |
|  | Is Test PDU35-36 successful? |  |  | Yes | No |

## MHO-1b Mobile-initiated handover w/ network assistance and deleting the connection to the old network after handover

This test case is the same as MHO-1a except that the connection to PoA1is removed after the handover.

### Test Sequence

The test sequence is the same as MHO-1a except that, after handover, MN detaches from PoA1.

### Test Case

| Test: | MHO-1b | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Mobile-initiated handover w/ network assistance and deleting the connection to the old network after handover |  |  |  |
| Purpose: |  | To verify that - The mobile node can initiate and complete a handover using MIH protocol message exchange - After a handover, the mobile node remove connection to the old network |  |  |  |
| Pre-Condition: |  | Use Figure 3 - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - MN recognizes that handovers to specific the point of attachment (i.e., PoA2) is served by PoS1. - MN should provide PoA2 in its candidate list when connected to PoA1. - PoS1 is configured to return PoA2 (if exists in the MN candidate list) as the best candidate for the MN. Network resource assumptions: - It is assumed that there is no resource reservation required. |  |  |  |
| Parameters |  | None. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN and PoS1 run MHO-1a. |  |  |  |  |
|  | Is MHO-1a successful? |  |  | Yes | No |
| - M | MN disconnects from PoA1 and break Link1 |  |  |  |  |
|  | Is MN disconnected from PoA1? |  |  | Yes | No |

## MHO-2a Mobile-initiated handover w/ network resource reservation signals and retaining the connection to the old network after handover

A mobile node is attached to PoA1 and discovers the network PoS via pre-configuration. The MN then decides to perform a handover with the assistance from the network within the same operator. Though there are MIH message exchanges for QoS resource information, the actual network resource reservation is performed by applying the resource reservation techniques for the specific access technology. The connection to PoA1as well as the registration to PoS1 is retained after the handover.

### Sequence

MN was registered to PoS1 through Com3 via Link1. When test starts, PoS1 provides a list of target networks to the MN. MN then establishes Link 2. Once attached to PoA2, the MN registers to PoS2 through Com4 via Link2.

See clause C.1 in IEEE Std 802.21-2008.

### Test Case

| Test: | MHO-2a | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Mobile-initiated handover with network resource reservation signals and retaining the connection to the old network after handover |  |  |  |
| Purpose: |  | To verify that - The mobile node can initiate and complete a handover using MIH protocol message exchange - The network PoS reserve network resources to assist handover using MIH protocol message exchange |  |  |  |
| Pre-Condition: |  | Use Figure 4 - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - PoS1 and MN are aware of the existence of PoS2 and recognize that handovers to specific the point of attachment (i.e., PoA2) is served by PoS2. - MN should provide PoA2 in its candidate list when connected to PoA1. - PoS1 is configured to return PoA2 (if exists in the MN candidate list) as the best candidate for the MN. Network resource assumptions: - PoS2 is able to reserve the QoS resource specified in the parameter QOS_LIST. - PoS1 is able to release the QoS resource at the end of the test. |  |  |  |
| Parameters |  | - QOS_LIST: a list of QoS requirements for this test case. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an MIH_MN_HO_Candidate_Query request message to PoS1 through Com3 via Link 1 (Reference: Test PDU16) |  |  |  |  |
|  | Is Test PDU16 successful? |  |  | Yes | No |
|  | PoS1 performs an MIH_N2N_HO_Query_Resources command with PoS2 via Com5 (Reference: Test PDU18-19) |  |  |  |  |
|  | Is Test PDU18-19 successful? |  |  | Yes | No |
|  | PoS1 responds with MIH_MN_HO_Query response message to MN through Com3 (Reference: Test PDU17) |  |  |  |  |
|  | Is Test PDU17 successful? |  |  | Yes | No |
|  | MN sends MIH_MN_HO_Commit request message to PoS1 through Com3 via Link 1 (Reference: Test PDU20) |  |  |  |  |
|  | Is Test PDU20 successful? |  |  | Yes | No |
|  | PoS1 performs an MIH_N2N_HO_Commit command with PoS2 through Com5 (Reference: Test PDU24-25) |  |  |  |  |
|  | Is Test PDU24-25 successful? |  |  | Yes | No |
| - P | PoS1 responds with MIH_MN_HO_Commit response message to MN through Com3 (Reference: Test PDU21) |  |  |  |  |
|  | Is Test PDU21 successful? |  |  | Yes | No |
|  | MN attaches to PoA2 and establishes Link2 |  |  |  |  |
|  | Is the MN attached to PoA2? |  |  | Yes | No |
|  | MN establishes Com4 and registers to PoS2 (Reference: Test PDU35-36) |  |  |  |  |
|  | Is Test PDU35-36 successful? |  |  | Yes | No |

## MHO-2b Mobile-initiated handover w/ network resource reservation signals and deleting the connection to the old network after handover

### Sequence

This test case is the same as MHO-2a except that the connection to PoA1as well as the registration to PoS1 is removed after the handover.

### Test Case

The test sequence is the same as MHO-2a except that, after handover, MN deregisters from PoS1 and detaches from PoA1.

| Test: | MHO-2b | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Mobile-initiated handover w/ network resource reservation signals and deleting the connection to the old network after handover |  |  |  |
| Purpose: |  | To verify that - The mobile node can initiate and complete a handover using MIH protocol message exchange - The network PoS reserve network resources to assist handover using MIH protocol message exchange - After a handover, the mobile node remove connection to the old network |  |  |  |
| Pre-Condition: |  | Use Figure 4 - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - PoS1and MN are aware of the existence of PoS2 and recognize that handovers to specific the point of attachment (i.e., PoA2) is served by PoS2. - MN should provide PoA2 in its candidate list when connected to PoA1. - PoS1 is configured to return PoA2 (if exists in the MN candidate list) as the best candidate for the MN. Network resource assumptions: - PoS2 is able to reserve the QoS resource specified in the parameter QOS_LIST. - PoS1 is able to release the QoS resource at the end of the test. |  |  |  |
| Parameters |  | - QOS_LIST: a list of QoS requirements for this test case. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN, PoS1 and PoS2 run MHO-2a |  |  |  |  |
|  | Is MHO-2a successful? |  |  | Yes | No |
|  | MN deregistered from PoS1. (Reference Test PDU37-38) |  |  |  |  |
|  | Is MN deregistered from PoS1? |  |  | Yes | No |
| - M | MN disconnects from PoA1 and break Link1 |  |  |  |  |
|  | Is MN disconnected from PoA1? |  |  | Yes | No |

# Network-initiated handover

The purpose of this test suite is to verify that the serving PoS can initiate and complete a handover-related sequence of operations from one network to the other network using MIH protocol with MN and the target PoS. Error: Reference source not found or Figure 4 is used for this test suite depending on whether network resource allocation in the target network is performed. This test suite describes make-before-break handover scenarios.

## NHO-1a Network-initiated handover w/o network resource reservation and retaining the connection to the old network after handover

A mobile node is attached to PoA1, discovers PoS1 via pre-configuration. PoS1 then decides to let MN perform a handover to a target network within the same operator. The PoS first execute a remote MIH_Net_HO_Commit command with MN to indicate that the MN is going to perform a handover to the target network. After the handover, the MN performs MIH re-registration to PoS1. The connection to PoA1 is retained after the handover.

See clause C.2 in IEEE Std 802.21-2008.

### Test Sequence

MN was registered to PoS1 through Com3 via Link1. When test starts, PoS1 indicates MN about its target point of attachment.  MN then establishes Link 2. Once attached to PoA2, the MN re-registers to PoS2 through Com4 via Link2.

### Test Case

| Test: | NHO-1a | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Network-initiated handover with network resource reservation signals. |  |  |  |
| Purpose: |  | To verify that - PoS1 can initiate a handover using MIH protocol message exchange. - MN successfully completes a handover using MIH protocol message exchange. |  |  |  |
| Pre-Condition: |  | Use Error: Reference source not found - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - MN recognizes that handovers to specific points of attachment (e.g., PoA2) are served by PoS1. - Based on the network preference, PoS1 is configured to return PoA2. |  |  |  |
| Parameter |  | None |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
| - P | PoS1 sends MIH_Net_HO_Commit request message to MN through Com3 (Reference: Test PDU22) |  |  |  |  |
|  | Is Test PDU22 successful? |  |  | Yes | No |
|  | MN attaches to PoA2 and establishes Link2 |  |  |  |  |
|  | Is the MN attached to PoA2? |  |  | Yes | No |
|  | MN establishes Com3’ and registers to PoS1 (Reference Test PDU35-36) where requester_code parameter in Test PDU35-36 is set to 1 (Re-Registration). |  |  |  |  |
|  | Is Test PDU35-36 successful? |  |  | Yes | No |
|  | MN responds with MIH_Net_HO_Commit response message to PoS1 through Com3 (Reference: Test PDU23) |  |  |  |  |
|  | Is Test PDU23 successful? |  |  | Yes | No |

## NHO-1b Network-initiated handover w/o network resource reservation and deleting the connection to the old network after handover

This test case is the same as NHO-1a except that the connection to PoA1as well as the registration to PoS1 is removed after the handover.

### Test Sequence

The test sequence is the same as NHO-1a except that, after handover, MN deregisters from PoS1 and detaches from PoA1.

### Test Case

| Test: | NHO-1b | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Network-initiated handover with network resource reservation signals and deleting the connection to the old network after handover |  |  |  |
| Purpose: |  | To verify that - PoS1 can initiate a handover using MIH protocol message exchange. - MN successfully completes a handover using MIH protocol message exchange. - After a handover, the mobile node remove connection to the old network |  |  |  |
| Pre-Condition: |  | Use Error: Reference source not found - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - MN recognizes that handovers to specific points of attachment (e.g., PoA2) are served by PoS1. - Based on the network preference, PoS1 is configured to return PoA2. |  |  |  |
| Parameter |  | None |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN, PoS1 and PoS2 run NHO-1a |  |  |  |  |
|  | Is NHO-1a successful? |  |  | Yes | No |
|  | MN deregistered from PoS1. (Reference Test PDU37-38) |  |  |  |  |
|  | Is Test PDU37-38 successful? |  |  | Yes | No |
| - M | MN disconnects from PoA1 and break Link1 |  |  |  |  |
|  | Is MN disconnected from PoA1? |  |  | Yes | No |

## NHO-2a Network-initiated handover w/ network resource reservation signals and retaining the connection to the old network after handover

A mobile node is attached to PoA1 and registered to PoS1. Operator O decides to request the MN to perform a handover to PoS2 with the assistance from the network (reserve network resources) within the same operator.

### Sequence

MN was registered to PoS1 through Com3 via Link1. When test starts, PoS1 provides a list of target networks to the MN. MN indicates PoS1 about its choice of target point of attachment which is PoA2. PoS1 then contacts PoS2 to reserve resources in the target network for MN. MN then establishes Link 2. Once attached to PoA2, the MN registers to PoS2 through Com4 via Link2.

See clause C.2 in IEEE Std 802.21-2008.

### Test Case

| Test: | NHO-2a | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Network-initiated handover with network resource reservation signals and retaining the connection to the old network after handover |  |  |  |
| Purpose: |  | To verify that - PoS1 can initiate and complete a handover using MIH protocol message exchange. - MN successfully completes a handover using MIH protocol message exchange. - The network PoS reserves network resources to assist handover using MIH protocol message exchange. |  |  |  |
| Pre-Condition: |  | Use Figure 4 - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - MN is aware of the existence of PoS2 and recognizes that handovers to specific points of attachment (e.g., PoA2) are served by PoS2. - Based on the network preference, PoS1 is configured to return PoA2. Network resource assumptions: - PoS2 is able to reserve the QoS resource specified in the parameter QOS_LIST. - PoS1 is able to release the QoS resource at the end of the test. |  |  |  |
| Parameter |  | - QOS_LIST: a list of QoS requirements for this test case. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | PoS1 sends MIH_Net_HO_Query request message to MN through Com3 via Link 1 (Reference: Test PDU14) |  |  |  |  |
|  | Is Test PDU14 successful? |  |  | Yes | No |
|  | MN responds with MIH_Net_HO_Query response message to PoS1 through Com3 (Reference: Test PDU15) |  |  |  |  |
|  | Is Test PDU15 successful? |  |  | Yes | No |
|  | PoS1 performs an MIH_N2N_HO_Query_Resources command with PoS2 via Com5 (Reference: Test PDU18-19) |  |  |  |  |
|  | Is Test PDU18-19 successful? |  |  | Yes | No |
|  | PoS1 performs an MIH_N2N_HO_Commit command with PoS2 through Com5 (Reference: Test PDU24-25) |  |  |  |  |
|  | Is Test PDU24-25 successful? |  |  | Yes | No |
| - P | PoS1 sends MIH_Net_HO_Commit request message to MN through Com3 (Reference: Test PDU22) |  |  |  |  |
|  | Is Test PDU22 successful? |  |  | Yes | No |
|  | MN attaches to PoA2 and establishes Link2 |  |  |  |  |
|  | Is the MN attached to PoA2? |  |  | Yes | No |
|  | MN establishes Com4 and registers to PoS2 (Reference Test PDU35-36) |  |  |  |  |
|  | Is Test PDU35 36 successful? |  |  | Yes | No |
|  | MN responds with MIH_Net_HO_Commit response message to PoS1 through Com3 (Reference: Test PDU23) |  |  |  |  |
|  | Is Test PDU23 successful? |  |  | Yes | No |

## NHO-2b Network-initiated handover w/ network resource reservation signals and deleting the connection to the old network after handover

This test case is the same as NHO-2a- except that the connection to PoA1as well as the registration to PoS1 is removed after the handover.

### Sequence

The test sequence is the same as NHO-2a except that, after handover, MN deregisters from PoS1 and detaches from PoA1.

### Test Case

| Test: | NHO-2b | Selection Criteria: |  | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- | --- |
| Title: |  | Network-initiated handover with network resource reservation signals and deleting the connection to the old network after handover |  |  |  |  |
| Purpose: |  | To verify that - PoS1 can initiate and complete a handover using MIH protocol message exchange. - MN successfully completes a handover using MIH protocol message exchange. - The network PoS reserves network resources to assist handover using MIH protocol message exchange. - After a handover, the mobile node remove connection to the old network |  |  |  |  |
| Pre-Condition: |  |  | Use Figure 4 - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - PoS1 and MN are aware of the existence of PoS2 and recognize that handovers to specific points of attachment (e.g., PoA2) are served by PoS2. - Based on the network preference, PoS1 is configured to return PoA2. Network resource assumptions: - PoS2 is able to reserve the QoS resource specified in the parameter QOS_LIST. - PoS1 is able to release the QoS resource at the end of the test. |  |  |  |
| Parameter |  | - QOS_LIST: a list of QoS requirements for this test case. |  |  |  |  |
| Step: | Test description |  |  |  | Verdict |  |
|  |  |  |  |  | Pass | Fail |
|  | MN, PoS1 and PoS2 run NHO-2a |  |  |  |  |  |
|  | Is NHO-2a successful? |  |  |  | Yes | No |
|  | MN deregistered from PoS1. (Reference Test PDU37-38) |  |  |  |  |  |
|  | Is Test PDU37-38 successful? |  |  |  | Yes | No |
| - M | MN disconnects from PoA1 and break Link1 |  |  |  |  |  |
|  | Is MN disconnected from PoA1? |  |  |  | Yes | No |

## NHO-3a Network initiated handover with candidate query without resource reservation and retaining the connection to the old network after handover

A mobile node is attached to PoA1 and registered to PoS1. Operator O decides to request the mobile node to send a list of candidate networks, and chooses a target network using the list sent by the mobile node, and let the mobile node perform a handover to PoS2 within the same operator.

The feasibility of this use case is based on there is no explicit requirement for using the MIH_N2N_HO_Query_Resources primitive in “7.4.17.3.4 Effect on receipt” for the 7.4.17.3 MIH_Net_HO_Candidate_Query.indication primitive. However, 7.4.17.1 General description provides that this message follows by the N2N resource signals.

### Sequence

MN was registered to PoS1 through Com3 via Link1. When test starts, PoS1 asks MN for providing a list of candidate networks. Once PoS1 receives the list of candidate networks from MN, it indicates MN about its choice of target point of attachment which is PoA2. Once attached to PoA2, the MN re-registers to PoS1 through Com3’ via Link1.

### Test Case

| Test: | NHO-3a | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Network initiated handover with candidate query without resource reservation and retaining the connection to the old network after handover |  |  |  |
| Purpose: |  | To verify that - PoS1 can query MN about candidate networks and initiate a handover with MN using MIH protocol message exchange. - MN successfully completes a handover using MIH protocol message exchange. |  |  |  |
| Pre-Condition: |  | Use Error: Reference source not found - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - MN recognizes that handovers to specific points of attachment (e.g., PoA2) are served by PoS1. - Based on the network preference, PoS1 is configured to return PoA2. Network resource assumptions: - None |  |  |  |
| Parameter |  | - QOS_LIST: a list of QoS requirements for this test case. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | PoS1 sends MIH_Net_HO_Candidate_Query request message to MN through Com3 via Link 1 (Reference: Test PDU14) |  |  |  |  |
|  | Is Test PDU14 successful? |  |  | Yes | No |
|  | MN responds with MIH_Net_HO_Query response message to PoS1 through Com3 (Reference: Test PDU15) |  |  |  |  |
|  | Is Test PDU15 successful? |  |  | Yes | No |
| - P | PoS1 sends MIH_Net_HO_Commit request message to MN through Com3 (Reference: Test PDU22) |  |  |  |  |
|  | Is Test PDU22 successful? |  |  | Yes | No |
|  | MN attaches to PoA2 and establishes Link2 |  |  |  |  |
|  | Is the MN attached to PoA2? |  |  | Yes | No |
|  | MN establishes Com3’ and re-registers to PoS1 (Test PDU35-36) where requester_code parameter in Test PDU35-36 is set to 1 (Re-Registration). |  |  |  |  |
|  | Is MN re-registered to PoS1? (Reference Test PDU35-36) |  |  | Yes | No |
|  | MN responds with MIH_Net_HO_Commit response message to PoS1 through Com3 (Reference: Test PDU23) |  |  |  |  |
|  | Is Test PDU23 successful? |  |  | Yes | No |

## NHO-3b Network initiated handover with candidate query without resource reservation and deleting the connection to the old network after handover

This test case is the same as NHO-3a except that the connection to PoA1as well as the registration to PoS1 is removed after the handover.

### Test Sequence

The test sequence is the same as NHO-3a except that, after handover, MN deregisters from PoS1 and detaches from PoA1.

### Test Case

| Test: | NHO-3b | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Network initiated handover with candidate query without resource reservation and deleting the connection to the old network after handover |  |  |  |
| Purpose: |  | To verify that - PoS1 can query MN about candidate networks and initiate a handover with MN using MIH protocol message exchange. - MN successfully completes a handover using MIH protocol message exchange. - After a handover, the mobile node remove connection to the old network |  |  |  |
| Pre-Condition: |  | Use Error: Reference source not found - MN is already attached to PoA1. - MN is already registered to PoS1 (Test PDU35-36). - MN is recognizes that handovers to specific points of attachment (e.g., PoA2) are served by PoS1. - Based on the network preference, PoS1 is configured to return PoA2. Network resource assumptions: None |  |  |  |
| Parameter |  | None |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN, PoS1 and PoS2 run NHO-3a |  |  |  |  |
|  | Is NHO-3a successful? |  |  | Yes | No |
|  | MN deregistered from PoS1. (Reference Test PDU37-38) |  |  |  |  |
|  | Is Test PDU37-38 successful? |  |  | Yes | No |
| - M | MN disconnects from PoA1 and break Link1 |  |  |  |  |
|  | Is MN disconnected from PoA1? |  |  | Yes | No |

# Information Service Test Cases

This section contains the detail description for MIH Information Service related test cases using the test architecture shown in Error: Reference source not found.

## IS-1a Query available networks without registration

The MN is connected to the network. It queries PoS1 for a suggested access network connection by providing a query message. MN does not perform MIH registration prior to sending a query.

### Test Case

| Test: | IS-1a | Selection Criteria: | Mandatory for IS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Query information on available access technologies |  |  |  |
| Purpose: |  | To verify that the Information Server is providing the correct information. - The Information Server is able to accept binary query or RDF query from a querier MIHF. - The Information Server returned the expected query response to the querier MIHF. |  |  |  |
| Pre-Condition: |  | Use Figure 1Error: Reference source not found - MN is the querier MIHF. - PoS1 is the Information Server. - MN has at least two access technology interfaces. - PoS1 contains information of all the access points and its location for the two access technology the MN supports. - Link type of Link1 is 23 (Wireless- UMTS). - Link type of Link2 is 19 (Wireless - IEEE 802.11). Link2 does not have to physically exist. |  |  |  |
| Parameters |  | - The following parameters are used for binary query. - Info_query_binary_list_ext: A list of binary queries set to Info_query_binary_list parameter in PDU 30-31. The default value is: - List size of LIST(IQ_BIN_DATA): 1 - List element #1:SEQUENCE { - CHOICE(NULL, QUERIER_LOC): QUERIER_LOC - SEQUENCE { - CHOICE(NULL, LOCATION): NULL - CHOICE(NULL, LINK_ADDR): LINK_ADDR - CHOICE(3GPP_2G_CELL_ID, MAC_ADDR): 3GPP_2G_CELL_ID: - SEQUENCE(PLMN_ID, LAC, CI) { any, any, 1000 } - CHOICE(NULL, NGHB_RADIUS): NULL } - CHOICE(NULL, NET_TYPE_INC): NET_TYPE_INC: - Bits 5 and 7 are set - CHOICE(NULL, NETWK_INC): NULL - CHOICE(NULL, RPT_TEMPL): RPT_TMPL - LIST(IE_TYPE): - List size: 4 - List element #1: - IE_CONTAINER_LIST_OF_NETWORKS - List element #2: - IE_CONTAINER_NETWORK - List element #3: - IE_NETWORK_TYPE - List element #4: - IE_NETWORK_ID - CHOICE(NULL, RPT_LIMIT): NULL - CHOICE(NULL, CURR_PREF): NULL } - info_response_binary_data_list_ext: The list of binary responses set to Info_response_binary_data_list parameter in PDU 30-31. The default value is: - List size of LIST(IR_BIN_DATA): 1 - List element #1: - LIST(INFO_ELEMENT) - List size: 2 - List element #1:IE_CONTAINER_NETWORK - IE_NETWORK_TYPE: SEQUENCE( - CHOICE(NULL, LINK_TYPE): LINK_TYPE: 19 - CHOICE(NULL, SUBTYPE): NULL - CHOICE(NULL, TYPE_EXT): NULL) - IE_NETWORK_ID: “opero.com” - List element #2:IE_CONTAINER_NETWORK - IE_NETWORK_TYPE: SEQUENCE( - CHOICE(NULL, LINK_TYPE): LINK_TYPE: 23 - CHOICE(NULL, SUBTYPE): NULL - CHOICE(NULL, TYPE_EXT): NULL) - IE_NETWORK_ID: “opero.com” - The following parameters are used for RDF query. - info_query_rdf_data_list_ext: A list of rdf queries set to info_query_rdf_data_list parameter in PDU 30-31. The default value is the same as the one for info_query_rdf_data_list parameter in PDU30-31 - info_response_rdf_data_list_ext: A list of RDF query responses set to info_response_rdf_data_list parameter in PDU Test 30-31. The default value is the same as the one for info_response_rdf_data_list parameter in PDU30-31 |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an information query to PoS1 and receives an information query response from PoS1 (Reference PDU 30-31). |  |  |  |  |
|  | Is Test PDU 30-31 successful? |  |  | YES | NO |

## IS-1b Query available networks with registration

The MN is connected to the network. It queries PoS1 for a suggested access network connection by providing a query message. MN performs MIH registration prior to sending a query.

### Test Case

| Test: | IS-1b | Selection Criteria: | Mandatory for IS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Query information on available access technologies |  |  |  |
| Purpose: |  | To verify that the Information Server is providing the correct information. - The Information Server is able to accept binary query or RDF query from a querier MIHF. - The Information Server returned the expected query response to the querier MIHF. |  |  |  |
| Pre-Condition: |  | Use Figure 1Error: Reference source not found - MN is the querier MIHF. - PoS1 is the Information Server. - MN has at least two access technology interfaces. - PoS1 contains information of all the access points and its location for the two access technology the MN supports. - Link type of Link1 is 23 (Wireless- UMTS). - Link type of Link2 is 19 (Wireless - IEEE 802.11). Link2 does not have to physically exist. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN registers with PoS1 (Reference: Test PDU35-36) |  |  |  |  |
|  | Is Test PDU35-36 successful? |  |  | Yes | No |
|  | MN and PoS1 run IS-1a |  |  |  |  |
|  | Is IS-1a successful? |  |  | Yes | No |

## IS-2 IS push mode

The MN is connected to the network. It registers with IS1. IS1 push suggested access network information to the MN.

### Test Case

| Test: | IS-2 | Selection Criteria: | Mandatory for IS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | Push information on available access technologies |  |  |  |
| Purpose: |  | To verify that the Information Server is providing the correct information. - The Information Server sent the expected information to the receiving MIHF. |  |  |  |
| Pre-Condition: |  | Use Error: Reference source not found - MN is the receiving MIHF. - PoS1 is the Information Server. - MN has at least two access technology interfaces. - PoS1 is an Information Server providing information of all the access points and its location for the two access technology the MN supports. - Link type of Link1 is 23 (Wireless- UMTS). - Link type of Link2 is 19 (Wireless - IEEE 802.11). Link2 does not have to physically exist. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN registers to PoS1 (Reference Test PDU35-36) |  |  |  |  |
|  | Is Test PDU35-36 successful? |  |  | YES | NO |
|  | PoS1 pushes information to MN (Reference PDU 32). When RDF information is pushed, the default values are used for all parameters of PDU32. When binary data is pushed, the following non-default values are used for specific parameters of PDU32: - info_query_binary_data_list: - List size of LIST(IQ_BIN_DATA): 1 - List element #1:SEQUENCE { - CHOICE(NULL, QUERIER_LOC): NULL - CHOICE(NULL, NET_TYPE_INC): NET_TYPE_INC: - Bi s 5 and 8 are set - CHOICE(NULL, NETWK_INC): NETWK_INC - LIST(NETWORK_ID): - List Size: 1 - List element #1: - NETWORK_ID: “opero.com” - CHOICE(NULL, RPT_TEMPL): RPT_TMPL - LIST(IE_TYPE): - List size: 3 - List element #1: - IE_CONTAINER_NETWORK - List element #2: - IE_NETWORK_TYPE - List element #3: - IE_NETWORK_AUX_ID - CHOICE(NULL, RPT_LIMIT): NULL - CHOICE(NULL, CURR_PREF): NULL } - info_query_rdf_data_list: - info_response_binary_data_list: - List size of LIST(IR_BIN_DATA): 1 - List element #1: LIST(INFO_ELEMENT) - List size: 2 - List element #1:IE_CONTAINER_NETWORK - IE_NETWORK_TYPE: SEQUENCE( - CHOICE(NULL, LINK_TYPE): LINK_TYPE: 19 - CHOICE(NULL, SUBTYPE): NULL - CHOICE(NULL, TYPE_EXT): NULL) - IE_NETWORK_ID: “opero.com” - List element #2:IE_CONTAINER_NETWORK - IE_NETWORK_TYPE: SEQUENCE( - CHOICE(NULL, LINK_TYPE): LINK_TYPE: 23 - CHOICE(NULL, SUBTYPE): NULL - CHOICE(NULL, TYPE_EXT): NULL) - IE_NETWORK_ID: “opero.com” - info_response_rdf_data_list: null |  |  |  |  |
|  | Is Test PDU 32 successful? |  |  | YES | NO |

# Common Test Cases

In the common test cases described in this section, the MIH protocol uses TCP or UDP transport as the MIH transport protocol. When TCP is used, MIH acknowledgment is disabled. When UDP is used, MIH acknowledgment is enabled.

## MIH Capability Discovery (PDU33 and PDU34)

| Test: | PDU33-34 | Selection Criteria: | Mandatory | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH capability discover request and response |  |  |  |
| Purpose: |  | To verify that - A requester MIHF sends a valid MIH_Capability_Discover request message to the responder MIHF. - The responder MIHF receives the message and responds to the requestor MIHF with a valid MIH_Capability_Discover response message. |  |  |  |
| Pre-Condition: |  | Use Figure 1Error: Reference source not found - MN is the requester MIHF - PoS1 is the responder MIHF - MN is attached to PoA1 - MN has discovered the PoS1 via pre-configuration - MN supports at least one of ES, CS or IS |  |  |  |
| Parameters |  | - requester_link_address_list of type LIST(NET_TYPE_ADDR) containing a list of network type and link address pair on the requester MIHF. The default value is null. - requester_supported_mih_event_list of type MIH_EVT_LIST containing a list of supported events on the requester MIHF. The default value is: - Bit 0: 1 - Other Bits: 0. - requester_supported_mih_command_list of type MIH_CMD_LIST containing a list of supported commands on the requester MIHF. The default value is: - Bit 0: 1 - Other Bits: 0. - requester_supported_is_query_type of type MIH_IQ_TYPE_LST containing a list of supported MIIS query types on the requester MIHF. The default value is: - Bit 0: 1 - Other Bits: 0. - requester_supported_transport_list of type MIH_TRANS_LST containing a list of supported transport types on the requester MIHF. The default value is: - Bit 0: 1 - Other Bits: 0. - requester_mbb_handover_support of type LIST(MBB_HO_SUPP) indicates if a make before break handover is supported on the requester MIHF. The default value is: - Bit 0: 1 - Other Bits: 0. - responder_link_address_list of type LIST(NET_TYPE_ADDR) containing a list of network type and link address pair on the responder MIHF. The default value is: - Bit 0: 1 - Other Bits: 0. - responder_supported_mih_event_list of type MIH_EVT_LIST containing a list of supported events on the responder MIHF. The default value is: - Bit 0: 1 - Other Bits: 0. - responder_supported_mih_command_list of type MIH_CMD_LIST containing a list of supported commands on the responder MIHF. The default value is: - Bit 0: 1 - Other Bits: 0. - responder_supported_is_query_type of type MIH_IQ_TYPE_LST containing a list of supported MIIS query types on the responder MIHF. The default value is: - Bit 0: 1 - Other Bits: 0. - responder_supported_transport_list of type MIH_TRANS_LST containing a list of supported transport types on the responder MIHF. The default value is: - Bit 0: 1 - Other Bits: 0. - responder_mbb_handover_support of type LIST(MBB_HO_SUPP) indicates if a make before break handover is supported on the responder MIHF. The default value is null. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an MIH_Capability_Discover request message to PoS1 via Com3 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing MN’s MIHF ID - Destination Identifier containing PoS1’s MIHF ID - If requester_link_address_list is non-null, LinkAddressList containing requester_link_address_list - If requester_supported_mih_event_list is non-null, SupportedMIHEventList containing requester_supported_mih_event_list - If requester_supported_mih_command_list is non-null, SupportedMIHCommandList containing requester_supported_mih_command_list - If requester_supported_is_query_type is non-null, SupportedISQueryTypeList containing requester_supported_is_query_type - If requester_supported_transport_list is non-null, SupportedTransportList containing requester_supported_transport_list - If requester_mbb_handover_support is non-null, MBBHandoverSupport containing requester_mbb_handover_support |  |  | Yes | No |
|  | PoS1 responds with an MIH_Capability_Discover response message |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing PoS1’s MIHF ID - Destination Identifier containing MN’s MIHF ID - If responder_link_address_list is non-null, LinkAddressList containing responder _link_address_list - If responder _supported_mih_event_list is non-null, SupportedMIHEventList containing responder_supported_mih_event_list - If responder_supported_mih_command_list is non-null, SupportedMIHCommandList containing responder_supported_mih_command_list - If responder_supported_is_query_type is non-null, SupportedISQueryTypeList containing responder_supported_is_query_type - If responder_supported_transport_list is non-null, SupportedTransportList containing responder_supported_transport_list - If responder_mbb_handover_support is non-null, MBBHandoverSupport containing responder_mbb_handover_support |  |  | Yes | No |
| Observations: |  |  |  |  |  |

## MIH Register (PDU35 and PDU36)

| Test: | PDU35-36 | Selection Criteria: | Mandatory for CS and IS push mode | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH register request and response |  |  |  |
| Purpose: |  | To verify that - A requester MIHF sends a valid MIH_Register request message to a responder MIHF. - The responder MIHF receives the message and registers the requester MIHF with infinite validity period. |  |  |  |
| Pre-Condition: |  | Use Figure 1Error: Reference source not found - MN is the requester MIHF - PoS1 is the responder MIHF - MN and PoS1 support CS or IS push mode - MN is attached to PoA1. - MN has discovered the PoS1 via pre-configuration |  |  |  |
| Parameters |  | - requester_code of type REG_REQUEST_CODE containing a request node. The default value is 0 (Regitration). |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an MIH_Register request message to PoS1 via Com3 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing MN’s MIHF ID - Destination Identifier containing PoS1’s MIHF ID - LinkIDList containing the link address of MN for Link1 and Link2 (Link2 is not connected) - RequestCode set to requester_code |  |  | Yes | No |
|  | PoS1 responds with a successful MIH_Register response message |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing PoS1’s MIHF ID - Destination Identifier containing MN’s MIHF ID - Status is 0 (Success) - ValidTimeInterval is 0 (infinite validity period) |  |  | Yes | No |
| Observations: |  |  |  |  |  |

## MIH Deregister (PDU37 and PDU38)

| Test: | PDU37-38 | Selection Criteria: | Mandatory for CS and IS push mode | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH deregister request and response |  |  |  |
| Purpose: |  | To verify that - A requesting MIHF sends a valid MIH_DeRegister request message to a responder MIHF. - The responder MIHF receives the message and deregisters the responder MIHF. |  |  |  |
| Pre-Condition: |  | Use Figure 1Error: Reference source not found - MN is the requester MIHF - PoS1 is the responder MIHF - MIH Register test case has passed. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an MIH_DeRegister request message to PoS1 via Com3 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing MN’s MIHF ID - Destination Identifier containing PoS1’s MIHF ID |  |  | Yes | No |
|  | PoS1 responds with a successful MIH_DeRegister response message |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing PoS1’s MIHF ID - Destination Identifier containing MN’s MIHF ID - Status is 0 (Success) |  |  | Yes | No |

## MIH Event Subscribe (PDU 39 and PDU40)

| Test: | PDU39-40 | Selection Criteria: | Mandatory for ES | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH event subscribe request and response |  |  |  |
| Purpose: |  | To verify that - A requester MIHF sends a valid MIH_Event_Subscribe request message to a responder MIHF. - The responder MIHF receives the message and subscribes the requester MIHF for a set of events. |  |  |  |
| Pre-Condition: |  | Use Figure 1Error: Reference source not found - MN is the requester MIHF - PoS1 is the responder MIHF - MN and PoS1 support ES - MN is attached to PoA1. - MN has discovered PoS1 via pre-configuration |  |  |  |
| Parameters |  | - link_identifer of type LINK_TUPLE_ID containing an identifier of the link for event subscription. The default value is the link identifier of Link1. - requested_mih_event_list of type MIH_EVT_LIST containing List of MIH events. The default value is: - Bit 0: 1 - Oher Bits: 0 - event_configuration_list of type LIST(EVT_CFG_INFO) containing a list of additional configuration information for event subscription. The default value is null. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an MIH_Event_Subscribe request message to PoS1 via Com3 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing MN’s MIHF ID - Destination Identifier containing PoS1’s MIHF ID - LinkIdentifier containing link_identifer - RequestedMIHEvent Id List containing requested_mih_event_list - If event_configuration_list is not null, EventConfigurationInfoList containing event_configuration_list |  |  | Yes | No |
|  | PoS1 responds with a successful MIH_Event_Subscribe response message |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing PoS1’s MIHF ID - Destination Identifier containing MN’s MIHF ID - Status is 0 (Success) - LinkIdentifier containing the link identifier of Link1 - ResponseMIHEventList containing a subset of the RequestedMIHEventIdList succeeding the subscription. |  |  | Yes | No |

## MIH Link Parameter Report (PDU 5)

| Test: | PDU5 | Selection Criteria: | Mandatory for ES | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH link parameter report indication |  |  |  |
| Purpose: |  | To verify that - A sender MIHF sends a valid MIH_Link_Parameter_Report request message to a receiver MIHF. - The receiver MIHF receives the message. |  |  |  |
| Pre-Condition: |  | Use Figure 1Error: Reference source not found - MN is the requester MIHF - PoS1 is the responder MIHF - MN and PoS1 support ES - MN is attached to PoA1 - MN has discovered PoS1 via pre-configuration - MN has successfully completed Test PDU39-40 (MIH Event Subscribe) with PoS1 with the following non-default parameters: - requested_mih_event_list: - Bit 3: 1 - Other Bits: 0 - event_configuration_list - List Size: 1 - List element #1: CHOICE(LIST(LINK_DET_CFG),LIST(LINK_CFG_PARAM)): LIST(LINK_CFG_PARAM) - List size: 1 - List element #1: SEQUENCE( - LINK_PARAM_TYPE: LINK_PARAM_GEN:2 (SINR) - CHOICE(NULL, TIMER_INTERVAL): TIMER_INTERVAL: 10 - TH_ACTION: 0 (no threshold) - LIST(THRESHOLD): List size: 0) |  |  |  |
| Parameters |  | - link_identifier of type LINK_TUPLE_ID containing Identifier of the link associated with the event. The default value is the link identifier of Link1. - link_parameter_report_list of type LIST(LINK_PARAM_RPT) containing a list of Link Parameter Reports. The default value is: SEQUENCE( - LINK_PARAM: SEQUENCE( - LINK_PARAM_TYPE: LINK_PARAM_GEN: - 2 (SINR) - CHOICE(LINK_PARAM_VAL, QOS_PARAM_VAL): LINK_PARAM_VAL: 30) - CHOICE(NULL, THRESHOLD): NULL) |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | After the timer interval specified in the event subscription, PoS1 sends an MIH_Link_Parameter_Report indication message |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing PoS1’s MIHF ID - Destination Identifier containing MN’s MIHF ID - Status is 0 (Success) - LinkIdentifier containing link_identifier - LinkParameterReportList containing link_parameter_report_list |  |  | Yes | No |

## MIH MN HO Commit (PDU20 and PDU21)

### MIH MN HO Commit request (PDU20)

| Test: | PDU20 | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH MN HO Commit request |  |  |  |
| Purpose: |  | To verify that - A requester MIHF sends a valid MIH_MN_HO_Commit request message to a responder MIHF. - The responder MIHF receives the message. |  |  |  |
| Pre-Condition: |  | Use Figure 1or Figure 3. - MN is the requester MIHF - PoS1 is the responder MIHF - MN and PoS1 support CS - MN is attached to PoA1 - MN has discovered PoS1 via pre-configuration and completed MIH registration with PoS1 |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an MIH_MN_HO_Commit request message to PoS1 via Com3 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing MN’s MIHF ID - Destination Identifier containing PoS1’s MIHF ID - LinkIDList containing the link address of MN for Link1 and Link2 (Link2 is not connected) - RequestCode set to 0 (Registration) |  |  | Yes | No |
| Observations: |  |  |  |  |  |

### MIH MN HO Commit response (PDU21)

| Test: | PDU21 | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH MN HO Commit response |  |  |  |
| Purpose: |  | To verify that - A responder MIHF sends a valid MIH_MN_HO_Commit response message to a requester MIHF. - The requester MIHF receives the message. |  |  |  |
| Pre-Condition: |  | Use Figure 1or Figure 3. - MN is the requester MIHF. - PoS1 is the responder MIHF - MN and PoS1 support CS - MN is attached to PoA1 - MN has discovered PoS1 via pre-configuration and completed MIH registration with PoS1 - PDU20 has been successfully completed. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | PoS1 responds with a successful MIH_MN_HO_Commit response message |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing PoS1’s MIHF ID - Destination Identifier containing MN’s MIHF ID - Status is 0 (Success) - ValidTimeInterval is 0 (infinite validity period) |  |  | Yes | No |
| Observations: |  |  |  |  |  |

## MIH MN HO Complete (PDU26 and PDU27)

### MIH MN HO Complete request (PDU26)

| Test: | PDU26 | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH MN HO Complete request |  |  |  |
| Purpose: |  | To verify that - A requester MIHF sends a valid MIH_MN_HO_Complete request message to a responder MIHF. - The responder MIHF receives the message. |  |  |  |
| Pre-Condition: |  | Use Error: Reference source not found or Figure 3. - MN is the requester MIHF - When Error: Reference source not found is used: - PoS1 is the responder MIHF - MN and PoS1 support CS - MN is attached to PoA1 - MN has discovered PoS1 via pre-configuration and completed MIH registration with PoS1 - When Figure 3 is used - PoS2 is the responder MIHF - MN and PoS2 support CS - MN is attached to PoA2 - MN has discovered PoS2 via pre-configuration and completed MIH registration with PoS2 |  |  |  |
| Parameters |  | - source_link_id of type LINK_TUPLE_ID containing the source link of the handover. - target_link_id of type LINK_TUPLE_ID containing the identifier of the target link of the handover. - handover_result of type HO_RESULT containing the handover result. The default value is 0 (Success). |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an MIH_MN_HO_Complete request message to the responder MIHF |  |  |  |  |
|  | Is the message received by the responder MIHF? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing MN’s MIHF ID - Destination Identifier containing the responder MIHF’s MIHF ID - SourceLinkId of type LINK_TUPLE_ID containing source_link_id. - TargetLinkId of type LINK_TUPLE_ID containing target_link_id. - HandoverResult containing handover_result. |  |  | Yes | No |
| Observations: |  |  |  |  |  |

### MIH MN HO Complete response (PDU27)

| Test: | PDU27 | Selection Criteria: | Mandatory for CS and IS push mode | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH MN HO Complete response |  |  |  |
| Purpose: |  | To verify that - A responder MIHF sends a valid MIH_MN_HO_Complete response message to a requester MIHF. - The requester MIHF receives the message. |  |  |  |
| Pre-Condition: |  | Use Figure 1or Figure 3. - MN is the requester MIHF. - When Error: Reference source not found is used: - PoS1 is the responder MIHF - MN and PoS1 support CS - MN is attached to PoA1 - MN has discovered PoS1 via pre-configuration and completed MIH registration with PoS1 - When Figure 3 is used - PoS2 is the responder MIHF - MN and PoS2 support CS - MN is attached to PoA2 - MN has discovered PoS2 via pre-configuration and completed MIH registration with PoS2 - PDU26 has been successfully completed. |  |  |  |
| Parameters |  | - source_link_id of type LINK_TUPLE_ID containing the source link of the handover. - target_link_id of type LINK_TUPLE_ID containing the identifier of the target link of the handover. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | The responder MIHF responds with a successful MIH_MN_HO_Complete response message |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing the responder MIHF’s MIHF ID - Destination Identifier containing MN’s MIHF ID - Status is 0 (Success) - SourceLinkIdentifier containing source_link_id - TargetLinkIdentifier containing target_link_id |  |  | Yes | No |
| Observations: |  |  |  |  |  |

## MIH MN HO Candidate Query (PDU16 and PDU17)

### MIH MN HO Candidate Query request (PDU16)

| Test: | PDU16 | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH MN HO Candidate Query request |  |  |  |
| Purpose: |  | To verify that - A requester MIHF sends a valid MIH_MN_HO_Candidate_Query request message to a responder MIHF. - The responder MIHF receives the message. |  |  |  |
| Pre-Condition: |  | Use Figure 1or Figure 3. - MN is the requester MIHF - PoS1 is the responder MIHF - MN and PoS1 support CS - MN is attached to PoA1 - MN has discovered PoS1 via pre-configuration and completed MIH registration with PoS1 |  |  |  |
| Parameters |  | - source_link_id of type LINK_TUPLE_ID containing the source link of the handover. - candidate_link_list of type LIST(LINK_POA_LIST) containing the list of identifiers of the candidate links. - qos_resource_req of type QOS_LIST containing the minimal QoS resources required for the candidate links. The default value is the same as the default value of requested_resource_set parameter of MIH N2N HO Complete test. - ip_config_methods of type IP_CFG_MTHDS containing IP configuration methods. The default value is null. - dhcp_server_addr of type IP_ADDR containing the DHCP server address. The default value is null. - fa_addr of type IP_ADDR containing the FA address. The default value is null. - ar_addr of type IP_ADDR containing the access router address. The default value is null. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an MIH_MN_HO_Candidate_Query request message to PoS1 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing MN’s MIHF ID - Destination Identifier containing the PoS1’s MIHF ID - SourceLinkIdentifier containing source_link_id - CandidateLinkList containing candidate_link_list - If Ip_config_methods is non-null, IPConfigurationMethods containing ip_config_methods - If dhcp_server_addr if non-null, DHCPServerAddress containing dhcp_server_addr - If fa_addr is non-null, FAAddress containing fa_addr - If ar_addr is non-null, AccessRouterAddress containing ar_addr |  |  | Yes | No |
| Observations: |  |  |  |  |  |

### MIH MN HO Candidate_Query response (PDU17)

| Test: | PDU17 | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH MN HO Candidate Query response |  |  |  |
| Purpose: |  | To verify that - A responder MIHF sends a valid MIH_MN_HO_Candidate_Query response message to a requester MIHF. - The requester MIHF receives the message. |  |  |  |
| Pre-Condition: |  | Use Figure 1or Figure 3. - MN is the requester MIHF. - PoS1 is the responder MIHF - MN and PoS1 support CS - MN is attached to PoA1 - MN has discovered PoS1 via pre-configuration and completed MIH registration with PoS1 - PDU16 has been successfully completed. |  |  |  |
| Parameters |  | - source_link_id of type LINK_TUPLE_ID containing the source link of the handover. - preferred_candidate_link_list of type LIST(RQ_RESULT) containing the new access networks to which handover initiation should be considered. The default value for preferred_candidate_link_list is: - List size of LIST(RQ_RESULT): 1 - List element #1 of LIST(RQ_RESULT): - LINK_POA_LIST: - List size: 1 - List Element #1 of LINK_POA_LIST: - Link identifier of Link2 - QOS_LIST: - NUM_COS_TYPES: 1 - List size of LIST(MIN_PK_TX_DELAY): 0 - List size of LIST(AVG_PK_TX_DELAY): 0 - List size of LIST(MAX_PK_TX_DELAY): 1 - List size of LIST(PK_DELAY_JITTER): 0 - List size of LIST(PK_LOSS_RATE): 0 - Element #1 of LIST(MAX_PK_TX_DELAY) - COS_ID: 0 - UNSIGNED_INT(2): 500 - CHOICE(NULL,BOOLEAN,IP_CFG_MTHDS): NULL - CHOICE(NULL, BOOLEAN, DHCP_SERV): NULL - CHOICE(NULL, BOOLEAN, FN_AGNT): NULL - CHOICE(NULL, BOOLEAN, ACC_RTR): NULL |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | The responder MIHF responds with a successful MIH_MN_HO_Candidate_Query response message |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing the responder MIHF’s MIHF ID - Destination Identifier containing MN’s MIHF ID - Status is 0 (Success) - SourceLinkIdentifier containing source_link_id - PreferredCandidateList containing preferred_candidate_link_list |  |  | Yes | No |
| Observations: |  |  |  |  |  |

## MIH Net HO Commit (PDU22 and PDU23)

### MIH Net HO Commit request (PDU22)

| Test: | PDU22 | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH Net HO Commit request |  |  |  |
| Purpose: |  | To verify that - A requester MIHF sends a valid MIH_Net_HO_Commit request message to a responder MIHF. - The responder MIHF receives the message. |  |  |  |
| Pre-Condition: |  | Use Figure 1or Figure 3. - MN is the responder MIHF - PoS1 is the requester MIHF - MN and PoS1 support CS - MN is attached to PoA1 - MN has discovered PoS1 via pre-configuration and completed MIH registration with PoS1 |  |  |  |
| Parameters |  | - link_type of type LINK_TYPE containing the target link type - target_network_info_list of type LIST(TGT_NET_INFO) containing target network information for assisting the mobile node to perform a handover. - assigned_resource_set of type ASGN_RES_SET containing the set of resources allocated to MN By default, the following information is set: - QOS_LIST: same as QOS_LIST in requested_resource_set. - TSP_CONTAINER: same as TSP_CONTAINER in requested_resource_set. - link_action_execution_delay of type UNSIGNED_INT(2) containing time (in ms) to elapse before an action needs to be taken. The default value is 0. - link_action_list (OPTIONAL) of type LIST(LINK_ACTION_REQ) containing a list of network controlled handover actions for the links. The default value is null. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | PoS1 sends an MIH_Net_HO_Commit request message to MN via Com3 |  |  |  |  |
|  | Is the message received by MN1? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing MN’s MIHF ID - Destination Identifier containing PoS1’s MIHF ID - LinkType containing link_type - TargetNetworkInfoList containing target_network_info_list - AssignedResourceSet containing assigned_resource_set - LinkActionExecutionDelay containing link_action_executio_delay - If link_action_list (OPTIONAL) is non-null, LinkActionsList containing link_action_list |  |  | Yes | No |
| Observations: |  |  |  |  |  |

### MIH Net HO Commit response (PDU23)

| Test: | PDU23 | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH Net HO Commit response |  |  |  |
| Purpose: |  | To verify that - A responder MIHF sends a valid MIH_Net_HO_Commit response message to a requester MIHF. - The requester MIHF receives the message. |  |  |  |
| Pre-Condition: |  | Use Figure 1or Figure 3. - PoS1 is the requester MIHF. - MN is the responder MIHF - MN and PoS1 support CS - MN is attached to PoA1 - MN has discovered PoS1 via pre-configuration and completed MIH registration with PoS1 - PDU22 has been successfully completed. |  |  |  |
| Parameters |  | - link_type of type LINK_TYPE containing the target link type - target_network_info of type TGT_NET_INFO containing the target network information for assisting the mobile node to perform a handover. - link_action_result_list (OPTIONAL) of type LIST(LINK_ACTION_RSP) containing A list of link actions result.. The default value is null. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN responds with a successful MIH_Net_HO_Commit response message |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing PoS1’s MIHF ID - Destination Identifier containing MN’s MIHF ID - Status is 0 (Success) - LinkType containing link_type - TargetNetworkInfo containing target_network_info - If link_action_result (OPTIONAL) is non-null, LinkActionsResultList containing link_action_result |  |  | Yes | No |
| Observations: |  |  |  |  |  |

## MIH Net HO Candidate Query (PDU14 and PDU15)

### MIH Net HO Candidate Query request (PDU14)

| Test: | PDU14 | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH Net HO Candidate Query request |  |  |  |
| Purpose: |  | To verify that - A requester MIHF sends a valid MIH_Net_HO_Candidate_Query request message to a responder MIHF. - The responder MIHF receives the message. |  |  |  |
| Pre-Condition: |  | Use Figure 1or Figure 3. - MN is the requester MIHF - PoS1 is the responder MIHF - MN and PoS1 support CS - MN is attached to PoA1 - MN has discovered PoS1 via pre-configuration and completed MIH registration with PoS1 |  |  |  |
| Parameters |  | - suggested_new_link_list of type LIST(LINK_POA_LIST) containing A list of PoAs for each link, suggesting the new access networks to which handover initiation should be considered. The default value is: - List size: 1 - List Element #1 of LINK_POA_LIST: - Link identifier of Link2 - query_resource_report_flag of type BOOLEAN indicating if resources need to be reported by MN (TRUE) or not (FALISE). The default value is FALSE. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | PoS1 sends an MIH_Net_HO_Candidate_Query request message to MN |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing PoS1’s MIHF ID - Destination Identifier containing MN’s MIHF ID - SourceLinkIdentifier containing source_link_id - CandidateLinkList containing candidate_link_list - If Ip_config_methods is non-null, IPConfigurationMethods containing ip_config_methods - If dhcp_server_addr if non-null, DHCPServerAddress containing dhcp_server_addr - If fa_addr is non-null, FAAddress containing fa_addr - If ar_addr is non-null, AccessRouterAddress containing ar_addr |  |  | Yes | No |
| Observations: |  |  |  |  |  |

### MIH Net HO Candidate_Query response (PDU15)

| Test: | PDU15 | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH Net HO Candidate Query response |  |  |  |
| Purpose: |  | To verify that - A responder MIHF sends a valid MIH_MN_HO_Candidate_Query response message to a requester MIHF. - The requester MIHF receives the message. |  |  |  |
| Pre-Condition: |  | Use Figure 1or Figure 3. - MN is the requester MIHF. - PoS1 is the responder MIHF - MN and PoS1 support CS - MN is attached to PoA1 - MN has discovered PoS1 via pre-configuration and completed MIH registration with PoS1 - PDU14 has been successfully completed. |  |  |  |
| Parameters |  | - source_link_id of type LINK_TUPLE_ID containing the source link of the handover. - handover_status of type HO_STATUS containing the acceptance status (permit/decline) of the handover request. The default value is 0 (HandoverPermitted) - preferred_candidate_link_list of type LIST(RQ_RESULT) containing the new access networks to which handover initiation should be considered. The default value for preferred_candidate_link_list is: - List size of LIST(RQ_RESULT): 1 - List element #1 of LIST(RQ_RESULT): - LINK_POA_LIST: - List size: 1 - List Element #1 of LINK_POA_LIST: - Link identifier of Link2 - QOS_LIST: - NUM_COS_TYPES: 1 - List size of LIST(MIN_PK_TX_DELAY): 0 - List size of LIST(AVG_PK_TX_DELAY): 0 - List size of LIST(MAX_PK_TX_DELAY): 1 - List size of LIST(PK_DELAY_JITTER): 0 - List size of LIST(PK_LOSS_RATE): 0 - Element #1 of LIST(MAX_PK_TX_DELAY) - COS_ID: 0 - UNSIGNED_INT(2): 500 - CHOICE(NULL,BOOLEAN,IP_CFG_MTHDS): NULL - CHOICE(NULL, BOOLEAN, DHCP_SERV): NULL - CHOICE(NULL, BOOLEAN, FN_AGNT): NULL - CHOICE(NULL, BOOLEAN, ACC_RTR): NULL |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN responds with a successful MIH_Net_HO_Candidate_Query response message |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Is the message in the right format with the following information: - Source Identifier containing PoS1’s MIHF ID - Destination Identifier containing MN’s MIHF ID - Status is 0 (Success) - SourceLinkIdentifier containing source_link_id - HandoverStatus containing handover_status - PreferredCandidateList containing preferred_candidate_link_list |  |  | Yes | No |
| Observations: |  |  |  |  |  |

## MIH N2N HO Commit (PDU24 and PDU25)

| Test: | PDU24-25 | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH N2N HO Commit request and response |  |  |  |
| Purpose: |  | To verify that when handover from PoS1 to PoS2 - A requester PoS sends a MIH_N2N_HO_Commit request message to a responder PoS. - The responder PoS receives the MIH_N2N_HO_Commit request message and responds to the requester PoS with an MIH_N2N_HO_Commit response message. It is not a purpose of this test to verify the requested resources are actually allocated in the target network. |  |  |  |
| Pre-Condition: |  | Use Figure 2Error: Reference source not found - PoS1 is the requester PoS - PoS2 is the responder PoS |  |  |  |
| Parameters |  | - mnid of type MIHF_ID containing an MIHF ID of a mobile node. - target_mn_linkid of type LINK_ID containing a link identifier of the mobile node. - target_poa of type LINK_ADDR containing a link address of the target PoA. - requested_resource_set of type REQ_RES_SET containing the set of parameters required for performing MN admission control and resource reservation at the target network By default, the following information is set: - QOS_LIST: - NUM_COS_TYPES: 1 - List size of LIST(MIN_PK_TX_DELAY): 0 - List size of LIST(AVG_PK_TX_DELAY): 0 - List size of LIST(MAX_PK_TX_DELAY): 1 - List size of LIST(PK_DELAY_JITTER): 0 - List size of LIST(PK_LOSS_RATE): 0 - Element #1 of LIST(MAX_PK_TX_DELAY) - COS_ID: 0 - UNSIGNED_INT(2): 500 - TSP_CONTAINER: - CHOICE value: 0 (NULL) - HO_CAUSE: 2 (No resource) - target_link_id of type LINK_TUPLE_ID containing the identifier of the target PoA for the MN. - assigned_resource_set of type ASGN_RES_SET containing the set of resources allocated to MN By default, the following information is set: - QOS_LIST: same as QOS_LIST in requested_resource_set. - TSP_CONTAINER: same as TSP_CONTAINER in requested_resource_set. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | PoS1 sends an MIH_N2N_HO_Commit request message to PoS2 via Com5 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Is the message in the right format with the following information? - Source Identifier containing PoS1’s MIHF ID - Destination Identifier containing PoS2’s MIHF ID - MNIdentifierLink containing mnid - TargetPoA containing target_poa - RequestedResourceSet containing requested_resource_set |  |  | Yes | No |
|  | PoS2 sends an MIH_N2N_HO_Commit response message to PoS1 via Com5 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Is the message in the right format with the following information? - Source Identifier containing PoS2’s MIHF ID - Destination Identifier containing PoS1’s MIHF ID - Status is 0 (Success) - MNIdentifierLink containing mnid - TargetLinkIdentifier containing target_link_id - AssignedResourceSet containing assigned_resource_set |  |  | Yes | No |

## MIH N2N HO Complete (PDU28 and PDU29)

| Test: | PDU28-29 | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH N2N HO Complete request and response |  |  |  |
| Purpose: |  | To verify that when handover from PoS1 to PoS2 - A requester PoS sends a MIH_N2N_HO_Complete request message to a responder PoS. - The responder PoS receives the MIH_N2N_HO_Complete request message and responds to the requester PoS with an MIH_N2N_HO_Complete response message. It is not a purpose of this test to verify the requested resources are actually allocated in the target network. |  |  |  |
| Pre-Condition: |  | Use Figure 2Error: Reference source not found - PoS1 is the requester PoS - PoS2 is the responder PoS |  |  |  |
| Parameters |  | - mnid of type MIHF_ID containing an MIHF ID of a mobile node. - target_mn_linkid of type LINK_ID containing a link identifier of the mobile node. - source_link_id of type LINK_TUPLE_ID containing the source link of the handover. - target_link_id of type LINK_TUPLE_ID containing the identifier of the target link of the handover. - handover_result of type HO_RESULT containing the handover result. The default value is 0 (Success). - resource_retention_status of type BOOLEAN containing TRUE (retain resource) or FALSE (release resource). The default value is FALSE. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | PoS1 sends an MIH_N2N_HO_Complete request message to PoS2 via Com5 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Is the message in the right format with the following information? - Source Identifier containing PoS1’s MIHF ID - Destination Identifier containing PoS2’s MIHF ID - MNIdentifierLink containing mnid - SourceLinkIdentifier containing source_link_id - TargetLinkIdentifier containing target_link_id - HandoverResult containing handover_result |  |  | Yes | No |
|  | PoS2 sends an MIH_N2N_HO_Complete response message to PoS1 via Com5 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Is the message in the right format with the following information? - Source Identifier containing PoS2’s MIHF ID - Destination Identifier containing PoS1’s MIHF ID - Status is 0 (Success) - MNIdentifierLink containing mnid - SourceLinkIdentifier containing source_link_id - TargetLinkIdentifier containing target_link_id - ResourceRetentionStatus containing resource_retention_status |  |  | Yes | No |

## MIH N2N HO Query Resources (PDU18 and PDU19)

| Test: | PDU18-19 | Selection Criteria: | Mandatory for CS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH N2N HO Query Resources request and response |  |  |  |
| Purpose: |  | To verify that when handover from PoS1 to PoS2 - A requester PoS sends a MIH_N2N_HO_Query_Resources request message to a responder PoS. - The responder PoS receives the MIH_N2N_HO_Query_Resources request message and responds to the requester PoS with an MIH_N2N_HO_Query_Resources response message. It is not a purpose of this test to verify the actual resources available in the target network. |  |  |  |
| Pre-Condition: |  | Use Figure 2Error: Reference source not found - PoS1 is the requester PoS - PoS2 is the responder PoS |  |  |  |
| Parameters |  | - qos_resource_req of type QOS_LIST containing the minimal QoS resources required for the candidate links. The default value is the same as the default value of requested_resource_set parameter of MIH N2N HO Complete test. - ip_config_methods of type IP_CFG_MTHDS containing IP configuration methods. The default value is null. - dhcp_server_addr of type IP_ADDR containing the DHCP server address. The default value is null. - fa_addr of type IP_ADDR containing the FA address. The default value is null. - ar_addr of type IP_ADDR containing the access router address. The default value is null. - candidate_link_list of type LIST(RQ_RESULT) containing the list of new access networks to which handover initiation should be considered. The default value for preferred_candidate_link_list is: - List size of LIST(RQ_RESULT): 1 - List element #1 of LIST(RQ_RESULT): - LINK_POA_LIST: - List size: 1 - List Element #1 of LINK_POA_LIST: - Link identifier of Link2 - QOS_LIST: - NUM_COS_TYPES: 1 - List size of LIST(MIN_PK_TX_DELAY): 0 - List size of LIST(AVG_PK_TX_DELAY): 0 - List size of LIST(MAX_PK_TX_DELAY): 1 - List size of LIST(PK_DELAY_JITTER): 0 - List size of LIST(PK_LOSS_RATE): 0 - Element #1 of LIST(MAX_PK_TX_DELAY) - COS_ID: 0 - UNSIGNED_INT(2): 500 - CHOICE(NULL,BOOLEAN,IP_CFG_MTHDS): NULL - CHOICE(NULL, BOOLEAN, DHCP_SERV): NULL - CHOICE(NULL, BOOLEAN, FN_AGNT): NULL - CHOICE(NULL, BOOLEAN, ACC_RTR): NULL |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | PoS1 sends an MIH_N2N_HO_Query_Resources request message to PoS2 via Com5 |  |  |  |  |
|  | Is the message received by PoS2? |  |  | Yes | No |
|  | Is the message in the right format with the following information? - Source Identifier containing PoS1’s MIHF ID - Destination Identifier containing PoS2’s MIHF ID - SourceLinkIdentifier containing source_link_id - If Ip_config_methods is non-null, IPConfigurationMethods containing ip_config_methods - If dhcp_server_addr if non-null, DHCPServerAddress containing dhcp_server_addr - If fa_addr is non-null, FAAddress containing fa_addr - If ar_addr is non-null, AccessRouterAddress containing ar_addr |  |  | Yes | No |
|  | PoS2 sends an MIH_N2N_HO_Query_Resources response message to PoS1 via Com5 |  |  |  |  |
|  | Is the message received by PoS1? |  |  | Yes | No |
|  | Is the message in the right format with the following information? - Source Identifier containing PoS2’s MIHF ID - Destination Identifier containing PoS1’s MIHF ID - Status is 0 (Success) - LinkResourceStatus is TRUE (AvailableS) - CandidateLinkList containing candidate_link_list |  |  | Yes | No |

## MIH Get Information (PDU30 and PDU31)

IEEE 802.21 supports both binary query and RDF query. This test uses RDF query by default.

| Test: | PDU30-31 | Selection Criteria: | Mandatory for IS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH Get Information request and response |  |  |  |
| Purpose: |  | To verify that - A requester MIHF sends a MIH_Get_Information request message to a responder MIHF. - The responder MIHF receives the MIH_Get_Information request message and responds to the requester MIHF with an MIH_Get_Information response message. |  |  |  |
| Pre-Condition: |  | Use Figure 1Error: Reference source not found - MN is the requester MIHF - PoS1 is the responder MIHF - Link type of Link1 is 23 (Wireless - UMTS). - Link type o Link2 is 19 (Wireless - IEEE 802.11). Link2 does not have to physically exist. |  |  |  |
| Parameters |  | - info_query_binary_data_list of type LIST(IQ_BIN_DATA) containing a list of TLV queries. The default value is null. - info_query_rdf_data_list of type LIST(IQ_RDF_DATA) containing a list of RDF queries. The default value is given as follows: - List size: 1 - List element #1:SEQUENCE( - CHOICE(NULL, MIME_TYPE): NULL OCTET_STRING: See Table 3: RDF Query Data - info_query_rdf_schema_url of type BOOLEAN indicating that the query is an RDF Schema URL query. The default value is FALSE. - info_query_rdf_schema_list of type LIST(IQ_RDF_SCHM) containing a list of RDF schema queries. The default value is null. - max_response_size of type UNSIGNED_INT(2) containing the maximum size of Info Response parameters in MIH_Get_Information response primitive in octets. The default value is null. - querier_network_type of type NETWORK_TYPE containing the type of the network being used by the querier. - unauth_information_request of type BOOLEAN indicating the value of UIR bit to be set in theMIH_Get_Information request message. - info_response_binary_data_list of type LIST(IR_BIN_DATA) containing a list of TLV query responses. The default value is null. - info_response_rdf_data_list of type LIST(IR_RDF_DATA) containing a list of RDF query responses. The default value is given as follows: - List size: 1 - List element #1:SEQUENCE( - CHOICE(NULL, MIME_TYPE): NULL OCTET_STRING: See Table 4 - info_response_rdf_schema_url_list of type LIST(IR_SCHM_URL) containing a list of RDF schema URL. The default value is null. - info_response_rdf_schema_list of type LIST(IR_RDF_SCHM) containing a list of RDF schema query responses. The default value is null. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | MN sends an MIH_Get_Information request message to PoS1 via Com8 |  |  |  |  |
|  | Is the message received by IS1? |  |  | Yes | No |
|  | Is the message in the right format with the following information? - Source Identifier containing MN’s MIHF ID - Destination Identifier containing IS1’s MIHF ID - If info_query_binary_data_list is non-null, IPInfoQueryBinaryDataList containing info_query_binary_data_list - If info_query_rdf_data_list is non-null, IPInfoQuery RDFDataList containing info_query_rdf_data_list - InfoQueryRDFSchemaURL containing Info_query_rdf_schema_url - If Info_query_rdf_schema_list is non-null, InfoQueryRDFSchemaList containing info_query_rdf_schema_list - If max_response_size is non-null, MaxResponseSize containing max_response_size - QuerierNetworkType containing querier_network_type - UnauthenticatedInformationRequest containing unauth_information_request |  |  | Yes | No |
|  | PoS1 sends an MIH_Get_Information response message to MN via Com8 |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Is the message in the right format with the following information? - Source Identifier containing IS1’s MIHF ID - Destination Identifier containing MN’s MIHF ID - Status is 0 (Success) - If info_response_binary_data_list is non-null, InfoResponseBinaryDataList containing info_response_binary_data_list - If info_response_rdf_data_list is non-null, InfoResponseRDFDataList containing info_response_rdf_data_list - InfoResponseRDFSchemaURLList containing Info_query_rdf_schema_url - If info_response_rdf_schema_url_list is non-null, InfoQueryRDFSchemaList containing info_response_rdf_schema_url_list - If info_response_rdf_schema_list is non-null, InfoResponseRDFSchemaList containing info_response_rdf_schema_list |  |  | Yes | No |

Table 3: RDF Query Data

PREFIX mihbasic: <http://www.mih.org/2005/04/rdf-basic-schema#> PREFIX xsd: <http://www.w3.org/2001/XMLSchema#> SELECT ?network_aux_id ?link_type WHERE { ?network mihbasic:ie_network_id ?network_id . ?network mihbasic:ie_network_type ?network_type . ?network_type mihbasic:link_type ?link_type . ?network mihbasic:ie_container_poa ?poa . ?poa mihbasic:ie_poa_location ?x1 . ?x1 mihbasic:location_cell_id “1000” . }

Table 4: RDF Query Response Data

<?xml version="1.0"?> <sparql xmlns:rdf="http://www.w3.org/1999/02/22-rdf-syntax-ns#" xmlns:xs="http://www.w3.org/2001/XMLSchema#" xmlns="http://www.w3.org/2005/sparql-results#" > <head> <variable name="link_type"/> <variable name="network_id"/> </head> <results> <result> <binding name="link_type"> <literal>23</literal> </binding> </result> <result> <binding name="network_id"> <literal>opero.com</literal> </binding> </result> </results> </sparql>

## MIH Push Information (PDU32)

IEEE 802.21 supports both binary query and RDF query. The corresponding PDU30 tests are split into two types of test cases.

| Test: | PDU32 | Selection Criteria: | Mandatory for IS | Selected: | Yes No |
| --- | --- | --- | --- | --- | --- |
| Title: |  | MIH Push Information indication |  |  |  |
| Purpose: |  | To verify that - A sender MIHF sends a MIH_Push_Information indication message to a receiver MIHF. - The receiver MIHF receives the MIH_Push_Information indication message. |  |  |  |
| Pre-Condition: |  | Use Error: Reference source not found or Error: Reference source not found - PoS1is the sender MIHF - MN is the receiver MIHF - Link type of Link1 is 23 (Wireless - UMTS). - Link type o Link2 is 19 (Wireless - IEEE 802.11). Link2 does not have to physically exist. |  |  |  |
| Parameters |  | - info_response_binary_data_list of type LIST(IR_BIN_DATA) containing a list of TLV query responses. The default value is given as follows: - List size of LIST(IR_BIN_DATA): 1 - List element #1: LIST(INFO_ELEMENT) - List size: 2 - List element #1:IE_CONTAINER_NETWORK - IE_NETWORK_TYPE: SEQUENCE( - CHOICE(NULL, LINK_TYPE): LINK_TYPE: 19 - CHOICE(NULL, SUBTYPE): NULL - CHOICE(NULL, TYPE_EXT): NULL) - IE_NETWORK_ID: “opero.com” - List element #2:IE_CONTAINER_NETWORK - IE_NETWORK_TYPE: SEQUENCE( - CHOICE(NULL, LINK_TYPE): LINK_TYPE: 23 - CHOICE(NULL, SUBTYPE): NULL - CHOICE(NULL, TYPE_EXT): NULL) - IE_NETWORK_ID: “opero.com” - info_response_rdf_data_list of type LIST(IR_RDF_DATA) containing a list of RDF query responses. The default value is null. - info_response_rdf_schema_url_list of type LIST(IR_SCHM_URL) containing a list of RDF schema URL. The default value is null. - info_response_rdf_schema_list of type LIST(IR_RDF_SCHM) containing a list of RDF schema query responses. The default value is null. |  |  |  |
| Step: | Test description |  |  | Verdict |  |
|  |  |  |  | Pass | Fail |
|  | PoS1 sends an MIH_Push_Information indication message to MN via Com8 |  |  |  |  |
|  | Is the message received by MN? |  |  | Yes | No |
|  | Is the message in the right format with the following information? - Source Identifier containing IS1’s MIHF ID - Destination Identifier containing MN’s MIHF ID - If info_response_binary_data_list is non-null, InfoResponseBinaryDataList containing info_response_binary_data_list - If info_response_rdf_data_list is non-null, InfoResponseRDFDataList containing info_response_rdf_data_list - InfoResponseRDFSchemaURLList containing Info_query_rdf_schema_url - If info_response_rdf_schema_url_list is non-null, InfoQueryRDFSchemaList containing info_response_rdf_schema_url_list - If info_response_rdf_schema_list is non-null, InfoResponseRDFSchemaList containing info_response_rdf_schema_list |  |  | Yes | No |

# Bibliography

- "IEEE Standard for Local and metropolitan area networks— Part 21: Media Independent Handover Services," IEEE 802 IEEE 802.21, Jan. 2009. - "Telecommunications and Internet Protocol Harmonization Over Networks (TIPHON) Release 4;Interoperability test methods and approaches;Part 1: Generic approach to interoperability testing," ETSI TS 102 237-1, Dec. 2003. [Online]. "http://portal.etsi.org/docbox/MTS/MTS/STFs/STF295/MBS/Referenced%20Documents/ts_102_237_01.pdf"

- Mapping the use cases to the test cases

The test cases can be combined to address general use cases. This section provides an example of such mappings to the FMCA use cases provided in http://mentor.ieee.org/802.21/dcn/09/21-09-0061-00-0000-fmca-use-cases.ppt. Table 5: FMCA use case numbering Table 5 provides the corresponding use case numbering to facilitate referencing the use cases later in this document.

Table 5: FMCA use case numbering

| Use Case | Title |
| --- | --- |
| FMCA-UC1 | Network controlled access technology selection |
| FMCA-UC2 | 3G to WLAN Handover |
| FMCA-UC3 | Per-flow |
| FMCA-UC4 | Resource limited backhaul |
| FMCA-UC5 | Seamless handover and service adaptation |
| FMCA-UC6 | Adaptive Handover for Application |
| FMCA-UC7 | Multiple Simultaneous Sessions (Data, Voice & Mobility) |

FMCA-UC1 can be achieved by network initiated handover or device handover using information service.  The device handover does not use a remote MIH handover command.  In the network initiated handover in which the PoS explicitly commands the user device to connect to a specific network.

Similarly, for handover from 3G to WLAN, FMCA-UC2 can be achieved by device handover using information service. For the handover from WLAN to 3G, the MIH event service (i.e., WLAN signal deteriorates,) can be addressed as an internal functionality of the device. When the event occurs, the device then performs mobile initiated handover using the information provided by the information service.

Per application flow handover is not considered in the IEEE Std. 802.21-2008. The deicision of which flow is sent out from which interface, described in FMCA-UC3, cannot be tested by MIH test cases. However, the activation to connect to a network can be addressed by MIH protocol message exchanges. This use case uses the IS query result to make decision on which WLAN to connect to. Instead of handover from 3G to WLAN, the device attaches to the WLAN network without disconnecting from the 3G network. When the application finishes transmitting or the device is out of the WLAN range, the device disconnects the WLAN connection.

FMCA-UC4 can be achieved by device performing mobile initiated handover or device handover based on IS query results that were updated by the network based on the resource availability. Alternatively, it can be acchieved by issueing a network initiated handover.

FMCA-UC5 is partially addressed by network initiated handover with the assumption that the call information and user location is accessible by the network. The application switching from Video conference to Audio conference is a software application and is not supported by 802.21. The network may utilize the MIH information stored in the IS server. For this use case, we assume that the IS server is implemented on the PoS.

FMCA-UC6 can be addressed by the user device performing a handover based on the user triggering a specific application.

FMCA-UC7 describes a use case that cannot be directly addressed by the MIH protocol. It requires MIH to interwork with a mobility protocol or a session continuity protocol.

Error: Reference source not found provides test sequences that address different FMCA use cases. Details on each test case  are described in later sections. Since these use cases contain overlapping MIH functionalities, one test case may cover mutliple use cases. Since some FMCA use cases can be achieved by using different test case sequences, one use case may be satisfied by more than one test case. For example, FMCA-UC1 can be achieved either by the MN performing handover based on the IS query results or by the network initiating a handover to a specific access technology.

Table 6: Test case sequences addressing FMCA use cases

| Test Case | Test Case Sequence | FMCA Use Case |
| --- | --- | --- |
| TC-NHO Network initiated handover | Precondition: Network contains the location information of the MN NHO-1a (Reference: Section 4.1) or NHO-1b (Reference: Section 4.2): The PoS commands the MN to handover to connect a specific PoA. | FMCA-UC1, FMCA-UC4, FMCA-UC5 |
| TC-ISDEV Device handover from 3G to WLAN using information service | - IS-1b (Section 5.2): MN query for WLAN radio via the 3G network link. - TC-DEV: MN attaches to the WLAN using the IS-1b query result. | FMCA-UC1, FMCA-UC2, FMCA-UC3, FMCA-UC4 |
| TC-DEV Device handover | Based on specific criteria, the MN connects to different network. Criteria: - WLAN signal strength deteriorates (from WLAN to 3G), - Device location (GPS) information (3G to WLAN), or - Based on user application (3G to WIMAX, or WLAN to 3G). | FMCA-UC6 |

- Interoperable Functions Statement

This part is a revised section of M8.2 Roles and M8.3 Major Capabilities in the Annex M (PICS).

  - Protocol Summary

| Protocol | MIH protocol |
| --- | --- |
| Protocol version | 1 |
| Features | MIH fragmentation |
|  | MIH acknowledgement (mandatory in the case of UDP transport) |

  - MIH entities

Table 7 lists the entities that are involved in MIH protocol exchanges. [NOTE: Figure2 in IEEE Std 802.21-2008 and its description is used. Not sure if we need to have “non-PoS” as one of the MIH entities in this section.]

Table 7 MIH Entities

| Item | MIH entities | Reference | Support |
| --- | --- | --- | --- |
|  | Mobile Node (MN) | [1]5.4.2 |  |
|  | Point of Service (PoS) | [1]5.4.2 |  |
|  | Non-PoS | [1]5.4.2 |  |
| Comments: |  |  |  |

    - Roles

Table 8: Pos Roles lists the different roles for a PoS. There is no separation of roles for other MIH entities, i.e., MN.

Table 8: Pos Roles

| Item | Role | Reference | Support |
| --- | --- | --- | --- |
|  | Source PoS |  |  |
|  | Target PoS |  |  |
|  | Information Server | [1]5.4.3 |  |
| Comments: The role “source” and “target” apply to a PoS role regarding a handover. Since a PoS is going to take each position during its usage, the capabilities are not listed separately in the corresponding clauses. |  |  |  |

    - MN and PoS Capabilities

Following are the capabilities that apply to both MN and PoS entities. [NOTE: Item and Mnemonic seems duplicating index referencing, can we just keep one? Suggest just keep Mnemonic]

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| *M.8.3.4 | Capability Discovery | [1]6.2.3 | M | Yes [ ] No [ ] | MC4 |
| *M.8.3.5 | Registration | [1]6.2.4 | MC2 OR MC3.3: M | Yes [ ] No [ ] | MC5 |
| *M.8.3.1 | Event Service | [1]6.3 | O.2 | Yes [ ] No [ ] | MC1 |
| *M.8.3.2 | Command Service | [1]6.4 | O.2 | Yes [ ] No [ ] | MC2 |
| *M.8.3.3 | Information Service | [1]6.5 | O.2 | Yes [ ] No [ ] | MC3 |
| Comment: |  |  |  |  |  |

    - MIH discovery

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| M.8.3.4.2 | Solicitated Uni-cast capability discovery | [1]8.2.4.3.4 | M | Yes [ ] No [ ] | MC4.1 |
|  | Soliciated Multi-cast capability discovery | [1]8.2.4.3.4 | O* | Yes [ ] No [ ] | MC4.2 |
| M.8.3.4.1 | Unsolicited capability discovery | [1]8.2.4.3.3 | O | Yes [ ] No [ ] | MC4.3 |
| Comment: |  |  |  |  |  |

Rationale:

- In [1] 8.2.4.3.4 the “Solicitated capability discovery” is described using either unicast or multicast to send an MIH_Capability_Request message. For this reason the “Soliciated capability discovery” capability is separated in to uni-cast and multicast.

- The “Unsolicited capability discovery” is the functionality of using media-specific broadcast control messages for discovering MIH enabled network entities.

    - Registration

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
|  | Registration | [1]6.2.4 | MC5:M | Yes [ ] No [ ] |  |
|  | Re-registration | [1]6.2.4 | MC5:O | Yes [ ] No [ ] N/A [ ] |  |
|  | Deregistration | [1]6.2.4 | MC5:M | Yes [ ] No [ ] |  |
| Comment: [Note: There’s no text describing re-registration. The term appears only in the primitive parameter description and message.] |  |  |  |  |  |

    - Command Service

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
|  | Get the status of link on a MN | [1]6.4.3.2.1 | MC2:O.5 | Yes [ ] No [ ] |  |
|  | Control the behavior of a set of links on a MN | [1]6.4.3.2.1 | MC2:O.5 | Yes [ ] No [ ] |  |
| M.8.3.6 | Mobile initiated handover | [1]6.4.3.2.3 | MC2:O.5 | Yes [ ] No [ ] | MC6 |
| M.8.3.7 | Network initiated handover | [1]6.4.3.2.4 | MC2:O.5 | Yes [ ] No [ ] | MC7 |
|  | Network resource reservation | [1]6.4.3.2.3, 6.4.3.2.4 | MC6 OR MC7:O | Yes [ ] No [ ] |  |
| Comment: M.8.3.10 is only relevant for the functional entity of an target PoS. |  |  |  |  |  |

Rationale:

- Define the different capabilities for command service. The actual PDUs are the implementation of the capability.

    - Event Notification

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
|  | Subscribe to events | [1]6.3.2.3 | MC1:M | Yes [ ] No [ ] |  |
|  | Unsubscribe to events | [1]6.3.2.3 | MC1:M | Yes [ ] No [ ] |  |
|  | Link thresholds report | [1]7.4.15.1 | MC1 and MC2:O | Yes [ ] No [ ] |  |
|  | Link notification |  |  |  |  |
| Comment: |  |  |  |  |  |

Rationale:

- The link thresholds notification capability is implemented by the MIH_Configure_Thresholds, MIH_Link_Parameter_Report in conjunction with the event subscriptions.

- Alternatively, have the MIH_Configure_Threshold, MIH_Link_Parameter_Report as a supporting group.

    - Information Service

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| M.8.3.3.1 | Binary query method | [1]6.5.6.2 | MC3:O.3 | Yes [ ] No [ ] N/A [ ] | MC3.1 |
| M.8.3.3.2 | RDF query method | [1]6.5.6.3 | MC3:O.3 | Yes [ ] No [ ] N/A [ ] | MC3.2 |
| M.8.3.3.3 | Push mode | [1]5.3.4 | MC5 AND MC3:O | Yes [ ] No [ ] N/A [ ] | MC3.3 |
| Comment: |  |  |  |  |  |

  - Protocol Data Units

This section remains the same as in IEEE Std 802.21 clause M.8.4 with updates on the Mnemonic predicate identification [Work In Progress].

| Item | Capability | Reference | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- |
| M.8.3.3.1 | MIH_Get_Parameters request | [1] | MCX | Yes [ ] No [ ] N/A [ ] | PDU8 |
| M.8.3.3.2 | MIH_Get_Parameters response | [1]6.5.6.3 | MC3:O.3 | Yes [ ] No [ ] N/A [ ] | PDU |
| M.8.3.3.3 | MIH_Link_Action request | [1]5.3.4 |  | Yes [ ] No [ ] N/A [ ] |  |
|  | MIH_Net_HO_Candidate request |  | MC6:M |  |  |
|  | MIH_Net_HO_Candidate response |  | MC6:M |  |  |