| Project | IEEE 802.21 <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Multi-Radio Power Management |  |
| Date Submitted | September, 2009 |  |
| Source(s) | Junghoon Jee (ETRI), Anthony Chan (Huawei) |  |
| Re: | IEEE 802.21 Session #34 in September 2009 |  |
| Abstract | Multi-Radio Power Management |  |
| Purpose | Multi-Radio Power Management Discussion |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Multi-Radio Power Management

Today, mobile phones and other handheld communication devices are often equipped with two or more access technologies such as WiFi (e.g., IEEE 802.11a/b/g/n), WiMAX, CDMA, UMTS, GSM and Bluetooth. We call these devices having more than two heterogeneous radio interfaces as multi-radio devices. One major bottleneck in the implementation of multi-mode terminals for any application area is the overall power management mechanism for different radios, each with its own mechanism.

Power saving by deactivating a radio interface affects the network reachability through that radio interface. One of the practical ways to minimize power consumption of multi-radio device is to deactivate an unused radio interface. For an example, WiFi radio interface can be turned off when device moves out of WiFi coverage area or there’s no active traffic through that radio interface. However, the deactivated radio interface looses network reachability thus impossible to serve incoming traffic. Therefore, we need a way to keep network reachability even for the deactivated radio interface.

Multi-Radio Power Management (MRPM) enables optimized power conservation for multi-radio devices. Previously, a powered-off radio interface looses network reachability thus can not deal with incoming traffic toward that interface. However, even the deactivated radio can be regarded as reachable by the help of another active radio in MRPM. MRPM defines multi-radio power states to provide an integrated view about the multi-radio device’s reachable state not depending on the physical individual radio’s power state. For an example, a powered-off WiFi interface’s multi-radio state can be described as “reachable via WiMAX” when a WiMAX radio interface is available and can deal with incoming traffic toward that deactivated WiFi interface. These multi-radio power states enable the individual radios to enter into or leave lower power states not affecting the network reachability. MRPM also enables power management through coordination across multiple networks of different radios using media independent services, taking into account the power management functions in individual radios and networks and QoS requirements.