| Project | IEEE 802.21 Media Independent Handover Services IEEE 802.21a: Security <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Candidate authenticator discovery mechanism |  |
| Date Submitted | Nov 13, 2009 |  |
| Source(s) | Dapeng Liu (China Moible) |  |
| Re: | IEEE 802.21 Session #35 in Atlanta |  |
| Abstract | This document introduces a mechanism used for 802.21a candidate authenticator discovery. This proposal is organized in the format of the 21 amendment and the purpose is to discuss and see how this proposal could fit in to 802.21 specifications. |  |
| Purpose | Task Group Discussion and Acceptance |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

The applicability of EAP pre-authentication and pro-active re-authentication is limited to the scenarios where the candidate authenticator(s) can be discovered prior to mobile node movement to the candidate network.

This proposal introduces the method of extending 802.21 standards to define new MIH Command and MIH Information Elements for candidate authenticator discovery.

This proposal is organized as the amendment of the base 802.21 specification and the purpose is to discuss and see how this proposal could fit in to 802.21 specifications.

# Extensions of MIH command for candidate authenticator discovery

This contribution proposes to extend 802.21 specification MIH command by the following way:

In section :

6.4.3.2.1 General

Table 7-MIH commands

Extend the following items (marked by yellow):

| MIH command | (L)ocal, (R)emote | Comments | Defined in |
| --- | --- | --- | --- |
| MIH_Net_HO_Candidate_Query | R | Network initiates handover and sends a list of suggested networks and associated points of attachment and also includes the candidate authenticator information in the candidate networks. | 7.4.17 |
| MIH_MN_HO_Candidate_Query | R | Command used by MN to query and obtain handover related information about possible candidate networks and the candidate authenticators’ information in the candidate networks. | 7.4.18 |

In section :

7.4.17.2.1 Function

The primitive is invoked by an MIH user on a network node to communicate to a peer MIH user about intent of handover initiation or the intent of acquiring the candidate network’s authenticator information that used for proactive authentication.

7.4.17.2.2 Semantics of service primitive

MIH_Net_HO_Candidate_Query.request(

DestinationIdentifier,

SuggestedNewLinkList,

CandidateAuthenticatorList,

QueryResourceReportFlag

)

Parameters:

| Name | Data type | Description |
| --- | --- | --- |
| DestinationIdentifier | MIHF_ID | This identifies a remote MIHF that will be the destination of this request. |
| SuggestedNewLinkList | LIST(LINK_POA_LIST) | A list of PoAs for each link, suggesting the new access networks to which handover initiation should be considered. The access networks towards the top of the list are more preferable that those towards the bottom of the list. |
| CandidateAuthenticatorList | LIST | A list of candidate authenticators’ information in the candidate networks. |
| QueryResourceReprotFlag | BOOLEAN | Flag to specify if resources need to be reported by MN: TURE: Required to report resource list FALSE: Not required to report resource list. |

7.4.17.2.3 When generated

This primitive is invoked by an MIH user to communicate with a remote MIH user about its intent of handover initiation or to acquire the candidate authenticator’s information in the candidate networks. Serving PoS requests MN to provide information about resources required to initiate a handover by setting the QueryResourceReportFlag parameter.

7.4.17.3.2 Semantics of Service primitive

MIH_Net_HO_Candidate_Query.indication(

SourceIdendifier,

SuggestedNewLinkList,

CandidateAuthenticatorList,

QueryResourceReportFlag

)

Parameters:

| Name | Data type | Description |
| --- | --- | --- |
| SourceIdentifier | MIHF_ID | This identifies the invoker of this primitive, which is a remote MIHF. |
| SuggestedNewLinkList | LIST(LINK_POA_LIST) | A list of PoAs for each link, suggesting the new access network to which handover initiation should be considered. The access networks towards the top of the list are more preferable than those towards the bottom of the list. |
| CandidateAuthenticatorList | LIST | A list of candidate authenticators’ information in the candidate networks. |
| QueryResourceReprotFlag | BOOLEAN | Flag to specify if resources need to be reported by MN: TURE: Required to report resource list FALSE: Not required to report resource list |

In section:

7.4.18.1.1 Function

This primitive is used by MIH users on an MN to inform MIHF to query candidates for possible handover initiation. The request includes queries on QoS resources and/or whether IP address configuration method of the ongoing data sessions can be supported in the candidate network. This primitive also includes the current IP configuration server address [e.g., DHCP server, foreign agent(FA) IP address AR IP address]when the current IP configuration method is included. This primitive also include the candidate authenticator’s information in the candidate nework.

7.4.18.1.2 Semantics of service primitive

MIH_MN_HO_Candidate_Query.request(

DestinationIdentifier,

SourceLinkIdentifier,

CandidateLinkList,

CandidateAuthenticatorList,

QoSResourceRequirments,

IPConfigurationMethods,

FA Address,

AccessRouterAddress

)

Parameters:

| Name | Date type | Description |
| --- | --- | --- |
| DestinationIdentifier | … | … |
| SourceLinkIdentifier | … | … |
| CandidateLinkList |  |  |
| CandidateAuthenticatorList | LIST | A list of candidate authenticators’ information in the candidate networks. |
| QoSResourceRequirements |  |  |
| IPConfigurationMethods |  |  |
| DHCPServerAddress |  |  |
| FAAddress |  |  |
| AccessRouterAddress |  |  |

7.4.18.2.2 Semantics of Service primitive

MIH_MN_HO_Candidate_Query.indication(

DestinationIdentifier,

SourceLinkIdentifier,

CandidateLinkList,

CandidateAuthenticatorList,

QoSResourceRequirments,

IPConfigurationMethods,

FA Address,

AccessRouterAddress

)

Parameters:

| Name | Date type | Description |
| --- | --- | --- |
| DestinationIdentifier | … | … |
| SourceLinkIdentifier | … | … |
| CandidateLinkList |  |  |
| CandidateAuthenticatorList | LIST | A list of candidate authenticators’ information in the candidate networks. |
| QoSResourceRequirements |  |  |
| IPConfigurationMethods |  |  |
| DHCPServerAddress |  |  |
| FAAddress |  |  |
| AccessRouterAddress |  |  |

# Extensions of MIH Information Element for candidate authenticator discovery

New MIH Information Element proposed to be defined that contains the candidate authenticator’s information such as its L2 or IP address, supported proactive authentication method etc.

In 802.21 base specification, section

    - Information elements

In table 10, insert the following item:

| Name of information element | Description | Data type |
| --- | --- | --- |
| PoA Specific Information Elements |  |  |
| IE_POA_AUTHENTICATOR_ADDR | The L2 address of the authenticator which serves the PoA. | LINK_ADDR |
| IE_POA_AUTHENTICATOR_METHOD_INFO | The supported proactive authentication method information of the authenticator which serves the PoA. | SYSTEM_INFO |
| PoA Specific Higher Layer Service Information Elements |  |  |
| IE_POA_AUTHENTICATOR_IP_ADDR | The IP address of the authenticator which serves the PoA. | IP_ADDR |