| Project | IEEE 802.21 Media Independent Handover Services IEEE 802.21b: Broadcast Service <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Group Handover Related MIH_SAP primitive |  |
| Date Submitted | February 27, 2010 |  |
| Source(s) | Hongseok Jeon, Junghoon Jee, Changmin Park (ETRI) |  |
| Re: | IEEE 802.21 Session #37 in Orlando, FL |  |
| Abstract | This submission contains normative text related to adding a new MIH_SAP primitive to the IEEE Std 802.21-2008. The new MIH_SAP primitive is used by an MIH user on the downlink-only network to recommend a handover to the specific group of MNs. This primitive allows the specific group of MNs to perform a group handover from the downlink-only networks to bidirectional networks. The text is aligned with the IEEE Std 802.21-2008. |  |
| Purpose | Task Group Discussion and Acceptance |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

3. Definitions

Insert the following new definitions alphabetically into clause 3:

3.x bidirectional network: The general communication network providing bidirectional transmission such as telecommunication system (3GPP, 3GPP2), mobile communication system (WiMAX, WiFi) and fixed communication system (Ethernet).

3.x DO (downlink-only) network: The mobile broadcasting network providing only downward transmission such as DVB-H, T-DMB, MediaFLO and ATSC-M/H.

3.x program: A specific instance of certain content, e.g. chapter 2 of a TV series.

3.x service: A sequence of programs under the control of a content aggregator, e.g. BBC One, BBC Two, etc.

4. Abbreviations and acronyms

Insert the following new acronyms alphabetically into clause 4:

DO				downlink-only

URI				uniform resource identifier

7.4 MIH_SAP primitives

Insert the following new subclause 7.4.27 in clause 7.4:

7.4.27 MIH_Net_Bcst_HO_Commit

7.4.27.1 MIH_Net_Bcst_HO_Commit.request

7.4.27.1.1 Function

This primitive is used by MIH users on a DO network to inform the remote MIH users on the specific group of MNs of a possible handover. This primitive is used to recommend a handover from the DO network to the bidirectional network based on the selected choices for candidate networks and PoAs. This primitive also includes service or program information to indentify a group of MNs to which the DO network recommends the handover.

7.4.27.1.2 Semantics of service primitives

MIH_Net_Bcst_HO_Commit.request    (

DestinationIdentifier,

TargetMNGroupInfo,

LinkType,

TargetNetworksInfoList,

LinkActionExecutionDelay,

LinkActionsList

)

Parameters:

| Name | Data Type | Description |
| --- | --- | --- |
| DestinationIdentifier | MIHF_ID | The remote MIHF, which is the destination of this request. |
| TargetMNGroupInfo | GROUP_INFO | Service or program information to identify the group of mobile nodes to which the DO networks recommend a handover. |
| LinkType | LINK_TYPE | Contains target link type |
| TargetNetworksInfoList | LIST(TGT_NET_INFO) | This list contains information of recommended target networks for assisting the mobile node to perform handover. |
| LinkActionExecutionDelay | UNSIGNED_INT(2) | Time (in ms) to elapse before an action needs to be taken. A value of 0 indicates that the action is taken immediately. Time elapsed is calculated from the instance the command arrives until the time when the execution of the action is carried out. |
| LinkActionList | LIST(LINK_ACTION_REQ) | A list of network controlled handover actions for the links. |

7.4.27.1.3 When generated

The MIH user generates this primitive to recommend a handover actions when a specific program or service will be no longer available on the DO network.

7.4.27.1.4 Effect on receipt

The recipient will return an MIH_Net_Bcst_HO_Commit indication message.

7.4.27.2 MIH_Net_Bcst_HO_Commit.indication

7.4.27.2.1 Function

This primitive is used by an MIHF for MIH users to perform a network-controlled handover or network-assisted handover. This primitive is the result of receipt of an MIH_Net_Bcst_HO_Commit indication message from a remote MIHF.

7.4.27.2.2 Semantics of service primitives

MIH_Net_Bcst_HO_Commit.indication (

SourceIdentifier,

TargetMNGroupInfo,

LinkType,

TargetNetworksInfoList,

LinkActionExecutionDelay,

LinkActionsList

)

Parameters:

| Name | Data Type | Description |
| --- | --- | --- |
| SourceIdentifier | MIHF_ID | The remote MIHF, which sent the MIH_Net_Bcst_HO_Commit indication message. |
| TargetMNGroupInfo | GROUP_INFO | Service or program information to identify the group of mobile nodes to which the DO networks recommend a handover. |
| LinkType | LINK_TYPE | Contains target link type |
| TargetNetworksInfoList | LIST(TGT_NET_INFO) | This list contains information of recommended target networks for assisting the mobile node to perform handover. |
| LinkActionExecutionDelay | UNSIGNED_INT(2) | Time (in ms) to elapse before an action needs to be taken. A value of 0 indicates that the action is taken immediately. Time elapsed is calculated from the instance the command arrives until the time when the execution of the action is carried out. |
| LinkActionList | LIST(LINK_ACTION_REQ) | A list of network controlled handover actions for the links. |

7.4.27.2.3 When generated

This primitive is generated by an MIHF upon receiving an MIH_Net_Bcst_HO_Commit indication message.

7.4.27.2.4 Effect on receipt

Upper layer entities take the applicable actions in the Link Action List.

8.6.3 MIH messages for command service

Insert the following new subclause 8.6.3.23 into clause 8.6.3:

8.6.3.23 MIH_Net_Bcst_HO_Commit indication

The corresponding MIH primitive of this message is defined in 7.4.27.1.

This message is used by the MIHF on the DO network to recommend a possible handover to a specific link and PoA.

The requesting MIHF entity may fill its destination MIHF ID with a multicast MIHF ID to send this message.

MIH Header Fixed Fields (SID=3, Opcode=3, AID=12)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

TargetMNGroupInfo (Target mobile node group info TLV)

LinkType (Link type TLV)

TargetNetworksInfoList (List of target netework info TLV)

LinkActionExecutionDelay (Time interval TLV)

LinkActionList (Link actions list TLV)

F.3 Derived data types

Insert the new rows into Table F.3 in Annex F.3:

Table. F.3—Data type for address

| Data type name | Derived from | Definition |
| --- | --- | --- |
| BCST_PROG_ID | OCTET_STRING | A type to represent the URI of a program that will be no longer available on DO networks. |
| BCST_SERVICE_ID | OCTET_STRING | A type to represent the URI of a service that will be no longer available on DO networks. |

Insert the following new subclause F.3.16 in Annex F.3:

F. 3. 16 Data type for group information

Table. F.24—Data type for group information

| Data type name | Derived from | Definition |
| --- | --- | --- |
| GROUP_INFO | CHOICE( BCST_PROG_ID, BCST_SERVICE_ID ) | A type to represent target MN group information. |

Annex L

(normative)

MIH protocol message code assignments

Insert the new row under MIH messages for command service into Table L.1 in Annex L:

Table. L.1—AID assignment

| MIH messages | AID |
| --- | --- |
| MIH messages for Command Service |  |
| MIH_Net_Bcst_HO_Commit | 12 |

Insert the new rows into Table L.2 in Annex L:

Table. L.2—Type values for TLV encoding

| TLV type name | TLV type value | Data type |
| --- | --- | --- |
| Broadcast program ID | 64 | BCST_PROG_ID |
| Broadcast service ID | 65 | BCST_SERVICE_ID |
| Target MN group information | 66 | GROUP_INFO |

Currently, IEEE 802.21-2008 has defined 11 action identifiers (AIDs) for MICS messages.