| Project | IEEE 802.21 Media Independent Handover Services IEEE 802.21a: Security <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Text of the option A of work item I: Proactive Authentication Through EAP (MSA is the authenticator) |  |
| Date Submitted | July 28, 2010 |  |
| Source(s) | Dapeng Liu (China Mobile) |  |
| Re: | IEEE 802.21a |  |
| Abstract | This document is used for down-selection discussion in July meeting for option A of work item I. |  |
| Purpose | Task Group Discussion |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

## Scope:

The scope of this document is to give detail text description of the option A of work item I in the summary of the current proposals in IEEE802.21a (21-10-0049-03-0sec-proposal-summary). The purpose of this proposal is for down-selection in July meeting.

## Purpose:

The purpose of this document is for the task group discussion regarding to the option A of work item I (Proactive authentication through EAP; MSA is the authenticator).

## Terminologies

EAP: Extensible Authentication Protocol

ERP : EAP Re-Authentication Protocol

SA : Serving Authenticator

CA : Candidate Authenticator

## Definitions

Authentication Process: The cryptographic operations and supporting data frames that perform the authentication

Proactive Authentication:  An authentication process that is performed between  MIA-KH and other entities attached to the other end of a link of a MSA-KH. This process occurs when the other entities intend to perform a handover to another link.

MIH Assisted Proactive Authentication: MIH (Media Independent Handover Service) could be used to assist proactive authentication. For example, MIH PoS may help to forward the proactive authentication message to the corresponding media specific authenticator. MIH could be used for candidate media specific authenticator discovery, etc. MIH Assisted Proactive Authentication means the procedure that using MIH service to assist the performance of proactive authentication.

Media Specific Authenticator and Key Holder (MSA-KH): Media specific authenticator and key holder is an entity that facilitates authentication of other entities attached to the other end a link specific to a media.

MIH Security Association (SA): An MIH SA is the security association between the peer MIH entities

Media AS: Authentication server of the specific media

## References

[IEEE802.21]   IEEE P802.21 Std-2008, IEEE Standard for Local and Metropolitan Area Networks- Part 21: Media Independent Handover Services.

# Architecture overview

Figure 1 Architecture of MIH Assisted Proactive EAP Authentication

Figure 1 illustrates the architecture of MIH assisted proactive EAP authentication. There are mainly four entities in the architecture:

MN: The Mobile Node is assumed to have MIH function.

PoS: PoS is the IEEE 802.21 point of service which provides MIH function and service. This document intends to extend the current IEEE 802.21 base specification to provide assistant for proactive authentication during handover. The purpose of proactive authentication is to reduce the authentication delay during handover.

There are two classes of proactive authentication: proactive authentication and EAP re-authentication (ERP). There are two forms to preform proactive authentication: direct and indirect mode.

The PoS could provide assistants for the proactive authentication. Such as: deliver proactive authentication message to the media specific candidate authenticator; help to perform candidate media specific authenticator discovery etc.

PoA: this is the candidate media specific authenticator that the MN may handover to after handover execution.

AAA: this is the EAP authentication server.

# Function description and procedures

# MN function description

The MN is assumed having MIH function. The MIHF also need to be extended to support proactive authentication signaling. The proactive authentication signaling is carried using extended MIH protocol which is running between MN and enhanced PoS.

The MN need to support the function of candidate media specific authenticator discovery.

# Enhanced PoS function description

The function of enhanced PoS is composed by the following:

- Receive MIH message which carries proactive EAP authentication frames from MN.

- Decapsulate the MIH message that carries the proactive authentication frames then forward the EAP frames to the candidate media specific authenticator. The protocol that used to carry EAP proactive authentication frames between PoS and media specific authenticator need to be further specified.

# Procedures of MIH assisted proactive authentication

The procedure of MIH assistant proactive authentication is consisted of the following steps:

1.  POS and Candidate Media Specific Authenticator Discovery

Before MN initiates MIH assisted proactive authentication, MN needs to know the PoS’s address and the candidate media specific authenticator’s MAC addresses.  The discovery of PoS’s address has already been specified by IEEE 21 base specification. The discovery of corresponding media specific candidate authenticator’s address could be done by using extended IEEE802.21 information service.

- Perform of Proactive EAP authentication

The proactive EAP authentication or ERP message could be encapsulated in to the extended MIH messages as L2 frames. When the PoS receives the encapsulated MIH message, it decapsulates it, then forwards the EAP message to the media specific candidate authenticator. The EAP message could be encoded as OCTET_STRING, the PoS needs not to implement EAP protocol; It may simply forwards the EAP messages to the media specific authenticator.
After successful proactive EAP authentication, the MN and AS derives the related keying material and candidate media specific authenticator can get the keying material from the AS using AAA protocol. The packet format of the MIH assisted EAP proactive authentication is depicted in figure 3.

- Security association

When the MN decide to handover to the candidate network, the MN and candidate media specific authenticator perform security association based on the keying material derived by the proactive EAP authentication. For example, this could be 4-way handshake in 802.11i.

MN

PoS

PoA

AS

Information Server

- Candidate authenticator discovery

2. MN or network initiates proactive EAP authentication

3. Security association

Figure 2 Procedure of MIH assisted proactive EAP authentication

Figure 3 Proactive EAP authentication packet format

# Extensions of IEEE802.21 Specification

# 4.1 Extension used for candidate media specific authenticator discovery

There are two possible ways proposed for media specific authenticator discovery:

- Extending Command Service

MIH_Net_HO_Candidate_Query and MIH_MN_HO_Candidate_Query command are extended for candidate media specific authenticator discover.

When the UE or network prepares for handover, it may use the MIH_Net_HO_Candidate_Query or MIH_MN_HO_Candidate_Query to get the candidate network information. This command is proposed to extend to  return candidate media specific authenticator’s information, such as media specific candidate authenticator’s IP address and L2 address.

| MIH command | (L)ocal, (R)emote | Comments | Defined in |
| --- | --- | --- | --- |
| MIH_Net_HO_Candidate_Query | R | Network initiates handover and sends a list of suggested networks and associated points of attachment and also includes the media specific candidate authenticator information in the candidate networks. | 7.4.17 |
| MIH_MN_HO_Candidate_Query | R | Command used by MN to query and obtain handover related information about possible candidate networks and the candidate media specific authenticators’ information in the candidate networks. | 7.4.18 |

- Detail extension of MIH_MN_HO_Candidate_Query.reques:

MIH_MN_HO_Candidate_Query.request(

DestinationIdentifier,

SourceLinkIdentifier,

CandidateLinkList,

QoSResourceRequirments,

IPConfigurationMethods,

FA Address,

AccessRouterAddress，

PreAuthenticationFlg

)

MIH_MN_HO_Candidate_Query.response(

DestinationIdentifier,

Status,

SourceLinkIdentifier,

PreferedCandidateLinkList,

PreferedCandidateAuthenticator

)

PreAuthenticationFlg is a BOOLEAN value, if this value is TURE, it expects that the MIH_MN_HO_Candidate_Query.response returns the corresponding candidate media specific authenticotor’s address of the prefered candidate PoAs. The candidate media specific authenticator’s address is carried by the PreferedCandidateAuthenticator parameter.

- Detail extension of MIH_Net_HO_Candidate_Query:

MIH_Net_Ho_Candidate_Query.request(

DestinationIdentifier,

SuggestedNewLinkList,

SuggestedNewLinkCandidateAuthenticatorList,

QueryResourceReportFlag

)

MIH_Net_HO_Candidate_Query.reponse(

DestinationIdentifier,

Status,

SourceLinkIdentifier,

HandoverStatus,

PerferredLinkList,

PreferedCandidateAuthenticator,

)

SuggestedNewLinkCandidateAuthenticatorList is the media specific authenticator’s address for the suggested candidate PoAs. The PreferedCandidateAuthenticator indicates the corresponding media specific authenticator’s address of the prefered candidate PoAs.

- Extending Information Service

The candidate media specific authenticator information could be provided by MIH information service. The following extensions are proposed for the information service to provide candidate media specific authenticator discovery.

The following Information element is extended to provide candidate media specific authenticator information.

| Name of information element | Description | Data type |
| --- | --- | --- |
| Access network specific information elements |  |  |
| IE_SEC_OPEN_AUTH | Whether the security policy allows open authentication. | BOOLEAN |
| IE_SEC_PASSWORD | Whether the security policy allows password based authentication. | TBD |
| IE_SEC_CA | Certificate authority ID | TBD |
| PoA Specific Information Elements |  |  |
| IE_POA_AUTHENTICATOR_ADDR | The L2 address of the authenticator which serves the PoA. | LINK_ADDR |
| IE_PoA_PoS_IP_ADDR | PoS’s IP address; this PoS is the serving PoS of the PoA | LINK_ADDR |
| IE_POA_AUTHENTICATOR_METHOD_INFO | The supported proactive authentication method information of the authenticator which serves the PoA. | SYSTEM_INFO |
| IE_SEC_AUTH_PROTOCOL | Which authentication protocol is supported for access authentication | TBD |
| IE_SEC_EAP_REAUTH | Whether to support re-authentication | BOOLEAN |
| IE_SEC_EAP_PREAUTH | Whether to support pre-authentication | BOOLEAN |
| PoA Specific Higher Layer Service Information Elements |  |  |
| IE_POA_AUTHENTICATOR_IP_ADDR | The IP address of the authenticator which serves the PoA. | IP_ADDR |
| IE_PoA_PoS_IP_ADDR | PoS’s IP address; this PoS is the serving PoS of the PoA | IP_ADDR |
| IE_SEC_MOBIKE | Whether to support MOBIKE | BOOLEAN |

# 4.2 Extensions used for carry EAP proactive authentication frames

# 4.2.1 MIH Command Extension

## Following table describes the MIH commands.

| MIH Commands | MIH Command type | Description |
| --- | --- | --- |
| MIH_Pro_Auth_Start | Remote | Starting Proactive authentication |
| MIH_Pro_Auth _Request | Remote | Carry proactive EAP signaling as L2 frame |
| MIH_Pro_Auth _Response | Remote | Carry proactive EAP signaling as L2 frame |

# 4.2.2 MIH Protocol Extension

- New MIH message is proposed to carry the EAP message over MIH.

MIH New Message Types

| Message name | Action ID |
| --- | --- |
| MIH_Pro_Auth_Start | TBD |
| MIH_Pro_auth Request | TBD |
| MIH_Pro_auth Response | TBD |

- MIH_Pro_Auth_Start

MIH Header Fields (SID=3, Opcode=3, AID-xx)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

Auth_Start TLV

- MIH_Pro_Auth Request

MIH Header Fields (SID=3, Opcode=1, AID-xx)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

Proactive EAP message Frame TLV

- MIH_Pro_Auth Response

MIH Header Fields (SID=3, Opcode=2, AID-xx)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

Proactive EAP message Frame TLV

- Encoding of proactive EAP message frame

The EAP message frame could be encoded as OCTET_STRING;

# 4.2.3 Protocol between PoS and PoA

New protocol need to be defined to carry EAP frame between PoS and PoA. Whether this protocol need to be specified in 21a’s scope and the details of this protocol need to be further studied.