| Project | IEEE 802.21 Media Independent Handover Services IEEE 802.21c: Single Radio Handover <http://www.ieee802.org/21/> |
| --- | --- |
| Title | IEEE 802.21c Proposal: Additional Information Elements for Single Radio Handover |
| Date Submitted | July 8, 2010 |
| Source(s) | Hongseok Jeon, Junghoon Jee, Changmin Park, Yoon-Young An (ETRI) |
| Re: | IEEE 802.21 Session #39 in San Diego, CA |
| Abstract | This proposal introduces a set of information elements to carry information on SRHO capability and SFF. |
| Purpose | Task Group Discussion and Acceptance |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |

# Introduction

This proposal is a contribution for the 802.21c in response to 802.21-10/0023r1-srho-802-21c-proposal-guidelines. It covers the network discovery and selection requirements presented in 802.21-10/0017r2-srho-requirements. Especially, this proposal satisfies the following functional requirements in the network discovery and selection:

- Define mechanisms that assist the mobile device to detect the presence of SFF and their services before attaching to the access network.

- Define mechanisms that assist the mobile device to know if the network supports Single radio HO.

This proposal tries to address the above functional requirements by utilizing the media independent information service (MIIS). Hence this proposal introduces a new information element container, IE_CONTAINER_SFF, which includes information elements for SFF. Also it expands an existing information element, IE_NET_CAPABILITIES, to provide capability for supporting SRHO.

This proposal is based on the document structure depicted in 802.21-10/0025r2-srho-802-21c-draft-template.

IEEE Standard for

Local and metropolitan area networks—

Part 21: Media Independent Handover Services

Amendment: Optimized Single Radio Handovers

Abstract: This amendment specifies the extensions to IEEE Standard 802.21-2008 for optimized single radio handovers to reduce the latency during handovers between heterogeneous access networks.

Keywords:

IEEE Standard for

Local and metropolitan area networks—

Part 21: Media Independent Handover

Services

Amendment: Optimized Single Radio Handovers

# Overview

## 1.3 General

# Normative references

# Definitions

# Abbreviations and acronyms

SFF		Signal Forwarding Function

SRHO		Single Radio Handover

# General architecture

# MIH Services

## 6.3 Media independent event service

## 6.4 Media independent command service

## 6.5 Media independent information service

6.5.1 Information elements

The Information Server provides the Signal Forwarding Function (SFF) information and the capability for supporting SRHO for each of the available access networks. The SFF information includes SFF addressing information and tunnel management protocol information.

Table 1 shows information element containers. The containers are used in the type-length-value (TLV) based query method.

Table 1—Information element container

| Name of container | Description |
| --- | --- |
| IE_CONTAINER_NETWORK | Access Network Container, containing information that depicts an access network. |
| IE_CONTAINER_SFF | SFF Container, containing information that depicts a SFF. |

Table 2 represents the list of Information Elements and their semantics. Each Information Element has an abstract data type (see Annex A for detailed definitions).

Table 2 – Information Element

| Name of information element | Description |  | Data type |
| --- | --- | --- | --- |
| Access network specific information elements |  |  |  |
| IE_NET_CAPABILITIES | Bitmap of access network capabilities. | NET_CAP |  |
| Signal Forwarding Function information elements |  |  |  |
| IE_SFF_IP_ADDR | IP address of SFF |  | IP_ADDR |
| IE_SFF_TUNN_MGMT_PRTO | Type of tunnel management protocol supported. |  | IP_TUNN_MGMT |
| IE_SFF_FQDN | FQDN of SFF. |  | FQDN |

6.5.2 IE Containers

In the binary representation method, the Information Element Containers are defined, namely the IE_CONTAINER_NETWORK and the IE_CONTAINER_SFF:

- IE_CONTAINER_NETWORK—contains all the information depicting an access network, as shown in Table 3.

Table 3 – IE_CONTAINER_NETWORK definition

| Information element ID = (see Table B.1) | Length = variable |
| --- | --- |
| IE_NET_CAPABILITIES |  |

- IE_CONTAINER_SFF – contains all the information depicting a SFF as shown in Table 4.

Table 4 – IE_CONTAINER_SFF definition

| Information element ID = (see Table B.1) | Length = variable |
| --- | --- |
| IE_SFF_IP_ADDR |  |
| IE_SFF_TUNN_MGMT_PRTO |  |
| IE_SFF_FQDN |  |

# General architecture

# MIH Services

Annex A

(normative)

Data type definition

A.1 Derived data types

Table A.1—Data types for information elements

| Data type name | Derived from | Definition |
| --- | --- | --- |
| NET_CAPS | BITMAP(32) | These bits provide high level capabilities supported on a network. Bitmap Values: Bit 0: Security – Indicates that some level of security is supported when set. Bit 1: QoS Class 0 – Indicates that QoS for class 0 is supported when set Bit 2: QoS Class 1 – Indicates that QoS for class 1 is supported when set Bit 3: QoS Class 2 – Indicates that QoS for class 2 is supported when set; Otherwise, no QoS for class 2 support is available. Bit 4: QoS Class 3 – Indicates that QoS for class 3 is supported when set; Otherwise, no QoS for class 3 support is available. Bit 5: QoS Class 4 – Indicates that QoS for class 4 is supported when set; Otherwise, no QoS for class 4 support is available. Bit 6: QoS Class 5 – Indicates that QoS for class 5 is supported when set; Otherwise, no QoS for class 5 support is available. Bit 7: Internet Access – Indicates that Internet access is supported when set; Otherwise, no Internet access support is available. Bit 8: Emergency Services – Indicates that some level of emergency services is supported when set; Otherwise, no emergency service support is available. Bit 9: MIH Capability – Indicates that MIH is supported when set; Otherwise, no MIH support is available. Bit 10: SRHO Capability – Indicates that SRHO is supported when set; Otherwise, no SRHO support is available. Bit 11–31: (Reserved) |
| IP_ADDR | TRANSPORT_ADDR | Represents an IP address. The Address Type is either 1 (IPv4) or 2 (IPv6). |
| IP_TUNN_MGT | BITMAP(16) | Indicates the supported tunnel management protocol on SFF. Bitmap Values: Bit 0: IPsec Bit 1–15: (Reserved) |
| FQDN | OCTET_STRING | The fully qualified domain name of a host as described in IETF RFC 2181. |

Annex B

(normative)

# Information element identifiers

Table B.1—Information element identifier values

| Name of information element or container | IE Identifier |
| --- | --- |
| IE_NET_CAPABILITIES | 0x1000010A |
| IE_SFF_IP_ADDR | 0x10000206 |
| IE_SFF_TUNN_MGMT_PRTO | 0x10000207 |
| IE_SFF_FQDN | 0x10000208 |
| IE_CONTAINER_NETWORK | 0x10000301 |
| IE_CONTAINER_SFF | 0x10000303 |