| Project | IEEE 802.21 Media Independent Handover Services IEEE 802.21a: Security <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Key derivation in 802.21a |  |
| Date Submitted | September 20, 2010 |  |
| Source(s) | Lily Chen (NIST) |  |
| Re: | Teleconference September 28, 2010 |  |
| Abstract | This document suggests a key derivation procedure for 802.21a based on proposal in document 21-10-0078-07. |  |
| Purpose | Task Group Discussion |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.21. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# MIH session key derivation

This document suggests a description of key derivation to derive MIIK and MIEK based on the selected ciphersuite.

We will introduce the following parameters:

- MSK or rMSK – master session key established through an EAP or EAP re-authentication. When the MSK or rMSK is 128 bits long, the PRF used for key derivation can be HMAC-SHA1, HMAC-SHA-256, or AES-CMAC. But if MSK is longer than 128 bits, then HMAC-SHA-1 or HMAC-SHA-256 shall be used as a PRF in key derivation.

- L – the length of keying material. L = |MIIK| + |MIEK|, that is, the sum of the binary length of MIIK and MIEK.

- h – The output length of the PRF used in key derivation.

  - For HMAC-SHA-1, h = 160

  - For HMAC-SHA-256, h = 256

  - For CMAC-AES, h = 128

- n = the number of iterations of PRF in order to generate L bit keying material.

The ciphersuites are coded as follows:

| Code | Integrity Algorithm | Encryption Algorithm | L (bits) |
| --- | --- | --- | --- |
| 00000000 | NULL | NULL | 0 |
| 00000001 | AES_CBC | NULL | 128 |
| 00000010 | AES_CBC | HMAC-SHA1-96 | 256 |
| 00000011 | AES_CBC | CMAC-AES | 256 |
| 00000100 | NULL | HMAC_SHA1-96 | 128 |
| 00000101 | NULL | CMAC_AES | 128 |
| 00000110 | AES_CCM |  | 128 |

The key derivation for MIH session keys can be described by the following procedure:

Fixed values:

    - h - The length of the output of the PRF in bits, and

    - t - The length of the binary representation of the counter I and L. A default value of t is 32.

Input: K = MSK or rMSK, Nonce-P, Nonce-S, and L.

Process:

- n: = L/h.

- If n > 2t-1, then indicate an error and stop.

- result(0):= Æ

- For i = 1 to n, do

  - K(i) :=  PRF (K, “MISK” || [i]2 || Nonce-P || Nonce-S || ciphersuite code ||  [L]2)

  - result (i) :=  result(i-1) || K(i)

7.  Return: MISK, i.e., the leftmost L bits of result(n).

Output: MISK.

The MISK is parsed in such a way that

MISK = MIIK || MIEK.

# MS root key derivation

In bundle case, an MS root key is derived from MSK or rMSK.  Then from the MS root key, it can derive multiple MS MSKs for different PoAs.

It is assumed that the same PRF is used for both MIH session keys and for MS root key. When the MSK or rMSK is 128 bits long, the PRF used for key derivation can be HMAC-SHA1, HMAC-SHA-256, or AES-CMAC. But if MSK is longer than 128 bits, then HMAC-SHA-1 or HMAC-SHA-256 shall be used as a PRF in key derivation.

The MS-root key can be derived by the following procedure.

Input: K = MSK or rMSK, Nonce-P, Nonce-S, and L.

Process:

- MS-RK :=  PRF (K, “MSROOTKEY” ||  Nonce-P || Nonce-S )

- Return: MS-RK.

Output: MS-RK.

.MS-RK can be 128 bits, 160 bits or 256 bits depending on the PRF used in the above.

# MS-PMK derivation

Multiple MS-PMK can be derived from MS root key for different PoAs. Each PoA is defined by its LINK layer address.

It is assumed that the same PRF is used for both MIH session keys and for MS root key. When the MSK or rMSK is 128 bits long, the PRF used for key derivation can be HMAC-SHA1, HMAC-SHA-256, or AES-CMAC. But if MSK is longer than 128 bits, then HMAC-SHA-1 or HMAC-SHA-256 shall be used as a PRF in key derivation.

The MS-root key can be derived by the following procedure.

Input: MS-RK, MN_LINK_ID, POA_LINK_ID.

Process:

- MS-PMK :=  PRF (MS-RK, “MS-PMK” || MN_LINK_ID || POS_LINK_ID )

- Return: MS-PMK.

Output: MS-PMK.

.MS-PMK can be 128 bits, 160 bits or 256 bits depending on the PRF used in the above.  The MS-PMK will be distributed to the PoA identified by POA_LINK_ID.  It will be used by the PoA to derive media specific session keys. The key derivation for media specific session keys is out of the scope of this standard.