| Project | IEEE 802.21c <https://mentor.ieee.org/802.21> |
| --- | --- |
| Title | Proactive pull key distribution through target PoS |
| DCN | 21-12-0020-01-srho |
| Date Submitted |  |
| Source(s) | Yoshihiro Ohba, Rafa Marin, Fernando Bernal, Antonio de la Oliva |
| Re: |  |
| Abstract | This contribution addresses the required extensions to support proactive pull key distribution. |
| Purpose | Proposes changes in the current draft |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |

Note to the editor: The two next figures must be in annex N, they are presented here for better understanding of the rest of the contribution.

Annex N

This first call flow shows how the identity is bootstrapped by TPoS, how the MNmirk is sent by the sPoS to the tPoS and how the MNMSRK is installed into the TPoS (AAA).

In the second one, the authentication between the MN and the tPoA is depicted. MNmsrk is installed on the tPoS in the bootstrap process in the first figure. After that, tPoS holds the MNmsrk and uses it for media-specific authentication in the next figure.  Therefore, another MNmsrk transfer is not needed in the next figure.

In the case the MN can directly contact the tPoS (this case is the same as when sPoS and tPoS are the same entity) the following example diagram applies for authentication at the tPoA:

Finally when the sPoS and tPoS are the same entity and the MIH_LL_Transfer is used to exchange L2 frames (no authentication related), the following example diagram can be applied:

7.4.29 MIH_LL_Transfer

The primitives are used to transport media specific link-layer frames between the MIHF located at the terminal and a target PoS. The target PoS may or may not be directly accessible, in this case the link-layer frames can optionally be relayed through the serving PoS.

7.4.29.1 MIH_LL_Transfer.request

7.4.29.1.1 Function

This primitive carries link-layer frames between the MN and the serving PoS.

7.4.29.1.2 Semantics of service primitive

MIH_LL_Transfer.request (

DestinationIdentifier,

LinkIdentifier,

LLInformation,

SFFIdentifier

)

Parameters:

| Name | Data type | Description |
| --- | --- | --- |
| DestinationIdentifier | MIHF_ID | This identifies a remote MIHF that will be the destination of this request. |
| LinkIdentifier | LINK_TUPLE_ID | This identifies the remote PoA that is the corresponding peer of the L2 exchange. |
| LLInformation | LL_FRAMES | This carries link layer frames. |
| SFFIdentifier | SFF_ID | (Optional) This identifies the target PoS (SFF) that will be the destination of the link-layer frames. |

7.4.29.1.3 When generated

This primitive is generated by an MIH user to start an authentication and association process based on link-layer frames.

7.4.29.1.4 Effect on receipt

After reception of this primitive, the MIHF must generate an MIH_LL_Transfer request message towards the MIHF of the serving PoS, which must rely the link-layer frames transported in this message to the target PoS.

7.4.29.2 MIH_LL_Transfer.indication

7.4.29.2.1 Function

This primitive is used by the remote MIHF to notify the corresponding MIH user about the reception of an MIH_LL_Transfer request message.

7.4.29.2.2 Semantics of service primitive

MIH_LL_Transfer.indication (

SourceIdentifier,

LinkIdentifier,

LLInformation,

SFFIdentifier

)

Parameters:

| Name | Data type | Description |
| --- | --- | --- |
| SourceIdentifier | MIHF_ID | This identifies the invoker, which is a remote MIHF. |
| LinkIdentifier | LINK_TUPLE_ID | This identifies the remote PoA that is the corresponding peer of the L2 exchange. |
| LLInformation | LL_FRAMES | This carries link layer frames. |
| SFFIdentifier | SFF_ID | (Optional) This identifies the target SFF |

7.4.29.2.3 When generated

This primitive is generated by a remote MIHF after receiving an MIH_LL_Transfer request message.

7.4.29.2.4 Effect on receipt

The MIH user must generate an MIH_LL_Transfer.response primitive or invoke an MIH_N2N_LL_Transfer.request primitive to send an MIH_N2N_LL_Transfer request message to the MIHF on the target PoS before invoking the MIH_LL_Transfer.response primitive.

7.4.29.3 MIH_LL_Transfer.response

7.4.29.3.1 Function

This primitive is used by an MIH user to provide the link-layer frames to the local MIHF.

7.4.29.3.2 Semantics of service primitive

MIH_LL_Transfer.response (

DestinationIdentifier,

LinkIdentifier,

LLInformation,

MNnetworkaccessid,

Status

)

Parameters:

| Name | Data type | Description |
| --- | --- | --- |
| DestinationIdentifier | MIHF_ID | This identifies a remote MIHF that will be the destination of this response. |
| LinkIdentifier | LINK_TUPLE_ID | This identifies the remote PoA that is the corresponding peer of the L2 exchange. |
| LLInformation | LL_FRAMES | This carries link layer frames. |
| MNnetworkaccessid | NAI | (Optional) This carries the MN’s Network Access Identifier in the case optimized pull key distribution is used |
| Status | STATUS | Status of the operation. |

7.4.29.3.3 When generated

This primitive is generated after receiving an MIH_LL_Transfer.indication primitive and possibly after receiving an MIH_N2N_LL_Transfer.confirm primitive from the local MIHF.

7.4.29.3.4 Effect on receipt

The local MIHF must generate an MIH_LL_Transfer response message in order to provide the required information until the authentication or association process is finished.

7.4.29.4 MIH_LL_Transfer.confirm

7.4.29.4.1 Function

This primitive is used to notify the corresponding MIH user about the reception of an MIH_LL_Transfer response message.

7.4.29.4.2 Semantics of service primitive

MIH_LL_Transfer.confirm (

SourceIdentifier,

LinkIdentifier,

LLInformation,

MNnetworkaccessid,

Status)

Parameters:

| Name | Data type | Description |
| --- | --- | --- |
| SourceIdentifier | MIHF_ID | This identifies the invoker, which is a remote MIHF. |
| LinkIdentifier | LINK_TUPLE_ID | This identifies the remote PoA that is the corresponding peer of the L2 exchange. |
| LLInformation | LL_FRAMES | This carries link layer frames. |
| MNnetworkaccessid | NAI | (Optional) This carries the MN’s Network Access Identifier in the case optimized pull key distribution is used |
| Status | STATUS | Status of the operation. |

7.4.29.4.3 When generated

This primitive is generated by the local MIHF after receiving an MIH_LL_Transfer response message.

7.4.29.4.4 Effect on receipt

The MIH user on the MN may generate an MIH_LL_Transfer.request primitive unless the authentication or association processes are completed.

7.4.30 MIH_N2N_LL_Transfer

The primitives defined are to transport media link-layer frames over MIH between the serving PoS and the target PoS. The media specific authentication is conducted with the media specific authenticator deployed in the target PoA.

7.4.30.1 MIH_N2N_LL_Transfer.request

7.4.30.1.1 Function

This primitive is used to transport link layer frames between the serving PoS and the target PoS (SFF).

7.4.30.1.2 Semantics of Service Primitive

MIH_N2N_LL_Transfer.request(

DestinationIdentifier,

LinkIdentifier,

LLInformation,

MNmirk

)

Parameters:

| Name | Data type | Description |
| --- | --- | --- |
| DestinationIdentifier | MIHF_ID | This identifies a remote MIHF that will be the destination of this request. |
| LinkIdentifier | LINK_TUPLE_ID | This identifies the remote PoA that is the corresponding peer of the L2 exchange. |
| LLInformation | LL_FRAMES | This carries link layer frames. |
| MNmirk | SEQUENCE( MIHF_ID, KEY) | (Optional) MN’s Media Independent Root Key to be transferred to the target PoS, and MIHF_ID of the MN to identify it. |

7.4.29.1.3 When generated

This primitive is generated by the serving PoS to relay link layer frames to the target PoS when the serving PoS receives an MIH_LL_Transfer request message.

7.4.29.1.4 Effect on receipt

The local MIHF shall generate an MIH_N2N_LL_Transfer request message to the remote MIHF.

7.4.30.2 MIH_N2N_LL_Transfer.indication

7.4.30.2.1 Function

This primitive is used by the local MIHF to notify the corresponding MIH user of the reception of an MIH_N2N_LL_Transfer request message.

7.4.30.2.2 Semantics of service primitive

MIH_N2N_LL_Transfer.indication (

SourceIdentifier,

LinkIdentifier,

LLInformation,

MNmirk

)

Parameters:

| Name | Data type | Description |
| --- | --- | --- |
| SourceIdentifier | MIHF_ID | This identifies the invoker, which is a remote MIHF. |
| LinkIdentifier | LINK_TUPLE_ID | This identifies the remote PoA that is the corresponding peer of the L2 exchange. |
| LLInformation | LL_FRAMES | This carries link layer frames. |
| MNmirk | SEQUENCE( MIHF_ID, KEY) | (Optional) MN’s Media Independent Root Key to be transferred to the target PoS, and MIHF_ID of the MN to identify it. |

7.4.30.2.3 When generated

This primitive is generated by the local MIHF after receiving an MIH_N2N_LL_Transfer request message.

7.4.30.2.4 Effect on receipt

The MIH user must generate an MIH_N2N_LL_Transfer.response primitive.

7.4.30.3 MIH_N2N_LL_Transfer.response

7.4.30.3.1 Function

This primitive is used by an MIH user to provide the link-layer frames to the local MIHF.

7.4.30.3.2 Semantics of service primitive

MIH_N2N_LL_Auth.response (

DestinationIdentifier,

LinkIdentifier,

LLInformation,

MNnetworkaccessid,

Status

)

Parameters:

| Name | Data type | Description |
| --- | --- | --- |
| DestinationIdentifier | MIHF_ID | This identifies a remote MIHF that will be the destination of this response. |
| LinkIdentifier | LINK_TUPLE_ID | This identifies the remote PoA that is the corresponding peer of the L2 exchange. |
| LLInformation | LL_FRAMES | This carries link layer frames. |
| MNnetworkaccessid | NAI | (optional)This carries the MN’s Network Access Identifier in the case optimized pull key distribution is used |
| Status | STATUS | Status of the operation. |

7.4.30.3.3 When generated

This primitive is generated after receiving an MIH_N2N_LL_Transfer.indication primitive.

7.4.30.3.4 Effect on receipt

The local MIHF must generate an MIH_N2N_LL_Transfer response message in order to provide the required information until the authentication is finished.

7.4.30.4 MIH_N2N_LL_Transfer.confirm

7.4.30.4.1 Function

This primitive is used to notify the corresponding MIH user about the reception of an MIH_N2N_LL_Transfer response message.

7.4.29.4.2 Semantics of service primitive

MIH_N2N_LL_Transfer.confirm (

SourceIdentifier,

LinkIdentifier,

LLInformation,

MNnetworkaccessid,

Status

)

Parameters:

| Name | Data type | Description |
| --- | --- | --- |
| SourceIdentifier | MIHF_ID | This identifies the invoker, which is a remote MIHF. |
| LinkIdentifier | LINK_TUPLE_ID | This identifies the remote PoA that is the corresponding peer of the L2 exchange. |
| LLInformation | LL_FRAMES | This carries link layer frames. |
| MNnetworkaccessid | NAI | (optional)This carries the MN’s Network Access Identifier in the case optimized pull key distribution is used |
| Status | STATUS | Status of the operation. |

7.4.30.4.3 When generated

This primitive is generated by the remote MIHF after receiving an MIH_N2N_LL_Transfer response message.

7.4.30.4.4 Effect on receipt

The MIH user invokes an MIH_LL_Transfer.response primitive with the information obtained from this primitive.

ADD FOLLOWING DATA TYPE TO TABLE F.24

| Data | Derived from | Definition |
| --- | --- | --- |
| NAI | OCTET_STRING | Represents a Network Access Identifier as for IETF RFC 4282 |
| SFF_ID | CHOICE( IP_ADDR, MIHF_ID) | Represents the identifier of the target SFF |

8.6.1.18 MIH_LL_Transfer request

This message is used for an MIHF to carry link layer frames to conduct an authentication. The corresponding primitive is defined in Figure 7.4.29.1.

MIH Header Fields (SID=1, Opcode=1, AID=9)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

LinkIdentifier (Link Identifier TLV)

LLInformation (Link Layer Information TLV)

SFFIdentifier (SFF Identifier TLV)

8.6.1.19 MIH_LL_Transfer response

This message is used for an MIHF to carry link layer frames to conduct an authentication. The corresponding primitive is defined in Figure 7.4.29.3.

MIH Header Fields (SID=1, Opcode=2, AID=9)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

LinkIdentifier (Link Identifier TLV)

LLInformation (Link Layer Information TLV)

MNnetworkaccessid (optional) (Network Access Identifier TLV)

Status (Status TLV)

8.6.1.20 MIH_N2N_LL_Auth request

This message is used for an MIHF to carry link layer frames to conduct an authentication. The correspond ing primitive is defined in Figure (must be section) 7.4.30.1

MIH Header Fields (SID=1, Opcode=1, AID=9)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

LinkIdentifier (Link Identifier TLV)

LLInformation (Link Layer Information TLV)

MNmirk (Media Independent root key TLV)(optional)

8.6.1.21 MIH_N2N_LL_Auth response

This message is used for an MIHF to carry link layer frames to conduct an authentication. The correspond ing primitive is defined in Figure 7.4.30.3

MIH Header Fields (SID=1, Opcode=2, AID=9)

Source Identifier = sending MIHF ID (Source MIHF ID TLV)

Destination Identifier = receiving MIHF ID (Destination MIHF ID TLV)

LinkIdentifier (Link Identifier TLV)

LLInformation (Link Layer Information TLV)

MNnetworkaccessid (Network Access Identifier TLV)(optional)

Status (Status TLV)

Add to table L.2

| TLV type Name | TLV type value | Data type |
| --- | --- | --- |
| Media independent root key | 77 | KEY |
| Network Access Identifier | 76 | NAI |
| SFF Identifier TLV | 77 | SFF_ID |

Changes to section 9.2.2 of .21a (now .21b)

9.2.2 Key derivation and key hierarchy

Upon a successful MIH service access authentication, the authenticator, PoS obtains a master session key (MSK) or a re-authentication master session key (rMSK). Alternatively upon an optimized pull key distribution process a Target PoS can obtain a MIRK from a Serving PoS (see clause 9.2.2.2).

9.2.2.1 Derivation of media independent session keys (MISKs)

From the MSK, rMSK or MIRK defined in 9.2.2.2 based on the ciphersuite agreed upon between an MN and a PoS, the session keys used for MIH message protection can consist of an encryption key (MIEK) only, an integrity key (MIIK) only, or both an encryption key (MIEK) and an integrity key (MIIK). The length, L, of the derived keying material, called media independent session key (MISK), depends on the ciphersuite, which are specified in Clause 9.2.3.

For the key derivation, the following notations and parameters are used.

— K - key derivation key. It is truncated from a master session key (MSK), re-authentication MSK

(rMSK) or media independent root key (MIRK). The length of K is determined by the pseudorandom function (PRF) used for key derivation. If HMAC-SHA-1 or HMAC-SHA-256 is used as a PRF, then the full MSK, rMSK or MIRK is used as key derivation key, K. If CMAC-AES is used as a PRF, then the first 128 bits of MSK or rMSK are used as key derivation key, K.

….

9.2.2.2 Derivation of media independent root keys (MIRKs)

During an optimized pull key distribution process that involves the MN, the Serving PoS and a target PoS, the serving PoS derives a Media Independent Root Key (MIRK) for a specific target PoS. For the master media independent root key (MIRK) derivation, the following notations and parameters are used:

— K - key derivation key. It is truncated from a master session key (MSK) or re-authentication MSK

(rMSK). The length of K is determined by the pseudorandom function (PRF) used for key derivation.

If HMAC-SHA-1 or HMAC-SHA-256 is used as a PRF, then the full MSK or rMSK is used as key

derivation key, K. If CMAC-AES is used as a PRF, then the first 128 bits of MSK or rMSK are used

as key derivation key, K.

— L - The binary length of derived keying material MIRK. L=64 bytes for the MIRK

— h - The output binary length of PRF used in the key derivation. That is, h is the length of the block of

the keying material derived by one PRF execution. Specifically, for HMAC-SHA-1, h = 160 bits; for

HMAC-256, h =256 bits; for CMAC-AES, h = 128 bits.

— n - The number of iterations of PRF in order to generate L-bits keying material.

— Nonce-T and Nonce-N - The nonces exchanged during the execution of service access authentication.

— c - The ciphersuite code is a one octet string specified for each ciphersuite. The code is defined in

9.2.3.

— v - The length of the binary representation of the counter and the length of keying material L. The

default value for v is 32.

— [a]2 - Binary representation of integer a with a given length.

— MN_MIHF_ID – Mobile node’s MIHF identity.

—    PoS_MIHF_ID – Target PoS’s MIHF identity.

—  “MIRK” - 0x4D49524B, ASCII code in hex for string “MIRK”.

Fixed input values: h and v.

Input: K, Nonce-T, Nonce-N, L=64, and ciphersuite code.

Process:

a) n := ;

b) If n > 2v -1, then indicate an error and stop.

c) Result (0) := empty string.

d) For i = 1 to n, do

i) K(i) := PRF(K, “MIRK” || [i]2 || Nonce-T || Nonce-N || MN_MIHF_ID || POS_MIHF_ID || c || [L]2).

ii) Result(i) = Result (i-1) || K(i).

e) Return Result (n) and MISK is the leftmost L bits of Result (n).

Output: MIRK

It is important to note that the key distribution of a MIRK from the Serving PoS to the target PoS may produce a security weakness so-called “domino effect” [rfc4962]. This weakness implies that the compromise of the serving PoS will also compromise the target PoS, since an attacker can know and derive the MIRK that is delivered to the target PoS.  Reducing the latency of proactive authentication based on transferring a MIRK is at the cost of taking such a risk.

Replace Section 9.7.2 and 9.7.3 of .21c draft with the following text:

Protecting communications between serving PoS and target PoS

MIH_N2N_LL_Transfer messages exchanged between the serving PoS and the target PoS may require security protection. Also, the target PoS may only accept these messages from an authenticated and authorized serving PoS. To protect the link between the serving PoS and the target PoS several approaches are possible.

An MIH SA (Security Association) defined in IEEE 802.21a can be used for protecting the communications between the serving PoS and the target PoS. In this case, the serving PoS acts as the initiating end-point of an MIH SA and the target PoS as the other end-point of the MIH SA.. As specified in IEEE 802.21a, the MIH SA can be established using (D)TLS over MIH or EAP over MIH. 
Other mechanisms such as IPsec and TLS over TCP can also be used for protecting the communications between the serving PoS and the target PoS. Details on such mechanisms are outside the scope of this standard.

Add the following Annex

Annex P

(informative)

MN’s Network Access Identifier Format

An MNnetworkaccessid attribute (of type NAI), which is optionally contained in  MIH_LL_Transfer.response, MIH_LL_Transfer.confirm, MIH_N2N_LL_Transfer.response, and  MIH_N2N_LL_Transfer.confirm primitives, is assigned by the target PoS to the MN such that the MN can use the value of this attribute as the EAP peer identity for subsequent reactive pull key distribution or optimized pull key distribution from the target PoS.  The username part of the NAI carried in this attribute may contain the identifier of the MSRK used between the MN and the target PoS, and the realm part of the NAI may contain a Fully Qualified Domain Name of the target PoS.

Subclause 10.1

Change Figure 46 (IEEE 802.21a) as follows

ADD NEW FIGURE

ADD THE FOLLOWING EXPLANATION

In a media access proactive authentication, a PoS passes authentication messages between the mobile node and a media specific authenticator (MSA). The protocol stacks in each interface are illustrated in Figure 43. In scenarios where MSA/Target PoA is reachable via same media as MN and PoS, EAP messages received at PoS are directly forwarded to the target PoA. In an optimized pull key distribution, a PoS passes authentication messages between the mobile node, the target PoS and a media specific authenticator (MSA). The protocol stacks in each interface are illustrated in Figure 44.

Note to the editor, this footnote should not appear in the final version. MNmirk stands for MN´s media independent root key.

Note to the editor, this footnote should not appear in the final version. MNMSRK stands for MN´s media specific root key.

Note that LINK_TUPLE_ID includes the LINK_ID of both sides of the link, the MN and the PoA.

Note that LINK_TUPLE_ID includes the LINK_ID of both sides of the link, the MN and the PoA.

Note that LINK_TUPLE_ID includes the LINK_ID of both sides of the link, the MN and the PoA.

Note that LINK_TUPLE_ID includes the LINK_ID of both sides of the link, the MN and the PoA.

Note that LINK_TUPLE_ID includes the LINK_ID of both sides of the link, the MN and the PoA.

Note that LINK_TUPLE_ID includes the LINK_ID of both sides of the link, the MN and the PoA.

Note that LINK_TUPLE_ID includes the LINK_ID of both sides of the link, the MN and the PoA.

Note that LINK_TUPLE_ID includes the LINK_ID of both sides of the link, the MN and the PoA.