| Project | IEEE 802.21 MIHS <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | On notations with respect to GKB method |  |
| DCN | 21-13-0216-00-MuGM |  |
| Date Submitted | 13th Nov, 2013 |  |
| Source(s) | Yoshikazu Hanatani (Toshiba) |  |
| Re: | IEEE 802.21 Session #59 in Dallas |  |
| Abstract | This contribution provides alternatives of notations to describe GKB method. |  |
| Purpose | This contribution provides a remedy for cmt #19, #31 and #135 in LB7a. |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

Alternative of the data format of device key : Remedy for 135

| Data type name | Derived form | Definition |
| --- | --- | --- |
| NODE_DEPTH | UNSIGNED_INT(2) | This is called “NODE_BIT_LENGTH” in the current draft. |
| NODE_INDEX_VALUE | CHOICE( UNSIGNED_INT(1), UNSIGNED_INT(2), UNSIGNED_INT(3), UNSIGNED_INT(4) ) | This is called “NODE_INDEX” in the current draft. |
| NODE_INDEX | SEQUENCE( NODE_DEPTH, NODE_INDEX_VALUE ) | This is called “GKB_INDEX” in the current draft. |
| NODE_KEY | OCTET(16) | This is the base data type of a key assigned each node in a management tree. This stores a key of 16 octets AES key. |
| DEVICE_KEY | SEQUENCE( NODE_INDEX, NODE_KEY ) | This is the base data type of DEVICE_KEYS. |
| DEVICE_KEYS | LIST(DEVICE_KEY) | The data type for a list of the device key. |

Note: Sec. 9.4 in the current draft does not follow the data type of NODE_INDEX.

Definition of Leaf key: Remedy for cmt#19

Leaf number: A node index value which is assigned to a node of depth d in a key tree of depth d.

Leaf key: A node key which is assigned to a node of depth d in a key tree of depth d.

Note:

“Key tree” is a term used in the contribution DCN#199-00.

“Node index value” is called “node index” in the current draft.

An alternative of information base: Remedy for cmt#31

In this part, I summarize information that each entity holds. Please brush up.

MIH user of PoS (Command Center)

Group_Management_Information_Base

- LIST(DEVICE_KEY_MANAGEMENT__Info_Base)

-- MIHF_ID

-- DEVICE_KEYS

- LIST(Group_Info_Base)

-- MIHF Group ID

-- MGK

-- GroupMembers

--- LIST(MIHF_ID)

Command_Center_Base

- LIST(Group_Info_Base)

-ManagedNodes

-- LIST(MIHF_ID)

-CREDENTIAL

-Certificate

Note: Both Group_Management_Information_Base and Command_Center_Base have LIST(Group_Info_Base). LIST(Group_Info_Base) in Group_Management_Information_Base stores group information to issue group manipulation commands, i.e., all group information in a system is stored in it. LIST(Group_Info_Base) in Command_Center_Base stores group information to issue group addressed commands, i.e., a part of group information in a system may be stored in it. A command center who can generate GKB may omit LIST(Group_Info_Base) in Command_Center_Base.

MIHF of PoS

RECIPIENT_MIHF_Base,

-DEVICE_KEYS,

-Certificate,

-LIST(Group_Membership_Base)

--MIHF Group ID,

--MulticastAddress,

--MGK,

--SequenceNumber,

--SAID

SigningKey

MIHF of Recipient

RECIPIENT_MIHF_Base

| Name | Type | Descriotion | Note |
| --- | --- | --- | --- |
| ManagedNodes | LIST(MIHF_ID) | The IDs of PoS/MN which can be managed by groups. |  |
| GroupMembers | LIST(MIHF_ID) | The IDs of PoS/MN which a belonged a group. |  |
| SigningKey | Data type of ECDSA’s signing key. | The key to generate a digital signature it can be verified using a CREDENTIAL. |  |
| Group_Info_Base | SEQUENCE( MIHF Group ID, MGK, GroupMembers ) | The information base stores MIHF Group ID and its group members. | MGK:OCTET(16) is optional. Does it also need to store GROUP_STATUS? |
| DEVICE_KEY_MANAGEMENT_Info_Base | SEQUENCE( MIHF_ID, DEVICE_KEYS ) | The information base stores MIHF_ID and device keys assigned to the MIHF_ID to generate GKB. |  |
| Group_Membership_Base | SEQUENCE( MIHF Group ID, MulticastAddress, MGK, SequenceNumber, SAID ) | The information base stores group membership information to manage a belonged group. | MGK, SequenceNumber and SAIS are optional. |
| Group_Management_Information_Base | SEQUENCE( LIST(DEVICE_KEY_MANAGEMENT_Info_Base), LIST(Group_Info_Base) ) | The information base stores all ManagedNode_Info_Base and all Group_Info_Base to manage group membership using GKB. |  |
| Command_Center_Base | SEQUENCE( LIST(Group_Info_Base), ManagedNodes, CREDENTIAL, Certificate ) | The information base stores information to issue group manipulation commands and group addressed commands. | Does it need to store CERT_STATUS? CREDENTIAL and Certificate are needed to issue MIH Push_Certificate command. |
| RECIPIENT_MIHF_Base | SEQUENCE( DEVICE_KEYS, Certificate, LIST(Group_Membership_Base) ) | The information base stores an individual device keys to decrypt GKB, Certificate to verify digital signatures, and Group_Membership_Base to manage belonged groups. |  |