| Project | IEEE 802.21d <http://www.ieee802.org/21/> |  |
| --- | --- | --- |
| Title | Remedy for Cmt#84 in LB7b |  |
| DCN | 21-14-0055-00-MuGM |  |
| Date Submitted | Mar, 19, 2014 |  |
| Source(s) | Yoshikazu Hanatani (Toshiba) |  |
| Re: | IEEE 802.21 Session #61 in Beijing |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document has been prepared to assist the IEEE 802.21 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that IEEE 802.21 may make this contribution public. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as stated in Section 6 of the IEEE-SA Standards Board bylaws <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and in Understanding Patent Issues During IEEE Standards Development http://standards.ieee.org/board/pat/faq.pdf |  |

Remedy for Cmt#84 in LB7b.

Change the original text to following new text.

Add new text and remove original text.

And, we should revise sentences on MIGMEK.

- Rename MIGMEK to MIGEK which is appeared in sub-clause 8.4.2.3.2, sub-clause 9.5.6.2.

- Remove sentences on MIGMEK in clause 4, sub-clause 9.4.2, sub-clause 9.5.4.

And rename MIGEK to MIGSK since they are the same key.

- Remove MIGEK in clause 4.

New text:

9.5.1 Group session key derivation

MGK

K

MIGSEK

- Figure 42—Key derivation example

When an MN successfully recovers a GKB, it obtains a master group key (MGK). The following key is derived from MGK:

- Group session key (MIGSK) used to protect the MIH Group command.

For the key derivation, the following notations and parameters are used.

- K: key derivation key. It is truncated from a master group key (MGK). The length of K is determined by the pseudorandom function (PRF) used for key derivation. If HMAC-SHA-1 or HMAC-SHA-256 is used as a PRF, then the full MGK is used as key derivation key, K. If CMAC-AES is used as a PRF, then the first 128 bits of MGK are used as derivation key, K.

- L: The binary length of derived keying material MIGSK. L is determined by selected multicast ciphersuites described in Error: Reference source not found.

- h: The output binary length of PRF used in the key derivation. That is, h is the length of the block of the keying material derived by one PRF execution. Specifically, for HMAC-SHA-1, h = 160 bits; for HMAC-256, h = 256 bits; for CMAC-AES, h = 128 bits.

- n: The number of iterations of PRF in order to generate L-bits keying material.

- c: The multicast ciphersuite code is a one octet string specified for each ciphersuite. The code is defined in Error: Reference source not found.

- v: The length of the binary representation of the counter and the length of keying material L. The default value for v is 32.

- “MIGSK”: 0x4D4947534B, ASCII code in hex for string “MIGSK.”

- [a]2: Binary representation of integer a with a given length.

For given PRF, the key derivation for MIGSK can be described in the following procedures:

Fixed input values: h and v.

Input: K, L, and multicast ciphersuite code.

Process:

- If n > 2v-1, then indicate an error and stop.

- Result(0) := empty string.

- For i = 1 to n, do

  - K(i) := PRF(K, “MIGSK” || [i]2 || c || [L]2).

  - Result(i) = Result(i-1) || K(i).

- Return Result(n) and MIGSK is the leftmost L bits of Result(n).

- Output: MIGSK.

- With the above procedure, a key hierarchy is derived as shown in .

Original text:

9.5.1 Group session key derivation

- Figure 42—Key derivation example

When an MN successfully recovers a GKB, it obtains a master group key (MGK). The following two keys are derived from MGK:

- Group manipulation encryption key (MIGMEK) used to protect a group manipulation command;

- Group encryption key (MIGEK) used to protect the MIH Group command.

The deriving key is specified by the different multicast ciphersuites described in Error: Reference source not found. For the key derivation, the following notations and parameters are used.

- K: key derivation key. It is truncated from a master group key (MGK). The length of K is determined by the pseudorandom function (PRF) used for key derivation. If HMAC-SHA-1 or HMAC-SHA-256 is used as a PRF, then the full MGK is used as key derivation key, K. If CMAC-AES is used as a PRF, then the first 128 bits of MGK are used as derivation key, K.

- L: The binary length of derived keying material MIGSK. L is determined by selected multicast ciphersuites described in Error: Reference source not found.

- h: The output binary length of PRF used in the key derivation. That is, h is the length of the block of the keying material derived by one PRF execution. Specifically, for HMAC-SHA-1, h = 160 bits; for HMAC-256, h = 256 bits; for CMAC-AES, h = 128 bits.

- n: The number of iterations of PRF in order to generate L-bits keying material.

- c: The multicast ciphersuite code is a one octet string specified for each ciphersuite. The code is defined in Error: Reference source not found.

- v: The length of the binary representation of the counter and the length of keying material L. The default value for v is 32.

- “MIGSK”: 0x4D4947534B, ASCII code in hex for string “MIGSK.”

- [a]2: Binary representation of integer a with a given length.

For given PRF, the key derivation for MIGSK can be described in the following procedures:

Fixed input values: h and v.

Input: K, L, and multicast ciphersuite code.

Process:

- If n > 2v-1, then indicate an error and stop.

- Result(0) := empty string.

- For i = 1 to n, do

  - K(i) := PRF(K, “MIGSK” || [i]2 || c || [L]2).

  - Result(i) = Result(i-1) || K(i).

- Return Result(n) and MIGSK is the leftmost L bits of Result(n).

- Output: MIGSK.

- The MIGSK is parsed in such a way that

- MIGSK = MIGMEK || MIGEK.

With the above procedure, a key hierarchy is derived as shown in .