IEEE P802.22
Wireless RANs

| Functional Requirements for the 802.22 WRAN Standard |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-03-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Carl R. Stevenson | WK3C Wireless LLC | 4991 Shimerville Road, Emmaus, PA 18049-4955 USA | +1 610-965-8799 | wk3c@wk3c.com |
| Carlos Cordeiro | Philips | 345 Scarborough Rd Briarcliff Manor, NY 10510 USA | +1 914 945-6091 | carlos.cordeiro@philips.com |
| Eli Sofer | Runcom | Israel |  | elisofer@runcom.co.il |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | +1 613-998-2500 | gerald.chouinard@crc.ca |

Abstract

Introduction

This document represents a collaborative effort by the 802.22 Functional Requirements Ad Hoc Group, starting from discussions held at the January 2005 interim session and drawing and adapting the structure and content of document IEEE 802.16.3-00/02r4 as a framework and source of ideas for further work by the Ad Hoc Group.

Abstract

This document provides functional requirements that are guidelines for developing an interoperable 802.22 air interface. The 802.22 committee desired to reach an understanding and consensus for functional requirements before proceeding with developing standards for 802.22 MAC and PHY protocols and thus formed a Functional Requirements Ad Hoc Group to produce this document.

Purpose

The 802.22 WRAN WG group will use this document to help qualify MAC and PHY protocol proposals.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Functional Requirements
for the 802.22 WRAN Standard

Xxxx Yyyy (editor)

# 1	Introduction

This document provides functional requirements that are guidelines for developing an interoperable 802.22 air interface for use in spectrum allocated to TV Broadcast Service, enabling Point to Multipoint (P-MP) Wireless Regional Area Network (WRAN). The WRAN system provides packet-based transport capabilities that can support a wide range of services  (e.g., data, voice and video) to residential, Small and Medium Enterprises (SME) and Small Office/Home Office (SOHO) locations.

For convenience, requirements are itemized in Appendix A.

Throughout this document, the words that are used to define the significance of particular requirements are capitalized. These words are:

- “MUST” or “SHALL” These words or the adjective "REQUIRED" means that the item is an absolute requirement.

- “MUST NOT” This phrase means that the item is an absolute prohibition.

- “SHOULD” This word or the adjective “RECOMMENDED” means that there may exist valid reasons in particular circumstances to ignore this item, but the full implications should be understood and the case carefully weighed before choosing a different course.

- “SHOULD NOT” This phrase means that there may exist valid reasons in particular circumstances when the listed behavior is acceptable or even useful, but the full implications should be understood and the case carefully weighed before implementing any behavior described with this label.

- “MAY” This word or the adjective “OPTIONAL” means that this item is truly optional. One implementation may include the item because the target marketplace requires it or because it enhances the product, for example; another implementation may omit the same item.

## 1.1	Scope

For the purposes of this document, a “system” constitutes an 802.22 MAC and PHY implementation in which at least one subscriber station communicates with a base station via a point-to-multipoint (P-MP) radio air interface, the interfaces to external networks, and services supported by the MAC and PHY protocol layers. So, “functional requirements” describes the functions of typical systems in terms of how they affect requirements of interoperable 802.22 MAC and PHY protocols. The functional requirements describe 802.22 systems and requirements in broad terms: what the required functions are but not how these functions work.

The ‘how’ part is left to the forthcoming 802.22 interoperability standard [1] which will describe in detail the interfaces, functions and procedures of the MAC and PHY protocols. This document focuses on the service capabilities that an 802.22 system is required to support. These service capabilities have a direct impact on the requirements of the 802.22 MAC and PHY protocols. When the 802.22 working group produces an interoperable air interface standard that meets these functional requirements, resulting 802.22-based implementations will provide the services required to interface into many conceivable WRAN systems.

Other goals of this document are to formulate reference models and terminology for both network topology and protocol stacks that help the 802.22 working group to discuss and develop the MAC and PHY protocols. As far as possible, these SHOULD be common across 802.22 systems.

Remove 801.10, change 802.16.3 by 802.22

Figure 1-1: Relationship between 802.22 and other Protocol Standards
(the numbers in the figure refer to IEEE standard numbers)
(figure needs to be fixed …)

The 802.22 air interface interoperability standard SHALL be part of a family of standards for wireless regional area networks. The 802.22 protocols relate to other 802 standards and to the OSI model as shown in Figure 1-1.

This family of standards deals with the Physical and Data Link layers as defined by the International Organization for Standardization (ISO) Open Systems Interconnection Basic Reference Model (ISO 7498: 1984).

The standards that define the services noted in the above diagram are as follows:

- IEEE Std 802: Overview and Architecture. This standard provides an overview to the family of IEEE 802 Standards. This document forms part of the 802.1 scope of work.

- ANSI/IEEE Std 802.1B [ISO/IEC 15802-2]: LAN/MAN Management. Defines an Open Systems Interconnection (OSI) management-compatible architecture, environment for performing remote management.

- ANSI/IEEE Std 802.1D [ISO/IEC 10038]: MAC Bridging. Specifies an architecture and protocol for the interconnection of IEEE 802 LANs below the MAC service boundary.

- ANSI/IEEE Std 802.1E [ISO/IEC 15802-4]: System Load Protocol. Specifies a set of services and protocols for those aspects of management concerned with the loading of systems on IEEE 802 LANs.

- ANSI/IEEE Std 802.2 [ISO/IEC 8802-2]: Logical Link Control.

## 1.2	Target Markets

The target markets described in this section are not an exhaustive set, but serve as guidelines and examples that suffice for meeting the broad applicability goals set forth by the air interface “Five Criteria” as described in the IEEE 802.22 Project Authorization Request (PAR) and “Five Criteria” [1, 2].

A Wireless Regional Area Network (WRAN) system based on 802.22 protocols is intended to make use, on a non-interfering basis, of unused TV broadcast channels to address, as a primary objective, rural and remote areas and low population density underserved markets with performance levels similar to those of broadband access technologies such as digital subscriber line (xDSL) technologies and Digital Cable modem service.  A secondary objective is to have this system scale to serve denser population areas where spectrum is available.

- The WRAN system must be capable of supporting a mix of  data, voice and audio/video applications.

The critical parameters for serving these markets using wireless access technology is the combination of coverage / capacity factors that affects access cost per user, the deployability, maintainability and product costs associated with the customer premise installation, and the spectrum efficiency / reuse for economically serving the required number of customer locations with a minimum number of base station locations and backhaul routes.

The target markets to be addressed by the 802.22 protocols in WRAN networks are single family residential, multi-dwelling units, SOHO, small businesses, multi-tenant buildings and public and private campuses..

In accordance with ITU-R [3] definitions, Fixed Wireless Access (FWA) (and hence WRAN) provides access to one or more (public and private) core networks, rather than forming an end-to-end communication system. 802.22 systems serve fixed location customers, but who might be geographically fixed, re-locatable or nomadic.

# 2	WRAN System Model

This section presents a high level description of a system model to be used as a framework for developing 802.22 standards. The model identifies the main features of an 802.22 system, and the terminology to be used by the 802.22-working group in the creation of the standards.

The 802.22 wireless regional area network system is aimed at providing broadband access with capabilities similar to the ADSL and cable modem technologies over less populated rural areas.  The typical range of the system is [33] km (based on 4 Watt CPE EIRP and F(90, 99.9)) for a coverage of population density of about 1.25 person/km2 and above, and up to a maximum of 100 km when transmit power in not a constraint.  The system will need to operate over a set of typical channels models as defined in Appendix D.

As mentioned in section 1.1, an 802.22 “system” constitutes an 802.22 MAC and PHY implementation in which at least one subscriber station communicates with a base station via a point-to-multipoint (P-MP) radio air interface, and services supported by the MAC and PHY protocol layers. Specific applications of the 802.22 point-to-multipoint (P-MP) radios are applicable to the VHF/UHF TV broadcast frequency range. Radio communications in the above range may be possible in near- and non-line-of-sight situations between a base station and subscriber station. Operation may include partial and even complete blockage by foliage which contributes to signal attenuation and multipath effects. Figure 2-1 shows an example deployment configuration including the optional use of macro diversity (optimization of link and use of repeaters) (2nd class base station). 802.22 systems should be deployable in multiple-cell frequency reuse systems and single cell (super cell) frequency reuse systems. The range of 802.22 radios varies with EIRP, channel characteristics, availability requirement, local regulations and atmospheric conditions as well as bandwidth and transmitter/receiver performance.

Figure 2-1 Example Deployment Configuration
(figure should be fixed to be more appropriate to the WRAN environment, need to include houses with TV receivers, from contributors)

To facilitate description, an 802.22 system consists of one base station radio and one or more subscriber stations. Thus, an 802.22 system also defines 802.22 base station and one or more stationary subscriber station radios that communicate using the 802.22 MAC and PHY protocols. The base station radio SHALL be P-MP, radiating its downstream signal (forward) toward the CPE’s  with an omni-directional, a shaped sector or optionally an adaptive array (spatial reuse) antenna achieving broad azimuthal beam width to “cover” a number of prospective subscribers.

For the purpose of coexistence, the 802.22 standard shall include mechanisms allowing cooperation between base stations for better sharing of the spectrum.  The MAC and PHY protocols MUST provide means for base stations to resolve interference problems due to collocation or coverage area overlapping.

The frequency bands used by 802.22 systems vary among governed geographies (see section 8.1.1.1).  The frequency range identified by 802.18 in its comments to the FCC was from TV channel 2 to 51, the 802.22 PAR identifies 54 MHz to 863 MHz but the international range is from 41 MHz to 910 MHz.  Since the 802.22 system will have to operate without causing interference to incumbent services, sensing of channel occupancy shall be done according to the regulatory requirements of the local Administration.

## 2.1	Wireless Access Reference Model

Figure 2-2 shows the 802.22 wireless access reference model. The model depicts the relevant points between subscriber networks and “core” networks (the networks that MAY be accessed via 802.22 air interface). A greater system encompassing user terminals, base station interconnection networks, network management facilities, etc. may be envisaged but the 802.22 protocol focuses on the air interface shown in the model. The Core Network Interface (CNI) and the User Network Interface (UNI) are also shown.

A single Customer Premise Equipment (CPE) MAY support multiple customer premises networks that transport data, voice and video through one or more UNIs. Base stations MAY support multiple core networks through one or more CNIs. For the purposes of 802.22, the UNI and CNI are abstract concepts. The details of these interfaces are beyond the scope of this document. The standard SHALL specify MAC layer protocols and PHY transmission techniques suitable for providing access between one or more CPE and base stations to support UNI and CNI requirements.

Figure 2-2: Wireless Access Reference Model
(figure needs to be fixed, sensing interface …)

## 2.2	Spectral efficiency vs Distance

Since the 802.22 WRAN system is to operate primarily in rural areas, more spectrum is expected to be available.  Consistent with what is indicated above, the required minimum peak throughput rate at edge of coverage shall be 1.5 Mbit/s in the forward direction and 384 kbit/s in the reverse direction per subscriber.  For example, if adaptive modulation is used (for example 256-QAM and 64-QAM could be used to serve close-in subscriber stations, down to 16-QAM, QPSK and BPSK at the edge of coverage, see 5.2), the system will operate with a minimum spectrum efficiency of 0.5 bit/(sec*Hz) and a typical efficiency of 3 bit/(sec*Hz) including error coding, hence providing a typical capacity of 18 Mbit/s in 6 MHz TV channel, 21Mbit/s in 7 MHz channel and 24 Mbit/s in 8 MHz TV channels in the forward direction.

The modulation capacity will be adaptive based on C/N, BER, etc.  This will translate in a reduction of capacity as a function of the distance from the base station, i.e., lower transmission capacity toward the edge of the coverage area.  [Realize that the environment Burst noise immunity: depth of interleaving and impact onswitching between modulation level.]  [Service availability in presence on thermal noise, interference, channel linear (multipath static) and non-linear characteristics and impulse noise: need to relate it to % availability.  How to apportion availability ‘budget’.]

## 2.3	Service availability:

The system should be designed to provide a service availability of 90% locations and 99.9% time representing, F(90, 99.9).  Consumers need high data availability, especially when streaming services and VoIP are provided.

## 2.4	Antenna Characteristics  (Information section, assumptions we made)

### 2.4.1	Base

#### 2.4.1.1	Operational

The PHY layer shall allow use of omni as well as directional (sectorial) antennas that cover a specified segment of the entire frequency range.  The antenna gain should be flat across usable segment.  The transmit antenna should use horizontal polarization whereas the receive antenna should be vertically polarized (to allow maximum isolation of the transmit CPE antenna toward nearby outdoor TV receiving antennaes and maximum isolation between transmit and receive CPE antennas).  Both omni and directional antenna may be used.

Log-periodic: more element, more BW, gain: 7 dBi VHF, 10 dBi UHF, more gain: stacking in H or V.

Input from Radio-Shack, Winnegard, ChannelMaster.

CEA antenna group.

Close-in depolarization is possible with coupling with angled piece of metal.

[Max to make the two reports on de-polarization availble on the reflector.]

Omni

Transmit and receive antenna at the base station would normally be omni-directional in the horizontal plane to cover the area surrounding the base station

Directional

Directional antennas (log-periodic, corner reflector, etc.) would be used for sectorized operation from the same tower and should allow implementation of sidelobe rejection for frequency re-use.

#### 2.4.1.2	Sense (omni)

The sensing antenna should be flat across the usable frequency segment within x dB, with a minimum gain 0dBi in both horizontal and vertical polarizations, omni-directional  in the horizontal plane with a minimum gain variation.

### 2.4.2	CPE

#### 2.4.2.1	Operational

The transmit antenna should use vertical polarization [?] whereas the receive antenna should be horizontally polarized (to allow maximum isolation of the transmit CPE antenna toward nearby outdoor TV receiving antenna and maximum isolation between transmit and receive CPE antennas mounted on the same mast or even integrated into a common physical antenna.) If separate transmit and receive antennas are installed on the same mast, a minimum distance should be secured between the two antenna to minimize coupling.  The transmit antenna would normally be located above the receive antenna to minimize the actual power transmitted through TPC. [If half duplex operation is selected for WRAN, the forward signal transmitted from the base station could also be vertically polarized because isolation between the transmit and receive CPE antenna would not be needed.]

Omni

Although omni-directional antennas at the CPE would avoid the need of re-aiming the antenna when a new WISP operator is selected, directional antennas will need to be used to avoid interference coming in and going out of the CPE in all directions,

Directional

Antennas with minimal gain and backlobe rejection should be used outdoors.  [Vertically polarized transmit antenna at the CPE may have a coupling problem with the metallic mast for Yagi of log-periodic antennas.  This needs to be investigated.]  The gain of the antennas should be flat over the given frequency segment to allow for dynamic frequency change.

#### 2.4.2.2	Sense (omni)

The sensing antenna will need to be mounted outside with the operational antennas.  The sensing antenna should be flat across the usable frequency segment within x dB, with a minimum gain 0dBi in both horizontal and vertical polarizations, omni-directional  in the horizontal plane with a minimum gain variation.  The horizontal polarization will be used for sensing TV transmission whereas the vertical polarization will be used to sense nearby CPE’s, whereas the polarization from wireless microphones could be anything.

## 2.5	Installation Requirements

Base stations will be professionally installed.

CPE should be user installable (Plug and play) and the antennas should be installed outdoor at 10m above ground with proper azimuth and elevation alignment.  However, operators may chose to provide for professional installation.

Verification of the installation should be done either visually or remotely from the base station if the proper means exist (i.e., compass and GPS receiver)

## 2.6	Equipment integrity

The WRAN equipment should be tamper-proof to avoid any unauthorized modification to the hardware, firmware and/or functionalities, e.g., cognitive functionality, RF sensing, DFS, TPC, tuning).

## 2.7	Optional Repeater Function

The 802.22 systems may support the optional deployment of repeater functions. The proponent should document the impact of the use of repeaters on the functionality PHY and MAC .  The repeater function SHOULD NOT affect the end-to-end operation of 802.22 protocols between the base station BS and the CPE’s.

[Types of repeaters: RF repeaters on-channel, off-channel (RF translators), re-generating repeaters off-channel.]

Need a trade-off analysis on multiple low power base station versus higher power base station.  2 or 3 operating modes

## 2.8	Topology

Since all data traffic in a single cell of an 802.22 network MUST go through the base station, that station SHALL serve as a radio resource supervisor and controller.  The base station will control the RF characteristics of all CPE’s.

In the downstream direction, the multiplexed traffic is transmitted by the base station. In the upstream direction, 802.22 protocols MUST provide the means to multiplex traffic from multiple subscriber stations and allocate capacity.

# 2.9	Wireless Media Characteristics

### 2.9.1	Multiple access techniques

Need to support ….(Carl)

## 2.9.2	Duplex Modes

This standard SHALL support duplex modes of operation.

If full duplex FDD is proposed, the proposal must include band plan or propose a scheme to do it.

## 2.9.3	Flexible Asymmetry

Some applications utilize naturally asymmetrical capacity, such as for generic Internet access where most of the capacity is consumed in the downstream direction. Some applications  utilize asymmetrical capacity, using more in the upstream direction, such as surveillance video from a CPE.. Other applications require symmetrical capacity, such as telephony and video conferencing [15].  WRAN systems must support fFlexible asymmetry.  It should be recognized that CPE’s at the edge of coverage may not be capable of achieving the same capacity in the upstream direction as the base station can provide to them in the downstream direction.

# 3	Supported Service Capabilities

This section describes typical service capabilities supported by an 802.22 air interface. The MAC and PHY protocols will not have explicit support for each and every service, due to the fact that generic data streams SHALL be used for transport. The MAC and PHY protocols SHALL provide for admission control, QoS service specific support, appropriate PER for data services, minimum latency and jitter for real time services, etc.

3.1	Data Transport Service Capabilities – Internet and VoIP

The 802.22 system should directly transport variable-length IP datagrams efficiently. Both IP versions 4 and 6 MUST be supported by packet prioritization, admission control and scheduling.

The 802.22 systems should provide support for real-time and non-real-time service capabilities. The 802.22 protocols should provide for VoIP services.  It SHOULD be possible to support IP Quality of Service (QoS) efforts: Differentiated Services [RFC 2475- Dec 1998, RFC2474, latest reference?].

The following types of services shall be supported: (see 802.1 and 802.3 for priority and access control)

- Constant bit rate service

- Real-time variable bit rate service

- Non-real-time variable bit rate service

- Best-effort service [802.1d]

## 3.2	Bridged LAN Service Capabilities

The 802.22 protocols SHALL support 802.1 bridged LAN service capabilities.

3.3	Other Services

Other services that for instance require QoS-based delivery of the MAC services may be added. These services SHALL NOT place any additional requirements on 802.22 systems (MAC and PHY protocols) not already covered in the above sections.

# 4	802.22 Protocols

The 802.22 protocols determine the interoperability of multiple vendors' equipment. Protocol interoperability occurs at each level in the protocol “stack” [9]. In this section, services refer to the services provided by the protocols that can appear in the layer sitting directly over the MAC layer. IEEE 802 protocols reside at layers 1 and 2 and consist primarily of Logical Link Control (802.2) [10] and the various MAC and PHY layers for each LAN or MAN standard. The IEEE Std 802-1990 Overview and Architecture [11] describes these layers.

The 802.22 protocol stack reference diagram is shown in Figure 4-1. In addition to the LLC, MAC and PHY layers suggested by the generic 802 architectures [11] [12] [13], 802.22 protocols support other categories of “upper protocols” that correspond to the requirements of the service capabilities described in section 3.

Figure 4-1: Protocol Stack Reference Model
(figure needs to be fixed, change 802.16.3 for 802.22 …)

This protocol stack reference model is intended to help develop terminology, and possibly protocol architecture. Each of the protocols above the PHY and MAC support “convergence  sub-layers.” The convergence sub-layers [14] [15] may be necessary to:

- Encapsulate PDU framing of upper layers into the native 802.22 MAC/PHY PDUs [15]

- Map an upper layer’s addresses into 802.22 addresses

- Translate upper layer CoS/QoS parameters into native 802.22 MAC constructs

- Adapt the asynchronous, synchronous or isochronous data pattern of the upper layer into the equivalent MAC service

- Reduce the need for complex inter-working functions (IWFs) [15]

Other convergence sub-layers may be required by additional higher layer service requirements.

The IEEE 802.22 MAC and PHY protocol stacks SHALL be the same for all the supported services. The central purpose of the MAC protocol layer in 802.22 is sharing of radio channel resources. The MAC protocol defines how and when a base station or subscriber station may initiate transmission on the channel. Since customer units will contend for capacity to/from one or more base stations, the MAC protocol MUST efficiently manage contention and resource allocation. The PHY layer may be subdivided between a convergence layer and a physical medium dependent (PMD) layer. The PMD is the “main” part of the PHY. Like the MAC convergence layers, the PHY convergence layers adapt/map the “special” needs of the MAC services to generic PMD services.

5	Performance and Capacity

This section addresses some issues regarding 802.22 system performance and capacity. Specifying protocols that can maintain specified/mandatory performance levels in the face of fluctuating and diverse channel characteristics (e.g., due to multipath and atmospheric conditions) is a problem that the 802.22 work group has to consider. This section specifies the target performance levels. This section also outlines some of the issues for 802.22 capacity planning.

Note that ITU-R has presented several questions regarding the need for performance objectives  for fixed wireless access radio systems, in particular, the activities being carried out within the  Joint Rapporteur Group (JRG) 8A/9B (Questions ITU-R 140/9, ITU-R 215/8, ITU-R 220/9).

## 5.1	Peak Data Rate

The minimum peak rate at the edge of service is 1.5 Mbit/s downstream and 384 kbit/s upstream.

802.22 protocols SHALL be optimized to support the minimum peak data rate in both directions to a CPE station within the specified distance from the base station. Optional modes that can provide different rates may be included.  The 802.22 MAC protocol SHOULD allow the peak data rate to scale up to the total practical channel capacity (see section xxx)..  We can assume e.g. 3 bps/Hz, but what fraction of the 6/7/8 MHz bandwidth should be assumed, given the spectral mask constraints are still TBD?  Need to document in an appendix, and just put conclusion in this section.  Need to decide on max modulation rate, BW of channel, FEC and other overhead.]

## 5.2	Propagation Delay

The large cell radius, up to 33 –100 km, will produce a large propagation delay with round trip delaysas large as 186us. The PHY and MAC protocol SHOULD provide for far CPEs’ propagation delay compensation.

## 5.3	Spectral Efficiency

The spectral efficiency is an important performance parameter of a wireless access system. This spectral efficiency will vary from location to location within the coverage area of the base station, depending on distance, propagation, channel impairment and interference.

A minimum spectral efficiency of 0.5 bps/Hz  and a typical efficiency of 3 bit/(s*Hz) shall be supported..

## 5.4	Radio Link Availability

An 802.22 system SHOULD be available to transport all services at better than their required maximum error rates (see section 5.7) from about 99.9% of the time [14, 16] and 90% of locations within the coverage area, not counting electrical power and equipment availability. The 802.22 specifications SHALL NOT preclude the ability of the radio link to be engineered for different link availabilities, based on the preference of the system operator.  [99.9% time availability means to be equated to a required fade margin and TPC set point.

[The coverage area is defined as reaching 90% of the population in an area.]

Service availability encompasses radio link (fade) availability, down time due to coexistence, down time due to network congestion, electrical power and equipment failure.  That is why current broadband access systems don’t promise service availability.

For a data network, a period of unavailable time begins at the onset of ten consecutive SES events based on the following definitions [17].

- Severely Errored Second (SES) is defined as a one-second period which contains 30% errored blocks.

- Errored Block (EB): A block is defined as a set of consecutive bits associated with the path. Consecutive bits may not be contiguous in time. A block is typified as data block containing an error detection code for service performance monitoring. An errored block is a block in which one or more bits are in error.

It is expected that the highest contributor to 802.22 system outage will be excessive attenuation due to blockage and multipath due to varying path impediments such as reflections. In a multicell environment, intercell interference can not be neglected as an outage increasing factor.  802.22 MAC and PHY protocols MUST accommodate these conditions, perhaps consuming more radio bandwidth and/or requiring smaller radio propagation distance (radius) to meet the availability requirements. Since statistical atmospheric and path conditions vary widely in geography, the 802.22 protocols MUST be flexible in consumed radio bandwidth (spectral efficiency), cell radius, and transmit power. Bandwidth and cell radius are critical components of system/cell capacity planning (also see section 5.9). [Bring in ‘adaptive’section? Or is it informative.]

5.6	Radio Link Error Performance

The error rate, after application of the appropriate error correction mechanism (e.g., FEC), delivered by the PHY layer to the MAC layer SHALL meet IEEE 802 functional requirements (10-8 undetected corrupted packet rate) and the radio link Packet Error Ratio (PER) SHALL be 10% or better for a packet length of 1000 octets at the minimum specified receiver sensitivity.

## 5.7	Delay

Delay and variation of delay, or jitter, are important factors to consider. For example, a high variation of delay can severely impact interactive services. However, generic Internet access can tolerate a high degree of delay variation.  Jitter for the services considered should be maintained to less that xx msec.  [Eli to provide table]

The end-to-end delay depends on an entire application-specific network encompassing all 7  layers of the OSI model.

Maximum latency caused by PHY and MAC should be xx msec.

## 5.8	Capacity

802.22 base station capacity requirement is defined as the product of the number of subscribers, their peak bandwidth requirements and load factor based on quality of service  guarantees. The standard SHALL support careful planning to ensure that subscribers’ quality of service guarantees and minimum error rates are met.

The delivered base station capacity can vary depending on attenuation due to atmospheric conditions, LOS blockage, transmit power, etc., and should be calculated as the aggregate capacity of all sectors supported by a base station. Sector capacity is defined as the product of two factors: the “modulation factor” and the “sector-bandwidth factor” based on quality of service guarantees. The “modulation factor” is defined as the sector’s aggregate bit rate divided by the bandwidth. The “sector-bandwidth factor” is defined as the total frequency band available for the WRAN service, adjusted by the appropriate frequency re-use factors. This reflects mainly the factor of frequency reallocation and the ability to optimize frequency usage.  WRAN should utilize multiple access technologies which offer excellent NLOS operation in multipath environment

Given the propagation characteristics in the target frequency bands and variability in differing geographic areafor the development of a link budgets [16], the following parameters of an 802.22 system SHOULD be addressed by the MAC and PHY protocols:

- Radio range (33 to 100 km)

- Upstream/downstream channels’ data rates

- Allocation of prospective subscriber data rate to channels. Note: the MAC and PHY standards MAY allow subscribers to hop between channels

- Types of modulation

The MAC and PHY protocols should accommodate channel capacity issues and changes in channel capacity. For example, flexible modulation types, power level adjustment, and bandwidth reservation schemes MAY be employed (see section 6) . Also, as subscribers are added to 802.22 systems, the protocols MUST accommodate them in an automated fashion.

The time-variant impairments (e.g., multipath interference) is expected to be the most significant contributor to channel impairments and complexity in cell capacity planning [16, 17, 19-26].

The document [which one?] has a good elaboration on capacity, reference range, upstream/downstream data rate, accept as is?

5.9	Delivered Bandwidth

The minimum delivered peak data rate per subscriber should be 1.5 Mbit/s forward and 384 kbit/s return).  The WRAN system should be able to provide to CPE’s, higher capacity than the previously stated minimum peak data rate if the bandwidth is available, up to the full forward capacity to one subscriber, at times of low system loading), and if the power is available on the return link, i.e.,a CPE relatively close to the base station may, as directed by the base station, use the power margin of its TPC to achieve higher return throughput.

5.10	Flexible data rate and resource allocation – Downstream & Upstream

System capacity allocation should be able to vary dynamically.

The system should provide its operator to flexibly allocate its system resources to any CPE for the purpose of limiting or enhancing its throughput,

In the case of a re-generating repeater, the system should be capable of providing for  more bandwidth to the ‘CPE’ that would act as the relay for this ‘2nd classbase station to extend the coverage.

5.10.1	Peak Data Rates

CPE’s should be able to receive the full capacity from the base station in the forward direction and utilize the full capacity of the return channel as long as the transmit power limit is not exceeded.  The base station should be capable of allocating full capacity to a user on forward and return directions.  On the return channel, the capacity will be RF power limited.  Maximum peak capacity will reduce toward fringe because of need for more robust and less efficient modulation.

5.10.2	Number of User Terminals serviced by base station

Typically, there will be more that one user terminal connected to the household and SOHO CPE.  In order to assess the number of user terminals that a base station can support, an  over-subscription ratio of 50 has been assumed.

Assuming an average 3 bit/(s*Hz) spectrum efficiency, a 6 MHz channel, and the required minimum peak data rate of 1.5 Mb/sec per user terminal, this results in a total of 600 user terminals potentially being served by a single base station. Assuming an average of 3 user terminals per CPE, the numer of CPEs addressed by a base station will be typically 200.

6	Flexible Adaptive Performance

## 6.1	Scalability

The 802.22 protocols SHOULD allow for different “scales” of capacity and performance for 802.22 system instances.  [Everything here applies except for SDO name.  Different scales for capacity and performance.]

Scalability in bit rate, in terms of deployment, in channel bandwidth (varying channel bandwidth or using a variable number of channels).

Other dimensions?

### 6.1.1	Per-Subscriber Rate Adaptation  (????)

802.22 systems can use different modulation and/or coding options for far and near subscriber stations. In this way the data rate to/from relatively near subscribers can be higher, increasing the overall system capacity. Additionally, far subscribers can experience different interference profiles and so would benefit from rate adaptation. The PHY and MAC protocols SHALL provide for multirate support.

Scalability/ adaptability/ reconfigurability chapter:

Minimum spectrum efficiency: 0.5 bit/(s*Hz) with a goal of at least of 5 bit/(s*Hz) where propagation permits.

TPC SHALL be included.  TPC should have the following characteristics:  TBD

Forward could be continuous TX power but there is a need for quiet period for sensing, thus burst modem may be preferable at the base station.
The base station needs to be heard by all terminals when it sends specific control frames such as for association.  These would need to be transmitted at power so all terminals can hear.  However the base station doesn't have to be hearable by everybody all the time.  The base station will transmit on a link-by-link basis. TPC characteristics will be linked to the scalability in modulation, the access mode whether it is TDD or FDD.  Close and open loop power control schemes will need to be considered.

Bandwidth scalability: depending on the findings on the LE RF mask and the possible adjacent channel interference situation with TV receivers, wireless microphones and other LE systems, the 802.22 standard may need to include scalability with respect to occupied bandwidth.  This would be related to the adjacent channel occupancy by these other systems.  This needs more work.

Channel bonding: where possible, more than one 6 MHz channel (contiguous or not) could be used to increase the capacity of the transmission link as long as each 6 MHz channel can become independent from the modulation point of view to be able to free any of these bonded channels in case of interference.  (802.18 commented to the FCC that such boning should not be persistent.)

Flexibility in system transmission rate asymmetry: should be discussed under TDD/FDD section in section 5.

Flexibility in Tuning: VHF, band 4, 5 UHF, front end flexibility, 802.18 range was from ch5 to ch51 but the international range is much wider, from 66 MHz to about 900 MHz, but what is physically practical.  It is likely that, for feasibility of the RF section of the base stations and subscriber terminals, this range will be split in more reasonable segments such as the VHF range, band 4 and 5, etc.

Flexibility in maximum range: see discussion related to MAC characteristics in section 7 ( limitation due to the contention mechanism, not for polling) to maximum transmit power in section 2.

Channel model and propagation:

Document to form the basis of the Appendix: Propagation, impulse response, industrial noise. (Model of 802.16: 500 MHz and up)

Cost 207, broadcast requirements

ATSC-A74 model, Brazilian model,

## 6.2	Channelization

The MAC and PHY protocols MUST permit the operation with channel spacing of integer multiples of 6, 7 and 8 MHz.  Subdivision of these channels may be used as long as the regulatory maximum power per device in a 6, 7 or 8 MHz is not exceeded.  The effective channel bandwidth will be dependent on the requirement for out-of-band emission in the adjacent channels due to filtering requirement.

Granularity (channelization)-Capability of WRAN to configure bandwidth resources of the user to a desirable share of the allocated BW meeting application requirement thereby enhancing system gain.  Within the channel itself.

7	Medium Access Control (MAC)

## 7.1 Network Entry

When CPE boots up. What are the steps the user has to go through?  What if many CPE’s try to enter a the same time?

The proposals should detail the mechanisms for network entry and proper authorization,

## 7.2	Support for Interference Mitigation/Coexistence

### 7.2.1	Sensing Measurements

802.22 Base Station SHALL be able to control CPEs to conduct measurement activities and obtain measurement results.  There may be a need for scheduled quiet periods for sensing to take place .  Repetition rate and integration time will need to be identified in order to meet the sensitivity down to the specified threshold in a timely manner.  Sensing should include capture of signal signature to identify the type of incumbent and other LE signals present.

#### 7.2.1.1 Sensing Control

802.22 protocols MUST provide mechanisms for sensing control in the CPEs. The Base Station SHALL be able to instruct the CPEs when to sense, which channels to sense and the duration of the sensing activity.

The sensing mechanism MAY be different in FDD and TDD modes.

7.2.1.2 Sensing Measurement Report

CPEs SHALL send to the Base Station sensing measurement reports in a form understandable by the Base Station.  In addition, CPEs SHALL be able to acquire and indicate to the Base Station the incumbent or other LE service profile, and SHALL also be able to accommodate service profiles according to different regulatory domains.

The IEEE 802.22 air interface SHALL provide for the transmission of the identification of CPEs and Base stations at the packet level, which are used for resolving interference problems amongst Base Stations.

[ [System section: Need to indicate that RF levelsreported to the base station are in absolute field strength levels.  The frequency range for CPE should be the same as the range over which the CPE can operate .]

### 7.2.2	PHY Operational Control

#### 7.2.2.1	Channel Management

#### 7.2.2.1.1	Channel Change

Once the CPE has received the request from the base station, the 802.22 MAC protocol SHALL be able to order the 802.22 PHY to change the channel of operation.

#### 7.2.2.1.2	Suspend/Resume Channel Operation

The 802.22 MAC protocol SHALL be able to suspend the operation in a specified channel. Operation in this channel MAY be resumed upon indication from the Base Station.

#### 7.2.2.1.3	Termination and Reestablishment of Channel Operation

The 802.22 MAC protocol SHALL be able to terminate the operation in a channel for an unspecified amount of time. Operation MAY be reestablished in another channel.

#### 7.2.2.2 Control of PHY Adaptability

The 802.22 MAC protocol SHALL be able to control the PHY adaptability features such as TPC, modulation/FEC, and channel bandwidth, timing for ranging.

#### 7.2.3	Base station management to CPE

#### 7.2.3.1 Base is master

Base station can order to change modulation, coding, encryption, bandwidth resource allocation, power level, channel, QoS (packet prioritization, request from the CPE and bases station), etc.

#### 7.2.3.1.2 Order CPEs to scan band and report back

#### 7.2.3.1.3 Analyze (ongoing) results, consider database, and implement correct behavior to avoid interference

CPE can contain simple tables to store sensing information to minimize sensing time (only sensing channels that can be available).

Data fusion from sufficiently large number of CPE’s to get a reliable spectrum occupancy figure.

### 7.2.4	Inter-Base Station Coordination

TBD: The IEEE 802.22 air interface protocols SHALL provide mechanisms to allow for coexistence, interference resistence, and sharing of radio resources amongst neighboring 802.22 Base Stations. CPEs SHALL be able to report interference received from another 802.22 base station, and base stations SHALL be able to coordinate in order to resolve issue.

In case of interference: CPE reporting interference from another WRAN, both base stations exchange info to resolve issue.  Exchange of operational information such as channel maps, extension of coverage, nested cells, different classes of base stations under the same operator.  Point-to-multipoint link between base stations or use of normal backhaul structure to exchange control packets.

CPE being bridged to a base station: combination on different classes stations (ex: CPE connected to a Wi-Fi AP)

Base stations from same operator with overlapping coverage could coordinate for load balancing.  Base stations from different operators could cooperate on use of same channels and for load balancing.  Synchronization between the base stations might be needed to avoid down time when CPE re-associate.  (Antenna may have to be re-aligned to re-associate.)

### 7.3	Class of Service and Quality of Service

This section describes the classes of service and quality of service for 802.22 systems. Terminology is borrowed from the Internet Engineering Task Force (IETF).

802.22 protocols MUST support classes of service (CoS) with various quality of service (QoS) guarantees to support the services that an 802.22 system MUST transport. Thus, 802.22 protocol standard MUST define interfaces and procedures that accommodate the needs of the services with respect to allocation and prioritization of resources. Additionally, 802.22 protocols MUST provide the means to enforce QoS contracts and Service Level Agreements. Table 1 provides a summary of the QoS requirements that the PHY and MAC SHALL provide. Note that parameters in the table are measured between the MAC input at the upper layer at the transmit station and the MAC output at the upper layer of the receiving station for information transmission. For example, delay does not include setup time, link acquisition, voice codecs, etc.

For QoS-based, connectionless services, the 802.22 protocols MUST support resource negotiation “on-demand”. For instance, the MAC protocol MAY allocate bursts of PDUs to services that require changes in resource allocation. Such allocation, for connectionless services, is thus performed in a semi-stateless manner.

A connection-oriented service may require “state” information to be maintained for the life of a connection. However, the 802.22 MAC layer interface MAY provide a connection-less service interface that requires a higher-layer “adaptation” to maintain the “state” of a connection and periodically allocate resources. For instance, the MAC may need to maintain “state” information about a QoS data flow only for the duration of an allocation.

Table 1: Services and QoS Requirements

| Service | Maximum Ratio | PER (packet length= ) | Maximum Access Delay (One way) |
| --- | --- | --- | --- |
| Full Quality Telephony (Vocoder MOS) 4.0) | BER10-6 |  | 20 ms |
| Standard Quality Telephony (Vocoder MOS) < 4.0) | BER10-4 |  | 40 ms |
| Time Critical Packet Services | BER 10-6 |  | 20 ms |
| Non- Time Critical Services: | BER 10-9 |  | not applicable |

QoS

See 802.1p, 802.15.3, 802.16.

Good but has a lot of implication on the complexity of the MAC, frequency switching, etc.

Services: UDP streaming, video conferencing, VoIP.

Relative QoS among services.

Toll quality QoS would probably not be possible because of RF sensing and frequency agility required by the interference environment (down time during on-channel RF sensing).

Latency and Packet Error Rate

Error Performance

802.16: BER at input of MAC: 10-6, latency: 20 ms

### 7.3.1	Types and Classes of Service (to be moved to an appendix)

The fundamental direction for the QoS model that will be exported to the WRAN endpoints will be IP based and conform to IETF DiffServ QoS model in conjunction with other IP based  protocols. The DiffServ QoS model defines traffic for all services as follows:

- Expedited forwarding (EF) EF requires a constant, periodic access to “bandwidth”. The “bandwidth” requirements vary over time, within a specified range, but delay and delay variance limits are specified. Examples that fall into this category are voice-over-IP (VoIP), videoconferencing, video on demand (VoD), and other “multimedia” applications.

- Assured Forwarding (AF): In AF the “bandwidth” varies within a specified range, but has loose delay and delay variance requirements. Applications, which are limited in their “bandwidth” usage, may fall into this category. In one example, corporate database transactions could be relegated to this category. 'Assured Forwarding' service allows the traffic to be divided into different classes. Using this service, an ISP can offer an “Olympic” service model, which provides three tiers of services: gold, silver and bronze with decreasing quality (i.e., the gold level of service receives a higher share of resources than silver during times of congestion). This service model would support, for example, the ability to provide preferential treatment to subscribers willing to pay a “premium” price for better service. Or it would support more granular priorities such as giving preference to VoIP traffic over other traffic e.g., HTTP).

- Best Effort Service (BES). The “bandwidth” varies within a wide range, and is allowed to burst up to the maximum link “bandwidth” when EF and AF traffic are not using “bandwidth.” The “bandwidth” and delay requirements may or may not be specified.  Higher variations of delay may be tolerable since applications that fall into this category allow for priority traffic to preempt their “bandwidth” consumption. “Bandwidth” is delivered on a “best effort” basis. Current Internet service is an example of this type of operation.

### 7.3.2	Parameters

802.22 protocols SHALL define a set of parameters that preserve the intent of QoS parameters or IP-based services.

### 7.3.3	Service QoS Mappings

The classes of service and QoS parameters of services SHALL be translated into a common set of parameters defined by 802.22. A QoS-based IP network may employ the Resource Reservation Protocol (RSVP) [27] to “signal” the allocation of resources along a routed IP path.  If 802.22 is to be a “link” in the IP network, an IWF (? ) MUST interface with 802.22 to negotiate resource allocation.

The basic mechanism available within 802.22 systems for supporting QoS requirements is to allocate bandwidth to various services. 802.22 protocols SHOULD include a mechanism that can support dynamically-variable-bandwidth channels and paths (such as those defined for IP environments).

## 7.4	Optional MAC Bridge to other networks

(e.g. 802.11x, others?) Requirement?, Option?

## 7.5	Single channel versus Multichannel Operation

The simultaneous use of more than one channel MAY be possible as long as persistent channel bonding is not employed. In other words, the 802.22 MAC protocols SHALL be able to terminate operation in a subset of channels belonging to a larger channel bonding if the need arises.

7.6	OA&M Support

The air interface SHALL provide a mechanism to enable the provisioning and collection of metrics, so that the network operator can effectively control, monitor, and tune the performance of the 802.22 air-interface. Provisional parameters, performance metrics and other OA&M values shall be made available through a standards compliant MIB.

7.7	Base station and CPE’s address space

What kind of addresses that specify the CPE and Base station IEEE addresses? Where in the header?

- Should we use 48-bit standard IEEE addresses? 
- Addresses could be either fixed or dynamic

# 8	Co-existence and interference mitigation

### 8.1	Licensed Incumbent Sensing and Avoidance

#### 8.1.1	Overview

[Database needs to be updated within ?1 hour? for the base stations to have timely access latest info.  Ability to enter disallowed channels at the base station on a regular basis shall be a requirement.]

8.1.1.1	TV broadcast: description and technical characteristics

Channel rasters of 6, 7, 8 MHz. In North America, 6 MHz and high transmit power up to MW.  Typical RF characteristics of receivers is documented in ATSC A-74. , To create international standard, the characteristics, channel raster (6, 7, 8 MHz) and extend of frequency bands tend to change with international regions.

References: ITU-R BT.470-6, ITU-R BT.655-6, ITU-R BT.417-4

Analogue (AM Video, FM Audio)

Vestigial Side Band NTSC M: US, Canada, Chile, Costa Rica, Cuba, El Salvador, Guatemala, Honduras, Japan, Mexico, Panama, Philippines, Puerto Rico, South Korea, Taiwan

Operating frequencies: 54 – 746 MHz (Japan 90 – 770 MHz)

Bandwidth: 6 MHz channel: 4.2 MHz video, 250kHz audio

Carriers: Colour 3.580 MHz above video carrier, Audio 4.5 MHz above video carrier

ERP: 100 W – 5 MW

Phase Alternating Line (PAL): Argentina, Australia, Belgium, Brazil, China, Denmark, Finland, Great Britain, India, Indonesia, Ireland, Italy, Norway, New Zealand, the Netherlands, Portugal, Spain, Sweden, Switzerland, Turkey.

PAL B

Operating frequencies: 48 – 798 MHz (New Zealand 44 – 798 MHz, Australia 45 – 806 MHz)

Bandwidth: 7 MHz channel: 5.0 MHz video, 250 kHz audio

Carriers: Colour 4.434 MHz above video carrier, Audio 5.5 MHz above video carrier

PAL G

Operating frequencies: 48 – 798 MHz (New Zealand 44 – 798 MHz)

Bandwidth: 8 MHz channel: 5.0 MHz video, 250 kHz audio

Carriers: Colour 4.434 MHz above video carrier, Audio 5.5 MHz above video carrier

PAL H

Operating frequencies: 47 – 798 MHz

Bandwidth: 8 MHz channel: 5.0 MHz video, 250 kHz audio

Carriers: Colour 4.434 MHz above video carrier, Audio 5.5 MHz above video carrier

PAL I

Operating frequencies: 174 – 798 MHz

Bandwidth: 8 MHz channel: 5.5 MHz video, 250 kHz audio

Carriers: Colour 4.434 MHz above video carrier, Audio 6.0 MHz above video carrier

PAL N

Operating frequencies: 54 – 806 MHz

Bandwidth: 6 MHz channel: 4.2 MHz video, 250 kHz audio

Carriers: Colour 3.582 MHz above video carrier, Audio 4.5 MHz above video carrier

PAL M

Operating frequencies: 54 – 806 MHz

Bandwidth: 6 MHz channel: 4.2 MHz video, 250 kHz audio

Carriers: Colour 3.576 MHz above video carrier, Audio 4.5 MHz above video carrier

Sequential Colour with Memory (SECAM): France, Egypt, Germany, Greece, Haiti, Iran, Iraq, North Korea, Poland, the former Soviet Union.

SECAM B

Operating frequencies: 47 – 798 MHz

Bandwidth: 7 MHz channel: 5.0 MHz video, 250 kHz audio

Carriers: Audio 5.5 MHz above video carrier

SECAM G, H

Operating frequencies: 47 – 798 MHz

Bandwidth: 8 MHz channel: 5.0 MHz video, 250 kHz audio

Carriers: Audio 5.5 MHz above video carrier

SECAM D, K, K1, L

Operating frequencies: 470 – 790 MHz

Bandwidth: 8 MHz channel: 6.0 MHz video, 250 kHz audio

Carriers: Audio 6.5 MHz above video carrier

Digital

Vestigial Side Band ATSC (North America)

Operating frequencies: 54 – 746 MHz

Bandwidth: 6 MHz channel

Pilot: 11.3 dB below the average data signal power

ERP: 100 W – 1 MW

Orthogonal Frequency Division Multiplexing DVB-T (Europe)

Bandwidth: 7 or 8 MHz channel

Operating frequencies: 48 – 798 MHz

TDS-OFDM (China)

Bandwidth: 7 or 8 MHz channel

Operating frequencies: 48 – 798 MHz

Orthogonal Frequency Division Multiplexing ISDB-T (Japan)

Bandwidth: 6 MHz channel

Operating frequencies: 54 – 746 MHz

Above information to be put in a table. (Winston)

Add discussion on service contours (Grade B), field strength, etc.

Distance between transmitters at planning stage

Receiver characteristics and typical installation (antenna gain, directivity, height)

Summary of ATSC A-74                       . (Winston)

8.1.1.2	Wireless Microphone description and technical characteristics

Operated by broadcaster, new crews, in-studio and remote productions.  Typical power: 10 mW, typical range: 300 m.  There is also a large number of unlicensed microphone operations.  Wireless microphone operators will need a beacon for reliable operation (e.g., simple narrowband carrier-like).  There is however possibility for false beacon operation: CW would easily disallow WRAN operation in the nearby area. There is still a possibility of having channels set aside for microphones by the FCC.

[Shure to provide more information: email sent to Shure.]

8.1.1.3	Public Safety description

This service is geographically based: base station would know whether to use the channel or not (channels 14-20 are off the table in the 14 main market, plus some more licenses outside of these market).  There is no need to sense by CPE’s, hence speeding up the process.  [Steve Kuffner to gather text on Part 90, succinct table of technical characteristics, ch 14-20, sensing may be quite difficult, therefore geographic use should be the way to share, developed at Motorola.]

#### 8.1.2	D/U Ratios

Depends on the modulations proposed for LE equipment.

Start with DTV into DTV and DTV into NTSC D/U’s.

RF front-end saturation versus receiver de-sensitization => RF spectrum mask for the LE devices. 
There are different ways to deal with RF mask, for example, channel adjacent to a DTV broadcast operation may never be used for WRAN, thus no need to specify LE device RF rejection in adjacent channels.

8.1.3 Sensing Thresholds: (See 802.18SG1 comments to FCC)

8.1.4 Response Time: Time during which Broadcast can be interfered with before the frequency change. (e.g., if new broadcast station comes on during night=> not critical)

Wireless microphone: more demanding => msec! Unless a beacon is turned on before the microphone use. [Need more information, Shure to provide.]  [We have to go by the rules of the FCC, Shure asked to reserve 6 TV channels for wireless microphones. Even with these channels, they should use beacons in order to provide a means to identify those that operate according to Part 74 so that those that are ‘unlicensed’ do not pre-empt the use of the spectrum for ‘License-Exemp’ operation.]

Need to investigate methods for enhancing the ability to sense licensed wireless microphone operation.

What was done at 5 GHz for DFS at 5 GHz: channel availability check time= 60s, channel move time= 10s, channel closing time= 200m].  ThenNumbers for WRAN will probably need to be different.

Ned analysis on the impact on LE client, on transmission continuity.

#### 8.1.3	Distributed Sensing

802.22 system shall employ distributed sensing where the base station and CPE have the capability to sence the RF sensing occupancy.  Information will considered at the base stationandmeasuresto avoid interference will be signalled to the CPE’s such as channelchange or power reduction.

At the base station: mapping interference situation based on CPE’s information according to their geographic coordinates and take decision on TV channels to be used.  The incumbent services will be better protected by such distribute RF sensing since it would avoid the ‘hidden node’ problem.  Such sensing will include the monitoring of broadcast operation, wireless microphone operation as well as the operation of other LE operation in the area.  Sensing on the operating channels will imply a command from the base station to set the exact time for sensing to generate a quiet period at the CPE’s and the base station.

8.1.4	Behaviors (including database updates)

TPC behaviour, link-by-link, sensing, response to sensing data, changing channels.  Although broadcasters have a reliable database and changes occur at a very slow pace, a 100% correct database cannot be assumed.  A database should be kept up-to-date on a regional basis.  [Taking the 1.91-1.93 GHz sharing as an example, the cost of keeping such a database could be supported by the LE operators.]

BSs and CPEs may maintain a "list" of backup channel(s) in case an incumbent is detected and a channel has to be vacated.  The exact scheme can be left to the proposals, as different approaches could be suggested to tackle this problem.  (Informative: appendix)

8.2	License-exempt system coexistence/sharing

Among WRAN’s as well as with other LE systems. If other LE systems don’t have  service profiles => problem

Need to have a common interface for sensing to later cooperate

Any license-exempt usage in the TV broadcast bands would need to use a beacon or a signature in its transmission signal that would include a transmitter identification.  This should be in the rule.  The identification of the transmitter would make resolving interference situations tractable.  802.22 will be the first group to define the sensing rules and the FCC need to monitor the work to develop the corresponding rules.

Coordination among 802.22 systems should be straightforward since all these system will operate according to the same standard.. Specific address space for each WRAN operation would be needed. This is required in order to ensure that interference resolution is always possible:

However, coordination with other LE systems will need to include conditions on the operating rules of these systems to allow resolution of interference.  Precedents exist for the 2 GHz LE PCS band where technical means of avoiding interference were defined in the ANSI standards.  These other LE systems should also have RF sensing and means to avoid interference.  There should be an industry group to work together to resolve the issue.

8.2.1	Overview

Extending the beacon concept to help cooperation among LE systems. Encourage the FCC to look into this capability.  FCC will insist to know how it will be tested.  Every 30 sec.  ANSI C63.17 1910 to 1930. Unlicensed PCS shared.    Need TX signature.

8.2.2	Sensing Thresholds and D/U Ratios

Other devices that could use beacons similar to those used for the wireless microphones would avoid interference but it would give them precedence over WRAN.  Need for a transmission signature to distinguish among DTV, wireless microphones, WRAN transmissions and other LE transmissions.

8.2.3	Response Time

802.19 is an outflow from 802.11 whereas 802.16h deals with this in the 802.16 context. Response time still to be defined.

8.2.4	Distributed Sensing

Since distributed sensing will exist for coexistence with incumbents, the same structure will be available for LE coexistence.

8.2.5	Behaviors  (including database updates)

## 8.3	RF Sensing

  - measurements that must be done are part of phy.  phy has to respond to commands from mac to do sensing and generate messages to the mac on results

  - cognitive aspects interwoven between phy and mac

  - sensing needs requirement in phy and mac.

  - bound problem and define what the standard has to specify but not specify explicitly.

  - sensing on occupied channel and other channels.  sensing on channel you're planning to occupy.  primary channel sensing, in-service monitoring, during quiet periods figure out if channel is occupied;  how frequently (dynamic nature of the usage), how long to sense? (sensitivity level)  besides initial scanning when CPE turns on, sensing is controlled by base station.  eg.  scan next available and others less frequently.

  - System shall sense at the specified threshold levels (Table of thresholds) How it will be done and give indication on the duration of the quiet period.

  - legitimate wireless microphones, use beacon? don't want to have to protect illegal mikes.

  - define rate at which sensing would be done, integration time, once we know thresholds.  spectrum signature, know what DTV looks like, know what mikes look like, NTSC looks like, DVB-T has null in centre of channel, narrow null would define observation of this.  need to include foreign signatures in this portion of the document.

## 8.3	Dynamic Frequency Selection

When interference is to be avoided, the systems shall change (vacate, etc. see section 7.2.2.1) to another channel that was found to be available. … time frame: detect, close, move, re-establish connection.

8.4	TPC

Define range and granularity. (802.16: 40 dB in 0.5 dB steps), Define acquisition process (short loop, long loop.), base station control.

(e.g., 802.11J: coverage class: timing for propagation in increments of 3 usec, TX power level: 256 steps in 0.5 dB from –102 dBm.)

# 9	Management

As outlined in IEEE Std 802-1990 [11], The LLC Sublayer, MAC Sublayer and Physical Layer standards also include a management component that specifies managed objects and aspects of the protocol machine that provide the management view of managed resources. The aspects of management considered are (FCAPS):

- Fault management

- Configuration management

- Accounting management

- Performance management

- Security (see also section 9)

The 802 standards define a framework for LAN/MAN management in ISO/IEC 15802-2:1995(E) [28]. The framework contains guidelines for managed objects, management protocol, and the relationship to ITU management protocols. 802.22 protocols SHALL comply with the above-mentioned standards and guidelines.

## 9.1	Service Level Agreements

The 802.22 protocols MUST permit operators to enforce service level agreements (SLAs) with subscribers by restricting access to the air link, discarding data, dynamically controlling bandwidth available to a user or other appropriate means [29]. The 802.22 protocols MUST also permit subscribers to monitor performance service levels of the 802.22 services being provided at the delivery point.

## 9.2	Malfunctioning CPE or Base Station

The operator MUST have means to shut down and reactivate a subscriber station if necessary, remote from the CPE, in the face of a malfunction. The operator also SHOULD have the means to securely shut down and reactivate a base station remotely. When such capabilities are available, the 802.22 protocols SHALL support a secure function. The 802.22 protocols SHOULD support functions that automatically shut down transmission from a CPE or base station in case of malfunction (e.g., power limits exceeded).

## 9.3	Accounting and Auditing

The 802.22 system management framework, architecture, protocols and managed objects MUST allow for operators to effectively administer accounting and auditing. An operator MUST be able to account for resource utilization and various service features for each subscriber service separately.  [The WRAN system protocols should allow control and management of the services on a subscriber basis provided to special CPE’s which bridge to a number of subscribers i.e., multi-apartment building served by a ‘first class’ CPE.)]  Also recall from section 2 (?) that a single CPE can interface to multiple subscribers that an operator could bill separately.

# 10	Security

The 802.22 system SHALL enforce security procedures described in this section.

The security system chosen by 802.22 SHALL be added to the protocol stack and reference points to include security protocols, and “database” servers for authentication, authorization, key management, service suspend/resume, relocation, anti-cloning, etc. [30, 31].

## 10.1	Authentication

There are two types of authentication for an 802.22 system. In the first type, a subscriber station MUST authenticate itself with the network every time it registers with the network. This authentication MUST prevent unauthorized subscriber station from entering the network or an unauthorized base station from emulating an authorized base station. This type of authentication MUST be supported by the 802.22 MAC layer.

The second type of authentication is between the subscriber and the WRAN system. This may or may not be the responsibility of the 802.22 protocols. It MAY be handled by higher layer protocols.

An additional level of authentication may exist between the other two. This additional layer is the authentication of the subscriber with the subscriber station. This is beyond the scope of the 802.22 protocols.

The authentication mechanisms MUST be secure so that an “enemy” subscriber station is not able to gain access to an 802.22 system, or to the core network beyond. Passwords and secrets MUST NOT be passed “in the clear” through the air interface.

## 10.2	Authorization

Authorization is a security process that determines what services an authenticated subscriber is permitted to invoke. Each subscriber has a set of credentials that describe what the subscriber is “allowed” to do. The 802.22 standard SHALL identify a standard set of credentials and allow for vendors to extend the defined credentials with non-standard credentials. Some possible credentials are:

- Permission to access the 802.22 system

- Permission to access certain services (IP, Remote Bridging, Digital Audio/Video, etc.)

- Permission to request specific service features, and/or QoS parameters, based on the SLA (resources, delay, etc.).

For each service offered, the 802.22 protocols MUST be capable of securely supporting authorization requests and responses.

## 10.3	Privacy

Privacy is a security concept that protects transmitted data from being intercepted and understood by third parties (e.g., an “enemy” subscriber station, base station or passively  “listening” radio). Wire-equivalent privacy (WEP) [5] and shared private key [5] privacy have been suggested as minimum required privacy levels for 802.22 systems.

802.22 standards SHOULD allow a suitable cryptographic algorithm to be employed that is internationally applicable. Facilities SHOULD also be defined in the protocol for the use of alternate cryptographic algorithms that can be used in certain localities and that can replace algorithms as they are obsoleted or “legalized” for international use.

# 11	802 Conformance

802.22 SHALLconform to the 802 architecture. The following  particulars with the 802 model (see IEEE Standards for Local and Metropolitan Area Networks: Overview and Architecture (IEEE Std 802-1990) [11]) shall apply:

- The 802.22 MAC supports 802 “universal” 48 bit addresses. (? 64 bit addresses? Carl:Tony Jeffrey, Goff Thompson and Floyd Backes to review)

- An 802.22 system supports MAC multicast in the downstream direction.

- The 802.22 protocols support 802.1 bridging services and protocols, including support of the 802.1q virtual LAN tag and 802.1D priority ID [32-34].

- The 802.22 protocols support encapsulation of 802.2 (LLC) [10] by the MAC protocol.

- Conform to the 802 conventions and structures for “interface primitives:” logical structures that are passed between protocol layers to invoke processes and transact data.

- Address the 802 system management guidelines (see section 8) [35].

- Provide a MAC service interface that complies to 802 conventions [12].

# 12	References

[1]	IEEE Standards Association. Project Authorization Request. Telecommunications and Information Exchange Between Systems - LAN/MAN Specific Requirements - Air Interface for Fixed Wireless Regional Area Network Systems in Licensed Bands from 2 to 11 GHz. March 30, 2000. http://ieee802.org/16/sub11/par/index.html.

[2]	IEEE LAN/MAN Standards Committee (IEEE 802 LMSC) Sponsor Executive Committee. 802.22 (Wireless Regional Area Network Air Interface Standard In Licensed Bands from 2 to 11 GHz): Meting the Five Criteria. March 9, 2000. http://ieee802.org/16/sub11/par/fivec.html.

[3]	Recommendation ITU-R F.1399. Vocabulary of terms for wireless access.

[5]	Marianna Goldhammer, 802.22sc-99/16 MAC Services.

[6]	S. Blake et al, "An Architecture for Differentiated Services", RFC 2475, December, 1998.

[7]	S. Blake et al, "A Framework for Differentiated Services", Internet Draft, October, 1998.

[8]	R. Braden et al., "Integrated Services in the Internet Architecture: An Overview", RFC 1633, June 1994.

[9] George Fishel, 802.22sc-99/22 Interface to MAC and LLC Protocols.

[10]	ISO/IEC 8802-2:1998. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 2: Logical Link Control.

[11]	IEEE Std 802-1990 "IEEE Standards for Local and Metropolitan Area Networks: Overview and Architecture. IEEE 1990.

[12]	ISO/IEC 10039: 1991. Information technology -- Open Systems Interconnection -- Local area networks -MAC service definition.

[13]	ISO 7498-1:1984. Information technology -- Open Systems Interconnection -- Basic Reference Model.

[14]	James Mollenauer, 802.16sc-99/5 Functional Requirements for Wireless Regional Area Network Networks.

[15]	Imed Frigui, 802.16sc-99/23 Services and Performance requirements for Broadband Fixed Wireless Access.

[16] Robert Duhamel, 802.16sc-99/17 LMDS Cell Sizing and Availability.

[17]	Recommendation ITU-T G.826: Error performance parameters and objectives for international, constant bit rate digital paths at or above the primary rate (2/99).

[18]	Recommendation ITU-R F.1491. Error performance objectives for real digital radio links used in the national portion of a 27 500 km hypothetical reference path at or above the primary rate.

[19]	C.W. Lundgren and W.D. Rummler, "Digital radio outage due to selective fading observation vs. prediction from laboratory simulation," Bell System Technical Journal, pp.1073-1100, May-June 1979.

[20]	M. Emshwiller, "Characterization on the performance of PSK digital radio transmission in the presence of multipath fading," ICC'78 Conference Record, Toronto, Ontario, CANADA, Paper 47.3.

[21]	Microwave digital radio systems criteria, Bellcore Technical Reference TR-TSY-000752, October 1989.

[22]	W.D. Rummler, R.P. Coutts, and M. Linger, "Multipath fading channel models for microwave digital radio," IEEE Communications Magazine, November 1986, pp.30-42.

[23]	R. K. Crane, "Prediction of Attenuation by Rain" IEEE, 1980.

[24]	Recommendation ITU-R F.1189-1. Error performance objectives for constant bit rate digital paths at or above the primary rate carried by digital radio-relay systems which may form part or all of the national portion of a 27500 km hypothetical reference path. (1995-1997).

[25]	CCIR Recommendation 749, Radio-Frequency channels arrangements for radio-relay systems operating in the 36.0 to 40.5 GHz band. (1992).

[26]	John Liebetreu, 802.22sc-99/13 Dispersive Fade Margin: A Physical Layer Performance Metric.

[27]	RFC-2205 Resource ReSerVation Protocol (RSVP) -- Version 1 Functional Specification. R. Braden, Ed., L. Zhang, S. Berson, S. Herzog, S. Jamin. September 1997. Status: PROPOSED STANDARD.

[28]	ISO/IEC 15802-2:1995. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 2: LAN/MAN management.

[29]	Jim Mollenauer, 802.22sc-99/7 Functional Requirements for the 802.22 Standard.

[30]	IEEE 802.10-1998. IEEE Standards for Local and Metropolitan Area Networks: Standard for Interoperable LAN/MAN Security (SILS).

[31]	IEEE 802.10c-1998. IEEE Standards for Local and Metropolitan Area Networks: Supplement to Standard for Interoperable LAN/MAN Security (SILS) -- Key Management (Clause 3).

[32]	ISO 15802-3:1998. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 3: Media Access Control (MAC) bridging.

[33]	ISO/IEC 15802-5:1998. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 5: Remote Media Access Control (MAC) bridging.

[34]	IEEE 802.1Q-1998. IEEE Standards for Local and Metropolitan Area Networks: Virtual Bridged Local Area Networks.

[35]	IEEE 802.1F-1993. IEEE Standards for Local and Metropolitan Area Networks: Common Definitions and Procedures for IEEE 802 Management Information.

# Appendix A
Requirements Summary (Informative)

This section contains tabular summaries or requirements found in the text of this document. Requirements are separated into three categories: required, recommended and optional.

Each requirement is numbered for easy reference. Future revisions of this document will keep the requirement reference numbers intact such that the number for a requirement will not change from revision to revision.

To better discern the meaning and intent of a requirement, please refer to the text.

## A.1	Mandatory

It is mandatory that the 802.22 standard support or specify the items in Table 2.

Table 2: Mandatory Requirements

| # | Section | Requirement |
| --- | --- | --- |
| M1 M2 | 1 1.1 | The forthcoming air interface standard MUST comply with the system requirements. The 802.22 air interface interoperability standard SHALL be part of a family of standards for local, metropolitan and wide area networks. |
| M3 | 2 | 802.22 systems SHALL be deployable in multiple-cell frequency reuse systems and single cell (super cell) frequency reuse systems. |
| M4 | 2.3 | Since all data traffic in a single cell of an 802.22 network MUST go through the base station, that station SHALL serve as a radio resource supervisor. |
| M5 | 2 | The base station radio SHALL be P-MP, radiating its downstream signal with a shaped sector or adaptive array (spatial reuse) antenna achieving broad azimuthal beam width to “cover” a prospective number of subscribers. |
| M6 M7 | 2.1 2.2 | The standard SHALL specify MAC layer protocols and PHY transmission techniques suitable for providing access between one or more subscriber stations and base stations to support UNI and CNI requirements. The 802.22 protocols SHALL support the optional deployment of repeater functions. |
| M8 | 2.2 | The repeater function SHALL NOT affect the MAC protocol other than the delay which the repeater might introduce into the system. |
| M9 M10 | 2.3 3 | In the upstream direction, 802.22 protocols MUST provide the means to multiplex traffic from multiple subscriber stations, resolve contention, and allocate capacity. The MAC and PHY protocols will not have explicit support for each and every service, due to the fact that generic data streams SHALL be used for transport. |
| M11 | 3 | The MAC and PHY protocols SHALL provide for QoS service specific support, resulting in appropriate BER for data services, limited delay for real |
| M12 | 3.1 | time services, etc. 802.22 systems SHALL support voice communications for subscribers in a way that eases the migration of legacy voice communications equipment and public switched telephone network (PSTN) access technologies to 802.22 systems. |
| M13 M14 | 3.1 3.2 | The 802.22 voice access transport SHALL be packet based (as opposed to circuit-switched based). The 802.22 system MUST directly transport variable-length IP datagrams efficiently. |
| M15 | 3.2 | Both IP versions 4 and 6 MUST be supported. |
| M16 M17 | 3.2 3.4 | The 802.22 IP service MUST provide support for real-time and non-real time service capabilities. These services SHALL NOT place any additional requirements on 802.22 systems (MAC and PHY protocols) not already covered in the above sections. |
| M18 | 4 | The IEEE 802.22 MAC and PHY protocol stacks SHALL be the same for all the supported services. |
| M19 M20 | 4 4 | Further details, and finalization of the protocol reference model, SHALL be worked out by the 802.22 MAC and PHY task groups while developing the air interface interoperability standard. Since key layers above the MAC require service guarantees, the MAC protocol MUST define interfaces and procedures to provide guaranteed service to the upper layers. |
| M21 M22 | 4 5.2 | Since customer units will contend for capacity to/from one or more base stations, the MAC protocol MUST efficiently resolve contention and resource allocation. 802.22 protocols SHALL be optimized to support the peak data rate in either or both directions to a subscriber station within the specified distance from the base station. |
| M23 M24 | 5.4 5.6 | The PHY and MAC protocols SHALL provide for multirate support. The 802.22 specifications SHALL NOT preclude the ability of the radio link to be engineered for different link availabilities, based on the preference of the system operator. |
| M25 M26 M27 | 5.6 5.6 5.7 | 802.22 MAC and PHY protocols MUST accommodate these conditions, perhaps consuming more radio bandwidth and/or requiring smaller radio propagation distance (radius) to meet the availability requirements. Since statistical atmospheric and path conditions vary widely in geography, the 802.22 protocols MUST be flexible in consumed radio bandwidth (spectral efficiency), cell radius, and transmit power. The error rate, after application of the appropriate error correction mechanism (e.g., FEC), delivered by the PHY layer to the MAC layer SHALL meet IEEE 802 functional requirements with the following exception: the radio link bit error ratio (BER) SHALL be 10E-6 (in accordance with ITU FWA |

| M28 | 5.9 | recommendations or better. The standard SHALL support careful planning to ensure that subscribers’ |
| --- | --- | --- |
| M29 | 5.9 | quality of service guarantees and minimum error rates are met. The delivered base station capacity can vary depending on attenuation due to |
| M30 | 5.9 | atmospheric conditions, LOS blockage, transmit power, etc., and SHALL be calculated as the aggregate capacity of all sectors supported by a base station. The MAC and PHY protocols MUST accommodate channel capacity issues |
|  |  | and changes in channel capacity to meet contracted service levels with customers. |
| M31 | 5.9 | As subscribers are added to 802.22 systems, the protocols MUST |
|  |  | accommodate them in an automated fashion. |
| M32 | 6.1 | This standard SHALL permit two duplex modes of operation: Frequency |
| M33 | 6.1 | Division Duplex (FDD) and Time Division Duplex (TDD). The PHY and MAC protocols MUST provide for duplex (i.e. bi-directional) |
| M34 | 6.1 | operation, while preserving the QoS, BER and spectral efficiency requirements for data and voice traffic. The MAC and PHY protocols MUST provide means to resolve applicable |
|  |  | collocation and interference problems. |
| M35 | 6.2 | The MAC and PHY protocols MUST permit the operation with channel |
| M36 | 7 | spacing of 1.75, 3.5 and 7MHz when using ETSI masks and 1.5 to 25MHz when using other masks. Table 1 provides a summary of the QoS requirements that the PHY and MAC |
|  |  | SHALL provide. |
| M37 | 7 | 802.22 protocols MUST support classes of service (CoS) with various |
| M38 | 7 | quality of service (QoS) guarantees to support the services that an 802.22 system MUST transport. Thus, 802.22 protocol standards MUST define interfaces and procedures that |
|  |  | accommodate the needs of the services with respect to allocation and prioritization of resources. |
| M39 | 7 | 802.22 protocols MUST provide the means to enforce QoS contracts and |
| M40 | 7 | Service Level Agreements. For QoS-based, connectionless services, the 802.22 protocols MUST support resource negotiation “on-demand” |
| M41 | 7 | If 802.22 is to be a “link” in the IP network, an IWF MUST interface with |
| M42 | 7.2 | 802.22 to negotiate resource allocation. 802.22 protocols SHALL define a set of parameters that preserve the intent |
| M43 | 7.3 | of QoS parameters for IP-based services. The classes of service and QoS parameters of services SHALL be translated |
|  |  | into a common set of parameters defined by 802.22. |
| M44 | 8.1 | The 802.22 protocols MUST permit operators to enforce service level |
|  |  | agreements (SLAs) with subscribers by restricting access to the air link, discarding data, dynamically controlling bandwidth available to a user or other |

| M45 | 8.1 | appropriate means. The 802.22 protocols MUST also permit subscribers to monitor performance |
| --- | --- | --- |
| M46 | 8.2 | service levels of the 802.22 services being provided at the delivery point. When such capabilities are available, the 802.22 protocols SHALL support a |
| M47 | 8.2 | secure function. The operator MUST have means to shut down and reactivate a subscriber |
|  |  | station if necessary, remote from the subscriber station, in the face of a malfunction.8.2 |
| M48 | 8.3 | The 802.22 system management framework, architecture, protocols and |
| M49 | 8.3 | managed objects MUST allow for operators to effectively administer accounting and auditing. An operator MUST be able to account for resource utilization and various |
|  |  | service features for each subscriber service separately. |
| M50 | 9 | The 802.22 system SHALL enforce security procedures described in this |
| M51 | 9 | section. The security system chosen by 802.22 SHALL be added to the protocol stack |
| M52 | 9.1 | and reference points to include security protocols, and “database” servers for authentication, authorization, key management, service suspend/resume, relocation, anti-cloning, etc. There are two types of authentication for an 802.22 system. In the first type, |
|  |  | a subscriber station MUST authenticate itself with the network every time it registers with the network. This authentication MUST prevent unauthorized subscriber station from entering the network or an unauthorized base station from emulating an authorized base station. This type of authentication MUST be supported by the 802.22 MAC layer. |
| M53 | 9.1 | The authentication mechanisms MUST be secure so that an “enemy” |
| M54 | 9.1 | subscriber station is not able to gain access to an 802.22 system, or to the core network beyond. Passwords and secrets MUST NOT be passed “in the clear” through the air |
| M55 | 9.2 | interface. The 802.22 standard SHALL identify a standard set of credentials and allow |
| M56 | 9.2 | for vendors to extend the defined credentials with non-standard credentials. For each service offered, the 802.22 protocols MUST be capable of securely |
|  |  | supporting authorization requests and responses. |

## A.2	Recommended (R)

- It is recommended that the 802.22 standard support or specify the items in Table 3 “Recommended” means that there may exist valid reasons in particular circumstances to ignore an item, but the full implications should be understood and the case carefully weighed before choosing a different course.

Table 3: Recommended Requirements

| # | Section | Requirement |
| --- | --- | --- |
| R1 | 1.1 | Other goals of this document are to formulate reference models and terminology for both network topology and protocol stacks that help the 802.22 working group to discuss and develop the MAC and PHY protocols. As far as possible, these SHOULD be common across 802.22 systems. |
| R2 | 2.2 | The repeater function SHOULD NOT affect the end-to-end operation of 802.22 protocols between BS and SS. |
| R3 R4 | 3.2 3.2 | For efficient transport of IPv6, TCP/IP header compression over the air interface SHOULD be supported. It SHOULD be possible to support the emerging IP Quality of Service (QoS) efforts: Differentiated Services and Integrated Services. |
| R5 | 3.3 | The 802.22 protocols SHOULD support bridged LAN service capabilities, whether directly or indirectly, including always on, ad hoc and on-demand communication in either or both directions. |
| R6 R7 | 4 5.1 | Note that the function of the MAC protocols SHOULD include error correction by retransmission, or Automatic Repeat Request (ARQ), whereas, in the 802 model, those functions if necessary, are provided by the LLC layer. The 802.22 protocols SHOULD allow for different “scales” of capacity and performance for 802.22 system instances. |
| R8 | 5.2 | The 802.22 MAC protocol SHOULD allow the peak data rate to scale beyond 10 Mbps. |
| R9 R10 | 5.3 5.5 | The PHY and MAC protocol SHOULD provide for far CPEs’ propagation delay compensation. 802.22 protocols SHOULD allow for flexibility between delivered upstream and downstream capacity and CoS/QoS. |
| R11 R12 | 5.6 5.6 | An 802.22 system SHOULD be available to transport all services at better than their required maximum error rates (see section 5.7) from about 99.9 to 99.99% of the time, assuming that the system and radios receive adequate mains power 100% of the time and not counting equipment availability. 802.22 MAC and PHY protocols SHOULD specify functions and procedures to adjust transmitter power, modulation, or other parameters to accommodate rapid changes in channel characteristics. |
| R13 | 5.9 | Given the propagation characteristics in a given frequency band and geographic area, and the development of a link budget, the following parameters of an 802.22 system SHOULD be addressed by the MAC and PHY protocols: ~ Radio range (up to 50 Km) ~ Upstream/downstream channels’ data rates ~ Allocation of prospective subscriber data rate to channels. Note: the MAC and PHY standards MAY allow subscribers to hop between channels ~ Types of modulation |

| R14 | 6.2 | The typical value [channel spacing] for performance analysis SHOULD be 3.5MHz for the ETSI mask and 3MHz for the MDS mask. |
| --- | --- | --- |
| R15 | 7.3 | 802.22 protocols SHOULD include a mechanism that can support dynamically-variable-bandwidth channels and paths (such as those defined for IP environments). |
| R16 | 8 | The 802 standards define a framework for LAN/MAN management in ISO/IEC 15802-2: 1995(E). The framework contains guidelines for managed objects, management protocol, and the relationship to ITU management protocols. 802.22 protocols SHOULD comply with the above-mentioned standards and guidelines. |
| R17 R18 R19 | 8.2 8.2 9.3 | The operator also SHOULD have the means to securely shut down and reactivate a base station remotely. The 802.22 protocols SHOULD support functions that automatically shuts down transmission from a subscriber station or base station in case of malfunction (e.g., power limits exceeded). 802.22 standards SHOULD allow a suitable cryptographic algorithm to be employed that is internationally applicable. |
| R20 | 9.3 | Facilities SHOULD also be defined in the protocol for the use of alternate cryptographic algorithms that can be used in certain localities and that can replace algorithms as they are obsoleted or “legalized” for international use. |
| R21 | 10 | 802.22 SHOULD strive to fit into the 802 system model. |

## A.3	Optional (O)

It is optional that the 802.22 standard support or specify the items in Table 4.

Table 4: Optional Requirements

| # | Section | Requirement |
| --- | --- | --- |
| O1 | 2.1 | The model [Figure 2-2: Wireless Access Reference Model] depicts the relevant |
| O2 | 2.1 | points between subscriber networks and “core” networks (the networks that MAY be accessed via 802.22 air interface). A single SS MAY support multiple customer premises networks that transport |
| O3 | 2.1 | data, voice and video through one or more UNIs. Base stations MAY support multiple core networks through one or more CNIs. |
| O4 | 5.9 | The MAC and PHY standards MAY allow subscribers to hop between |
| O5 | 5.9 | channels. Flexible modulation types, power level adjustment, and bandwidth reservation |
| O6 | 7 | schemes MAY be employed [which affect how the MAC and PHY protocols meet contracted service levels with customers]. [To support on-demand resource allocation,] The MAC protocol MAY allocate |
|  |  | bursts of PDUs to services that require changes in resource allocation. |
| O7 | 7 | The 802.22 MAC layer interface MAY provide a connection-less service |
|  |  | interface that requires a higher-layer “adaptation” to maintain the “state” of a connection and periodically allocate resources. |
| O8 | 9.1 | The second type of authentication is between the subscriber and the WRAN |
|  |  | system. This may or may not be the responsibility of the 802.22 protocols. It MAY be handled by higher layer protocols. |

# Appendix B
Vocabulary of Terms (Normative)

Editor’s Note: Some definitions below are as yet undefined. And a few other are as yet incomplete or in-progress. The 802.22 task group should provide updates for these definitions and release a new version of this document. The in-progress definitions are in italics.

| Term | Definition | Reference |
| --- | --- | --- |
| Access | End-user connection(s) to core networks NOTE 1 - Core networks include, for example, PSTN, ISDN, PLMN, PSDN, Internet, WAN/LAN, CATV, etc. NOTE 2 - The end-user may be a single user or a user accessing the services on behalf of multiple users. | Based on Rec. ITU-R F.1399 |
| Accounting | A function which apportions the revenue obtained by the service providers to network operators in line with commercial arrangements. | Rec. ITU-R M.1224 |
| Ad hoc communication | Use of the wireless access system without reservation or QoS guarantees at sporadic times, by activating the link when needed. |  |
| Air interface | The common boundary between the subscriber station and the radio equipment in the network, defined by functional characteristics, common radio (physical) interconnection characteristics, and other characteristics, as appropriate. NOTE 1 – An interface standard specifies the bi directional interconnection between both sides of the interface at once. The specification includes the type, quantity and function of the interconnecting means and the type, form and sequencing order of the signals to be interchanged by those means. | Based on Rec. ITU-R M.1224 |
| Always on communication | Use of the wireless access system at any time by having the link continuously available. |  |
| Authentication | The process of verifying the identity of a user, terminal, or service provider. | Rec. ITU-R M.1224 |
| Authorization | A property by which the rights to resources are established and enforced. | Rec. ITU-R M.1224 |
| Backhaul service | Transport of aggregate communication signals from base stations to the core network. | IEEE 802.22 |
| Bandwidth; communication channel bandwidth | The information payload capacity of a communication channel available to a user for services (expressed in bit/s or multiples thereof). |  |
| Bandwidth; RF channel bandwidth | The frequency extent of a specified portion of the RF spectrum capable of carrying information over the radio interface (expressed in Hz or multiples thereof). |  |
| Bandwidth; transmission channel bandwidth | The frequency extent required for the transmission of a specified signal (expressed in Hz or multiples thereof). |  |
| Base station | The common name for all the radio equipment located at one and the same place used for serving one or several cells. (See also “station”). | ITU-R Rec. M.1224 |
| Bearer service | A type of telecommunication service that provides the capability for the transmission of signals between user-network interfaces. | ITU-T Rec. I.112 |
| Broadband wireless access | wireless access in which the connection(s) capabilities are higher than the primary rate. | Rec. ITU-R F.1399 |
| Cell | The radio coverage area of a base station, or of a subsystem (e.g. sector antenna) of that base station corresponding to a specific logical identification on the radio path, whichever is smaller. | Based on Rec. ITU-R M.1224 |
| Cell | A block of fixed length which is identified by a label at the asynchronous transfer mode layer of the B-ISDN protocol reference model. | ITU-T Rec. I.113 |
| Cell delay variation | A component of cell transfer delay, induced by buffering and cell scheduling. The proportion of lost cells over the total number of transmitted cells for a connection. | ATM Forum ATM Forum |
| Channel; communication channel | A specific portion of the information payload capacity, available to the user for services. | ITU-T Rec. I.113 |
| Channel; radio- frequency (RF) channel | A specified portion of the RF spectrum with a defined bandwidth and a carrier frequency and is capable of carrying information over the radio interface. | Rec. ITU-R M.1224 |
| Channel; transmission channel Class of Service Core network Customer premises | A means of unidirectional transmission of signals between two points. This is either a Customer Class of Service, a Trunk Class of Service, or a Private-Facility Class of Service. It may refer to either originating or terminating accesses. Core networks include, for example, PSTN, ISDN, PLMN, PSDN, Internet, WAN/LAN, CATV, etc. The equipment/network administered by the user. | ITU-T Rec. I.112 ITU-T Rec. Q.1290 (95), 2 Based on Rec. ITU-R F.1399 Based on ITU-T Rec. |

| equipment/network |  | H.310 |
| --- | --- | --- |
| Downstream | The direction from base station to subscriber station(s). | IEEE 802.22 |
| Dynamically variable bandwidth | A capability of a system to be able to change the bandwidth of the information payload capacity of a communication channel available to a user for services according to negotiated user requirements. |  |
| External Network | Any network that is physically located past the base station or the subscriber station. |  |
| Fixed wireless access | Wireless access application in which the base station and the subscriber station are fixed. | Based on Rec. ITU-R F.1399 |
| Frequency Division Duplex | Separation of upstream and downstream transmission in the frequency domain at the same time. | IEEE 802.22 |
| Internet protocol | Networking protocol defined by IETF standards. | IETF |
| Interoperability | The ability of multiple entities in different networks or systems to operate together without the need for additional conversion or mapping of states and protocols. | Rec. ITU-R M.1124 |
| Inter-working | The means of supporting communications interactions between entities in different networks or systems. | Rec. ITU-R M.1124 |
| Inter-working function | Mechanism which masks the differences in physical, link, and network technologies by converting or mapping states and protocols into consistent network and user services. | Rec. ITU-R M.1124 |
| Link Budget | The allocation of power within a link. Link budget consists of the difference between Tx power and receiver sensitivity. |  |
| Line-of-sight (LOS) | A radio communication link for which 60% or greater of the first Fresnel zone is unobstructed. (i.e., the geographical region where the rules of free space propagation applies). Two points for which the signal suffers temporal fading of less than 10 dB 99% of the time. Note: This definition does not imply that a direct optical line of sight exists. |  |
| Modulation Factor | The sector’s aggregate payload bit rate divided by the bandwidth. |  |
| Multi-tenant | A primarily non-commercial building designed to |  |

| dwelling | contain more than two residential units. |  |
| --- | --- | --- |
| Near LOS | A radio communication link for which 40% or greater of the first Fresnel zone is obstructed. Two points for which the signal suffers temporal fading greater than or equal to 10 dB exceeding 1% of the time and an average delay spread of less than 10 microseconds. |  |
| Non LOS | A radio communication link that is obstructed. Two points for which the signal suffers an average delay spread of greater than or equal to 10 microseconds. |  |
| Network Nomadic wireless access | A set of nodes and links that provides connections between two or more defined points to facilitate telecommunication between them. Wireless access application in which the subscriber station may be in different places but must be stationary while in use. | Rec. ITU-R M.1224 Based on ITU-R Rec. F.1399 |
| On demand communication | Use of the wireless access system for services based on negotiated QoS. |  |
| Peak Data Rate | The maximum bit rate available to a user for communications. |  |
| plesiochronous mode Point-to-multipoint system | A mode where the essential characteristic of time scales or signals such that their corresponding significant instants occur at nominally the same rate, any variation in rate being constrained within specified limits. a system that establishes connections between a single specified point and more than one other specified points. | ITU-T Rec. G.810 (96), 4.3.5 ITU-R Rec. F.1399 |
| Privacy Quality of service | The provision of capabilities to prevent access of information by unauthorized parties. The collective effect of service performance which determine the degree of satisfaction of a user of the service. NOTE 1 - The quality of service is characterized by the combined aspects of service support performance, service operability performance, serviceability performance, service security performance and other factors specific to each service. NOTE 2 - The term "quality of service" is not used to express a degree of excellence in a comparative sense nor is it used in a quantitative sense for | ANSI T1.702-1995 ITU-T Rec. E.800 (94), 2101 |

|  | technical evaluations. In these cases a qualifying adjective (modifier) should be used. |  |
| --- | --- | --- |
| Radio interface Real-Time (adjective) | See air interface Pertaining to the processing or transmission of data according to defined time requirements . | Rec. ITU-R M.1224 Based on ITU-T Rec. Q.9 (88), 6103 |
| Re-use Factor; Frequency re-use factor | As yet undefined |  |
| Sector | As yet undefined |  |
| Sector Bandwidth Factor | As yet undefined |  |
| Security | The protection of information availability, integrity and confidentiality, as well as authentication and authorization. | Based on Rec. ITU-R M.1224 |
| Semi-stateless | As yet undefined | Sect 7 |
| Service Service Capability | A set of functions offered to a user by an organization. Defines possible service arrangements with respect to the number and type of channels that can be supported by the equipment. In this way a subscriber station access capability is the set of possible channel configurations supported by the equipment. Similarly, the base station access capability may be considered to be a super-set with respect to the subscriber station. | Recs. ITU-R M.1224, M.1308 Based on ITU-T Rec. Q.1063 (88), 2.4 |
| Single Residential | A non-commercial building intended to be inhabited by an individual. |  |
| Small Business | As yet undefined |  |
| Small Office/Home Office Spectrum (Spectral) Efficiency | As yet undefined Measures of the accuracy and completeness of system tasks relative to the spectral resources used to achieve the specific system tasks | Based on ITU-T Rec. F.901 (93), 2 |
| Standard | A document established by consensus and approved by a recognized body, that provides, for common and repeated use, rules, guideline or characteristics for activities or their results, aimed at the achievement of the optimum degree of order in a given context. | ISO/IEC Guide 2, 1996 |
| Station | the common name for all the radio equipment at one and the same place. NOTE - The term "station" may refer to any end | Rec. ITU-R M.1224 |

|  | user radio equipment (“subscriber station”) or network radio equipment (“base station”). |  |
| --- | --- | --- |
| Subscriber Subscriber station Supplementary service Synchronous transfer mode | A person or other entity that has a contractual relationship with a service provider on behalf of one or more users. (A subscriber is responsible for the payment of charges due to that service provider.) the common name for all the radio equipment at one and the same place serving one or more users. (See also “station”). A service which modifies or supplements a basic telecommunication service. Consequently, it can not be offered to a customer as a standalone service, rather, it must be offered together with or in association with a basic telecommunication service. The same supplementary service may be common to a number of telecommunication services. A transfer mode which offers periodically to each connection a fixed-length block. | Rec. ITU-R M.1224 Based on Rec. ITU-R M.1224 Rec. ITU-R M.1224 Based on ITU-T Rec. I.113 |
| System | A regularly interacting or interdependent group of items forming a unified whole technology. | Recs. ITU-R M.1224, M.1308 |
| Telecommuter | As yet undefined |  |
| Time Division Duplex | Separation of upstream and downstream transmission in the time domain using the same frequency. | IEEE 802.22 |
| Upstream | The direction from subscriber station(s) to base station. | IEEE 802.22 |
| User | Any entity external to the network which utilizes connections through the network for communication. | ITU-T Rec. E.600 |
| Virtual point-to- point connections | Providing a point-to-point connection to a subscriber using a point to multipoint system. | IEEE 802.22 |
| Wireless access | end-user radio connection(s) to core networks. NOTE 1 - Core networks include, for example, PSTN, ISDN, PLMN, PSDN, Internet, WAN/LAN, CATV, etc. NOTE 2 - The end-user may be a single user or a user accessing the services on behalf of multiple users. | Rec. ITU-R F.1399 |

# Appendix C
Acronyms and Abbreviations

| Acronym/Abbreviation | Definition |
| --- | --- |
| AF | Assured Forwarding |
| ANSI | American National Standards Institute |
| ARQ | Automatic repeat Request |
| BER | Bit Error Ratio |
| BES | Best Effort Service |
| BS | Base Station |
| WRAN | Wireless Regional Area Network |
| CNI | Core Network Interface |
| CoS | Class of Service |
| CSMA/CD | Carrier Sense multiple Access with Collision Detection |
| DFM | Dispersive Fade Margin |
| DSL | Digital Subscriber Line |
| EF | Expedited Forwarding |
| ER | Errored Block |
| ETSI | European Telecommunications Standard Institute |
| FCAPS | Fault, Configuration, Accounting, Performance, Security Management |
| FDD | Frequency Division Duplex |
| FEC | Forward Error Correction |
| FWA | Fixed Wireless Access |
| HFC | Hybrid fiber coax |
| HTTP | Hypertext Transfer Protocol |
| IEC | International Electrotechnical Commission |
| IETF | Internet Engineering Task Force |
| IP | Internet protocol |
| ISDN | Integrated Services Digital Network |
| ISO | International Organization for Standardization |
| ITU-R | International Telecommunications Union- Radiocommunications Sector |
| IWF | Inter-working function |
| JRG | Joint Rapporteur group |
| LAN | Local area network |
| LLC | Logical link control |
| LOS | Line of Sight |
| MAC | Medium Access Control |
| MAN | Metropolitan area network |
| MGMT | Management |
| OSI | Open Systems Interconnection |
| PDU | Protocol Data Unit |
| PHY | Physical layer |
| PMD | Physical Medium Dependent |
| P-MP | Point-to-multipoint |
| PSTN | Public Switched Telephone Network |
| QoS | Quality of service |
| RF | Repeater Function |
| SDE | Secure Data Exchange |
| SEC | Security |
| SES | Severely Errored Second |
| SLA | Service Level Agreement |
| SME | Small and Medium Enterprises |
| SNMP | Simple Network Management Protocol |
| SOHO | Small Office/Home Office |
| SS | Subscriber Station |
| TC | Transmission convergence |
| TCP/IP | Transmission Control Protocol/Internet Protocol |
| TDD | Time Division Duplex |
| UNI | User Network interface |
| VoIP | Voice over IP |
| WEP | Wire-Equivalent Privacy |

# Appendix D
Propagation Model

# 1- ITU-R P1546-1

# 2- Specific sites with real terrains at these sites (Carl and Gregg)

# Appendix E

# Channel Models Eli  (Gerald, Victor)

- channel models

  - 802.16 had lower limit of 2GHz, need to look

  - ITU model (Rec. 370 and later Rec 1546) uses FCC F-curves updated and augmented.

  - 802.16 doc (80216a-03_01_potential_channel_model.pdf) is a good basis for developing this Appendix.  It deals with all physical aspects of the channel.  It will need to be adjusted to the VHF and UHF bands.  It will need to include the consideration of industrial (man-made) noise that is not negligeable in these lower frequency bands.  Broadcast models developed for DTV for DTVwill need to be considered.  Channel delay spread impulse response models developed for mobile telephony such as the European model COST 207 would be useful to characterize multipath to be considered in the development of the modulation for 802.22.

  - COST 207 [1989]  Digital land mobile radio communications.  Final Report of COST Project 207, Commission of the European Communities, Brussels.

  - presence of on-channel repeaters if it is include in the 802.22 standard will change the range of possible multipath excess delays because of the presence of passive as well as active echoes will need to be considered.  Passive channel multipath range is around 25usec for echoes with sizable power but active echoes will extend it greatly.  DTV receiver equalizers typically cover a range of -20 to +40 usec.