IEEE P802.22
Wireless RANs

| Functional Requirements for the 802.22 WRAN Standard |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-03-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Carl R. Stevenson | WK3C Wireless LLC | 4991 Shimerville Road, Emmaus, PA 18049-4955 USA | +1 610-965-8799 | wk3c@wk3c.com |
| Carlos Cordeiro | Philips | 345 Scarborough Rd Briarcliff Manor, NY 10510 USA | +1 914 945-6091 | carlos.cordeiro@philips.com |
| Eli Sofer | Runcom | Israel |  | elisofer@runcom.co.il |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | +1 613-998-2500 | gerald.chouinard@crc.ca |

Abstract

Introduction

This document represents a collaborative effort by the 802.22 Functional Requirements Ad Hoc Group, starting from discussions held at the January 2005 interim session and drawing and adapting the structure and content of document IEEE 802.16.3-00/02r4 as a framework and source of ideas for further work by the Ad Hoc Group.

Abstract

This document provides functional requirements that are guidelines for developing an interoperable 802.22 air interface. The 802.22 committee desired to reach an understanding and consensus for functional requirements before proceeding with developing a standard for 802.22 MAC and PHY protocols and thus formed a Functional Requirements Ad Hoc Group to produce this document.

Purpose

The 802.22 WRAN WG group will use this document to help qualify MAC and PHY protocol proposals.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Functional Requirements
for the 802.22 WRAN Standard

Xxxx Yyyy (editor)

# 1	Introduction

This document provides functional requirements that are guidelines for developing an interoperable 802.22 air interface for use in spectrum allocated to TV Broadcast Service, enabling Point to Multipoint (P-MP) Wireless Regional Area Network (WRAN). The WRAN system provides packet-based transport capabilities that can support a wide range of services (e.g., data, voice and video) to residential, Small and Medium Enterprises (SME) and Small Office/Home Office (SOHO) locations.

For convenience, requirements are itemized in Appendix A.

Throughout this document, the words that are used to define the significance of particular requirements are capitalized. These words are:

- “MUST” or “SHALL” These words or the adjective "REQUIRED" means that the item is an absolute requirement.

- “MUST NOT” This phrase means that the item is an absolute prohibition.

- “SHOULD” This word or the adjective “RECOMMENDED” means that there may exist valid reasons in particular circumstances to ignore this item, but the full implications should be understood and the case carefully weighed before choosing a different course.

- “SHOULD NOT” This phrase means that there may exist valid reasons in particular circumstances when the listed behavior is acceptable or even useful, but the full implications should be understood and the case carefully weighed before implementing any behavior described with this label.

- “MAY” This word or the adjective “OPTIONAL” means that this item is truly optional. One implementation may include the item because the target marketplace requires it or because it enhances the product, for example; another implementation may omit the same item

# 2	Scope

For the purposes of this document, a “system” constitutes an 802.22 MAC and PHY implementation in which at least one subscriber station communicates with a base station via a point-to-multipoint (P-MP) radio air interface, the interfaces to external networks, and services supported by the MAC and PHY protocol layers. Hence, “functional requirements” describe the functions of typical systems in terms of how they affect requirements of interoperable 802.22 MAC and PHY protocols. The functional requirements describe 802.22 systems and requirements in broad terms: what the required functions are but not how these functions work.

The ‘how’ part is left to the forthcoming 802.22 interoperability standard [1] which will describe in detail the interfaces, functions and procedures of the MAC and PHY protocols. This document focuses on the service capabilities that an 802.22 system is required to support. These service capabilities have a direct impact on the requirements of the 802.22 MAC and PHY protocols. When the 802.22 working group produces an interoperable air interface standard that meets these functional requirements, resulting 802.22-based implementations will be able to utilize and interconnect multi-vendor WRAN devices to provide the expected services to the end users.

Other goals of this document are to formulate reference models and terminology for both network topology and protocol stacks that help the 802.22 working group to discuss and develop the MAC and PHY protocols. As far as possible, these SHOULD be common across 802.22 systems.

The 802.22 protocols relate to other 802 standards and to the OSI model as shown in Figure 1-1.

Figure 1-1: Relationship between 802.22 and other Protocol Standards
(the numbers in the figure refer to IEEE standard numbers)
(Remove 801.10, change 802.16.3 for 802.22)

This standard deals with the Physical and Data Link layers as defined by the International Organization for Standardization (ISO) Open Systems Interconnection Basic Reference Model (ISO 7498: 1984).

The standards that define the services noted in the above diagram are as follows:

- IEEE Std 802: Overview and Architecture. This standard provides an overview to the family of IEEE 802 Standards. This document forms part of the 802.1 scope of work.

- ANSI/IEEE Std 802.1B [ISO/IEC 15802-2]: LAN/MAN Management. Defines an Open Systems Interconnection (OSI) management-compatible architecture, environment for performing remote management.

- ANSI/IEEE Std 802.1D [ISO/IEC 10038]: MAC Bridging. Specifies an architecture and protocol for the interconnection of IEEE 802 LANs below the MAC service boundary.

- ANSI/IEEE Std 802.1E [ISO/IEC 15802-4]: System Load Protocol. Specifies a set of services and protocols for those aspects of management concerned with the loading of systems on IEEE 802 LANs.

- ANSI/IEEE Std 802.2 [ISO/IEC 8802-2]: Logical Link Control.

# 3	PAR Summary

[Informative, no MUST and SHOULD.]  Carl to develop.

# 4	Target Markets

The target markets described in this section are not an exhaustive set, but serve as guidelines and examples that suffice for meeting the broad applicability goals set forth by the air interface “Five Criteria” as described in the IEEE 802.22 Project Authorization Request (PAR) and “Five Criteria” [1, 2].

A Wireless Regional Area Network (WRAN) system based on 802.22 protocols is intended to make use of unused TV broadcast channels, on a non-interfering basis, to address, as a primary objective, rural and remote areas and low population density underserved markets with performance levels similar to those of broadband access technologies serving urban and suburban areas The WRAN system should also be able to scale to serve denser population areas where spectrum is available.

- The WRAN system MUST be capable of supporting a mix of data, voice (VoIP) and audio/video applications with corresponding QoS guarantees.  The RF link availability assumed for the provision of these WRAN applications is 99.9% of time.

The critical parameters for serving these markets using wireless access technology is the combination of coverage / capacity factors that affects access cost per user, the deployability, maintainability and product costs associated with the customer premise installation, and the spectrum efficiency / reuse for economically serving the required number of customer locations with a minimum number of base station locations and backhaul routes.

The target markets to be addressed by the 802.22 protocols in WRAN networks are single-family residential, multi-dwelling units, SOHO, small businesses, multi-tenant buildings and public and private campuses.

In accordance with ITU-R [3] definitions, Fixed Wireless Access (FWA), to which WRAN belongs provides access to one or more (public and private) core networks, rather than forming an end-to-end communication system. 802.22 systems serve fixed location customers who might be geographically fixed or re-locatable.

# 5	WRAN System Model

This section presents a high level description of a system model to be used as a framework for developing the 802.22 standard. The model identifies the main features of an 802.22 system, and the terminology to be used by the 802.22 working group in the creation of the standard.

The 802.22 wireless regional area network system is aimed at providing broadband access with capabilities similar to the ADSL and cable modem technologies over less populated rural areas.  The typical range of the system is [33] km (based on 4 Watt CPE EIRP and F(50, 99.9)) for a coverage of population density of about 1.25 person/km2 and above, and up to a maximum of [100] km when transmit power in not a constraint.  The system will need to operate over a set of typical channels models as defined in Appendix D.

As mentioned in section 1.1, an 802.22 “system” constitutes an 802.22 MAC and PHY implementation in which at least one subscriber station communicates with a base station via a point-to-multipoint (P-MP) radio air interface, and services supported by the MAC and PHY protocol layers. Specific applications of the 802.22 point-to-multipoint (P-MP) radios are aimed at the use of the VHF/UHF TV broadcast frequency range. Radio communications in the above range may be possible in near- and non-line-of-sight situations between a base station and subscriber station. Operation may include partial and even complete blockage by foliage.  This will contribute to signal attenuation and multipath effects. Figure 2-1 shows an example deployment configuration including the optional use of macro diversity (optimization of link and use of repeaters) (2nd class base station, see section 5.6.1.2.1). 802.22 systems should be deployable in multiple-cell frequency reuse systems and single cell (super cell) frequency reuse systems. The range of 802.22 radios varies with EIRP, local topography, atmospheric conditions, channel characteristics, availability requirement and local regulations as well as bandwidth and transmitter/receiver performance.

Figure 2-1 Example Deployment Configuration
(figure should be fixed to be more appropriate to the WRAN environment, need to include houses with TV receivers, from contributors)

An 802.22 system MUST consist of one base station radio and one or more Consumer Premise Equipment (CPE) radios.  It defines an 802.22 base station and one or more stationary CPE radios communicating using the 802.22 MAC and PHY protocols. Proposals for 802.22 should also include description of how repeaters could be accommodated.  The base station radio SHALL be P-MP, radiating its downstream signal (forward) toward the CPE’s with an omni-directional, a shaped sector or optionally an adaptive array (spatial reuse) antenna achieving broad azimuthal beam width to “cover” a number of prospective subscribers.

For the purpose of coexistence with incumbent services operating in these TV broadcast bands (TV broadcasting, wireless microphones operating according to FCC Part 74 and Public Safety), the 802.22 standard shall include mechanisms in the PHY and MAC protocols to allow the base stations to dynamically change the frequency of operation based on the sensing of the use of the spectrum by these incumbent services by the CPE’s to avoid interference to these services (see section 8).  This will constitute an essential part of this License-Exempt transmission standard.  For the purpose of coexistence among WRAN systems operating in the same area, the 802.22 standard shall include mechanisms allowing cooperation between base stations for better sharing of the spectrum.  The MAC and PHY protocols MUST provide means for base stations to resolve interference problems due to collocation or coverage area overlapping.

The frequency bands used by 802.22 systems vary among governed geographies (see section 8.1.1.1).  In the case of the USA, the frequency range identified by 802.18 in its comments to the FCC was from TV channel 2 to 51, (54 MHz to 698 MHz) the 802.22 PAR identifies 54 MHz to 863 MHz but the extremes of the international range is from 41 MHz to 910 MHz.  Since the 802.22 system will have to operate without causing interference to incumbent services, sensing of channel occupancy shall be done according to the regulatory requirements of the local Administration.

## 5.1	Wireless Access Reference Model

Figure 2-2 shows the 802.22 wireless access reference model. The model depicts the relevant points between subscriber networks and “core” networks. A greater system encompassing user terminals, base station interconnection networks, network management facilities, etc. may be envisaged but the 802.22 protocol focuses on the air interface shown in the model. The Core Network Interface (CNI) and the User Network Interface (UNI) are also shown.

A single Customer Premise Equipment (CPE) MAY support multiple customer premises networks that transport data, voice and video through one or more UNIs. Base stations MAY support multiple core networks through one or more CNIs. For the purposes of 802.22, the UNI and CNI are abstract concepts. The details of these interfaces are beyond the scope of this document. The standard SHALL specify MAC layer protocols and PHY transmission techniques suitable for providing access between one or more CPE and base stations to support UNI and CNI requirements.

Figure 2-2: Wireless Access Reference Model
(figure needs to be fixed, sensing interface …)

### 5.2	IEEE 802 Reference Architecture model

[“The 802.22 standard shall be in conformance with the IEEE 802.1 Architecture, Management and Interworking documents as follows: 802. Overview and Architecture, 802.1D, 802.1Q and parts of 802.1f.”]

Optional MAC Bridge to other networks (e.g. 802.11x, others?)

Data link layer: MAC and AAC.  Done at LLC layer, would be transparent to the lower layers such as MAC and PHY.  LLC bridging would be easily done when the standard is set.  This has more to do with interoperability and higher performance of bridges as compared to routers.  Various LANs can use bridging and switching to interconnect.  802.22 should not be an isolated standard: talk to other 802 standards through MAC (LLC) bridging.  Interconnection to other IEEE standards will need to connect at the network layer through routers.

(System Management, Database Management interface, Station Management, Database Management, MAC, PHY)

Have to confirm that the top of the 802.22 MAC protocol (LLC) will be fully compatible with the 802.1 bridging, 802.architecture, etc.  [Carl, Gerald]

## 5.3	Service capacity

The required minimum peak throughput rate at edge of coverage shall be 1.5 Mbit/s per subscriber in the forward direction and 384 kbit/s per subscriber in the reverse direction.  The capacity of the base station will need to be much higher to provide service to a number of subscribers in this P-MP system.  The system SHALL operate with a minimum spectrum efficiency of 0.5 bit/(sec*Hz) and the average spectrum efficiency should be 3 bit/(s*Hz). CPE should be built to operate at as high spectrum efficiency as possible, as allowed by regulations or/and standard characteristics.

## A.1	Spectral efficiency vs Distance

Since the 802.22 WRAN system is to operate primarily in lightly populated rural areas where more spectrum is expected to be available than in urban areas since broadcasting occupies fewer channels.    If adaptive modulation is used (for example 256-QAM and 64-QAM could be used to serve close-in subscriber stations, down to 16-QAM, QPSK and BPSK at the edge of coverage, see 5.2), the system will operate with a minimum spectrum efficiency of 0.5 bit/(sec*Hz) and a average efficiency of 3 bit/(sec*Hz) including error coding.

The modulation capacity will be adaptive based on C/N, BER, etc.  This will translate in a reduction of throughput as a function of the distance from the base station, i.e., lower transmission capacity toward the edge of the coverage area.  [The environment noise, including interference generated by incumbents and other WRAN systems on the same channel or on adjacent channels (see ATSC A-74 Receiver Guideline), and burst noise will have an impact on the choice of depth of interleaving and switching mechanism between modulation levels.]  [Service availability in presence of thermal noise, interference, channel linear (multipath static) and non-linear (transmitter and receiver non-linear operation) characteristics and impulsive noise will need to be considered and apportioned among these various causes of defects.]

## A.2	Antenna Characteristics )

### A.2.1	Base

#### A.2.1.1	Operational

The PHY layer should allow the use of omni, sectorized as well as directional antennas that cover a specified segment of the entire frequency range (see section 9.2 of the Functional Requirement document).  The system should use vertical polarization from CPE to base station to allow maximum isolation of the transmit CPE antenna toward nearby outdoor TV receiving antennas. The transmit antenna could use either horizontal or vertical polarization but horizontal polarization would allow for better isolation between transmit and receive antennas at the base station and at the CPE’s in the case where full duplex operation is used).

Omni

Transmit and receive antennas at the base station could be omni-directional to cover the area surrounding the base station

Sectorized

Transmit and receive antennas at the base station could also be sectorized to allow for the system capacity to be concentred per sector.  Some frequency re-use would be possible through sufficient antenna sidelobe rejection.

Directional

Directional antennas (log-periodic, corner reflector, etc.) could be used to concentrate the  coverage in specific directions.

#### A.2.1.2	Sense (omni)

The sensing antenna should be omni-directional  with a minimum gain of 0dBi .

### A.2.2	CPE

#### A.2.2.1	Operational

The transmit antenna should use vertical polarization to allow maximum isolation of the transmit CPE antenna toward nearby outdoor TV receiving antennas (note however that depolarization is possible from coupling with near-by angled piece of metal), whereas the receive antenna could be either horizontally or vertically polarized.  [Max Muterspaugh to make the two reports on de-polarization available on the reflector.]

Vertical polarization would allow for better isolation between transmit and receive CPE antennas mounted on the same mast or even integrated into a common physical antenna in the case of full duplex operation. If half duplex operation is selected for WRAN, the forward signal transmitted from the base station could also be vertically polarized because isolation between the transmit and receive CPE antenna would not be needed.

Different types of antennas (Yagi-Uda, log-periodic, corner reflectors, etc.) could be used at the CPE.  Realistic antenna gains seem to be 7 dBi in VHF and 10 dBi in UHF.  Higher gain could be achieved by properly stacking similar antennas in horizontal and/or vertical directions.  [Input from Radio-Shack, Winnegard, ChannelMaster, CEA antenna group.]

If separate transmit and receive antennas are installed on the same mast, a minimum distance should be secured between the two antennas to minimize coupling.  The transmit antenna would normally be located above the receive antenna to minimize the actual power transmitted. Transmit Power Control would automatically compensate for the better channel propagation..

Omni

Although omni-directional antennas at the CPE would avoid the need of re-aiming the antenna when a new WISP operator is selected, directional antennas SHALL to be used to avoid interference coming in and going out of the CPE in all directions.

Directional

Antennas with minimal gain and backlobe rejection should be used outdoors.  Special care will need to be used for vertically polarized antennas at the CPE to minimize the coupling problem with the metallic mast, especially for Yagi-Uda and log-periodic antennas.

#### A.2.2.2	Sense (omni)

The sensing antenna will need to be mounted outside with the operational antennas.  The sensing antenna SHALL be omni-directional  in the horizontal plane. It SHALL have a minimum gain of 0dBi on both horizontal and vertical polarizations at any frequency in which the terminal is capable of operating including the necessary adjacent channels. .  The horizontal polarization will be needed for sensing TV transmission and the vertical polarization will be used to sense nearby CPE’s, whereas the polarization from wireless microphones could be anything.

## 5.4	Installation Requirements

Base stations will be professionally installed.

The CPE antennas SHALL be installed outdoor at 10m above ground with proper azimuth and elevation alignment.  CPE’s could be user installable (plug and play) but operators may chose to provide for professional installation.

Verification of the installation should be done either visually or remotely from the base station if the proper means exist (i.e., compass and GPS receiver at the CPE antenna)

## A.3	Equipment integrity

The WRAN equipment should be tamper-proof to avoid any unauthorized modification to the hardware, firmware and/or functionalities, e.g., cognitive functionality, RF sensing, DFS, TPC, tuning).

## 5.6	Network Topology

### 5.6.1	P-MP Star

In the downstream direction, all the traffic to be sent to the CPE’s SHALL be transmitted by the base station. In the upstream direction, 802.22 protocols MUST provide the means to efficiently multiplex traffic from multiple CPE’s and allocate transmission channel capacity.  The base station SHALL serve as a radio resource supervisor and controller.

A Master/Slave relationship between the base station and the CPE’s  SHALL be established whereby all the RF characteristics of the CPE’s will be remotely controlled by the base station.  The base station SHALL also remotely control the distributed sensing of RF Environment.

5.6.2	Optional Repeater Function

The 802.22 systems may support the optional deployment of repeater functions. Proponent may wish to propose mechanisms for repeaters to extend coverage into areas that could not otherwise be reached by the service and document the impact of the use of these repeaters on the functionality of the PHY and MAC layers.  The repeater function SHOULD NOT affect the end-to-end operation of 802.22 protocols between the base station and the CPE’s.

There can be four types of repeaters:

- Re-generating repeaters (2nd class base station),

- on-channel, (need large isolation between transmit and receive antennas)

- off-channel

- Non-re-generating repeaters (possible with low modulation complexity)

- on-channel (RF repeaters), (need very large isolation between transmit and receive antennas)

- off-channel (RF translators)

## 5.7	Wireless Media Characteristics

### 5.7.1	Multiple access techniques

Need to support ….(Carl) (???)

### 5.7.2	Duplex Modes

This standard SHALL support duplex modes of operation. Half-duplex can also be considered as long as service latency is kept to an acceptable level.  This covers the cases of the TDD mode and a FDD mode where duplex operation is assumed at the base station but half-duplex operation is used at the CPE to reduce the user terminal complexity.  If FDD is selected, the proposal must include band plan or propose a scheme to achieve it.  A study of the relative merits of FDD versus TDD can be found in documents:

22-05-0020-00-0000_TDD_FDD_Tradeoffs.doc

22-05-0022-02-0000_TDD_vs_FDD.doc

5.7.3	Flexible Asymmetry

Symmetry is hard to predict and is very bursty depending on the type of traffic.  Some applications utilize naturally asymmetrical bit rate, such as for generic Internet access where most of the throughput is consumed in the downstream direction. Some applications utilize asymmetrical bit rate in the reverse direction, using more in the upstream direction, such as surveillance video from a CPE. Other applications require symmetrical capacity, such as telephony and video conferencing [15].  Systems should have the flexibility to adjust to the bit rate requirements in both directions.  WRAN systems must support flexible asymmetry.  It should be recognized that CPE’s at the edge of coverage may not be capable of achieving the same bit rate in the upstream direction as the base station can provide to them in the downstream direction.

6	Supported Service Capabilities

This section describes typical service capabilities supported by an 802.22 air interface. The MAC and PHY protocols will not need to have explicit support for each and every service, due to the fact that generic data streams SHALL be used for transport. The MAC and PHY protocols SHALL provide for admission control, QoS service specific support, appropriate PER for the various types of services, and acceptable latency and jitter for real time services.

6.1	Data Transport Service Capabilities – Internet and VoIP

The 802.22 system should directly transport variable-length IP datagrams efficiently. Both IP versions 4 and 6 MUST be supported with packet prioritization, admission control and scheduling.

The 802.22 systems should provide support for real-time and non-real-time service capabilities. The 802.22 protocols should provide for VoIP services.  It SHOULD be possible to support IP Quality of Service (QoS) efforts: Differentiated Services [RFC 2475- Dec 1998, RFC 2474, latest reference?].

The following types of services shall be supported: (see 802.1 and 802.3 for priority and access control)

- Constant bit rate service

- Real-time variable bit rate service

- Non-real-time variable bit rate service

- Best-effort service

## 6.2	Bridged LAN Service Capabilities

The 802.22 protocols SHALL support 802.1 bridged LAN service capabilities.

## 6.3	Other Services

Other services that, for instance, require QoS-based delivery of the MAC services may be added. These services SHALL NOT place any additional requirements on 802.22 systems (MAC and PHY protocols) not already covered in the above sections.

# 7	802.22 Protocols

The 802.22 protocols determine the interoperability of multiple vendors' equipment. Protocol interoperability occurs at each level in the protocol “stack” [9]. In this section, services refer to the services provided by the protocols that can appear in the layer sitting directly over the MAC layer. IEEE 802 protocols reside at layers 1 and 2 and consist primarily of Logical Link Control (802.2) [10] and the various MAC and PHY layers for each LAN or MAN standard. The IEEE Std 802-1990 Overview and Architecture [11] describes these layers.

The 802.22 protocol stack reference diagram is shown in Figure 4-1. In addition to the LLC, MAC and PHY layers suggested by the generic 802 architectures [11] [12] [13], 802.22 protocols support other categories of “upper protocols” that correspond to the requirements of the service capabilities described in section 6 of the Functional Requirements document.

Figure 4-1: Protocol Stack Reference Model
(figure needs to be fixed, change 802.16.3 for 802.22 …)

This protocol stack reference model is intended to help develop terminology, and possibly protocol architecture. Each of the protocols above the PHY and MAC support “convergence  sub-layers.” The convergence sub-layers [14] [15] may be necessary to:

- Encapsulate PDU framing of upper layers into the native 802.22 MAC/PHY PDUs [15]

- Map an upper layer’s addresses into 802.22 addresses

- Translate upper layer CoS/QoS parameters into native 802.22 MAC constructs

- Adapt the asynchronous, synchronous or isochronous data pattern of the upper layer into the equivalent MAC service

- Reduce the need for complex inter-working functions (IWFs) [15]

Other convergence sub-layers may be required by additional higher layer service requirements.

The IEEE 802.22 MAC and PHY protocol stacks SHALL be the same for all the supported services. The central purpose of the MAC protocol layer in 802.22 is sharing of radio channel resources. The MAC protocol defines how and when a base station or subscriber station may initiate transmission on the channel. Since CPE’s will contend for capacity to/from one or more base stations, the MAC protocol MUST efficiently manage contention and resource allocation. The PHY layer may be subdivided between a convergence layer and a physical medium dependent (PMD) layer. The PMD is the “main” part of the PHY. Like the MAC convergence layers, the PHY convergence layers adapt/map the “special” needs of the MAC services to generic PMD services.

# 8	Performance and Capacity

This section addresses some issues regarding 802.22 system performance and capacity. Specifying protocols that can maintain specified/mandatory performance levels in the face of fluctuating and diverse channel characteristics (e.g., due to multipath and atmospheric conditions) is a problem that the 802.22 work group has to consider. This section specifies the target performance levels. This section also outlines some of the issues for 802.22 capacity planning.

Note that ITU-R has presented several questions regarding the need for performance objectives  for fixed wireless access radio systems, in particular, the activities being carried out within the  Joint Rapporteur Group (JRG) 8A/9B (Questions ITU-R 140/9, ITU-R 215/8, ITU-R 220/9).

## 8.1	Peak Data Rate

The minimum peak rate at the edge of service is 1.5 Mbit/s downstream and 384 kbit/s upstream.

802.22 protocols SHALL be optimized to support the minimum peak data rate in both directions to a CPE station within the specified distance from the base station. Optional modes that can provide different rates may be included.  The 802.22 MAC protocol SHOULD allow the peak data rate to scale up to the total practical channel capacity (see section xxx).  We can assume e.g. 3 bps/Hz, but the total system capacity will depend on what fraction of the 6/7/8 MHz bandwidth should be assumed, given the spectral mask constraints are still TBD?  Need to document in an appendix, and just put conclusion in this section.  Need to decide on max modulation rate, BW of channel, FEC and other overhead.]

## 8.2	Propagation Delay

The large cell radius, up to [33 –100] km, will produce a large propagation delay with round trip delays as large as 186us. The PHY and MAC protocol SHOULD provide for far CPEs’ propagation delay compensation.

## 8.3	Multipath/Delay Spread Performance

See channel model in Appendix D

## 8.4	Ranging / Synchronization / Acquisition / / Access

The first time a CPE is turned on, it will start by sweeping the RF range in which it is to operate in to identify presence and signal signature of incumbent operation, as well as access information from the WRAN networks accessible in the area.  A list of these WRAN networks will be presented to the user for a choice.  The CPE will then try to contact the selected WRAN base station according to the parameters sent over broadcast control packets.  Initial hand-shaking with the base station will be done for ranging, initial synchronization and CPE initialization.

Ranging SHALL be done in a way to minimize the required guard-time interval to maximize the transmission medium capacity.  Part of the channel can be used for ranging.    Synchronization of the CPE in frequency and time SHALL be re-established with the base station every time the CPE tries to access the network.  Acquisition should then be made following the conditions (e.g., network security) stipulated by the network.

Access time should be no more than 10 sec.  Within this time frame, up to 12 CPE’s will try to access the network at the same time.  However there are some applications such as tele-voting where faster access time would be required by a large number of users for short messages.  The WRAN system SHALL also allow for such applications.

8.5	Frequency/time tolerances

The frequency stability of 802.22 base stations SHALL be better than 2 ppm (re. 802.16 (2004) Section 8.4.14.1). Base stations SHALL be able to operate with or without GPS lock.  Tolerance for the CPE’s should be more relaxed than that of the base station to minimize CPE complexity.  [Phase noise for free running CPE oscillators could be as high as 20-30 ppm.]  However, the CPE’s SHALL be capable of synchronizing to the base station to adjust their drift in frequency and time within the 2 ppm tolerance of the base station.  The CPE shall have enough range to capture the base station frequency and time.  The CPE shall be capable of acquiring frequency and time synchronization in a sufficiently short period to secure proper service.

## 8.6	Spectral Efficiency

The spectral efficiency is an important performance parameter of a wireless access system. This spectral efficiency will vary from location to location within the coverage area of the base station, depending on distance, propagation, channel impairment and interference.

A minimum spectral efficiency of 0.5 bit/(s*Hz) and a average efficiency of 3 bit/(s*Hz) shall be supported.

## 8.7	Radio Link Availability

An 802.22 system SHOULD be available to transport all services at better than their required maximum error rates (see section 10.3) based on a link availability of 99.9% of the time [14, 16], (not counting access time, quiet periods for RF sensing, and electrical power and equipment down-time. The 802.22 specifications SHALL NOT preclude the ability of the radio link to be engineered for different link availabilities, based on the preference of the system operator.

Guideline to the proponents as to how to respond: Produce a curve for each proposed system linking spectrum efficiency to field strength (i.e., fade, distance) for given PERs.  Another curve may need to be produced linking spectrum efficiency and distance for a same field strength to see the impact of varying the round-trip delay on MAC.

WRAN receivers will likely be affected by the presence of high power broadcast transmitters operating on adjacent channels due to the apparent increase in noise level.  This will need to be considered in meeting the 50% availability location figure. In addition, the quiet periods required for RF sensing will need to be considered in meeting the time availability figure.

Unlike link availability, service availability encompasses radio link (fade) availability, down time due to coexistence, down time due to network congestion, electrical power and equipment failure.  That is why current broadband access systems don’t promise service availability.

For a data network, a period of unavailable time begins at the onset of ten consecutive SES events based on the following definitions [17].

- Severely Errored Second (SES) is defined as a one-second period which contains 30% errored blocks.

- Errored Block (EB): A block is defined as a set of consecutive bits associated with the path. Consecutive bits may not be contiguous in time. A block is typified as data block containing an error detection code for service performance monitoring. An errored block is a block in which one or more bits are in error.

It is expected that the highest contributor to 802.22 system outage will be excessive attenuation due to blockage and multipath due to varying path impediments such as reflections. In a multicell environment, intercell interference can not be neglected as an outage increasing factor.  802.22 MAC and PHY protocols MUST accommodate these conditions, perhaps consuming more radio bandwidth and/or requiring smaller radio propagation distance (radius) to meet the availability requirements. Since statistical atmospheric and path conditions vary widely in geography, the 802.22 protocols MUST be flexible in consumed radio bandwidth (spectral efficiency), cell radius, and transmit power. Bandwidth and cell radius are critical components of system/cell capacity planning (also see section 5.9).

## 8.8	Radio Link Error Performance

The error rate, after application of the appropriate error correction mechanism (e.g., FEC), delivered by the PHY layer to the MAC layer SHALL meet IEEE 802 functional requirements (10-8 undetected corrupted packet rate) and the radio link Packet Error Ratio (PER) SHALL be 10% or better for a packet length of 1000 octets at the minimum specified receiver sensitivity.

## 8.9	Delay

Delay and variation of delay, or jitter, are important factors to consider. For example, a high variation of delay can severely impact interactive services. However, generic Internet access can tolerate a high degree of delay variation.  Jitter for the services considered should be maintained to less that 0.5 msec.

The end-to-end delay depends on an entire application-specific network encompassing all 7 layers of the OSI model.

Maximum latency caused by PHY and MAC should be 20 msec (VoIP).

| Service Class / Application | service class | Typical data rate/volume | 1-way delay | Round trip delay | Variation (jitter) | PER / PLR | Security |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Broadcast video | Class A/B | 8 Mbps | 100 ms | N.A. | ±500 ns | 1×10-7 | 64 bit |
| Two-way videophone | Class A | 144 kbps | N.A. | 250 ms | ±20 ms | 1% | 64 bit |
| Broadcast audio | Class A | 128 kbps | 100 ms | N.A. | ±500 ns | 1×10-6 | 64 bit |
| Voice over IP | Class A | 64 kbps | 10 ms | 20 ms | ±10 ms | 1% | 64 bit |
| - Voice messaging | Class A | 32 kbps | 1000 ms | N.A. | ±10 ms | 3% | 40 bit |
| Two-way data for |  |  |  |  |  |  |  |
| Command / Control | Class D | ~1 kB/ 80 bps | 100 ms | 200 ms | N.A. | 5% | 40 bit |
| Webbrowsing | Class C | ~100 kB |  | 250 ms | N.A. | Zero | 64 bit |
| Webhosting | Class C | ~500 kB |  | 100 ms | N.A. | Zero | 64 bit |
| Interactive games | Class A/B | <1 kB | 10 ms | 20 ms | N.A. | Zero | 64 bit |
| Transactions | Class D | <10 kB | 2000 ms | 3000 ms | N.A. | Zero | 64 bit |
| Fax | Class B | 64 kbps | 2000 ms | 3000 ms | N.A. | Zero | 64 bit |
| Photography | Class C | <100 kB | 1000 ms | 2000 ms | N.A. | Zero | 64 bit |
| Emergency/tele-voting |  |  |  |  |  |  |  |

Figure 8.1:  QoS requirements for different service classes and applications

8.10	Capacity

802.22 base station capacity requirement is defined as the product of the number of subscribers, their peak bandwidth requirements and load factor based on quality of service guarantees. The standard SHALL support careful planning to ensure that subscribers’ quality of service guarantees and minimum error rates are met.

Done!

The delivered base station capacity can vary depending on attenuation due to atmospheric conditions, LOS blockage, transmit power, etc., and should be calculated as the aggregate capacity of all sectors supported by a base station. Sector capacity is defined as the product of two factors: the “modulation factor” and the “sector-bandwidth factor” based on quality of service guarantees. The “modulation factor” is defined as the sector’s aggregate bit rate divided by the bandwidth. The “sector-bandwidth factor” is defined as the total frequency band available for the WRAN service, adjusted by the appropriate frequency re-use factors. This reflects mainly the factor of frequency reallocation and the ability to optimize frequency usage.  WRAN should utilize multiple access technologies which offer excellent NLOS operation in multipath environment

Given the propagation characteristics in the target frequency bands and variability in differing geographic area for the development of a link budgets [16], the following parameters of an 802.22 system SHOULD be addressed by the MAC and PHY protocols:

- Radio range ([33] to [100] km)

- Upstream/downstream channels’ data rates

- Allocation of prospective subscriber data rate to channels. Note: the MAC and PHY standard MAY allow subscribers to hop between channels

- Types of modulation

The MAC and PHY protocols should accommodate channel capacity issues and changes in channel capacity. For example, flexible modulation types, power level adjustment, and bandwidth reservation schemes MAY be employed (see section 6) . Also, as subscribers are added to 802.22 systems, the protocols MUST accommodate them in an automated fashion.

The time-variant impairments (e.g., multipath interference) is expected to be the most significant contributor to channel impairments and complexity in cell capacity planning [16, 17, 19-26].

## 8.11	Delivered Bandwidth

The minimum delivered peak data rate per subscriber should be 1.5 Mbit/s forward and 384 kbit/s return).  The WRAN system should be able to provide to CPE’s, higher bit rate than the previously stated minimum peak data rate if the system capacity is available, up to the full forward throughput to one subscriber, at times of low system loading), and if the power is available on the return link, i.e., a CPE relatively close to the base station may, as directed by the base station, use the power margin of its TPC to achieve higher return throughput.

## 8.12	Flexible data rate and resource allocation – Downstream & Upstream

System capacity allocation should be able to vary dynamically.

The system should provide its operator to flexibly allocate its system resources to any CPE for the purpose of limiting or enhancing its throughput. , In the case of a re-generating repeater, the system should be capable of providing for more capacity to the ‘CPE’ that would act as the relay for this ‘2nd class’ base station to extend the coverage.

### 8.12.1	Peak Data Rates

CPE’s SHALL be able to receive the full throughput from the base station in the forward direction and utilize the full throughput of the return channel as long as the transmit power limit is not exceeded.  The base station should be capable of allocating full system throughput to a user on forward and return directions.  On the return channel, the throughput will be RF power limited.  Maximum peak bit rate will reduce toward fringe because of need for more robust and less efficient modulation.

### 8.12.2	Number of User Terminals serviced by base station

Typically, there will be more that one user terminal connected to the household and SOHO CPE.  In order to assess the number of user terminals that a base station can support, an over-subscription ratio of 50 has been assumed.

Assuming an average 3 bit/(s*Hz) spectrum efficiency, a 6 MHz channel, and the required minimum peak data rate of 1.5 Mb/sec per user terminal, this results in a total of 600 user terminals potentially being served by a single base station. Assuming an average of 3 user terminals per CPE, the number of CPEs addressed by a base station will be typically 200.

# 9	Flexible Adaptive Performance

## 9.1	Scalability

The 802.22 protocols SHOULD allow for different “scales” of capacity and performance for the 802.22 WRAN systems.  In order to take advantage of such scalability in capacity and performance, the WRAN systems will need to be adaptable.  In a P-MP type of operation, such adaptability shall be fully controlled by the base station, leaving the CPE’s to be fully remote controllable.  Adaptability refers both to modification of specific transmission parameters as well as to download of updated firmware and software for the CPE`s.

Scalability is needed in terms of bit rate, channel bandwidth (varying channel bandwidth within the TV channel or using a variable number of TV channels), extent of coverage, deployment, etc.

CPE should be built to support 100% duty cycle at rated power to cover for cases where maximum upstream throughput is required from a terminal at the edge of the coverage.  It may not be a requirement for all CPE’s available on the market but the option should be available.

There is a possibility for base stations having different classes of transmit power with different conditions of operation from the regulatory environment.

### 9.1.1	Per-Subscriber Rate Adaptation

802.22 systems can use different modulations and/or coding options depending on the distance between the base station and the CPE’s, the extent of blockage and the interference level. For example, the data rate to/from relatively near subscribers can be higher, increasing the overall system capacity. Additionally, far subscribers can experience different signal blockage and interference situations and so would benefit from rate adaptation. The PHY and MAC protocols SHALL provide for multirate support.

The rate adaptability SHALL operate within the following range: minimum spectrum efficiency of 0.5 bit/(s*Hz), with a goal of at least of 5 bit/(s*Hz) where propagation permits.

### 9.1.2	 Per-Subscriber Power Adaptation (TPC)

First, in the case of specific control frames such as for association, the base station needs to be heard by all terminals.  These control frames will need to be sent at constant high power so that the fringe terminals can also receive it.  However the base station doesn't have to be hearable by everybody all the time.  The base station will transmit data packets on a link-by-link basis and Transmit Power Control (TPC) will allow a reduction of the transmit power at the base station and at the CPE’s to lower levels still sufficient to establish a reliable connection (e.g., 10 dB above minimum C/N for the given modulation and FEC).

PC settings will be related to the actual channel conditions for each connection and will also be linked to the modulation and FEC used for the link and the access mode (e.g., TDD, FDD, contention, pooled).  The range for TPC will be at least 40 dB with 0.5 dB steps. (802.16 values) (e.g., 802.11J: coverage class: timing for propagation in increments of 3 usec, Tx power level: 256 steps in 0.5 dB from –102 dBm.)  Short loop TPC will need to be used at the CPE’s at the time of the link acquisition whereas long loop TPC controlled at the base station will be used once the link is established.  This central control will help manage interference situations.

Long loop TPC on the return link will allow all CPE’s to be received at similar power at the base station receiving antenna (constant received level in TDD and equivalent carrier levels hitting the FFT in the base station receiver).

Even though the power of the data packets will be adjusted on a link-by-link basis to always be at a reasonable reception level, the receivers will need to have good linearity over a large dynamic range in order to also be able to receive the control frames that will be transmitted at high power.

Forward link is:

1)	Burst transmission: needs synch, preamble, etc. can change modulation per frame, can change power per frame.  In case of broadcast frames used for channel acquisition, etc., these will be sent at maximum power to acquire terminals at edge of coverage.  The dynamic range of CPE’s will need to be very large to sustain such transmission when located close to the base station.

2)	Continuous transmission (i.e., broadcast stream), the base would be always transmitting, except during on-channel sensing period.  The transmission would therefore not be completely continuous and the transmitter frame structure will need to include frequency, timing and frame re-synchronization information.  Adaptive modulation may not be possible in the forward direction.

Return channel will need to use burst transmission and TPC.

[99.9% time availability means to be equated to a required fade margin and TPC set point.]

9.1.3	Bandwidth scalability

Depending on the findings on  out-of-band emission limits (RF mask) for the License-Exempt devices operating in the TV broadcast bands and the susceptibility of TV receivers to adjacent channels interference, a specific fraction of the TV channel will be usable by the 802.22 WRAN system. (see document 22-05-00xx-00-0000)  The operation of the WRAN system could also be restrained to a smaller portion when interference to wireless microphones and other LE systems is considered, and even for operational reason.  Depending on the above considerations, the 802.22 standard may need to include scalability with respect to occupied bandwidth.  This needs more work to be able to conclude.  As a minimum, the 802.22 standard SHALL include the scalability to make the best use of the 6, 7 and 8 MHz TV bandwidths that exist in the various part of the world.

Where the spectrum is available, it may be useful that a WRAN system could use  more than one TV channel (contiguous or not) to increase the capacity of the transmission link as long as each TV channel can become independent from the modulation point of view to be able to free any one of these channels in case of interference.  (802.18 commented to the FCC that channel bonding should not be persistent, see doc. 22-05-0003-00-0000_Suggested WRAN Functional Requirements.doc.)

#### 9.1.3.1	Channel Aggregation

Multi-channel aggregation, contiguous and non-contiguous to augment capacity  (e.g., OFDM occupying a number of TV channels where the carriers in a TV channel can be turned off if TV broadcast/wireless microphone operation starts).

In-channel aggregation: aggregation of operators within same TV channel.

Channel versus “TV channel”: need to be defined and consistent.

If multi-channel aggregation is proposed, each TV channel should be independent from modulation point of view (no channel bounding).

### 9.1.4	Coverage reach scalability

The typical reach of the WRAN systems should be [33] km and the 802.22 standard should allow a reach of up to 100 km.  In practical situations, 802.22 systems will also operate over much smaller radius of coverage.  The scalability of WRAN systems with respect to coverage range will involve the transmission power, the antenna height, the modulation and forward error correction (FEC) as well as the flexibility of the MAC protocol to absorb various propagation delays (acceptable delay in contention and pooling access algorithms) and the RF sensing characteristics.  The 802.22 standard SHALL include the necessary means and flexibility to allow for a full range of coverage reach up to 100 km.

9.1.5	Link symmetry scalability

The 802.22 standard SHOULD accommodate some level of  transmission rate asymmetry to allow efficient usage of the medium by all the applications that it is intended to carry. (See further discussion  under TDD/FDD in section 5.7.2.

9.2	Flexibility in Tuning

Although the total range over which the 802.22 systems may operate worldwide is from low VHF (42 MHz) to mid UHF (910 MHz), it is unlikely that, in practice, the total range could be covered by a common set of antennas and RF front ends at the base station and subscriber terminals.  It is expected that this range will be divided in more reasonable segments such as the low-VHF range (band 1), high VHF range (band 2), low-UHF (band 4) and medium UHF (band 5).  Although the flexibility in dynamic frequency selection to avoid interference will be limited to these segments, it is likely that such flexibility will be sufficient in most cases.  In more difficult cases, equipment covering more than one range could be used.  The 802.22 standard SHALL accommodate for operation in these frequency segments as well as the amalgamation of more than one of these segments.

9.3	Channelization

The MAC and PHY protocols and the resulting silicon implementation MUST permit the operation with channel spacing of integer multiples of 6, 7 and 8 MHz although the RF front-end filters and channel IF filters would need to be specific to the raster.  Also, the center frequencies may be different in various regions.  The CPE RF front-end and tuning will need to adapt accordingly.

Subdivision of these nominal channels may be used as long as the regulatory maximum power per device in a 6, 7 or 8 MHz is not exceeded.  The effective channel bandwidth will be dependent on the requirement for WRAN out-of-band emission in the adjacent channels and on the susceptibility of DTV receivers to adjacent channel interference from WRAN (see section 9.1.3).

In order to reduce the peak power transmitted from a CPE, the 802.22 should allow sharing of the upstream channel capacity on a FDM basis to allow simultaneous longer transmission frames to share the medium.

9.3.1	Adaptable to differing regulatory domains ( 6, 7, and 8 MHz TV raster friendly as appropriate)

The 802.22 standard SHALL support 6, 7 and 8 MHz channellization.  With the average spectrum efficiency indicated above (section 5.3), the WRAN systems should be able to provide 18 Mbit/s in 6 MHz TV channels, 21Mbit/s in 7 MHz channels and 24 Mbit/s in 8 MHz TV channels in the forward direction.

#### 9.3.1.1	NA

#### 9.3.1.2	SA

#### 9.3.1.3	EU

#### 9.3.1.4	Asia

#### 9.3.1.5	Africa

#### 9.3.1.6	Other?

# 10	Medium Access Control (MAC)

## 10.1 Network Operations

The 802.22 protocols shall provide mechanisms for authorization, registration, continued operation and deregistration of CPEs to 802.22 networks while avoiding interference to incumbent users.  These operations shall take place under the control of the base station, which shall be capable of handling multiple CPEs.

## 10.2	Support for Interference Mitigation/Coexistence

### 10.2.1	Sensing Measurements

802.22 Base Station SHALL control CPEs to conduct measurement activities and obtain measurement results.  There may be a need for scheduled quiet periods for sensing to take place.  Repetition rate and integration time SHALL be identified in order to meet the sensitivity down to the specified threshold in a timely manner.  Sensing should include capture of signal signature to identify the type of incumbent and other LE signals.

#### 10.2.1.1 Sensing Control

802.22 protocols MUST provide mechanisms for sensing control in the CPEs. The Base Station SHALL be able to instruct the CPEs when to sense, which channels to sense and the duration of the sensing activity.

Additionally, the base station could specify to the CPEs a threshold per incumbent type for comparison purposes so that sensing information report is optimized.

#### 10.2.1.2 Sensing Measurement Report

CPEs SHALL send to the Base Station sensing measurement reports in a form understandable by the Base Station.  In addition, CPEs SHALL be able to acquire and convey to the Base Station the incumbent or other LE RF signal profile.  CPE’s could also recognize RF signal profiles, and in such cases, these CPE’s SHALL be able to recognize the incumbent services present in the regulatory area of operation (see Section 11.1.4).  The IEEE 802.22 air interface SHALL provide identifiers for CPEs and Base stations in view of resolving interference problems (see Section 10.7).

### 10.2.2	PHY Operational Control

#### 10.2.2.1	Channel Management

#### 10.2.2.1.1	Channel Change

Once the CPE has received the channel change request from the base station, the 802.22 MAC SHALL order the 802.22 PHY to change the channel of operation.

#### 10.2.2.1.2	Suspend/Resume Channel Transmission

The 802.22 MAC SHALL be able to suspend the transmission in a specified channel. Transmission in this channel MAY be resumed upon indication from the Base Station.

#### 10.2.2.1.3	Termination and Reestablishment of Channel Operation

The 802.22 MAC SHALL be able to terminate the operation in a channel. Operation MAY be reestablished in another channel.

#### 10.2.2.2 Control of PHY Adaptability

The 802.22 MAC SHALL be able to control the PHY adaptability features such as TPC, modulation/FEC, channel bandwidth, and timing.

### 10.2.3	Base Station Management of CPEs

#### 10.2.3.1 Base station and CPE Relationship

The 802.22 system follows a master/slave relationship where the base station performs the role of the master    and the CPE’s are the slaves.  Hence, in an 802.22 system, the base station SHALL be responsible for managing CPEs in numerous aspects including modulation, coding, encryption, power level and bandwidth usage.

#### 10.2.3.1.2 Measurement Management

Measurement activities in an 802.22 system are also managed by the base station.  Thus, the base station shall be able to order CPEs to perform measurements and report the results back to the base station.

#### 10.2.3.1.3 Measurement analysis

Measurements analysis is an ongoing process conducted by the base station and which is based upon measurement results reported by CPEs. Once sufficient measurement information is available to the base station, the latter shall be able to analyze them and, if necessary, take appropriate steps to correct any misbehavior by changing its operating parameters such as frequencies of operation and power level. To augment this functionality, the base station may rely on other features such as databases (see Section 11.1.7), information from other 802.22 base stations, and service provider specific information.

### 10.2.4	Inter-Base Station Coordination

The IEEE 802.22 air interface protocols SHALL provide mechanisms to allow for coexistence, interference resistance, and sharing of radio resources amongst neighboring 802.22 Base Stations. CPEs SHALL be able to report interference received from another 802.22 base station, and base stations SHALL be able to take measures in order to resolve issue.

## 10.3	Class of Service and Quality of Service

This section describes the classes of service and quality of service for 802.22 systems. Terminology is borrowed from the Internet Engineering Task Force (IETF).

802.22 protocols MUST support classes of service (CoS) with various quality of service (QoS) guarantees to support the services that an 802.22 system MUST transport. Thus, 802.22 protocols MUST define interfaces and procedures that accommodate the needs of the services with respect to allocation and prioritization of resources. Additionally, 802.22 protocols MUST provide the means to ensure that QoS levels are satisfied under the constraints of license-exempt operation in presence of incumbent services. Table 1 provides a summary of the QoS requirements that the PHY and MAC SHALL provide. Note that parameters in the table are measured between the MAC input at the upper layer at the transmit station and the MAC output at the upper layer of the receiving station for information transmission. For example, delay does not include setup time, link acquisition, voice codecs, etc.  For evaluation purposes, proponents should indicate the Packet Error Rates (PER) requirements for each type of service shown in Table 1.

For QoS-based, connectionless services, the 802.22 protocols MUST support resource negotiation “on-demand”. For instance, the MAC protocol MAY allocate bursts of PDUs to services that require changes in resource allocation. Such allocation, for connectionless services, is thus performed in a semi-stateless manner.

A connection-oriented service may require “state” information to be maintained for the life of a connection. However, the 802.22 MAC layer interface MAY provide a connection-less service interface that requires a higher-layer “adaptation” to maintain the “state” of a connection and periodically allocate resources. For instance, the MAC may need to maintain “state” information about a QoS data flow only for the duration of an allocation.

Table 1: Services and QoS Requirements

| Service | Maximum Ratio | Maximum Latency Delay (One way) |
| --- | --- | --- |
| Full Quality Telephony (Vocoder MOS) 4.0) | BER10-6 | 20 ms |
| Standard Quality Telephony (Vocoder MOS) < 4.0) | BER10-4 | 40 ms |
| Time Critical Packet Services | BER 10-6 | 20 ms |
| Non- Time Critical Services: | BER 10-9 | not applicable |

10.3.1	Types and Classes of Service

The fundamental direction for the QoS model that will be exported to the WRAN endpoints will be IP based and conform to IETF DiffServ QoS model in conjunction with other IP based protocols. The DiffServ QoS model defines traffic for all services according to the following levels: Expedited Forwarding (EF), Assured Forwarding (AF), and Best Effort Service (BES).  For more information, please refer to Appendix F.

- Expedited forwarding (EF) EF requires a constant, periodic access to “bandwidth”. The “bandwidth” requirements vary over time, within a specified range, but delay and delay variance limits are specified. Examples that fall into this category are voice-over-IP (VoIP), videoconferencing, video on demand (VoD), and other “multimedia” applications.

- Assured Forwarding (AF): In AF the “bandwidth” varies within a specified range, but has loose delay and delay variance requirements. Applications, which are limited in their “bandwidth” usage, may fall into this category. In one example, corporate database transactions could be relegated to this category. 'Assured Forwarding' service allows the traffic to be divided into different classes. Using this service, an ISP can offer an “Olympic” service model, which provides three tiers of services: gold, silver and bronze with decreasing quality (i.e., the gold level of service receives a higher share of resources than silver during times of congestion). This service model would support, for example, the ability to provide preferential treatment to subscribers willing to pay a “premium” price for better service. Or it would support more granular priorities such as giving preference to VoIP traffic over other traffic (e.g., HTTP).

- Best Effort Service (BES). The “bandwidth” varies within a wide range, and is allowed to burst up to the maximum link “bandwidth” when EF and AF traffic are not using “bandwidth.” The “bandwidth” and delay requirements may or may not be specified.  Higher variations of delay may be tolerable since applications that fall into this category allow for priority traffic to preempt their “bandwidth” consumption. “Bandwidth” is delivered on a “best effort” basis. Current Internet service is an example of this type of operation.

### 10.3.2	Parameters

802.22 protocols SHALL define a set of parameters that preserve the intent of QoS parameters or IP-based services.

### 10.3.3	Service QoS Mappings

The classes of service and QoS parameters of services SHALL be translated into a common set of parameters defined by 802.22. A QoS-based IP network may employ the Resource reSerVation Protocol (RSVP) [27] to “signal” the allocation of resources along a routed IP path.  If 802.22 is to be a “link” in the IP network, an IWF MUST interface with 802.22 to negotiate resource allocation.

The basic mechanism available within 802.22 systems for supporting QoS requirements is to allocate bandwidth to various services. 802.22 protocols SHOULD include a mechanism that can support dynamically-variable-bandwidth channels and paths (such as those defined for IP environments).

## 10.4	Single channel versus Multichannel Operation

The simultaneous use of more than one TV channel (contiguous or not) MAY be possible as long as it is possible to free up any of these channels in case of interference to the incumbent services.  To this end, the 802.22 MAC protocols SHALL be able to terminate operation in any of the channels being used if the need arises.

## 10.5	OA&M Support

The air interface SHALL provide a mechanism to enable the provisioning and collection of metrics, so that the network operator can effectively control, monitor, and tune the performance of the 802.22 air-interface. Provisional parameters, performance metrics and other OA&M values shall be made available through a standards compliant MIB.

### 10.6	Base station and CPE’s address space

The 802.22 protocols shall support addresses that uniquely identify CPEs and base stations and which shall be used for, among other things, resolving interference. The 802.22 MAC shall regularly transmit the address of the transmitter device as to identify itself and hence allow for interference resolution with incumbents as well as other 802.22 systems.

# 11	Co-existence and interference mitigation

The context in which the WRAN systems will be operated is not usual for License-Exempt type services.  The TV broadcast bands in which they will operate are already used by TV broadcasting, wireless microphones operating according to FCC Part 74 and Public Safety.  [A more detailed description of this special interference context along with the various means identified to allow harmonious operation are contained in document “22-05-0003-00-0000_Suggested WRAN Functional Requirements.doc”.]  The PHY and MAC protocols of the WRAN standard MUST include the necessary means to operate in such environment without causing interference to these incumbent services.  Furthermore, these protocols will also need to include means to allow coexistence of WRAN systems and coexistence with other license-exempt systems in these bands for fair and efficient use of the spectrum.

## 11.1	Licensed Incumbent Sensing and Avoidance

### 11.1.1	Overview

Interference avoidance will be based on RF sensing at the base station and at the CPE’s and will rely on cognitive radio techniques and intelligence at the network level, centralized at the base station.  The interference potential would be assessed by mapping the results of the sensing of spectrum occupancy performed by the base station and the CPE’s against information on their physical location acquired through registration information or other geolocation mechanisms.  Data fusion from sufficiently large number of CPE’s will be needed to get a reliable spectrum occupancy figure.

Such sensing will include the monitoring of broadcast operation, wireless microphone operation, Public Safety as well as the operation of other LE operation in the area.  Sensing on the operating channels will imply a command from the base station to set the exact time for sensing to generate a quiet period at the CPE’s and at the base station.

What are the rules?  Within 802, existence of 802.22, more pressure for co-existence, more demand.  All players will have to play with the FCC rules.  Equipment will have to be certified.  We need to specified how to sense incumbent.

#### 11.1.1.1	TV broadcast: description and technical characteristics

Channel rasters of 6, 7, 8 MHz. In North America, 6 MHz and high transmit power up to MW.  Typical RF characteristics of receivers is documented in ATSC A-74. , To create international standard, the characteristics, channel raster (6, 7, 8 MHz) and extend of frequency bands tend to change with international regions.

References: ITU-R BT.470-6, ITU-R BT.655-6, ITU-R BT.417-4

Analogue (AM Video, FM Audio)

Vestigial Side Band NTSC M: US, Canada, Chile, Costa Rica, Cuba, El Salvador, Guatemala, Honduras, Japan, Mexico, Panama, Philippines, Puerto Rico, South Korea, Taiwan

Operating frequencies: 54 – 746 MHz (Japan 90 – 770 MHz)

Bandwidth: 6 MHz channel: 4.2 MHz video, 250kHz audio

Carriers: Colour 3.580 MHz above video carrier, Audio 4.5 MHz above video carrier

ERP: 100 W – 5 MW

Phase Alternating Line (PAL): Argentina, Australia, Belgium, Brazil, China, Denmark, Finland, Great Britain, India, Indonesia, Ireland, Italy, Norway, New Zealand, the Netherlands, Portugal, Spain, Sweden, Switzerland, Turkey.

PAL B

Operating frequencies: 48 – 798 MHz (New Zealand 44 – 798 MHz, Australia 45 – 806 MHz)

Bandwidth: 7 MHz channel: 5.0 MHz video, 250 kHz audio

Carriers: Colour 4.434 MHz above video carrier, Audio 5.5 MHz above video carrier

PAL G

Operating frequencies: 48 – 798 MHz (New Zealand 44 – 798 MHz)

Bandwidth: 8 MHz channel: 5.0 MHz video, 250 kHz audio

Carriers: Colour 4.434 MHz above video carrier, Audio 5.5 MHz above video carrier

PAL H

Operating frequencies: 47 – 798 MHz

Bandwidth: 8 MHz channel: 5.0 MHz video, 250 kHz audio

Carriers: Colour 4.434 MHz above video carrier, Audio 5.5 MHz above video carrier

PAL I

Operating frequencies: 174 – 798 MHz

Bandwidth: 8 MHz channel: 5.5 MHz video, 250 kHz audio

Carriers: Colour 4.434 MHz above video carrier, Audio 6.0 MHz above video carrier

PAL N

Operating frequencies: 54 – 806 MHz

Bandwidth: 6 MHz channel: 4.2 MHz video, 250 kHz audio

Carriers: Colour 3.582 MHz above video carrier, Audio 4.5 MHz above video carrier

PAL M

Operating frequencies: 54 – 806 MHz

Bandwidth: 6 MHz channel: 4.2 MHz video, 250 kHz audio

Carriers: Colour 3.576 MHz above video carrier, Audio 4.5 MHz above video carrier

Sequential Colour with Memory (SECAM): France, Egypt, Germany, Greece, Haiti, Iran, Iraq, North Korea, Poland, the former Soviet Union.

SECAM B

Operating frequencies: 47 – 798 MHz

Bandwidth: 7 MHz channel: 5.0 MHz video, 250 kHz audio

Carriers: Audio 5.5 MHz above video carrier

SECAM G, H

Operating frequencies: 47 – 798 MHz

Bandwidth: 8 MHz channel: 5.0 MHz video, 250 kHz audio

Carriers: Audio 5.5 MHz above video carrier

SECAM D, K, K1, L

Operating frequencies: 470 – 790 MHz

Bandwidth: 8 MHz channel: 6.0 MHz video, 250 kHz audio

Carriers: Audio 6.5 MHz above video carrier

Digital

Vestigial Side Band ATSC (North America)

Operating frequencies: 54 – 746 MHz

Bandwidth: 6 MHz channel

Pilot: 11.3 dB below the average data signal power

ERP: 100 W – 1 MW

Orthogonal Frequency Division Multiplexing DVB-T (Europe)

Bandwidth: 7 or 8 MHz channel

Operating frequencies: 48 – 798 MHz

TDS-OFDM (China)

Bandwidth: 7 or 8 MHz channel

Operating frequencies: 48 – 798 MHz

Orthogonal Frequency Division Multiplexing ISDB-T (Japan)

Bandwidth: 6 MHz channel

Operating frequencies: 54 – 746 MHz

Above information to be put in a table. (Winston)

Add discussion on service contours (Grade B), field strength, etc.

Distance between transmitters at planning stage

Receiver characteristics and typical installation (antenna gain, directivity, height)

Summary of ATSC A-74                       . (Winston)

#### 11.1.1.2	Wireless Microphone description and technical characteristics

Operated by broadcaster, news crews, in-studio and remote productions.  Typical power: 10 mW, typical range: 300 m.  There is also a large number of unlicensed microphone operations.  Wireless microphone operators will need a beacon for reliable operation (e.g., simple narrowband carrier-like).  There is however possibility for false beacon operation: CW would easily disallow WRAN operation in the nearby area. There is still a possibility of having channels set aside for microphones by the FCC.  Shure asked to reserve 6 TV channels for wireless microphones.

[Shure to provide more information: email sent to Shure.]

#### 11.1.1.3	Public Safety description and technical characteristics

This service is geographically based: base station would know whether to use the channel or not (channels 14-20 are off the table in the 14 main market, plus some more licenses outside of these market).  There is no need to sense by CPE’s, hence speeding up the process.  [Steve Kuffner to gather text on Part 90, succinct table of technical characteristics, ch 14-20, sensing may be quite difficult, therefore geographic use should be the way to share, developed at Motorola.]

#### 11.1.2	D/U Ratios

The Desired to Undesired ratios (D/U) will depends on the modulation proposed for the WRAN standard and the incumbent services that need to be considered.  As an initial step to determine the coexistence feasibility, it was assumed that the impact of the WRAN transmission on TV broadcasting will be similar to a DTV signal.  The same D/U’s as for

DTV into DTV and DTV into NTSC were used for the co-channel case and adjacent channels cases.  More precise values will be found from actual measurements once the WRAN air interface is better defined.

### 11.1.3 Sensing Thresholds

### The base station and the CPE’s will sense licensed transmissions using an omni-directional antenna with a gain of 0 dBi or greater (where all losses between the antenna and the input to the receiver are included) in any azimuthal direction. The base station will need to change the frequency of operation of the network if licensed signals are detected above the following thresholds, referred to the receiver input:

(1)	DTV threshold: -116 dBm (total DTV power in the 6 MHz channel) (using spectrum analysis techniques to sense the pilot carrier of the DTV signal which is at –11.3 dB in a 10 kHz effective bandwidth relative to the total DTV power)

(2)	Analog TV threshold: -94 dBm (measured at peak of sync of the picture carrier)

(3)	Wireless microphone threshold: -107 dBm (measured in 200 kHz bandwidth).  Lower threshold and more reliable sensing levels would be achievable if narrowband beacons are used for the wireless microphone operation.  A ‘dead beacon’ could be used per occupied TV channel or a single beacon modulated by a low data rate (e.g., FSK) indicating the TV channels used by wireless microphones in the immediate area could also be used.

### 11.1.4 Response Time:

The response time is the time during which broadcast operation can withstand interference before the WRAN system vacates the channel.  In the case of interference to broadcasting, this response time is not likely to be critical . (e.g., a new broadcast station comes on, likely during the night.  In the case of interference to wireless microphone operation, the response time will be dictated by the time the beacon can be turned on before the microphones begin to be used for the event.  In practice, this could be in the range of a few minutes.  If a beacon is not used with wireless microphone operation, the base station and the CPEs will have to sense and detect the microphone operation within few tens of millisecond because of their intermittent nature (see section 11.1.1.2).

Legitimate wireless microphone operators that operate according to Part 74 should use beacons in order to distinguish themselves from those that are ‘unlicensed’ so that the latter do not pre-empt the use of the spectrum for ‘License-Exempt’ operation.]

.

[DFS at 5 GHz: channel availability check time= 60s, channel move time= 10s, channel closing time= 200m.  The numbers for WRAN will probably need to be different.  Need analysis on the impact on LE client, on transmission continuity, QoS].

### 11.1.5	Distributed Sensing

802.22 systems shall employ distributed sensing where the base station and CPEs have the capability to sense the RF channel occupancy.  Information will be sent and collated at the base station where mapping of the interference situation will be made based on the sensing information from the CPE’s and according to their respective geographic coordinates.  Solutions will be deduced at the base station and measures to avoid interference will be signalled to the CPE’s such as channel change or power reduction.  The MAC and PHY protocols SHALL allow the transmission of the necessary control packets in both directions to permit such distributed sensing and the CPEs shall react in a timely manner to the commands from the base station.

The incumbent services will be better protected by such distributed RF sensing since it would avoid the ‘hidden node’ problem.

11.1.6	Channel utilization databases

Although broadcasters have a reliable database and changes occur at a very slow pace, a 100% correct database cannot be assumed.  Such database should be used as a starting point by the base station to identify the TV channels that can be used in the area.  The database should be kept up-to-date on a regional basis and it should be updated within one (?) hour.  Furthermore, there should be a way to enter disallowed channels at the base station on a regular basis. [Taking the 1.91-1.93 GHz sharing as an example, the cost of keeping such a database could be supported by the LE operators.]

The base station and CPEs should maintain a "list" of backup channel(s) in case an incumbent is detected and a channel has to be quickly vacated.  The exact scheme can be left to the proposals, as different approaches could be suggested to tackle this problem.  (Informative: appendix)

11.1.5	Minimum separation distances between WRAN base stations and DTV operation

Base stations will need to be located outside the Grade B contour of the DTV stations that use the same channel and first adjacent channels.  Minimum distances are indicated in Table x.  In the case of the second adjacent channels and beyond, the base station will need to be located at specified minimum distances from the closest DTV receiver as indicated in Table x.  These distances are based on the assumption that the D/U ratios between WRAN transmission and DTV reception will be the same as DTV into DTV.  Some refinements to these distances will possibly be needed once actual tests are conducted with the modulation selected for WRAN transmission.

| ATSC A-74 DTV RX Performance Guidelines | Free Space (m) | ITU-R P.1546 F(10,10) (m) |
| --- | --- | --- |
| N (continuous) Para. 4.4.1 | 1,365,406 | 42,770 |
| N (impulsive) Para. 4.4.4 | 171,894 | 18,800 |
| N+/-1 Para. 4.4.2 | 868 | 868 |
| N+/-2 Para. 4.4.3.1 | 246 | 196 |
| N+/-3 Para. 4.4.3.1 | 155 | 123 |
| N+/-4 Para. 4.4.3.1 | 78 | 62 |
| N+/-5 Para. 4.4.3.1 | 62 | 49 |
| N+/-6 to N+/-13 Para. 4.4.3.1 | 123 | 98 |
| N+/-14 and 15 Para. 4.4.3.1 | 55 | 44 |
| RF front-end overload Para. 4.2 | 39 | 31 |

Table x:  Minimum distance between the WRAN base station and the DTV Grade B contour in the case of co- and adjacent channel operation and distance to the closest DTV receiver for larger channel separation

### 11.1.6 Maximum power for WRAN CPE’s to avoid interference to DTV operation

The worst-case interference situation between a transmission from a CPE and a DTV reception is assumed to occur when the CPE antenna, which is mounted at 10 m above the ground, is in close proximity to a DTV receive antenna.  A minimum distance of 10 m is assumed to be realistic.  In the case of co-channel and first adjacent channels operation, both antennas are assumed to be looking away from each other since the base station has to be located at a certain distance outside the Grade B contour of the DTV station as indicated in the previous section.  The back-lobe rejection of both antennas can therefore be relied upon.

For the second adjacent channels and beyond, the antennas can be aimed toward any angle and only polarization discrimination can provide isolation since the transmit antenna will be vertically polarized whereas the DTV receive antenna is horizontally polarized.  The polarization discrimination is assumed to be equal to the DTV antenna backlobe rejection.  It is very unlikely that metallic objects at 45 deg. which could depolarize the signal will be in proximity of the short path that will be needed between the CPE transmit antenna and the DTV receive antenna.  This likelihood will increase when the separation distance becomes larger but the larger free-space loss will compensate for it.  No reduction in polarization discrimination is therefore applied..

Table Y indicates the CPE transmit power level that will be allowed before interference is created at the nearby DTV receiver.  These levels are based on the assumption that the D/U ratios between WRAN transmission and DTV reception will be the same as DTV into DTV.  Some refinements to these levels may be needed once actual tests are conducted with the modulation selected for WRAN transmission.  Powers beyond 6 dBW will not be allowed according to the FCC NPRM 04-186.

When the DTV receiver is equipped with lower gain antenna, lower antenna height or even indoor-mounted antenna, the required DTV field strength will need to be higher in the area in order to provide for adequate reception to compensate for the lower antenna gain and indoor/outdoor attenuation, therefore easing the interference situation.

Table y:  Maximum permitted transmit power for the CPE’s assuming a minimum separation of 10 m between the CPE antenna and the closest DTV receiving antenna

### 11.1.7	Out-of-band emission mask for WRAN systems

Once the maximum level of transmit power for the CPE’s is established for specific channel separation between the one used by the WRAN system and the TV channel that the DTV receivers are to tune to, and that the level of DTV receiver desensitization is established at the worst reception case, i.e., Grade B contour field strength level, the levels of signal rejection at the output of the CPE can be established.  These are given in Table z for 1 dB DTV receiver desensitization at the Grade B contour.

WRAN CPE transmissions will need to comply with this RF mask.  Some refinements to this mask will possibly be needed once actual tests are conducted with the modulation selected for WRAN transmission.  It is worth noting that this RF mask has taken advantage of the fact that RF sensing and dynamic frequency selection will exist in the operation of the WRAN systems to avoid interference to the incumbent services.  It is based on a good balance between desensitization of the DTV receiver by WRAN out-of-band emission, and on the other hand on the DTV RF front-end saturation and adjacent channel D/U’s.

For a detailed treatment of the WRAN RF mask development, please consult doc. “22. 04.0002.04 .0000 WRAN Reference Model.xls”.

Table z:  Out-of-band emission levels for various adjacent channel operation

## 11.2	WRAN systems coexistence/sharing

Coexistence among WRAN systems.

More than one WRAN base stations in the same geographic area.  Will allow multiple 802.22 systems with service area overlap.  Sensing techniques should be operable in presence of other WRAN systems operating co-channel in the same channel.

Any license-exempt usage in the TV broadcast bands would need to use a beacon or a signature in its transmission signal that would include a transmitter identification.  This should be in the rule.  The identification of the transmitter would make resolving interference situations tractable.  802.22 will be the first group to define the sensing rules and the FCC need to monitor the work to develop the corresponding rules.

Coordination among 802.22 systems should be straightforward since all these systems will operate according to the same standard.. Specific address space for each WRAN operation would be needed. This is required in order to ensure that interference resolution is always possible:

- Should we use 48-bit standard IEEE addresses?

- Addresses could be either fixed or dynamic

### 11.2.1	Overview

Extending the beacon concept to help cooperation among LE systems. Encourage the FCC to look into this capability.  FCC will insist to know how it will be tested.  Every 30 sec.  ANSI C63.17 1910 to 1930. Unlicensed PCS shared.    Need Transmitter signature.

### 11.2.2	Sensing Thresholds and D/U Ratios

Other devices that could use beacons similar to those used for the wireless microphones would avoid interference but it would give them precedence over WRAN.  Need for a transmission signature to distinguish among DTV, wireless microphones, WRAN transmissions and other LE transmissions.

### 11.2.3	Response Time

802.19 is an outflow from 802.11 whereas 802.16h deals with this in the 802.16 context. Response time still to be defined.

### 11.2.4	Distributed Sensing

Since distributed sensing will exist for coexistence with incumbents, the same structure will be available for LE coexistence.

### 11.2.5	Channel utilization databases

## Coexistence/sharing with other license-exempt systems

We are aware of the problem with other LE systems.  Discussing the evolutional of this section will depend on the FCC decisions.

If other LE systems don’t have RF signal profiles => problem.  Need to have a common interface for sensing to later cooperate.

Any license-exempt usage in the TV broadcast bands would need to use a beacon or a signature in its transmission signal that would include a transmitter identification.  This should be in the rule.  The identification of the transmitter would make resolving interference situations tractable.

Coordination with other LE systems will need to include conditions on the operating rules of these systems to allow resolution of interference.  Precedents exist for the 1.9 GHz LE PCS band where technical means of avoiding interference were defined in the ANSI standards.  These other LE systems should also have RF sensing and means to avoid interference.  There should be an industry group to work together to resolve the issue.

## 11.4	RF Sensing

  - measurements that must be done are part of PHY.  PHY has to respond to commands from MAC to do sensing and generate messages to the MAC on results (see section 10.2.1).

  - cognitive aspects interwoven between PHY and MAC

  - sensing needs requirement in PHY and MAC.

  - bound problem and define what the standard has to specify but not specify explicitly.

  - sensing on occupied channel and other channels.  sensing on channel you're planning to occupy.  primary channel sensing, in-service monitoring, during quiet periods figure out if channel is occupied;  how frequently (dynamic nature of the usage), how long to sense? (sensitivity level)  besides initial scanning when CPE turns on, sensing is controlled by base station.  eg.  scan next available and others less frequently.

  - System shall sense at the specified threshold levels (section 11.1.4) How it will be done and give indication on the duration of the quiet period.

  - legitimate wireless microphones, use beacon? don't want to have to protect illegal mikes.

  - define rate at which sensing would be done, integration time, once we know thresholds.  spectrum signature, know what DTV looks like, know what mikes look like, NTSC looks like, DVB-T has null in centre of channel, narrow null would define observation of this.  need to include foreign signatures in this portion of the document.

  - [System section: Need to indicate that RF levels reported to the base station are in absolute  levels (e.g. field strength, dBm, etc.).  The general frequency range (band 1, 2, 3, 4 and 5) for CPE should be the same as the range over which the CPE can operate … plus the adjacent channels

## 11.5	Dynamic Frequency Selection

When interference is to be avoided, the systems shall change (vacate, etc. see section 10.2.2.1) to another channel that was found to be available. … time frame: detect, close, move, re-establish connection.

12	Network Management

As outlined in IEEE Std 802-1990 [11], the LLC Sublayer, MAC Sublayer and Physical Layer standards also include a management component that specifies managed objects and aspects of the protocol machine that provide the management view of managed resources. The aspects of management considered are (FCAPS):

- Fault management

- Configuration management

- Accounting management

- Performance management

- Security (see also section 13)

The 802 standards define a framework for LAN/MAN management in ISO/IEC 15802-2:1995(E) [28]. The framework contains guidelines for managed objects, management protocol, and the relationship to ITU management protocols. 802.22 protocols SHALL comply with the above-mentioned standards and guidelines.

## 12.1	Support for Service Level Agreements

The 802.22 protocols MUST permit operators to enforce service level agreements (SLAs) with subscribers by restricting access to the air link, discarding data, dynamically controlling bandwidth available to a user or other appropriate means [29]. The 802.22 protocols MUST also permit subscribers to monitor performance service levels of the 802.22 services being provided at the delivery point.

## 12.2	Support for Admission Control

The 802.22 standard SHALL a flow by flow admission control to support  QoS.

## 12.3	Support for Accounting and Auditing

The 802.22 system management framework, architecture, protocols and managed objects MUST allow for operators to effectively administer accounting and auditing. An operator must be able to account for resource utilization and various service features for each subscriber service separately.  [The WRAN system protocols should allow control and management of the services on a subscriber basis as well as services provided to special CPE’s which bridge to a number of subscribers (i.e., multi-apartment building served by a ‘first class’ CPE.)]  Also recall from section 8.12 that a single CPE can interface to multiple subscribers that an operator could bill separately.

## 12.4	Base Station to Base Station Communications/Coordination

### 12.4.1	In Same Network

Should it be at layers 3 and above or it would need to be at layers 1 and 2 and thus need to be defined in this standard?

### 12.4.2	Between Co-located/Adjacent/Overlapping Networks

## 12.5	Timing, sync, etc.

## 12.6	Malfunctioning CPE or Base Station

The operator MUST have means to shut down and reactivate a subscriber station if necessary, remote from the CPE, in the face of a malfunction. The operator also SHOULD have the means to securely shut down and reactivate a base station remotely. When such capabilities are available, the 802.22 protocols SHALL support a secure function. The 802.22 protocols SHOULD support functions that automatically shut down transmission from a CPE or base station in case of malfunction (e.g., power limits exceeded).

The WRAN equipment should be tamper-proof to avoid any unauthorized modification to the hardware, firmware and/or functionalities, e.g., cognitive functionality, RF sensing, DFS, TPC, tuning).

# 13	Security

The 802.22 system SHALL enforce security procedures described in this section.

The security system chosen by 802.22 SHALL be added to the protocol stack and reference points to include security protocols, and “database” servers for authentication, authorization, key management, service suspend/resume, relocation, anti-cloning, etc. [30, 31].

## 13.1	Authentication for network access

There are two types of authentication for an 802.22 system. In the first type, a subscriber station MUST authenticate itself with the network every time it registers with the network. This authentication MUST prevent unauthorized subscriber station from entering the network or an unauthorized base station from emulating an authorized base station. This type of authentication MUST be supported by the 802.22 MAC layer.

The second type of authentication is between the subscriber and the WRAN system. This may or may not be the responsibility of the 802.22 protocols. It MAY be handled by higher layer protocols.

An additional level of authentication may exist between the other two. This additional layer is the authentication of the subscriber with the subscriber station. This is beyond the scope of the 802.22 protocols.

The authentication mechanisms MUST be secure so that an “enemy” subscriber station is not able to gain access to an 802.22 system, or to the core network beyond. Passwords and secrets MUST NOT be passed “in the clear” through the air interface.

## 13.2	Authorization (Access control)

Authorization is a security process that determines what services an authenticated subscriber is permitted to invoke. Each subscriber has a set of credentials that describe what the subscriber is “allowed” to do. The 802.22 standard SHALL identify a standard set of credentials and allow for vendors to extend the defined credentials with non-standard credentials. Some possible credentials are:

- Permission to access the 802.22 system

- Permission to access certain services (IP, Remote Bridging, Digital Audio/Video, etc.)

- Permission to request specific service features, and/or QoS parameters, based on the SLA (resources, delay, etc.).

For each service offered, the 802.22 protocols MUST be capable of securely supporting authorization requests and responses.

## 13.3	Privacy

Privacy is a security concept that protects transmitted data from being intercepted and understood by third parties (e.g., an “enemy” subscriber station, base station or passively  “listening” radio). Wire-equivalent privacy (WEP) [5] and shared private key [5] privacy have been suggested as minimum required privacy levels for 802.22 systems.

802.22 standard SHOULD allow a suitable cryptographic algorithm to be employed that is internationally applicable. Facilities SHOULD also be defined in the protocol for the use of alternate cryptographic algorithms that can be used in certain localities and that can replace algorithms as they are obsoleted or “legalized” for international use.

## 13.4	 Message integrity

## 13.5	Data (payload) encryption

## 13.6	Protection of network control information

## 13.7	Protection against Denial of Service and other attacks

## 13.8	Security algorithm(s)

## 14	802 Conformance

802.22 SHALLconform to the 802 architecture. The following  particulars with the 802 model (see IEEE Standards for Local and Metropolitan Area Networks: Overview and Architecture (IEEE Std 802-1990) [11]) shall apply:

- The 802.22 MAC supports 802 “universal” 48 bit addresses. (? 64 bit addresses? Carl:Tony Jeffrey, Goff Thompson and Floyd Backes to review)

- An 802.22 system supports MAC multicast in the downstream direction.

- The 802.22 protocols support 802.1 bridging services and protocols, including support of the 802.1q virtual LAN tag and 802.1D priority ID [32-34].

- The 802.22 protocols support encapsulation of 802.2 (LLC) [10] by the MAC protocol.

- Conform to the 802 conventions and structures for “interface primitives:” logical structures that are passed between protocol layers to invoke processes and transact data.

- Address the 802 system management guidelines (see section 8) [35].

- Provide a MAC service interface that complies to 802 conventions [12].

# 15	References

[1]	IEEE Standards Association. Project Authorization Request. Telecommunications and Information Exchange Between Systems - LAN/MAN Specific Requirements - Air Interface for Fixed Wireless Regional Area Network Systems in Licensed Bands from 2 to 11 GHz. March 30, 2000. http://ieee802.org/16/sub11/par/index.html.

[2]	IEEE LAN/MAN Standards Committee (IEEE 802 LMSC) Sponsor Executive Committee. 802.22 (Wireless Regional Area Network Air Interface Standard In Licensed Bands from 2 to 11 GHz): Meting the Five Criteria. March 9, 2000. http://ieee802.org/16/sub11/par/fivec.html.

[3]	Recommendation ITU-R F.1399. Vocabulary of terms for wireless access.

[5]	Marianna Goldhammer, 802.22sc-99/16 MAC Services.

[6]	S. Blake et al, "An Architecture for Differentiated Services", RFC 2475, December, 1998.

[7]	S. Blake et al, "A Framework for Differentiated Services", Internet Draft, October, 1998.

[8]	R. Braden et al., "Integrated Services in the Internet Architecture: An Overview", RFC 1633, June 1994.

[9] George Fishel, 802.22sc-99/22 Interface to MAC and LLC Protocols.

[10]	ISO/IEC 8802-2:1998. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 2: Logical Link Control.

[11]	IEEE Std 802-1990 "IEEE Standards for Local and Metropolitan Area Networks: Overview and Architecture. IEEE 1990.

[12]	ISO/IEC 10039: 1991. Information technology -- Open Systems Interconnection -- Local area networks -MAC service definition.

[13]	ISO 7498-1:1984. Information technology -- Open Systems Interconnection -- Basic Reference Model.

[14]	James Mollenauer, 802.16sc-99/5 Functional Requirements for Wireless Regional Area Network Networks.

[15]	Imed Frigui, 802.16sc-99/23 Services and Performance requirements for Broadband Fixed Wireless Access.

[16] Robert Duhamel, 802.16sc-99/17 LMDS Cell Sizing and Availability.

[17]	Recommendation ITU-T G.826: Error performance parameters and objectives for international, constant bit rate digital paths at or above the primary rate (2/99).

[18]	Recommendation ITU-R F.1491. Error performance objectives for real digital radio links used in the national portion of a 27 500 km hypothetical reference path at or above the primary rate.

[19]	C.W. Lundgren and W.D. Rummler, "Digital radio outage due to selective fading observation vs. prediction from laboratory simulation," Bell System Technical Journal, pp.1073-1100, May-June 1979.

[20]	M. Emshwiller, "Characterization on the performance of PSK digital radio transmission in the presence of multipath fading," ICC'78 Conference Record, Toronto, Ontario, CANADA, Paper 47.3.

[21]	Microwave digital radio systems criteria, Bellcore Technical Reference TR-TSY-000752, October 1989.

[22]	W.D. Rummler, R.P. Coutts, and M. Linger, "Multipath fading channel models for microwave digital radio," IEEE Communications Magazine, November 1986, pp.30-42.

[23]	R. K. Crane, "Prediction of Attenuation by Rain" IEEE, 1980.

[24]	Recommendation ITU-R F.1189-1. Error performance objectives for constant bit rate digital paths at or above the primary rate carried by digital radio-relay systems which may form part or all of the national portion of a 27500 km hypothetical reference path. (1995-1997).

[25]	CCIR Recommendation 749, Radio-Frequency channels arrangements for radio-relay systems operating in the 36.0 to 40.5 GHz band. (1992).

[26]	John Liebetreu, 802.22sc-99/13 Dispersive Fade Margin: A Physical Layer Performance Metric.

[27]	RFC-2205 Resource ReSerVation Protocol (RSVP) -- Version 1 Functional Specification. R. Braden, Ed., L. Zhang, S. Berson, S. Herzog, S. Jamin. September 1997. Status: PROPOSED STANDARD.

[28]	ISO/IEC 15802-2:1995. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 2: LAN/MAN management.

[29]	Jim Mollenauer, 802.22sc-99/7 Functional Requirements for the 802.22 Standard.

[30]	IEEE 802.10-1998. IEEE Standards for Local and Metropolitan Area Networks: Standard for Interoperable LAN/MAN Security (SILS).

[31]	IEEE 802.10c-1998. IEEE Standards for Local and Metropolitan Area Networks: Supplement to Standard for Interoperable LAN/MAN Security (SILS) -- Key Management (Clause 3).

[32]	ISO 15802-3:1998. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 3: Media Access Control (MAC) bridging.

[33]	ISO/IEC 15802-5:1998. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 5: Remote Media Access Control (MAC) bridging.

[34]	IEEE 802.1Q-1998. IEEE Standards for Local and Metropolitan Area Networks: Virtual Bridged Local Area Networks.

[35]	IEEE 802.1F-1993. IEEE Standards for Local and Metropolitan Area Networks: Common Definitions and Procedures for IEEE 802 Management Information.

# Appendix A
Requirements Summary (Informative)

This section contains tabular summaries or requirements found in the text of this document. Requirements are separated into three categories: required, recommended and optional.

Each requirement is numbered for easy reference. Future revisions of this document will keep the requirement reference numbers intact such that the number for a requirement will not change from revision to revision.

To better discern the meaning and intent of a requirement, please refer to the text.

## A.1	Mandatory

It is mandatory that the 802.22 standard support or specify the items in Table 2.

Table 2: Mandatory Requirements

| # | Section | Requirement |
| --- | --- | --- |
| M1 M2 | 1 1.1 | The forthcoming air interface standard MUST comply with the system requirements. The 802.22 air interface interoperability standard SHALL be part of a family of standards for local, metropolitan and wide area networks. |
| M3 | 2 | 802.22 systems SHALL be deployable in multiple-cell frequency reuse systems and single cell (super cell) frequency reuse systems. |
| M4 | 2.3 | Since all data traffic in a single cell of an 802.22 network MUST go through the base station, that station SHALL serve as a radio resource supervisor. |
| M5 | 2 | The base station radio SHALL be P-MP, radiating its downstream signal with a shaped sector or adaptive array (spatial reuse) antenna achieving broad azimuthal beam width to “cover” a prospective number of subscribers. |
| M6 M7 | 2.1 2.2 | The standard SHALL specify MAC layer protocols and PHY transmission techniques suitable for providing access between one or more subscriber stations and base stations to support UNI and CNI requirements. The 802.22 protocols SHALL support the optional deployment of repeater functions. |
| M8 | 2.2 | The repeater function SHALL NOT affect the MAC protocol other than the delay which the repeater might introduce into the system. |
| M9 M10 | 2.3 3 | In the upstream direction, 802.22 protocols MUST provide the means to multiplex traffic from multiple subscriber stations, resolve contention, and allocate capacity. The MAC and PHY protocols will not have explicit support for each and every service, due to the fact that generic data streams SHALL be used for transport. |
| M11 | 3 | The MAC and PHY protocols SHALL provide for QoS service specific support, resulting in appropriate BER for data services, limited delay for real |
| M12 | 3.1 | time services, etc. 802.22 systems SHALL support voice communications for subscribers in a way that eases the migration of legacy voice communications equipment and public switched telephone network (PSTN) access technologies to 802.22 systems. |
| M13 M14 | 3.1 3.2 | The 802.22 voice access transport SHALL be packet based (as opposed to circuit-switched based). The 802.22 system MUST directly transport variable-length IP datagrams efficiently. |
| M15 | 3.2 | Both IP versions 4 and 6 MUST be supported. |
| M16 M17 | 3.2 3.4 | The 802.22 IP service MUST provide support for real-time and non-real time service capabilities. These services SHALL NOT place any additional requirements on 802.22 systems (MAC and PHY protocols) not already covered in the above sections. |
| M18 | 4 | The IEEE 802.22 MAC and PHY protocol stacks SHALL be the same for all the supported services. |
| M19 M20 | 4 4 | Further details, and finalization of the protocol reference model, SHALL be worked out by the 802.22 MAC and PHY task groups while developing the air interface interoperability standard. Since key layers above the MAC require service guarantees, the MAC protocol MUST define interfaces and procedures to provide guaranteed service to the upper layers. |
| M21 M22 | 4 5.2 | Since customer units will contend for capacity to/from one or more base stations, the MAC protocol MUST efficiently resolve contention and resource allocation. 802.22 protocols SHALL be optimized to support the peak data rate in either or both directions to a subscriber station within the specified distance from the base station. |
| M23 M24 | 5.4 5.6 | The PHY and MAC protocols SHALL provide for multirate support. The 802.22 specifications SHALL NOT preclude the ability of the radio link to be engineered for different link availabilities, based on the preference of the system operator. |
| M25 M26 M27 | 5.6 5.6 5.7 | 802.22 MAC and PHY protocols MUST accommodate these conditions, perhaps consuming more radio bandwidth and/or requiring smaller radio propagation distance (radius) to meet the availability requirements. Since statistical atmospheric and path conditions vary widely in geography, the 802.22 protocols MUST be flexible in consumed radio bandwidth (spectral efficiency), cell radius, and transmit power. The error rate, after application of the appropriate error correction mechanism (e.g., FEC), delivered by the PHY layer to the MAC layer SHALL meet IEEE 802 functional requirements with the following exception: the radio link bit error ratio (BER) SHALL be 10E-6 (in accordance with ITU FWA |

| M28 | 5.9 | recommendations or better. The standard SHALL support careful planning to ensure that subscribers’ |
| --- | --- | --- |
| M29 | 5.9 | quality of service guarantees and minimum error rates are met. The delivered base station capacity can vary depending on attenuation due to |
| M30 | 5.9 | atmospheric conditions, LOS blockage, transmit power, etc., and SHALL be calculated as the aggregate capacity of all sectors supported by a base station. The MAC and PHY protocols MUST accommodate channel capacity issues |
|  |  | and changes in channel capacity to meet contracted service levels with customers. |
| M31 | 5.9 | As subscribers are added to 802.22 systems, the protocols MUST |
|  |  | accommodate them in an automated fashion. |
| M32 | 6.1 | This standard SHALL permit two duplex modes of operation: Frequency |
| M33 | 6.1 | Division Duplex (FDD) and Time Division Duplex (TDD). The PHY and MAC protocols MUST provide for duplex (i.e. bi-directional) |
| M34 | 6.1 | operation, while preserving the QoS, BER and spectral efficiency requirements for data and voice traffic. The MAC and PHY protocols MUST provide means to resolve applicable |
|  |  | collocation and interference problems. |
| M35 | 6.2 | The MAC and PHY protocols MUST permit the operation with channel |
| M36 | 7 | spacing of 1.75, 3.5 and 7MHz when using ETSI masks and 1.5 to 25MHz when using other masks. Table 1 provides a summary of the QoS requirements that the PHY and MAC |
|  |  | SHALL provide. |
| M37 | 7 | 802.22 protocols MUST support classes of service (CoS) with various |
| M38 | 7 | quality of service (QoS) guarantees to support the services that an 802.22 system MUST transport. Thus, 802.22 protocol standard MUST define interfaces and procedures that |
|  |  | accommodate the needs of the services with respect to allocation and prioritization of resources. |
| M39 | 7 | 802.22 protocols MUST provide the means to enforce QoS contracts and |
| M40 | 7 | Service Level Agreements. For QoS-based, connectionless services, the 802.22 protocols MUST support resource negotiation “on-demand” |
| M41 | 7 | If 802.22 is to be a “link” in the IP network, an IWF MUST interface with |
| M42 | 7.2 | 802.22 to negotiate resource allocation. 802.22 protocols SHALL define a set of parameters that preserve the intent |
| M43 | 7.3 | of QoS parameters for IP-based services. The classes of service and QoS parameters of services SHALL be translated |
|  |  | into a common set of parameters defined by 802.22. |
| M44 | 8.1 | The 802.22 protocols MUST permit operators to enforce service level |
|  |  | agreements (SLAs) with subscribers by restricting access to the air link, discarding data, dynamically controlling bandwidth available to a user or other |

| M45 | 8.1 | appropriate means. The 802.22 protocols MUST also permit subscribers to monitor performance |
| --- | --- | --- |
| M46 | 8.2 | service levels of the 802.22 services being provided at the delivery point. When such capabilities are available, the 802.22 protocols SHALL support a |
| M47 | 8.2 | secure function. The operator MUST have means to shut down and reactivate a subscriber |
|  |  | station if necessary, remote from the subscriber station, in the face of a malfunction.8.2 |
| M48 | 8.3 | The 802.22 system management framework, architecture, protocols and |
| M49 | 8.3 | managed objects MUST allow for operators to effectively administer accounting and auditing. An operator MUST be able to account for resource utilization and various |
|  |  | service features for each subscriber service separately. |
| M50 | 9 | The 802.22 system SHALL enforce security procedures described in this |
| M51 | 9 | section. The security system chosen by 802.22 SHALL be added to the protocol stack |
| M52 | 9.1 | and reference points to include security protocols, and “database” servers for authentication, authorization, key management, service suspend/resume, relocation, anti-cloning, etc. There are two types of authentication for an 802.22 system. In the first type, |
|  |  | a subscriber station MUST authenticate itself with the network every time it registers with the network. This authentication MUST prevent unauthorized subscriber station from entering the network or an unauthorized base station from emulating an authorized base station. This type of authentication MUST be supported by the 802.22 MAC layer. |
| M53 | 9.1 | The authentication mechanisms MUST be secure so that an “enemy” |
| M54 | 9.1 | subscriber station is not able to gain access to an 802.22 system, or to the core network beyond. Passwords and secrets MUST NOT be passed “in the clear” through the air |
| M55 | 9.2 | interface. The 802.22 standard SHALL identify a standard set of credentials and allow |
| M56 | 9.2 | for vendors to extend the defined credentials with non-standard credentials. For each service offered, the 802.22 protocols MUST be capable of securely |
|  |  | supporting authorization requests and responses. |

## A.2	Recommended (R)

- It is recommended that the 802.22 standard support or specify the items in Table 3 “Recommended” means that there may exist valid reasons in particular circumstances to ignore an item, but the full implications should be understood and the case carefully weighed before choosing a different course.

Table 3: Recommended Requirements

| # | Section | Requirement |
| --- | --- | --- |
| R1 | 1.1 | Other goals of this document are to formulate reference models and terminology for both network topology and protocol stacks that help the 802.22 working group to discuss and develop the MAC and PHY protocols. As far as possible, these SHOULD be common across 802.22 systems. |
| R2 | 2.2 | The repeater function SHOULD NOT affect the end-to-end operation of 802.22 protocols between BS and SS. |
| R3 R4 | 3.2 3.2 | For efficient transport of IPv6, TCP/IP header compression over the air interface SHOULD be supported. It SHOULD be possible to support the emerging IP Quality of Service (QoS) efforts: Differentiated Services and Integrated Services. |
| R5 | 3.3 | The 802.22 protocols SHOULD support bridged LAN service capabilities, whether directly or indirectly, including always on, ad hoc and on-demand communication in either or both directions. |
| R6 R7 | 4 5.1 | Note that the function of the MAC protocols SHOULD include error correction by retransmission, or Automatic Repeat Request (ARQ), whereas, in the 802 model, those functions if necessary, are provided by the LLC layer. The 802.22 protocols SHOULD allow for different “scales” of capacity and performance for 802.22 system instances. |
| R8 | 5.2 | The 802.22 MAC protocol SHOULD allow the peak data rate to scale beyond 10 Mbps. |
| R9 R10 | 5.3 5.5 | The PHY and MAC protocol SHOULD provide for far CPEs’ propagation delay compensation. 802.22 protocols SHOULD allow for flexibility between delivered upstream and downstream capacity and CoS/QoS. |
| R11 R12 | 5.6 5.6 | An 802.22 system SHOULD be available to transport all services at better than their required maximum error rates (see section 5.7) from about 99.9 to 99.99% of the time, assuming that the system and radios receive adequate mains power 100% of the time and not counting equipment availability. 802.22 MAC and PHY protocols SHOULD specify functions and procedures to adjust transmitter power, modulation, or other parameters to accommodate rapid changes in channel characteristics. |
| R13 | 5.9 | Given the propagation characteristics in a given frequency band and geographic area, and the development of a link budget, the following parameters of an 802.22 system SHOULD be addressed by the MAC and PHY protocols: ~ Radio range (up to 50 Km) ~ Upstream/downstream channels’ data rates ~ Allocation of prospective subscriber data rate to channels. Note: the MAC and PHY standard MAY allow subscribers to hop between channels ~ Types of modulation |

| R14 | 6.2 | The typical value [channel spacing] for performance analysis SHOULD be 3.5MHz for the ETSI mask and 3MHz for the MDS mask. |
| --- | --- | --- |
| R15 | 7.3 | 802.22 protocols SHOULD include a mechanism that can support dynamically-variable-bandwidth channels and paths (such as those defined for IP environments). |
| R16 | 8 | The 802 standards define a framework for LAN/MAN management in ISO/IEC 15802-2: 1995(E). The framework contains guidelines for managed objects, management protocol, and the relationship to ITU management protocols. 802.22 protocols SHOULD comply with the above-mentioned standards and guidelines. |
| R17 R18 R19 | 8.2 8.2 9.3 | The operator also SHOULD have the means to securely shut down and reactivate a base station remotely. The 802.22 protocols SHOULD support functions that automatically shuts down transmission from a subscriber station or base station in case of malfunction (e.g., power limits exceeded). 802.22 standard SHOULD allow a suitable cryptographic algorithm to be employed that is internationally applicable. |
| R20 | 9.3 | Facilities SHOULD also be defined in the protocol for the use of alternate cryptographic algorithms that can be used in certain localities and that can replace algorithms as they are obsoleted or “legalized” for international use. |
| R21 | 10 | 802.22 SHOULD strive to fit into the 802 system model. |

## A.3	Optional (O)

It is optional that the 802.22 standard support or specify the items in Table 4.

Table 4: Optional Requirements

| # | Section | Requirement |
| --- | --- | --- |
| O1 | 2.1 | The model [Figure 2-2: Wireless Access Reference Model] depicts the relevant |
| O2 | 2.1 | points between subscriber networks and “core” networks (the networks that MAY be accessed via 802.22 air interface). A single SS MAY support multiple customer premises networks that transport |
| O3 | 2.1 | data, voice and video through one or more UNIs. Base stations MAY support multiple core networks through one or more CNIs. |
| O4 | 5.9 | The MAC and PHY standard MAY allow subscribers to hop between |
| O5 | 5.9 | channels. Flexible modulation types, power level adjustment, and bandwidth reservation |
| O6 | 7 | schemes MAY be employed [which affect how the MAC and PHY protocols meet contracted service levels with customers]. [To support on-demand resource allocation,] The MAC protocol MAY allocate |
|  |  | bursts of PDUs to services that require changes in resource allocation. |
| O7 | 7 | The 802.22 MAC layer interface MAY provide a connection-less service |
|  |  | interface that requires a higher-layer “adaptation” to maintain the “state” of a connection and periodically allocate resources. |
| O8 | 9.1 | The second type of authentication is between the subscriber and the WRAN |
|  |  | system. This may or may not be the responsibility of the 802.22 protocols. It MAY be handled by higher layer protocols. |

# Appendix B
Vocabulary of Terms (Normative)

Editor’s Note: Some definitions below are as yet undefined. And a few other are as yet incomplete or in-progress. The 802.22 task group should provide updates for these definitions and release a new version of this document. The in-progress definitions are in italics.

| Term | Definition | Reference |
| --- | --- | --- |
| Access | End-user connection(s) to core networks NOTE 1 - Core networks include, for example, PSTN, ISDN, PLMN, PSDN, Internet, WAN/LAN, CATV, etc. NOTE 2 - The end-user may be a single user or a user accessing the services on behalf of multiple users. | Based on Rec. ITU-R F.1399 |
| Accounting | A function which apportions the revenue obtained by the service providers to network operators in line with commercial arrangements. | Rec. ITU-R M.1224 |
| Ad hoc communication | Use of the wireless access system without reservation or QoS guarantees at sporadic times, by activating the link when needed. |  |
| Air interface | The common boundary between the subscriber station and the radio equipment in the network, defined by functional characteristics, common radio (physical) interconnection characteristics, and other characteristics, as appropriate. NOTE 1 – An interface standard specifies the bi directional interconnection between both sides of the interface at once. The specification includes the type, quantity and function of the interconnecting means and the type, form and sequencing order of the signals to be interchanged by those means. | Based on Rec. ITU-R M.1224 |
| Always on communication | Use of the wireless access system at any time by having the link continuously available. |  |
| Authentication | The process of verifying the identity of a user, terminal, or service provider. | Rec. ITU-R M.1224 |
| Authorization | A property by which the rights to resources are established and enforced. | Rec. ITU-R M.1224 |
| Backhaul service | Transport of aggregate communication signals from base stations to the core network. | IEEE 802.22 |
| Bandwidth; communication channel bandwidth | The information payload capacity of a communication channel available to a user for services (expressed in bit/s or multiples thereof). |  |
| Bandwidth; RF channel bandwidth | The frequency extent of a specified portion of the RF spectrum capable of carrying information over the radio interface (expressed in Hz or multiples thereof). |  |
| Bandwidth; transmission channel bandwidth | The frequency extent required for the transmission of a specified signal (expressed in Hz or multiples thereof). |  |
| Base station | The common name for all the radio equipment located at one and the same place used for serving one or several cells. (See also “station”). | ITU-R Rec. M.1224 |
| Bearer service | A type of telecommunication service that provides the capability for the transmission of signals between user-network interfaces. | ITU-T Rec. I.112 |
| Broadband wireless access | wireless access in which the connection(s) capabilities are higher than the primary rate. | Rec. ITU-R F.1399 |
| Cell | The radio coverage area of a base station, or of a subsystem (e.g. sector antenna) of that base station corresponding to a specific logical identification on the radio path, whichever is smaller. | Based on Rec. ITU-R M.1224 |
| Cell | A block of fixed length which is identified by a label at the asynchronous transfer mode layer of the B-ISDN protocol reference model. | ITU-T Rec. I.113 |
| Cell delay variation | A component of cell transfer delay, induced by buffering and cell scheduling. The proportion of lost cells over the total number of transmitted cells for a connection. | ATM Forum ATM Forum |
| Channel; communication channel | A specific portion of the information payload capacity, available to the user for services. | ITU-T Rec. I.113 |
| Channel; radio- frequency (RF) channel | A specified portion of the RF spectrum with a defined bandwidth and a carrier frequency and is capable of carrying information over the radio interface. | Rec. ITU-R M.1224 |
| Channel; transmission channel Class of Service Core network Customer premises | A means of unidirectional transmission of signals between two points. This is either a Customer Class of Service, a Trunk Class of Service, or a Private-Facility Class of Service. It may refer to either originating or terminating accesses. Core networks include, for example, PSTN, ISDN, PLMN, PSDN, Internet, WAN/LAN, CATV, etc. The equipment/network administered by the user. | ITU-T Rec. I.112 ITU-T Rec. Q.1290 (95), 2 Based on Rec. ITU-R F.1399 Based on ITU-T Rec. |

| equipment/network |  | H.310 |
| --- | --- | --- |
| Downstream | The direction from base station to subscriber station(s). | IEEE 802.22 |
| Dynamically variable bandwidth | A capability of a system to be able to change the bandwidth of the information payload capacity of a communication channel available to a user for services according to negotiated user requirements. |  |
| External Network | Any network that is physically located past the base station or the subscriber station. |  |
| Fixed wireless access | Wireless access application in which the base station and the subscriber station are fixed. | Based on Rec. ITU-R F.1399 |
| Frequency Division Duplex | Separation of upstream and downstream transmission in the frequency domain at the same time. | IEEE 802.22 |
| Internet protocol | Networking protocol defined by IETF standards. | IETF |
| Interoperability | The ability of multiple entities in different networks or systems to operate together without the need for additional conversion or mapping of states and protocols. | Rec. ITU-R M.1124 |
| Inter-working | The means of supporting communications interactions between entities in different networks or systems. | Rec. ITU-R M.1124 |
| Inter-working function | Mechanism which masks the differences in physical, link, and network technologies by converting or mapping states and protocols into consistent network and user services. | Rec. ITU-R M.1124 |
| Link Budget | The allocation of power within a link. Link budget consists of the difference between Tx power and receiver sensitivity. |  |
| Line-of-sight (LOS) | A radio communication link for which 60% or greater of the first Fresnel zone is unobstructed. (i.e., the geographical region where the rules of free space propagation applies). Two points for which the signal suffers temporal fading of less than 10 dB 99% of the time. Note: This definition does not imply that a direct optical line of sight exists. |  |
| Modulation Factor | The sector’s aggregate payload bit rate divided by the bandwidth. |  |
| Multi-tenant | A primarily non-commercial building designed to |  |

| dwelling | contain more than two residential units. |  |
| --- | --- | --- |
| Near LOS | A radio communication link for which 40% or greater of the first Fresnel zone is obstructed. Two points for which the signal suffers temporal fading greater than or equal to 10 dB exceeding 1% of the time and an average delay spread of less than 10 microseconds. |  |
| Non LOS | A radio communication link that is obstructed. Two points for which the signal suffers an average delay spread of greater than or equal to 10 microseconds. |  |
| Network Nomadic wireless access | A set of nodes and links that provides connections between two or more defined points to facilitate telecommunication between them. Wireless access application in which the subscriber station may be in different places but must be stationary while in use. | Rec. ITU-R M.1224 Based on ITU-R Rec. F.1399 |
| On demand communication | Use of the wireless access system for services based on negotiated QoS. |  |
| Peak Data Rate | The maximum bit rate available to a user for communications. |  |
| plesiochronous mode Point-to-multipoint system | A mode where the essential characteristic of time scales or signals such that their corresponding significant instants occur at nominally the same rate, any variation in rate being constrained within specified limits. a system that establishes connections between a single specified point and more than one other specified points. | ITU-T Rec. G.810 (96), 4.3.5 ITU-R Rec. F.1399 |
| Privacy Quality of service | The provision of capabilities to prevent access of information by unauthorized parties. The collective effect of service performance which determine the degree of satisfaction of a user of the service. NOTE 1 - The quality of service is characterized by the combined aspects of service support performance, service operability performance, serviceability performance, service security performance and other factors specific to each service. NOTE 2 - The term "quality of service" is not used to express a degree of excellence in a comparative sense nor is it used in a quantitative sense for | ANSI T1.702-1995 ITU-T Rec. E.800 (94), 2101 |

|  | technical evaluations. In these cases a qualifying adjective (modifier) should be used. |  |
| --- | --- | --- |
| Radio interface Real-Time (adjective) | See air interface Pertaining to the processing or transmission of data according to defined time requirements . | Rec. ITU-R M.1224 Based on ITU-T Rec. Q.9 (88), 6103 |
| Re-use Factor; Frequency re-use factor | As yet undefined |  |
| Sector | As yet undefined |  |
| Sector Bandwidth Factor | As yet undefined |  |
| Security | The protection of information availability, integrity and confidentiality, as well as authentication and authorization. | Based on Rec. ITU-R M.1224 |
| Semi-stateless | As yet undefined | Sect 7 |
| Service Service Capability | A set of functions offered to a user by an organization. Defines possible service arrangements with respect to the number and type of channels that can be supported by the equipment. In this way a subscriber station access capability is the set of possible channel configurations supported by the equipment. Similarly, the base station access capability may be considered to be a super-set with respect to the subscriber station. | Recs. ITU-R M.1224, M.1308 Based on ITU-T Rec. Q.1063 (88), 2.4 |
| Single Residential | A non-commercial building intended to be inhabited by an individual. |  |
| Small Business | As yet undefined |  |
| Small Office/Home Office Spectrum (Spectral) Efficiency | As yet undefined Measures of the accuracy and completeness of system tasks relative to the spectral resources used to achieve the specific system tasks | Based on ITU-T Rec. F.901 (93), 2 |
| Standard | A document established by consensus and approved by a recognized body, that provides, for common and repeated use, rules, guideline or characteristics for activities or their results, aimed at the achievement of the optimum degree of order in a given context. | ISO/IEC Guide 2, 1996 |
| Station | the common name for all the radio equipment at one and the same place. NOTE - The term "station" may refer to any end | Rec. ITU-R M.1224 |

|  | user radio equipment (“subscriber station”) or network radio equipment (“base station”). |  |
| --- | --- | --- |
| Subscriber Subscriber station Supplementary service Synchronous transfer mode | A person or other entity that has a contractual relationship with a service provider on behalf of one or more users. (A subscriber is responsible for the payment of charges due to that service provider.) the common name for all the radio equipment at one and the same place serving one or more users. (See also “station”). A service which modifies or supplements a basic telecommunication service. Consequently, it can not be offered to a customer as a standalone service, rather, it must be offered together with or in association with a basic telecommunication service. The same supplementary service may be common to a number of telecommunication services. A transfer mode which offers periodically to each connection a fixed-length block. | Rec. ITU-R M.1224 Based on Rec. ITU-R M.1224 Rec. ITU-R M.1224 Based on ITU-T Rec. I.113 |
| System | A regularly interacting or interdependent group of items forming a unified whole technology. | Recs. ITU-R M.1224, M.1308 |
| Telecommuter | As yet undefined |  |
| Time Division Duplex | Separation of upstream and downstream transmission in the time domain using the same frequency. | IEEE 802.22 |
| Upstream | The direction from subscriber station(s) to base station. | IEEE 802.22 |
| User | Any entity external to the network which utilizes connections through the network for communication. | ITU-T Rec. E.600 |
| Virtual point-to- point connections | Providing a point-to-point connection to a subscriber using a point to multipoint system. | IEEE 802.22 |
| Wireless access | end-user radio connection(s) to core networks. NOTE 1 - Core networks include, for example, PSTN, ISDN, PLMN, PSDN, Internet, WAN/LAN, CATV, etc. NOTE 2 - The end-user may be a single user or a user accessing the services on behalf of multiple users. | Rec. ITU-R F.1399 |

# Appendix C
Acronyms and Abbreviations

| Acronym/Abbreviation | Definition |
| --- | --- |
| AF | Assured Forwarding |
| ANSI | American National Standards Institute |
| ARQ | Automatic repeat Request |
| BER | Bit Error Ratio |
| BES | Best Effort Service |
| BS | Base Station |
| WRAN | Wireless Regional Area Network |
| CNI | Core Network Interface |
| CoS | Class of Service |
| CSMA/CD | Carrier Sense multiple Access with Collision Detection |
| DFM | Dispersive Fade Margin |
| DSL | Digital Subscriber Line |
| EF | Expedited Forwarding |
| ER | Errored Block |
| ETSI | European Telecommunications Standard Institute |
| FCAPS | Fault, Configuration, Accounting, Performance, Security Management |
| FDD | Frequency Division Duplex |
| FEC | Forward Error Correction |
| FWA | Fixed Wireless Access |
| HFC | Hybrid fiber coax |
| HTTP | Hypertext Transfer Protocol |
| IEC | International Electrotechnical Commission |
| IETF | Internet Engineering Task Force |
| IP | Internet protocol |
| ISDN | Integrated Services Digital Network |
| ISO | International Organization for Standardization |
| ITU-R | International Telecommunications Union- Radiocommunications Sector |
| IWF | Inter-working function |
| JRG | Joint Rapporteur group |
| LAN | Local area network |
| LLC | Logical link control |
| LOS | Line of Sight |
| MAC | Medium Access Control |
| MAN | Metropolitan area network |
| MGMT | Management |
| OSI | Open Systems Interconnection |
| PDU | Protocol Data Unit |
| PHY | Physical layer |
| PMD | Physical Medium Dependent |
| P-MP | Point-to-multipoint |
| PSTN | Public Switched Telephone Network |
| QoS | Quality of service |
| RF | Repeater Function |
| SDE | Secure Data Exchange |
| SEC | Security |
| SES | Severely Errored Second |
| SLA | Service Level Agreement |
| SME | Small and Medium Enterprises |
| SNMP | Simple Network Management Protocol |
| SOHO | Small Office/Home Office |
| SS | Subscriber Station |
| TC | Transmission convergence |
| TCP/IP | Transmission Control Protocol/Internet Protocol |
| TDD | Time Division Duplex |
| UNI | User Network interface |
| VoIP | Voice over IP |
| WEP | Wire-Equivalent Privacy |

# Appendix D
Channel Models Eli  (Gerald, Victor)

- channel models

  - 802.16 had lower limit of 2GHz, need to look

  - 802.16 doc (80216a-03_01_potential_channel_model.pdf) is a good basis for developing this Appendix.  It deals with all physical aspects of the channel.  It will need to be adjusted to the VHF and UHF bands.  It will need to include the consideration of industrial (man-made) noise that is not negligeable in these lower frequency bands.  Broadcast models developed for DTV for DTVwill need to be considered.  Channel delay spread impulse response models developed for mobile telephony such as the European model COST 207 would be useful to characterize multipath to be considered in the development of the modulation for 802.22.

  - COST 207 [1989]  Digital land mobile radio communications.  Final Report of COST Project 207, Commission of the European Communities, Brussels.

  - presence of on-channel repeaters if it is include in the 802.22 standard will change the range of possible multipath excess delays because of the presence of passive as well as active echoes will need to be considered.  Passive channel multipath range is around 25usec for echoes with sizable power but active echoes will extend it greatly.  DTV receiver equalizers typically cover a range of -20 to +40 usec.

  - ATSC-A74 model, Brazilian model

Impulse response, Industrial noise. (Model of 802.16: 500 MHz and up)

# Appendix E
Propagation Model

# 1- ITU-R P1546-1

  - ITU model (Rec. 370 and later Rec 1546) uses FCC F-curves updated and augmented.

# 2- Specific sites with real terrains at these sites (Carl and Gregg)