IEEE P802.22
Wireless RANs

| Functional Requirements for the 802.22 WRAN Standard |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2026-07-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Carl R. Stevenson | WK3C Wireless LLC | 4991 Shimerville Road, Emmaus, PA 18049-4955 USA | +1 610-965-8799 | wk3c@wk3c.com |
| Carlos Cordeiro | Philips | 345 Scarborough Rd Briarcliff Manor, NY 10510 USA | +1 914 945-6091 | carlos.cordeiro@philips.com |
| Eli Sofer | Runcom | Israel |  | elisofer@runcom.co.il |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | +1 613-998-2500 | gerald.chouinard@crc.ca |

Abstract

Introduction

This document represents a collaborative effort by the 802.22 Functional Requirements Ad Hoc Group, starting from discussions held at the January 2005 interim session and drawing and adapting the structure and content of document IEEE 802.16.3-00/02r4 as a framework and source of ideas for further work by the Ad Hoc Group.

Abstract

This document provides functional requirements that are guidelines for developing an interoperable 802.22 air interface. The 802.22 committee desired to reach an understanding and consensus for functional requirements before proceeding with developing a standard for 802.22 MAC and PHY protocols and thus formed a Functional Requirements Ad Hoc Group to produce this document.

Purpose

The 802.22 WRAN WG group will use this document to help qualify MAC and PHY protocol proposals.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Functional Requirements
for the 802.22 WRAN Standard

Xxxx Yyyy (editor)

# 1	Introduction

This document provides functional requirements that are guidelines for developing an interoperable 802.22 air interface for use in spectrum allocated to TV Broadcast Service, enabling Point to Multipoint (P-MP) Wireless Regional Area Network (WRAN). The WRAN system provides packet-based transport capabilities that can support a wide range of services (e.g., data, voice and video) to residential, Small and Medium Enterprises (SME) and Small Office/Home Office (SOHO) locations.

For convenience, requirements are itemized in Appendix A.

Throughout this document, the words that are used to define the significance of particular requirements are capitalized. These words are:

- “MUST” or “SHALL” These words or the adjective "REQUIRED" means that the item is an absolute requirement.

- “MUST NOT” This phrase means that the item is an absolute prohibition.

- “SHOULD” This word or the adjective “RECOMMENDED” means that there may exist valid reasons in particular circumstances to ignore this item, but the full implications should be understood and the case carefully weighed before choosing a different course.

- “SHOULD NOT” This phrase means that there may exist valid reasons in particular circumstances when the listed behavior is acceptable or even useful, but the full implications should be understood and the case carefully weighed before implementing any behavior described with this label.

- “MAY” This word or the adjective “OPTIONAL” means that this item is truly optional. One implementation may include the item because the target marketplace requires it or because it enhances the product, for example; another implementation may omit the same item

# 2	Scope

For the purposes of this document, a “system” constitutes an 802.22 MAC and PHY implementation in which at least one subscriber station communicates with a base station via a point-to-multipoint (P-MP) radio air interface, the interfaces to external networks, and services supported by the MAC and PHY protocol layers. Hence, “functional requirements” describe the functions of typical systems in terms of how they affect requirements of interoperable 802.22 MAC and PHY protocols. The functional requirements describe 802.22 systems and requirements in broad terms: what the required functions are but not how these functions work.

The ‘how’ part is left to the forthcoming 802.22 interoperability standard [1] which will describe in detail the interfaces, functions and procedures of the MAC and PHY protocols. This document focuses on the service capabilities that an 802.22 system is required to support. These service capabilities have a direct impact on the requirements of the 802.22 MAC and PHY protocols. When the 802.22 working group produces an interoperable air interface standard that meets these functional requirements, resulting 802.22-based implementations will be able to utilize and interconnect multi-vendor WRAN devices to provide the expected services to the end users.

Other goals of this document are to formulate reference models and terminology for both network topology and protocol stacks that help the 802.22 working group to discuss and develop the MAC and PHY protocols. As far as possible, these SHOULD be common across 802.22 systems.

The 802.22 protocols relate to other 802 standards and to the OSI model as shown in Figure 1-1.

Figure 1-1: Relationship between 802.22 and other Protocol Standards
(the numbers in the figure refer to IEEE standard numbers)
(Remove 801.10, change 802.16.3 for 802.22)

This standard deals with the Physical and Data Link layers as defined by the International Organization for Standardization (ISO) Open Systems Interconnection Basic Reference Model (ISO 7498: 1984).

The standards that define the services noted in the above diagram are as follows:

- IEEE Std 802: Overview and Architecture. This standard provides an overview to the family of IEEE 802 Standards. This document forms part of the 802.1 scope of work.

- ANSI/IEEE Std 802.1B [ISO/IEC 15802-2]: LAN/MAN Management. Defines an Open Systems Interconnection (OSI) management-compatible architecture, environment for performing remote management.

- ANSI/IEEE Std 802.1D [ISO/IEC 10038]: MAC Bridging. Specifies an architecture and protocol for the interconnection of IEEE 802 LANs below the MAC service boundary.

- ANSI/IEEE Std 802.1E [ISO/IEC 15802-4]: System Load Protocol. Specifies a set of services and protocols for those aspects of management concerned with the loading of systems on IEEE 802 LANs.

- ANSI/IEEE Std 802.2 [ISO/IEC 8802-2]: Logical Link Control.

# 3	PAR Summary

Carl to develop.

# 4	Target Markets

The target markets described in this section are not an exhaustive set, but serve as guidelines and examples that suffice for meeting the broad applicability goals set forth by the air interface “Five Criteria” as described in the IEEE 802.22 Project Authorization Request (PAR) and “Five Criteria” [1, 2].

A Wireless Regional Area Network (WRAN) system based on 802.22 protocols is intended to make use of unused TV broadcast channels, on a non-interfering basis, to address, as a primary objective, rural and remote areas and low population density underserved markets with performance levels similar to those of broadband access technologies serving urban and suburban areas The WRAN system should also be able to scale to serve denser population areas where spectrum is available.

- The WRAN system MUST be capable of supporting a mix of data, voice (VoIP) and audio/video applications with corresponding provisions for QoS.  The RF link availability assumed for the provision of these WRAN applications is 99.9% of time.

The critical parameters for serving these markets using wireless access technology are the combination of coverage/capacity factors that affects access cost per user, deployability, maintainability, and product costs associated with the customer premise installation, and spectrum efficiency/reuse for economically serving the required number of customer locations with a minimum number of base station locations and backhaul routes.

The target markets to be addressed by the 802.22 protocols in WRAN networks are single-family residential, multi-dwelling units, SOHO, small businesses, multi-tenant buildings, and public and private campuses.

In accordance with ITU-R [3] definitions, Fixed Wireless Access (FWA), to which WRAN belongs provides access to one or more (public and private) core networks, rather than forming an end-to-end communication system. 802.22 systems serve fixed location customers who might be geographically fixed or re-locatable.

# 5	WRAN System Model/Requirements

This section presents a high level description of a system model to be used as a framework for developing the 802.22 standard. The model identifies the main features of an 802.22 system, and the terminology to be used by the 802.22 working group in the creation of the standard.

The 802.22 wireless regional area network system is aimed at providing broadband access with capabilities similar to ADSL and cable modem technologies, but capable of more economical deployment over less populated rural areas.  The typical range of the system is [33] km (based on 4 Watt CPE EIRP and F(50, 99.9)) for a coverage of population density of about 1.25 person/km2 and above, and up to a maximum of [100] km if higher base station transmit power is permitted in some regulatory domains.  The system will need to operate over a set of typical channels models as defined in Appendix D.

As mentioned in section 1.1, an 802.22 “system” constitutes an 802.22 MAC and PHY implementation in which at least one subscriber station communicates with a base station via a point-to-multipoint (P-MP) radio air interface, and services supported by the MAC and PHY protocol layers. Specific applications of the 802.22 point-to-multipoint (P-MP) radios are aimed at the use of the VHF/UHF TV broadcast frequency range. Radio communications in the above range may be possible in near and non-line-of-sight situations between a base station and subscriber stations. Operation may include partial and even complete blockage by foliage.  This will contribute to signal attenuation and multipath effects. Figure 2-1 shows an example deployment configuration including the optional use of macro diversity (optimization of link and use of repeaters) (2nd class base station, see section 5.6.1.2.1). 802.22 systems should be deployable in multiple-cell frequency reuse systems and single cell (super cell) frequency reuse systems. The range of 802.22 radios varies with EIRP, local topography, atmospheric conditions, channel characteristics, availability requirement and local regulations as well as bandwidth and transmitter/receiver performance.

Figure 2-1 Example Deployment Configuration
(figure should be fixed to be more appropriate to the WRAN environment, need to include houses with TV receivers, hills, etc. to illustrate more rural environment)

An 802.22 system MUST consist of one base station radio and one or more Consumer Premise Equipment (CPE) radios.  It defines an 802.22 base station and one or more stationary CPE radios communicating using the 802.22 MAC and PHY protocols.

Proposals for 802.22 MAY also include a description of how repeaters could be accommodated.

The base station radio SHALL be P-MP, radiating its downstream signal (forward) toward the CPE’s with an omni-directional, a shaped sector, or optionally an adaptive array (spatial reuse) antenna achieving broad azimuthal beam width to serve a number of prospective subscribers.

For the purpose of coexistence with incumbent services operating in these TV broadcast bands (TV broadcasting, wireless microphones operating according to FCC Part 74 and Public Safety), the 802.22 standard SHALL include mechanisms in the PHY and MAC protocols to allow the base stations to dynamically change the frequency of operation of the network based on the sensing of the use of the spectrum by these incumbent services by the base station and the CPE’s to avoid interference to these services (see section 8).  This will constitute an essential part of this standard.

For the purpose of coexistence among WRAN systems operating in the same area, the 802.22 standard SHALL include mechanisms allowing cooperation between base stations for better sharing of the spectrum.  The MAC and PHY protocols MUST provide means for base stations to resolve interference problems due to collocation or overlapping coverage areas.

The frequency bands used by 802.22 systems MAY vary across various regulatory domains (see section 8.1.1.1).  In the case of the USA, the frequency range identified by IEEE 802 in its comments to the FCC was from TV channel 2 to 51, (54 MHz to 698 MHz) the 802.22 PAR identifies 54 MHz to 862 MHz, but the extremes of the international range are from 41 MHz to 910 MHz.  [CRS note:  If true, we may wish to submit a request to modify the .22 PAR to encompass at least the upper end of this frequency range.]

Since the 802.22 system MUST operate without causing interference to incumbent licensed services, sensing of channel occupancy SHALL be done according to the regulatory requirements in the regulatory domain where a 802.22 system is installed and operated.

## 5.1	Wireless Access Reference Model

Figure 2-2 shows the 802.22 wireless access reference model. The model depicts the relevant points between subscriber networks and “core” networks. A greater system encompassing user terminals, base station interconnection networks, network management facilities, etc. MAY be envisaged but the 802.22 protocol focuses on the air interface shown in the model. The Core Network Interface (CNI) and the User Network Interface (UNI) are also shown.

A single Customer Premise Equipment (CPE) MAY support multiple customer premises networks that transport data, voice and video through one or more UNIs.

Base stations MAY support multiple core networks through one or more CNIs.

For the purposes of 802.22, the UNI and CNI are abstract concepts. The details of these interfaces are beyond the scope of this document.

The standard SHALL specify MAC layer protocols and PHY transmission techniques suitable for providing access between one or more CPE and base stations to support UNI and CNI requirements.

Figure 2-2: Wireless Access Reference Model
(figure needs to be fixed, change SS to CPE …)

### 5.2	IEEE 802 Architecture Conformance

The 802.22 standard SHALL conform to the requirements of the IEEE 802.1 Architecture, Management and Interworking documents as follows: 802. Overview and Architecture, 802.1D, 802.1Q and parts of 802.1f (Optional MAC Bridge to other networks (e.g. 802.11x, others?)

## 5.3	Service capacity

The required minimum peak throughput rate at edge of coverage SHALL be 1.5 Mbit/s per subscriber in the forward direction and 384 kbit/s per subscriber in the reverse direction.  The capacity of the base station will need to be higher to provide service to a number of subscribers in this P-MP system.

The system SHALL operate with a minimum spectrum efficiency of 0.5 bits/sec/H and a maximum goal of 5 bit/(s*Hz) or better where propagation permits. CPEs should be designed to operate at as a high spectrum efficiency as possible, as allowed by regulations or/and standard characteristics.

## 5.4	Installation Requirements

Base stations SHALL be designed to be professionally installed.

CPE’s MAY be user installable (plug and play) but operators MAY choose to provide for professional installation.

The CPE antenna SHALL be assumed to be installed outdoors at a nominal height of 10 meters above ground level.  Omnidirectional sense antenna chacteristics, with the sense antenna co-located with the transmit antenna SHALL be assumed.  Sensing thresholds in other sections of this document are based on these assumptions.  Installation and deployment  requirements  should be further elaborated in proposals with the understanding that, due to the unique requirements of assuring incumbent services are protected, they will be incorporated in a normative manner in the standard.  Proposals may discuss the implications of WRAN antenna installations at heights other than 10 meters above ground level.

Means SHALL be provided to allow verification of proper CPE installation either visually or remotely from the base station. Proposers are encouraged to submit  ideas for mechanisms by which such verification could be done automatically. If such a mechanism is adopted, it SHALL ensure that CPEs will not transmit absent reliable verification.

[need to renumber to not skip 5.5]

## 5.6	Network Entity Relationships and Topology

The topology is P-MP with optional use of repeaters. Repeaters can be used to extend the coverage area or enhance network capacity. In all cases, all device types SHALL remain under the control of the base station with the BS providing centralized power, spectrum management, and scheduling control.

### 5.6.1	Base Station/CPE Master/Slave Relationship

The base station SHALL serve as a radio resource supervisor and controller for its “cell,” including all associated CPEs and/or optional repeaters, if implemented.

A Master/Slave relationship between the base station and the CPE’s  SHALL be established whereby all the RF characteristics of the CPEs are remotely controlled by the base station.

The base station SHALL also be capable of remotely controlling the distributed sensing of the RF Environment by its associated CPEs (in addition to CPEs’ inherent autonomous sensing capabilities).

### 5.6.2 P-MP Star  [CRS note:  need better sub-section title???]

In the downstream direction, all traffic directed to CPEs SHALL be transmitted by the base station.  However,  mechanisms may be proposed that avoid unnecessary retransmissions by the base station and hence improve spectrum efficiency.

In the upstream direction, 802.22 protocols MUST provide the means to efficiently multiplex traffic from multiple CPE’s and allocate transmission channel capacity.

##### 5.6.3	Optional Repeater Function

802.22 systems MAY support the optional deployment of repeater functions. Proponents may wish to propose mechanisms for repeaters to extend coverage into areas that could not otherwise be reached by the service and document the impact of the use of these repeaters on the functionality of the PHY and MAC layers.  The repeater function[, if implemented SHALL preserve the end-to-end operation of 802.22 protocols between the base station and the CPE’s.

## 5.7	Wireless Media Characteristics

### 5.7.1	Multiple access techniques

Need to support ….(Carl) (???)

### 5.7.2	Duplex Modes

This standard SHALL support duplex modes of operation.

Half-duplex MAY also be considered as long as service latency is kept to an acceptable level.  [TBD???]  This covers the cases of the TDD mode and a FDD mode where duplex operation is assumed at the base station but half-duplex operation is used at the CPE to reduce the user terminal complexity.

If FDD is proposed, the proposal MUST include a proposed band plan scheme and address system impacts such as duplex filter requirements, antenna considerations, and other factors affecting performance and overall system complexity.

If TDD is proposed, the proposal MUST include diagrams illustrating the proposed framing/timing and analysis of the impact of TDD turn-around time on system capacity, latency, and other factors affecting performance and overall system complexity.

### 5.7.3	Flexible Asymmetry

Symmetry is hard to predict and is very bursty depending on the type of traffic.  Some applications utilize naturally asymmetrical bit rate, such as for generic Internet access where most of the throughput is consumed in the downstream direction. Some applications utilize asymmetrical bit rate in the reverse direction, using more in the upstream direction, such as surveillance video from a CPE. Other applications require symmetrical capacity, such as telephony and video conferencing [15].

The 802.22 WRAN system SHALL support flexible asymmetry.

802.22 WRAN systems SHALL also have the flexibility to satisfy the bit rate requirements of a mix of applications in both directions.

It should be recognized that CPE’s at the edge of coverage may not be capable of achieving the same bit rate in the upstream direction as the base station can provide to them in the downstream direction.

# 6	Supported Service Capabilities

This section describes typical service capabilities that MAY be supported by an 802.22 air interface. The MAC and PHY protocols will not need to have explicit support for each and every service, due to the fact that generic data streams SHALL be used for transport. The MAC and PHY protocols SHALL provide for admission control, QoS service specific support, appropriate PER for the various types of services, and acceptable latency and jitter for real time services.

## 6.1	Data Transport Service Capabilities – Internet and VoIP

The 802.22 system SHOULD directly transport variable-length IP datagrams efficiently. Both IP versions 4 and 6 MUST be supported with packet prioritization, admission control and scheduling.

802.22 systems SHOULD support both (quasi) real-time and non-real-time service capabilities.

The 802.22 protocols SHOULD support VoIP services.

It SHOULD be possible to support IP Quality of Service (QoS) efforts: Differentiated Services [RFC 2475- Dec 1998, RFC 2474, latest reference?].

The following types of services SHALL be supported, in those situations where capacity is available, recognizing that avoiding interference to incumbent licensed services takes precedence over maintaining WRAN service(s): (see 802.1 and 802.3 for priority and access control)

- Constant bit rate service

- Real-time variable bit rate service

- Non-real-time variable bit rate service

- Best-effort service

## 6.2	Bridged LAN Service Capabilities

The 802.22 protocols SHALL support 802.1 bridged LAN service capabilities.

## 6.3	Other Services

Other services that, for instance, require QoS-based delivery of the MAC services MAY be added.

These services SHALL NOT place any additional requirements on 802.22 systems (MAC and PHY protocols) not already covered in the above sections.

# 7	802.22 Protocols

The IEEE 802.22 MAC and PHY protocol stacks SHALL be the same for all the supported services. The central purpose of the MAC protocol layer in 802.22 is sharing of radio channel resources. The MAC protocol defines how and when a base station or subscriber station may initiate transmission on the channel.

Since CPE’s MAY contend for capacity to/from one or more base stations, the MAC protocol MUST efficiently manage contention and resource allocation.

The PHY layer MAY optionally be subdivided between a convergence layer and a physical medium dependent (PMD) layer. The PMD is the “main” part of the PHY. Like the MAC convergence layers, the PHY convergence layers adapt/map the “special” needs of the MAC services to generic PMD services.

# 8	Performance and Capacity

This section addresses some issues regarding 802.22 system performance and capacity. Specifying protocols that can maintain specified/mandatory performance levels in the face of fluctuating and diverse channel characteristics (e.g., due to multipath and atmospheric conditions) is a problem that the 802.22 work group has to consider. This section specifies the target performance levels. This section also outlines some of the issues for 802.22 capacity planning.

The PHY and MAC protocols SHALL flexibly accommodate communication between a base station and CPEs at distances of up to 100 km.

## 8.1	Peak Data Rate

The minimum peak rate at the edge of service SHALL be 1.5 Mbit/s downstream and 384 kbit/s upstream.

802.22 protocols SHALL be optimized to support the minimum peak data rate in both directions to a CPE station within the specified distance from the base station.

Optional modes that can provide different rates MAY be included.

The 802.22 MAC protocol SHOULD allow the peak data rate to scale up to the total practical channel capacity (see section xxx).

## 8.3	Multipath/Delay Spread Performance

See channel model in Appendix D

## 8.4	Ranging / Synchronization / Acquisition / Access

The first time a CPE is turned on, it MUST start by sweeping the RF range in which it is to operate to identify the presence of incumbent operations, as well as to access information from the WRAN networks accessible in the area.

A list of available WRAN networks and their received signal strength at the user location should be available for e.g., presentation to the user for choosing the appropriate WRAN network.  [to annex]

The CPE SHALL then attempt to associate with the selected WRAN base station according to the parameters sent over broadcast control packets.  Initial hand shaking with the base station SHALL be done for ranging, initial synchronization and CPE initialization.  Ranging SHALL be done with the goal to minimize the required guard-time interval in order to maximize the transmission medium capacity.

The CPE SHALL be synchronized with its associated base station before attempting any transmission.

Network entry time for a CPE SHOULD be no more than 10 seconds under normal operating conditions.

## 8.5	Frequency/time tolerances

The frequency stability of 802.22 base stations SHALL be better than 2 ppm (re. 802.16 (2004) Section 8.4.14.1).

Tolerance for the CPEs SHOULD be more relaxed than that of the base station to minimize CPE cost and complexity.

CPEs SHALL be capable of synchronizing to the base station to adjust their drift in frequency and time to within the 2 ppm tolerance of the base station.

The CPE’s frequency/timing control system(s) SHALL have enough range to achieve synchronization to the base station frequency and timing over worst case initial offsets, tolerances, and environmental extremes.

The CPE SHALL be capable of acquiring frequency and time synchronization in a sufficiently short period to secure proper service.	Should this be kept???

The frequency and time stability of the base station and CPE’s as well as the dynamic behavior of the clock synchronization should be documented by the proponents.

## 8.6	Spectral Efficiency

Spectral efficiency is an important performance parameter of a wireless access system. This spectral efficiency will vary from location to location within the coverage area of the base station, depending on distance, propagation, channel impairments, interference, and their effect on the usable modulation and coding parameters over any given path.

The WRAN system should be capable of providing spectral efficiencies in the range from 0.5 bit/(s*Hz) up to a goal of 5 bit/(s*Hz) or better depending on the transmission channel conditions.  The proponent should document the way they quantify these minimum and maximum spectral efficiencies and in which operational conditions they would apply.

## 8.7	Radio Link Availability

802.22 systems are expected to support all intended services at their respectively rated QoS (see section 14.3) except for 0.1% of the time when the RF link may not be available (based on a link availability of 99.9% of the time [14, 16]), (not counting network entry time, quiet periods for RF sensing, and electrical power and equipment down-time).  Information on how the technology will allow to achieve this requirement and help conceal and react to the downtimes should be provided.  The proposed WRAN technology should allow for graceful degradation through a reduction of the throughput rather than a complete loss of service.

The coverage of WRAN service will be based on 50% location availability at the edge of the coverage area for a median location.

The 802.22 technology should also allow for the radio link to be engineered for different link availabilities, based on the preference of the system operator.

## 8.8	Radio Link Error Performance

The error rate, after application of the appropriate error correction mechanism (e.g., FEC), delivered by the PHY layer to the MAC layer SHALL meet IEEE 802 functional requirements (10-8 undetected corrupted packet rate) and the radio link Packet Error Ratio (PER) SHALL be no more than 10% for a packet length of 1000 octets at the minimum specified receiver sensitivity.

Furthermore, in order to assure that the base station is able to control all of its CPEs, control frames SHALL be transmitted in a manner to assure a probability of [tbd%] that all CPEs will be able to receive [tbd%] of the control frames.

## 8.9	Delay, Jitter, and Latency

Delay and variation of delay, or jitter, are important factors to consider. For example, a high variation of delay can severely impact interactive services. However, generic Internet access can tolerate a high degree of delay variation.

Jitter for the services considered SHOULD be maintained to less than [0.5 msec] (see Table XXX in Annex YYY).

Maximum latency caused by the 802.22 PHY and MAC SHOULD be no more than [20 msec] to support VoIP (see section 14.3).

## 8.10	Capacity

The 802.22 base station capacity requirement is defined as the product of the number of subscribers, their peak bandwidth requirements and load factor, based on quality of service goals.

Given the propagation characteristics in the target frequency bands and variability in differing geographic area for the development of a link budget [16], the following parameters of an 802.22 system SHOULD be addressed by the proponents in the development of  their MAC and PHY protocols:

- Radio range ([33] to [100] km)

- Upstream/downstream channels’ data rates

- Allocation of prospective subscriber data rate to channels. Note: the MAC and PHY standard MAY allow subscribers to hop between channels

- Types of modulation

The MAC and PHY protocols SHOULD accommodate channel capacity issues and changes in channel capacity by employing, for example, flexible modulation types, power level adjustment, and bandwidth reservation schemes when needed(see section 6).

## 8.11	Delivered Bandwidth

The minimum delivered peak data rate per subscriber SHALL be 1.5 Mbit/s forward and 384 kbit/s return.

## 8.12	Flexible data rate and resource allocation – Downstream & Upstream

The standard SHALL support dynamic capacity allocation to ensure that subscribers’ quality of service goals are met.

System capacity allocation SHOULD be able to be varied dynamically to accommodate a wide mix of usage scenarios and user demands for varied services.

The system SHALL provide its operator to flexibly allocate its system resources to any CPE for the purpose of limiting or enhancing its throughput.

In the case of an optional repeater, the system SHOULD be capable of providing for more capacity to such a ‘2nd class’ base station.

### 8.12.1	Peak Data Rates

The WRAN base station SHOULD be able to provide to CPEs, higher bit rates than the previously stated minimum peak data rate if the system capacity is available, up to the full forward throughput to one subscriber, at times of low system loading and CPE’s SHOULD be able to receive the full throughput from the base station.

In the return direction, CPEs SHOULD be able to increase its transmission capacity at times of low system loading, up to the full channel throughput as long as the transmit power limit is not exceeded, i.e., a CPE relatively close to the base station may, as directed by the base station, use part of the power margin of its Transmit Power Control (TPC) to achieve higher return throughput.

8.12.2	Number of User Terminals serviced by a base station [to be sent to an annex]

Typically, there will be more that one user terminal connected to the household and SOHO CPE.  In order to assess the number of user terminals that a base station can support, an over-subscription ratio of 50 has been assumed.

Assuming an average 3 bit/(s*Hz) spectrum efficiency, a 6 MHz channel, and the required minimum peak data rate of 1.5 Mb/sec per user terminal, this results in a total of 600 user terminals potentially being served by a single base station. Assuming an average of 3 user terminals per CPE, the number of CPEs addressed by a base station will be typically 200.

## 8.13	Connection loss and network outage response

[CPE initialization sequence:  The CPE SHALL go through the following initialization sequence when it is moved, turned on or has lost power for more than 1 hour.

- Cold start:

- scan the channels, ….

- user to enter ZIP code, address, or (lat. long)

- D

- d

- Moved CPE: enter ZIP code, address, or (lat. long)

- power outage:

- lost connection: If the CPE has not received the necessary control packets from the base station for more than tbd sec, the terminal has to re-initialize]

[Flowchart needed]

[Carl to develop.]

# 9	Base Station and CPE Duty Cycles

Base stations SHALL support 100% transmit duty cycle at rated power.

CPEs SHALL support 100% duty cycle at rated power to support cases where maximum upstream throughput is required from a terminal at the edge of the coverage.

# 10	Adaptability and Scalability

## 10.1	Adaptability

Adaptability refers both to modification of specific transmission parameters as well as to download of updated firmware and software for the CPEs.

### 10.1.1	Per-Subscriber Rate Adaptation

The PHY and MAC protocols SHALL provide the ability to adaptively deliver different bit rates/capacities to individual subscribers.

Also, the PHY and MAC protocols SHALL provide for adaptable channel capacity as a function of channel performance. With a minimum spectrum efficiency of 0.5 bit/(s*Hz) and a goal of 5 bit/(s*Hz) or better where propagation permits.

### 10.1.2	 Per-Subscriber Power Adaptation (TPC)

While the control frames need to be sent by the base station at constant high power so that all CPEs, including those in fringe conditions can also receive it, other packets don’t need to be heard  by all CPEs all the time and therefore need less power/protection.

The WRAN system standard SHALL support Transmit Power Control (TPC) on a link-by-link basis to allow a reduction of the transmit power at the base station and at the CPEs to as low levels as possible while still maintaining a reliable connection at the rated QoS.  A trade-off will be possible between such low transmit power level and the need to increase the complexity of the modulation scheme to maximize the link throughput  as much as possible.  The TPC range SHALL be at least [40 dB] with [0.5 dB] steps.

## 10.2	Scalability

Scalability refers to varying system operational parameters such as: bit rate, channel bandwidth (varying channel bandwidth within the TV channel and/or using a variable number of TV channels), extent of coverage (e.g., TPC), deployment, etc.

### 10.2.1	Bandwidth scalability

10.2.1.1 	Use of fraction of TV channel

The operation of the WRAN system may need to be restrained to a smaller portion of the TV channel when interference to wireless microphones operating on an adjacent TV channel and other LE systems is considered, and even for operational reason.  The 802.22 standard may need to include scalability with respect to occupied bandwidth.  As a minimum, the 802.22 standard SHALL include the scalability necessary to adapt to the 6, 7 and 8 MHz TV bandwidths that exist in the various parts of the world.

10.2.1.2 	Use of multiple TV channels

Where spectrum is available, it may be useful for a WRAN system to use more than one TV channel (contiguous or not) to increase the capacity of the transmission link as long as each TV channel can become independent from the modulation point of view to be able to free any one of these channels in case of interference.  (Note: 802 commented to the FCC that channel bonding should not be permitted to be persistent.)

### 10.2.2	Link symmetry scalability

In addition to the minimum requirement of 1.5 Mbit/s and 384 kbit/s specified in section 8.1, the 802.22 standard MAY provide higher return link capacity, up to and including a fully symmetrical link.

# 11.0 Support for Different Classes of Base Stations

The 802.22 standard SHALL support different classes of base stations with different conditions of operation where permitted by regulatory domains such as transmit power levels, etc.

[TABLE HERE???]

# 12.0	Flexibility in Tuning

Although the total range over which the 802.22 systems may operate worldwide is from low VHF (42 MHz) to mid UHF (910 MHz), it is unlikely that, in practice, the total range will be covered by a common set of antennas and RF front ends at the base station and subscriber terminals.

It is expected that this range will be divided in more reasonable segments such as the low-VHF range (band 1), high VHF range (band 2), low-UHF (band 4) and medium UHF (band 5).

Although the flexibility in dynamic frequency selection to avoid interference will be limited to these segments, it is presumed likely that such flexibility will be sufficient in most cases.  In more difficult cases, equipment covering more than one range may be needed.

The 802.22 standard SHALL support operation in these frequency segments as well as the amalgamation of more than one of these segments.

# 13	Channelization

System implementation SHALL support channel spacings of 6, 7 and 8 MHz  and proper centre frequencies to align with the frequency raster of the incumbent broadcast systems in the area.

The CPE RF front-end and tuning SHALL be capable of adapting accordingly.

Subdivision of these nominal channels MAY be used as long as the regulatory maximum EIRP per device in a 6, 7 or 8 MHz channel is not exceeded.

In order to reduce the peak power transmitted from a CPE, the 802.22 protocols MAY allow sharing of the upstream channel capacity on a FDM basis to allow simultaneous longer transmission frames to share the medium. [to annex]

# 14.0	Medium Access Control (MAC)

[Need introduction to following sections on what the base station and CPE’s will do to avoid interference.]

## 14.1 Network Operations

The 802.22 protocols SHALL provide mechanisms for authorization, registration, continued operation and deregistration of CPEs to 802.22 networks while avoiding interference to incumbent users.

All operations SHALL take place under the control of the base station, which SHALL be capable of simultaneously supporting multiple CPEs.

## 14.2	Support for Interference Mitigation/Coexistence

### 14.2.1	Sensing Measurements

802.22 Base Station SHALL control CPEs to conduct measurement activities and obtain measurement results.  There may be a need for scheduled quiet periods for sensing to take place.

Sensing repetition rate and integration time SHALL be identified in order to meet the sensitivity down to the specified threshold in a timely manner.

Sensing SHOULD include capture of signal signature to identify the type of incumbent and other LE signals.

#### 14.2.1.1 Sensing Control

802.22 protocols MUST provide mechanisms for sensing control in the CPEs.

The Base Station SHALL be able to instruct the CPEs when to sense, which channels to sense and the duration of the sensing activity.

Additionally, the base station SHALL specify to the CPEs a threshold per incumbent type for comparison purposes to assure maximum incumbent protection .

#### 14.2.1.2 Sensing Measurement Report

CPEs SHALL send to the Base Station sensing measurement reports in a form understandable by the Base Station.

In addition, CPEs SHALL be able to acquire and convey to the Base Station the incumbent or other LE RF signal profile.

CPE’s COULD also recognize RF signal profiles, and in such cases, these CPE’s SHALL be able to recognize the incumbent services present in the regulatory area of operation (see Section 11.1.4).

The IEEE 802.22 air interface SHALL provide identifiers for CPEs and Base stations to facilitate the resolution of any  interference problems that might occur (see Section 10.7).

### 14.2.2	PHY Operational Control

#### 14.2.2.1	Channel Management

#### 14.2.2.1.1	Channel Change

Once the CPE has received the channel change request from the base station, the 802.22 MAC SHALL order the 802.22 PHY to change the channel of operation.

#### 14.2.2.1.2	Suspend/Resume Channel Transmission

The 802.22 MAC SHALL be able to suspend the transmission in a specified channel. Transmission in this channel MAY be resumed upon indication from the Base Station.

#### 14.2.2.1.3	Termination and Reestablishment of Channel Operation

The 802.22 MAC SHALL be able to terminate the operation in a channel. Operation MAY be reestablished in another channel.

#### 14.2.2.2 Control of PHY Adaptability

The 802.22 MAC SHALL be able to control the PHY adaptability features such as TPC, modulation/FEC, channel bandwidth, and timing.

### 14.2.3	Base Station Management of CPEs

#### 14.2.3.1 Base station and CPE Relationship

The 802.22 system follows a master/slave relationship where the base station performs the role of the master    and the CPE’s are the slaves.  Hence, in an 802.22 system, the base station SHALL be responsible for managing CPEs in numerous aspects including modulation, coding, encryption, power level and bandwidth usage.

#### 14.2.3.1.2 Measurement Management

Measurement activities in an 802.22 system are also managed by the base station.  Thus, the base station SHALL be able to order CPEs to perform measurements and report the results back to the base station.

#### 14.2.3.1.3 Measurement analysis

Measurements analysis is an ongoing process conducted by the base station and which is based upon measurement results reported by CPEs as well as the base station’s own measurements. Once sufficient measurement information is available to the base station, the base station SHALL be able to analyze that data and, if necessary, take appropriate steps to resolve any interference potential by changing its operating parameters such as frequencies of operation and power level.

To augment this functionality, the base station MAY rely on other features such as externally available databases (see Section 15.2.5), information from other 802.22 base stations, and service provider specific information.

### 14.2.4	Inter-Base Station Coordination

The IEEE 802.22 [air interface] protocols SHALL provide mechanisms to allow for coexistence, interference resistance, and sharing of radio resources amongst neighboring 802.22 Base Stations.

CPEs SHALL be able to report interference received from another 802.22 base station, and base stations SHALL be able to take measures in order to resolve the issues.

## 14.3	Class of Service and Quality of Service

This section describes the classes of service and quality of service for 802.22 systems. Terminology is borrowed from the Internet Engineering Task Force (IETF).

802.22 protocols MUST support classes of service (CoS) with various quality of service (QoS) goals to support the services that an 802.22 system MUST transport.

Thus, 802.22 protocols MUST define interfaces and procedures that accommodate the needs of the services with respect to allocation and prioritization of resources.

Additionally, 802.22 protocols MUST provide the means to ensure that QoS levels are satisfied under the constraints of license-exempt operation in presence of incumbent services. Table 1 provides a summary of the QoS requirements that the PHY and MAC SHALL provide. Note that parameters in the table are measured between the MAC input at the upper layer at the transmit station and the MAC output at the upper layer of the receiving station for information transmission. For example, delay does not include setup time, link acquisition, voice codecs, etc.

For evaluation purposes, proponents SHOULD indicate the Packet Error Rates (PER) that their proposed solution can provide for each type of service shown in Table 1.

Either connection-less or connection-oriented services, or a mix of both, MAY be proposed for consideration.

For QoS-based, connectionless services, the 802.22 protocols MUST support resource negotiation “on-demand”. For instance, the MAC protocol MAY allocate bursts of PDUs to services that require changes in resource allocation. Such allocation, for connectionless services, is thus performed in a semi-stateless manner.

A connection-oriented service MAY require “state” information to be maintained for the life of a connection. However, the 802.22 MAC layer interface MAY provide a connection-less service interface that requires a higher-layer “adaptation” to maintain the “state” of a connection and periodically allocate resources. For instance, the MAC may need to maintain “state” information about a QoS data flow only for the duration of an allocation.

Table 1: Services and QoS Requirements

| Service | Maximum Ratio | Maximum Latency Delay (One way) |
| --- | --- | --- |
| Full Quality Telephony (Vocoder MOS) 4.0) | BER10-6 | 20 ms |
| Standard Quality Telephony (Vocoder MOS) < 4.0) | BER10-4 | 40 ms |
| Time Critical Packet Services | BER 10-6 | 20 ms |

### 14.3.1	Types and Classes of Service

- The fundamental QoS model that will be exported to the WRAN endpoints SHALL be IP based and conform to IETF DiffServ QoS model in conjunction with other IP based protocols. (For more information, please refer to document 22-05-00xx-0000, section A.12, and relevant IETF documents [reference?]).

### 14.3.2	Parameters

802.22 protocols SHALL define a set of parameters that preserve the intent of QoS parameters or IP-based services.

### 14.3.3	Service QoS Mappings

The classes of service and QoS parameters of services SHALL be translated into a common set of parameters defined by 802.22.

A QoS-based IP network MAY employ the Resource reSerVation Protocol (RSVP) [27] to “signal” the allocation of resources along a routed IP path.

If 802.22 is to be a “link” in the IP network, an IWF MUST interface with 802.22 to negotiate resource allocation.

The basic mechanism available within 802.22 systems for supporting QoS requirements is to allocate bandwidth to various services.

802.22 protocols SHOULD include a mechanism that can support dynamically-variable-bandwidth channels and paths (such as those defined for IP environments).

## 14.4	Single channel versus Multichannel Operation

The simultaneous use of more than one TV channel (contiguous or not) MAY be possible as long as it is possible to free up any of these channels in case of interference to the incumbent services.

The 802.22 MAC protocols SHALL be able to terminate operation in any of the channels being used if the need arises.

## 14.5	OA&M Support  [add definition to annex]

The 802.22 air interface SHALL provide a mechanism to enable the provisioning and collection of metrics, so that the network operator can effectively control, monitor, and tune the performance of the 802.22 air-interface.

Provisioning parameters, performance metrics and other OA&M values SHALL be made available through a standards compliant MIB.

### 14.6	Base station and CPE’s address space

The 802.22 protocols SHALL support addresses that uniquely identify CPEs and base stations and which SHALL be used for, among other things, resolving interference.

[802.22 transmitted frames SHALL contain, at a minimum, the address of the transmitter device to identify the source of the transmission for the purpose of facilitating interference resolution with incumbents as well as other 802.22 systems.]

# 15.0	Co-existence and interference mitigation

The context in which WRAN systems will be operated is not usual for License-Exempt type services.  The TV broadcast bands in which they will operate are already used by TV broadcasting, Part 74  devices (for example wireless microphones in the US) and terrestrial mobile radio services including Public Safety (for example Part 90 in the US).

#### The PHY and MAC protocols of the WRAN standard MUST include the necessary means to operate in such an environment without causing interference to these incumbent services.

#### The WRAN system SHALL also provide a fail-safe operation mode in the CPEs and base stations to avoid interference should anomalous situations occur.

Proposals SHALL provide flowcharts or other means of fully explaining and characterizing the algorithms and mechanisms they employ to meet these requirements with the understanding that the solutions selected will be incorporated in a normative manner in the standard.

Furthermore, these protocols SHALL also include means to allow coexistence among multiple WRAN systems and with other license-exempt systems in these bands for fair and efficient use of the spectrum.

## 15.1	Licensed Incumbent Sensing and Avoidance

### 15.1.1	Overview

Interference avoidance SHALL be based on RF sensing at the base station and at the CPE’s and SHALL rely on cognitive radio techniques and intelligence at the network level.

The WRAN system SHALL provide the means for the base station operator to assess interference potential by mapping the results of the sensing of spectrum occupancy performed by the base station and the CPE’s against information on their physical location acquired through registration information or other geolocation mechanisms.

Data fusion from a sufficiently large number of CPE’s will be needed to get a reliable spectrum occupancy figure.

#### Such sensing will include the monitoring of broadcast operation, wireless microphone operation, mobile radio servicesand other WRAN system operations in the area.

If the implementation requires a quiet period, it MUST be done within the requirements of providing service QoS (i.e., network throughput and latency) as much as possible.

#### 15.1.1.1	TV broadcast: description and technical characteristics

In the context of developing an international standard for WRAN, the characteristics, channel rasters (6, 7, 8 MHz) and extend of frequency bands which tend to change with international regions will have to be accommodated.  Appendix A.13 includes tables that give details on the various channellization schemes and signal formats.  The WRAN system will need to sense the presence of these TV systems to avoid potential interference.

#### 15.1.1.2	Wireless Microphone description and technical characteristics

Wireless microphones are operated by broadcasters, news crews, in-studio and in remote production locations.    The typical system characteristics are detailed in Appendix I.  Various regulatory environments exist in the world with respect to the operation of wireless microphones in the TV bands.  The WRAN systems shall be able to sense the presence of such microphone operation and avoid interference.

#### 15.1.1.3	Terrestrial mobile radio services description and technical characteristics

Licensed operation of this service is geographically based, hence the base stations would normally know whether to use the channel or not (channels 14-20 are off the table in the 11 main markets) based on their location.  There would be no need to sense by CPE’s, hence speeding up the process.  However, some public safety applications are also allowed to use the TV bands outside of these markets, e.g., some car racing safety crews are allowed to operate in the TV bands anywhere in the US).  [Steve Kuffner and Greg Buchwald to gather text on Part 90, succinct table of technical characteristics, ch 14-20, sensing may be quite difficult, therefore geographic use should be the way to share, developed at Motorola.]

#### 15.1.1.4        Medical telemetry

While indications are that >>80% of all medical telemetry systems have already migrated to CH37 and the remainder have been warned/advised for years to do likewise, it is recommended that base station operatorsnotify and/or coordinate with potentially affected facilties.

### 15.1.2	D/U Ratios for Television Broadcasting

The Desired to Undesired ratios (D/Us) necessary to preclude interference to TV broadcast services will depend on the WRAN waveform (i.e., ‘U’)  and the waveform of the TV broadcast service (i.e., ‘D’) .  As an initial step to determine the coexistence feasibility, it was assumed that the impact of the WRAN waveform on TV broadcasting will be similar to that of a DTV signal.  The DTV into DTV and DTV into analog TV D/Us for ATSC are provided below as a baseline for comparison.    The actual required D/U’s will need to be determined through measurements or simulations.  Proposals SHALL provide information on expected D/U ratios for interference into incumbent systems for their particular waveform(s).

### 15.1.3 Sensing Thresholds

The base station and the CPE’s SHALL sense licensed transmissions using an omni-directional antenna with a gain of 0 dBi or greater (where all losses between the antenna and the input to the receiver are included) in any azimuthal direction and polarization. The base station SHALL take appropriate actions to assure that the network and all of its components will behave in a manner that mitigates any interference that would result if licensed signals are detected above the following thresholds, referenced to the receiver input:

(1)	DTV threshold: -116 dBm (total ATSC DTV power in the 6 MHz channel)  This could be done for the ATSC DTV system by using spectrum analysis techniques to sense the pilot carrier of the DTV signal which is at –11.3 dB below the total DTV power.  Different threshold values may be needed to protect the various digital TV systems.

- Analog TV threshold: -94 dBm (measured at peak of sync of the NTSC picture carrier).  Different threshold values may be needed to protect the various analog TV systems.

- Wireless microphone threshold: -107 dBm (measured in 200 kHz bandwidth) for operating microphones.

Proposers SHALL provide detailed descriptions of how they will meet the requirements of this section, along with supporting analysis and, if appropriate, simulations.  Proposers are encouraged to propose solutions that will exceed the minimum requirements outlined herein and provide supporting analysis and/or simulations. (e.g. better sensing capabilities such as the ability to achieve lower thresholds, the flexibility to adapt sensing mechanisms to recognize various signal “signatures” as needed to function in an evolving environment and make such information available to the base station, etc.).

### 15.1.4 Response Time:

For the purpose of detecting a new DTV station, the response time is not likely to be critical  (e.g., a new broadcast station comes on over night).  The minimum rate of sensing for new TV broadcast  stations SHALL be one hour and the vacate time SHALL be no more than 30 minutes, allowing for multiple sensing to confirm presence of DTV operation.

[In the case where TV stations are not operating in a continuous mode, e.g., turned off during the night, much faster sensing will be needed to vacate the channel when the TV station comes on the air.   The minimum rate of sensing for such known stations SHALL be 5 minutes and the vacate time SHALL be no more than 1 minute and the base station should retain some knowledge of such a station’s usage of a channel.]

The following model based on Dynamic Frequency Selection outlines the requirements to avoid interference to wireless microphone operations from 802.22 services.

DFS Parameter Values:

| DFS Parameter | Value for Wireless Microphone |
| --- | --- |
| Channel Availability Check Time | 30 sec (recommended) |
| Non-Occupancy Period | 10 minutes (recommended) |
| Channel Detection Time | 500 msec – 2 sec to >=90% PD |
| Channel Setup Time | 2 sec |
| Channel Opening Transmission Time (Aggregate transmission time) | 100 msec |
| Channel Move Time (In-service monitoring) | 2 sec |
| Channel Closing Transmission Time (Aggregate transmission time) | 100 msec |
| Interference Detection Threshold | -107 dBm (200kHz BW) |

Table 11.1.5: DFS-like parameters for sensing and vacating channels used by wireless microphones

DFS Parameter Definitions:

Channel Availability Check Time: the time during which a TV channel SHALL be checked for the presence of a wireless microphone signal with a level above the Interference Detection Threshold prior to the commencement of WRAN operation in that channel.  Unlicensed device transmissions SHALL NOT occur during this time.

Non-Occupancy Period: the required period in which, once a TV channel has been recognized as containing a wireless microphone signal by an unlicensed device, the TV channel SHALL NOT be selected as an available channel.  Unlicensed device transmissions SHALL NOT occur during this time.

Channel Detection Time: the maximum time taken by the unlicensed device to detect a wireless microphone signal above the Interference Detection Threshold within the current TV channel.

Channel Setup Time: the time taken by an unlicensed device to transmit control information to other unlicensed devices in order to establish an available TV channel.

Channel Opening Transmission Time: the aggregate duration of control transmissions by the unlicensed devices during the Channel Setup Time, which starts at the end of the Channel Availability Check Time.  The aggregate duration of all transmissions SHALL NOT count quiet periods in between transmissions.

Channel Move Time: the time taken by an unlicensed device to cease all transmissions on the current TV channel upon detection of a wireless microphone above the Interference Detection Threshold.

Channel Closing Transmission Time: the aggregate duration of control transmissions by the unlicensed devices during the Channel Move Time, which starts upon detection of a wireless microphone above the Interference Detection Threshold.  The aggregate duration of all transmissions SHALL NOT count quiet periods in between transmissions.

In-Service Monitoring: a mechanism to check a TV channel in use by an unlicensed device for the presence of a wireless microphone signal above the Interference Detection Threshold.

Interference Detection Threshold: is the level to be used by the DFS function to detect wireless microphone signals

General operational behavior of unlicensed devices implementing DFS for wireless microphones is outlined below.

Base Station Devices

- All Base Stations operating in the TV bands SHALL use DFS to detect and avoid co-channel interference to wireless microphones for signals above the Interference Detection Threshold.

- The base station initiates an unlicensed network by transmitting short control signals during the Channel Setup Time that will enable CPEs to associate with the network.  The aggregate transmissions during the Channel Setup Time SHALL be limited to the Channel Opening Transmission Time.

- Before initiating a network on a channel, the Base Station SHALL perform a Channel Availability Check for the duration of the Channel Availability Check Time to ensure there are no wireless microphones operating on that channel.

- During normal operation, the Fixed/Access device SHALL continuously monitor the operating channel (In-Service Monitoring) in order to detect wireless microphone signals.  The amount of time the Fixed/Access device can take to detect a wireless microphone signal above the detection threshold is the Channel Detection Time.

- If the Fixed/Access device detects a wireless microphone signal, it SHALL stop transmissions on that channel and that channel is made unavailable for the Non-Occupancy Period. The Fixed/Access device SHALL instruct associated CPE devices to stop transmitting on this channel, which they SHALL do within the Channel Move Time.  The aggregate transmissions during the Channel Move Time SHALL be limited to the Channel Closing Transmission Time.

CPE Devices

- All CPE devices operating in the TV bands SHALL use DFS to detect and avoid co-channel interference to wireless microphones for signals above the Interference Detection Threshold.

- An unlicensed CPE device SHALL NOT transmit unless it receives a signal from a Fixed/Access device.

- A CPE device SHALL stop all transmissions whenever instructed by a Fixed/Access device.  The CPE device SHALL NOT resume transmissions until it has again received a signal from a Fixed/Access device.

- The amount of time the CPE device can take to detect a wireless microphone signal above the detection threshold is the Channel Detection Time. During normal operation, a CPE device SHALL continuously monitor the operating channel at intervals sufficient to meet the Channel Detection Time  in order to detect wireless microphone signals (In-Service Monitoring).

- If the CPE device detects a wireless microphone signal, it SHALL inform the Fixed/Access device and stop transmissions on that channel and that channel is made unavailable for the Non-Occupancy Period.

### 15.1.5	Distributed Sensing

802.22 systems shall employ distributed sensing. The base station and CPEs SHALL have the capability to sense the RF channel occupancy.  Information will be sent and collated at the base station where mapping of the interference situation will be made based on the sensing information from the CPE’s and according to their respective geographic coordinates.  Solutions to avoid interference will be deduced at the base station and measures will be signaled to the CPE’s such as channel change or power reduction.  The intelligence for the distributed sensing system may be centralized at a Network Operating Center (NOC) but the base station SHALL have the capability to meet the requirements related to the protection of incumbent services in the case where connectivity to the NOC is lost.  The MAC and PHY protocols SHALL allow the transmission of the necessary control packets in both directions to permit such distributed sensing and the CPEs SHALL react in a timely manner to commands from the base station.  The MAC SHALL support control packets to and from higher layers to assure protection of incumbent services at the network level.

### 15.1.6	Channel Utilization Table

The system SHALL provide a way to populate a table of channels that characterizes channel availability – such as disallowed, occupied,  available, etc., and to update that table at any time.

Disallowed channels SHALL not be used and therefore will not need to be sensed by the system.

Updating of the table MAY occur either as a result of data entry by a system operator or as an automatic function of  the network sensing mechanism for the other channels, which will result in non dis-allowed channels being designated as either occupied or available.

Such automatic updating by the sensing mechanism SHALL operate on a quasi-realtime basis with parameters as specified in section xxx.

For the available channels, conditions for operation SHOULD be included (e.g., TPC cap).  [?????]

The base station and optionally the CPEs SHOULD maintain a "list" of backup channel(s) in case an incumbent is detected and a channel has to be quickly vacated.  The exact scheme can be left to the proposals, as different approaches could be suggested to address this problem.

### 15.1.5	Minimum separation distances between WRAN base stations and DTV operation

Base stations MUST be located outside the Grade B contour of the DTV stations that use the same channel and first adjacent channels.

Minimum distances are indicated in Table x.

In the case of the second adjacent channels and beyond, the base station SHALL be located at specified minimum distances from the closest DTV receiver as indicated in Table x.

These distances are based on the assumption that the D/U ratios between WRAN transmission and DTV reception will be the same as DTV into DTV.  Some refinements to these distances may be needed once actual tests are conducted with the modulation selected for WRAN transmission.

| ATSC A-74 DTV RX Performance Guidelines | Free Space (m) | ITU-R P.1546 F(10,10) (m) |
| --- | --- | --- |
| N (continuous) OEC Bulletin 69 | 1,365,406 | 42,770 |
| N (impulsive) Para. 4.4.4 | 171,894 | 18,800 |
| N+/-1 OEC Bulletin 69 | 4,845 | 2,940 |
| N+/-2 Para. 4.4.3.1 | 196 | 196 |
| N+/-3 Para. 4.4.3.1 | 123 | 123 |
| N+/-4 Para. 4.4.3.1 | 78 | 78 |
| N+/-5 Para. 4.4.3.1 | 49 | 49 |
| N+/-6 to N+/-13 Para. 4.4.3.1 | 98 | 98 |
| N+/-14 and 15 Para. 4.4.3.1 | 44 | 44 |
| RF front-end overload Para. 4.2 | 31 | 31 |

Table x:  Minimum distance between the WRAN base station and the DTV Grade B contour in the case of co- and adjacent channel operation and distance to the closest DTV receiver for larger channel separation

### 15.1.6 Maximum power for WRAN CPE’s to avoid interference to DTV operation

The interference potential to a DTV receiver resulting from a transmission from a CPE  is analyzed under the following conditions:

- 4 Watt maximum CPE transmit EIRP;

- CPE antenna mounted 10 m above the ground;

- A separation distance of 10 m between the CPE and DTV receiving antennas;

- In the case of co-channel and first adjacent channel operation, both antennas are assumed to be looking away from each other since the base station has to be located at a certain distance outside the Grade B contour of the DTV station as indicated in the previous section.  The back-lobe rejection of both antennas can therefore be relied upon.

- Main beam coupling will exist between the antennas for channels N±2 and beyond inside the Grade B countour.

- For channels N±2 and beyond, polarization discrimination is the only way to increase isolation based on the fact that the CPE transmit antenna and the DTV receive antenna will be orthogonally polarized;

- The antenna polarization discrimination is assumed to be equal to the DTV antenna backlobe rejection.

- No signal depolarization is assumed between the two antennas

Table y:  Maximum permitted transmit power for the CPE’s assuming a minimum separation of 10 m between the CPE antenna and the closest DTV receiving antenna

| ATSC A-74 DTV RX Performance Guidelines |  | D/U at Grade B contour (-84 dBm) (dB) | Grade B field strength (uV/m) | CPE TX and DTV RX Antenna discrimination (dB) | Polarization discrimination (dB) | Min. dist. between CPE antenna and DTV RX antenna (m) | Maximum CPE transmit EIRP (dBW) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N (continuous) | OET Bulletin 69 | 23 | 41.0 | 30.0 | 0.0 | 10 | -66.8 |
| N (impulsive) | Para. 4.4.4 | 5 | 41.0 | 30.0 | 0.0 | 10 | -48.8 |
| N+/-1 | Para. 4.4.2 | -28/-26 | 41.0 | 30.0 | 0.0 | 10 | -15.8/-17.8 |
| N+/-2 | Para. 4.4.3.1 | tbd | 41.0 | 0.0 | 14.0 | 10 | tbd |
| N+/-3 Grade B | Para. 4.4.3.1 | tbd | 41.0 | 0.0 | 14.0 | 10 | tbd |
| N+/-4 | Para. 4.4.3.1 | tbd | 41.0 | 0.0 | 14.0 | 10 | tbd |
| N+/-5 | Para. 4.4.3.1 | tbd | 41.0 | 0.0 | 14.0 | 10 | tbd |
| N+/-6 to N+/-13 | Para. 4.4.3.1 | tbd | 41.0 | 0.0 | 14.0 | 10 | tbd |
| N+/-14 and 15 | Para. 4.4.3.1 | tbd | 41.0 | 0.0 | 14.0 | 10 | tbd |
| RF front-end overload | Para. 4.2 | -8.0 | -- | 0.0 | 14.0 | 10 | 16.1 |

The CPE SHALL operate according to an EIRP profile as depicted in Figure y as an example.  The system SHALL allow for modification of such EIRP profile and remote control from the base station such that, the allowable EIRP can be adjusted on a channel by channel basis.  The values for each channel will represent the clipping level of the TPC.  Actual values will be defined later.  So far the D/U values available for deducing this profile are contained in ATSC A-74 DTV Receiver Guideline for Strong, Moderate and Weak signal levels.  Further analysis and testing will be needed to come up with the D/U values for signal levels representative of reception at the noise-limited (Grade B) contour. (see Table qqq)

Figure y – CPE EIRP Profile

### 15.1.7	Out-of-band emission mask for WRAN systems

In order to protect DTV receivers and to protect wireless microphones, 4 W WRAN CPEs and base station devices SHALL meet the limits specified in Table zzz

|  | If WRAN operates |  |
| --- | --- | --- |
|  | First adjacent channel to [TV or] wireless microphone | Second adjacent channel and beyond to TV or wireless microphone |
| WRAN first adjacent channel limit | 4.8 uV/m | 200 uV/m |
| WRAN second adjacent channel and beyond limit | 4.8 uV/m | 4.8 uV/m |

Table zzz:  Emission levels (measured at 3 m in 120 kHz)

For a detailed treatment of the WRAN RF mask development, please consult doc. “22. 04.0002.04 .0000 WRAN Reference Model.xls”.

## 15.2	WRAN systems coexistence/sharing [highlighted sections need work]

Coexistence among WRAN systems.

More than one WRAN base stations in the same geographic area.  Will allow multiple 802.22 systems with service area overlap.  Sensing techniques should be operable in presence of other WRAN systems operating co-channel in the same channel.

Any license-exempt usage in the TV broadcast bands would need to use a beacon or a signature in its transmission signal that would include a transmitter identification.  This should be in the rule.  The identification of the transmitter would make resolving interference situations tractable.  802.22 will be the first group to define the sensing rules and the FCC need to monitor the work to develop the corresponding rules.

Coordination among 802.22 systems should be straightforward since all these systems will operate according to the same standard.. Specific address space for each WRAN operation would be needed. This is required in order to ensure that interference resolution is always possible:

- Should we use 48-bit standard IEEE addresses?

- Addresses could be either fixed or dynamic

### 15.2.1	Overview

Extending the beacon concept to help cooperation among LE systems. Encourage the FCC to look into this capability.  FCC will insist to know how it will be tested.  Every 30 sec.  ANSI C63.17 1910 to 1930. Unlicensed PCS shared.    Need Transmitter signature.

### 15.2.2	Sensing Thresholds and D/U Ratios

Other devices that could use beacons similar to those used for the wireless microphones would avoid interference but it would give them precedence over WRAN.  Need for a transmission signature to distinguish among DTV, wireless microphones, WRAN transmissions and other LE transmissions.

### 15.2.3	Response Time

### 15.2.4	Distributed Sensing

Since distributed sensing will exist for coexistence with incumbents, the same structure will be available for LE coexistence.

### 15.2.5	Channel utilization databases

## 15.3  Coexistence/sharing with other license-exempt systems

We are aware of the problem with other LE systems.  Discussing the evolutional of this section will depend on the FCC decisions.

If other LE systems don’t have RF signal profiles => problem.  Need to have a common interface for sensing to later cooperate.

Any license-exempt usage in the TV broadcast bands would need to use a beacon or a signature in its transmission signal that would include a transmitter identification.  This should be in the rule.  The identification of the transmitter would make resolving interference situations tractable.

Coordination with other LE systems will need to include conditions on the operating rules of these systems to allow resolution of interference.  Precedents exist for the 1.9 GHz LE PCS band where technical means of avoiding interference were defined in the ANSI standards.  These other LE systems should also have RF sensing and means to avoid interference.  There should be an industry group to work together to resolve the issue.

## 15.4	RF Sensing

  - measurements that must be done are part of PHY.  PHY has to respond to commands from MAC to do sensing and generate messages to the MAC on results (see section 10.2.1).

  - cognitive aspects interwoven between PHY and MAC

  - sensing needs requirement in PHY and MAC.

  - bound problem and define what the standard has to specify but not specify explicitly.

  - sensing on occupied channel and other channels.  sensing on channel you're planning to occupy.  primary channel sensing, in-service monitoring, during quiet periods figure out if channel is occupied;  how frequently (dynamic nature of the usage), how long to sense? (sensitivity level)  besides initial scanning when CPE turns on, sensing is controlled by base station.  eg.  scan next available and others less frequently.

  - System shall sense at the specified threshold levels (section 11.1.4) How it will be done and give indication on the duration of the quiet period.

  - legitimate wireless microphones, use beacon? don't want to have to protect illegal mikes.

  - define rate at which sensing would be done, integration time, once we know thresholds.  spectrum signature, know what DTV looks like, know what mikes look like, NTSC looks like, DVB-T has null in centre of channel, narrow null would define observation of this.  need to include foreign signatures in this portion of the document.

[System section: Need to indicate that RF levels reported to the base station are in absolute  levels (e.g. field strength, dBm, etc.).  The general frequency range (band 1, 2, 3, 4 and 5) for CPE should be the same as the range over which the CPE can operate … plus the adjacent channels

## 15.5	Dynamic Frequency Selection

When interference is to be avoided, the systems shall change (vacate, etc. see section 10.2.2.1) to another channel that was found to be available. … time frame: detect, close, move, re-establish connection.

# 16	Network Management

As outlined in IEEE Std 802-1990 [11], the LLC Sublayer, MAC Sublayer and Physical Layer standards also include a management component that specifies managed objects and aspects of the protocol machine that provide the management view of managed resources. The aspects of management considered are (FCAPS):

- Fault management

- Configuration management

- Accounting management

- Performance management

- Security (see also section 13)

The 802 standards define a framework for LAN/MAN management in ISO/IEC 15802-2:1995(E) [28]. The framework contains guidelines for managed objects, management protocol, and the relationship to ITU management protocols. 802.22 protocols SHALL comply with the above-mentioned standards and guidelines.

## 16.1	Support for Service Level Agreements

The 802.22 protocols MUST permit operators to enforce service level agreements (SLAs) with subscribers by restricting access to the air link, discarding data, dynamically controlling bandwidth available to a user or other appropriate means [29]. The 802.22 protocols MUST also permit subscribers to monitor performance service levels of the 802.22 services being provided at the delivery point.

## 16.2	Support for Admission Control

The 802.22 standard SHALL ?????  a flow by flow admission control to support  QoS.

## 16.3	Support for Accounting and Auditing

The 802.22 system management framework, architecture, protocols and managed objects MUST allow for operators to effectively administer accounting and auditing. An operator must be able to account for resource utilization and various service features for each subscriber service separately.

[The WRAN system protocols should allow control and management of the services on a subscriber basis as well as services provided to special CPE’s which bridge to a number of subscribers (i.e., multi-apartment building served by a ‘first class’ CPE.)]  Also recall from section 8.12 that a single CPE can interface to multiple subscribers that an operator could bill separately.

## 16.4	Base Station to Base Station Communications/Coordination

### 16.4.1	In Same Network

Should it be at layers 3 and above or it would need to be at layers 1 and 2 and thus need to be defined in this standard?

### 16.4.2	Between Co-located/Adjacent/Overlapping Networks

## 16.5	Timing, sync, etc.

## 16.6	Malfunctioning CPE or Base Station

The operator MUST have means to shut down and reactivate a CPE if necessary, remote from the CPE. The operator also MUST have the means to securely shut down and reactivate a base station remotely. When such capabilities are available, the 802.22 protocols SHALL support a secure function. The 802.22 protocols SHALL support functions that automatically shut down transmission at a CPE or base station in case of malfunction (e.g., power limits exceeded).

The WRAN equipment SHALL be tamper-proof to prevent unauthorized modification to firmware and/or functionalities, e.g., cognitive functionality, RF sensing, DFS, TPC, tuning).

# 17.0	Security

The 802.22 system SHALL enforce security procedures described in this section.

The security system chosen by 802.22 SHALL be added to the protocol stack and reference points to include security protocols, and “database” servers for authentication, authorization, key management, service suspend/resume, relocation, anti-cloning, etc. [30, 31].

## 17.1	Authentication for network access

There are two types of authentication for an 802.22 system. In the first type, a subscriber station MUST authenticate itself with the network every time it registers with the network. This authentication MUST prevent unauthorized subscriber station from entering the network or an unauthorized base station from emulating an authorized base station. This type of authentication MUST be supported by the 802.22 MAC layer.

The second type of authentication is between the subscriber and the WRAN system. This may or may not be the responsibility of the 802.22 protocols. It MAY be handled by higher layer protocols.

An additional level of authentication may exist between the other two. This additional layer is the authentication of the subscriber with the subscriber station. This is beyond the scope of the 802.22 protocols.

The authentication mechanisms MUST be secure so that an “enemy” subscriber station is not able to gain access to an 802.22 system, or to the core network beyond. Passwords and secrets MUST NOT be passed “in the clear” through the air interface.

## 17.2	Authorization (Access control)

Authorization is a security process that determines what services an authenticated subscriber is permitted to invoke. Each subscriber has a set of credentials that describe what the subscriber is “allowed” to do. The 802.22 standard SHALL identify a standard set of credentials and allow for vendors to extend the defined credentials with non-standard credentials. Some possible credentials are:

- Permission to access the 802.22 system

- Permission to access certain services (IP, Remote Bridging, Digital Audio/Video, etc.)

- Permission to request specific service features, and/or QoS parameters, based on the SLA (resources, delay, etc.).

For each service offered, the 802.22 protocols MUST be capable of securely supporting authorization requests and responses.

## 17.3	Privacy

Privacy is a security concept that protects transmitted data from being intercepted and understood by third parties (e.g., an “enemy” subscriber station, base station or passively  “listening” radio). Wire-equivalent privacy (WEP) [5] and shared private key [5] privacy have been suggested as minimum required privacy levels for 802.22 systems.

802.22 standard SHOULD allow a suitable cryptographic algorithm to be employed that is internationally applicable. Facilities SHOULD also be defined in the protocol for the use of alternate cryptographic algorithms that can be used in certain localities and that can replace algorithms as they are obsoleted or “legalized” for international use.

## 17.4	 Message integrity

## 17.5	Data (payload) encryption

## 17.6	Protection of network control information

## 17.7	Protection against Denial of Service and other attacks

## 17.8 	Security algorithm(s)

# 18	802 Conformance

802.22 SHALLconform to the 802 architecture. The following  particulars with the 802 model (see IEEE Standards for Local and Metropolitan Area Networks: Overview and Architecture (IEEE Std 802-1990) [11]) shall apply:

- The 802.22 MAC supports 802 “universal” 48 bit addresses. (? 64 bit addresses? Carl:Tony Jeffrey, Goff Thompson and Floyd Backes to review)

- An 802.22 system supports MAC multicast in the downstream direction.

- The 802.22 protocols support 802.1 bridging services and protocols, including support of the 802.1q virtual LAN tag and 802.1D priority ID [32-34].

- The 802.22 protocols support encapsulation of 802.2 (LLC) [10] by the MAC protocol.

- Conform to the 802 conventions and structures for “interface primitives:” logical structures that are passed between protocol layers to invoke processes and transact data.

- Address the 802 system management guidelines (see section 8) [35].

- Provide a MAC service interface that complies to 802 conventions [12].

# 19	References

[1]	IEEE Standards Association. Project Authorization Request. Telecommunications and Information Exchange Between Systems - LAN/MAN Specific Requirements - Air Interface for Fixed Wireless Regional Area Network Systems in Licensed Bands from 2 to 11 GHz. March 30, 2000. http://ieee802.org/16/sub11/par/index.html.

[2]	IEEE LAN/MAN Standards Committee (IEEE 802 LMSC) Sponsor Executive Committee. 802.22 (Wireless Regional Area Network Air Interface Standard In Licensed Bands from 2 to 11 GHz): Meting the Five Criteria. March 9, 2000. http://ieee802.org/16/sub11/par/fivec.html.

[3]	Recommendation ITU-R F.1399. Vocabulary of terms for wireless access.

[5]	Marianna Goldhammer, 802.22sc-99/16 MAC Services.

[6]	S. Blake et al, "An Architecture for Differentiated Services", RFC 2475, December, 1998.

[7]	S. Blake et al, "A Framework for Differentiated Services", Internet Draft, October, 1998.

[8]	R. Braden et al., "Integrated Services in the Internet Architecture: An Overview", RFC 1633, June 1994.

[9] George Fishel, 802.22sc-99/22 Interface to MAC and LLC Protocols.

[10]	ISO/IEC 8802-2:1998. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 2: Logical Link Control.

[11]	IEEE Std 802-1990 "IEEE Standards for Local and Metropolitan Area Networks: Overview and Architecture. IEEE 1990.

[12]	ISO/IEC 10039: 1991. Information technology -- Open Systems Interconnection -- Local area networks -MAC service definition.

[13]	ISO 7498-1:1984. Information technology -- Open Systems Interconnection -- Basic Reference Model.

[14]	James Mollenauer, 802.16sc-99/5 Functional Requirements for Wireless Regional Area Network Networks.

[15]	Imed Frigui, 802.16sc-99/23 Services and Performance requirements for Broadband Fixed Wireless Access.

[16] Robert Duhamel, 802.16sc-99/17 LMDS Cell Sizing and Availability.

[17]	Recommendation ITU-T G.826: Error performance parameters and objectives for international, constant bit rate digital paths at or above the primary rate (2/99).

[18]	Recommendation ITU-R F.1491. Error performance objectives for real digital radio links used in the national portion of a 27 500 km hypothetical reference path at or above the primary rate.

[19]	C.W. Lundgren and W.D. Rummler, "Digital radio outage due to selective fading observation vs. prediction from laboratory simulation," Bell System Technical Journal, pp.1073-1100, May-June 1979.

[20]	M. Emshwiller, "Characterization on the performance of PSK digital radio transmission in the presence of multipath fading," ICC'78 Conference Record, Toronto, Ontario, CANADA, Paper 47.3.

[21]	Microwave digital radio systems criteria, Bellcore Technical Reference TR-TSY-000752, October 1989.

[22]	W.D. Rummler, R.P. Coutts, and M. Linger, "Multipath fading channel models for microwave digital radio," IEEE Communications Magazine, November 1986, pp.30-42.

[23]	R. K. Crane, "Prediction of Attenuation by Rain" IEEE, 1980.

[24]	Recommendation ITU-R F.1189-1. Error performance objectives for constant bit rate digital paths at or above the primary rate carried by digital radio-relay systems which may form part or all of the national portion of a 27500 km hypothetical reference path. (1995-1997).

[25]	CCIR Recommendation 749, Radio-Frequency channels arrangements for radio-relay systems operating in the 36.0 to 40.5 GHz band. (1992).

[26]	John Liebetreu, 802.22sc-99/13 Dispersive Fade Margin: A Physical Layer Performance Metric.

[27]	RFC-2205 Resource ReSerVation Protocol (RSVP) -- Version 1 Functional Specification. R. Braden, Ed., L. Zhang, S. Berson, S. Herzog, S. Jamin. September 1997. Status: PROPOSED STANDARD.

[28]	ISO/IEC 15802-2:1995. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 2: LAN/MAN management.

[29]	Jim Mollenauer, 802.22sc-99/7 Functional Requirements for the 802.22 Standard.

[30]	IEEE 802.10-1998. IEEE Standards for Local and Metropolitan Area Networks: Standard for Interoperable LAN/MAN Security (SILS).

[31]	IEEE 802.10c-1998. IEEE Standards for Local and Metropolitan Area Networks: Supplement to Standard for Interoperable LAN/MAN Security (SILS) -- Key Management (Clause 3).

[32]	ISO 15802-3:1998. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 3: Media Access Control (MAC) bridging.

[33]	ISO/IEC 15802-5:1998. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 5: Remote Media Access Control (MAC) bridging.

[34]	IEEE 802.1Q-1998. IEEE Standards for Local and Metropolitan Area Networks: Virtual Bridged Local Area Networks.

[35]	IEEE 802.1F-1993. IEEE Standards for Local and Metropolitan Area Networks: Common Definitions and Procedures for IEEE 802 Management Information.

# Appendix A
Requirements Summary

# NEEDS REWORK for Renumbering(Informative)

This section contains tabular summaries or requirements found in the text of this document. Requirements are separated into three categories: required, recommended and optional.

Each requirement is numbered for easy reference. Future revisions of this document will keep the requirement reference numbers intact such that the number for a requirement will not change from revision to revision.

To better discern the meaning and intent of a requirement, please refer to the text.

## A.1	Mandatory

It is mandatory that the 802.22 standard support or specify the items in Table 2.

Table 2: Mandatory Requirements

| # | Section | Requirement |
| --- | --- | --- |
| M1 M2 | 1 1.1 | The forthcoming air interface standard MUST comply with the system requirements. The 802.22 air interface interoperability standard SHALL be part of a family of standards for local, metropolitan and wide area networks. |
| M3 | 2 | 802.22 systems SHALL be deployable in multiple-cell frequency reuse systems and single cell (super cell) frequency reuse systems. |
| M4 | 2.3 | Since all data traffic in a single cell of an 802.22 network MUST go through the base station, that station SHALL serve as a radio resource supervisor. |
| M5 | 2 | The base station radio SHALL be P-MP, radiating its downstream signal with a shaped sector or adaptive array (spatial reuse) antenna achieving broad azimuthal beam width to “cover” a prospective number of subscribers. |
| M6 M7 | 2.1 2.2 | The standard SHALL specify MAC layer protocols and PHY transmission techniques suitable for providing access between one or more subscriber stations and base stations to support UNI and CNI requirements. The 802.22 protocols SHALL support the optional deployment of repeater functions. |
| M8 | 2.2 | The repeater function SHALL NOT affect the MAC protocol other than the delay which the repeater might introduce into the system. |
| M9 M10 | 2.3 3 | In the upstream direction, 802.22 protocols MUST provide the means to multiplex traffic from multiple subscriber stations, resolve contention, and allocate capacity. The MAC and PHY protocols will not have explicit support for each and every service, due to the fact that generic data streams SHALL be used for transport. |
| M11 | 3 | The MAC and PHY protocols SHALL provide for QoS service specific support, resulting in appropriate BER for data services, limited delay for real |
| M12 | 3.1 | time services, etc. 802.22 systems SHALL support voice communications for subscribers in a way that eases the migration of legacy voice communications equipment and public switched telephone network (PSTN) access technologies to 802.22 systems. |
| M13 M14 | 3.1 3.2 | The 802.22 voice access transport SHALL be packet based (as opposed to circuit-switched based). The 802.22 system MUST directly transport variable-length IP datagrams efficiently. |
| M15 | 3.2 | Both IP versions 4 and 6 MUST be supported. |
| M16 M17 | 3.2 3.4 | The 802.22 IP service MUST provide support for real-time and non-real time service capabilities. These services SHALL NOT place any additional requirements on 802.22 systems (MAC and PHY protocols) not already covered in the above sections. |
| M18 | 4 | The IEEE 802.22 MAC and PHY protocol stacks SHALL be the same for all the supported services. |
| M19 M20 | 4 4 | Further details, and finalization of the protocol reference model, SHALL be worked out by the 802.22 MAC and PHY task groups while developing the air interface interoperability standard. Since key layers above the MAC require service guarantees, the MAC protocol MUST define interfaces and procedures to provide guaranteed service to the upper layers. |
| M21 M22 | 4 5.2 | Since customer units will contend for capacity to/from one or more base stations, the MAC protocol MUST efficiently resolve contention and resource allocation. 802.22 protocols SHALL be optimized to support the peak data rate in either or both directions to a subscriber station within the specified distance from the base station. |
| M23 M24 | 5.4 5.6 | The PHY and MAC protocols SHALL provide for multirate support. The 802.22 specifications SHALL NOT preclude the ability of the radio link to be engineered for different link availabilities, based on the preference of the system operator. |
| M25 M26 M27 | 5.6 5.6 5.7 | 802.22 MAC and PHY protocols MUST accommodate these conditions, perhaps consuming more radio bandwidth and/or requiring smaller radio propagation distance (radius) to meet the availability requirements. Since statistical atmospheric and path conditions vary widely in geography, the 802.22 protocols MUST be flexible in consumed radio bandwidth (spectral efficiency), cell radius, and transmit power. The error rate, after application of the appropriate error correction mechanism (e.g., FEC), delivered by the PHY layer to the MAC layer SHALL meet IEEE 802 functional requirements with the following exception: the radio link bit error ratio (BER) SHALL be 10E-6 (in accordance with ITU FWA |

| M28 | 5.9 | recommendations or better. The standard SHALL support careful planning to ensure that subscribers’ |
| --- | --- | --- |
| M29 | 5.9 | quality of service guarantees and minimum error rates are met. The delivered base station capacity can vary depending on attenuation due to |
| M30 | 5.9 | atmospheric conditions, LOS blockage, transmit power, etc., and SHALL be calculated as the aggregate capacity of all sectors supported by a base station. The MAC and PHY protocols MUST accommodate channel capacity issues |
|  |  | and changes in channel capacity to meet contracted service levels with customers. |
| M31 | 5.9 | As subscribers are added to 802.22 systems, the protocols MUST |
|  |  | accommodate them in an automated fashion. |
| M32 | 6.1 | This standard SHALL permit two duplex modes of operation: Frequency |
| M33 | 6.1 | Division Duplex (FDD) and Time Division Duplex (TDD). The PHY and MAC protocols MUST provide for duplex (i.e. bi-directional) |
| M34 | 6.1 | operation, while preserving the QoS, BER and spectral efficiency requirements for data and voice traffic. The MAC and PHY protocols MUST provide means to resolve applicable |
|  |  | collocation and interference problems. |
| M35 | 6.2 | The MAC and PHY protocols MUST permit the operation with channel |
| M36 | 7 | spacing of 1.75, 3.5 and 7MHz when using ETSI masks and 1.5 to 25MHz when using other masks. Table 1 provides a summary of the QoS requirements that the PHY and MAC |
|  |  | SHALL provide. |
| M37 | 7 | 802.22 protocols MUST support classes of service (CoS) with various |
| M38 | 7 | quality of service (QoS) guarantees to support the services that an 802.22 system MUST transport. Thus, 802.22 protocol standard MUST define interfaces and procedures that |
|  |  | accommodate the needs of the services with respect to allocation and prioritization of resources. |
| M39 | 7 | 802.22 protocols MUST provide the means to enforce QoS contracts and |
| M40 | 7 | Service Level Agreements. For QoS-based, connectionless services, the 802.22 protocols MUST support resource negotiation “on-demand” |
| M41 | 7 | If 802.22 is to be a “link” in the IP network, an IWF MUST interface with |
| M42 | 7.2 | 802.22 to negotiate resource allocation. 802.22 protocols SHALL define a set of parameters that preserve the intent |
| M43 | 7.3 | of QoS parameters for IP-based services. The classes of service and QoS parameters of services SHALL be translated |
|  |  | into a common set of parameters defined by 802.22. |
| M44 | 8.1 | The 802.22 protocols MUST permit operators to enforce service level |
|  |  | agreements (SLAs) with subscribers by restricting access to the air link, discarding data, dynamically controlling bandwidth available to a user or other |

| M45 | 8.1 | appropriate means. The 802.22 protocols MUST also permit subscribers to monitor performance |
| --- | --- | --- |
| M46 | 8.2 | service levels of the 802.22 services being provided at the delivery point. When such capabilities are available, the 802.22 protocols SHALL support a |
| M47 | 8.2 | secure function. The operator MUST have means to shut down and reactivate a subscriber |
|  |  | station if necessary, remote from the subscriber station, in the face of a malfunction.8.2 |
| M48 | 8.3 | The 802.22 system management framework, architecture, protocols and |
| M49 | 8.3 | managed objects MUST allow for operators to effectively administer accounting and auditing. An operator MUST be able to account for resource utilization and various |
|  |  | service features for each subscriber service separately. |
| M50 | 9 | The 802.22 system SHALL enforce security procedures described in this |
| M51 | 9 | section. The security system chosen by 802.22 SHALL be added to the protocol stack |
| M52 | 9.1 | and reference points to include security protocols, and “database” servers for authentication, authorization, key management, service suspend/resume, relocation, anti-cloning, etc. There are two types of authentication for an 802.22 system. In the first type, |
|  |  | a subscriber station MUST authenticate itself with the network every time it registers with the network. This authentication MUST prevent unauthorized subscriber station from entering the network or an unauthorized base station from emulating an authorized base station. This type of authentication MUST be supported by the 802.22 MAC layer. |
| M53 | 9.1 | The authentication mechanisms MUST be secure so that an “enemy” |
| M54 | 9.1 | subscriber station is not able to gain access to an 802.22 system, or to the core network beyond. Passwords and secrets MUST NOT be passed “in the clear” through the air |
| M55 | 9.2 | interface. The 802.22 standard SHALL identify a standard set of credentials and allow |
| M56 | 9.2 | for vendors to extend the defined credentials with non-standard credentials. For each service offered, the 802.22 protocols MUST be capable of securely |
|  |  | supporting authorization requests and responses. |

## A.2	Recommended (R)

- It is recommended that the 802.22 standard support or specify the items in Table 3 “Recommended” means that there may exist valid reasons in particular circumstances to ignore an item, but the full implications should be understood and the case carefully weighed before choosing a different course.

Table 3: Recommended Requirements

| # | Section | Requirement |
| --- | --- | --- |
| R1 | 1.1 | Other goals of this document are to formulate reference models and terminology for both network topology and protocol stacks that help the 802.22 working group to discuss and develop the MAC and PHY protocols. As far as possible, these SHOULD be common across 802.22 systems. |
| R2 | 2.2 | The repeater function SHOULD NOT affect the end-to-end operation of 802.22 protocols between BS and SS. |
| R3 R4 | 3.2 3.2 | For efficient transport of IPv6, TCP/IP header compression over the air interface SHOULD be supported. It SHOULD be possible to support the emerging IP Quality of Service (QoS) efforts: Differentiated Services and Integrated Services. |
| R5 | 3.3 | The 802.22 protocols SHOULD support bridged LAN service capabilities, whether directly or indirectly, including always on, ad hoc and on-demand communication in either or both directions. |
| R6 R7 | 4 5.1 | Note that the function of the MAC protocols SHOULD include error correction by retransmission, or Automatic Repeat Request (ARQ), whereas, in the 802 model, those functions if necessary, are provided by the LLC layer. The 802.22 protocols SHOULD allow for different “scales” of capacity and performance for 802.22 system instances. |
| R8 | 5.2 | The 802.22 MAC protocol SHOULD allow the peak data rate to scale beyond 10 Mbps. |
| R9 R10 | 5.3 5.5 | The PHY and MAC protocol SHOULD provide for far CPEs’ propagation delay compensation. 802.22 protocols SHOULD allow for flexibility between delivered upstream and downstream capacity and CoS/QoS. |
| R11 R12 | 5.6 5.6 | An 802.22 system SHOULD be available to transport all services at better than their required maximum error rates (see section 5.7) from about 99.9 to 99.99% of the time, assuming that the system and radios receive adequate mains power 100% of the time and not counting equipment availability. 802.22 MAC and PHY protocols SHOULD specify functions and procedures to adjust transmitter power, modulation, or other parameters to accommodate rapid changes in channel characteristics. |
| R13 | 5.9 | Given the propagation characteristics in a given frequency band and geographic area, and the development of a link budget, the following parameters of an 802.22 system SHOULD be addressed by the MAC and PHY protocols: ~ Radio range (up to 50 Km) ~ Upstream/downstream channels’ data rates ~ Allocation of prospective subscriber data rate to channels. Note: the MAC and PHY standard MAY allow subscribers to hop between channels ~ Types of modulation |

| R14 | 6.2 | The typical value [channel spacing] for performance analysis SHOULD be 3.5MHz for the ETSI mask and 3MHz for the MDS mask. |
| --- | --- | --- |
| R15 | 7.3 | 802.22 protocols SHOULD include a mechanism that can support dynamically-variable-bandwidth channels and paths (such as those defined for IP environments). |
| R16 | 8 | The 802 standards define a framework for LAN/MAN management in ISO/IEC 15802-2: 1995(E). The framework contains guidelines for managed objects, management protocol, and the relationship to ITU management protocols. 802.22 protocols SHOULD comply with the above-mentioned standards and guidelines. |
| R17 R18 R19 | 8.2 8.2 9.3 | The operator also SHOULD have the means to securely shut down and reactivate a base station remotely. The 802.22 protocols SHOULD support functions that automatically shuts down transmission from a subscriber station or base station in case of malfunction (e.g., power limits exceeded). 802.22 standard SHOULD allow a suitable cryptographic algorithm to be employed that is internationally applicable. |
| R20 | 9.3 | Facilities SHOULD also be defined in the protocol for the use of alternate cryptographic algorithms that can be used in certain localities and that can replace algorithms as they are obsoleted or “legalized” for international use. |
| R21 | 10 | 802.22 SHOULD strive to fit into the 802 system model. |

## A.3	Optional (O)

It is optional that the 802.22 standard support or specify the items in Table 4.

Table 4: Optional Requirements

| # | Section | Requirement |
| --- | --- | --- |
| O1 | 2.1 | The model [Figure 2-2: Wireless Access Reference Model] depicts the relevant |
| O2 | 2.1 | points between subscriber networks and “core” networks (the networks that MAY be accessed via 802.22 air interface). A single SS MAY support multiple customer premises networks that transport |
| O3 | 2.1 | data, voice and video through one or more UNIs. Base stations MAY support multiple core networks through one or more CNIs. |
| O4 | 5.9 | The MAC and PHY standard MAY allow subscribers to hop between |
| O5 | 5.9 | channels. Flexible modulation types, power level adjustment, and bandwidth reservation |
| O6 | 7 | schemes MAY be employed [which affect how the MAC and PHY protocols meet contracted service levels with customers]. [To support on-demand resource allocation,] The MAC protocol MAY allocate |
|  |  | bursts of PDUs to services that require changes in resource allocation. |
| O7 | 7 | The 802.22 MAC layer interface MAY provide a connection-less service |
|  |  | interface that requires a higher-layer “adaptation” to maintain the “state” of a connection and periodically allocate resources. |
| O8 | 9.1 | The second type of authentication is between the subscriber and the WRAN |
|  |  | system. This may or may not be the responsibility of the 802.22 protocols. It MAY be handled by higher layer protocols. |

# Appendix B
Vocabulary of Terms (Normative)

Editor’s Note: Some definitions below are as yet undefined. And a few other are as yet incomplete or in-progress. The 802.22 task group should provide updates for these definitions and release a new version of this document. The in-progress definitions are in italics.

| Term | Definition | Reference |
| --- | --- | --- |
| Access | End-user connection(s) to core networks NOTE 1 - Core networks include, for example, PSTN, ISDN, PLMN, PSDN, Internet, WAN/LAN, CATV, etc. NOTE 2 - The end-user may be a single user or a user accessing the services on behalf of multiple users. | Based on Rec. ITU-R F.1399 |
| Accounting | A function which apportions the revenue obtained by the service providers to network operators in line with commercial arrangements. | Rec. ITU-R M.1224 |
| Ad hoc communication | Use of the wireless access system without reservation or QoS guarantees at sporadic times, by activating the link when needed. |  |
| Air interface | The common boundary between the subscriber station and the radio equipment in the network, defined by functional characteristics, common radio (physical) interconnection characteristics, and other characteristics, as appropriate. NOTE 1 – An interface standard specifies the bi directional interconnection between both sides of the interface at once. The specification includes the type, quantity and function of the interconnecting means and the type, form and sequencing order of the signals to be interchanged by those means. | Based on Rec. ITU-R M.1224 |
| Always on communication | Use of the wireless access system at any time by having the link continuously available. |  |
| Authentication | The process of verifying the identity of a user, terminal, or service provider. | Rec. ITU-R M.1224 |
| Authorization | A property by which the rights to resources are established and enforced. | Rec. ITU-R M.1224 |
| Backhaul service | Transport of aggregate communication signals from base stations to the core network. | IEEE 802.22 |
| Bandwidth; communication channel bandwidth | The information payload capacity of a communication channel available to a user for services (expressed in bit/s or multiples thereof). |  |
| Bandwidth; RF channel bandwidth | The frequency extent of a specified portion of the RF spectrum capable of carrying information over the radio interface (expressed in Hz or multiples thereof). |  |
| Bandwidth; transmission channel bandwidth | The frequency extent required for the transmission of a specified signal (expressed in Hz or multiples thereof). |  |
| Base station | The common name for all the radio equipment located at one and the same place used for serving one or several cells. (See also “station”). | ITU-R Rec. M.1224 |
| Bearer service | A type of telecommunication service that provides the capability for the transmission of signals between user-network interfaces. | ITU-T Rec. I.112 |
| Broadband wireless access | wireless access in which the connection(s) capabilities are higher than the primary rate. | Rec. ITU-R F.1399 |
| Cell | The radio coverage area of a base station, or of a subsystem (e.g. sector antenna) of that base station corresponding to a specific logical identification on the radio path, whichever is smaller. | Based on Rec. ITU-R M.1224 |
| Cell | A block of fixed length which is identified by a label at the asynchronous transfer mode layer of the B-ISDN protocol reference model. | ITU-T Rec. I.113 |
| Cell delay variation | A component of cell transfer delay, induced by buffering and cell scheduling. The proportion of lost cells over the total number of transmitted cells for a connection. | ATM Forum ATM Forum |
| Channel; communication channel | A specific portion of the information payload capacity, available to the user for services. | ITU-T Rec. I.113 |
| Channel; radio- frequency (RF) channel | A specified portion of the RF spectrum with a defined bandwidth and a carrier frequency and is capable of carrying information over the radio interface. | Rec. ITU-R M.1224 |
| Channel; transmission channel Class of Service Core network Customer premises | A means of unidirectional transmission of signals between two points. This is either a Customer Class of Service, a Trunk Class of Service, or a Private-Facility Class of Service. It may refer to either originating or terminating accesses. Core networks include, for example, PSTN, ISDN, PLMN, PSDN, Internet, WAN/LAN, CATV, etc. The equipment/network administered by the user. | ITU-T Rec. I.112 ITU-T Rec. Q.1290 (95), 2 Based on Rec. ITU-R F.1399 Based on ITU-T Rec. |

| equipment/network |  | H.310 |
| --- | --- | --- |
| Downstream | The direction from base station to subscriber station(s). | IEEE 802.22 |
| Dynamically variable bandwidth | A capability of a system to be able to change the bandwidth of the information payload capacity of a communication channel available to a user for services according to negotiated user requirements. |  |
| External Network | Any network that is physically located past the base station or the subscriber station. |  |
| Fixed wireless access | Wireless access application in which the base station and the subscriber station are fixed. | Based on Rec. ITU-R F.1399 |
| Frequency Division Duplex | Separation of upstream and downstream transmission in the frequency domain at the same time. | IEEE 802.22 |
| Internet protocol | Networking protocol defined by IETF standards. | IETF |
| Interoperability | The ability of multiple entities in different networks or systems to operate together without the need for additional conversion or mapping of states and protocols. | Rec. ITU-R M.1124 |
| Inter-working | The means of supporting communications interactions between entities in different networks or systems. | Rec. ITU-R M.1124 |
| Inter-working function | Mechanism which masks the differences in physical, link, and network technologies by converting or mapping states and protocols into consistent network and user services. | Rec. ITU-R M.1124 |
| Link Budget | The allocation of power within a link. Link budget consists of the difference between Tx power and receiver sensitivity. |  |
| Line-of-sight (LOS) | A radio communication link for which 60% or greater of the first Fresnel zone is unobstructed. (i.e., the geographical region where the rules of free space propagation applies). Two points for which the signal suffers temporal fading of less than 10 dB 99% of the time. Note: This definition does not imply that a direct optical line of sight exists. |  |
| Modulation Factor | The sector’s aggregate payload bit rate divided by the bandwidth. |  |
| Multi-tenant | A primarily non-commercial building designed to |  |

| dwelling | contain more than two residential units. |  |
| --- | --- | --- |
| Near LOS | A radio communication link for which 40% or greater of the first Fresnel zone is obstructed. Two points for which the signal suffers temporal fading greater than or equal to 10 dB exceeding 1% of the time and an average delay spread of less than 10 microseconds. |  |
| Non LOS | A radio communication link that is obstructed. Two points for which the signal suffers an average delay spread of greater than or equal to 10 microseconds. |  |
| Network Nomadic wireless access | A set of nodes and links that provides connections between two or more defined points to facilitate telecommunication between them. Wireless access application in which the subscriber station may be in different places but must be stationary while in use. | Rec. ITU-R M.1224 Based on ITU-R Rec. F.1399 |
| On demand communication | Use of the wireless access system for services based on negotiated QoS. |  |
| Peak Data Rate | The maximum bit rate available to a user for communications. |  |
| plesiochronous mode Point-to-multipoint system | A mode where the essential characteristic of time scales or signals such that their corresponding significant instants occur at nominally the same rate, any variation in rate being constrained within specified limits. a system that establishes connections between a single specified point and more than one other specified points. | ITU-T Rec. G.810 (96), 4.3.5 ITU-R Rec. F.1399 |
| Privacy Quality of service | The provision of capabilities to prevent access of information by unauthorized parties. The collective effect of service performance which determine the degree of satisfaction of a user of the service. NOTE 1 - The quality of service is characterized by the combined aspects of service support performance, service operability performance, serviceability performance, service security performance and other factors specific to each service. NOTE 2 - The term "quality of service" is not used to express a degree of excellence in a comparative sense nor is it used in a quantitative sense for | ANSI T1.702-1995 ITU-T Rec. E.800 (94), 2101 |

|  | technical evaluations. In these cases a qualifying adjective (modifier) should be used. |  |
| --- | --- | --- |
| Radio interface Real-Time (adjective) | See air interface Pertaining to the processing or transmission of data according to defined time requirements . | Rec. ITU-R M.1224 Based on ITU-T Rec. Q.9 (88), 6103 |
| Re-use Factor; Frequency re-use factor | As yet undefined |  |
| Sector | As yet undefined |  |
| Sector Bandwidth Factor | As yet undefined |  |
| Security | The protection of information availability, integrity and confidentiality, as well as authentication and authorization. | Based on Rec. ITU-R M.1224 |
| Semi-stateless | As yet undefined | Sect 7 |
| Service Service Capability | A set of functions offered to a user by an organization. Defines possible service arrangements with respect to the number and type of channels that can be supported by the equipment. In this way a subscriber station access capability is the set of possible channel configurations supported by the equipment. Similarly, the base station access capability may be considered to be a super-set with respect to the subscriber station. | Recs. ITU-R M.1224, M.1308 Based on ITU-T Rec. Q.1063 (88), 2.4 |
| Single Residential | A non-commercial building intended to be inhabited by an individual. |  |
| Small Business | As yet undefined |  |
| Small Office/Home Office Spectrum (Spectral) Efficiency | As yet undefined Measures of the accuracy and completeness of system tasks relative to the spectral resources used to achieve the specific system tasks | Based on ITU-T Rec. F.901 (93), 2 |
| Standard | A document established by consensus and approved by a recognized body, that provides, for common and repeated use, rules, guideline or characteristics for activities or their results, aimed at the achievement of the optimum degree of order in a given context. | ISO/IEC Guide 2, 1996 |
| Station | the common name for all the radio equipment at one and the same place. NOTE - The term "station" may refer to any end | Rec. ITU-R M.1224 |

|  | user radio equipment (“subscriber station”) or network radio equipment (“base station”). |  |
| --- | --- | --- |
| Subscriber Subscriber station Supplementary service Synchronous transfer mode | A person or other entity that has a contractual relationship with a service provider on behalf of one or more users. (A subscriber is responsible for the payment of charges due to that service provider.) the common name for all the radio equipment at one and the same place serving one or more users. (See also “station”). A service which modifies or supplements a basic telecommunication service. Consequently, it can not be offered to a customer as a standalone service, rather, it must be offered together with or in association with a basic telecommunication service. The same supplementary service may be common to a number of telecommunication services. A transfer mode which offers periodically to each connection a fixed-length block. | Rec. ITU-R M.1224 Based on Rec. ITU-R M.1224 Rec. ITU-R M.1224 Based on ITU-T Rec. I.113 |
| System | A regularly interacting or interdependent group of items forming a unified whole technology. | Recs. ITU-R M.1224, M.1308 |
| Telecommuter | As yet undefined |  |
| Time Division Duplex | Separation of upstream and downstream transmission in the time domain using the same frequency. | IEEE 802.22 |
| Upstream | The direction from subscriber station(s) to base station. | IEEE 802.22 |
| User | Any entity external to the network which utilizes connections through the network for communication. | ITU-T Rec. E.600 |
| Virtual point-to- point connections | Providing a point-to-point connection to a subscriber using a point to multipoint system. | IEEE 802.22 |
| Wireless access | end-user radio connection(s) to core networks. NOTE 1 - Core networks include, for example, PSTN, ISDN, PLMN, PSDN, Internet, WAN/LAN, CATV, etc. NOTE 2 - The end-user may be a single user or a user accessing the services on behalf of multiple users. | Rec. ITU-R F.1399 |

# Appendix C
Acronyms and Abbreviations

| Acronym/Abbreviation | Definition |
| --- | --- |
| AF | Assured Forwarding |
| ANSI | American National Standards Institute |
| ARQ | Automatic repeat Request |
| BER | Bit Error Ratio |
| BES | Best Effort Service |
| BS | Base Station |
| WRAN | Wireless Regional Area Network |
| CNI | Core Network Interface |
| CoS | Class of Service |
| CSMA/CD | Carrier Sense multiple Access with Collision Detection |
| DFM | Dispersive Fade Margin |
| DSL | Digital Subscriber Line |
| EF | Expedited Forwarding |
| ER | Errored Block |
| ETSI | European Telecommunications Standard Institute |
| FCAPS | Fault, Configuration, Accounting, Performance, Security Management |
| FDD | Frequency Division Duplex |
| FEC | Forward Error Correction |
| FWA | Fixed Wireless Access |
| HFC | Hybrid fiber coax |
| HTTP | Hypertext Transfer Protocol |
| IEC | International Electrotechnical Commission |
| IETF | Internet Engineering Task Force |
| IP | Internet protocol |
| ISDN | Integrated Services Digital Network |
| ISO | International Organization for Standardization |
| ITU-R | International Telecommunications Union- Radiocommunications Sector |
| IWF | Inter-working function |
| JRG | Joint Rapporteur group |
| LAN | Local area network |
| LLC | Logical link control |
| LOS | Line of Sight |
| MAC | Medium Access Control |
| MAN | Metropolitan area network |
| MGMT | Management |
| OSI | Open Systems Interconnection |
| PDU | Protocol Data Unit |
| PHY | Physical layer |
| PMD | Physical Medium Dependent |
| P-MP | Point-to-multipoint |
| PSTN | Public Switched Telephone Network |
| QoS | Quality of service |
| RF | Repeater Function |
| SDE | Secure Data Exchange |
| SEC | Security |
| SES | Severely Errored Second |
| SLA | Service Level Agreement |
| SME | Small and Medium Enterprises |
| SNMP | Simple Network Management Protocol |
| SOHO | Small Office/Home Office |
| SS | Subscriber Station |
| TC | Transmission convergence |
| TCP/IP | Transmission Control Protocol/Internet Protocol |
| TDD | Time Division Duplex |
| UNI | User Network interface |
| VoIP | Voice over IP |
| WEP | Wire-Equivalent Privacy |

# Appendix D
Channel Models Eli  (Gerald, Victor)

- channel models

  - 802.16 had lower limit of 2GHz, need to look

  - 802.16 doc (80216a-03_01_potential_channel_model.pdf) is a good basis for developing this Appendix.  It deals with all physical aspects of the channel.  It will need to be adjusted to the VHF and UHF bands.  It will need to include the consideration of industrial (man-made) noise that is not negligeable in these lower frequency bands.  Broadcast models developed for DTV for DTVwill need to be considered.  Channel delay spread impulse response models developed for mobile telephony such as the European model COST 207 would be useful to characterize multipath to be considered in the development of the modulation for 802.22.

  - COST 207 [1989]  Digital land mobile radio communications.  Final Report of COST Project 207, Commission of the European Communities, Brussels.

  - presence of on-channel repeaters if it is include in the 802.22 standard will change the range of possible multipath excess delays because of the presence of passive as well as active echoes will need to be considered.  Passive channel multipath range is around 25usec for echoes with sizable power but active echoes will extend it greatly.  DTV receiver equalizers typically cover a range of -20 to +40 usec.

  - ATSC-A74 model, Brazilian model

Impulse response, Industrial noise. (Model of 802.16: 500 MHz and up)

# Appendix E
Propagation Model

# 1- ITU-R P1546-1

  - ITU model (Rec. 370 and later Rec 1546) uses FCC F-curves updated and augmented.

  - Other model using real digital terrain if feasible and agreed

# 2- Specific sites with real terrains at these sites (Carl and Gregg)

# Appendix I

# Wireless Microphone Parameters

Wireless microphones are classed as Low Power Auxiliary Stations (LPAS) and operate in the VHF and UHF television bands as licensed secondary users under Section 74 of the FCC Rules in the United States.

| Wireless Microphone Parameter | Value |
| --- | --- |
| Operating Frequency | 54-72, 76-88, 174-216, 470-488, 488-494 (except Hawaii), 494-608, and 614-806 MHz |
| Output Power Limit (Conducted) | 54-72, 76-88, and 174-216 MHz: 50 mW; 470-608 and 614-806 MHz: 250 mW. |
| Occupied Bandwidth Limit | 200 kHz |
| Modulation | Any, subject to Occupied Bandwidth Limit. Maximum Deviation of 75 kHz if FM is used. |

The above information has been extracted from Section 74.801 – 74.861 of the FCC Rules.

Typical wireless microphones are handheld and body-worn and operate with antenna gains of less than 0dBi.  Due to shadowing and body attenuation effects, the resulting effective radiated power (ERP) is less than the maximum power limits allowed in Part 74 of the FCC rules.

The incumbent services will be better protected by such distributed RF sensing since it would avoid the ‘hidden node’ problem.

These levels are based on the assumption that the D/U ratios between WRAN transmission and DTV reception will be the same as DTV into DTV.  Some refinements to these levels may be needed once actual tests are conducted with the modulation selected for WRAN transmission.  In the United States, more than 4 Watts EIRP would not be allowed according to the current proposals in FCC NPRM 04-186.