IEEE P802.22
Wireless RANs

| Functional Requirements for the 802.22 WRAN Standard |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2026-07-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Carl R. Stevenson | WK3C Wireless LLC | 4991 Shimerville Road, Emmaus, PA 18049-4955 USA | +1 610-965-8799 | wk3c@wk3c.com |
| Carlos Cordeiro | Philips | 345 Scarborough Rd Briarcliff Manor, NY 10510 USA | +1 914 945-6091 | carlos.cordeiro@philips.com |
| Eli Sofer | Runcom | Israel |  | elisofer@runcom.co.il |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | +1 613-998-2500 | gerald.chouinard@crc.ca |

Abstract

Introduction

This document represents a collaborative effort by the 802.22 Functional Requirements Ad Hoc Group, starting from discussions held at the January 2005 interim session and drawing and adapting the structure and content of document IEEE 802.16.3-00/02r4 as a framework and source of ideas for further work by the Ad Hoc Group.

Abstract

This document provides functional requirements that are guidelines for developing an interoperable 802.22 air interface. The 802.22 WRAN Working Group desired to reach an understanding and consensus for functional requirements before proceeding with developing a standard for 802.22 MAC and PHY protocols and thus formed a Functional Requirements Ad Hoc Group to produce this document.

Purpose

The 802.22 WRAN WG group will use this document to help qualify MAC and PHY protocol proposals.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Functional Requirements
for the 802.22 WRAN Standard

# Introduction

This document provides functional requirements that are guidelines for developing an interoperable 802.22 air interface for use in spectrum allocated to TV Broadcast Service, enabling Point-to-Multipoint (P-MP) Wireless Regional Area Network (WRAN). The WRAN system provides packet-based transport capabilities that can support a wide range of services (e.g., data, voice and video) to residential, Small and Medium Enterprises (SME) and Small Office/Home Office (SOHO) locations.

Throughout this document, the words that are used to define the significance of particular requirements are capitalized. These words are:

- “MUST” or “SHALL” These words or the adjective "REQUIRED" means that the item is an absolute requirement.

- “MUST NOT” This phrase means that the item is an absolute prohibition.

- “SHOULD” This word or the adjective “RECOMMENDED” means that there may exist valid reasons in particular circumstances to ignore this item, but the full implications should be understood and the case carefully weighed before choosing a different course.

- “SHOULD NOT” This phrase means that there may exist valid reasons in particular circumstances when the listed behavior is acceptable or even useful, but the full implications should be understood and the case carefully weighed before implementing any behavior described with this label.

- “MAY” This word or the adjective “OPTIONAL” means that this item is truly optional. One implementation may include the item because the target marketplace requires it or because it enhances the product, for example; another implementation may omit the same item

- The above adjectives are to be interpreted as guidance to proposers as to what the members of the WG believe to be requirements.  However, they are only guidance, and, in the interest of not inhibiting the ability of proposers to be innovative, proposers are free to take exception to requirements whose intent they believe can be met by other approaches, as long as they provide detailed justification for such exceptions that the members of the WG find acceptable.  No proposal will be rejected from consideration on the basis of having taken exception(s) to the guideline requirements presented herein.

# Scope

For the purposes of this document, an 802.22 MAC and PHY implementation constitutes a “system”  in which a base station communicates with least one Customer Premise Equipment (CPE)  via a point-to-multipoint (P-MP) radio air interface. Hence, “functional requirements” describe the requirements of interoperable 802.22 MAC and PHY protocols to meet the needs of a WRAN system. The functional requirements describe 802.22 systems and requirements in broad terms so that the required functions are specified but not how they work.

The ‘how’ part is left to the forthcoming 802.22 interoperability standard [1] which will describe in detail the interfaces, functions and procedures of the MAC and PHY protocols. This document focuses on the service capabilities that an 802.22 system is required to support. These service capabilities have a direct impact on the requirements of the 802.22 MAC and PHY protocols. When the 802.22 Working Group produces an interoperable air interface standard that meets these functional requirements, resulting 802.22-based implementations will be able to utilize and interconnect multi-vendor WRAN devices to provide the expected services to the end users.

Other goals of this document are to formulate reference models and terminology for both network topology and protocol stacks that will help the 802.22 Working Group to discuss and develop the MAC and PHY protocols.

The 802.22 protocols relate to other 802 standards and to the OSI model as shown in Figure 1-1.

Figure 1-1: Relationship between 802.22 and other Protocol Standards
(the numbers in the figure refer to IEEE standard numbers)

This standard deals with the Physical and Data Link layers as defined by the International Organization for Standardization (ISO) Open Systems Interconnection Basic Reference Model (ISO 7498: 1984).

The standards that define the services noted in the above diagram are as follows:

- IEEE Std 802: Overview and Architecture. This standard provides an overview to the family of IEEE 802 Standards. This document forms part of the 802.1 scope of work.

- ANSI/IEEE Std 802.1B [ISO/IEC 15802-2]: LAN/MAN Management. Defines an Open Systems Interconnection (OSI) management-compatible architecture, environment for performing remote management.

- ANSI/IEEE Std 802.1D [ISO/IEC 10038]: MAC Bridging. Specifies an architecture and protocol for the interconnection of IEEE 802 LANs below the MAC service boundary.

- ANSI/IEEE Std 802.1E [ISO/IEC 15802-4]: System Load Protocol. Specifies a set of services and protocols for those aspects of management concerned with the loading of systems on IEEE 802 LANs.

- ANSI/IEEE Std 802.2 [ISO/IEC 8802-2]: Logical Link Control.

# PAR Summary

The IEEE 802.22 WG on Wireless Regional Area Networks (“WRANs”) has the following charter, per its approved PAR:

To develop “Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications” … “(specifying) the air interface, including the medium access control layer (MAC) and physical layer (PHY), of fixed point-to-multipoint wireless regional area networks operating in the VHF/UHF TV broadcast bands between 54 MHz and 862 MHz.” [47-910 MHz with PAR modification]

“This standard is intended to enable deployment of interoperable 802 multivendor wireless regional area network products, to facilitate competition in broadband access by providing alternatives to wireline broadband access and extending the deployability of such systems into diverse geographic areas, including sparsely populated rural areas, while preventing harmful interference to incumbent licensed services in the TV broadcast bands.”

The goal is a global standard, capable of use in different regulatory domains where different TV technologies are used (NTSC, PAL, SECAM, ATSC-8VSB, DVB-T, etc.)

# Target Markets

The target markets described in this section are not an exhaustive set, but serve as guidelines and examples that suffice for meeting the broad applicability goals set forth by the air interface “Five Criteria” as described in the IEEE 802.22 Project Authorization Request (PAR) and “Five Criteria” [1, 2].

A Wireless Regional Area Network (WRAN) system based on 802.22 protocols is intended to make use of unused TV broadcast channels, on a non-interfering basis, to address, as a primary objective, rural and remote areas and low population density underserved markets with performance levels similar to those of broadband access technologies serving urban and suburban areas The WRAN system should also be able to scale to serve denser population areas where spectrum is available.

- The WRAN system MUST be capable of supporting a mix of data, voice (VoIP) and audio/video applications with corresponding provisions for QoS.  The RF link availability assumed for the provision of these WRAN applications is 99.9% of time.

The critical parameters for serving these markets using wireless access technology are the combination of coverage/capacity factors that affect access cost per user, deployability, maintainability, and product costs associated with the Customer Premise Equipment (“CPE”) installation,  and spectrum reuse to economically serve the required number of customer locations with a minimum number of base station locations and backhaul routes.

The target markets to be addressed by the 802.22 protocols in WRAN networks are single-family residential, multi-dwelling units, SOHO, small businesses, multi-tenant buildings, and public and private campuses.

In accordance with ITU-R [2] definitions, Fixed Wireless Access (FWA), to which WRAN belongs provides access to one or more (public and private) core networks, rather than forming an end-to-end communication system. 802.22 systems serve fixed location CPEs that might be geographically fixed or re-locatable.

# WRAN System Model/Requirements

This section presents a high level description of a system model to be used as a framework for developing the 802.22 standard. The model identifies the main features of an 802.22 system, and the terminology to be used by the 802.22 working group in the creation of the standard.

The 802.22 wireless regional area network system is aimed at providing broadband access with capabilities similar to ADSL and cable modem technologies, but capable of more economical deployment over less populated rural areas.  The typical range of the system is 33 km (based on 4 Watt CPE EIRP and 50% location availability at the edge of the coverage area for a median location and 99.9% time availability F(50, 99.9)) for a coverage of population density of about 1.25 person/km2 and above, and up to a maximum of 100 km where higher base station transmit power is permitted in the relevant regulatory domains.  The system will need to operate over a set of typical channels models as defined in Appendix C.

As mentioned in section 1.1, an 802.22 “system” constitutes an 802.22 MAC and PHY implementation in which at least one subscriber station communicates with a base station via a point-to-multipoint (P-MP) radio air interface and supports the required services. Specific applications of the 802.22 point-to-multipoint (P-MP) radios are aimed at the use of the VHF/UHF TV broadcast frequency range. Radio communications in the above range may be possible in line-of-sight as well as non-line-of-sight situations between a base station and subscriber stations. Operation may include partial and even complete blockage by foliage and other obstructions.  This will contribute to signal attenuation and multipath effects. Figure 2-1 shows an example deployment configuration including the optional use of macro diversity (optimization of link and use of repeaters). 802.22 systems should be deployable in multiple-cell frequency reuse systems and single cell (super cell) frequency reuse systems. The range of 802.22 links will vary with EIRP, local topography, atmospheric conditions, channel characteristics, availability requirement and local regulations as well as bandwidth and transmitter/receiver performance.

Figure 2-1 Example Deployment Configuration

An 802.22 system MUST consist of one base station radio and one or more Consumer Premise Equipment (CPE) radios, i.e.  an 802.22 base station and one or more stationary CPE radios communicating using the 802.22 MAC and PHY protocols.

Proposals for 802.22 MAY also include a description of how repeaters could be accommodated to extend the range of a WRAN system.

The base station radio SHALL be P-MP, capable of  radiating its downstream signal (forward) toward the CPEs with an omni-directional, a shaped sector, or optionally an adaptive array (spatial reuse) antenna achieving broad azimuthal beam width to serve a number of prospective subscribers.

For the purpose of coexistence with incumbent services operating in these TV broadcast bands (TV broadcasting, FCC Part 74 devices, e.g. wireless microphones, wireless intercoms, etc.,  and PLMRS), the 802.22 standard SHALL include mechanisms in the PHY and MAC protocols to allow the base stations to dynamically change the power and/or frequency of operation of the network based on the sensing of the use of the spectrum by these incumbent services by the base station and the CPEs to avoid interference to these services (see section 15.1).  This will constitute an essential part of this standard.

For the purpose of coexistence among WRAN systems operating in the same area, the 802.22 standard SHALL include mechanisms allowing cooperation between base stations for better sharing of the spectrum.  The MAC and PHY protocols MUST provide means for base stations to resolve interference problems due to collocation or overlapping coverage areas.

The frequency bands used by 802.22 systems MAY vary across various regulatory domains (see Appendix D).  In the case of the USA, the frequency range identified by IEEE 802 in its comments to the FCC was from TV channel 2 to 51, (54 MHz to 698 MHz) the 802.22 PAR identifies 54 MHz to 862 MHz but the extremes of the international range are from 47 MHz to 910 MHz.

Since the 802.22 system MUST operate without causing interference to incumbent licensed services, sensing of channel occupancy SHALL be done according to the more stringent of either the regulatory requirements in the regulatory domain where an 802.22 system is installed and operated or the requirements specified in the 802.22 Standard.

## Wireless Access Reference Model

Figure 2-2 shows the 802.22 wireless access reference model. The model depicts the relevant points between subscriber networks and “core” networks. A greater system encompassing user terminals, base station interconnection networks, network management facilities, etc. MAY be envisaged but the 802.22 protocol focuses on the air interface shown in the model. The Core Network Interface (CNI) and the User Network Interface (UNI) are also shown.

A single Customer Premise Equipment (CPE) MAY provide connection to multiple customer premises networks that transport data, voice and video through one or more UNIs.

Base stations MAY provide connection to multiple core networks through one or more CNIs.

For the purposes of 802.22, the UNI and CNI are abstract concepts. The details of these interfaces are beyond the scope of this document.

The standard SHALL specify MAC layer protocols and PHY transmission techniques suitable for providing access between one or more CPE and base stations to support UNI and CNI requirements.

Figure 2-2: Wireless Access Reference Model

## IEEE 802 Architecture Conformance

The 802.22 standard SHALL conform to the requirements of the IEEE 802.1 Architecture, Management and Interworking documents as follows: 802. Overview and Architecture, 802.1D, 802.1Q and parts of 802.1f (Optional MAC Bridge to other networks, e.g. other 802 networks.)

## Service capacity

The required minimum peak throughput rate at edge of coverage SHALL be 1.5 Mbit/s per subscriber in the forward direction and 384 kbit/s per subscriber in the return direction.  The capacity of the base station will need to be higher to provide service to a number of subscribers in this P-MP system.

The system SHALL operate with a minimum spectrum efficiency per transmission link of 0.5 bits/sec/Hz and a maximum goal of 5 bit/(s/Hz) or better where propagation permits. CPEs should be designed to operate at the highest spectrum efficiency possible per transmission link, as allowed by regulations and/or operating conditions.

## Installation Requirements

Base stations SHALL be designed to be professionally installed.

CPEs MAY be user installable (plug and play) but operators MAY choose to provide for professional installation.

The CPE antenna (transmit and receive) SHALL be assumed to be installed outdoors at a nominal height of 10 meters above ground level.  It is assumed that the sense antenna will be omnidirectional and will be co-located with the transmit antenna.  Sensing thresholds specified in section 15.1.3 are based on these assumptions.  Installation and deployment requirements should be further elaborated in proposals with the understanding that, due to the unique requirement of assuring incumbent services are protected, they will be incorporated in a normative manner in the standard.  Proposals may discuss the implications of WRAN antenna installations at heights other than 10 meters above ground level.

For the system to operate correctly, proper CPE installations SHALL be verifiable.  Means MAY be provided to allow verification of proper CPE installation remotely from the base station. Proposers are strongly encouraged to submit ideas for mechanisms by which such verification could be done automatically. If such a mechanism is adopted, it SHALL ensure that CPEs will not transmit absent reliable verification.  In the absence of any automated verification method, visual verification of proper installation will be necessary.

## Network Entity Relationships and Topology

The topology is fixed P-MP. Repeaters (also fixed) MAY optionally be used to extend the coverage area or enhance network capacity. In all cases, all device types SHALL remain under the control of the base station with the BS providing centralized power control, spectrum management, and scheduling control.

### Base Station/CPE Master/Slave Relationship

The base station SHALL serve as a radio resource supervisor and controller for its “cell,” including all associated CPEs and/or optional repeaters, if implemented, in its cell.

A Master/Slave relationship between the base station and the CPEs  SHALL be established.

The base station SHALL also be capable of remotely controlling the distributed sensing of the RF Environment by its associated CPEs (in addition to CPEs’ inherent autonomous sensing capabilities).

### P-MP Star Architecture

In the downstream direction, all traffic directed to CPEs SHALL be transmitted by the base station or relayed by a repeater if that option is implemented.

In the upstream direction, 802.22 protocols MUST provide the means to efficiently multiplex traffic from multiple CPEs and allocate transmission channel capacity.

#### Optional Repeater Function

802.22 systems MAY support the optional deployment of repeater functions. Proponents may wish to propose mechanisms for repeaters to augment the capacity and/or extend coverage into areas that could not otherwise be reached by the service and document the impact of the use of these repeaters on the functionality of the PHY and MAC layers.  The repeater function, if implemented SHALL preserve the end-to-end operation of 802.22 protocols between the base station and the CPEs.  Such repeater function, in the case of the use of on-channel re-transmitters, will have some impact on the channel multipath performance requirements of the receivers (see section 8.3) and the proponents SHALL take such impact into account in their proposal if they decide to include on-channel re-transmitters.

## Wireless Media Characteristics

### Duplex Modes

This standard SHALL support duplex modes of operation.

If FDD is proposed, the proposal MUST include a proposed band plan scheme and address system impacts such as duplex filter requirements, antenna considerations, and other factors affecting performance and overall system complexity.

If TDD is proposed, the proposal MUST include diagrams illustrating the proposed framing/timing and analysis of the impact of TDD turn-around time on system capacity, latency, and other factors affecting performance and overall system complexity.

Half-duplex MAY also be considered as long as service latency is kept to an acceptable level.  This covers the cases of the TDD mode and the FDD mode where duplex operation is assumed at the base station but half-duplex operation is used at the CPE to reduce the user terminal complexity.

The proposal MUST also include a discussion on the impact of the duplex or half-duplex configuration on the way interference situations involving other WRAN systems will be resolved to ensure co-existence among WRAN systems (see section 15.2).

### Flexible Asymmetry

Symmetry is hard to predict and is very bursty depending on the type of traffic.  Some applications utilize naturally asymmetrical bit rate, such as for generic Internet access where most of the throughput is consumed in the downstream direction. Some applications utilize asymmetrical bit rate in the reverse direction, using more in the upstream direction, such as surveillance video from a CPE. Other applications require symmetrical capacity, such as telephony and video conferencing.

The 802.22 WRAN system SHALL support flexible asymmetry.

802.22 WRAN systems SHALL also have the flexibility to satisfy the bit rate requirements of a mix of applications in both directions.

It should be recognized that CPEs at the edge of coverage may not be capable of achieving the same bit rate in the upstream direction as the base station can provide to them in the downstream direction.

# Supported Service Capabilities

This section describes typical service capabilities supported by an 802.22 air interface. The MAC and PHY protocols will not need to have explicit support for each and every service, due to the fact that generic data streams SHALL be used for transport. The MAC and PHY protocols SHALL provide for admission control, QoS service specific support, appropriate PER for the various types of services, and acceptable latency and jitter tolerances for (quasi) real time services and VoIP.

## Data Transport Service Capabilities – Internet and VoIP

The 802.22 system SHOULD directly transport variable-length IP datagrams efficiently. Both IP versions 4 and 6 MUST be supported with packet prioritization, admission control and scheduling.

802.22 systems SHOULD support both (quasi) real-time and non-real-time service capabilities.

The 802.22 protocols SHOULD support VoIP services.

The 802.22 protocols SHOULD support IP Quality of Service (QoS) efforts: Differentiated Services [RFC 2475, 1998 and RFC 2474, 1998].

The following types of services SHALL be supported, in those situations where capacity is available, recognizing that avoiding interference to incumbent licensed services takes precedence over maintaining WRAN service(s):

- Constant bit rate service

- Real-time variable bit rate service

- Non-real-time variable bit rate service

- Best-effort service

## Bridged Network Service Capabilities

The 802.22 protocols SHALL support 802.1 bridging service capabilities to other IEEE 802 networks.

## Other Services

Other services that, for instance, require QoS-based delivery of the MAC services MAY be added.

These services SHALL NOT place any additional requirements on 802.22 systems (MAC and PHY protocols) not already covered in the above sections.

# 802.22 Protocols

The IEEE 802.22 MAC and PHY protocol stacks SHALL be the same for all the supported services. The central purpose of the MAC protocol layer in 802.22 is sharing of radio channel resources. The MAC protocol defines how and when a base station or subscriber station may initiate transmission on the channel.

Since CPEs MAY contend for capacity to/from one or more base stations, the MAC protocol MUST efficiently manage contention and resource allocation.

The PHY layer MAY optionally be subdivided between a convergence layer and a physical medium dependent (PMD) layer. The PMD is the “main” part of the PHY. Like the MAC convergence layers, the PHY convergence layers adapt/map the “special” needs of the MAC services to generic PMD services.

# Performance and Capacity

This section addresses some issues regarding 802.22 system performance and capacity. Specifying protocols that can maintain specified/mandatory performance levels in the face of fluctuating and diverse channel characteristics (e.g., due to multipath and atmospheric conditions) is a problem that the 802.22 Working Group has to consider. This section specifies the target performance levels. This section also outlines some of the issues for 802.22 capacity planning.

The PHY and MAC protocols SHALL flexibly accommodate communication between a base station and CPEs at distances of up to 100 km where feasible.

## Multipath/Delay Spread Performance

Proposers SHALL provide analysis and/or simulations to demonstrate the performance of the proposed system using the multipath profiles supplied in Appendix D.

In addition, if on-channel repeaters are used to extend/improve coverage (see section 5.6.3), this may create artificial multipath that will go beyond the multipath conditions described in the channel model developed in Appendix D.

The amplitude of these echoes becomes dependent on the location of the receiver relative to the various re-transmitters.  Delayed echoes may become more powerful than earlier echoes if the receiver is located closer to the re-transmitters emitting these signals.

This translates into a potential for a large amount of pre-echoes.  Furthermore, depending upon the distance between these on-channel repeaters, the echoes produced may have very large excess delays, well beyond those typically seen for passive echoes as described in Appendix D.

If on-channel repeaters are used, the performance of the demodulation algorithms SHALL be adapted to the presence of these artificial multipath environments and proven to perform adequately in all on-channel repeater arrangements.

Multipath test profiles SHOULD be developed accordingly by the proposer.

## Ranging / Synchronization / Acquisition / Access

The first time a CPE is turned on, it MUST start by sweeping the RF range in which it is to operate to identify the presence of incumbent operations, as well as to access information from the WRAN networks accessible in the area.

The CPE SHALL then attempt to associate with the selected WRAN base station according to the parameters sent by the base station in its broadcast control packets.  Initial hand shaking with the base station SHALL be done for ranging, initial synchronization, and CPE initialization.

Ranging SHALL be done to synchronize CPEs with a common time reference relative to base station timing  in order to maximize the transmission medium capacity by minimizing guard time requirements.

The CPE SHALL be synchronized with its associated base station before attempting any transmission.

Network entry time at initial power on for a CPE SHOULD be no more than 10 seconds under normal operating conditions.

## Frequency/time tolerances

The frequency and time stability of the base station and CPEs as well as the dynamic behavior of the clock synchronization should be documented by the proposer.

Tolerance for the CPEs SHOULD be more relaxed than that of the base station to minimize CPE cost and complexity.

CPEs SHALL be capable of synchronizing to the base station to adjust their drift in frequency and time to within the same tolerance as the base station.

The CPEs frequency/timing control system(s) SHALL have enough range to achieve synchronization to the base station frequency and timing over documented worst case initial offsets, tolerances, and environmental extremes.

The CPE SHALL be capable of acquiring frequency and time synchronization in a sufficiently short period and then maintaining synchronization to secure proper service.

## Spectral Efficiency

Spectral efficiency is an important performance parameter of a wireless access system. This spectral efficiency will vary from location to location within the coverage area of the base station, depending on distance, propagation, channel impairments, interference, and their effect on the usable modulation and coding parameters over each given path.

The WRAN system SHALL be capable of providing spectral efficiencies in the range from 0.5 bit/sec/Hz up to a goal of 5 bits/sec/Hz or better per transmission link depending on the transmission channel conditions.  Proposers SHALL document the way they quantify these minimum and maximum spectral efficiencies and in which operational conditions they would apply.

## Radio Link Availability

802.22 systems are expected to support all intended services at their respectively rated QoS (see section 14.3) except for 0.1% of the time when the RF link may not be available (based on a link availability of 99.9% of the time), (not counting network entry time, quiet periods for RF sensing, and electrical power and equipment down-time).  Information on how the technology will allow to achieve this requirement and help conceal and react to the downtimes SHALL be provided.  The proposed WRAN technology SHOULD allow for graceful degradation through a reduction of the throughput rather than a complete loss of service.

The coverage of WRAN service will be based on 50% location availability at the edge of the coverage area for a median location.

The 802.22 technology SHOULD also allow for the radio link to be engineered for different link availabilities, based on the preference of the system operator.

## Radio Link Error Performance

The error rate, after application of the appropriate error correction mechanism (e.g., FEC), delivered by the PHY layer to the MAC layer SHALL meet IEEE 802 functional requirements for undetected corrupted packet rate and radio link Packet Error Rate (PER) for a packet length of 1000 octets at the minimum specified receiver sensitivity.

In order to assure that the base station is able to control all of its CPEs, control frames SHALL be transmitted in a manner to assure extremely robust packet error rate performance.  Proposers SHALL state the packet error rate for control packets for their proposed solution and demonstrate through analysis and/or simulation how they will meet their stated performance.

## Delay, Jitter, and Latency

Delay and variation of delay, or jitter, are important factors to consider. For example, a high variation of delay can severely impact interactive services. However, generic Internet access can tolerate a high degree of delay variation.

Jitter for the services considered SHOULD be able to be maintained to less than 10 msec.

Maximum average CPE to base station and base station to CPE latency caused by the 802.22 PHY and MAC SHOULD be no more than 20 msec. to support VoIP (see section 14.3).

## Capacity

The 802.22 base station capacity requirement is defined as the product of the number of subscribers, their peak bandwidth requirements and load factor, based on quality of service goals.

Given the propagation characteristics in the target frequency bands and variability in differing geographic area for the development of a link budget, the following parameters of an 802.22 system SHOULD be addressed by the proponents in the development of their MAC and PHY protocols:

- Radio range

- Upstream/downstream channels’ data rates

- Allocation of prospective subscriber data rate to channels. Note: the MAC and PHY standard MAY allow subscribers to hop between channels

- Types of modulation

The MAC and PHY protocols SHOULD accommodate channel capacity issues and changes in channel capacity as described in section 6 by employing, for example, flexible modulation types, power level adjustment, and bandwidth reservation schemes when needed (see section 6).

## Delivered Bandwidth

The minimum delivered peak data rate per subscriber SHALL be 1.5 Mbit/s forward and 384 kbit/s return.

## Flexible data rate and resource allocation – Downstream & Upstream

The WRAN system SHALL support dynamic capacity allocation to try to meet subscribers’ quality of service expectations.

System capacity allocation SHOULD be able to be varied dynamically to accommodate a wide mix of usage scenarios and user demands for varied services.

The system SHALL allow its operator to flexibly allocate its system resources to any CPE for the purpose of limiting or enhancing its throughput.

In the case of an optional regenerative off-channel repeater, the system SHOULD be capable of providing for more capacity to such a repeater.

### Peak Data Rates

The WRAN base station SHOULD be able to provide to CPEs, higher bit rates than the previously stated minimum peak data rate if the system capacity is available, up to the maximum forward throughput allowed by the channel capacity and propagation conditions to one subscriber, at times of low system loading and CPEs SHOULD be able to receive the full throughput from the base station.

In the return direction, CPEs SHOULD be able to increase their transmission capacity at times of low system loading, up to the maximum channel throughput allowed by the channel capacity, propagation conditions and as long as the transmit power limit is not exceeded, i.e., a CPE relatively close to the base station may, as directed by the base station, use part of the power margin of its Transmit Power Control (TPC) to achieve higher return throughput.

### Number of User Terminals serviced by a base station

A WRAN base station SHALL be able to support at least 512 simultaneously associated CPEs (minimum size of MAC address table – this does not imply servicing all CPEs simultaneously).

## Connection loss and network outage response

Proposers SHALL describe in detail (text, flowcharts, SDL, etc.) how fault-tolerant the proposed system is and how a CPE that has lost network connectivity will re-establish connectivity.

Representative, but not exhaustive examples of the causes of loss of network connectivity include:

- power outage (due to moving CPE to a different location)

- power outage (due to utility failure)

- loss of connection due to link failure (proposer to describe what constitutes “link failure”)

- other (proposer to enumerate and explain)

## Antenna Systems

WRAN CPE transmit antennas SHOULD be polarized orthogonally to nearby DTV receive antennas to provide additional isolation through cross-polarization discrimination.

CPE antenna systems SHALL support the requirements of 15.1.1.7 (sensing thresholds), 15.1.6 (maximum transmitted EIRP) and 15.1.7 (out of band emissions). Proposers SHOULD comment on how to integrate the various elements of antenna systems to meet these requirements.

Provisions SHALL be made to prevent the use of antenna equipment that could impair or defeat the requirements of the preceding two paragraphs and proposers SHALL elaborate their proposed approaches.

Base station antenna systems SHALL be designed and installed to support the requirements of 15.1.1.7, 15.1.6, and 15.1.7.

# Base Station and CPE Duty Cycles

Base stations SHALL support 100% transmit duty cycle at rated power.

CPEs SHALL support 100% transmit duty cycle at rated power to support cases where maximum upstream throughput is required from a terminal at the edge of the coverage area.

# Adaptability and Scalability

## Adaptability

Adaptability refers both to modification of specific transmission parameters as well as to download of updated firmware and software for the CPEs.

### Per-Subscriber Rate Adaptation

The PHY and MAC protocols SHALL provide the ability to adaptively deliver different bit rates/capacities to individual subscribers.

Also, the PHY and MAC protocols SHALL provide for adaptable channel capacity as a function of channel performance. With a minimum spectrum efficiency of 0.5 bit/s/Hz and a goal of 5 bits/s/Hz or better  per transmission link as propagation permits.

### Per-Subscriber Power Adaptation (TPC)

While the control frames need to be sent by the base station at constant high power so that all CPEs, including those in fringe conditions can also receive it, other packets don’t need to be heard  by all CPEs all the time and therefore may need less power/protection.

The WRAN system standard SHALL support Transmit Power Control (TPC) on a link-by-link basis to allow a reduction of the transmit power at the CPEs (and perhaps also at the base station, at the option of the proposer) to the lowest levels possible while still maintaining a reliable connection.  A trade-off will be possible between such low transmit power level and the need to increase the complexity of the modulation scheme in order  to increase the link throughput as much as possible.

Proposers SHALL specify their proposed TPC range and granularity which SHALL be at least 30 dB with 1 dB steps.

## Scalability

Scalability refers to varying system operational parameters such as: bit rate, channel bandwidth (varying channel bandwidth within the TV channel and/or using a variable number of TV channels), extent of coverage (e.g., TPC), deployment, etc.

### Bandwidth scalability

#### Use of a fraction of a TV channel

The operation of the WRAN system may need to be restrained to a smaller portion of the TV channel when interference to wireless microphones operating on an adjacent TV channel and other LE systems is considered, and even for operational reason.  The 802.22 standard may need to include scalability with respect to occupied bandwidth.  As a minimum, the 802.22 standard SHALL include the scalability necessary to adapt to the 6, 7 and 8 MHz TV bandwidths that exist in the various parts of the world.

#### Use of multiple TV channels

Where spectrum is available, it may be useful for a WRAN system to use more than one TV channel (contiguous or not) to increase the capacity of the transmission link as long as each TV channel can become independent from the modulation point of view to be able to free any one of these channels in case of interference.  (Note: 802 commented to the FCC that channel bonding SHOULD NOT be permitted to be persistent.)

### Link symmetry scalability

In addition to the minimum requirement of 1.5 Mbit/s and 384 kbit/s specified in section 8.1, the 802.22 standard MAY provide higher return link capacity, up to and including a fully symmetrical link.

# Support for Different Classes of Base Stations

The default class of base station SHALL operate at a maximum of 4 W EIRP.

The 802.22 standard SHOULD support different classes of base stations with different conditions of operation such as transmit power levels, etc. where permitted by regulatory domains

Proposers SHALL elaborate in detail the technical characteristics of each class proposed (transmit EIRP, out of band emissions mask, required separation distance from TV receivers, EIRP profile if required, etc.)

Proposers SHALL provide analysis and simulation to justify the proposed technical parameters of each class proposed and provide a very high degree of confidence that those parameters will not cause harmful interference to licensed services.

# Flexibility in Tuning

Although the total range over which the 802.22 systems may operate worldwide is from low VHF (54 MHz) to UHF (862 MHz) [47-910 MHz verified – may want/need to submit PAR rev], it is unlikely that, in practice, the total range will be covered by a common set of antennas and RF front ends at the base station and subscriber terminals.

It is expected that this range will be divided in more reasonable segments such as the low-VHF range (band 1), high VHF range (band 2), low-UHF (band 4) and medium UHF (band 5).

Although the flexibility in dynamic frequency selection to avoid interference will be limited to these segments, it is presumed likely that such flexibility will be sufficient in most cases.  In more difficult cases, equipment covering more than one range may be needed.

The 802.22 standard SHALL support operation in these frequency segments as well as the amalgamation of more than one of these segments.

# Channelization

The 802.22 Standard SHALL support channel spacings of 6, 7 and 8 MHz and proper center frequencies to align with the frequency raster of the incumbent broadcast systems in the area.

The CPE RF front-end and tuning SHALL be capable of adapting accordingly.

Subdivision of these nominal channels MAY be used as long as the regulatory maximum EIRP per device in a 6, 7 or 8 MHz channel is not exceeded.

# Medium Access Control (MAC)

## Network Operations

The 802.22 protocols SHALL provide mechanisms for authorization, registration, continued operation and deregistration of CPEs to 802.22 networks while avoiding interference to incumbent users.

All operations other than CPE initialization SHALL take place under the control of the base station, which SHALL be capable of simultaneously supporting multiple CPEs.

## Support for Interference Mitigation/Coexistence

### Sensing Measurements

802.22 Base Station SHALL control CPEs to conduct measurement activities and obtain measurement results.  There may be a need for scheduled quiet periods for sensing to take place.

Sensing repetition rate and integration time SHALL be identified in order to meet the sensitivity down to the specified threshold in a timely manner.

Sensing SHOULD include capture of signal signature to identify the type of incumbent and other LE signals and possibly the transmit unit identification.

#### Sensing Control

802.22 protocols MUST provide mechanisms for sensing control in the CPEs.

The Base Station SHALL be able to instruct the CPEs when to sense, which channels to sense and the duration of the sensing activity.

Additionally, the base station SHALL maintain and control a threshold per incumbent type for comparison purposes to assure maximum incumbent protection.

#### Sensing Measurement Report

CPEs SHALL send to the Base Station sensing measurement reports in a form understandable by the Base Station.

In addition, CPEs SHALL, at a minimum, be able to acquire and convey to the Base Station sufficient data to permit the identification of the incumbent or other WRAN RF signal profile.

CPEs COULD also recognize RF signal profiles, and in such cases, these CPEs SHOULD be able to recognize the incumbent services present in the regulatory area of operation.

Proposers SHALL elaborate in detail how they propose to implement incumbent profile identification.

The IEEE 802.22 air interface SHALL provide identifiers for CPEs and Base stations to facilitate the resolution of any interference problems that might occur (see Section15.3).  The CPEs and Base stations SHALL be able to extract the WRAN device identification from the received RF signal.

### PHY Operational Control

#### Channel Management

##### Channel Change

Once the CPE has received the channel change request from the base station, the 802.22 MAC SHALL order the 802.22 PHY to change the channel of operation.

##### Suspend/Resume Channel Transmission

The 802.22 MAC SHALL be able to suspend the transmission in a specified channel. Transmission in this channel MAY be resumed upon indication from the WRAN base station.

##### Termination and Reestablishment of Channel Operation

The 802.22 MAC SHALL be able to terminate the operation in a channel. Operation MAY be reestablished in another channel.

#### Control of PHY Adaptability

The 802.22 MAC in any device (CPE or base station) SHALL be able to control the PHY adaptability features such as TPC, modulation/FEC, channel bandwidth, and timing only in response to commands from the base station.

### Base Station Management of CPEs

#### Base station and CPE Relationship

The 802.22 system follows a master/slave relationship where the base station performs the role of the master    and the CPEs are the slaves.  Hence, in an 802.22 system, the base station SHALL be responsible for managing CPEs in numerous aspects including modulation, coding, encryption, power level, and bandwidth usage.

#### Measurement Management

Measurement activities in an 802.22 system are also managed by the base station.  Thus, the base station SHALL be able to order CPEs to perform measurements and report the results back to the base station.

#### Measurement analysis

Measurements analysis is an ongoing process conducted by the base station and which is based upon measurement results reported by CPEs as well as the base station’s own measurements. Once sufficient measurement information is available to the base station, the base station SHALL be able to analyze that data and, if necessary, take appropriate steps to resolve any interference potential by changing its or any associated CPEs operating parameters such as frequencies of operation and power level.

To augment this functionality, the base station MAY rely on other features such as externally available databases (see Section 15.2.5), information from other 802.22 base stations, and service provider specific information.

### Inter-Base Station Coordination

The IEEE 802.22 protocols SHALL provide mechanisms for the exchange of information between WRAN base stations to allow for coexistence, interference avoidance, and sharing of radio resources amongst neighboring 802.22 Base Stations, whether they are operated by the same entity or not.

The communications channel for this function MAY use the 802.22 air interface, backhaul channels, or other means.

CPEs and base stations SHALL be able to report interference received from other 802.22 base stations or CPEs, and neighboring base stations SHALL be REQUIRED to cooperatively take measures in order to resolve the issues.

Proposers SHALL describe their proposed solution and provide data on how their proposed solution works, including information exchanged, metrics of performance (including impact on the capacity of the networks involved), etc.

## Class of Service and Quality of Service

This section describes the classes of service and quality of service for 802.22 systems. Terminology is borrowed from the Internet Engineering Task Force (IETF).

802.22 protocols MUST support classes of service (CoS) with various quality of service (QoS) goals to support the services that an 802.22 system MUST transport.

Thus, 802.22 protocols MUST define interfaces and procedures that accommodate the needs of the services with respect to allocation and prioritization of resources.

Additionally, 802.22 protocols MUST provide the means to ensure that QoS levels are satisfied under the constraints of license-exempt operation in presence of incumbent services. Table 1 provides a summary of the QoS requirements that the PHY and MAC SHALL provide. Note that the parameters in the table are measured between the MAC input at the upper layer at the transmit station and the MAC output at the upper layer of the receiving station for information transmission. For example, delay does not include setup time, link acquisition, voice codecs, etc.

For evaluation purposes, proponents SHOULD indicate the Packet Error Rates (PER) that their proposed solution can provide for each type of service shown in Table 1.

Either connection-less or connection-oriented services, or a mix of both, MAY be proposed for consideration.

For QoS-based, connectionless services, the 802.22 protocols MUST support resource negotiation “on-demand”. For instance, the MAC protocol MAY allocate bursts of PDUs to services that require changes in resource allocation. Such allocation, for connectionless services, is thus performed in a semi-stateless manner.

A connection-oriented service MAY require “state” information to be maintained for the life of a connection. However, the 802.22 MAC layer interface MAY provide a connection-less service interface that requires a higher-layer “adaptation” to maintain the “state” of a connection and periodically allocate resources. For instance, the MAC may need to maintain “state” information about a QoS data flow only for the duration of an allocation.

Table 1: Services and QoS Requirements

| Service | Maximum Ratio | Maximum Latency Delay (One way) |
| --- | --- | --- |
| Full Quality Telephony (Vocoder MOS) 4.0) | BER10-6 | 20 ms |
| Standard Quality Telephony (Vocoder MOS) < 4.0) | BER10-4 | 40 ms |
| Time Critical Packet Services | BER 10-6 | 20 ms |

### Types and Classes of Service

- The fundamental QoS model that will be exported to the WRAN endpoints SHALL be IP based and conform to IETF DiffServ QoS model in conjunction with other IP based protocols. (See IETF RFC 2474 and RFC 2475).

### Parameters

802.22 protocols SHALL define a set of parameters that preserve the intent of QoS parameters or IP-based services.

### Service QoS Mappings

The classes of service and QoS parameters of services SHALL be translated into a common set of parameters defined by 802.22.

A QoS-based IP network MAY employ the Resource reSerVation Protocol (RSVP) [7] to “signal” the allocation of resources along a routed IP path.

If 802.22 is to be a “link” in the IP network, an IWF MUST interface with 802.22 to negotiate resource allocation.

The basic mechanism available within 802.22 systems for supporting QoS requirements is to allocate bandwidth to various services.

802.22 protocols SHOULD include a mechanism that can support dynamically-variable-bandwidth channels and paths (such as those defined for IP environments).

## Single channel versus Multichannel Operation

The simultaneous use of more than one TV channel (contiguous or not) MAY be possible as long as it is possible to free up any of these channels in case of interference to the incumbent services.

The 802.22 MAC protocols SHALL be able to terminate operation in any of the channels being used if the need arises.

## OA&M Support

The 802.22 air interface SHALL provide a mechanism to enable the provisioning and collection of metrics, so that the network operator can effectively control, monitor, and tune the performance of the 802.22 air-interface.

Provisioning parameters, performance metrics and other OA&M values SHALL be made available through a standards compliant MIB.

## Base station and CPE address spaces

The 802.22 protocols SHALL support addresses that uniquely identify CPEs and base stations.

802.22 transmitted frames SHALL contain, at a minimum, the address of the transmitting device to identify the source of the transmission for the purpose of facilitating interference resolution with incumbents as well as other 802.22 systems.

# Co-existence and interference mitigation

The context in which WRAN systems will be operated is not usual for License-Exempt type services.  The TV broadcast bands in which they will operate are already used by TV broadcasting, Part 74 devices (for example wireless microphones in the US) and terrestrial mobile radio services including Public Safety (for example Part 90 in the US).

The PHY and MAC protocols of the WRAN standard MUST include the necessary means to operate in such an environment without causing interference to these incumbent services.

The WRAN system SHALL also provide a fail-safe operation mode in the CPEs and base stations to avoid interference should anomalous situations occur.

Proposals SHALL provide flowcharts or other means of fully explaining and characterizing the algorithms and mechanisms they employ to meet these requirements with the understanding that the solutions selected will be incorporated in a normative manner in the standard.

Furthermore, these protocols SHALL also include means to allow coexistence among multiple WRAN systems and with other license-exempt systems in these bands for fair and efficient use of the spectrum.

## Licensed Incumbent Sensing and Avoidance

### Overview

Interference avoidance SHALL be based on RF sensing at the base station and at the CPEs and SHALL rely on cognitive radio techniques and intelligence at the network level.

The WRAN system SHALL provide the means for the base station operator to assess interference potential by mapping the results of the sensing of spectrum occupancy performed by the base station and the CPEs against information on their physical location acquired through registration information or other geolocation mechanisms.

Data fusion from a sufficiently large number of CPEs will be needed to get a reliable spectrum occupancy figure.

Such sensing SHALL include the monitoring of broadcast operation and Part 74 licensed device operation.

If the implementation requires a quiet period, it MUST be done within the requirements of providing and maintaining QoS (i.e., network throughput and latency) in an interference free environment.

#### TV broadcast: description and technical characteristics

In the context of developing an international standard for WRAN, the characteristics, channel rasters (6, 7, 8 MHz) and extend of frequency bands which tend to change with international regions will have to be accommodated.  Appendix D includes tables that give details on the various channelization schemes and signal formats.  .

#### Wireless Microphone description and technical characteristics

Wireless microphones are operated in the US as licensed Part 74 devices  by broadcasters, news crews, in-studio and in remote production locations.    The typical system characteristics are detailed in Appendix E.  Various regulatory environments exist in the world with respect to the operation of wireless microphones in the TV bands.

#### Terrestrial mobile radio services (PLMRS) description and technical characteristics

WRAN stations SHALL be configured to result in less than 1dB of degradation to nearby land mobile base stations receiving in 455-460 MHz and 465 - 470 MHz nationwide, and in channels 14 - 20 where operation is authorized.  When the WRAN base station is co-located with the land mobile base station, the maximum field strength that can be produced at the land mobile station is -11 dBuV/m over 12.5 kHz bandwidth in the peak of the PLMRS antenna main lobe.

The resulting out-of-band emission requirement for the WRAN base station could be relaxed by locating the WRAN base station antenna to take advantage of physical separation and/or WRAN/land mobile antenna pattern/polarization discrimination.

#### Medical telemetry

While indications are that greater than 80% of all medical telemetry systems have already migrated to TV channel 37 and the remainder have been warned/advised for years to do likewise, it is recommended that base station operators notify and/or coordinate with potentially affected facilities.

#### Algorithms for Avoidance of interference to incumbents:

A WRAN CPE device SHALL NOT transmit unless it periodically receives an enabling signal from a Base Station. Proposers SHALL elaborate their proposed means of meeting this requirement and SHALL propose a maximum period without receipt of an enabling signal from a base station, after which the CPE will cease transmission.  (It is suggested that this time should be commensurate with the Channel Move time specified in Table 15.1.2.)

Both CPEs and Base Stations SHALL sense the presence of incumbent television and wireless microphones as specified in Paragraph 15.1.3.

When, during normal WRAN system operation, a CPE senses that either a television or wireless microphone signal is present above the specified threshold(s), the CPE SHALL communicate sufficient information about the signal detection to the Base Station and wait for instructions from the Base Station.

It is desirable for the sensing terminal to extract from the signal detection as many features of the signal as possible and to likewise communicate these to the Base Station.

The Base Station SHALL contain a programmable, flexible and responsive computational capability to process data gathered by itself or its CPEs related to the detection of incumbent signals.

It is expected that the algorithms, which determine the response of the WRAN system to the detection of incumbent signals, will mature and be changed over time.

The Base Station SHALL incorporate the capability to easily and securely make changes in these algorithms without an interruption in service.

At a minimum, the proposed WRAN system SHALL contain and function efficiently using these features:

If a television signal is detected at either the Base Station or one (or more) of its CPEs:

The Base Station SHALL modify system operating parameters as required to avoid harmful interference, up to and including ceasing operations.

However, if the proposed WRAN system has indisputable knowledge of the geographic location of the television station transmitter which has been detected, the Base Station MAY make a determination that the Base Station or its associated CPEs are outside the boundaries of the television Grade B/noise limited protected areas and to adjust power, frequency, etc. accordingly to adhere to D/U levels at the boundary specified in paragragh 15.1.2. If the WRAN system incorporates the latter capability, the proposal SHALL specify how the current location of the television station was determined and updated in timely manner and how this information is used in the establishment and setting of WRAN system transmitting parameters.

If a Part 74 device is detected:

The Base Station SHALL modify the system operating parameters of all network members within a specified radius of the reporting device as required to avoid harmful interference, up to and including ceasing operations.

#### D/U Ratios for Television Broadcasting

The Desired to Undesired ratios (D/Us) necessary to preclude interference to TV broadcast services will depend on the WRAN waveform (i.e., ‘U’)  and the waveform of the TV broadcast service (i.e., ‘D’) .  As an initial step to determine the coexistence feasibility, it was assumed that the impact of the WRAN waveform on TV broadcasting will be similar to that of a DTV signal.   The actual required D/U’s will need to be determined through measurements or simulations.  Proposals SHALL provide information on expected D/U ratios for interference into incumbent systems for their particular waveform(s).

#### Sensing Thresholds

The base station and the CPEs SHALL sense licensed transmissions using an omni-directional antenna with a gain of 0 dBi or greater (where all losses between the antenna and the input to the receiver are included) in any azimuthal direction and polarization. The base station SHALL take appropriate actions to assure that the network and all of its components will behave in a manner that mitigates any interference that would result if licensed signals are detected above the following thresholds, referenced to the receiver input:

(1)	DTV threshold: -116 dBm (total ATSC DTV power in the 6 MHz channel). For example, this could be done for the ATSC DTV system by using narrowband spectrum analysis techniques to sense the pilot carrier of the DTV signal which is at –11.3 dB below the total DTV power.  Different threshold values may be needed to protect the various digital TV systems.

- Analog TV threshold: -94 dBm (measured at peak of sync of the NTSC picture carrier).  Different threshold values may be needed to protect the various analog TV systems.

- Wireless microphone threshold: -107 dBm (measured in 200 kHz bandwidth) for operating microphones.

Proposers SHALL provide detailed descriptions of how they will meet the requirements of this section, along with supporting analysis and, if appropriate, simulations.  Proposers are encouraged to propose solutions that will exceed the minimum requirements outlined herein and provide supporting analysis and/or simulations. (e.g. better sensing capabilities such as the ability to achieve lower thresholds, the flexibility to adapt sensing mechanisms to recognize various signal “signatures” as needed to function in an evolving environment and make such information available to the base station, etc.).

### DFS Timing Requirements

The Dynamic Frequency Selection (DFS) timing requirements to avoid interference to incumbent licensed services from WRAN system operation are described below.  The parameters used are first defined; specific values are given in Table 15.1.2; and the needed DFS operational behaviour of the WRAN base station and CPEs are then described.

Incumbent detection: the action of detecting the presence of a licensed incumbent operation in a given TV channel by recognizing the incumbent signal signature and measuring the signal level as being above the Incumbent Detection Threshold.

Incumbent Detection Threshold: is the received power level in dBm to be used by the DFS function to detect licensed incumbent signals (see values in Table 15.1.2).

Channel Availability Check Time: the time during which a TV channel SHALL be checked for the presence of licensed incumbent signals having a level above the Incumbent Detection Threshold prior to the commencement of WRAN operation in that channel, and additionally in the case of TV, a related channel at an EIRP level that can affect the measured TV channel.  The Channel Availability Check Time SHALL conclude no more than 6 seconds prior to the start of the Channel Setup Time period. WRAN transmissions SHALL NOT occur during this time.

Non-Occupancy Period: the required period during which WRAN device transmissions SHALL NOT occur in a given TV channel because of the detected presence of an incumbent signal in that channel above the Incumbent Detection Threshold, and additionally in the case of TV, above a given EIRP level (see section 15.1.6) on a  related TV channel.  A WRAN device SHALL NOT use this channel or, in the case of TV, SHALL NOT utilize its full rated EIRP in the given channel.

Channel Detection Time: the maximum time for a WRAN device to detect a licensed incumbent signal, commencing at the time at which incumbent signal exceeds the Incumbent Detection Threshold within a given TV channel during normal WRAN operation.

Channel Setup Time: the window of time that may be taken by a WRAN CPE to transmit control information to a WRAN base station in order to establish operation with that base station at the prescribed power or, in the case of TV, at or below the allowable EIRP within a given TV channel (see section 15.1.6).  The In-Service Monitoring period SHALL start once the Channel Setup Time period has ended.

Channel Opening Transmission Time: the aggregate duration of control transmissions by WRAN devices during the Channel Setup Time, which starts at the end of the Channel Availability Check Time.  The aggregate duration of all transmissions SHALL NOT count quiet periods in between transmissions.

Channel Move/EIRP Reduction Time: the time taken by a WRAN system to cease all interfering transmissions on the current TV channel upon detection of a licensed incumbent signal above the relevant Incumbent Detection Threshold, or in the case TV, to alternatively reduce its EIRP to that which is allowable within a given TV channel upon detection of a TV signal in the same or a related channel (see. section 15.1.6).

Channel Closing Transmission Time: the aggregate duration of control transmissions by the WRAN devices during the Channel Move/EIRP Reduction Time, which starts upon detection of a licensed incumbent signal above the relevant Incumbent Detection Threshold.  The aggregate duration of all transmissions using the initial power level SHALL NOT count quiet periods between transmissions.

In-Service Monitoring: a mechanism to check a TV channel in use by WRAN devices for the presence of a licensed incumbent signal above the relevant Incumbent Detection Threshold during normal WRAN operation.

| DFS Parameter | Value for Part 74 Devices | Value for TV Broadcasting |
| --- | --- | --- |
| Channel Availability Check Time | 30 sec (recommended) | 30 sec (recommended) |
| Non-Occupancy Period (minimum) | 10 minutes (recommended) | 10 minutes (recommended) |
| Channel Detection Time | <=2 sec to >=90% Probability of Detection with a False Alarm rate of <=10% | <=2 sec to >=90% Probability of Detection with a False Alarm rate of <= 10% |
| Channel Setup Time | 2 sec | 2 sec |
| Channel Opening Transmission Time (Aggregate transmission time) | 100 msec | 100 msec |
| Channel Move Time (In-service monitoring) | 2 sec | 2 sec |
| Channel Closing Transmission Time (Aggregate transmission time) | 100 msec | 100 msec |
| Incumbent Detection Threshold | -107 dBm (200kHz BW) | -116 dBm (6 MHz BW) for DTV |

Table 15.1.2: DFS-like parameters for sensing and vacating channels used by incumbent services in the US

General operational behaviours of WRAN devices implementing DFS for licensed incumbent protection are outlined below.

Base Station Devices

- All WRAN Base Stations SHALL use DFS to sense, detect, and avoid interference to licensed incumbent services whose signals are sensed above the relevant Incumbent Detection Thresholds as given in Table 15.1.2.

- The base station initiates  WRAN operation by transmitting short control signals during the Channel Setup Time that will enable CPEs to associate with the network.  The aggregate transmissions during the Channel Setup Time SHALL be limited to the Channel Opening Transmission Time.

- Before initiating  WRAN operation on a channel, the Base Station SHALL perform a Channel Availability Check for the duration of the Channel Availability Check Time to ensure there are no licensed incumbent signals operating on that channel at received signal levels above the relevant Incumbent Detection Thresholds.  In the case of TV, the Base Station SHALL perform a Channel Availability Check on the potentially affected channels for the duration of the Channel Availability Check Time to adjust the maximum level of EIRP that can be used in this channel depending on the presence of DTV signals operating on the potentially affected channels (see. section 15.1.6)..

- During normal operation, the Base Station SHALL monitor the potentially affected channels (In-Service Monitoring) at intervals sufficient to meet the Channel Detection Time

- If the Base Station or any CPE detects a licensed incumbent signal above the relevant Incumbent Detection Threshold, the Base Station SHALL modify the system operating parameters of all network members within a specified radius of the reporting device as required to avoid harmful interference, up to and including ceasing operations entirely.

CPE Devices

- All CPE devices operating in the TV bands SHALL use DFS to sense, detect, and  report on  signals above the relevant Incumbent Detection Thresholds in the operating channel and in the first adjacent channels to the channel occupied by the WRAN and, in the case of TV,  all potentially affected channels (see section 15.1.8) at intervals sufficient to meet the Channel Detection Time (In-Service Monitoring).

- A CPE device SHALL reduce its transmit power or stop all transmissions whenever instructed by a Base Station.  The CPE device SHALL NOT increase its transmit power or resume transmissions respectively until it has again received an enabling signal from a Base Station.

- The amount of time the CPE device can take to detect a licensed incumbent signal at received signal levels above the relevant Incumbent Detection Threshold is the Channel Detection Time.

### Distributed Sensing

802.22 systems SHALL employ distributed sensing.

The base station and CPEs SHALL have the capability to sense RF channel occupancy.  Information will be sent and collated at the base station where mapping of the interference situation will be made based on the sensing information from the CPEs and according to their respective geographic coordinates.  Solutions to avoid interference will be deduced at the base station and measures will be signaled to the CPEs such as channel change or power reduction.

The intelligence for the distributed sensing system SHOULD be located at the base station or MAY alternatively be centralized at a Network Operating Center (NOC).  If the base station operation is dependent on connectivity to the NOC, then the base station SHALL cease operation if connectivity to the NOC is lost.

The MAC and PHY protocols SHALL allow the transmission of the necessary control packets in both directions to permit such distributed sensing and the CPEs SHALL react in a timely manner to commands from the base station.

The MAC SHALL support control packets to and from higher layers to assure protection of incumbent services at the network level.

### Channel Utilization Table

The system SHALL provide a way to populate a table of channels that characterizes channel availability – such as disallowed, occupied, available, available at reduced power, etc., and to update that table at any time.

Disallowed channels SHALL not be used and therefore will not need to be sensed by the system.

Updating of the table MAY occur either as a result of data entry by a system operator or as an automatic function of the network sensing mechanism, which will result in non disallowed channels being designated as either occupied, available, available at reduced power, etc.

Such automatic updating by the sensing mechanism SHALL operate utilizing parameters as specified in section 15.1.2.

The base station, and optionally the CPEs, SHOULD maintain a "list" of backup channel(s) in case a new incumbent is detected and a channel has to be quickly vacated.  The exact scheme can be left to the proposers, as different approaches could be suggested to address this issue.

### Minimum separation distances between WRAN base stations and TV operation

Proposers should be mindful in terms of system architectural considerations that, in the interest of coexistence and avoidance of interference to incumbent TV operations, WRAN base stations will have to be located outside the  noise-limited protected/Grade B contour of TV stations that use the same channel or first adjacent channels.

Proposers should also note that exact required separation distances from the noise-limited protected/Grade B contour of TV stations that use the same channel or first adjacent channels may vary in different regulatory domains.

In the case of second adjacent channels and beyond, the WRAN base station will need to be located at some minimum distance from the closest TV receiver (see Section 11).

### Maximum power for WRAN CPEs to avoid interference to TV operation

The interference potential to a TV receiver resulting from a transmission from a CPE is analyzed under the following conditions:

- 4 Watt maximum CPE transmit EIRP;

- CPE antenna mounted 10 m above the ground;

- A separation distance of 10 m between the CPE and DTV receiving antennas;

- In the case of co-channel and first adjacent channel operation, both antennas are assumed to be looking away from each other since the base station has to be located at a certain distance outside the Grade B contour of the DTV station as indicated in the previous section.  The back-lobe rejection of both antennas can therefore be relied upon.

- Main beam coupling will exist between the antennas for channels N±2 and beyond inside the Grade B contour.

- For channels N±2 and beyond, polarization discrimination is the only way to increase isolation based on the fact that the CPE transmit antenna and the DTV receive antenna will be orthogonally polarized;

- Some antenna polarization discrimination MAY be assumed and proposers SHOULD document their assumptions in this regard.

- The possibility of some signal depolarization between the two antennas SHOULD be considered as well.

- Chassis radiation is assumed to be and SHOULD be insignificant (e.g. >= -20 dB) compared to radiation from the intended path via the antenna.

WRAN systems will have to have cognisance of the presence of TV broadcast stations in the surrounding area over a range that could extend up to +-15 channels. Accordingly, the maximum transmitted CPE EIRP will have to be limited (by capping the TPC range) to a value that is related to the channel separation between WRAN and TV operations.   Actual values will be defined later.  The D/U values available for deducing these limits for DTV are contained in ATSC A-74 (available at http://www.atsc.org).  Further analysis and testing will be needed to come up with the D/U values for signal levels representative of reception at the noise-limited/Grade B contour. (For an example of a related parametric study, see document 22-05-0002-12-0000 (revision 12, or later).

### Out-of-band emission mask for WRAN systems

According to the model of 15.1.6, it is tentatively concluded that in order to protect DTV receivers and to protect wireless microphones, 4 W EIRP WRAN CPEs and base stations SHALL meet the limits specified in Table 15.1.7.

|  | If WRAN operates |  |
| --- | --- | --- |
|  | First adjacent channel to wireless microphone | Second adjacent channel and beyond to TV or wireless microphone |
| WRAN first adjacent channel limit | 4.8 uV/m | 200 uV/m |
| WRAN second adjacent channel and beyond limit | 4.8 uV/m | 4.8 uV/m* |

Table 15.1.7:  Emission levels (measured at 3 m in 120 kHz bandwidth)

* This number is derived from a 20 dBuV/m field strength at 10m, measured in 6 MHz. and assumes no polarization discrimination.   If a proposer suggests a relaxation based on the use of polarization discrimination or some other approach, they SHALL provide supportive documentation to justify such a relaxation.

## WRAN systems coexistence/sharing

### Coexistence among WRAN systems

Due to the proposed license-exempt (or, perhaps some form of “light licensing” for base stations) nature of WRAN service that is expected, it is likely that more than one WRAN base station may operate in the same geographic area.

The 802.22 protocols SHALL allow multiple 802.22 WRAN systems with service area overlap, whether operated by the same operator or different operators, and whether co-channel or operating on adjacent channels.

Sensing techniques SHALL be operable in the presence of other WRAN systems operating co-channel or on adjacent channels, both for the purpose of protecting incumbent licensed operation and for facilitating sharing and coexistence between multiple WRAN systems.

802.22 systems SHALL have means to coordinate with each other to facilitate sharing, coexistence, and interference mitigation amongst neighbors.

Proposers SHALL elaborate via narrative descriptions, analysis, and simulation as applicable, the performance of their systems in terms of sensing and coordinating with other WRAN systems.

### Unique Address Space/Node Identification

Each WRAN system MUST have a unique address space in order to ensure that interference resolution is always possible.

The IEEE 802.22 protocols SHALL use IEEE standard 48-bit addresses, in accordance with the applicable IEEE standards.

Each WRAN base station and CPE SHALL, in each transmission, transmit sufficient identifying information to facilitate the prompt resolution of any interference that might occur.

### Distributed Sensing

802.22 WRAN base stations and CPEs SHALL use the same fundamental distributed sensing mechanism and base station to CPE master-slave relationship used to protect incumbent services to detect and identify other WRAN systems operating co-channel or on adjacent channels to facilitate sharing, coexistence, and interference mitigation amongst neighbors.  (See also 15.1.3)

Proposers SHALL propose, and justify through analysis and simulation, the sensing thresholds required.

### Channel utilization databases

Each 802.22 WRAN base station (and optionally CPEs) SHALL maintain a table of frequencies used by other WRAN systems operating in the same geographic area, and SHALL use that information to facilitate sharing, coexistence, and interference mitigation amongst neighbors.

### Inter-Base Station Coordination

Proposers SHALL elaborate how their proposed system will meet the requirements of  Section 15.2 by outlining how WRAN base stations will communicate to resolve coexistence issues (over the air, via “backhaul”, over the internet, etc.)  in sufficient detail, and with performance metrics such as delay, latency, etc. to allow the proposed mechanisms to be evaluated quantitatively and qualitatively by the members of the Working Group.

## Coexistence/sharing with other license-exempt systems

Proposers SHOULD  propose and elaborate potential methods to address coexistence with non-802.22-based license-exempt systems that might in the future be allowed access to the TV bands.

# Network Management

As outlined in IEEE Std 802-1990 [4], the LLC Sublayer, MAC Sublayer and Physical Layer standards also include a management component that specifies managed objects and aspects of the protocol machine that provide the management view of managed resources. The aspects of management considered are (FCAPS):

- Fault management

- Configuration management

- Accounting management

- Performance management

- Security (see also section 13)

The 802 standards define a framework for LAN/MAN management in ISO/IEC 15802-2:1995(E) [8]. The framework contains guidelines for managed objects, management protocol, and the relationship to ITU management protocols. 802.22 protocols SHALL comply with the above-mentioned standards and guidelines.

## Support for Service Level Agreements

The 802.22 protocols MUST permit operators to enforce service level agreements (SLAs) with subscribers by restricting access to the air link, discarding data, dynamically controlling bandwidth available to a user or other appropriate means. The 802.22 protocols MUST also permit subscribers to monitor performance service levels of the 802.22 services being provided at the delivery point.

## Support for Admission Control

The 802.22 standard SHALL support a flow by flow admission control mechanism to support  QoS.

## Support for Accounting and Auditing

The 802.22 system management framework, architecture, protocols, and managed objects MUST allow for operators to effectively administer accounting and auditing. An operator MUST be able to account for resource utilization and various service features for each subscriber service separately.

## Malfunctioning CPE or Base Station

The WRAN system operator MUST have secure means to shut down and reactivate a CPE, if necessary, from the WRAN base station (or NOC). The operator also MUST have the means to securely shut down and reactivate a base station remotely.

The 802.22 protocols SHALL support  fail-safe functions that automatically shut down transmission at a CPE or base station in case of malfunction (e.g., power limits exceeded).

# Security

The 802.22 system SHALL enforce security procedures described in this section.

The security system chosen by 802.22 SHALL be added to the protocol stack and reference points to include security protocols, and “database” functions for authentication, authorization, key management, service suspend/resume, relocation, anti-cloning, etc.

## Firmware Protection

The WRAN firmware SHALL meet regulatory requirements to be tamper-proof to prevent unauthorized modification to firmware and/or functionalities (e.g., cognitive functionality, RF sensing, DFS, TPC, tuning). Any attempt to load unapproved firmware into a WRAN device shall render it inoperable.

Proposers SHALL elaborate how they propose to meet this requirement.

## Authentication for network access

There SHALL be two types of authentication for an 802.22 system.

In the first type, a CPE MUST authenticate itself with the network every time it registers with the network. This authentication MUST prevent unauthorized CPE from entering the network or an unauthorized base station from emulating an authorized base station. This type of authentication MUST be supported by the 802.22 MAC layer.

The second type of authentication is between the subscriber and the WRAN system. This may or may not be the responsibility of the 802.22 protocols. It MAY be handled by higher layer protocols.

An additional level of authentication MAY exist between the other two. This additional layer is the authentication of the subscriber with the subscriber station. This is beyond the scope of the 802.22 protocols.

The authentication mechanisms MUST be secure so that an unauthorized or “rogue” CPE is not able to gain access to an 802.22 system, or to the core network beyond. Passwords and keying information MUST NOT be passed “in the clear” through the air interface.

## Authorization (Access control)

Authorization is a security process that determines what services an authenticated subscriber is permitted to invoke. Each subscriber has a set of credentials that describe what the subscriber is “allowed” to do. The 802.22 standard SHALL identify a standard set of credentials and allow for vendors to extend the defined credentials with non-standard credentials. Some possible credentials are:

- Permission to access the 802.22 system

- Permission to access certain services (IP, Remote Bridging, Digital Audio/Video, etc.)

- Permission to request specific service features, and/or QoS parameters, based on the SLA (resources, delay, etc.).

For each service offered, the 802.22 protocols MUST be capable of securely supporting authorization requests and responses.

## Privacy

Privacy is a security concept that protects transmitted data from being intercepted and understood by third parties (e.g., an unauthorized or “rogue” CPE, base station or passively “listening” radio).

The 802.22 WRAN standard SHALL allow suitable cryptographic algorithms to be employed.

Facilities SHALL be defined in the protocol for the use of alternate cryptographic algorithms that can be used in varying localities and that can replace algorithms as they are obsoleted.

Over the air rekeying (OTAR) of cryptographic functionality SHOULD be supported and proposers are expected to provide sufficient detail on this aspect of their proposed approach, as well as the basic cryptographic functionality, to allow the feasibility and efficacy of the proposed methods to be evaluated.

### Message integrity

The 802.22 WRAN protocols SHALL assure message integrity, including the accuracy of delivered data and that the data has not been altered in an unauthorized manner.

### Data (payload) encryption

The 802.22 WRAN protocols SHALL protect the privacy and security of user payload data transmitted over the system by employing suitable cryptographic algorithms to encrypt and verify the integrity of such data.

### Protection of network control information

The 802.22 WRAN protocols SHALL protect network control message data transmitted over the system by employing suitable cryptographic algorithms to encrypt and verify the integrity of such data.

## Protection against Denial of Service and other attacks

Proposers SHALL provide a description of how their proposed approach will protect against denial of service attacks and other malicious actions intended to disrupt system operation.

# 802 Conformance

The 802.22 Standard SHALL conform to the 802 architecture.

The following  particulars with the 802 model (see IEEE Standards for Local and Metropolitan Area Networks: Overview and Architecture (IEEE Std 802-1990) [11]) SHALL apply:

- The 802.22 MAC SHALL support 802 IEEE standard 48 bit addresses, in accordance with the applicable IEEE Standards.

- An 802.22 system SHALL support MAC multicast in the downstream direction.

- The 802.22 protocols SHALL support 802.1 bridging services and protocols, including support of the 802.1q virtual LAN tag and 802.1D priority ID [32-34].

- The 802.22 protocols SHALL support encapsulation of 802.2 (LLC) [3] by the MAC protocol.

- The 802.22 protocols  SHALL conform to the 802 conventions and structures for “interface primitives” (logical structures that are passed between protocol layers to invoke processes and transact data).

- The 802.22 protocols  SHALL conform to the 802 system management guidelines (see section 8) [9].

- The 802.22 MAC  SHALL provide a MAC service interface that complies to 802 conventions [5].

# References

[1]	IEEE Standards Association. Project Authorization Request. Information Technology -Telecommunications and information exchange between systems – Wireless Regional Area Networks (WRAN) - Specific requirements - Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Policies and procedures for operation in the TV Bands  http://www.ieee802.org/22/802-22_PAR.pdf

[2]	Recommendation ITU-R F.1399. Vocabulary of terms for wireless access.

[3]	ISO/IEC 8802-2:1998. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 2: Logical Link Control.

[4]	IEEE Std 802-1990 "IEEE Standards for Local and Metropolitan Area Networks: Overview and Architecture. IEEE 1990.

[5]	ISO/IEC 10039: 1991. Information technology -- Open Systems Interconnection -- Local area networks -MAC service definition.

[7]	RFC-2205 Resource ReSerVation Protocol (RSVP) -- Version 1 Functional Specification. R. Braden, Ed., L. Zhang, S. Berson, S. Herzog, S. Jamin. September 1997. Status: PROPOSED STANDARD.

[8]	ISO/IEC 15802-2:1995. Information technology -- Telecommunications and information exchange between systems -- Local and metropolitan area networks -- Common specifications -- Part 2: LAN/MAN management.

[9]	IEEE 802.1F-1993. IEEE Standards for Local and Metropolitan Area Networks: Common Definitions and Procedures for IEEE 802 Management Information.

# Appendix A
Vocabulary of Terms

Editor’s Note: Some definitions below are as yet undefined. And a few other are as yet incomplete or in-progress. The 802.22 task group should provide updates for these definitions and release a new version of this document. The in-progress definitions are in italics.

| Term | Definition | Reference |
| --- | --- | --- |
| Adaptive Array | A group of antennas in which the relative phases of the respective signals feeding the antennas are varied in such a way that the effective radiation pattern of the array is reinforced in a desired direction and suppressed in undesired directions. |  |
| Admission Control | The network’s method of controlling access to the radio interface and higher protocol layers. |  |
| Air Interface | The radio-frequency portion of the circuit between the customer premises equipment and the active base station. |  |
| Audio Carrier | In analog broadcast television, the audio signal is normally modulated separately from the video signal and combined at the transmitter. The audio carrier presents a distinct spectral line in the broadcast spectrum. |  |
| Authentication | The process of verifying the identity of a user, terminal, or service provider. | Rec. ITU-R M.1224 |
| Authorization | A property by which the rights to resources are established and enforced. | Rec. ITU-R M.1224 |
| Backhaul service | Transport of aggregate communication signals from base stations to the core network. | IEEE 802.22 |
| Backup Channel | A recommended approach where the base station and optionally the CPE maintain a list of channels that may be used in the event that the current channel must be vacated. |  |
| Bandwidth; communication channel bandwidth | The information payload capacity of a communication channel available to a user for services (expressed in bit/s or multiples thereof). |  |
| Bandwidth Reservation | A means to address channel capacity issues and changes in channel capacity. |  |
| Base station | The common name for all the radio equipment located at one and the same place used for serving one or several cells. (See also “station”). | ITU-R Rec. M.1224 |
| Bridged LAN | LAN bridging service capabilities | IEEE 802.1 |
| Broadband wireless access | Wireless access in which the connection(s) capabilities are higher than the primary rate. | Rec. ITU-R F.1399 |
| Broadcast Control Packets | Information packets broadcast by a base station to assist CPE in making network association attempts. |  |
| Cell | The radio coverage area of a base station, or of a subsystem (e.g. sector antenna) of that base station corresponding to a specific logical identification on the radio path, whichever is smaller. | Based on Rec. ITU-R M.1224 |
| Cell delay variation | A component of cell transfer delay induced by buffering and cell scheduling. The proportion of lost cells over the total number of transmitted cells for a connection. | ATM Forum |
| Channel Availability Check Time | The time during which a TV channel SHALL be checked for the presence of licensed incumbent signals having a level above the Incumbent Detection Threshold prior to the commencement of WRAN operation in that channel, and in the case of TV, a related channel at an EIRP level that can affect the measured TV channel. |  |
| Channel bonding | A method of spectrum utilization where two or more channels are permanently associated with each other. |  |
| Channel capacity | The limit of communications traffic that may be transported in a given transmission channel. |  |
| Channel Closing Transmission Time | The aggregate duration of control transmissions by the WRAN devices during the Channel Move/EIRP Reduction Time, which starts upon detection of a licensed incumbent signal above the relevant Incumbent Detection Threshold. |  |
| Channel Detection Time | The maximum time taken by a WRAN device to detect a licensed incumbent signal above the Incumbent Detection Threshold within a given TV channel during normal WRAN operation. |  |
| Channel Move/EIRP Reduction Time | The time taken by a WRAN system to cease all interfering transmissions on the current TV channel upon detection of a licensed incumbent signal above the relevant Incumbent Detection Threshold, or in the case TV, to alternatively reduce its EIRP to that which is allowable within a given TV channel upon detection of a TV signal in the same or a related channel (see. section 15.1.6) |  |
| Channel Opening Transmission Time | The aggregate duration of control transmissions by WRAN devices during the Channel Setup Time, which starts at the end of the Channel Availability Check Time. |  |
| Channel Setup Time | The window of time that may be taken by a WRAN CPE to transmit control information to a WRAN base station in order to establish operation with that base station at the prescribed power or, in the case of TV, at or below the allowable EIRP within a given TV channel (see section 15.1.6). |  |
| Class of Service | A means of unidirectional transmission of signals between two points. | ITU-T Rec. I.112 |
| Cognitive radio | A radio communication technology where either the network or a wireless node itself changes particular transmission or reception parameters to fulfill specific tasks that ultimately enable more efficient use of the spectrum. |  |
| Core network | This is either a Customer Class of Service, a Trunk Class of Service, or a Private-Facility Class of Service. It may refer to either originating or terminating accesses. Core networks include, for example, PSTN, ISDN, PLMN, PSDN, Internet, WAN/LAN, CATV, etc. | ITU-T Rec. Q.1290 (95), 2 Based on Rec. ITU-R F.1399 |
| Customer premises equipment/network | The equipment/network administered by the user. | Based on ITU-T Rec. H.310 |
| Desired to Undesired Ratios | The ratio of the desired signal to an interfering or undesired signal used to determine service contours. |  |
| Disallowed channels | Predefined channels within the WRAN network that are not available for use by the network. |  |
| Distributed sensing | The use of all available base stations and CPEs to sense and report on detected licensed incumbent emissions. |  |
| Dynamic Frequency Selection | A mechanism to avoid interference to incumbent licensed services. |  |
| Dynamically variable bandwidth | A capability of a system to be able to change the bandwidth of the information payload capacity of a communication channel available to a user for services according to negotiated user requirements. |  |
| External Network | Any network that is physically located past the base station or the CPE. |  |
| F(X,Y) | A propagation statistic that represents a field strength for X % of locations and Y% of time. (see F(50,50), etc. for definitions of specific functions applicable to TV and WRAN operations) |  |
| F(50,10) | The geographic points at which the field strength predicted for 50% of locations and 10% of time by FCC curves is at least as great as the values given for the appropriate D/U ratio. This is typically used as a criteria to define interference contours. | OET-69 |
| F(50,50) | The geographic points at which the field strength predicted for 50% of locations and 50% of time by FCC curves is at least as great as the values given for the appropriate D/U ratio. The relevant curves for predicting these fields are the F(50, 50) curves found in Section 73.699 of FCC rules, 47 C.F.R. § 73.699. | OET-69 |
| F(50,90) | For digital television stations, service is evaluated inside contours determined by DTV planning factors in combination with field strength curves derived for 50% of locations and 90% of the time from curves which are also found in Section 73.699 of FCC rules. | OET-69 |
| F(50,99.9) | For WRAN, the typical range of the system at 50% location availability at the edge of the coverage area for a median location and 99.9% time availability. |  |
| Fixed wireless access | Wireless access application in which the base station and the subscriber station are fixed. | Based on Rec. ITU-R F.1399 |
| Frequency Division Duplex | Separation of upstream and downstream transmission in the frequency domain at the same time. | IEEE 802.22 |
| Grade A Contour | The approximate extent of coverage over average terrain in the absence of interference from other television stations as defined by the f(50,50) D/U ratios for Grade A service. Only applicable to NTSC systems. |  |
| Grade B Contour | The approximate extent of coverage over average terrain in the absence of interference from other television stations as defined by the f(50,50) D/U ratios for Grade B service. Only applicable to NTSC systems. |  |
| Incumbent Detection | The action of detecting the presence of a licensed incumbent operation in a given TV channel by recognizing the incumbent signal signature and measuring the signal level as being above the Incumbent Detection Threshold. |  |
| Incumbent Detection Threshold | The received power level in dBm to be used by the DFS function to detect licensed incumbent signals. |  |
| Incumbent Licensed Services | Licensed services in the TV bands that include broadcast TV as well as Part 74 Low Power Auxiliary Station devices. |  |
| In-Service Monitoring | A mechanism to check a TV channel in use by WRAN devices for the presence of a licensed incumbent signal above the relevant Incumbent Detection Threshold during normal WRAN operation. |  |
| Internet protocol | Networking protocol defined by IETF standards. | IETF |
| Interoperability | The ability of multiple entities in different networks or systems to operate together without the need for additional conversion or mapping of states and protocols. | Rec. ITU-R M.1124 |
| Inter-working | The means of supporting communications interactions between entities in different networks or systems. | Rec. ITU-R M.1124 |
| Inter-working function | Mechanism which masks the differences in physical, link, and network technologies by converting or mapping states and protocols into consistent network and user services. | Rec. ITU-R M.1124 |
| Link Budget | The allocation of power within a link. Link budget consists of the difference between Tx power and receiver sensitivity. |  |
| Medical Telemetry Systems | Medical telemetry and remote control systems are the only unlicensed devices authorized to operate in the TV bands under Part 15 rules. |  |
| Multi-tenant buildings | A primarily non-commercial building designed to contain more than two residential units. |  |
| Network | A set of nodes and links that provides connections between two or more defined points to facilitate telecommunication between them. | Rec. ITU-R M.1224 |
| Noise-limited contour | For digital television stations, service is evaluated inside contours determined by DTV planning factors in combination with field strength curves derived for 50% of locations and 90% of the time from curves that are also found in Section 73.699 of FCC rules. |  |
| Non-Occupancy Period | The required period during which WRAN device transmissions SHALL NOT occur in a given TV channel because of the detected presence of an incumbent signal in that channel above the Incumbent Detection Threshold, or in the case of TV, above a given EIRP leve |  |
| On demand communication | Use of the wireless access system for services based on negotiated QoS. |  |
| OFDM | Orthogonal Frequency Division Multiplexing |  |
| Peak Data Rate | The maximum bit rate available to a user for communications. |  |
| Pilot | A narrow band signal used by a receiver to recover carrier frequency in support of demodulation. It may also be used to indicate the presence of a certain transmission format (e.g., ATSC DTV Pilot). |  |
| Point-to-multipoint system | A system that establishes connections between a single specified point and more than one other specified points. | ITU-R Rec. F.1399 |
| Privacy | The provision of capabilities to prevent access of information by unauthorized parties. | ANSI T1.702-1995 |
| Quality of service | The collective effect of service performance that determine the degree of satisfaction of a user of the service. NOTE 1 - The quality of service is characterized by the combined aspects of service support performance, service operability performance, serviceability performance, service security performance and other factors specific to each service. NOTE 2 - The term "quality of service" is not used to express a degree of excellence in a comparative sense nor is it used in a quantitative sense for technical evaluations. In these cases a qualifying adjective (modifier) should be used. | ITU-T Rec. E.800 (94), 2101 |
| Real-Time | Pertaining to the processing or transmission of data according to defined time requirements . | Based on ITU-T Rec. Q.9 (88), 6103 |
| Sector | As yet undefined |  |
| Security | The protection of information availability, integrity and confidentiality, as well as authentication and authorization. | Based on Rec. ITU-R M.1224 |
| Semi-stateless | As yet undefined | Sect 7 |
| Sense antenna | An antenna used in a WRAN CPE distributed sensing function to detect incumbent licensed services or other WRAN devices in order to avoid interfering with those systems. |  |
| Service | A set of functions offered to a user by an organization. | Recs. ITU-R M.1224, M.1308 |
| Single Family Residential | A non-commercial building intended to be inhabited by one or more individuals. |  |
| Small Business | As yet undefined |  |
| Small Office/Home Office | As yet undefined |  |
| Spectrum (Spectral) Efficiency | A measure of a system’s efficiency of spectral use, based on the net delivered data rate per unit bandwidth, expressed in bits/sec/Hz. |  |
| Standard | A document established by consensus and approved by a recognized body, that provides, for common and repeated use, rules, guideline or characteristics for activities or their results, aimed at the achievement of the optimum degree of order in a given context. | ISO/IEC Guide 2, 1996 |
| Station | The common name for all the radio equipment at one and the same place. NOTE - The term "station" may refer to any end user radio equipment (“subscriber station”) or network radio equipment (“base station”). | Rec. ITU-R M.1224 |
| Subscriber | A person or other entity that has a contractual relationship with a service provider on behalf of one or more users. (A subscriber is responsible for the payment of charges due to that service provider.) | Rec. ITU-R M.1224 |
| System | A regularly interacting or interdependent group of items forming a unified whole technology. | Recs. ITU-R M.1224, M.1308 |
| Time Division Duplex | Separation of upstream and downstream transmission in the time domain using the same frequency. | IEEE 802.22 |
| Transmit Power Control | To permit transmission at full power limits when necessary, but constrain the transmitter power to a lower level to allow greater sharing of spectrum when higher power operation is not necessary. |  |
| Upstream | The direction from subscriber station(s) to base station. | IEEE 802.22 |
| User | Any entity external to the network that utilizes connections through the network for communication. | ITU-T Rec. E.600 |
| Wireless access | End-user radio connection(s) to core networks. NOTE 1 - Core networks include, for example, PSTN, ISDN, PLMN, PSDN, Internet, WAN/LAN, CATV, etc. NOTE 2 - The end-user may be a single user or a user accessing the services on behalf of multiple users. | Rec. ITU-R F.1399 |
| Wireless microphones | Part 74 licensed devices that typically use FM modulation to convey audio to a receiver. Digital modulation may also be used. |  |

Appendix B

# Acronyms and Abbreviations

| Acronym/Abbreviation | Definition |
| --- | --- |
| ADSL | Asymmetric Digital Subscriber Loop |
| ANSI | American National Standards Institute |
| ATSC | Advanced Television Systems Committee |
| BER | Bit Error Ratio |
| BS | Base Station |
| CNI | Core Network Interface |
| CoS | Class of Service |
| CPE | Customer Premises Equipment |
| DFS | Dynamic Frequency Selection |
| DTV | Digital Television |
| EIRP | Effective Isotropic Radiated Power |
| FCAPS | Fault, Configuration, Accounting, Performance, Security Management |
| FDD | Frequency Division Duplex |
| FEC | Forward Error Correction |
| FWA | Fixed Wireless Access |
| IEC | International Electrotechnical Commission |
| IETF | Internet Engineering Task Force |
| IP | Internet Protocol |
| ISDN | Integrated Services Digital Network |
| ISO | International Organization for Standardization |
| ITU-R | International Telecommunications Union- Radiocommunications Sector |
| IWF | Inter-Working Function |
| LAN | Local Area Network |
| LE | License Exempt |
| LLC | Logical Link Control |
| MAC | Medium Access Control |
| MAN | Metropolitan area network |
| NOC | Network Operations Center |
| NTSC | National Television System Committee |
| OA&M | Operations, Administration, and Maintenance |
| OSI | Open Systems Interconnection |
| PAL | Phase Alternating Line |
| PDU | Protocol Data Unit |
| PER | Packet Error Rate |
| PHY | Physical layer |
| PLMRS | Public Land Mobile Radio System |
| PMD | Physical Medium Dependent |
| P-MP | Point-to-Multipoint |
| PSTN | Public Switched Telephone Network |
| QoS | Quality of service |
| RSVP | Resource reSerVation Protocol |
| SECAM | Sequential Color with Memory |
| SLA | Service Level Agreement |
| SME | Small and Medium Enterprises |
| SOHO | Small Office/Home Office |
| SS | Subscriber Station |
| TDD | Time Division Duplex |
| TPC | Transmit Power Control Conffff |
| UHF | Ultra High Frequency |
| UNI | User Network interface |
| VHF | Very High Frequency |
| VoIP | Voice over IP |
| VSB | Vestigial Side Band |
| WRAN | Wireless Regional Area Network |

# Appendix C
WRAN Channel Model

# The contents of this Appendix are presently in document

# 22-05-0055-07-0000_ WRAN_channel _Modeling.doc

# And will be incorporated herein as approved with only purely editorial/formatting changes such as making section numbers, page numbers, and formatting consistent with this document.

Once the material from document 22-05-0055-07-0000_WRAN_channel_Modeling.doc has been incorporated, the revision number of this document will be “bumped” to r47 and that version will be posted to the IEEE 802.22 website as the final, approved Requirements Document to be used as guidance by proposers.  In the meantime, proposers can use the contents of this document in combination with 22-05-0055-07-0000_WRAN_channel_Modeling.doc.

# Appendix D
TV Broadcast: description and technical characteristics

Channel rasters of 6, 7, 8 MHz. In North America, 6 MHz and high transmit power up to MW.  Typical RF characteristics of receivers is documented in ATSC A-74. , To create international standard, the characteristics, channel raster (6, 7, 8 MHz) and extend of frequency bands tend to change with international regions.

### References: ITU-R BT.470-6, ITU-R BT.655-6, ITU-R BT.417-4

|  |  | Countries |  | Frequencies | Bandwidth | Color carrier | Audio carrier |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Analog | Vestigial Side Band (VSB) | US, Canada, Chile, Costa Rica, Cuba, El Salvador, Guatemala, Honduras, Japan, Mexico, Panama, Philippines, Puerto Rico, South Korea, Taiwan | NTSC M | 54 – 746 MHz (Japan 90 – 770 MHz) | 6 MHz channel: 4.2 MHz video, 250kHz audio | 3.580 MHz above video carrier | 4.5 MHz above video carrier |
|  | Phase Alternating Line (PAL) | Argentina, Australia, Belgium, Brazil, China, Denmark, Finland, Great Britain, India, Indonesia, Ireland, Italy, Norway, New Zealand, the Netherlands, Portugal, Spain, Sweden, Switzerland, Turkey | PAL B | 48 – 798 MHz (New Zealand 44 – 798 MHz, Australia 45 – 806 MHz) | 7 MHz channel: 5.0 MHz video, 250 kHz audio | 4.434 MHz above video carrier | 5.5 MHz above video carrier |
|  |  |  | PAL G | 48 – 798 MHz (New Zealand 44 – 798 MHz) | 8 MHz channel: 5.0 MHz video, 250 kHz audio |  |  |
|  |  |  | PAL H | 47 – 798 MHz | 8 MHz channel: 5.0 MHz video, 250 kHz audio | 4.434 MHz above video carrier | 5.5 MHz above video carrier |
|  |  |  | PAL I | 174 – 798 MHz | 8 MHz channel: 5.5 MHz video, 250 kHz audio |  | 6.0 MHz above video carrier |
|  |  |  | PAL N | 54 – 806 MHz | 6 MHz channel: 4.2 MHz video, 250 kHz audio | 3.582 MHz above video carrier | 4.5 MHz above video carrier |
|  |  |  | PAL M |  |  | 3.576 MHz above video carrier |  |
|  | Sequential Color with Memory (SECAM) | France, Egypt, Germany, Greece, Haiti, Iran, Iraq, North Korea, Poland, the former Soviet Union | SECAM B | 47 – 798 MHz | 7 MHz channel: 5.0 MHz video, 250 kHz audio | N/A | 5.5 MHz above video carrier |
|  |  |  | SECAM G, H |  | 8 MHz channel: 5.0 MHz video, 250 kHz audio |  |  |
|  |  |  | SECAM D, K, K1, L | 470 – 790 MHz | 8 MHz channel: 6.0 MHz video, 250 kHz audio | N/A | 6.5 MHz above video carrier |
|  |  |  |  |  |  | Pilot |  |
| Digital | Vestigial Side Band | North America | ATSC | 54 – 746 MHz | 6 MHz channel | 11.3 dB below the average data signal power |  |
|  | Orthogonal Frequency Division Multiplexing | Europe | DVB-T | 48 – 798 MHz | 7 or 8 MHz channel | N/A |  |
|  |  | China | TDS-OFDM | 48 – 798 MHz | 7 or 8 MHz channel |  |  |
|  |  | Japan | ISDB-T | 54 – 746 MHz | 6 MHz channel |  |  |

[The remainder of the contribution for this section of this Requirements Document is only applicable to the FCC and the conditions found only in the US.  In any other country under any other government regulations, entirely different rules, recommendations, and assumptions are to be applied in our considerations.]

Reference FCC Rules Part 73:

NTSC Field Strength Contours:

This topic is defined in part 73.683 of the FCC rules as follows:

“In the authorization of TV stations, two field strength contours are considered. These are specified as Grade A and Grade B and indicate the approximate extent of coverage over average terrain in the absence of interference from other television stations. Under actual conditions, the true coverage may vary greatly from these estimates because the terrain over any specific path is expected to be different from the average terrain on which the field strength charts were based. The required field strength, F(50,50), in dB above one micro-volt per meter (dBu) for the Grade A and Grade B contours are as follows:

|  | Grade A (dBu) | Grade B (dBu) |
| --- | --- | --- |
| ###### Channels 2-6 | 68 | 47 |
| Channels 7-13 | 71 | 56 |
| Channels 14-69 | 74 | 64 |

“The Grade A and Grade B contours are only applicable to the NTSC service.  The different grade levels of the quality of received NTSC television service were determined through subjective tests where human subjects provided a rating of the quality of a television picture with which they were presented.

The concept of a Grade B contour has no meaning when considering the DTV service.  Because of the digital nature of the DTV service, a receiver either can acquire and tune to a signal or it cannot.  Therefore there is no “Grade B.”

However, the determination of the bounds of a station's DTV service area was determined through the consideration of the station's Grade B contour.  Defining the DTV service area involved an attempt to match the area of service to the station's associated to its Grade B contour as much as possible.  In doing so, a station's allowed transmit power was regulated while assuming a new set of field strengths, listed below, and a different time availability, F(50,90).

After the station's DTV service areas were determined and a new table of allotments was created, some stations were allowed to extend their service contour or “maximize” their service.  A station was allowed to transmit at a maximum EIRP of 1 MW, as long as this maximization did not cause greater than regulated de minimis interference.

DTV Service Areas:

This topic is defined in part 73.622(e) of the FCC rules as follows:

“(1) The service area of a DTV station is the geographic area within the station's noise-limited F(50,90) contour where its signal strength is predicted to exceed the noise-limited service level. The noise-limited contour is the area in which the predicted F(50,90) field strength of the station's signal, in dB above 1 microvolt per meter (dBu) as determined using the method in section 73.625(b) exceeds the following levels (these are the levels at which reception of DTV service is limited by noise):

|  | (dBu) |
| --- | --- |
| ### Channels 2-6 | 28 |
| Channels 7-13 | 36 |
| Channels 14-69 | 41 |

“Part 73.625(b) describes the method to determine the predicted F(50,90) field strength using the F(50,50) and F(50,10) field strength charts given in the Figures in part 73.699.

F(50,90) = 2 * F(50,50) – F(50,10)

“Within this contour, service is considered available at locations where the station's signal strength, as predicted using the terrain dependent Longley-Rice point-to-point propagation model, exceeds the levels above. Guidance for evaluating coverage areas using the Longley-Rice methodology is provided in OET Bulletin No. 69.“

Reference OET Bulletin No. 69:

The planning factors used in OET 69 for DTV reception:

| Planning Factor | Symbol | Low VHF | High VHF | UHF |
| --- | --- | --- | --- | --- |
| Geometric mean frequency (MHz) | F | 69 | 194 | 615 |
| Dipole factor (dBm-dBu) | Kd | -111.8 | -120.8 | -130.8 |
| Dipole factor adjustment | Ka | none |  | 20 * log[615/(channel mid-frequency in MHz)] |
| Thermal noise (dBm) | Nt | -106.2 |  |  |
| Antenna gain | G | 4 | 6 | 10 |
| Downlead line loss (dB) | L | 1 | 2 | 4 |
| System noise figure (dB) | Ns | 10 |  | 7 |
| Required carrier to noise ratio (dB) | C/N | 15 |  |  |

Field + Kd + Ka + G – L – Nt – Ns = C/N

Interference Criteria:

|  | D/U Ratio, dB |  |
| --- | --- | --- |
| Channel Offset | DTV into Analog | DTV into DTV |
| -1 (lower adjacent) | -14 | -28 |
| 0 (co-channel) | 34 | 15 |
| +1 (upper adjacent) | -17 | -26 |
| N - 8 | -32 | Not Considered |
| N - 7 | -35 |  |
| N - 4 | -34 |  |
| N - 3 | -30 |  |
| N - 2 | -24 |  |
| N + 2 | -28 |  |
| N + 3 | -34 |  |
| N + 4 | -25 |  |
| N + 7 | -43 |  |
| N + 8 |  |  |
| N + 14 | -33 |  |
| N + 15 | -31 |  |

The above co-channel D/U ratios are only valid where the S/N ratio is 28 dB or greater.  At the edge of the noise-limited service area, where the S/N ratio is 16 dB, the co-channel D/U ratio should instead be 23 dB.  For S/N ratios greater than 16 dB but lower than 28 dB, the minimum DTV into DTV co-channel D/U ratio is to be calculated from:

D/U = 15 + 10 * log[1.0/(1.0 – 10^(-x/10))],	x = S/N – 15.19 dB.

Front-to-Back ratios assumed for receiving antennae:

|  | Front-to-Back Ratios, dB |  |  |
| --- | --- | --- | --- |
| TV Service | Low VHF | High VHF | UHF |
| Analog | 6 |  |  |
| DTV | 10 | 12 | 14 |

Minimum distance separations:

As defined in part 73.610 of the FCC rules:

Reference FCC Rules Part 73:

Minimum co-channel allotment and station separations:

(1)

| Zone | Channels 2-13 | Channels 14-69 |
| --- | --- | --- |
| I | 272.7 km (169.5 miles) | 248.6 km (154.5 miles) |
| II | 304.9 km (189.5 miles) | 280.8 km (174.5 miles) |

(2) The minimum co-channel distance separation between a station in one zone and a station in another zone shall be that of the zone requiring the lower separation.

Minimum allotment and station adjacent channel separations applicable to all zones:

| Channels | Distance |
| --- | --- |
| 2-13 | 95.7 km (59.5 miles) |
| 14-69 | 87.7 km (54.5 miles) |

The zone in which the transmitter of a television station is located or proposed to be located determines the applicable rules with respect to co-channel distance separations where the transmitter is located in a different zone from that in which the channel to be employed is located.

“The United States is divided into three zones.  These zones are depicted in part 73.699 of the FCC's rules.

Reference ATSC Recommended Practice A/74:

Receiver Performance Guidelines

This Recommended Practice addresses the front-end portion of a receiver of digital terrestrial television broadcasts. The recommended performance guidelines enumerated in this document are intended to assure that reliable reception will be achieved. Guidelines for interference rejection are based on the FCC planning factors that were used to analyze coverage and interference for the initial DTV channel allotments. Guidelines for sensitivity and multipath handling reflect field experience accumulated by testing undertaken by ATTC, MSTV, NAB, and receiver manufacturers.

ATSC A/74 assumes the receiver is resolving an 8-VSB transmission.

Sensitivity:

“A DTV receiver should achieve a bit error rate in the transport stream of no worse than ...” 3E-6 “... for input RF signal levels directly to the tuner from -83 dBm to -8 dBm ... with no noise and no multipath interference.”

Multi-Signal Overload:

For the purpose of preventing internal intermodulation products, multiple signals should be limited to -8 dBm each at the input of the receiver.

Rejection Thresholds:

| Channel | Weak Desired (-68 dBm) | Moderate Desired (-53 dBm) | Strong Desired (-28 dBm) |
| --- | --- | --- | --- |
| N | 15.5 |  |  |
| N +/- 1 | -33 |  | -20 |
| N +/- 2 | -44 | -40 |  |
| N +/- 3 | -48 |  |  |
| N +/- 4 | -52 |  |  |
| N +/- 5 | -56 | -42 |  |
| N +/- 6 to N +/- 13 | -57 | -45 |  |
| N +/- 14 and 15 | -50 |  |  |

The adjacent rejection threshold includes an additional 6 dB margin so that during tests with lab generators, “... it is assured that the limiting interference in the field will be adjacent splatter rather than overload.”

Burst Noise Performance:

“The receiver should tolerate a noise burst of at least 165 microseconds duration at a 10 Hz repetition rate without visible errors.”

Multipath:

Due to the “... wide range of varying multipath and noise conditions,” no suggested model is given to describe “... the diversity of conditions observed in the field.”

# Appendix E

# Wireless Microphone Parameters

Wireless microphones are classed as Low Power Auxiliary Stations (LPAS) and operate in the VHF and UHF television bands as licensed secondary users under Section 74 of the FCC Rules in the United States.

| Wireless Microphone Parameter | Value |
| --- | --- |
| Operating Frequency | 54-72, 76-88, 174-216, 470-488, 488-494 (except Hawaii), 494-608, and 614-806 MHz |
| Output Power Limit (Conducted) | 54-72, 76-88, and 174-216 MHz: 50 mW; 470-608 and 614-806 MHz: 250 mW. |
| Occupied Bandwidth Limit | 200 kHz |
| Modulation | Any, subject to Occupied Bandwidth Limit. Maximum Deviation of 75 kHz if FM is used. |

The above information has been extracted from Section 74.801 – 74.861 of the FCC Rules.

Typical wireless microphones are handheld and body-worn and operate with antenna gains of less than 0dBi.  Due to shadowing and body attenuation effects, the resulting effective radiated power (ERP) is less than the maximum power limits allowed in Part 74 of the FCC rules.