| WRAN Channel Modeling |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-08-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eli Sofer | Runcom | 2 Hachoma St. Rishon Lezion Israel | +972 544 997 996 | elisofer@runcom.co.il |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa Ontario, Canada, K2H 8S2 | +1 613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This document reflects collective efforts of the working group for deriving Channel Model appropriate to the specific environment where WRAN radio link is operating.

Appropriate models for the signal distortion in WRAN radio channel and the non-ideal RF  components deemed crucial in the design process were also provided.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Table of Contents

1	Introduction	8

2	RADIO CHANNEL MODELS	8

2.1 Environment and Propagation Types	8

2.2 Path Loss Calculation	8

2.3 Wideband Channel Models	9

2.3.1 Power Delay Profile	9

2.3.2 Delay Spread and K-Factor	9

2.3.3 Antenna Directivity Gain Degradation	9

2.4	Derivation of a practical Multipath model (based on field measurements)	10

2.4.1 Multipath Model	10

2.4.1.1	Multipath versus frequency	10

2.4.1.2    Effect of echoes with large excess dela                                                               10

2.4.1.3    Effect of echoes with medium excess delay	11

2.4.1.4	Effect of echoes with small excess delay	11

2.4.1.5	Effect of pre-echoes	12

2.4.2 Free Space Theoretical Multipath Model   	12

2.4.3 Results of Multipath Field Measurements 	12

2.4.4 Channel Equalization performance of DTV Recedivers	13

2.4.4.1 ATSC & VSB                                                                                 	   	       13

2.4.4.2 DVB-T									       13

2.4.4.3 Concluding remarks							       14

2.4.5 Multipath Profiles    							      18

2.5 Additive Noise Model	17

2.6 Interference into WRAN	19

2.6.1 Narrow Band Jamming	19

2.6.2 Partial Band Jamming	20

2.6.3 Pulse Jamming	20

3.	Non-ideal RF DEVICES MODELS	20

4. CONCLUSION							             22

ANNEX  A-  Propagation Models                   				                  23

ANNEX  B-  Discrete Time Multipath Channel Model       			                  28

ANNEX  C-  Free Space Theoretical Multipath Channel model	                              29

ANNEX D-   COST 207 Multipath Model						      31

ANNEX  E-  Channel Bandwidth versus Frequency Selective Fading Performance      33

ANNEX  F-  Non-Ideal RF Devices Models					                   35

ANNEX  G-  Phase Noise and Power-Law Model 				                 38                                                     ANNEX  H- Quadrature Modulator							      41

References & Standards

[1]	“Fixed wireless routes for internet access”, IEEE Spectrum, September 1999.

[2]	T. S. Rappaport, “Wireless communications”, Prentice-Hall, 1996.

[3]	P. Karlsson, N. Löwendahl, J. Jordana, “Narrowband and wideband propagation measurements and models in the 27-29 GHz band”, COST 259 TD(98)17, COST 259 Workshop, Berne, Switzerland, February 1998.

[4]	T.-S. Chu, L. J. Greenstein, “A Quantification of Link Budget Differences Between the Cellular and PCS Bands”, IEEE Trans. Veh. Tech., vol.48, no. 1, January 1998.

[5]	A. Bohdanowicz et al., “Wideband Indoor and Outdoor Channel Measurements at 17 GHz”, VTC 1999.

[6]	M. Mitsuhiko et al., “Measurement of Spatiotemporal Propagation Characteristics in Urban Microcellular Environment”, VTC 1999.

[7]	V. Erceg et al., “A Model for the Multipath Delay Profile of Fixed Wireless Channels”, IEEE JSAC, vol. 17, no. 3, March 1999.

[8]	D. Falconer, “Multipath Measurements and Modelling for Fixed Broadband Wireless Systems in a Residential Environment”, IEEE 802.16.1pc-00/01, IEEE 802.16 Broadband Wireless Access Working Group, 21/12/1999.

[9]	P. Karlsson et al., “Outdoor Spatio-Temporal Propagation Measurements for Evaluation of Smart Antennas”, 3TRS091A.doc, ETSI EP BRAN #9, July 1998.

[10]	AC085 – The Magic WAND, “Deliverable 2D8: Evaluation of the WAND System for Outdoor Point-to-Multipoint Configurations”, Aug. 1998.

[11]		AC085 – The Magic WAND, “Deliverable 2D9: Results of outdoor measurements and experiments for the WAND system at 5 GHz”, Dec. 1998.

[12]	N. Patwari, G. D. Durgin, T. S. Rappaport, R. J. Boyle, “Peer-to-peer low antenna outdoor radio wave propagation at 1.8 GHz” Proc. of the IEEE Vehicular Technology Conference (VTC '99 Spring), Houston, TX, vol. I, pp. 371-375, May 1999.

[13]	M. Pettersen, P. H. Lehne, J. Noll, O. Rostbakken, E. Antonsen, R. Eckhoff, “Characterisation of the directional wideband radio channel in urban and suburban areas”, Proc. of the IEEE Vehicular Technology Conference (VTC '99 Fall), Amsterdam, The Netherlands, vol. I, pp. 1454-1459, September 1999.

[14]	A. Plattner, N. Prediger, W. Herzig, “Indoor and outdoor propagation measurements at 5 and 60 GHz for radio LAN applications”, IEEE MTT-S International Microwave Symposium Digest, vol. 2, pp. 853-856, 1993.

[15]	M. P. M. Hall, L. W. Barclay, M. T. Hewitt, “Propagation of Radiowaves”, The Institution of Electrical Engineers, London, UK, 1996.

[16]	S. R. Saunders, “Antennas and Propagation for Wireless Communication Systems”, John Wiley & Sons, Chichester, UK, 1999.

[17]	L. J. Greenstein, V. Erceg, Y. S. Yeh, M. V. Clark, “A New Path-Gain/Delay-Spread propagation model for Digital Cellular Channels”, IEEE Trans. Veh. Tech., vol. 48, no. 2, May 1997.

[18]	L. J. Greenstein, V. Erceg, “Gain Reductions Due to Scatter on Wireless Paths with Directional Antennas”, IEEE Comm. Letters, vol. 3, no. 6, June 1999.

[19]	ITT, “Reference Data for Radio Engineers”, Sixth Edition, 1975. Howard W. Sams and Co., Indianapolis.

[20]	J. Rutman, “Characterization of phase and frequency instabilities in precision frequency sources: Fifteen years of progress”, IEEE Proc., vol. 66, no. 9, pp. 1048-1076, Sep. 1978.

[21]	T. H. Lee, A Hajimiri, “Oscillator Phase Noise: A Tutorial”, IEEE J. Solid-State Circuits, vol. 35, no. 3, pp. 326-336,  Mar. 2000.

[22]	A. Demir, A. Mehrotra, and J. Roychowdhury, “Phase noise in oscillators: A unifying theory and numerical methods for characterization”, IEEE Trans. on Circuits and Systems-I, vol. 47, no. 5, pp. 655-674, May 2000.

[23]	L. Tomba, “On the effect of wiener phase noise in OFDM systems”, IEEE Trans. Commun., vol. 46, pp. 580–583, May 1998.

[24]	T. Pollet, M. van Bladel, and M. Moeneclaey, “BER sensitivity of OFDM systems to carrier frequency offset and Wiener phase noise”, IEEE Trans. Commun., vol. 43, pp. 191–193, Feb./March./April 1995.

[25]	G. Foschini, “Characterizing filtered light waves corrupted by phase noise”, IEEE Trans. Inform. Theory, vol. 34, Nov. 1988.

[26]	I. T. Monroy and G. Hooghiemstra, “On a recursive formula for the moments of phase noise”, IEEE Trans Commum., vol. 48, no. 6, June 2000.

[27]      WRAN_2crln006a.doc, “Effects of Climate on WRAN system performance”, WRAN

Design note, Aug. 2000.

[28]      R.L. Freeman, “Telecommunication Transmission Handbook”, New York: J. Wiley & Sons,

1991.

[29]      H. Xu, T. Rappaport et al, “Measurements and Models for 38-GHz Point-to-Multipoint

Radiowave Propagation”, IEEE J. Selected Areas Commun., SAC-18, No. 3, March 2000, pp.

310-321.

[30]     IEEE802.16.1.pc-00/12r1, “Multipath Measurements and Modeling for Fixed Broadband

Point-to-Multipoint Radiowave Propagation Links under different Weather Conditions”,

Contribution in IEEE 802.16.1, 25-02-2000.

[31]     T. Pratt, C.H. Bostan, “Satellite Communications”, New York: J.Wiley, 1986.

[32]    H. Masui et al, “Difference of Path Loss Characteristics due to Mobile Antenna Heights in

Microwave Urban Propagation”, IEICE Trans. Fundamentals, Vol. E82-A, No. 7, July 1999,

pp. 1144-1150.

[33]    G. Durgin, T. Rappaport and H. Xu, “Measurements and Models for Radio Path Loss and

Penetration Loss In and Around Homes and Trees at 5.85 GHz”, IEEE Trans. on Commun.,

COM-46, No. 11, Nov. 1998, pp. 1484-1496.

[34]	COST 207 Report, Digital land mobile radio communications, Commission of European Communities, Directorate General, Telecommunications, Information Industries and Innovation, Luxembourg, 1989

[35]		Culver, R., Final Report of the Channel Characterization Task Group: The Derivation and Rationale for Multipath Simulation Parameters for the EIA-DAR Laboratory Testing, prepared for the EIA DAR Subcommittee, July 1995.

[36]	Kahwa, T. and McLarnon, B., Channel characterization and modeling for digital radio, 2nd International Symposium on Digital Audio Broadcasting, Toronto, 1994.

[37]	Lee, W.C.Y., Mobile Communications Engineering. New York: McGraw-Hill, 1982, p.43.

[38]	Springer, K., Multipath propagation and fading statistics for digital audio broadcasting in the VHF and UHF bands, NAB Broadcast Engineering Conference Proceedings, 1993.

[39]	McLarnon, B., Further results on the characterization of VHF broadcast channels, October 1995 (report tabled at an EIA DAR Subcommittee meeting).

[40]	R. Voyer and R. Paiement, The Field Strength Variability of a 1.5 MHz Wide Signal at 1.5 GHz. Document submitted to the CCIR WP 10B (doc. 10B/85), Geneva (Switzerland), October 1993.

[41]      ADAMAS  IST Project- Channel Modeling (Public Deliverable)

[42]      Evaluation of a DVB-T compliant digital terrestrial television system – IBC 97 publication

[43]      The echo performance of DVB-T receivers – EBU Technical review – Septembre 2001

List of abbreviations & symbols

| OFDM | Orthogonal Frequency-Division Multiplexing |
| --- | --- |
| RF | Radio Frequency |
| LOS | Line-of-Sight |
| NLOS | Non-Line-of-Sight |
| PDP | Power Delay Profile |
| RMS | Root Mean Square |
| MAN | Metropolitan Area network |
| FFT | Fast Fourier Transform |
| ECC | Error Correction Codes |
| SNR | Signal-to-Noise Ratio |
| AM | Amplitude Modulation |
| PM | Phase Modulation |
| BER | Bit Error Rate |
| OBO | Output Back-Off |
| QPSK | Quadrature Phase-Shift Keying |
| TPD | Total Power Degradation |
| ICI | Inter-Channel Interference |
| PN | Phase Noise |
| LO | Local Oscillator |
| DAC | Digital-to-Analog Converter |
| DDS | Direct Digital Synthesizer |
| SFDR | Spurious-Free Dynamic Range |
| CIC | Cascaded Integrator-Comb |
| IF | Intermediate Frequency |

List of Figures

Figure 1:   Inter-symbol interference resulting from channel multipath			11

Figure 2:   Improvement of ATSC-DTV receivers against channel multipath		13

Figure 3:   DVB-T receiver performance             						14

Figure 4:   Multipath Profile 1								15

Figure 5:   Multipath Profile 2                                  					           15

Figure 6:   Multipath Profile 3								16

Figure 7:   Multipath Profile 4								16

Figure 8:   Medium value of man-made noise power						17

Figure 9:   Noise amplitude distribution at base station (150 MHz)				19

Figure 10:   Voltage and phase transfer characteristics of the ‘perfect clipper’		20

Figure 11:   AM/AM power characteristic for sinusoidal and Gaussian signal inputs

to the ‘perfect clipper’								21

Figure 12:   3rd order Intercept Point of the ‘perfect clipper’					21

List of Tables

Table 1:  Values of the constants c and d                                       			           18 Table 2:  Summary of channel model configuration parameters  				23

Error: Reference source not found

# Introduction

The development of appropriate models for the signal distortion in the radio channel, the antenna subsystems and the non-ideal RF unit components is a crucial step in the design process. These models support the algorithms design by providing a means for validation. Furthermore, the results obtained by computer simulations employing realistic models support the choice of the suitable transmission schemes as well as the selection of the appropriate RF technologies. Finally, they enable a forecast of the achievable coverage of the WRAN system.

The rest of this document is organised as follows. In Sect. 2 the WRAN radio channel models are defined. Sect. 3 contains the various models for the non-ideal RF parts.

# RADIO CHANNEL MODELS

The following channel models target wave propagation scenarios in the context of the WRAN system, i.e., outdoor broadband wireless transmission using fixed transmitter and receiver stations. A stochastic channel model is defined generating random impulse responses, which is suitable for employment in the WRAN system simulation chain. The base and terminal stations are assumed separated by a few hundred meters up to a few tens of kilometres. Sect 2.1 describes the different environment and propagation types that are distinguished. In Sect. 2.2 the path loss is calculated both for situations where the direct path between transmitter and receiver is unobstructed and for obstructed propagation situations. The wide-band behaviour of the channel is addressed in Sect. 2.3. In sec 2.4 a practical and simplified Multipath Channel is described, based on extensive field measurements conducted by CRC , and Sect. 2.5 contains a study of the additive noise in the receiver. Sect. 2.6 deals with the modelling of the interference in non-licensed bands.

## 2.1 Environment and Propagation Types

A classification into urban, suburban and rural environments is adopted within this document. Following Hata’s model, urban environments are further divided into large and small/medium cities while rural environments are assumed as flat.

Additionally, line-of-sight (LOS) and non-line-of-sight (NLOS) propagation scenarios are distinguished. In LOS situations there is no attenuation of the direct signal due to obstructing objects. This requires the direct transmitter-receiver path including the space within 0.6 times the radius of the first order Fresnel zone to be free [1]. All other propagation scenarios are attributed NLOS.

## 2.2 Path Loss Calculation

In the following section, the LOS propagation model is presented as well as the proposed reference NLOS prediction model.   This NLOS prediction model is proposed following an evaluation of two well know prediction methods, the first method was based on the extrapolation of Hata’s model for the frequencies where WRAN is expected to operate, while the second one is the ITU-R prediction method contained in ITU-R Recommendation P.1546-1.  These methods are described in more details in Appendix A which also provides a comparison of the results of these two methods for similar conditions.  After consideration, the ITU-R model was selected.

Error: Reference source not found

## 2.3 Wideband Channel Models

Multipath wave propagation leads to additional variations of the signal attenuation, called small-scale fading, with rapid changes when moving the antenna positions locally. Moreover, for broadband transmission multipath leads to dispersion in the time domain and in the same time a frequency selectivity of the channel. The dispersion of the transmitted signal induced by the channel is modelled by a convolution with the channel impulse response, for which in this section statistical models are defined.

### 2.3.1 Power Delay Profile

The power delay profile (PDP) provides statistical a-priori information about the impulse response. Specifically, the PDP provides the expected signal energy arriving at a specific delay from the transmission of a Dirac impulse. The earliest arriving contribution is assigned delay zero and normally originates from the signal part travelling in a direct transmitter-receiver path, resulting in a peak in the PDP. The energies in the indirect, reflected or scattered signal parts typically decay exponentially in the mean. This leads to the common spike-plus-exponential shape of the PDP, given by

ϕC(τ)=c0δ(τ)+(c1τ1)exp(ττ1),    τ  0,	(2.3.1)

where δ(·) is the Dirac delta function. In the above formula, c0 and c1 determine the mean energies in the direct and indirect signal parts, respectively, and τ1 specifies the exponential decay in the indirect components. The mean total signal energy returning from a transmitted unit energy pulse equals c0+c1. For LOS scenarios

c0=10LLOS10,

whereas for NLOS scenarios and wide angle terminal station antennas

c0+c1=10LNLOS10.

The ratio c0/c1 is referred to as the K-factor K0, providing information about the presence and strength of the direct propagation path. The root mean square (RMS) delay spread τRMS [2] of the PDP defined in (2.3.1) is given by

τRMS=1−c0c0+c12τ1.	(2.3.2)

### 2.3.2 Delay Spread and K-Factor

Both the delay spread and the K-factor heavily depend on the environment and the antenna types. In LOS scenarios the K-factor is much larger than for NLOS scenarios even with omnidirectional antennas. In NLOS scenarios, K0 is determined by the presence and strength of a dominant signal path. If the area between the transmitter and the receiver is totally obstructed, K0 is close to zero.

### 2.3.3 Antenna Directivity Gain Degradation

Clearly, the K-factor also increases when narrowing the terminal station antenna beamwidth because of increased fading of reflected signal parts arriving from “blind” angles. When the centre of the antenna beam is oriented towards the impinging direct signal, the gain degradation concerns only the power in the indirect signal parts. Hence, the PDP in (2.3.1) is replaced by

ϕC(D)(τ)=c0δ(τ)+10−μD10(c1τ1)exp(ττ1),   τ  0.

Here, μD represents the antenna directivity gain degradation factor in dB. In [18], the model

μD(βT,IS)=−(0.35+0.1IS)ln(βT360)+(0.50+0.04IS)(ln(βT360))2

was proposed for μD based on measurements in the 1.9 GHz band in suburban downlinks. The gain degradations have actually turned out to depend on the half-power-beamwidth βT of the terminal station antenna and on the season IS (IS = +1 for winter, IS = -1 for summer), while being relatively independent of d. For a 60 degree antenna beamwidth for instance, 2.5 dB and 1.9 dB reductions result for winter and summer, respectively, whereas for a 17 degree antenna the degradations are 6.4 dB and 5.1 dB, respectively. The model is formulated for βT between 17˚ and 65˚, while extrapolations below 17˚ and beyond 65˚ are plausible. The above formula is adopted as the general model for lower frequency bands

## Derivation of a practical Multipath model (based on field measurements)

### 2.4.1 Multipath Models

Usually simulating a multipath channel is done using a number of discrete signal paths according to the given power delay profiles.  This should represent a good approximation of the real phenomenon appearing in the field and corresponds to a finite set of ‘specular’ reflections.  In reality, the transmission channel produces ‘specular’ as well as ‘diffused’ echoes and the total effect is a composite of these multiple echoes.

#### Multipath versus frequency

During the EIA-DAR laboratory tests [35], the question arose whether channel models similar to those used in COST 207 (see Appendix B) would be applicable to VHF and L-Band frequencies.

There is unquestionably a significant frequency dependence in terms of propagation losses – the attenuation resulting from diffraction around objects or penetration through them almost invariably increases with frequency.  Multipath, however, is a different matter.  One good indicator of the amount of multipath present on a channel is the “delay spread” parameter.  One authority (Lee [37]) concludes that “the data available show that the delay spread is independent of the operating frequency at frequencies above 30 MHz”.  The explanation for this is as follows: as the wavelength increases, the energy scattered off a given object tends to decrease (more absorption and diffraction), which would decrease the amplitude of the multipath reflections.  On the other hand, path loss decreases with increasing wavelength (i.e., the effective aperture of the isotropic antenna at that frequency), and these two effects tend to balance each other, making delay spread roughly independent of frequency.

A similar conclusion is reached by Springer [38], who states that “there is support for the proposition that most of the important statistical parameters are relatively constant across the VHF and UHF bands”. This proposition is also supported by measurements previously performed by CRC [36,34, 39].

#### 2.4.1.2 Effect of echoes with large excess delay

Long-delay echoes are produced by the RF signal reflecting from large and distant structures such as neighboring mountains.  With light propagating at 300 m/sec, an echo with a 25 sec excess delay results from an RF signal having gone through a 7.5 km longer RF path than the direct path.  In the frequency domain, this results in a comb-like ripple structure across the channel.  Because of the extra spreading loss and partial absorption of energy by the reflecting surface, this long echo is usually received at lower power than the direct path.

#### 2.4.1.3 Effect of echoes with medium excess delay

Echoes with excess delays in the range between about 1 sec and 10 sec are the most prominent.  They are produced by reflective surfaces in the neighborhood of the receiver or transmitter.  They correspond to excess path lengths of 300 m to 3 km.  These echoes are clearly the most powerful and the most numerous, due to the probability of finding sizeable reflecting surfaces in this range.  Communication systems must compensate for these echoes.   For wideband signals, these echoes produce frequency selective fading with a coarser comb-like ripple structure within the transmission channel and create inter-symbol interference in typical transmissions, as illustrated in Figure 5.  This can be corrected by time equalization, or by discarding the ISI present in the symbol guard interval of a multi-carrier modulation.  For narrowband signal, these echoes may result in flat fading within the channel bandwidth.  Other means, as indicated in section 2.4.1.2, will then need to be used to recover the signal.

#### Effect of echoes with small excess delay

Short echoes are produced by the RF signal being reflected by structures close to the receiver or the transmitter.  Because of the small excess distance traveled, the reflected signal may approach the power of the direct signal if the nearby surface is very reflective for RF signals.  As an example, a 0.3 sec echo would be produced by a 90m extra length on the reflected path.  This could be produced by a reflector located as close as  45 m from the receiver.  These short echoes tend to create flat fading at the receiver, depending on the channel bandwidth.  The relationship between the channel bandwidth and the echo excess delays that can be corrected with time or frequency equalizers is covered in Appendix C.   If flat fading is experienced over the whole channel bandwidth, then one has to resort to antenna diversity to recover the signal.  Fortunately, the occurrence of such short echoes seems to be less than that for echoes with medium excess delay in the case where outside antennas with some directivity are used.

Figure 1:  Inter-symbol interference resulting from channel multipath

#### Effect of pre-echoes

It is possible that the main signal is received at a lower power level than the reflected signals.  In such case, the receiver will have to work in presence of pre-echoes and still synchronize, and either equalizing it or discard it.  Because of the geometry involved in these signal reflection, attenuation and blockage, the extend of such pre-echoes is typically less than the post-echoes but they should not be neglected.  A typical range for these pre-echoes is typically found to be around [5 μsec].

### 2.4.2	Free Space Theoretical multipath model

In order to establish the worst channel multipath situation possible to bound the phenomenon, a theoretical model was developed where free-space propagation was assumed on any direct and indirect transmission paths.  For a given distance between a transmitter and a receiver, the effect of reflectors located at regular azimuth and distance intervals around the receiver and assumed to be oriented to reflect the signal from the transmitter toward the receiver were simulated for the generation of multipath signals at the receiver.  The relationship between the level of each echo and its excess delay could then be established by this sampling mechanism. (See Appendix C)

.

### 2.4.3	Results of multipath field measurements

Multipath results were measured at a number of locations in the United States as well as in Canada and Europe.  The longest delay witness was in Hildesheim, Germany (62uSec., -18dB ) and near Paris, France (67uSec., -21dB)  In the United States, approximately 85% of the test locations indicated a total delay spread (pre-echo through –40dB post echo) of less than 35uSec.  Certain markets, particularly those situated in valleys near mountainous terrain, do possess long delay time characteristics.  Most notably are the markets of San Francisco, Phoenix, and Salt Lake City.  In each case, line of site to the source is partially blocked, thus reducing the received signal strength of the main ray by some 10 dB or more.  The effect of this is to effectively raise the amplitude of distant reflection rays with respect to the direct ray.  In the case of the San Francisco market, a delay spread of 56uSec was observed.  The value of the long delay echo was –17dB.

Under weak signal conditions, long delay echoes at levels below approximately –16 to -18dB (exact system dependant) do not pose a problem in that modulation of the constellation will have been reduced to QPSK.  In the less frequent case of long delay under strong signal conditions, such as those observed in the San Francisco market, a system which has not been designed to work with long delay, strong reflections will be forced to revert to QPSK modulation even though the main ray signal strength would support higher order modulations.  Such a system would prove to exhibit inefficient use of available channel bandwidth.  Corrective action under these conditions could include one or more of the following:  System design to accept longer delay spread, use of an optional second mode of transmission that can accept a longer delay spread, or incorporation of a channel equalizer in the receiver.

The following conclusions can be drawn from the field data collections for systems operating substantially below 1GHz:  1) System designs capable of 35uSec delay spread will operate with higher order modulation constellations approximately 90% of the time, 2) System designs that are capable of 60uSec delay spread will raise the reliability of operation of higher order modulation constellations to 99%, 3) Worst case testing has proven to be strong signal, partially blocked main ray conditions where higher order modulation constellations are warranted, but operation must be reduced to QPSK data rates to achieve an acceptable D/U ratio at the demodulator.

### 2.4.4 Channel equalization performance of DTV receivers

Special attention was given to channel multipath in developing the transmission standards for Digital TV.  Models such as those described above were used to identify the extent of the problem and design the DTV modulation accordingly.  When it can to the practical implementation of the DTV systems and utilization of DTV receivers by the public, the effects were found to be markedly more severe.  This could be explained by the fact that the models described above were developed for mobile reception where critical multipath situations will result in loss in time availability due to the time variation of the channel at the mobile user terminal.  However, in the case of fixed reception, a bad multipath situation is much more objectionable because I tends to last.  As a result, the manufacturers have put a lot of effort in the last few years to improve the robustness of the DTV receivers against multipath.

2.4.4.1	ATSC 8-VSB

Figure 2 depicts the typical performance of ATSC-DTV receivers as a function of time.

Figure 2:  Improvement of ATSC-DTV receivers against channel multipath

      - DVB-T

Since DVB-T was planned to technically operate in Single Frequency Networks (SFN) the system was specifically designed to deal with high power echoes. For that purpose a guard interval has been introduced between the OFDM symbols to guarantee the immunity of the signal with respect to multipath.

Echoes that are shorter than the guard interval duration add useful signal power. They however create an uneven channel response that requires an efficient channel estimation mechanism to be compensated.

The DVB-T receiver can generally withstand a 0 dB echo within the guard interval assuming a high C/N. Measurements carried out on some receivers (consumer electronics equipment) show that the echo attenuation level is 2 dB when the C/N ratio is 25 dB. The echo attenuation becomes 4 dB for a C/N ratio of 22 dB.

If the echo delay exceeds the guard interval, a fraction of each delayed symbol adds incoherently to the direct signal. This results in a noise-like inter-symbol interference that impairs the performances of the receiver.

Figure 3 (extracted from [42]) depicts a typical DVB-T characteristic where the plots represent the maximum tolerable echo power versus delay.

Figure 3 – DVB-T receiver performance

The system under test has the following features:

- Symbol duration: 231 µs;

- Guard interval duration: 7 µs;

- Outer channel coding: Reed-Solomon [204, 188] with Forney interleaver depth I=12;

- Inner coding: Punctured convolutional codes based on a mother code of rate ½ with 64 states. The generator polynomials are 171oct and 133oct.

With 64-QAM rate 2/3 the limit to operation with a 0 dB echo is the guard interval duration. The sharp fall at 60 µs is caused by the equalization mechanism.

With a very rugged code such as QPSK rate ½ the limiting factor is the channel equalizer. This figure shows that the system may operate with echoes significantly higher than the guard interval.

      - Concluding remarks

It can be seen from the figure that the multipath performance for DTV receivers needs to be much better that predicted by the models described above, especially with respect to pre-echoes.  Since WRAN user terminals will use similar antenna setup and RF electronics as for DTV reception, it is expected that such more demanding multipath performance will also be needed.  The various technologies proposed for WRAN should therefore be tested with a real-time multipath simulator of the type used to test DTV.

### 2.4.5 Multipath Profiles

The following four multipath profiles, defined in Table 1 below, and Figures 4-7,  are provided  for testing the proposed WRAN systems using a real-time multipath simulator (6 paths can be reproduced by a commonly available hardware channel simulators.

| # Profile A | Path 1 | Path 2 | Path 3 | Path 4 | Path 5 | Path 6 |
| --- | --- | --- | --- | --- | --- | --- |
| ## Excess delay | 0 | 3 μsec | 8 μsec | 11 μsec | 13 μsec | 21 μsec |
| Relative amplitude | 0 | -7 dB | -15 dB | -22 dB | -24 dB | -19 dB |
| Doppler frequency | 0 | 0.10 Hz | 2.5 Hz | 0.13 Hz | 0.17 Hz | 0.37 Hz |
| # Profile B | Path 1 | Path 2 | Path 3 | Path 4 | Path 5 | Path 6 |
| Excess delay | -3 μsec | 0 | 2 μsec | 4 μsec | 7 μsec | 11 μsec |
| Relative amplitude | -6 dB | 0 | -7 dB | -22 dB | -16 dB | -20 dB |
| Doppler frequency | 0.1 Hz | 0 | 0.13 Hz | 2.5 Hz | 0.17 Hz | 0.37 Hz |
| # Profile C | Path 1 | Path 2 | Path 3 | Path 4 | Path 5 | Path 6 |
| Excess delay | -2 μsec | 0 | 5 μsec | 16 μsec | 24 μsec | 33 μsec |
| Relative amplitude | -9 dB | 0 | -19 dB | -14 dB | -24 dB | -16 dB |
| Doppler frequency | 0.13 Hz | 0 | 0.17 Hz | 2.5 Hz | 0.23 Hz | 0.10 Hz |
| # Profile D | Path 1 | Path 2 | Path 3 | Path 4 | Path 5 | Path 6 |
| Excess delay | -2 μsec | 0 | 5 μsec | 16 μsec | 22 μsec | 0 to 60 μsec |
| Relative amplitude | -10 dB | 0 | -22 dB | -18 dB | -21 dB | -30 to +10 dB |
| Doppler frequency | 0.23 Hz | 0 | 0.1 Hz | 2.5 Hz | 0.17 Hz | 0.13 Hz |

Note: Amplitude in dB relative to the strongest signal path, i.e., reference signal path.

### Table 1: Reference channel multipath profiles for evaluation of 802.22 WRAN technologies

Figure 4- Multipath Profile A

Figure 5- Multipath Profile B

Figure 6- Multipath Profile C

Figure 7- Multipath Profile D

## 2.5 Additive Noise Model

The total effective input receiving noise consists of two parts, namely manmade noise and receiver noise, i.e.,

10 log (Ptrn / kTo) = 10 log [(Pmm + Prn) / kTo].	(2.5.1)

Median values of man-made noise, in dB above kTo (-174 dBm), is given in Figure 8 versus frequency, for various environment categories (ITU=R Recommendation P.372-8).

Median values of man-made noise power for a number of environments are shown in Fig. 8. The Figure also includes a curve for galactic noise.  In all cases results are consistent with a linear variation of the median value, Fam, with frequency f of the form:

Fam c – d log f           (11)

With f expressed in MHz, c and d take the values given in Table 1. Note that equation (11) is valid in the range 0.3 to 250 MHz for all the environmental categories except those of curves D and E as indicated on the Figure.  For frequencies above 250 MHz, the curves can be considered to extend linearly downward until they hit the level of thermal noise of the environment captured in the horizontal direction by the antenna, i.e., 290 deg K (Fam= 0dB).

Figure 8- Medium value of man made noise power

Table 1: Values of the constants c and d

An analysis of available measurement data for business areas (essentially the only area for which

data are available) in the frequency range 200 MHz to 900 MHz also shows a linear variation with the logarithm of frequency, but with a more gradual slope. The result is, with f in MHz,

Fam 44.3 – 12.3 log f for 200 MHz f 900 MHz          (12)

At VHF a significant component of man-made noise is due to ignition impulses from motor

vehicles. For this contribution noise may be presented as an impulsive noise amplitude distribution (NAD) (the impulsive noise spectrum amplitude as a function of impulse rate). Figure 9 is an example of the noise amplitude distribution at 150 MHz for three categories of motor vehicle density. The NAD for other frequencies may be determined from the relationship:

A C 10 log V – 28 log f dB(V/MHz) (13)

where:

C : 106 dB(V/MHz)

V : traffic density (vehicles/km2)

f : frequency (MHz).

Figure 9- Noise amplitude distribution at base station (150 MHz)

The noise levels decrease with frequency until the receiver noise becomes dominant, and then rise slowly with frequency

## 2.6 Interference into WRAN

Proponents should describe the mechanisms that they propose to include in their technology to cope with these various kinds of interference and how this should be quantified.

The interference in the WRAN environment can be categorised into:

- Coexistence with DTV broadcast and Land Mobile Systems operating in upper UHF band.

- Narrow band jamming

- Partial band jamming

- Pulse jamming

### 2.6.1 Narrow Band Jamming

Narrow band jamming can be treated by:

- Using time shaping on the symbol and then equalisation (the more FFT points used the better the shape is)

- Using jamming detection and then a smart ECC, which can erase bad symbols.

### 2.6.2 Partial Band Jamming

Detecting bad symbol can treat partial band jamming, which allow the usage of smart ECC, which can erase bad symbols.

### 2.6.3 Pulse Jamming

Short time interference can be sold by time interleaving the data. The usage of the sub-channel notion enables time interleaving of the sub-channel over time, the small packet length enables easy time interleaving and better statistical multiplexing.

Error: Reference source not found

# Non-ideal RF DEVICES MODELS

The various non-ideal RF components such as high-power amplifiers and receiver RF front-ends will have an impact on the performance of the WRAN systems.  In order to assess the susceptibility of the various technologies proposed, it is proposed to use a simple model of a non-linear device for simulation, testing and evaluation.  This device is a ‘perfect clipper’ which has a perfect linear characteristic in voltage up to a point where perfect saturation occurs.  It is assumed to also have a constant phase shift independent of the input signal level.

The voltage and phase transfer characteristics of this ‘perfect clipper’, its AM/AM power characteristic for sinusoidal and Gaussian signal inputs as well as the 3rd order Intercept Point are illustrated in the three Figures below.

Figure 10:  Voltage and phase transfer characteristics of the ‘perfect clipper’

Figure 11:  AM/AM power characteristic for sinusoidal and Gaussian signal inputs to the ‘perfect clipper’

Figure 12:  3rd order Intercept Point of the ‘perfect clipper’

For these figures, the 0 dB input reference point is assumed to be the level at which the sinusoidal signal starts to ‘clip.  For example, this corresponds to 10 dBm in a 50  load for clipping levels at –1 Volt and +1 Volt.  The 1 dB compression point corresponds to 2 dB higher input power as compared to the 0 dB input reference point and the 3rd order intercept point (IP3) corresponds to 7.76 dB relative to the 0 dB input reference point.

Closed form expressions for the compression of the waveforms versus backoff from the compression point have been derived.  “Compression” in this context is defined as the ratio of the output power to the input power.  These expressions were derived for the Gaussian and sinusoidal cases described above.  The hard clipper characteristic is linear with unity gain to the clip point (generalized to C in the equations; C = 1 in Figure xx).  The signals are expressed in terms of their rms strength (“σ” for the Gaussian analysis, “A” for the sinusoidal analysis), and the compression in terms of the ratio of the clip point C to the rms strength (either σ or A).  The ratio of the clip point to the rms strength is termed the “backoff”.

For the Gaussian input waveform with rms value σ (variance σ2), the compression can be expressed as

Ey2σ2=Cσ2−2πCσe−(Cσ)22+1−Cσ2erf12Cσ,

where E{y2} is the variance of the (zero-mean) output waveform y.  The output y is compressed by 1 dB when the ratio of C/σ = 1.54062.

For the sinusoidal waveform, there is no compression when the rms value of the input is more than 3 dB below the clip point.  When the rms value is less than 3 dB below the clip point, the compression can be shown to be

a12A=2πCA1−12CA2+2πsin−112CA,  CA≤2,

where a1 is the fundamental Fourier coefficient of the clipped sinusoid and A is again the rms amplitude of the input.  When C/A =2, the ratio of output to input amplitude is 1, indicating that there is no compression.  As C/A drops below2 (that is, A gets closer to C), the sinusoid starts clipping.  One dB compression is reached when C/A = 1.12202.  Note that since this expression is for the ratio of amplitudes rather than powers (as it was for the Gaussian case), 20 log10 must be used to determine the amount of compression.

Error: Reference source not foundError: Reference source not foundError: Reference source not foundError: Reference source not found

### 4 Conclusion

As a result of the above transmission channel considerations, the propagation model to be used as reference is the one described in ITU-R Rec, 1546-1.

WRAN system should be able to withstand the presence of multipath signals of up to 50 μsec excess delay and 10 μsec pre-echoes.  Such pre- and post echoes could be as powerful as the main signal or any other echo falling within this interval.  The minimum bandwidth that should be considered to minimize the effect of flat fading should be 2 MHz.  However, if on-channel repeaters are to be implemented as part of the WRAN deployment, a much larger window over which multipath signals will need to be corrected will be needed.  The extent of this window will depend on the physical distance among these on-channel repeaters and the base station.   Section 4.2.5 contains the reference multipath profiles against which the various proposed WRAN technologies will be tested.

The WRAN system will need to work in environments where man-made noise such as described in section 2.5 and interference as described in section 2.6 will exist.

The performance of the WRAN technologies will also be evaluated under reference non-linear transmission channel conditions as defined by the ‘perfect clipper’ model described in section 3,

The parameters of the WRAN channel and their respective values and ranges of values are summarised in Table 2.

| Parameter | Values/range |
| --- | --- |
| Transmitter-receiver separation (d) | A few hundred metres up to a several tens of kilometres |
| Radio frequency (f) | VHF/UHF TV frequency bands |
| Base station antenna gain (GB) | As specified |
| Customer premise equipment antenna gain (GT) | As specified |
| Customer premise equipment antenna half power beamwidth (βT) | Typically 60 degrees |
| Propagation conditions | LOS, NLOS |
| Environment type | Rural, suburban, urban small/medium cities; |
| Base station height hT | 30–1000 m |
| CPE antenna height hB | 10 m |
| Multipath profiles | See section 4.2.5 |
| Seasons of operation | All seasons |

Table 2 : Summary of channel model configuration parameters.

_ Error: Reference source not foundError: Reference source not foundError: Reference source not foundError: Reference source not found

ANNEX A

Propagation Models

### A.1 LOS Propagation Mode

The loss in the direct signal propagation path under LOS condition including the loss in the antennas is given by the Friis free space equation [2]

LLOS=L0+20logd+20logf−10logGB−10logGT    [dB],

In the above formula L0=20log4πc, where c is the speed of light. Additionally, d, f, GB and GT specify the transmitter-receiver-distance in meters, the frequency in Hertz and the base and terminal station antenna gain values in dBi, respectively. This model does not take the contributions from additional reflected and scattered signal paths into account.

### A.2  NLOS Okumura-Hata Model

In NLOS scenarios an additional path loss results from scattering, diffraction and reflection effects. This is modelled by the term Lexcess, i.e.,

LNLOS=LLOS+Lexcess(f,d,hB,hT)    [dB].	(2.2.2)

Besides of the frequency and the transmitter-receiver separation the excess path loss depends on the base and terminal station antenna heights denoted by hB and hT, respectively, in meters.

For frequencies within 150-1500 MHz and distances from one up to 20 kilometres, Hata’s model [2,16] may be employed for the excess path loss prediction. The model is based on extensive measurements in Tokyo and makes a distinction between small/medium and large cities as well as between urban, suburban and open rural areas. For f0 = 1 GHz, the excess path loss according to Hata’s model can be expressed by

LHATA(hB,hT)+μHATA(hB,hT)logd.

The applicable coefficients LHATA and μHATA for different hB and hT are summarised in the following Tablesfor small/medium cities, for large cities and for suburban and open areas.

These Tables provide the coefficients LHATA and μHATA for the excess path loss calculation based on f0 = 1 GHz. for different hB and hT according to Hata’s model.

|  | hT = 1 m | hT = 4 m | hT = 7 m | hT = 10 m |
| --- | --- | --- | --- | --- |
| hB = 30 m | LHATA = -9.22 μHATA = 15.22 | LHATA = -17.02 μHATA = 15.22 | LHATA = -24.82 μHATA = 15.22 | LHATA = -32.62 μHATA = 15.22 |
| hB = 50 m | LHATA = -7.93 μHATA = 13.77 | LHATA = -15.73 μHATA = 13.77 | LHATA = -23.53 μHATA = 13.77 | LHATA = -31.33 μHATA = 13.77 |
| hB = 100 m | LHATA = -6.17 μHATA = 11.80 | LHATA = -13.97 μHATA = 11.80 | LHATA = -21.77 μHATA = 11.80 | LHATA = -29.57 μHATA = 11.80 |
| hB = 200 m | LHATA = -4.42 μHATA = 9.83 | LHATA = -12.22 μHATA = 9.83 | LHATA = -20.02 μHATA = 9.83 | LHATA = -27.82 μHATA = 9.83 |

Table A1: Hata's model excess path loss coefficients for small/medium cities at f0 = 1 GHz.

|  | hT = 1 m | hT = 4 m | hT = 7 m | hT = 10 m |
| --- | --- | --- | --- | --- |
| hB = 30 m | LHATA = -9.19 μHATA = 15.22 | LHATA = -14.48 μHATA = 15.22 | LHATA = -17.27 μHATA = 15.22 | LHATA = -19.24 μHATA = 15.22 |
| hB = 50 m | LHATA = -7.90 μHATA = 13.77 | LHATA = -13.18 μHATA = 13.77 | LHATA = -15.97 μHATA = 13.77 | LHATA = -17.95 μHATA = 13.77 |
| hB = 100 m | LHATA = -6.15 μHATA = 11.80 | LHATA = -11.43 μHATA = 11.80 | LHATA = -14.22 μHATA = 11.80 | LHATA = -16.19 μHATA = 11.80 |
| hB = 200 m | LHATA = -4.39 μHATA = 9.83 | LHATA = -9.67 μHATA = 9.83 | LHATA = -12.46 μHATA = 9.83 | LHATA = -14.44 μHATA = 9.83 |

TableA2: Hata's model excess path loss coefficients for large cities at f0 = 1 GHz.

|  | Suburban areas | Open areas |
| --- | --- | --- |
| hB = 30 m | LHATA = -20.72 μHATA = 15.22 | LHATA = -39.47 μHATA = 15.22 |
| hB = 50 m | LHATA = -19.43 μHATA = 13.77 | LHATA = -38.18 μHATA = 13.77 |
| hB = 100 m | LHATA = -17.67 μHATA = 11.80 | LHATA = -36.42 μHATA = 11.80 |
| hB = 200 m | LHATA = -15.92 μHATA = 9.83 | LHATA = -34.67 μHATA = 9.83 |

Table A3: Hata's model excess path loss for suburban and open areas at f0 = 1 GHz.

#### A.2.1 Frequency Dependency of the Excess Path Loss

As the above calculations are based on a radio frequency of 1 GHz they are not directly applicable to the WRAN system. Clearly, the excess path loss is frequency dependent as at least the diffraction losses increase with f. Reliable investigations on the frequency dependency of the path loss based on measurements are rare since most channel sounders operate within a very limited band only. In [4], a model was proposed based on experiments at 0.45, 0.9, and 3.7 GHz. An excess path loss exponent of 0.6 was found appropriate for the modelling of the frequency dependency. With this extrapolation, the excess path loss in (2.2.1) can be predicted according to

Lexcess(f,d,hB,hT)=LHATA(hB,hT)+μHATA(hB,hT)logd+μexcesslog(ff0),

where μexcess = 6.

#### A.2.2 Location Variability

The large-scale path loss values depend on a great number of environmental factors. When the terminal or base stations move around in space, the received signal strength varies since the situation changes in terms of shadowing, number of reflected paths etc.. It has turned out that the signal strength variations are quite well described by a lognormal distribution. Hence, Lexcess provides the mean excess path loss in dB while σL is the standard deviation of the normal distributed signal strength in dB, known as the location variability [16]. In fact, σL depends on the frequency and the environment. In [16], σL is modelled according to

σL=0.65(log(f106))2−1.3log(f106)+SE

with SE = 5.2 for urban and SE = 6.6 for suburban environments, respectively.

Since the location availability for the WRAN systems is planned to be 50%, this equation will in fact not be exercised.

### A.3 NLOS ITU-R P.1546-1 Propagation Prediction Model

The propagation prediction model contained in ITU-R Recommendation P.1546-1 has been developed over the years for the point-to-area prediction of field-strength for the broadcasting, land mobile, maritime mobile and certain fixed services (e.g., those employing point-to-multipoint systems) in the frequency range 30 MHz to 3 000 MHz and for the distances range 1 km to 1000 km.

The model consists in a set of propagation curves that represent field-strength values for 1 kW effective radiated power (e.r.p.) at nominal frequencies of 100, 600 and 2 000 MHz, respectively, as a function of various parameters.  The model includes the methods to interpolate and extrapolate field-strength values from these three nominal frequencies.  These propagation curves were derived from the original “F-curves” still in use by the FCC in the USA for prediction broadcasting coverage.

The model also includes the method to obtain the effective height of the transmitting/base antenna above terrain height averaged between distances of 3 to 15 km in the direction of the receiving antenna.  For paths shorter than 15 km, the method can take account of the height of the transmitting/base antenna above the height of a representative clutter around its location.  The curves are produced for a receive antenna height corresponding to the representative height of the ground cover surrounding the receiving antenna location, i.e., 30m for dense urban area, 20 m for urban area and 10 m for suburban, rural and sea paths.  A correction method is provided if receiving antennas at different heights.

The model includes curves for 1%, 10% and 50% time availability and a method is given to interpolate for time availability in the range from 1% to 99%.  It also includes prediction over mixed land and see propagation paths.  The propagation curves represent the field strength value exceeded at 50% of locations within any area of typically 200 m by 200 m.  A method is given for a correction for different percentages of location based on a standard deviation of 5.5 dB for wideband digital broadcasting.

### A.4 Comparison between the two NLOS models

It is reported in Annex 7 of the ITU-R P.1546-1 Recommendation that this model gives results compatible with the Okumura-Hata model in the condition of mobile services in an urban environment for receive antenna height of 1.5 m, clutter height of 15 m and distances up to 10 km.

However, a comparison was made with the two models at 600 MHz, 10 m receive antenna height, and for 50% location availability in open rural areas and the results indicate that the Okumura-Hata model predicts higher received field-strengths than the P.1546-1 model. Tables 1 gives the results of this comparison.

Here are some further comparative notes on the two models:

- The Hata model covers the range of base antenna height of 30 m to 200 m whereas the P.1546-1 covers a range of 10 m to 1200 m;

- The Hata model covers the range of user terminal antenna heights from 1 m to 10 m whereas the P-1546-1 model assumes that the antenna is at the same height as the local clutter or ground cover  (i.e., 30 m in dense urban, 20 m in urban and 10 m elsewhere) with a correction factor depending on the path length for different antenna height;

- The Hata model predict up to a range of 20 km whereas the P-1546-1 model goes up to 1 000 km;

- The frequency range of the Hata model is from 150 MHz to 2 000 MHz whereas the P-1546-1 allows interpolation and extrapolation from 30 MHz to 3 000 MHz based on three nominal frequencies (100, 600 and 2 000 MHz);

- The excess path loss increases by 6 dB per decade in the Hata model whereas it is distance independent in the P-1546-1 model.

- The standard deviation for the location variability is frequency dependent and found to be 8 dB at 600 MHz whereas it is equal to 5.5 dB for wideband digital broadcast signals and frequency independent in the P.1546-1 model;

- The Hata model does not allow for a variation of time availability;

- The Hata model does not include prediction over sea paths;

Table A1:  Field-strength prediction difference between the Okumura-Hata model and ITU-R P.1546-1 model expressed in dB and in % (for 50% and 50%)
(positive values indicate larger excess path loss predicted by P.1546-1)

ITU model is more adequate and it can be justified.  The Hata model could be used at the implementation model for fine tuning.

ANNEX B

## Discrete Time Multipath Channel Model

The discrete-time impulse response model suitable for baseband Monte Carlo simulations is given by

h(τ)=k=0,1,...hkδ(τ−ktΔ),

where tΔ is the sampling interval. The complex-valued coefficients h0, h1, … for the tapped-delay-line model are randomly generated. It is reasonable to assume uncorrelated scattering, i.e., E[hk (hl)*] = 0 for k ≠ l. Also, a zero-mean complex Gaussian distribution is assumed for each coefficient with the variance given by

Ε[h02]=c0+c1(1−exp(−tΔτ1)),

Ε[hk2]=c1(1−exp(−tΔτ1))exp(−ktΔτ1),    k = 1,2,….

The complex Gaussian distribution of all tap coefficients leads to a Rayleigh fading characteristic in the absence of a direct path (i.e. c0 = 0), whereas otherwise a rician fading results. Taking the location variability into account, the parameters c0 and c1 are also random variables having a log-normal distribution.

ANNEX C

## Free Space Theoretical Multipath Channel Model

In order to establish the worst channel multipath situation possible to bound the phenomenon, a theoretical model was developed where free-space propagation was assumed on any direct and indirect transmission paths.  For a given distance between a transmitter and a receiver, the effect of reflectors located at regular azimuth and distance intervals around the receiver and assumed to be oriented to reflect the signal from the transmitter toward the receiver were simulated for the generation of multipath signals at the receiver.  The relationship between the level of each echo and its excess delay could then be established by this sampling mechanism.

The distance between the transmitter and receiver could be changed as well as the reflection coefficient (percentage of energy reflected).  The transmit antenna was assumed to be omni-directional while the receive antenna was either omni of directional by a factor of ‘cos5’ with a backlobe of 16 dB.

Figure C1 gives a scatter plot of the echoes present at the receiver for an omni-directional antenna and a 10 km transmit-receive separation and 10% reflectivity while Figure C2 gives the scatter plot for the directional receive antenna.

Figure C1:  Multipath scatter plot for omni-directional receive antenna and
10 km separation between transmitter and receiver

Figure C3 shows the trends for the maximum channel spread as a function of the separation distances between the transmitter and receiver.  In all cases, there is a decreasing trend in relative echo levels as a function of excess delay.  The worst case to be considered comes at the maximum distance that is expected to be reached by the transmitter.  The tapering of the multipath equalization window should therefore be based on this trend.

Because of the fact that the line of sight signal and any close-in echoes may be faded at the receiver, this simple downward trend would not apply all the time and pre-echoes will tend to come up since the receiver will tend to synchronize on the strongest signal which may be at some specific excess delay (relative to the line-of-sight signal).  The receiver synchronization within, and not only at the beginning of, the echo spread channel response needs to be considered in developing the multipath profiles to be used to test the various proposed WRAN technologies.

Figure C2:  Multipath scatter plot for directional receive antenna and 10 km separation between transmitter and receiver

Figure C3:  Maximum multipath trends for various separation distances between the transmitter and the receiver

### Annex D

### COST 207 Multipath Model

An extensive amount of work has been done to characterize the multipath, which occurs in the UHF range.  The main reference is the work done in Europe as part of the development of the GSM mobile radio system.  Although this work was done for mobile communication, some useful information could be extracted for fixed point-to-point operation by considering the somewhat limited directivity of the user terminal antennas (typically 60º in low UHF).  The GSM channel characterization work resulted in the following four mobile channel models, each representative of a different geographical environment, developed by the COST 207 committee on Digital Land Mobile Radio Communications

[34]:

Figure D1:  Multipath power delay profile for rural areas

Figure D2:  Multipath power delay profile for urban areas

Figure D3:  Multipath power delay profile for typical hilly terrain

Figure D4:  Multipath power delay profile for bad case hilly terrain

Extensive field measurements were carried out in the US as part of the EIA/NRSC DAR effort for digital radio broadcasting to mobile receivers in the 1992-1995 period [35] as well as in Canada, conducted by the CRC during the same time period [3].  All these results tended to confirm the validity of the COST 207 model for mobile reception, and similar models were used to derive the multipath parameters used to simulate typical channels in the EIA/NRSC DAR system tests [35].

ANNEX E

### Channel bandwidth versus frequency selective fading performance

A modulation system that is capable of fully taking advantage of a multipath environment (i.e., operating on the power sum of all echoes) is able to fully correct the effect of frequency selective fading due to multipath.  However, in the case of echoes with very small excess delays, which would produce flat fading over the entire transmission channel bandwidth, this modulation system will unfortunately have no means to recover the signal during these fades.

The only means to reduce the occurrence of this defect is to use a wider channel bandwidth, thus limiting the range of short echoes that will bring the system in a flat fading situation.  The results of a field measurement program undertaken by the CRC were reported to the ITU-R in 1993 [40].  A wideband pseudo-noise signal was transmitted by a 1.5 GHz high power transmitter over the Ottawa region and was received by a spectrum analyzer with a range of IF filter bandwidths.  The statistics of flat fading occurrence were analyzed.  Figure 6 summarizes the findings.  These results were used to confirm the need for a minimum channel bandwidth to allow adequate frequency diversity to cover for short echoes in a typical channel.  The results showed that the channel bandwidth has a major impact on the possible reduction in transmitter power for a same service availability.

The information about the possible reduction in transmit power for a given service availability lies in the difference in decibels between the cumulative distribution curves of the different bandwidths, at specific percentages of service availability.  Figure E1 shows that significant reductions in required transmit power can be obtained by increasing the channel bandwidth to avoid flat fading due to close-in echoes.  These curves represent a different way of showing the probability of occurrence of these echoes in a way that is readily usable for system design consideration.  Each curve can be divided into two sections, the first part being from 100 kHz to a bandwidth value that corresponds to a knee in the curve, the second part being from the knee position to the widest bandwidth measured (5 MHz). The criterion used to consistently locate the knee position is to find the point along the 99% service availability curve that corresponds to 1 dB transmit power increase as compared to the value obtained at 5 MHz.

This method of quantifying the effect of the bandwidth on the required transmit power was applied to different types of reception environments for an omni-directional antenna and the results are summarized in Table E1. This table shows the possible reduction in transmit power as the channel bandwidth is increased from 100 kHz to 5 MHz for service availability objectives of 90% and 99%.

Typically, the 90% service availability objective curves show a reduction in required transmit power in the order of 4 dB, from 100 kHz to the knee (1.1 to 1.9 MHz), and a reduction remaining below 0.7 dB from the knee to the 5 MHz bandwidth value whereas this reduction is in the order of 8 dB for 99% service availability.

Figure E1:  Reduction in required transmit power resulting from a widening of the transmission channel bandwidth (Dense urban, Ottawa)

| Type of environment | Knee position MHz | Typical reduction in required transmit power (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 100kHz-to-knee |  | Knee-to-5 MHz |  |
|  |  | 90% | 99% | 90% | 99% |
| Dense urban | 1.8 | 5.4 | 8.6 | 0.5 | 1.0 |
| Urban | 1.6 | 4.5 | 7.0 | 0.6 | 1.0 |
| Suburban | 1.9 | 4.1 | 8.1 | 0.6 | 1.0 |
| Rural, forest | 1.7 | 3.7 | 6.0 | 0.7 | 1.0 |
| Rural, open | 1.1 | 1.2 | 1.8 | 0.7 | 1.0 |

Table D1:  Multipath fade margins for service availabilities of 90% and 99%

The minimum transmission channel bandwidth for WRAN should therefore be around 2 MHz for the types of channels and reception installation assumed. Below 2 MHz, the multipath fading due to micro-reflections increases rapidly.

### ANNEX F

# Non-ideal RF DEVICES MODELS

In this section the impairments from various non-ideal RF components are addressed. Sect. 3.1 contains the models for the non-linear signal amplification in the transmitter, together with some results from corresponding simulations. Models for the phase noise process are addressed in Sect. 3.2 and Appendix C. In Sect. 3.3 and Appendix D, models for quadrature modulation and demodulation are derived.

## 3.1 Non-Linear Signal Amplification

The power amplifier is usually the most critical device as it exhibits non-linear characteristics, which distort the transmitted signal and hence degrade the error performance. In the following, an analysis of the expected performance loss is given for a non-linear input-output-system, based on appropriate parametric models which determine the deviation from an ideal linear device. Additionally, the necessary back-off for the signal amplification in the transmitter and the total signal-to-noise ratio (SNR) degradation due to the non-linearity is found, facilitating the RF components design process.

### 3.1.1 General Non-Linear System Models

The general model for a memory-less, non-linear system is the simple functional relationship between the input and output signal given by

yt=Fxt.

In communication systems, amplifiers and transducers are the devices that contribute most significantly to the non-linear characteristics. These devices are commonly modelled as memory-less non-linear systems, exhibiting non-linear gain (AM/AM) as well as amplitude-to-phase conversion (AM/PM). In the baseband representation, the AM/AM distortion only concerns the magnitude of the complex signal, whereas AM/PM converts the amplitude variations of the input signal into a phase modulation.

From the input bandpass signal

xt=Atcos2πf0t+φt

and after making the substitution

α=2πf0t+φt

the output signal can be expressed as

yα=FAcosα

in the neighbourhood of t. It is periodic in α and can hence be expanded in the Fourier series

yα=a0+k=1∞akcoskα+bksinkα.

Only the first-zone output, i.e. the k=1 terms in the above formula, are of interest. The bandpass signal at the output of the device can therefore be expressed as

yt=a1cos2πf0t+φt+b1sin2πf0t+φt	(3.1.1)

with

a1=1π02πFAcosαcosα dα	(3.1.2)

and

b1=1π02πFAcosαsinα dα.

It follows from the symmetry F(x) = F(-x) that the b1-term vanishes. The baseband equivalent model follows from (3.1.1) and (3.1.2), i.e.,

yl(t)=xl(t)xltfxlt,

where f(A) is defined as

fA=1π02πFAcosαcosα dα.

Here, xl(t) and yl(t) denote the complex envelopes of the input and the output signal, respectively.

### 3.1.2 Parametric RF Amplification Model

For the characterisation of the non-ideal amplification in the transmitter devices the common model

Fx=1+x−1s−s

is adopted, in which the extent of the non-linear behaviour is determined by the value of the parameter s ≥ 0. Figure F1 depicts the mapping F(·) for different values of s.

1dB compression point: P1dB

Figure F1: Parametric non-linear amplification model.

The 1 dB compression point determines the power level at which the output power deviation equals 1 dB.

## 3.2 Phase Noise

One of the impairments in multi-carrier or single carrier systems is the phase noise of the oscillators, which are used for signal up and down conversion. This non-ideal oscillator characteristic induces inter-channel interference (ICI) which degrades the overall system performance.

Appendix C covers some characteristics of the phase noise process, presents in brief the Power-Law Spectral Density Model, and proposes a statistical model and a simulation model, which is based on current bibliography.

## 3.3 Non-Ideal Modulator/Demodulator

Appendix D derives models for quadrature modulation and demodulation. It discusses the performance of digital modulation and provides a simple model for digital demodulation.

Quantisation noise is not discussed within the following sections, but should be taken in to account when implementing the models within a system simulation.

### ANNEX G

### Phase Noise and Power-Law Model

The phase noise topic is of great theoretical and practical interest. Although many studies have been made about it, phase noise (PN) remains a crucial issue in communication system design. Phase instabilities are phenomena that characterise any practical non-ideal oscillator. These fluctuations are strongly related to several physical mechanisms. There are systematic variations (drifts) that are called “long-term instabilities” and are due to ageing of the resonator material or due to frequency modulation by periodic signals. These are deterministic processes, which don’t afford any statistical treatment. Random short-term instabilities are caused due to thermal and flicker noise or by the oscillator’s environment. These are instabilities that need statistical treatment and respective models in order to be described.

The power spectral density S(f) of oscillator instabilities can be described from the Power-Law model, according to which

S(f)=hαfa

where a typically takes integer values, but also non-integer values may be encountered. The constant hα is a measure of the noise level. The factor f0(i.e., independent of f) represents the white noise, which originates from the additive white noise sources that reside in the oscillator’s loop. The factor f−1represents the flicker noise and the factor f−2 the effect of the environment on the oscillator (temperature, vibration, and shocks) which can be modelled as Random Walk noise [20].

Experimental data are usually plotted using log-log scaling, where the power law appears as straight lines. In practice the power-law model often used is [21]

S(f)=a=−33hafa,0≤f≤fco0,f>fco

where a cut-off frequency fcois introduced in order to avoid mathematical difficulties as infinite power.

### Wiener Statistical Phase Noise Model

In [20] it is justified that the oscillator’s environment (temperature, vibration shocks, etc.) can produce phase instabilities that can be modelled as a random walk (Brownian motion). From another point of view it has been proven [22] that the phase noise in an oscillator, regardless of its operating mechanism, has stochastic characteristics that can be very much like Brownian motion. So, the oscillators’ phase noise can be described by a Wiener process, with the zero-mean probability density function

f(θ,τ)=18π2βτe−θ28πβτ

and variance

E[θ(τ+τ0)−θ(τ0)]2=4πβτ

where β is the two-sided 3-dB linewidth of  the Lorenzian power density spectrum  of the oscillator.

This is a good model for phase noise, whenever the main impact upon the oscillator is the environment (as opposed to the flicker noise).

The impact of Wiener phase noise on OFDM systems performance has been under investigation by several authors [23], [24]. The great sensitivity that arises leads to the need for more extensive research of phase noise statistics. Several authors have tried to explore them, some using simulation techniques [25], and others through analytical methods. In [26] an analytical recursive method is presented which accounts for moments of the phase noise for any integral of a function of the Brownian motion:

I(t)=0tf(Bθ0(τ))dτ=0tf(θ0+B(τ))dτ

where

B(t),t≥0

is the Brownian motion staring from θ0=0 and

Bθο(t),t≥0

is the Brownian with an arbitrary starting value θ0, in case that it is measurable, bounded from below and satisfies

−∞∞f(y)e−|y|λdy<∞ with λ>0.

It is proved that

Eθ0I(t)n=n−∞∞f(y)0tEyI(t)n−1p(t−τ;θ0,y)dτdy	(3.2.1)

where Exis the expectation of a Brownian motion starting from xand

p(τ;θ0,y)=(4π2βτ)−12exp−(y−θ0)24π2βτ.	(3.2.2)

In case that the process is

I(t)=0tej(θ0+θ(τ))dτ

it can be decomposed into real and imaginary parts

I(t)=0tcos(θ0+θ(τ))dτ+j0tsin(θ0+θ(τ))dτ=Xθ0(t)+jYθ0(t).

Following (3.2.1-2), it is computed that

Eθο(I(t))=Eθο(Xθ0(t))+jEθο(Yθ0(t))

or

Eθο(I(t))=2(cosθ0+jsinθ0)1−e−2πβτ4πβτ.

Higher order moments can be found in the same fashion. Approximate pdf’s can be estimated by use of orthogonal series expansion or through a maximum entropy approach [26].

In order to simulate the effect of phase noise on OFDM, we can use the corresponding discrete-time Wiener process, where the phase noise term affecting the n-th sample of the m-th OFDM symbol can be modelled

φm(n)=φm−1(N−1)+l=0nθl,0≤n≤N−1, with   φ0(n)=l=0nθl

where θl  are stationary, zero-mean Gaussian samples with variance σθ2=4πβ.

It must be noted that in order to estimate or simulate the phase noise effect, both oscillators (at the base station and the receiver) should be accounted for. However, in most of the cases, the oscillator of the base station is stable enough to disregard its effect [23].

ANNEX H

Quarature Modulator

### 1.	General

Quadrature modulation is a technique to modulate a carrier using the quadrature components of the signal.  This technique has the advantage of being able to directly modulate to relatively high frequencies (up to 3GHz and above).  Its disadvantages include inband distortions.  These are caused by local oscillator (LO) leakage, imperfect amplitude/phase balance, and DC offsets.  Generally these detrimental effects become harder to contain as the carrier frequency and/or pass bandwidth is increased.

LO leakage at the output of the modulator will be generated from the internal mixers’ finite LO isolation, as well as DC offsets in the signal and/or in the mixers’ biasing.  Finite phase imbalance will always be present due to the fact that it is impossible to achieve a perfect 90 degree phase shift.  The tolerance of this specification will depend on the pass bandwidth within which the modulator is designed to operate.  Amplitude imbalance will be caused by the finite differences in the two quadrature paths of the modulator.  For instance differences in the phase splitter balance, mixer gain, combiner gain, and interstage matching will always contribute to amplitude imbalance.  Both amplitude and phase imbalance will generate a negative frequency component at the output of the modulator.

Generally quadrature modulator specifications state for their bandwidth of operation, LO leakage (dBc), amplitude imbalance (dB), and phase imbalance (max or RMS degrees).  Typical values for a good device would be –35dBc, 0.2dB, and 2 degrees max respectively.  However many devices state limits as poor as –25dBc, 0.5dB, and 5 degrees respectively.

#### 1.1 Derivation of Model

An equation for a modulated signal:

s(t)=a(t)⋅cos(ωct+θ(t))

can be represented by its quadrature component parts:

s(t)=x(t)⋅cos(ωct)−y(t)⋅sin(ωct)	(3.3.1)

where

x(t)=a(t)⋅cos(θ(t))y(t)=a(t)⋅sin(θ(t))

Equation (3.3.1) can be visualised in the diagram of a quadrature modulator:

Figure H1: Quadrature modulator.

To simplify the following algebra, let a=θ(t), b=ωct, and leave out the amplitude component a(t) until later on in the derivation.

In an ideal situation the output from the modulator can be derived thus:

s(t)=cos(a)⋅cos(b)−cos(a+π2)⋅cos(b+π2)cos(a)⋅cos(b)−sin(a)⋅sin(b)cos(a+b)2+cos(a−b)2−cos(a−b)2+cos(a+b)2=cos(a+b)

However by introducing amplitude imbalance, phase imbalance, LO leakage, and DC offset into the equation, we get:

s(t)=(cos(a)+m)⋅cos(b)−(x⋅sin(a)+n)⋅sin(b+c)=cos(a+b)2+cos(a−b)2+m⋅cos(b)−x⋅cos(a−b−c)2+x⋅cos(a+b+c)2−n⋅sin(b+c)

where ‘m’ and ‘n’ are the dc offsets (which are identical to LO leakage), ‘c’ is the phase imbalance, and 20⋅log10(x) represents amplitude imbalance in dB’s.

If we remove the LO leakage terms until later and use the following identities:

cos(a−b−c)=cos(a−b)⋅cos(c)+sin(a−b)⋅sin(c)cos(a+b+c)=cos(a+b)⋅cos(c)−sin(a+b)⋅sin(c)

we get:

s(t)=cos(a+b)⋅(12+x⋅cos(c)2)+sin(a+b)⋅(−x⋅sin(c)2)+cos(a−b)⋅(12−x⋅cos(c)2)+sin(a−b)⋅(−x⋅sin(c)2)sign(x⋅cos(c)−1)⋅12+x⋅cos(c)22+−x⋅sin(c)22cosa+b+atanx⋅sin(c)2(12+x⋅cos(c)2)+sign(1−x⋅cos(c))⋅12−x⋅cos(c)22+−x⋅sin(c)22cosa−b+atanx⋅sin(c)2(12−x⋅cos(c)2)sign(x⋅cos(c)−1)⋅12⋅1+2⋅x⋅cos(c)+x2⋅cosb+a+atanx⋅sin(c)1+x⋅cos(c)+sign(1−x⋅cos(c))⋅12⋅1−2⋅x⋅cos(c)+x2⋅cosb−a+atanx⋅sin(c)x⋅cos(c)−1

(3.3.2)

The term for LO leakage can be derived as follows:

m⋅cos(b)−n⋅sin(b+c)=m⋅cos(b)−n⋅sin(b)⋅cos(c)−n⋅sin(c)⋅cos(b)=cos(b)⋅m−n⋅sin(c)−sin(b)⋅n⋅cos(c)

=sign(m−n⋅cos(c))⋅m−n⋅sin(c)2+n⋅cos(c)2cosb+atann⋅cos(c)m−n⋅sin(c)	(3.3.3)

The formula for a quadrature modulator is given by (3.3.2) and (3.3.3) after multiplying them by the amplitude component a(t) and replacing variables ‘a’, ‘b’, and ‘c’.  However, a more convenient form for equation (3.3.3) can be used:

mean(a(t))⋅10LO20⋅cos(ωct)

where LO is the LO leakage in dBc.

#### 1.2 Performance of Model

The graphs in Figure H2, FigureH3 and Figure H4 show the effects of phase and amplitude imbalance and DC offsets.

Figure H2: Phase imbalance effects.

Note: 	The amplitude of the wanted or positive frequency term cos(b+a), will not be effected significantly by phase imbalance.

For negative phase offsets, the –ve frequency phase has a 180 degrees phase shift and is therefore not shown in the graph.

Figure H3: Amplitude imbalance effects.

Note:  Amplitude imbalance does not cause phase shifts in the signal.

Figure H4: DC offset effects.

Note:	DC offset is relative to signal at 1 unit peak.

Depending on the level of the DC offset on each path, the phase of the LO leakage will range between +/-90 degrees relative to the Local Oscillator phase.

### 2. Quadrature Demodulator

Quadrature demodulation is a technique to demodulate a modulated signal from its carrier to its quadrature components.

Generally quadrature demodulator specifications state for their bandwidth of operation, amplitude imbalance (dB), and phase imbalance (max degrees).  Typical values for a good device would be 0.2dB, and 2 degrees max respectively.  However many devices state limits as poor as 0.5dB, and 5 degrees respectively.

#### 2.1 Derivation of Model

90

Phase

Splitter

0

.y(t)

.x(t)

Figure H5: Quadrature demodulator. [reversal of figure]

Referring to the quadrature demodulator sketched in Figure H5.  If we let a=θ(t), b=ωct, and leave out the amplitude component a(t) until later on in the derivation, the equations for the quadrature outputs can be given as:

X=cos(b)⋅(cos(a+b)+L)=cos(2⋅b+a)2+cos(a)2+L⋅cos(b),Y=x⋅cos(b+π2+c)⋅cos(a+b)+L=−x⋅sin(2⋅b+a+c)2−x⋅sin(c−a)2+x⋅L⋅cos(b+π2+c)=−12⋅x⋅sin(2⋅b+a+c)+12⋅x⋅sin(a−c)−x⋅L⋅sin(b+c)

where as before, ‘x’ represents amplitude imbalance, and ‘c’ represents phase imbalance. For convenience LO leakage (or DC offset) on both sides is represented by ‘L’.

To evaluate the distortion of the demodulation process, the quadrature components are re-modulated:

s(t)=cos(b)⋅cos(2⋅b+a)2+cos(a)2+L⋅cos(b)−sin(b)⋅−12⋅x⋅sin(2⋅b+a+c)+12⋅x⋅sin(a−c)−x⋅L⋅sin(b+c)=14⋅cos(3⋅b+a)+cos(b+a)+14⋅cos(b+a)+cos(b−a)+x4⋅cos(b+a+c)+cos(3⋅b+a+c)−x4⋅cos(b−a+c)−cos(b+a−c)+L⋅cos(2⋅b)2+12+cos(c)2−cos(2⋅b+c)2=cos(a+b)⋅12+x2⋅cos(c)+cos(b−a)⋅14−x4⋅cos(c)−sin(b−a)⋅x4⋅sin(−c)+14⋅cos(3⋅b+a)−14⋅cos(3⋅b+a+c)+L⋅cos(2⋅b)2+12+cos(c)2−cos(2⋅b+c)2

The LO leakage generates components at DC and twice carrier frequency.  These can be ignored along with the third order components.  Re-introducing the amplitude component a(t) and leaving variables ‘a’, ‘b’, and ‘c’ in place for simplicity, we finally get:

s(t)=a(t)⋅12+x2⋅cos(c)⋅cos(b+a)+a(t)⋅sign(1−x⋅cos(c))⋅14⋅1−2⋅x⋅cos(c)+x2⋅cosb−a+atanx⋅sin(c)x⋅cos(c)−1

### 3. Digital Modulation

It is possible to modulate directly to a low IF frequency with a high-speed digital-to-analog converter (DAC).  A typical performance of 70dBc spurious free dynamic range can be expected by modulating to a carrier at a tenth of the sampling update rate.

The digital signal processing required to drive the DAC can be simplified by using a purpose built CMOS device.  For instance, analogue devices produce a digital quadrature modulator, which incorporates quadrature DDS and DAC.  The particular device is capable of offering 70dBc SFDR at 60MHz carrier frequency.  It incorporates interpolation, inv SINC and inv CIC filters, DDS, and a 14bit 200MSPS DAC.

In this type of modulation, apart from quantisation noise, other deficiencies are difficult to model.

### 4. Digital Demodulation

This type of modulation involves the technique of undersampling.  The analogue signal is sampled at a rate high enough to include all the information in the baseband signal.  Since the aperture time is very fast compared to the sampling rate, the process downconverts the signal from the carrier to baseband.

The current RF design for WRAN uses a 415MHz IF, which is probably too high for a cost effective undersampling solution.  However, a 2nd IF at 30MHz to 50MHz can be used without too much additional cost and complexity.

Undersampling offers improved performance over quadrature demodulation, and less resolution is required on the A-D converter.  In simulations aperture jitter could be increased as high as 50pS rms, whereas typically 1pS rms is usually specified.

#### 4.1 Derivation of Model

Apart from quantisation noise the main source of distortion is aperture jitter.  A formula for aperture jitter at baseband can be represented by:

s(t)=a(t)⋅cos(θ(t)+ωif⋅J(t))

where ωifis the 2nd IF carrier frequency, and J(t) represents normally distributed noise with an rms value equal to the aperture jitter of the device to be modelled.

___________________________

Re: 22-05-0061-01-0000_Non-linear_Channel_Model.xls and 22-05-0074-01-0000_Clipper_compression

In the entire document, log denotes the logarithm to the basis 10, i.e., log10(·).