IEEE P802.22
Wireless RANs

| Compression Point Calculations for a Hard Clipper with Gaussian and Sinusoidal Inputs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-08-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Steve Kuffner | Motorola Labs | 1301 E. Algonquin Road Schaumburg, IL, USA | 847-538-4158 | kuffner@labs.mot.com |

Abstract

This analysis complements the numerical results presented in [1].  Analytic solutions are presented for the compression (reduction of linear gain) of Gaussian and sinusoidal inputs to a hard clipper that is linear below the clipping points.  The 1 dB compression for the Gaussian input occurs for a backoff of 3.754 dB, while for the sinusoid it occurs for a backoff of 1 dB, where backoff is defined as the ratio of the clipping level to the rms signal level.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

### Gaussian Waveform

For a signal x governed by

p(x)=12πσe−x22σ2,	(1)

it is necessary to find the output power when x is passed through a clipper that can be described by

y(x)=x,    −C≤x≤C,=−C,          x<−C,=C,            x>C.	(2)

See Figure 1.

Figure 1.  Clipper characteristic.

E{y} remains zero if E{x} is zero since the clipping characteristic is symmetric.  The variance of y, E{y2}, is (where the 2’s are due to symmetry)

Ey2=0C2x2p(x)dxA+C∞2C2p(x)dxB.	(3)

The integral A is, using integration by parts (letting u=x,dv=xe−x22σ2),

A=20Cx2p(x)dx=σ2erf(C2σ)−2πCσ⋅e−C22σ2,	(4)

while the integral B is the familiar integral for the Q function but is expressed here in terms of the error function

B=2C∞C2p(x)dx=C2erfc(C2σ)=C2[1−erf(C2σ)].	(5)

The variance of the clipper output y, divided by the variance of the input x, E{x2} = σ2, is a measure of the compression.  This is described by

Ey2σ2=Cσ2−2πCσe−(Cσ)22+1−Cσ2erf12Cσ,	(6)

where C/σ is representative of the power amplifier backoff amount.  For example, C/σ very large is representative of a large backoff, while C/σ ≈ 1 is representative of severe clipping.  Call C/σ = b for ‘backoff’.  Figure 2 shows a plot of 10 log10(Eq. (6)) versus 20 log10b.  The 1 dB compression point occurs for a backoff of b = 1.54062 (3.754 dB).  That is, C = 1.54062σ, and this clipping point means the waveform is being clipped with a 12.34% probability.

Figure 2.  Plot of compression versus amplifier backoff for a hard clipper with Gaussian input.  One dB gain compression occurs for 3.75 dB backoff.

### Sinusoidal Waveform

For the sinusoidal waveform, the fundamental component of the Fourier series was computed versus C/2A, where A is the rms amplitude of the sinusoid:

a1=1π−ππf(θ)cosθ⋅dθ,	(7)

where f (θ) is the clipped cosine wave, which is, for θc=cos−1(C2A),

f(θ)=C,                 −θc≤θ≤θc,2Acosθ,          otherwise−C,               −π≤θ≤−π+θc,π−θc≤θ≤π.	(8)

Though formidable looking, the integrals are fairly simple to evaluate.  When a1 is divided by2A, this results in a measure of compression:

Compression =1+2πCA1−12CA2−2πcos−112CA.	(9)

This curve is plotted in Figure 3 versus backoff b = C/A.  The compression is now in 20 log10 since it is defined as the ratio of the distorted and undistorted sinusoid amplitudes as opposed to powers.  Since A is an rms value, the clipper doesn’t start distorting until the sine wave is backed off less than 3 dB (the peak to rms value for a sine wave).  The 1 dB gain compression occurs for a backoff of 1 dB, when it is 2 dB into clipping.

Figure 3.  Plot of compression versus amplifier backoff for a hard clipper with sinusoidal input.

References:

[1]  22-05-0061-00-0000, Non-linear_Channel_Model.xls, submitted by G. Chouinard, 17 July 2005.