IEEE P802.22
Wireless RANs

| Multipath experience in DVB |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-09-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Patrick Pirat | France Telecom | Rue du Clos Courtel 35512 Cesson-Sevigné France | 33299124806 | ppirat.ext@rd.francetelecom.com |

Abstract

This document reports on the results of experimental tests carried out on DVB receivers with respect to echo tolerance.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Since DVB-T was planned to technically operate in Single Frequency Networks (SFN) the system was specifically designed to deal with high power echoes. For that purpose a guard interval has been introduced between the OFDM symbols to guarantee the immunity of the signal with respect to multipath.

Echoes that are shorter than the guard interval duration add useful signal power. They however create an uneven channel response that requires an efficient channel estimation mechanism to be compensated.

The DVB-T receiver can generally withstand a 0 dB echo within the guard interval assuming a high C/N. Measurements carried out on some receivers (consumer electronics equipment) show that the echo attenuation level is 2 dB when the C/N ratio is 25 dB. The echo attenuation becomes 4 dB for a C/N ratio of 22 dB.

If the echo delay exceeds the guard interval, a fraction of each delayed symbol adds incoherently to the direct signal. This results in a noise-like inter-symbol interference that impairs the performances of the receiver.

Figure 1 (extracted from [1]) depicts a typical DVB-T characteristic where the plots represent the maximum tolerable echo power versus delay.

Figure 1

The system under test has the following features:

- Symbol duration: 231 µs;

- Guard interval duration: 7 µs;

- Outer channel coding: Reed-Solomon [204, 188] with Forney interleaver depth I=12;

- Inner coding: Punctured convolutional codes based on a mother code of rate ½ with 64 states. The generator polynomials are 171oct and 133oct.

With 64-QAM rate 2/3 the limit to operation with a 0 dB echo is the guard interval duration. The sharp fall at 60 µs is caused by the equalization mechanism.

With a very rugged code such as QPSK rate ½ the limiting factor is the channel equalizer. This figure shows that the system may operate with echoes significantly higher than the guard interval.

References:

[1] Evaluation of a DVB-T compliant digital terrestrial television system – IBC 97 publication

[2] The echo performance of DVB-T receivers – EBU Technical review – Septembre 2001