IEEE P802.22
Wireless RANs

| A PHY/MAC Proposal for IEEE 802.22 WRAN Systems Part 1: The PHY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-01-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Martial Bellec | France Telecom | France | +33 2 99 12 48 06 | Martial.bellec@francetelecom.com |
| Yoon Chae Cheong | SAIT | Korea | +82-31-280-9501 | Yc.cheong@samsung.com |
| Carlos Cordeiro | Philips | USA | +1 914 945-6091 | Carlos.Cordeiro@philips.com |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Hak-Sun Kim | Samsung Electro-mechanics | Korea | +82-31-210-3500 | hszic.kim@samsung.com |
| Joy Laskar | Georgia Institute of Technology | USA | +1-404-894-5268 | joy.laskar@ece.gatech.edu |

Abstract

Single carrier and multi-carrier modulation are well known and have been deployed for several years around the world for broadcasting applications. Wireless regional area network (WRAN) applications differ from broadcasting since they require flexibility on the downstream with support for variable number of users with possibly variable throughput. WRANs also need to support multiple access on the upstream. Multi-carrier modulation is very flexible in this regard, as it enables to control the signal in both time and frequency domains. This provides an opportunity to define two-dimensional (time and frequency) slots and to map the services to be transmitted in both directions onto a subset of these slots. We propose to consider OFDMA modulation for downstream and upstream links with some technological improvements such as spreading,   duo-binary turbo codes, LDPC, beam forming etc. The proposal also describes methods to scan for vacant TV bands and use a single or a multiple TV bands (through channel bonding) for WRAN applications.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

| Co-Author(s): |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Company | Address | Phone | email |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Soon-Ik Jeon | ETRI | Korea | +82-42-860-5947 | sijeon@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Bub-Joo Kang | ETRI | Korea | +82-42-860-5446 | kbj64370@etri.re.kr |
| Chung Gu Kang | ETRI | Korea | +82-2-3290-3236 | ccgkang@korea.ac.kr |
| KyungHi Chang | ETRI | Korea | +82-32-860-8422 | khchang@inha.ac.kr |
| Yun Hee Kim | ETRI | Korea | +82-31-201-3793 | yheekim@khu.ac.kr |
| Moon Ho Lee | ETRI | Korea | +82-63-270-2463 | moonho@chonbuk.ac.kr |
| HyungRae Park | ETRI | Korea | +82-2-300-0143 | hrpark@mail.hangkong.ac.kr |
| Denis Callonnec | France Telecom | France | +33-4-76-764412 | Denis.Callonnec@francetelecom.com |
| Luis Escobar | France Telecom | France | +33-2-45-294622 | Luis.Escobar@francetelecom.com |
| Francois Marx | France Telecom | France | +33-4-76-764109 | Francois.Marx@francetelecom.com |
| Patrick Pirat | France Telecom | France | +33-2-99-124806 | Ppirat.ext@francetelecom.com |
| Kyutae Lim | Georgia Institute of Technology | USA | +1-404-385-6008 | ktlim@ece.gatech.edu |
| Youngsik Hur | Georgia Institute of Technology | USA | +1-404-385-6008 | yshur @ece.gatech.edu |
| Dagnachew Birru | Philips | USA | +1-914-945-6401 | Dagnachew.Birru@philips.com |
| Kiran Challapali | Philips | USA | +1-914 945-6356 | Kiran.challapali@philips.com |
| Vasanth Gaddam | Philips | USA | +1-914-945-6424 | Vasanth.Gaddam@philips.com |
| Monisha Ghosh | Philips | USA | +1-914-945-6415 | Monisha.Ghosh@philips.com |
| Duckdong Hwang | SAIT | Korea | +82-31-280-9513 | duckdong.hwang@samsung.com |
| Ashish Pandharipande | SAIT | Korea | +82-010-6335-7784 | pashish@ieee.org |
| Jeong Suk Lee | Samsung Electro-Mechanics | Korea | +82-31-210-3217 | js0305.lee@samsung.com |
| Chang Ho Lee | Samsung Electro-Mechanics | Korea | +82-31-210-3217 | changholee@samsung.com |
| Wangmyong Woo | Samsung Electro-Mechanics | Korea | +82-31-210-3217 | wmwoo@samsung.com |
| David Mazzarese | Samsung Electronics Co. Ltd. | Korea | +82 10 3279 5210 | d.mazzarese@samsung.com |
| Baowei Ji | Samsung Telecom America | USA | +1-972-761-7167 | Baowei.ji@samsung.com |

Contents

1.	References	6

2.	Disclaimer	6

3.	Introduction	6

4.	Symbol description	6

4.1	OFDMA Symbol description	6

4.1.1	Time domain description	7

4.1.2	Frequency domain description	7

4.2	Symbol parameters	8

4.2.1	System frequency	8

4.2.2	Inter-carrier spacing	8

4.2.3	Symbol duration for different guard interval options	9

4.2.4	Transmissions parameters	9

5.	Data rates	10

6.	Superframe and frame structure	10

6.1	Preamble definition	12

6.1.1	Superframe preamble	12

6.1.2	Frame preamble	13

6.1.3	US Burst preamble	14

6.1.4	CBP preamble	14

6.2	Control header and map definitions	14

6.2.1	Superframe control header (SCH)	14

6.2.1.1	Sub-carrier allocation for SCH	15

6.2.2	Frame control header (FCH)	15

6.2.3	US Burst control header (BCH)	15

6.2.4	Downstream MAP (DS-MAP), Upstream MAP (US-MAP), Downstream Channel Descriptor (DCD) and Upstream Channel Descriptor (UCD)	16

7.	OFDMA sub-carrier allocation	16

7.1	Sub-carrier allocation in downstream (DS)	16

7.2	Sub-carrier allocation in Upstream (US)	18

8.	Channel coding	18

8.1	Data scrambling	19

8.2	Forward Error Correction (FEC)	19

8.2.1	Convolutional code (CC) mode	19

8.2.1.1	Convolutional coding	19

8.2.1.2	Puncturing	20

8.2.2	Duo-binary convolutional Turbo code (CTC) mode	21

8.2.2.1	Duo-binary convolutional turbo coding	21

8.2.2.2	CTC interleaver	21

8.2.2.3	Determination of the circulation states	22

8.2.2.4	Code rate and puncturing	22

8.3	Bit interleaving	23

9.	Constellation mapping and modulation	23

9.1	Spread OFDMA modulation	23

9.1.1	Data modulation	23

9.1.1.1	Spread OFDMA	24

9.1.2	Pilot modulation	24

10.	Base station requirements	25

10.1	Transmit and receive center frequency tolerance	25

10.2	Symbol clock frequency tolerance	25

10.3	Clock synchronization	25

11.	Channel Measurements/Sensing	25

11.1	Overall Sensing Scheme and Procedure	25

11.2	Energy Detection	27

11.2.1	RSSI measurement	27

11.2.2	Multi-Resolution Spectrum Sensing	27

11.3	Fine/Feature Detection	28

11.3.1	Fine Energy-based detection:	28

11.3.2	Signal Feature Detection	29

11.3.2.1	Part 74 Devices.	29

11.3.2.2	ATSC DTV  Detection	29

11.3.3	Cyclo-Stationary Feature Detection	30

11.3.4	Detailed requirements on signal detection	32

12.	Control mechanisms	33

12.1	CPE synchronization	33

12.1.1	Initial synchronization	33

12.1.2	Carrier synchronization	33

12.1.3	Targeted tolerances	34

12.2	Ranging	34

12.3	Power control	34

List of Figures

Figure 1 – OFDMA symbol format	7

Figure 2 – Frequency domain description of OFDMA signal. Note that this is a representative diagram. The number of sub-carriers and the relative positions of the sub-carriers do not correspond with the symbol parameters provided in Table 4.	8

Figure 6 – Superframe structure	11

Figure 7 – Frame structure	11

Figure 8 – PREF pseudo random sequence generator	12

Figure 9 – Superframe preamble format. ST – short training sequence, LT – long training sequence	12

Figure 10 – Frame preamble format. FST – frame short training sequence, FLT – frame long training sequence	13

Figure 11 – Scrambler initialization vector for BCH	16

Figure 12 – Channel coding process	18

Figure 13 – Partitioning of a data burst into data blocks	19

Figure 14 – Pseudo random binary sequence generator for data scrambler	19

Figure 15 – Data scrambler initialization vector for the data bursts	19

Figure 16 – Rate – ½ convolutional coder with generator polynomials 171o, 133o. The delay element represents a delay of 1 bit	20

Figure 17 – Duo-binary convolutional turbo code: Encoding scheme	21

List of Tables

Table 1: System frequency for different single TV channel bandwidth options	8

Table 2: Inter-carrier spacing and FFT/IFFT period values for different bandwidth options	8

Table 3: Symbol duration for different guard intervals and different bandwidth options	9

Table 4: OFDMA parameters for the 3 bandwidths with different channel bonding options	9

Table 5: PHY Mode dependent parameters. Note that the data rates are derived based on 2K sub-carriers and a TGI to TFFT ratio of 1/16	10

Table 6: Pilot allocation in each of the sub-channels for DS	17

Table 7: Puncturing and bit-insertion for the different coding rates	20

Table 8: Circulation state correspondence table	22

Table 9: Puncturing patterns for turbo codes (“1”=keep, “0”=delete)	22

Table 10: Modulation dependent normalization factor	23

Table 11: The number of coded bits per block (NCBPB) and the number of data bits per block (NDBPB) for the different constellation type and coding rate combinations	24

Table 13: Tolerance in time, frequency and synchronization for different coding rates. Ts= Symbol duration, Cs= Carrier spacing	34

# References

[1] C. Cordeiro et. al., “A Cognitive PHY/MAC Proposal for IEEE 802.22 WRAN Systems: Part 2 MAC Specification”, proposal to IEEE 802.22, Nov 2005.

[2]	Functional requirements for the IEEE 802.22 WRAN standard – doc IEEE 802.22-05/0007r46

[3]	WRAN Channel Modelling – doc IEEE802.22-05/0055r7

[4]	IEEE P802.22 Call for proposal –

# Disclaimer

Due to lack of time, this document does NOT FULLY  describe the merged proposal of ETRI, FT, Philips and Samsung. It is provided here as an initial reference only.  Some of the areas that need to be updated or included are parameters (such as the number   used subcarriers), additional FFT modes, additional sub-carrier allocation scheme, beam forming and LDPC.  Refer to the power point presentation for details.

# Introduction

This document provides an overview of the basic technologies for the standardization of the physical (PHY) layer for WRAN systems. The specification provides a flexible system that uses a vacant TV channel or a multiple of vacant TV channels to provide wireless communication over a large distance (up to 100 Km).

The following sections of the document provide details on the various aspects of the PHY specifications.

The system parameters defined in this document will be further refined based on full simulation results.

# Symbol description

## OFDMA Symbol description

The transmitted RF signal can be represented mathematically as

sRF(t)=Ren=0N−1sn(t−nTSYM)exp(j2πfct),                         Equation 1

where Re(.) represents the real part of the signal, N is the number of symbols in the PPDU, TSYM is the OFDM symbol duration, fc is the carrier centre frequency and sn(t) is the complex base-band representation of the nth symbol.

sn(t)=00>t≥TSYM

The exact form of sn(t) is determined by the n and whether the symbol is part of the DS or US.

### Time domain description

The time-domain signal is generated by taking the inverse Fourier transform of the length NFFT vector. The vector is formed by taking the constellation mapper output and inserting pilot and guard tones. At the receiver, the time domain signal is transformed to the frequency domain representation by using a Fourier transform. Fast Fourier Transform (FFT) algorithm is usually used to implement Fourier transform and its inverse.

Let TFFT represent the time duration of the IFFT output signal. The OFDMA symbol is formed by inserting a guard interval of time duration TGI (shown in Figure 1), resulting in a symbol duration of TSYM = TFFT + TGI

Figure 1 – OFDMA symbol format

The specific values for TFFT, TGI and TSYM are given in Section . The BS determines these parameters and then conveys the information to the CPEs.

### Frequency domain description

In the frequency domain, an OFDMA symbol is defined in terms of its sub-carriers. The sub-carriers are classified as: 1) data sub-carriers, 2) pilot sub-carriers, 3) guard and Null (includes DC) sub-carriers. The classification is based on the functionality of the sub-carriers. The DS and US may have different allocation of sub-carriers. The total number of sub-carriers is determined by the FFT/IFFT size. Figure 2 shows the frequency domain description of an OFDMA symbol for 6 MHz based TV bands. This representation can be extended to 7 and 8 MHz based TV bands. Except for the DC sub-carrier, all the remaining guard/Null sub-carriers are placed at the band-edges. The guard sub-carriers do not carry any energy. The pilot sub-carriers are distributed across the bandwidth. The exact location of the pilot and data sub-carrier and the symbol’s sub-channel allocation is determined by the particular configuration used. The 6 MHz and 12 MHz version of the symbol are generated by nulling out sub-carriers outside the corresponding bandwidths.

Figure 2 – Frequency domain description of OFDMA signal. Note that this is a representative diagram. The number of sub-carriers and the relative positions of the sub-carriers do not correspond with the symbol parameters provided in Table 4.

## Symbol parameters

### System frequency

The system frequency is an important parameter of the system since it is the frequency at which the transmitter and the receiver equipment work. Two criteria should be considered for the choice of this frequency:

- The simplicity of its generation from the 10 MHz delivered by a GPS receiver;

- Its asynchronous behaviour with respect to the line frequency of the existing analogue TV system. In being asynchronous with the line frequency of the TV signals (15.625 kHz in the case of PAL and SECAM) the system frequency reduces the level of interference of WRAN in an analogue TV co-channel.

For these reasons, the following system frequencies reported in Table 1 are proposed.

Table 1: System frequency for different single TV channel bandwidth options

|  | 6 MHz | 7 MHz | 8 MHz |
| --- | --- | --- | --- |
| System frequency | 48/7 MHz | 8 MHz | 64/7 MHz |

### Inter-carrier spacing

The inter-carrier spacing F is dependent on the bandwidth of a single TV band (6, 7 or 8 MHz). The inter-carrier spacing remains same when multiple TV bands are bonded and is equal to the corresponding single TV band inter-carrier spacing. Table 2 Shows the proposed inter-carrier spacing and the corresponding FFT/IFFT period (TFFT) values for the different channel bandwidth options.

Table 2: Inter-carrier spacing and FFT/IFFT period values for different bandwidth options

|  | 6 MHz based channels (6, 12 and 18 MHz) | 7 MHz based channels (7, 14 and 21 MHz) | 8 MHz based channels (8, 16 and 24 MHz) |
| --- | --- | --- | --- |
| Inter-carrier spacing, F (Hz) | 6×106256∗6= 3348.214 | 7×106256∗6 = 3906.625 | 8×106256∗6 = 4464.286 |
| FFT/IFFT period, TFFT (s) | 298.666 | 256.000 | 224.000 |

### Symbol duration for different guard interval options

The guard interval duration TGI could be one of the following derived values: TFFT/32, TFFT/16, TFFT/8 and TFFT/4. The OFDM symbol duration for different values of guard interval is given in Table 3.

Table 3: Symbol duration for different guard intervals and different bandwidth options

|  |  | GI = TFFT/32 | GI = TFFT/16 | GI = TFFT/8 | GI = TFFT/4 |
| --- | --- | --- | --- | --- | --- |
| TSYM = TFFT + TGI (s) | 6 MHz | 308.000 | 317.333 | 336.000 | 373.333 |
|  | 7 MHz | 264.000 | 272.000 | 288.000 | 320.000 |
|  | 8 MHz | 231.000 | 238.000 | 252.000 | 280.000 |

### Transmissions parameters

Table 4 shows the different parameters and their values for the three bandwidths. Note that these parameters can be further refined based on regulatory requirements.

Table 4: OFDMA parameters for the 3 bandwidths with different channel bonding options

| Parameter | 3 TV bands |  |  | 2 TV bands |  |  | 1 TV band |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 18 | 21 | 24 | 12 | 14 | 16 | 6 | 7 | 8 |
| Inter-carrier spacing, F (Hz) | 3348 | 3906 | 4464 | 3348 | 3906 | 4464 | 3348 | 3906 | 4464 |
| FFT period, TFFT (s) | 298.66 | 256.00 | 224.00 | 298.66 | 256.00 | 224.00 | 298.66 | 256.00 | 224.00 |
| Total no. of sub-carriers, NFFT | 6144 |  |  | 4096 |  |  | 2048 |  |  |
| No. of guard sub-carriers, NG (L, DC, R) | 960 (480, 1, 479) |  |  | 640 (320, 1, 319) |  |  | 320 (160, 1, 159) |  |  |
| No. of used sub-carriers, NT = ND+ NP | 5184 |  |  | 3456 |  |  | 1728 |  |  |
| No. of data sub-carriers, ND | 4608 |  |  | 3072 |  |  | 1536 |  |  |
| No. of pilot sub-carriers, NP | 576 |  |  | 384 |  |  | 192 |  |  |
| Signal bandwidth (MHz) | 17.356 | 20.249 | 23.141 | 11.571 | 13.500 | 15.428 | 5.785 | 6.750 | 7.714 |

# Data rates

Table 5 defines the different gross data rates (using a single TV channel of 6 MHz bandwidth) and their associated parameters.

Table 5: PHY Mode dependent parameters. Note that the data rates are derived based on 2K sub-carriers and a TGI to TFFT ratio of 1/16

| PHY Mode | Modulation | Coding Rate | Spreading Factor | Spreading Matrix | Data rate (Mb/s) |
| --- | --- | --- | --- | --- | --- |
| 0 | QPSK | ½ | 4 |  | SCH |
| 1 | QPSK | ½ | 1 | spreading | 4.84 |
| 2 | QPSK | ½ | 1 | Identity | 4.84 |
| 3 | QPSK | ¾ | 1 | spreading | 7.26 |
| 4 | QPSK | ¾ | 1 | Identity | 7.26 |
| 5 | 16-QAM | ½ | 1 | Identity | 9.68 |
| 6 | 16-QAM | ¾ | 1 | Identity | 14.52 |
| 7 | 64-QAM | ½ | 1 | Identity | 14.52 |
| 8 | 64-QAM | 2/3 | 1 | Identity | 19.36 |
| 9 | 64-QAM | ¾ | 1 | Identity | 21.78 |
| 10 | 64-QAM | 5/6 | 1 | Identity | 24.20 |
|  |  | Additional coding rate to be described |  |  |  |

# Superframe and frame structure

The proposed superframe structure and frame structure are shown in Figure 3 and Figure 4 respectively. See the MAC specification [1] for a full description of the superframe and frame structures.

Figure 3 – Superframe structure

Figure 4 – Frame structure

## Preamble definition

The frequency domain sequences for the preambles are derived from the following length 5184 vector.

PREF(−2592:−1)=...PREF(0)=0PREF(1:2592)=...

PREF can be generated by using length-8192 pseudo random sequence generators and by forming the QPSK symbols by mapping the first 5184 bits of these sequence to the I and Q components respectively. The generator polynomials of the pseudo random sequence generator are given as

X13+X11+X10+X9+X5+X3+1(shown in Figure 5) and

X13+X11+X10+1

The generators are initialized with a value of 0 1000 0000 0000. Figure 5 shows the pseudo noise generator for PREF.

The first 32 output bits generated by the generator are 0000 0000 0001 0110 0011 1001 1101 0100 and the corresponding reference preamble symbols are given as PREF(-2592:2561) = {-1-j, -1-j, -1-j, -1-j, -1-j, -1+j, -1-j, -1-j, -1+j, -1-j , -1-j, +1+j, -1-j, +1+j, +1-j,-1-j, -1+j, -1-j, +1+j, +1+j, +1+j, -1+j, -1-j, +1-j, +1-j, +1-j, -1-j, +1+j, -1+j, +1-j, -1+j, -1+j}.

Figure 5 – PREF pseudo random sequence generator

### Superframe preamble

The superframe preamble is used by the receiver for frequency and time synchronization. Since the receiver also has to decode the SCH, it needs to determine the channel response. Therefore, the superframe preamble also includes a channel estimation field.

The format of the superframe preamble is shown in Figure 6. The superframe preamble is 2 symbols in duration and consists of 5 repetitions of the short training sequence and 2 repetitions of the long training sequence. The guard interval is only used for the long training sequence. The length of the guard interval for the Superframe preamble is given as TGI=14TFFT.

The duration of superframe preamble is Tsuperframe preamble = 746.666 ms (assuming 6 MHz based TV channels).

Figure 6 – Superframe preamble format. ST – short training sequence, LT – long training sequence

The short training sequence is generated from the above PREF sequence using the following equation

PST(k)=45×1728378PREF(k)k≤756,andkmod4=00otherwise

This will generate 4 repetitions of a 512-sample vector. Another replica of this vector is transmitted in the GI. The factor 45×1728378is used to normalize the signal energy. Note that the preamble symbols are transmitted at 3 dB higher power compared to the control and payload symbols. The short training sequence can be used for initial burst detection, AGC tuning, coarse frequency offset estimation and timing synchronization.

The long training sequence is generated from the reference frequency domain sequence as shown below:

PLT(k)=1728756PREF(k)k≤756,andkmod2=00otherwise

This will generate 2 repetitions of a 1024-sample vector. The GI precedes the long training sequence. The long training sequence is used for channel estimation and for fine frequency offset estimation.

For both the short training sequence and the long training sequence, the DC sub-carrier should be mapped to the center frequency of a single TV band. The superframe preamble is transmitted/repeated in all the available bands.

### Frame preamble

The format of the frame preamble is shown in Figure 7. The frame preamble will use the TGI specified by SCH.

Figure 7 – Frame preamble format. FST – frame short training sequence, FLT – frame long training sequence

The short and long training sequence of the frame preamble are derived according to the following equations

PFrame,ST(k)=2×45PREF(k)k≤864×Nbands,andkmod4=00otherwise

PFrame,LT(k)=2PREF(k)k≤864×Nbands,andkmod2=00otherwise

where Nbands represents the number of bonded TV bands.

The duration of superframe is relatively large and as a result the channel response may change within the superframe duration. Moreover the superframe preamble is transmitted per band, while the frame could be transmitted across multiple bands. Therefore, the channel estimates that were derived using the superframe preamble may not be accurate for the frames. In addition, the channel estimation sequence can be used by the CPEs to re-initialize the fine frequency offset calculation. Therefore, the transmission of the long training sequence in the frame preamble is mandatory. In order to save system resources, the BS may optionally choose not to transmit the short training sequence in the frame preamble under certain conditions. This information is carried in the FCH and is used to determine if the next frame’s preamble includes the short training sequence.

### US Burst preamble

The burst preamble is derived from the following equation

PBurst(k)=PREF(k),

where k represents the sub-carrier indices in the CPE’s allocated sub-channels.

The burst preamble is transmitted in the first symbol of the burst transmission.

The burst preamble is used by the BS to estimate the channel from the CPE to the BS. Transmission of the burst preamble on each burst is not very efficient under certain channel conditions. Therefore, the burst preamble field is made optional. The US-MAP field contains the information on burst preamble. The CPEs will only use their allocated sub-channels to transmit the burst preamble.

### CBP preamble

The structure of the CBP preamble is similar to that of the Superframe preamble. The CBP preamble is generated similar to the one for the Superframe preamble except that the last instead of the first 5184 samples from the 8191-length sequence are used to generate the I and Q components of the reference symbol sequence.

## Control header and map definitions

### Superframe control header (SCH)

The super frame control header includes information such as the number of channels, number of frames, channel number, etc. It also includes a variable number of IEs, due to which the length of SCH is also variable (with a minimum of 19 bytes and a maximum of 42 bytes). Additional details on the SCH are provided in the MAC specification.

The superframe control header is encoded using the methods/modules described in Section . The SCH is transmitted using the basic data rate mode. The 15-bit randomizer initialisation sequence shall be set to all 1s (i.e. 1111 1111 1111 111).  The SCH shall be decoded by all the CPEs associated with that BS (or in the region of that BS).

The super frame control header is transmitted in all the sub-channels. Since the superframe control header has to be decoded by all the CPEs in the range of the BS, the SCH has to be repeated in all the bands.

The 42 bytes of the SCH are encoded by a rate-1/2 convolutional coder and after interleaving are mapped using QPSK constellation resulting in 336 symbols. In order to improve the robustness of SCH and to make better utilization of the available sub-carriers, spreading by a factor of 4 is applied to the output of the mapper. This will result in 1344 symbols occupying 28 sub-channels (see Section  for the definition of sub-channel). This will free up 2 sub-channels on each of the band-edges, which are therefore defined as guard sub-channels. The additional guard sub-carriers at the band-edges will enable the CPEs to better decode the SCH. The 2K IFFT vector thus formed is replicated to generate the 4K and 6K length IFFT vectors. The TGI to TFFT ratio is ¼ for the SCH.

#### Sub-carrier allocation for SCH

The SCH uses only 28 sub-channels. The sub-carrier allocation is defined by the following equation.

SubCarrier(n,k)=Nch×(k−28)+(n−1)n=1,2,...,Nch=28k=1,2,...,27SubCarrier(n,k)=Nch×(k−27)+(n−1)n=1,2,...,Nch=28,k=28,29,...,54

The 6 pilot sub-carriers are then identified within each sub-channel. The pilot sub-carriers are distributed uniformly across the used sub-carries in the SCH symbol. Every 9th sub-carrier in the symbol is designated as the pilot sub-carrier. The sub-carrier indices of the pilots in the SCH are: {-756, -747, -738… -18, -9, 9, 18… 738, 747, 756}. The rest of the sub-carriers in the sub-channel are then designated as data sub-carriers.

### Frame control header (FCH)

The frame control header is transmitted as part of the DS PPDU in the DS sub-frame. The length of FCH is 6 bytes and it contains among others the length (in bytes) information for DS-MAP, US-MAP, DCD and UDC. The FCH shall be sent in the first two sub-channels in the symbol immediately following the preamble symbols.

The FCH is encoded using the channel coding modules described in Section . The FCH is transmitted using the basic data rate mode. The 15-bit randomizer is initialised using the 15 LSBs of the BS ID. The BS ID is transmitted as part of the SCH and is available to the CPEs. The 48 FCH bits are encoded and mapped onto 48 data sub-carriers in sub-channel #1 (Note that the sub-carrier allocation for FCH is as defined in Section ). In order to increase the robustness of the FCH, the encoded and mapped FCH data is re-transmitted in sub-channel #2. If SFCH,1(k) represents the symbol transmitted on sub-carrier k in sub-channel 1, then the symbol transmitted on sub-channel k in sub-channel 2, SFCH,2(k) is given as

SFCH,2(k)=SFCH,1((k+24),mod48)k=0,1,2...,47

The receiver should combine corresponding symbols form the two sub-channels and decode the FCH data to determine the lengths of the following fields in the frames.

### US Burst control header (BCH)

The burst control header is sent as part of the US PPDUs in the US sub-frame. Each CPE will use it allocated sub-channels to send the BCH in the symbol immediately following the US preamble symbols. If US preamble is not transmitted, then the BCH symbol shall be the first symbol of the US PPDUs. The BCH contains the BS ID and CPE ID information.

The BCH is encoded using the channel coding modules described in Section . The BCH is transmitted at the same data rate as the rest of the payload symbols. The randomizer is initialized using the 8 LSBs of the BS ID and 7 LSBs of CPE ID as shown in Figure 8.

Figure 8 – Scrambler initialization vector for BCH

### Downstream MAP (DS-MAP), Upstream MAP (US-MAP), Downstream Channel Descriptor (DCD) and Upstream Channel Descriptor (UCD)

The lengths of DS-MAP, US-MAP, DCD and UCD fields are variable and are defined in FCH. These fields are transmitted using the base data rate mode. The DS-MAP is transmitted in the logical channels numbers immediately following the FCH logical channel numbers. The DS-MAP is followed by the US-MAP, DCD and UCD in that order. The number of sub-channels required to transmit these fields is determined by their lengths and could possibly exceed the number of sub-channels allocated per symbol. In that scenario, the transmission of these fields will continue in the next symbol starting with the first logical sub-channel. It is anticipated that no more than 2 symbols would be required to transmit the FCH, MAP and descriptor information. The unused sub-channels in the second symbol can be used for DS transmissions.

# OFDMA sub-carrier allocation

Based on the parameters defined Table 4, there will be 32 sub-channels each with 54 sub-carriers in the 2K mode. For the 4K and 6K, the number of sub-channels will be 64 and 96 respectively. Each of the sub-channels will have 48 data sub-carriers and 6 pilot sub-carriers. Other modes with 1 or more sub-carriers per sub-channel are also possible, but are not defined at this time.

## Sub-carrier allocation in downstream (DS)

In the downstream, the sub-carrier allocation is done in two steps.

In the first step, each sub-channel is allocated 54 sub-carriers with the following criteria and is given by Equation 2:

- The sub-carriers are distributed across the bandwidth, and

- The sub-carrier indices represent the mirror images

SubCarrier(n,k)=Nch×(k−28)+(n−1)n=1,2,...,Nchk=1,2,...,27SubCarrier(n,k)=Nch×(k−27)+(n−1)n=1,2,...,Nch,k=28,29,...,54,                Equation 2

where n and k represent the sub-channel index and sub-carrier index respectively, and Nch represents the number of sub-channels and is equal to 32, 64 and 96 for single TV band, 2 TV bands and 3 TV bands respectively.

In the second step, 6 pilot sub-carriers are identified within each sub-channel. The pilot sub-carriers are distributed uniformly across the OFDMA symbol. Every 9th sub-carrier in the symbol is designated as the pilot sub-carrier. Table 6 gives the pilot sub-carrier index for the all the 32 sub-channels. It also gives the corresponding sub-carrier numbers within the sub-channel that are defined as pilots.

The above defined sub-carrier allocation is used for all the fields in the DS except for the SCH.

Table 6: Pilot allocation in each of the sub-channels for DS

| Sub-Channel # | Sub-carrier # within the sub-channel | Sub-carrier index | Sub-Channel # | Sub-carrier # within the sub-channel | Sub-carrier index | Sub-Channel # | Sub-carrier # within the sub-channel | Sub-carrier index | Sub-Channel # | Sub-carrier # within the sub-channel | Sub-carrier index |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | -864 | 9 | 3 | -792 | 17 | 5 | -720 | 25 | 7 | -648 |
| 1 | 10 | -576 | 9 | 12 | -504 | 17 | 14 | -432 | 25 | 16 | -360 |
| 1 | 19 | -288 | 9 | 21 | -216 | 17 | 23 | -144 | 25 | 25 | -72 |
| 1 | 36 | 288 | 9 | 34 | 216 | 17 | 32 | 144 | 25 | 30 | 72 |
| 1 | 45 | 576 | 9 | 43 | 504 | 17 | 41 | 432 | 25 | 39 | 360 |
| 1 | 54 | 864 | 9 | 52 | 792 | 17 | 50 | 720 | 25 | 48 | 648 |
| 2 | 8 | -639 | 10 | 1 | -855 | 18 | 3 | -783 | 26 | 5 | -711 |
| 2 | 17 | -351 | 10 | 10 | -567 | 18 | 12 | -495 | 26 | 14 | -423 |
| 2 | 26 | -63 | 10 | 19 | -279 | 18 | 21 | -207 | 26 | 23 | -135 |
| 2 | 29 | 63 | 10 | 36 | 279 | 18 | 34 | 207 | 26 | 32 | 135 |
| 2 | 38 | 351 | 10 | 45 | 567 | 18 | 43 | 495 | 26 | 41 | 423 |
| 2 | 47 | 639 | 10 | 54 | 855 | 18 | 52 | 783 | 26 | 50 | 711 |
| 3 | 6 | -702 | 11 | 8 | -630 | 19 | 1 | -846 | 27 | 3 | -774 |
| 3 | 15 | -414 | 11 | 17 | -342 | 19 | 10 | -558 | 27 | 12 | -486 |
| 3 | 24 | -126 | 11 | 26 | -54 | 19 | 19 | -270 | 27 | 21 | -198 |
| 3 | 31 | 126 | 11 | 29 | 54 | 19 | 36 | 270 | 27 | 34 | 198 |
| 3 | 40 | 414 | 11 | 38 | 342 | 19 | 45 | 558 | 27 | 43 | 486 |
| 3 | 49 | 702 | 11 | 47 | 630 | 19 | 54 | 846 | 27 | 52 | 774 |
| 4 | 4 | -765 | 12 | 6 | -693 | 20 | 8 | -621 | 28 | 1 | -837 |
| 4 | 13 | -477 | 12 | 15 | -405 | 20 | 17 | -333 | 28 | 10 | -549 |
| 4 | 22 | -189 | 12 | 24 | -117 | 20 | 26 | -45 | 28 | 19 | -261 |
| 4 | 33 | 189 | 12 | 31 | 117 | 20 | 29 | 45 | 28 | 36 | 261 |
| 4 | 42 | 477 | 12 | 40 | 405 | 20 | 38 | 333 | 28 | 45 | 549 |
| 4 | 51 | 765 | 12 | 49 | 693 | 20 | 47 | 621 | 28 | 54 | 837 |
| 5 | 2 | -828 | 13 | 4 | -756 | 21 | 6 | -684 | 29 | 8 | -612 |
| 5 | 11 | -540 | 13 | 13 | -468 | 21 | 15 | -396 | 29 | 17 | -324 |
| 5 | 20 | -252 | 13 | 22 | -180 | 21 | 24 | -108 | 29 | 26 | -36 |
| 5 | 35 | 252 | 13 | 33 | 180 | 21 | 31 | 108 | 29 | 29 | 36 |
| 5 | 44 | 540 | 13 | 42 | 468 | 21 | 40 | 396 | 29 | 38 | 324 |
| 5 | 53 | 828 | 13 | 51 | 756 | 21 | 49 | 684 | 29 | 47 | 612 |
| 6 | 9 | -603 | 14 | 2 | -819 | 22 | 4 | -747 | 30 | 6 | -675 |
| 6 | 18 | -315 | 14 | 11 | -531 | 22 | 13 | -459 | 30 | 15 | -387 |
| 6 | 27 | -27 | 14 | 20 | -243 | 22 | 22 | -171 | 30 | 24 | -99 |
| 6 | 28 | 27 | 14 | 35 | 243 | 22 | 33 | 171 | 30 | 31 | 99 |
| 6 | 37 | 315 | 14 | 44 | 531 | 22 | 42 | 459 | 30 | 40 | 387 |
| 6 | 46 | 603 | 14 | 53 | 819 | 22 | 51 | 747 | 30 | 49 | 675 |
| 7 | 7 | -666 | 15 | 9 | -594 | 23 | 2 | -810 | 31 | 4 | -738 |
| 7 | 16 | -378 | 15 | 18 | -306 | 23 | 11 | -522 | 31 | 13 | -450 |
| 7 | 25 | -90 | 15 | 27 | -18 | 23 | 20 | -234 | 31 | 22 | -162 |
| 7 | 30 | 90 | 15 | 28 | 18 | 23 | 35 | 234 | 31 | 33 | 162 |
| 7 | 39 | 378 | 15 | 37 | 306 | 23 | 44 | 522 | 31 | 42 | 450 |
| 7 | 48 | 666 | 15 | 46 | 594 | 23 | 53 | 810 | 31 | 51 | 738 |
| 8 | 5 | -729 | 16 | 7 | -657 | 24 | 9 | -585 | 32 | 2 | -801 |
| 8 | 14 | -441 | 16 | 16 | -369 | 24 | 18 | -297 | 32 | 11 | -513 |
| 8 | 23 | -153 | 16 | 25 | -81 | 24 | 27 | -9 | 32 | 20 | -225 |
| 8 | 32 | 153 | 16 | 30 | 81 | 24 | 28 | 9 | 32 | 35 | 225 |
| 8 | 41 | 441 | 16 | 39 | 369 | 24 | 37 | 297 | 32 | 44 | 513 |
| 8 | 50 | 729 | 16 | 48 | 657 | 24 | 46 | 585 | 32 | 53 | 801 |

## Sub-carrier allocation in Upstream (US)

The 2-step sub-carrier allocation is also used for the US. In the first step, Equation 2 is used to allocate 54 sub-carriers in each of the 32 sub-channels. In the second step, 6 pilot sub-carriers are identified within each sub-channel.

The following equation defines the location of pilot sub-carriers within the given sub-channel’s 54 sub-carriers:

PilotSubCarrierInd(n,m)=5+(m−1)×9,                            Equation 3

where m∈[1,2,...,6] is the pilot number in sub-channel n.

Optionally, the pilot sub-carriers in Upstream transmission can be transmitted at a higher power (about 3 dB) compared to the data sub-carriers.

The remaining indices are designated as data sub-carriers.

# Channel coding

Channel coding includes data scrambling, convolutional coding/duo-binary convolutional turbo coding, puncturing, bit interleaving and constellation mapping. Figure 9 shows the mandatory channel coding process. The channel coder processes the control headers and the PSDU portion of the PPDU. The channel coder shall not process the preamble part of the PPDU.

For the purpose of channel coding, each data burst is further sub-divided in to data blocks as shown in Figure 10. Each block of encoded data will be mapped and transmitted on a sub-channel. It is assumed here that distributed sub-carrier allocation is used to define sub-channels. If contiguous sub-carrier allocation is used then multiple blocks of encoded data will be mapped and transmitted on multiple sub-channels (not defined in this section).

Figure 9 – Channel coding process

Figure 10 – Partitioning of a data burst into data blocks

## Data scrambling

The PSDU data is first processed by the data scrambler using a pseudo random binary sequence (PRBS) generator. The PRBS generator polynomial is 1+X14+X15 and is shown in Figure 11. The preamble and the control header fields of the PPDU are not scrambled. The data scrambler is initialised on each burst and the 15-bit initialisation vector is created as shown in Figure 12. The generation of the initialization vector for control headers is described in Section . The pad bits if present shall be scrambled.

Figure 11 – Pseudo random binary sequence generator for data scrambler

Figure 12 – Data scrambler initialization vector for the data bursts

## Forward Error Correction (FEC)

Forward Error Correction can be either one or both of the below described modes

### Convolutional code (CC) mode

#### Convolutional coding

The data burst is encoded using a rate – ½ binary convolutional encoder. The constraint length of this coder is equal to 7 and its generator polynomials are 171o and 133o. Figure 13 shows the pictorial depiction of the generator polynomials. Output A and output B represent the first and second output bits respectively of this encoder.

The convolutional coder shall be initialised at the beginning of the control headers and at the beginning of the each burst. Tail biting shall be used with control headers, while tail bits (6 in number) should be used with the burst or PSDU data. This implies that for the case of control headers, the encoder memory is initialised with the last 6 bits in the header; and for the case of PSDU data, the encoder memory is initialised to all zero state.

Figure 13 – Rate – ½ convolutional coder with generator polynomials 171o, 133o. The delay element represents a delay of 1 bit

#### Puncturing

Different coding rates can be obtained by puncturing the output of the convolutional coder. Table 7 shows the different rates that can be derived from the output of rate – ½ convolutional coder and the associated puncturing patterns.

Decoding by using Viterbi algorithm is recommended. At the receiver, zeros are inserted in the locations of the punctured bits before the Viterbi decoder.

Table 7: Puncturing and bit-insertion for the different coding rates

| Code rate | ½ | 2/3 | ¾ | 5/6 |
| --- | --- | --- | --- | --- |
| Convolutional coder output | A1B1 | A1B1A2B2 | A1B1A2B2A3B3 | A1B1A2B2A3B3A4B4A5B5 |
| Puncturer output/bit-inserter input | A1B1 | A1B1B2 | A1B1B2A3 | A1B1B2A3B4A5 |
| Decoder input | A1B1 | A1B10B2 | A1B10B2A30 | A1B10B2A300B4A50 |

### Duo-binary convolutional Turbo code (CTC) mode

#### Duo-binary convolutional turbo coding

The duo-Binary Turbo Codes use Circular Recursive Systematic Convolutional (CRSC) Codes as component codes, with double-binary input.

The encoding system is fed by blocks of k bits or N couples (k=2xN). N is a multiple of 4 (k is a multiple of 8) and should be comprised between 32 and 4096. It is illustrated in Figure 14.

Figure 14 – Duo-binary convolutional turbo code: Encoding scheme

The polynomials, which shall be used for the connections, are described in octal and symbolic notations as follows:

- for the feedback branch: 15 (in octal), equivalently 1+D+D3 (in symbolic notation);

- for the Y1 and Y2 parity bits, 13, equivalently 1+D2+D3;

The input A shall be connected to tap “1” of the shift register and the input “B” shall be connected to the input taps “1”, D and D2.

This first encoding is called C1 encoding. After initialisation by the circulation state Sc1, the encoder shall be fed by the sequence in the natural order with incremental address i = 0,…,N-1.

This second encoding is called C2 encoding. After initialisation by the circulation state Sc2, the encoder shall be fed by the interleaved sequence with incremental address j = 0,… N-1.

The function (j) that gives the natural address i of the considered couple, when reading it at place j for the second encoding, is given in 1.1.1.5.

#### CTC interleaver

In the CTC interleaver, the permutation shall be done on two levels:

- the first one inside the couples (level 1),

- the second one between couples (level 2),

The permutation is described in the following algorithm.

- Set the permutation parameters P0, P1, P2 and P3.

These parameters depend on the size of the sequence to be encoded. For example, for MPEG2-TS packet size (188 bytes): P0 = 19, P1 = 376, P2 = 224 and P3 = 600.

- j = 0,… N-1.

- level 1

if j mod. 2 = 0, let (A,B) = (B,A) (invert the couple)

- level 2

- if j mod. 4 = 0, then P = 0;

- if j mod. 4 = 1, then P = N/2 + P1;

- if j mod. 4 = 2, then P = P2;

- if j mod. 4 = 3, then P = N/2 + P3.

    - i = P0*j + P + 1 mod. N

#### Determination of the circulation states

The state of the encoder is denoted S (0  S  7) with S = 4.s1 + 2.s2 + s3 (see Table 2). The circulation states Sc1and Sc2shall be determined by the following operations:

- Initialise the encoder with state 0. Encode the sequence in the natural order for the determination of Sc1or in the interleaved order for the determination of Sc2(without producing redundancy). In both cases, the final state of the encoder is denoted SN−10.

- According to the length N of the sequence, the following correspondence shall be used to find Sc1and Sc2(see the following table).

Table 8: Circulation state correspondence table

| SN−10 Nmod.7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Sc=0 | Sc=6 | Sc=4 | Sc=2 | Sc=7 | Sc=1 | Sc=3 | Sc=5 |
| 2 | Sc=0 | Sc=3 | Sc=7 | Sc=4 | Sc=5 | Sc=6 | Sc=2 | Sc=1 |
| 3 | Sc=0 | Sc=5 | Sc=3 | Sc=6 | Sc=2 | Sc=7 | Sc=1 | Sc=4 |
| 4 | Sc=0 | Sc=4 | Sc=1 | Sc=5 | Sc=6 | Sc=2 | Sc=7 | Sc=3 |
| 5 | Sc=0 | Sc=2 | Sc=5 | Sc=7 | Sc=1 | Sc=3 | Sc=4 | Sc=6 |
| 6 | Sc=0 | Sc=7 | Sc=6 | Sc=1 | Sc=3 | Sc=4 | Sc=5 | Sc=2 |

#### Code rate and puncturing

Three code rates are defined here (more code rates can be defined if required): R = ½, 2/3, and ¾.

These rates shall be achieved through selectively deleting the parity bits (puncturing). The puncturing pattern defined in the following table shall be applied.

Table 9: Puncturing patterns for turbo codes (“1”=keep, “0”=delete)

| Code Rate | Puncturing vector |
| --- | --- |
| 1/2 | Y = [1 1 1 1 1 1] |
| 2/3 | Y = [1 0 1 0 1 0] |
| 3/4 | Y = [1 0 0 1 0 0] |

## Bit interleaving

A two-step block interleaver shall be used to interleave the encoded and punctured data. The block size of the interleaver is determined by the parameter NCBPB (number of coded bits per encoded block, see Table 11). The first step of the interleaving process ensures that the adjacent coded bits are mapped onto non-adjacent sub-carriers in a sub-channel, while the second step of the interleaving process ensures that the adjacent coded bits are mapped alternately onto less or more significant bits of the constellation.

Let k, i, and j represent the index of the coded bits before the first permutation, after the first permutation and after the second permutation respectively. The first permutation is defined by the rule:

i=(NCBPBd)(kmodd)+floor(kd)k=0,1,...,NCBPB−1

The second permutation is defined by the rule:

j=s×floor(is)+(i+NCBPB−floor(d×iNCBPB))modsi=0,1,...,NCBPB−1

The value of s is determined from the parameter NCBPC (see Table 10) and is given as

s=NCBPC2

The parameter d is dependent on number of sub-carriers allocated per sub-channel. For the case of 48 sub-carriers per sub-channel, the value of d is equal to 16.

# Constellation mapping and modulation

## Spread OFDMA modulation

### Data modulation

The output of the bit interleaver is entered serially to the constellation mapper. The input data to the mapper is first divided into groups of NCBPC (see Table 10) bits and then converted into complex numbers representing QPSK, 16-QAM or 64-QAM constellation points. The mapping is done according to Gray-coded constellation mapping. The complex valued number is scaled by a modulation dependent normalization factor KMOD. Table 10 shows the KMOD values for the different modulation types defined in this section. The number of coded bits per block (NCBPB) and the number of data bits per block for the different constellation type and coding rate combinations are summarized in Table 11. Note that a block corresponds to the data transmitted in a single sub-channel.

Table 10: Modulation dependent normalization factor

| Modulation Type | NCBPC | KMOD |
| --- | --- | --- |
| QPSK | 2 | 12 |
| 16-QAM | 4 | 110 |
| 64-QAM | 6 | 142 |

Table 11: The number of coded bits per block (NCBPB) and the number of data bits per block (NDBPB) for the different constellation type and coding rate combinations

| Constellation type | Coding rate | NCBPB | NDBPB |
| --- | --- | --- | --- |
| QPSK | ½ | 96 | 48 |
| QPSK | ¾ | 96 | 72 |
| 16-QAM | ½ | 192 | 96 |
| 16-QAM | ¾ | 192 | 144 |
| 64-QAM | ½ | 288 | 144 |
| 64-QAM | 2/3 | 288 | 192 |
| 64-QAM | ¾ | 288 | 216 |
| 64-QAM | 5/6 | 288 | 240 |

#### Spread OFDMA

A 16X16 matrix is used to spread the output of the constellation mapper. The type of the matrix to be used for different configurations is shown in Table 5. For purpose of spreading, the output of constellation mapper is grouped into a symbol block of 16 symbols. Since each data block results in 48 symbols, a data block will generate 3 such symbol blocks.

The spreading is performed according to the following equation

S=CX,

where X represents the constellation mapper output vector and is given asX=x1,x2,...x16T,

S represents the spreaded symbols and are defined as S=s1,s2,...s16T, and C = H16 represents the hadamard spreading matrix and is given by the following Equation

H2n=H2n−1H2n−1H2n−1H2n−1,

where H1 = [1] and H2=111−1

The spreading matrix C = I16x16, an identity matrix, when non-spreading mode is selected.

The spreading matrix for SCH is defined in .

### Pilot modulation

The pilots are mapped using QPSK constellation mapping. Spreading is not used on the pilots. The pilots are defined as

Sp(k)=PREF(k)k<0,andk∈pilotindices0otherwise, and

Sp(k)=conj(PREF(−k))k>0,andk∈pilotindices0otherwise

# Base station requirements

## Transmit and receive center frequency tolerance

The transmitter and receive center frequency tolerance should be within ±2 ppm.

## Symbol clock frequency tolerance

The symbol clock frequency tolerance should be within ±2 ppm.

## Clock synchronization

The transmitter center frequency and the symbol clock frequency should be derived from the same reference oscillator

# Channel Measurements/Sensing

When channel measurement is mandated by the BS, CPEs shall make the required channel measurement. The channel measurements can range from simple received signal strength measurements (RSSI) or signal energy  in a given TV band  or frequency or the detection of the characteristics of the signal.  The RSSI can be used for quality measurement of the signal from the BS station, or for detecting the presence of any other signal in a TV band.  The measurement results are reported via ???? messages.

## Overall Sensing Scheme and Procedure

In this proposal, we suggest the spectrum sensing architecture shown in Fig. 1. The sensing system comprised of (i) a wideband antenna, (ii) a wideband RF front-end for down converting received signal (iii) energy detection for determine candidate channels and (iv) find/feature detection for identifying the type of incoming signal and detecting low power signals.

Figure 15: Sensing architecture

The unoccupied channel selection is done by two step approach to meet the time and sensitivity requirement from the MAC. First, multiple unoccupied channel candidates are determined by energy detection method. The threshold can be set by BS or CPE. The swapping time is more important than the sensing sensitivity at this stage. This info is sent to MAC for selecting one candidate channel for communication. Then Fine/Feature sensing is performed for the selected channel for identifying the type of incoming signal. Also at this stage, very low power narrowband signal can be detected. If any signal is detected the given channel, then MAC will select another candidate channel for fine/feature detection until finding unoccupied channel.

Figure 16: Sensing Procedure

Even, while in communication, the sensing block is working for finding other unoccupied channels, check if there is any attempt on the channel currently being used from a primary user and detecting the selected channel by request from base station for distributed sensing purposes. There could be many different type of detection method can be applied for the sensing block.

## Energy Detection

### RSSI measurement

The RSSI measurement shall be reported in  units of dBm. The actual implementation of the RSSI measurement is left to individual implementation. However, one possible method of implementation  is by measuring the energy in a given band and converting that to the input signal strength using

RSSI=10−G10?x104Vc222BR(E2) mw

where G is the RF front-end gain from antenna to ADC input in dB, Vc is the ADC input clip level,   E2  is the measured signal power. The signal power, E2, can be estimated using various techniques and is left to the particular implementation. However in order to have good interoperability,  the particular implementation need to result in a measurement that is similar to the value obtained using the following method. Assume the input signal in one TV band is r(k)=xI(k)+jxQ(k). Then, average this signal, y(k),   over a window of K samples.

p(k)=1Ki=0K−1r(k+i)2

Then estimate the mean and variance of  p(k)   using a first-order low-pass filter as

μ(k)=δμ(k−1)+(1−δ)p(k)

where μ(k) is the mean and δ is a constant set by the BS.  The mean and variance are reported back to the BS upon request using the equation provided above for RSSI.

### Multi-Resolution Spectrum Sensing

The MRSS is suggested as a flexible energy detection based spectrum sensing technique. The wavelet transform is applied to the input signal and the resulting coefficient values stand for the representation of the input signal’s spectral contents with the given detection resolution. Figure 3 shows the functional block diagram of the suggested MRSS technique. The building blocks consist of the analog wavelet waveform generator, the analog multiplier and the analog integrator for computing the correlation, and the low speed analog-to-digital signal converter (ADC) to digitize the calculated analog correlation values.

Figure 3. Functional block diagram of the MRSS

The wavelet pulse is generated and modulated with I-, Q-sinusoidal carrier with the given frequency. The correlations are calculated with the wavelet waveform with the given spectral width, i.e. the spectrum sensing resolution. By sweeping the local oscillator (LO) frequency with the certain interval, the signal power and the frequency values are detected over the spectrum range of interest. The resulting correlation with the I-, Q- components of the wavelet waveforms are digitized and their magnitudes are recorded. If these magnitudes are greater than the certain threshold level, the sensing scheme determines the meaningful interferer reception.

Since the analysis is performed in the analog domain, the high-speed operation and low-power consumption can be achieved. By applying the narrow wavelet pulse and the large tuning step size of LO, this MRSS is able to examine the very wide spectrum span in the fast and sparse manner. On the contrary, very precise spectrum searching is realized with the wide wavelet pulse and the delicate adjusting LO frequency. By virtue of the scalable feature of the wavelet transform, multi-resolution is achieved without any additional digital hardware burdens. Moreover, unlike the heterodyne-based spectrum analysis techniques, this MRSS technique does not need any physical filters for image rejection due to the band pass filtering effect of the window signal.

## Fine/Feature Detection

Upon request by the BS, the CPE shall perform signal detection in a given band. This would be from simple energy-based detection to detecting a specific feature of the signal.

### Fine Energy-based detection:

Energy based detection is simply comparing the energy estimated by using the above method to a threshold. The energy-based detection shall perform the following

μ(k)>cσ12,

where c is a constant and σ12 in the noise power of the RF input. The noise power can simply be estimated form the thermal noise adjusted for any other gain of the RF front-end. Alternatively, the CPE can also periodically estimate its input noise power using a vacant channel or by disconnecting the antenna. The CPE shall also report the confidence of this detection [TBD]. The BS shall provide adequate time for estimating the energy.

### Signal Feature Detection

Upon request by the BS, the CPE shall identify the type of the signal seen at its input, example ATSC TV,  DVB-T, Part 74 devices. The following subsections describe some of the method to be used for this signal feature detection.

#### Part 74 Devices.

Part 74 Devices (wireless microphone) occupy only a small portion of the TV spectrum.  This fact is used to detect if  there is enough energy in a part of a spectrum as follows:

First, an FFT operation is performed on the input signal as follows

Y(k,m)=n=0N−1rn+N∗ke−j2πnk

where Y(k,m) is the k’th block FFT, rn is the received data and N is the size of the FFT (N=2048 for one TV channel).  Generally, rn is composed of the noise and the narrower-band signal to be detected.     After performing the FFT, the received power spectrum is then computed and averaged over each freq bin

P(k,m)=1Ki=0K−1Y(k+i,m)2

Where K is set by the BS.    Further spectral averaging is performed by filtering this estimate, P(k,m), with the expected spectrum of the signal being detected.  In the presence of frequency selective multipath between the detecting device the transmitter of the signal being detected, the expected spectrum is not known. Then, a simple rectangular filter with bandwidth equal to the bandwidth of the signal can be used. The mean and a modified variance are computed using

μk=m=0N−1P(k,m)βk=m=0N−1P(k,m)−μk

The generalized detection method is described below.

max(P(k,m))>k1μk+k2βk

where k1 and k2 constants set by the BS.

#### ATSC DTV  Detection

The proposed technique here is based on correlating the received signal with a copy of a known reference signal. For US DTV, this will be the PN511 sequence. After the necessary frequency correction, the CPE shall correlate the input signal with the known PN511 sequence as.

p(k)=n=0N−1rn+kx(n)

A running mean and variance of this correlation output is computed using

μ(k)=δμ(k−1)+(1−δ)p(k)η(k)=δη(k−1)+(1−δ)(μ(k)−p(k))2

where the filter parameters are set by the BS. When the correlation output is random, then the mean and variance are identical. However, if the output is not random (such as when field sync is present  and a sample of p(k) exhibits a value very different from the normal range (example, a peak), then  the variance will respond faster than the mean. An ATSC DTV is declared detected when

η(k')>cμ2(k')

where k’ is the sample where the peak sample is shown in p(k) and c is constant set by the BS.

### Cyclo-Stationary Feature Detection

Communication signals have traditionally being modeled as stationary. A large class of signals like AM, FM, VSB, PSK, QAM, OFDM, CDMA in fact exhibit underlying periodicities in their signal structure. The scheme described herein aims to exploit these signal properties to detect and classify signals, and is based on the theory of cyclostationarity.

Consider a zero-mean discrete-time signal. We say that is cyclostationary with period if its autocorrelation function is -periodic, i.e.

We define the cyclic autocorrelation function (CAF) as

with the discrete time Fourier transform giving the cyclic spectrum density (CSD), also known as the spectral correlation function (SCF), defined as

The parameter  is called the cycle frequency; each cyclic frequency is an integer multiple of the fundamental time period of the signal. Note that for , the CAF and CSD reduce to the conventional autocorrelation and power spectral density functions. Also, due to the symmetry and periodicity in , the entire function is completely specified over . Further we note that a cyclostationary signal passed through a linear time-invariant channel, which the channels in [2] are modeled by, remains cyclostationary. Also, if the signal is composed of signals , i.e.

where signal has cycle frequency , we can extract the CSD  from  by considering the particular cycle frequency , i.e.

.

Different signals exhibit different underlying signal periodicities, i.e. exhibit distinct spectral characteristics at their cycle frequencies. For a large class of signals, we can determine what the cycle frequency is. We now consider a simple example of a BPSK signal and specify its cycle frequency. Consider the BPSK signal

where is the average power, the IF carrier frequency, the bit duration and a random message sequence. One can easily derive the expression for the CSD to show that it has spectral components at , for integer . Thus to detect a BPSK signal with known characteristics, one only has to analyze the CSD at .

To gain an intuition into how cyclostationarity based detection works, let us revisit the problem of binary hypothesis testing. We want to determine whether the signal of interest to be detected , that is transmitted over a channel with channel impulse response in the presence of additive white Gaussian noise (AWGN) , is present or not on the basis of the measured received signal . That is, we want to determine which of the following hypothesis is true:

It is easy to show that we have the following relation

The conventional energy detector corresponds to testing the energy levels obtained from  at for the signal absent and signal present cases. When the signal is heavily attenuated and/or is in the presence of a strong noise component, it becomes difficult to discriminate between the energy levels of signal+noise and noise. However a cyclostationary detector is not faced with this problem since detection can theoretically be done irrespective of the noise power level.

Different forms of detection statistics are possible, each derived from . Some examples are

  - Single-cycle magnitude detector:

where  is the sliding cyclic auto-periodogram and the observation window length.

  - Multi-cycle magnitude detector:

This is simply a detector, , that is evaluated over all or a set of cycle frequencies.

In fact, we can also consider one-dimensional processing to alleviate some of the computational burden that arises from two-dimensional processing by projecting on the cycle frequency axis to obtain the projection .

### Detailed requirements on signal detection

Even without using distributing sensing, there are already several possibilities to detect an incumbent thanks to the spectrum density:

- threshold on the signal energy in a sub-band

- threshold on a pilot frequency or on several pilot frequencies

- threshold on a correlation between the spectrum received and a known signature

The more general method is the correlation between the spectrum received and a known signature, it is know in the literature under the name Optimal Radiometer.  This is however not the purpose of this document to discuss these methods. However, some further details are needed to deeply specify the expected performance of the signal detection.

The detailed requirements for the signal detection are as follows:

- Gaussian channel and multipath channel. The effect of multipath channel on the ATSC and NTSC signal are given by the document WRAN channel modelling IEEE802.22-05/0055r7. (The approximation of quasi-static channel is valid because the sensing period is short compared with the inverse of the Doppler frequency).

- The video carrier (NTSC) or the digital pilot frequency (ATSC) are in different offsets in the 6 MHz. However, the maximum offset deviation is 10 kHz in comparison to the nominal values. According to the Shared Spectrum Comments, Appendix A to Federal Communications Commission, "In the Matter of Unlicensed Operation in the TV Broadcast Bands Additional Spectrum for Unlicensed Devices Below 900 MHz and in the 3 GHz Band", ET Docket No. 04-186 and ET Docket No. 02-380, the carrier and pilot can be set to:

  - The “standard” frequency

  - 10 kHz above the “standard” frequency

  - 10 kHz below the “standard” frequency.

  - Low power analog TV stations have different rules. Apparently they can put their video carrier anywhere between minus 10 kHz and plus 10 kHz in relation to the standard carrier frequency

- Noise Figure is set to zero. A simple shift from the Noise Figure level on the energy of the ATSC signal or peak of the NTSC carrier can be done for the test requirements.

# Control mechanisms

## CPE synchronization

All the CPEs will be synchronized with the BS using the superframe preamble. It is required that all the US transmissions will be received at the BS within 25% of the minimum guard interval.

We propose to define a two-step synchronization process: an initial (coarse) synchronization phase and a fine synchronization based on the ranging procedure.

### Initial synchronization

The initial synchronization process provides the CPE with a minimum time and frequency accuracy to enable it to recover the ranging information. The purpose of the initial synchronization is to provide to the CPE:

- The time of the next upstream transmission frame;

- The information to initiate its internal clock and reach the required time and frequency accuracy.

### Carrier synchronization

During this phase, the CPE can synchronize the carriers in phase and frequency to the RF upstream channel by using phase locked techniques to synchronize the local oscillator driving the CPE to the reference clock transmitted by the Base Station.

### Targeted tolerances

Table 12 sums up, for different encoding rates, the experimental (and theoretical) tolerances on the return channel transmitter characteristics:

- Time (t);

- Frequency (f/Cs);

- Synchronization accuracy (A).

Table 12: Tolerance in time, frequency and synchronization for different coding rates. Ts= Symbol duration, Cs= Carrier spacing

| Coding rate | no coding | 3/4 | 2/ 3 | 1/2 |
| --- | --- | --- | --- | --- |
| t | ± Ts/10 | ± Ts/6 | ± Ts /6 | ± Ts/5 |
| f/Cs | ± 0.03 | ± 0.04 | ± 0.05 | ± 0.075 |
| A | 20 dB | 17 dB | 17 dB | 20 dB |

## Ranging

## Power control