IEEE P802.22
Wireless RANs

| OFDMA Single Channel Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-05-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eli Sofer | Runcom | 2 Hachoma St. Rishon Lezion | +972 544 997 996 | elisofer@runcom.co.il |
| Ramon Khalona | NextWave | 2075 Las Palmas Dr Carlsbad, CA 92009 | 1-760-710-2063 | rkhalona@nextwavetel.com |
| Wendong Hu | STMicroelectronics | Brokaw Rd. San Jose, CA 95131, USA | 1-408-467-8410 | wendong.hu@st.com |
| Itzik Kitroser | Runcom | 2 Hachoma St. Rishon Le zion, Israel | +972-52-8701606 | itzikk@runcom.co.il |
| Zion Hadad | Runcom | 2 Hachoma St. Rishon Le zion, Israel | +972-54-4997996 | zionh@runcom.co.il |
| George Vlantis | STMicroelectronics 1 | Brokaw Rd. San Jose, CA 95131, USA | 408-451-8109 | george.vlantis@st.com |
| Paul Piggin | Cygnus |  |  |  |

Abstract

This document provides a full proposal of the OFDMA Physical (PHY) Layer specifications for the 802.22 (WRAN) standard. This proposal is compliant with the 802.22 Functional Requirement Document.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Table of Contents

1.	OFDMA symbol description, symbol parameters and transmitted signal	    4

1.1.	Time domain description	4

1.2.	Frequency domain description	4

1.3.	Primitive parameters	5

1.4.	Derived parameters	5

1.5.	Transmitted signal	5

1.6.	OFDMA subcarrier allocations	6

1.6.1.	Downlink	6

1.6.2.	Uplink	17

1.7.	OFDMA ranging	17

1.7.1.	Initial-ranging transmissions	18

1.7.2.	Periodic-ranging and bandwidth-request transmissions	18

1.7.3.	Ranging codes	19

1.7.4.	Ranging and BW request opportunity size	20

# OFDMA symbol description, symbol parameters and transmitted signal

## Time domain description

Inverse-Fourier-transforming creates the OFDMA waveform; this time duration is referred to as the useful symbol time Tb. A copy of the last Tg of the useful symbol period, termed CP, is used to collect multipath, while maintaining the orthogonality of the tones.  illustrates this structure.

Figure 1: OFDMA symbol time structure

The transmitter energy increases with the length of the guard time while the receiver energy remains the same (the cyclic extension is discarded), so there is a  dB loss in Eb/N0. Using a cyclic extension, the samples required for performing the FFT at the receiver can be taken anywhere over the length of the extended symbol. This provides multipath immunity as well as a tolerance for symbol time synchronization errors.

On initialization, an SS should search all possible values of CP until it finds the CP being used by the BS. The SS shall use the same CP on the uplink. Once a specific CP duration has been selected by the BS for operation on the downlink, it should not be changed. Changing the CP would force all the SSs to resynchronize to the BS.

## Frequency domain description

The frequency domain description includes the basic structure of an OFDMA symbol.

An OFDMA symbol is made up of subcarriers, the number of which determines the FFT size used. There are several subcarrier types:

- Data subcarriers: for data transmission

- Pilot subcarriers: for various estimation purposes

- Null carrier: no transmission at all, for guard bands and DC carrier

The purpose of the guard bands is to enable the signal to naturally decay and create the FFT “brick wall” shaping.

In the OFDMA mode, the active subcarriers are divided into subsets of subcarriers, each subset is termed a subchannel. In the downlink, a subchannel may be intended for different (groups of) receivers; in the uplink, a transmitter may be assigned one or more subchannels, several transmitters may transmit simultaneously. The subcarriers forming one subchannel may, but need not be adjacent. The concept is shown in Figure 2.

The symbol is divided into logical subchannels to support scalability, multiple access, and advanced antenna array processing capabilities.

Figure 2: OFDMA frequency description (3 channel schematic example)

## Primitive parameters

The following four primitive parameters characterize the OFDMA symbol:

- BW: This is the nominal channel bandwidth.

- Nused: Number of used subcarriers (which includes the DC subcarrier).

- n: Sampling factor. This parameter, in conjunction with BW and Nused determines the subcarrier spacing, and the useful symbol time. This value is set as follows: for channel bandwidths that are a multiple of 1.75 MHz then n = 8/7 else for channel bandwidths that are a multiple of any of 1.25, 1.5, 2 or 2.75 MHz then n = 28/25 else for channel bandwidths not otherwise specified then n = 8/7.

- G: This is the ratio of CP time to “useful” time. The following values shall be supported: 1/32, 1/16, 1/8, and 1/4.

## Derived parameters

The following parameters are defined in terms of the primitive parameters of 1.3:

- NFFT: Smallest power of two greater than Nused

- Sampling Frequency:

- Subcarrier spacing:

- Useful symbol time:

- CP Time:

- OFDMA Symbol Time:

- Sampling time:

## Transmitted signal

Equation (1) specifies the transmitted signal voltage to the antenna, as a function of time, during any OFDMA symbol.

where

- t is the time, elapsed since the beginning of the subject OFDMA symbol, with

- ck is a complex number; the data to be transmitted on the subcarrier whose frequency offset index is k, during the subject OFDMA symbol. It specifies a point in a QAM constellation.

- Tg is the guard time.

- Ts is the OFDMA symbol duration, including guard time.

- is the subcarrier frequency spacing.

  - OFDMA subcarrier allocations

For OFDMA, . Subtracting the guard tones from NFFT, one obtains the set of “used” subcarriers Nused. For both uplink and downlink, these used subcarriers are allocated to pilot subcarriers and data subcarriers. However, there is a difference between the different possible zones. For FUSC and PUSC, in the downlink, the pilot tones are allocated first; what remains are data subcarriers, which are divided into subchannels that are used exclusively for data. For PUSC in the uplink, the set of used subcarriers is first partitioned into subchannels, and then the pilot subcarriers are allocated from within each subchannel. Thus, in FUSC, there is one set of common pilot subcarriers and in PUSC of the downlink, there is one set of common pilot subcarriers in each major group, but in PUSC of the uplink, each subchannel contains its own set of pilot subcarriers.

### Downlink

The downlink can be divided into a three segment structure and includes a preamble which begins the transmission. This preamble subcarriers are divided into three carrier-sets. There are three possible groups consisting of a carrier-sets each, that may used by any segment.

A downlink period will follow Figure 3.

Figure 3: Downlink transmission basic structure

#### Preamble

The first symbol of the downlink transmission is the preamble; there are three types of preamble carrier-sets, those are defined by allocation of different subcarriers for each one of them; those subcarriers are modulated using a boosted BPSK modulation with a specific Pseudo-Noise (PN) code.

The preamble carrier-sets are defined using Equation (2)

where,

- specifies all subcarriers allocated to the specific preamble,

- n is the number of the preamble carrier-set indexed 0...2,

- k is a running index 0...567.

Each segment uses twoone types of preamble out of the three sets in the following manner:

Each segment uses a preamble composed of a carrier-set out of the three available carrier-sets in the following manner: (In the case of segment 0, the DC carrier will not be modulated at all and the appropriate PN will be discarded; therefore, DC carrier shall always be zeroed. For the preamble symbol there will be 172 guard band subcarriers on the left side and the right side of the spectrum).

- Segment 0 uses preamble carrier-set 0

- Segment 1 uses preamble carrier-set 1

- Segment 2 uses preamble carrier-set 2

Therefore, each segment eventually modulates each third subcarrier. As an example, Figure   depicts the preamble of segment 1 (in this figure subcarrier 0 corresponds to the first subcarrier used on the preamble symbol).

Figure 4: Downlink basic structure

The PN series modulating the preamble carrier-set are defined in Table 1. The series modulated depends on the segment used and IDcell parameter. The defined series shall be mapped onto the preamble subcarriers in ascending order. Table  includes the PN sequence in an Hexadecimal format. The value of the PN is obtained by converting the series to a binary series (Wk) and starting mapping the PN from the MSB of each symbol to the LSB (0 mapped to +1 and 1 mapped to –1, for example for Index=0, segment =0, Wk = 110000010010..., and the mapping shall follow: –1 –1 +1 +1 +1 +1 +1 –1 +1 +1 –1 +1 ...) :

Table 1: Preamble modulation series per segment and IDcell for the 2k FFT mode

| Index | IDcell | Segment | Series to modulate (Wk) | PAPR (informative) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0xC12B7F736CFFB14B6ABF4EB50A60B7A3B4163EA3360 F697C45075997ACE17BB1512C7C0CEBB34B389D8784553 C0FC60BDE4F166CF7B04856442D97539FB915D80820CED D858483 | 4.33 |
| 1 | 1 | 0 | 0xA9F7AC1BD0A4BD694D3EDC2991CC3B2D24BF26A223 46F8DB370202CDA25D382D4119AAC676E320A938A95762 C4078689B6024E477F0EDA8F563106F0D70EBE3E006F75B 50B537D | 4.21 |
| 2 | 2 | 0 | 0x56531FBB87033E4F362273BAF0F8879B45B9F19143E549 4F7B025D138DF057756DE625196292AF6D28FD0AA08453 E5B9871EDAE3E680B848C67BFBD7ADE73CFBBBA4E811 91267A | 4.32 |
| 3 | 3 | 0 | 0xB397F552DEB2717CC19DDF0D59674DD6F6D3866A3FD 023A009F592B56460660F1D585E3078AFE272D97FDF42807 90C3A9E5FCF9910895E9DAF2BF65728F7390C930428B4E6 793C | 4.36 |
| 4 | 4 | 0 | 0x1BD4C84B42DF6B7DC53F6C7B8E223A3B16D8E214CFA 5469A8D22246BCF297E5F92159406608B8A0BB55EF64A85 B1241C5CDFA048CF0492AB3BCF46A8E8FE986F06E246F1 E06C68 | 4.49 |
| 5 | 5 | 0 | 0x4E00947B6722B09389EFB4F6951C488B368393E82549483 59287441709C6F0E4463C067733C42A7FA89645D7D69AF2 ACE5402AC473DBF2C75ECB8B630BAF4B27F282249BD52 660 | 4.49 |
| 6 | 6 | 0 | 0x494CAB6935E10DB5D6E985997849EA45F0D5E2EDF767 0BFD9643531760D9F7CC01DD63BEEDFECB7E806F3F189 291C074C8289D93A95324D131391E23EB9CEEAB0E789DA 1F5B9CB | 4.35 |
| 7 | 7 | 0 | 0x4C5F10264D6C5085346E86BF8567294523C1B683D2A220 D9BEDCEBBA110620BB53ECB0338BE7109240E22EC902F CA05F97338BB9DF2DDEAF7C795BCB160BD4F01A6DBF2 A729373 | 4.48 |
| 8 | 8 | 0 | 0x79797D9AB260C20D5A460CDC49B2D0285E095E835EAF 2ECC74E010DD8A53797CE0EC2EEBA51E779AA6B749B8 E69FFDD632AC79D64143467E73017113BCDF45E787D0A9 EAC3D22E | 4.48 |
| 9 | 9 | 0 | 0xA1B9AC2C3D5B9BAAE5067C9E4A83C167076BE7D8699 ACA710FF205DE774FD46DD5F7851A2149D61E57152B98B 6AF4194B6FED90ADA008D1D5F8DD87E8060F943BC9124 C1999236 | 4.55 |
| 10 | 10 | 0 | 0x066E5FA91D00D63B26036009F8C69142B9D936396FF9E13 786478BBFF5DE6F184A0F844663950F69AFEDEA93CA4A3 BF94B13175A2CBBA3836A34E5CE6D763767B35515F332D 836 | 4,54 |
| 11 | 11 | 0 | 0xF443E9FBF763DA2A5137A57C7DA504D194EC1797AD3 3365BAC2F0C94541F4D47A664A7A17308C37E06BB0826F F999C15EE430A3CC54159E3B7EEBD5FF307BB24A939AB 261E2B3C | 4.53 |
| 12 | 12 | 0 | 0xF38BE6D2108483C056088C5E7C8BF92E9C973E0B2ADA 9342B46C06C4C2516CF7B9E6043E2947AD40F41734E02A9 ADCE9C70E03C4D50E7EAC73DAD56BDBB796289DDCC3 57776DE2 | 4.55 |
| 13 | 13 | 0 | 0x104AA84E70B163A42654A45995182B1C3DD63F4BCB09 ECA79A0D6D2D2A784DD6015794598310BE087F75019227 F899744B7C73A9008C83C0923D5DC154FB2DCBF8983E70 9BCDF3 | 4.54 |
| 14 | 14 | 0 | 0x0B49A507AC4EAAC7551FC4B00658A28D951FC81723C C1C024AAC6A9DE9686383C28036C762C020012D797866D E589B36BB95DFDAC2B3D0AB9DDE0B9719918062FE824E 063BA3EC | 4.52 |
| 15 | 15 | 0 | 0x64C14C7D3725A74923E6B2FB1C3BDC77FEE58CB0AF3 10EC37F22C93E2C809AE8410963E6CF5E7E192502960F027 2244A31D2CDDC657BCFF422E29C50D5E82EDCB4457918 1BBA4D | 4.41 |
| 16 | 16 | 0 | 0x210D8A8E602BD53F981BE763E10F4730BDA53D2F89BD 1D91C8F2DD5B96732935F789F643911937344E9F2ECF3222 AB076BC2B5EE407DC581F0EF9FFBD56D14D137A0418A DA06D0 | 4.47 |
| 17 | 17 | 0 | 0x88960A88E3F79C95D525DB49679C20A736D0E9E4D1FC B9DE7735AE1E947F4E93637E98143D6BB779394C58F2AC 5A9BD7B2074E98F1B2026B67B507CAAD8076082B09FB34 5DA02D | 4.54 |
| 18 | 18 | 0 | 0x1D2D5C8CDEAFCA5EC180D9638CD0F277AF08AB5133 E6D60C919AADAD00569E5F902D40500542631FB729FC3A F456C9A47E3EB967D51E09D712D8D49A028E738BEC9006 1B089C9F | 4.55 |
| 19 | 19 | 0 | 0xA063E03DE6C137F3FC56F970052BCF7333C8451BF5D18 D1B9AA5342E79C25451C1D862ECB5CFF21B7CC203817D 78C192EE1A68976652E1740C4B123552C85CEE524A2AA90 D428B | 4..66 |
| 20 | 20 | 0 | 0xD2A7F126A9599093A9262E66A2471B6B6A2ACB0A4330 A114011366CBC3B01CC85CF1915982BE64DDDF8EEA098 5D8F47BC4B41381C58271C30578960EEFB054F299C721B8 1D5DFE | 4.52 |
| 21 | 21 | 0 | 0x7682842351C76BC8E4A7EA4EDDB0F92F6E876FCFDFD AA4987B38FC4FA47C52EF0070DCC8C77FA622B20BEBC2 373011660B4960EF49FB5E519D79E12029C7D13C553EDC4 8564A52 | 4.58 |
| 22 | 22 | 0 | 0xAD6143F875C4C965A7018B8230D8D50297DA2C54A6D D52EA6207620F4A66EAFEBC4DD56233FF5DF78FB20CD7 4ECC6D01232FCFB9CBD36B3381F0224EF5DE7BE0AFEF0 A1AEF3D82 | 4.63 |
| 23 | 23 | 0 | 0x9A14B722E05D8455A80B4A1B1D12A30C1E25D9488BA D486C639CC7BDF651E957E041A7C092A916BF3E3642121 350579B3F8F8F4A30570237E722A6DC532A26F4FD4A0767 D91A8B | 4.72 |
| 24 | 24 | 0 | 0xE6944DEEE85D75E3C5D9B90912177D8A85909D87AC21 FA4A51660E11D30DBEED391E5972D000EF4E9BF30B63B 18C0285FE4151A4231C289A824D405142B7C775C3C68D8A A1D8A7 | 4.64 |
| 25 | 25 | 0 | 0x9927326FDFEC99025AA1B79364F06C63AFE4A96C2A20 FF8B151EF97AFD08E161EA6B10A1FA74794521DE02645C 2561D3BEA5D382AE3707112619403E23C724B36B791DFA FEAA3A | 4.52 |
| 26 | 26 | 0 | 0x03C19F38117AE5BDADF256FB4A223A660E2D626598F5 6580E30FA2E40A521FE5D68709B7F62E4C08CB9A26AE12 002AD2FA9DE6C2B298538556EFBA71626A02745C3DB5E BADD3F3 | 4.43 |
| 27 | 27 | 0 | 0xCBA035B40EB7C8A3A048C490E38935CBF956C58AFC8 91A6C112C0321CF5262498915794DCA703BD31A96FF4C06 36F2D5E9F17C23F1486B90715597D565017CB8E424DE9A8 E464E | 4.76 |
| 28 | 28 | 0 | 0x9321B7BE085143649644BCDF8342FCAD3462DA1C5722 27B039BBC6F58B52EEED2ACFB38F9CAA2BA2F513A87B 10DD19DEFB6A9972EE12D81C83DBFB3CFCC93D35ED25 2D0E1A3D1E | 4.67 |
| 29 | 29 | 0 | 0x215F6EA7C7F95C74828485AABF6A5F54FD32D1A8F4F6 F1C20E6CDB57FA81ED70DFE44ACCD4B37D4F01AD3BF3 1AFBB38A4DDBC613C8809E46C1247222E5041D8CDC08F 37F679878 | 4.51 |
| 30 | 30 | 0 | 0xB5ABE9FC329600031F97DEA8CF5B17EC432BB9F19082 A3CFA2682AAF121EE855873119A78869AF988BD90C64A7 F31224727D22F74D7499AF6CD3B649C54AED6DC84DD8A B876B84 | 4.74 |
| 31 | 31 | 0 | 0x956D097E914338D226020B8A3BA5B3BB8733A9723CF19 485DD9D22670B1328B825A6BA154586EDE60EB328AF8D F114182EDAEE401620A1E870BFFFF430922893C1F54A87A 90BD3 | 4.78 |
| 32 | 0 | 1 | 0x251D994101EDA04D8BD0B8EA6FA20AE590C2CC199A B083C6AE61F091F2DD41D989EC164B1481D611BE9CEA0 094AFE9DB56A4763F55B26E54EAB73ACD7D4BBA64C14 21BC3EB9D67 | 4.61 |
| 33 | 1 | 1 | 0x113A5FB9C529AADC9CAB1FB882905601778659CDB69 AFCBADDF8B42314A7985B5F87C20692309D350454FF932 6481683FADAE4711DD0CC5DACEDF7CD5DF1177D60EB A4DBE657F1 | 4.68 |
| 34 | 2 | 1 | 0x9F08189EFC6B5DE6C2CFDCD13195DE077586B8EE01E0 0B6468B10A53FAAC1DD846E2A01681980D444B6AD0D34 C34EC9CFD9341507878EC9FBAE498F5A20614BDF3E4B22 DD285E6 | 4.63 |
| 35 | 3 | 1 | 0x3ECD476669A04A260414FD16F3F525AA060F20ADD933 4A29A9D9F90618916EF51840C8F53AB596297F0782BEF42 6E8B8539C9FDE970455B58F533FDAC1711DE6310E7596E D285E | 4.58 |
| 36 | 4 | 1 | 0x3D6BD09A3DBD9ECCC1C584E71C87221CD266087C7A6 92D3EFF2D5F84DF2011EA3675853A61CD75D23600F8C11 5E03406AF914938170256B86DA5646CE0211FFDCD76A9A 5E8D840 | 4.68 |
| 37 | 5 | 1 | 0x27F0DA91D4AD1F39F0EAD459E2705CB2CA029A8E575 92F1697877199FF707D0411D6068A0664594D89568460F268 A225BB2AC0ED043659D779EA84656DEC0322F8C0CB111 AD2C8 | 4.60 |
| 38 | 6 | 1 | 0x616FBCE479AAAC98B483FCF6EC06BBA84580EA98FA5 17B3065A418CAF2C965B7AF2E7866B257390517016F25214 90088193372879FAA8954651E7B3C80BA1725CB781726F32 328 | 4.71 |
| 39 | 7 | 1 | 0x357714863C5F477BE963806EA9D6EF6350BAFC1C183FC C6BB47FFFEAA9FFD86358918F6A218266D624CA07092EC 24466C7F7120C1887A3F59A48EBAB67F24A6E8930B862F5 09A3 | 4.60 |
| 40 | 8 | 1 | 0xABE49D0F8B9C8406BD70B3FD83758768CDBA98164B9 29A1EA18D59BD44B80F9BBEF9D1CE51E4EE1CF21F6CC F18A7D4A92C26C121A22FC95663F0B55B892CF7D3D6581 2A503DA48 | 4.63 |
| 41 | 9 | 1 | 0x6898A2FC8C36DF0B84380FBCDE70812390B644E3B5BE A87D76C9123477638B331BEBC075664EA58C15680263664 C48BF3411C3C13789C504A01FB4C7B9AC86AA524075E52 C6A90 | 4.65 |
| 42 | 10 | 1 | 0xE7709988D2D2D6ABC6CEFB025FFCAFCA4C0E75C8835 29EA439B75229ECE88FB5BD5D3BCA17C25BEB6575D932 D01B5A63E044102E208C071C734EBA55712E122822ED2F2 B379A11 | 4.84 |
| 43 | 11 | 1 | 0xE49DCC8627542BA30FA500DCC23EEBF5A54B490EE76 32C6BE57C724C3E74CD199930AB1D929D425185E2E1220 CD2300F487392F4DC29416D332F13F8E760571D99617B263 F387D | 4.66 |
| 44 | 12 | 1 | 0xEE4CBD9B0EC65DE6DA78A2A205E5908B74127BDE612 A9BD2D8F0C6A2B9E675401A9DDAA30FF9A55E87DAFF A3A33E53AAA1D96A60B326D7F6FA147098DD825BD0FB 13ADDFE01569 | 4.65 |
| 45 | 13 | 1 | 0xAD7DAD0BCA42DEDAFCCF7E57DC58D00E691E81C04 B98B2EDB66C66570B204B8352A08744D8A603C2A7769C7 A9EE938189A45737B86871E5C4025EE594D827C603E3A49 FD45519F | 4.58 |
| 46 | 14 | 1 | 0xF8F29BA0D2FA2D529EAF2CE9383E614F5AE8CA06658 DD039AB2C9912DFC7CD1BA9744339E537850B7E4EA564 819772D3320B1C7BA73EC24D90B8DCE17EA5DAD53771F 68B050F43 | 4.68 |
| 47 | 15 | 1 | 0x9CC0ECC9E7E8940DFED1332AF492CFD39A21F2820394 EF0523019EE5290A2B4281FF032C238A6BE41116C274E91 8F34F3A27B5F147E10D41658CDC7EFEDC3135255C2B83B 0AE6A | 4.81 |
| 48 | 16 | 1 | 0xFCE9C41A74CBB56634447836109869E557C5A0FAA1D4 566E36A51258CE6D096FBD3E0B7193418D9DFCBD27693F 8A5072425D4E3F33DB5AB45B1EF3E11A6730BED42961D F0354CD | 4.72 |
| 49 | 17 | 1 | 0x7941B66A275FB8F0BAA8EF7FFCC36AAA660113B66BE 476D629AE512E489341F6C9F84EC1BE1C05CA3C850D20B 1A12AA9C94E1A6541C29A9B4BCD41B94460DEF2E9643A DCE86728 | 4.62 |
| 50 | 18 | 1 | 0x0A91F390213C9B2ED372BA19FC42EE85AAB2598B58D2 F7184EA920546D6A81ED316551B74B341E238A7FB83A4E F7D9EB0939B7771A6F4D0AF1F72752FE3234793D3CDC19 BDFE08 | 4.85 |
| 51 | 19 | 1 | 0x96949ACCD785385AE8DE99CE42BBB73B996A886115A 78D0606AEC14D2E46E849BF88F9A2E17C2494704F1020CE F85FFDE16B7483DBC6A130488E3AC586E528A00B901347 76E08C | 4.59 |
| 52 | 20 | 1 | 0x53FFBA26676A4FD1A6C30B8E4EA02DF535C922978CD 24F6099C25003567F207CC5851656C5FD0D3F071942A16F1 DB48DFBC26BACC15A1E618FF35F3DC3E141E3666BCA5 07ED72E | 4.70 |
| 53 | 21 | 1 | 0x7F3219ABE1389DBED8FC2F1C9C0FCE1974E71C224E19 22F4CAD42E40AF15A5ECDB14221480F964E67BBD345C4 4DCA0853548B399E3DF4D054D176C0804D1B1154152BE9 73A8896 | 4.66 |
| 54 | 22 | 1 | 0x8D5552CCA9EA46C991FF81A35873F43C963E02ED24C4 102A79F5EB5EC25814511BA5DD2FE9FB9699E7ED76F965 B24748AE1A4A3A590F4F13E4722CCE399006F79AD8CE67 3178F7 | 4.88 |
| 55 | 23 | 1 | 0xDB74EFA478268CDDE2596ADF9410FD83FCEDBB07C6 DEC7A3422A6CC66EF901C1534EC2A83E1A89BA207C721 ECF3D42918FF40B3863379FDF3CED7A9CC86E348CAB03 2F8FAAD9CD | 4.59 |
| 56 | 24 | 1 | 0x5811ADD9240A7B3AF35CACA6EFCCF4090A54EBF33D E54C077192354DDEB81CC968D18354090B09D7472C83E16 96E19545F08136EF20CD74656BCD31296066E03CC89E22E 47CF47 | 4.80 |
| 57 | 25 | 1 | 0x871ECE60EDD2E19360D13862A15242250635774424B224 65B3EF625E72072B7C45D81076DB4A5BFD5BE146F15CA A80DA031763DAC23BBBC54249E9878EC465F3EABE0B7 AA497B8 | 4.78 |
| 58 | 26 | 1 | 0xD8259BAF89D3E13242CC1CDBA9C0281A09919D24ECF 5BA83CDCD81E698EAA37DFE3E5802B3395B80A3DE91C F6C4BE2D34BBE985EE4041C4290D0A9185F115C963AD53 6E4133426 | 4.70 |
| 59 | 27 | 1 | 0x1203A1FD3F7B8E9D97A3812D4375D42BC9E8F0E393BD 669A8099407EC0356DC45FEF848C98F3EF32A9A850CC67 CE432339CBAF38BBA7DD0C94BC03B4704866509255E284 50E459 | 4.91 |
| 60 | 28 | 1 | 0x6A78A3F0DC5E4FA504580C37F5416BCC4A2BD51FC1A 71471BD1433EC3DD924E7130A7B2B331AB0B4AF6CF94C 045A9C246965F46478D939795887EE3320BBC2D5DD5FFB 06F894E8 | 4.73 |
| 61 | 29 | 1 | 0x3042F24E050BAB38880A07BDE5D28AEE4AD59E0D71A B59824122E80F8FF67BEA1ECC865F50B25CF5095C642B80 0E6A4D132B49E5968DEBDDA029A227AF332CF034BB937 B471603 | 4.74 |
| 62 | 30 | 1 | 0xC82853465FBB213A85A52888464E5D38D997F6C31966A 94B452A2DE853CE38010BF9EA930BBD318189D5D2D0BD B4465248A2E8B481021531BE01F5E0FF1BAB75370C57B36 BE6E9 | 4.80 |
| 63 | 31 | 1 | 0x2B17EC947632DABA3A2E11022033F20F873032F51F064 711111D0C215E9E84C11A9E70950977527960700B37C6FEC CA57C35A91873C935D7EDC22CB44DCC251396173CAC82 B912 | 4.63 |
| 64 | 0 | 2 | 0x83FC05D6DED982EB95B06E16C91EF94B441BBD4868C0 9E9B3A251AD72427F2124607E151796070C2819E395EB68 A2C597391636333A7E492B70D8EA7397FFA1B28C20E0820 CC45 | 4.89 |
| 65 | 1 | 2 | 0x45818FBAE49B983DB9B8FBEA1E816823430CE47BD159 3173605CA255CCCCAE73C7283336BCBC94133DCA64D67 5BBA848A3E1C2EEE35D6085F06E72EAA696FBED6EA545 F27D3692 | 4.89 |
| 66 | 2 | 2 | 0xA005745E452ACE6BDDFC4A9F6253BF4B467C93ACFF0 F663A3F5949F15A1D266DEB0D26EC16D2A083F830E878A 0300D74CFA3266CBBF3F0244ED56344D6AD5D887B3179 CE56890D | 4.74 |
| 67 | 3 | 2 | 0xE52F56367EA45E4683E020856D05D08391D3D84766CA2 2531B3EC6BE682E76B6ED7BCAABC3AB6BDE32C4F700D 4CDFF26F79AE16499D2B70EC389AC3ED5E02FDF5C43B2 96CF965D | 4.75 |
| 68 | 4 | 2 | 0xC5C62E3A911A0F28BF6A67E1DA2486FED7110B08F0A9 34C930AA035290D098857ECF3A069203A2560DADD50168 02D9C1596526C7F1DA7C7B53360B0A673AA8634FB94D58 38DC3E | 4.77 |
| 69 | 5 | 2 | 0xF9E176A9837BF970D5FD51732ECB8D90FBD12F62B62F 938BE07915BABB0C6596080C832CE7FF914C849B9DA66F 2380F3058F66340A34CA43583EC8EDC1E5CB5A2A25436E 72292D | 4.64 |
| 70 | 6 | 2 | 0x5C26DB2489BE5197B20CD6B38B181B7789DCB90881A AC4B4317B2F40B44884144A1B15BCB53C8E30FEC419861 C54B56158D9719E448B8A8F455F5B116275D796329059CD F682D3 | 4.64 |
| 71 | 7 | 2 | 0xEF35242D5C426C1EBD9563A761CFBF11A531ACB93892 2EBBA5227D8292585B777783972DA79C853A2A178601E6 CFEA35380045B50EE628F13AE3EC5B72FED52F92F731BB 594DE8 | 4.63 |
| 72 | 8 | 2 | 0x17FBB33193C68A1CAA8CEF5CB9A7FDFA1E89994F177 9D2D0DE69DE75A6B338A07635C80A58722BB01398252F6 C46AB7AEA79A6FC05F383F89ED65C49A2B9FB0D82A6E C03DD61F5 | 4.82 |
| 73 | 9 | 2 | 0x076C1A1DCA870DF36638307F891A52F737BA2B54EC0A D1FC5424D4F32DDE168ACE9B08653DEF8A23BF37CA3D 306138D698A133834BFB65BDE57048B9EEC630B13E91ED B1B52E8F | 4.86 |
| 74 | 10 | 2 | 0xA6800969ED0CE80A76F0F9BF7597ABE76DF60F243EE5 29C63AF72BAD6AB8B3F0B09DFC132C2B006827E55E352 E3940A3BB8EE3526522ADF65AB76492BA41A393740A4B8 5CD5113 | 4.78 |
| 75 | 11 | 2 | 0x64817485E539E02AAB074982A56DC6E867C606313194B D66B4CF8B9E92C4F9FD138B0E6EF36F699A3E6DCC46741 B8CB16389EBA2C745398A30EA3102B6BA4FE9A8DA9605 F929FF | 4.73 |
| 76 | 12 | 2 | 0xA3E438CB9EC48C4F4DD92C24950D0F1EAE7EEE92050 1C2C82531EDF8AEE3531F8D6B82D28C1FE0731088489FD 215D19202DAEE0A57E3E1634C7A1BD5395CA64C64C14E 5C02D436 | 4.68 |
| 77 | 13 | 2 | 0x8F9339B406037D35ADB9858576A62AF6139FD2B02D381 C7DF147A274E145F76DE5687AEC5BD3A715E0E893EEB6 F24573D4017B24B30D4357E339B104601FC2DD184DC9A8 F0D76A | 4.67 |
| 78 | 14 | 2 | 0xDA85062511E22DEDB53797BDBEC8B0281818E890D438 CB5A48B2E4011FD5EFB2192C0FEDEE598372C7C06BEF2 5F9B9702A8A9F0F52B197E9C910BB63F467E53BF46A45F7 5C22F6 | 4.69 |
| 79 | 15 | 2 | 0x914DE1436839A8E2FDD4EACCD645C3D29E522E19E005 5A2510679A977772830824C7363461CBF5D662456DB798B A72AEB67FEB2FC28DAAD3FAE0048727CF6E9B237A8248 9790D6 | 4.84 |
| 80 | 16 | 2 | 0xA21E09CF0A98EB012E8914A31BE5BE53F47AF5650B6B CE2812F65C994A100EA41F732830EB3F6C6F7028BC9FB3 C5D108F63315DEDE8EE82CF5DE892032688E1C367D8567 A20ACA | 4.75 |
| 81 | 17 | 2 | 0x6E11F29AE45A99D74D911777D1DE60495C2DAF1705C7 844FB7FC0A01247F3265F45D90A198ADCD0DB98A3CE22 ACF24A77C737E5BF99DCD7EFA6B6096B70C572996B62E 7814236B | 4.72 |
| 82 | 18 | 2 | 0xE9F8FC17F5361DBCDD8F18F28CA90B618DF6B56D3481 C9E3B7FDBEA6D55FAB32A4310A52AD7AAA26D082B38 D4D8A2FCB70A3C6A4167515CA710E8F9B237F64B4D9A8 C3CE8DF085 | 4.70 |
| 83 | 19 | 2 | 0x11FDC3B4712101D717D0EDD7556EAE0940AA1683D4C A4C22A7959436ECCA5E08A4BF2BF9EEF4BCE5E3E48DD 77EB418F6B84BF8937CE0CF9DAD247A64E9E850373FEE3 D673F47C2 | 4.74 |
| 84 | 20 | 2 | 0xE8784553C091233730B7DA704B8A02BEBE45E5DEF436 1394E3B0E417FE3B571E641ADF2603402B8084A2D1318AF 30CD95AD014D553408393AD345C05D62F435C708948233E F55B | 4.86 |
| 85 | 21 | 2 | 0xCDD6E932F9D2FAD131E7AE666B758CC4BB60C60230F E14494D0F77E89A9BE726FAF8F9465AD0262D75C0A5374 165A4FD2B4C602C0FF123F416360C112F6F95BD6790F81A CD858A | 4.84 |
| 86 | 22 | 2 | 0xA2702DE422E1CBADAA8285C1C3B1F41D44561BCBF10 5466DF8070E604C733DE579755BCB8237C8DDB55A865B2 13D1929EDC553CE9B55994985F9EBEDF2A9F524301E3DA 0498817 | 4.76 |
| 87 | 23 | 2 | 0x54487F7BDCDF87B1AA252798D7E5AD97E6F5263B7986 B1E3E637852EDC83FA360676C04E35A1F5045B0A0B7DE9 269F8A0E17F100D9AC78D873AE59BA0BA3E8AB3DDF92 8AD58F9E | 4.99 |
| 88 | 24 | 2 | 0x3461AA27EDE0A9F7955B469C41AE1485EFBFE4EB233B 0BBEB5F31BB36AC1E72CA6BF06B1E58F8612096CFA728 9DEA8927B6368DC845DC8476074B83F3C1545A17F73EFE 214A3C9 | 4.91 |
| 89 | 25 | 2 | 0x2DDBBF4F82EB33001E46F08D17DB89DCE3C7CC127F6 B7D17839FE17A86F69178A1903E91857147348B491631336 CB5D710382B59FF71416FEC2BAF0A0584F2155EEB71C54 F84C4 | 4.89 |
| 90 | 26 | 2 | 0x762E2454401F66455358322AC0CFDF76EB18EFA9684A1 0F0F527537A54E75FAD77BD89E0D47980793C7B79B922C 17792CC84BBEA81F6637192B74407A5B859EA1C873ED29 D48FD | 4.76 |
| 91 | 27 | 2 | 0xF74BBB6C4B97071EBC19F3FE7840A67A3959BD993633 5A4F8BD10C9CFD925D8388C31B947BFD318FFC8B0967C 132A602BC31B29835AE070006A50554CF3C5F85D56832A BA9CA5B | 4.89 |
| 92 | 28 | 2 | 0x06493F32F3CA54692CAE2579388B97D99B5D540DE71F8 B2405944C3A4FF18D7D45D4026DB9A867B85870BE6E23C 9A8F84332D29B84B0303BB5179DFD89B56A14A37ABE053 A0277 | 4.94 |
| 93 | 29 | 2 | 0xC88A3A3C0211A21661FD2B30937F0A187B6601E366A8F C5BCD4210E2D5D3365B22D4D63273F822D89EC1745304F BA4D0A9295AA51212C11C9D0A31FABB066289D8227B5B FDE8A0 | 4.82 |
| 94 | 30 | 2 | 0xA81E35C6A92953C584FE5FB3B6F1B0A532E91A49DB70 3D6E20D796F4532630C1D64DCEA580188BDDAB37722AD 5DDCC9DFE7CFEDE1518D8E2ACA842F3570C7F381EAB9 C5E4D485C | 4.80 |
| 95 | 31 | 2 | 0x08C0CC1C53E52AA366AFA63A48EBE2F7389C8A33CEB 20173432B4828D68A547D4673E27F942FCA95942029CFE9 F413FDABE1D0BCF95022C5B99C1B229D151E9D3CA0A12 2F1BEEF | 4.97 |
| 96 | 0 | 0 | 0x9774EF2FA326AB19DE599803EB48740C90995A4508064 B6B19E58304229C5EDC578EF2C7030D4D2A01C9FB7618E 7CB8564816354DD61EE144D7C94AFE8AB966875131B9F7 C18BB | 4.85 |
| 97 | 1 | 1 | 0xB9FBD947B7F9F3C8F6D3799E095BE558E6A2D0550C0D D0DDC92CC7BB53C1FE80D536B1FAE89C9224E3504629D BF0C5457944A72769B7162FBB0BBE18189749D3E7E264CF BA7A0E | 4.75 |
| 98 | 2 | 2 | 0xFD0E15EC140B2E87817AECC16F134B66221C759CCF0E 5000CDC0A3BADBB354D6845D745C22B1FB78C4205ABC F689495DE555CFFB4E4164A9ED06E484A192308A8CA890 48A92C32 | 4.81 |
| 99 | 3 | 0 | 0x28237E963DE488B97083F5A76BF5A861773DB61108461 A8CB8FAE918887897033207CEBFB83380BCC45748732F97 52C86DEA5F5EE4BA741C6DAB59375DDCBDC6EFEDBC D10DF3C2 | 4.84 |
| 100 | 4 | 1 | 0x023B7D4F9CA92D1E796C749B7664CCC4E8558C5CF20B F702E39BC3AE525A9FAA6581F4A22EF6829A44156DAE4 CABEA9C6A41D5A4325C02980C8FA4621A7FD08D874C68 7B68C706 | 4.94 |
| 101 | 5 | 2 | 0xB7FF5E696A6923C504E2A64A097EB201EC52D7963D9D 5DA46051A4EBA8B2C2DB9FC4249ABF2D8CCC881F8AA D20230F1B66D5D48CF2BCC5CADE7217E25FB9F6CB93C CE4111A33C6 | 4.86 |
| 102 | 6 | 0 | 0x6870AA97FE0FF504C4247EBFA8EF1A21B6EEE100E407 F293086E1F48C7292BEC491DDAF0E2CF02455825089FCD 985F77CDC4B561A6B8CD60CE31CBE6D467CFB4D153746 FB7BE0D | 4.92 |
| 103 | 7 | 1 | 0x91466310F3C4F355233B54C0AB8CB818780691443781B7 1AB6FB8F6CD516661E39075B4207E55400E081FD79C5246 28C8FE1277BE1A6165ACB5F154158D26593FED2C48EF66 268 | 4.89 |
| 104 | 8 | 2 | 0x45F8EB9235B6DC375771B69789AFEEEBB806965E6931 A844F370CA14AA982635C54EA0BA973373D9FE010993B4 1EB8BF2C219B09AD13B4FE7FDC7295C5585883449067463 7ED95 | 4.93 |
| 105 | 9 | 0 | 0x3AA7974B18884644F5C782A5E71AF70D91220EB0C468 D079AFB7DF8033D3AB54BC728657D60B349C575C8B3DE C403A6D406E3FC4D016655D406B0B78389CEFCFF8A37D8 67A44DF | 4.89 |
| 106 | 10 | 1 | 0x1140B404D18CA769BDC1E1188BBB5BFA3B87668D158 B0875F4D4EE90ED42974B5A02A6AAFC6977EACD194CB 9E8423E2931F2CD9AEF6C90F44EC626C56518360D20AE97 219FDE89 | 4.90 |
| 107 | 11 | 2 | 0x76BE5786CE3C33A20A3776587F83E6C5280BD4DF20FE 6C52D6BB582957E0CAEB988B32C3DB58027815D8618FB6 FDB1BCF9E871D6C552AED5679BE98189D95708FE92750C 5ADE33 | 4.84 |
| 108 | 12 | 0 | 0x33597C2D850E76B116A82F95C766D2002B9822D52E09B 1968BEF3DFD48D9F53D5296F1559BEB0BC7791C1F6B666 EE68C605A2098A4A0BD57CE4F7A843068A8BA3BF0065A CA53C6 | 4.95 |
| 109 | 13 | 1 | 0x894B11E2BB6884D9FFD78C6A8103F3BD44E6DFE48CD 0DC89C63A4F8BA95858545D37EC1652AB2C073B99BC667 D1F396C87F9902FCB08686E563D0D30EBF3D65756A63F00 37C240 | 4.83 |
| 110 | 14 | 2 | 0xDD08538B0939E852443E8801AB36C0FF50A6A0B63BBB E969F6A5A60BD6EEF19D070C3A14366EC789D39D07CD8 891491FDB3C7EF57A0A310C8A4DC0A03D5DB84DA0D69 11C4CBA9B | 5.03 |
| 111 | 15 | 0 | 0x7FFA4EF380C6504225EA6C8339E130DB7E69577E9C46C A494F66E2D5B25A256444606103A821615C2CDEA721D15 3669E5025CDC37904CFC16A84E3B745079E5F1F3E08B068 4BBB | 4.83 |
| 112 | 16 | 1 | 0x8A608CAD1CD85FD846FB2A39FB61EBF9A219B9B7499 179C2C066F3F78F3B3EDEF15B7227C650BEFB63C950E1B 52632D78D1A0F34552BA138C877F09FCCFD30511E340F79 4D154A | 4.90 |
| 113 | 17 | 2 | 0x775CA156A4C0BDB8FE5FF3CFB91FC7BC9DEAF1B8B3 362D06C9738D332868BBA3B18A0A907EE7918D95510298 E42F44B7BFC39D9E002EE24D1806EE0436B92DEC06DA3 FDA2230F6 | 5.14 |

#### Symbol structure

##### Symbol structure for PUSC

The symbol structure is constructed using pilots, data, and zero subcarriers. The symbol is first divided into basic clusters and zero carriers are allocated. Pilots and data carriers are allocated within each cluster. Table 2   summarizes the parameters of the symbol structure.

Table 2: OFDMA downlink carrier allocations—PUSC

| Parameter | Value | Comments |
| --- | --- | --- |
| Number of DC subcarriers | 1 | Index 1024 (counting from 0) |
| Number of Guard subcarriers, Left | 184 |  |
| Number of Guard subcarriers, Right | 183 |  |
| Number of used subcarriers ( ) | 1681 | Number of all subcarriers used within a symbol, including all possible allocated pilots and the DC carrier. |
| Number of subcarriers per cluster | 14 |  |
| Number of clusters | 120 |  |
| Renumbering sequence | 1 | Used to renumber clusters before allocation to subchannels: 6, 108, 37, 81, 31, 100, 42, 116, 32, 107, 30, 93, 54, 78, 10, 75, 50, 111, 58, 106, 23, 105, 16, 117, 39, 95, 7, 115, 25, 119, 53, 71, 22, 98, 28, 79, 17, 63, 27, 72, 29, 86, 5, 101, 49, 104, 9, 68, 1, 73, 36, 74, 43, 62, 20, 84, 52, 64, 34, 60, 66, 48, 97, 21, 91, 40, 102, 56, 92, 47, 90, 33, 114, 18, 70, 15, 110, 51, 118, 46, 83, 45, 76, 57, 99, 35, 67, 55, 85, 59, 113, 11, 82, 38, 88, 19, 77, 3, 87, 12, 89, 26, 65, 41, 109, 44, 69, 8, 61, 13, 96, 14, 103, 2, 80, 24, 112, 4, 94, 0 |
| Number of data subcarriers in each symbol per subchannel | 24 |  |
| Number of subchannels | 60 |  |
| Basic permutation sequence 12 (for 12 subchannels) |  | 6,9,4,8,10,11,5,2,7,3,1,0 |
| Basic permutation sequence 8 (for 8 subchannels) | 4 | 7,4,0,2,1,5,3,6 |

Figure 5 depicts the cluster structure.

Figure 5: Cluster structure

###### Downlink subchannels subcarrier allocation in PUSC

The carrier allocation to subchannels is performed using the following procedure:

- Dividing the subcarriers into 120 physical clusters containing 14 adjacent subcarriers each (starting from carrier 0).

- Renumbering the physical clusters into logical clusters using the following formula:

- Dividing the clusters into six major groups. Group 0 includes clusters 0–23, group 1 includes clusters 24–39, group 2 includes clusters 40–63, group 3 includes clusters 64–79, group 4 includes clusters 80–103, group 5 includes clusters 104–119. These groups may be allocated to segments, if a segment is being used, then at least one group shall be allocated to it (by default group 0 is allocated to sector 0, group 2 is allocated to sector 1, and group 4 to is allocated sector 2).

- Allocating subcarriers to subchannels in each major group is performed separately for each OFDMA symbol by first allocating the pilot carriers within each cluster, and then taking all remaining data carriers within the symbol and using the same procedure described in FUSC (with the parameters from Table 2, using the PermutationBasebasic permutation sequence appropriate for each major group, basic permutation sequence 12 for even numbered major groups, and basic permutation sequence 8 for odd numbered major groups) to partition the subcarriers into subchannels containing 24 data subcarriers in each symbol. Note that the preamble IDcell is used for the first PUSC zone in Equation xx). The subcarrier indexing within each group shall start from 0, where 0 is the lowest number subcarrier in the lowest numbered logical cluster belonging to the group.

##### Symbol structure for FUSC

The symbol structure is constructed using pilots, data, and zero subcarriers. The symbol is first allocated with the appropriate pilots and with zero subcarriers, and then all the remaining subcarriers are used as data subcarriers (these will be divided into subchannels).

There are two variable pilot-sets and two constant pilot-sets. In FUSC, each segment uses both sets of variable/constant pilot-sets. Table 3 summarizes the parameters of the symbol.

Table 3: OFDMA downlink carrier allocations—FUSC

| Parameter | Value | Comments |
| --- | --- | --- |
| Number of DC Subcarriers | 1 | Index 1024 (counting from 0) |
| Number of Guard Subcarriers, Left | 173 |  |
| Number of Guard Subcarriers, Right | 172 |  |
| Number of Used Subcarriers (Nused) | 1703 | Number of all subcarriers used within a symbol, including all possible allocated pilots and the DC carrier. |
| Pilots |  |  |
| VariableSet #0 | 71 | 0,72,144,216,288,360,432,504,576,648,720,792,864, 936,1008,1080,1152,1224,1296,1368,1440,1512,1584, 1656,48,120,192,264,336,408,480,552,624,696,768, 840,912,984,1056,1128,1200,1272,1344,1416,1488, 1560,1632,24,96,168,240,312,384,456,528,600,672, 744,816,888,960,1032,1104,1176,1248,1320,1392, 1464,1536,1608,1680 |
| ConstantSet #0 | 12 | 9,153,297,441,585,729,873,1017,1161,1305,1449 ,1593, |
| VariableSet #1 | 71 | 36,108,180,252,324,396,468,540,612,684,756,828, 900,972,1044,1116,1188,1260,1332,1404,1476,1548, 1620,1692,12,84,156,228,300,372,444,516,588,660, 732,804,876,948,1020,1092,1164,1236,1308,1380, 1452,1524,1596,1668,60,132,204,276,348,420,492, 564,636,,708,780,852,924,996,1068,1140,1212,1284, 1356,1428,1500,1572,1644 |
| ConstantSet #1 | 24 | 81,225,369,513,657,801,945,1089,1233,1377,152 1,1665 |
| Number of data subcarriers | 1536 |  |
| Number of data subcarriers per subchannel | 48 |  |
| Number of Subchannels | 32 |  |
| Basic permutation sequence |  | 3, 18, 2, 8, 16, 10, 11, 15, 26, 22, 6, 9, 27, 20, 25, 1, 29, 7, 21, 5, 28, 31, 23, 17, 4, 24, 0, 13, 12, 19, 14, 30 |

The Variable set of pilots embedded within the symbol of each segment shall obey the following rule:

where FUSC_SymbolNumber counts the FUSC symbols used in the current zone starting from 0.

Figure 6 depicts as an example of the symbol allocation for segment 0 on the first symbol.

Figure 6: Downlink symbol structure for segment 0 on first symbol using FUSC

###### Downlink subchannels subcarrier allocation

Each subchannel is composed of 48 subcarriers. The subchannel indices are formulated using a Reed-Solomon series, and is allocated out of the data subcarriers domain. The data subcarriers domain includes 48*32=1536 subcarriers, which are the remaining subcarriers after removing from the subcarrier's domain (0-2047) all possible pilots and zero subcarriers (including the DC subcarrier).

After allocating the data subcarriers domain, the procedure of partitioning those subcarriers into subchannels shall be as specified in the following section.

###### Partitioning of data subcarriers into subchannels in downlink FUSC

After mapping all pilots, the remainder of the used subcarriers are used to define the data subchannels.

To allocate the data subchannels, the remaining subcarriers are partitioned into groups of contiguous subcarriers. Each subchannel consists of one subcarrier from each of these groups. The number of groups is therefore equal to the number of subcarriers per subchannel, and it is denoted Nsubcarriers. The number of the subcarriers in a group is equal to the number of subchannels, and it is denoted Nsubchannels. The number of data subcarriers is thus equal to .

The exact partitioning into subchannels is according to Equation (4), called a permutation formula.

where,

- subcarrier(k,s) is the subcarrier index of subcarrier k in subchannel s,

- s is the index number of a subchannel, from the set [0...Nsubchannels-1],

- where k is the subcarrier-in-subchannel index from the set [0...Nsubcarriers-1]

- Nsubchannels is the number of subchannels (for PUSC use number of subchannels in the currently partitioned Major group),

- Ps[j] is the series obtained by rotating basic permutation sequence cyclically to the left s times,

- Nsubcarriers is number of data subcarriers allocated to a subchannel in each OFDMA symbol,

- DL_PermBase is an integer ranging from 0 to 31, which is set to preamble IDCell in the first zone and determined by the DL-MAP for other zones.

The numerical parameters are given in Table 3.

On initialization, an SS must search for the downlink preamble. After finding the preamble, the user shall know the IDcell used for the data subchannels.

### Uplink

The following section defines the uplink transmission and symbol structure. The uplink follows the downlink model, therefore it also supports up to three segments.

The uplink supports 70 subchannels where each transmission uses 48 data carriers as the minimal block of processing. Each new transmission for the uplink commences with the parameters as given in the following table.

Table 4: OFDMA uplink subcarrier allocations

| Parameter | Value |
| --- | --- |
| Number of dc subcarriers | 1 (Index 1024, counting from 0) |
| Guard subcarriers: Left, Right | 184, 183 |
| TilePermutation | 6, 48, 58, 57, 50, 1, 13, 26, 46, 44, 30, 3, 27, 53, 22, 18, 61, 7, 55, 36, 45, 37, 52, 15, 40, 2, 20, 4, 34, 31, 10, 5, 41, 9, 69, 63, 21, 11, 12, 19, 68, 56, 43, 23, 25, 39, 66, 42, 16, 47, 51, 8, 62, 14, 33, 24, 32, 17, 54, 29, 67, 49, 65, 35, 38, 59, 64, 28, 60, 0 |
| Nsubchannels | 70 |
| Nsubcarriers | 48 |
| Ntiles | 420 |
| Tiles per subchannel | 6 |

#### Symbol structure for subchannel (PUSC)

A slot in the uplink is composed of three OFDMA symbols and one subchannel, within each slot, there are 48 data subcarriers and 24 fixed-location pilots.

The subchannel is constructed from six uplink tiles, each tile has four successive active subcarriers and its configuration is illustrated in the following figure.

Figure 7: Description of an uplink tile

#### Partitioning of subcarriers into subchannels in the uplink

The allocated frequency band shall be divided into 420 tiles, the allocation of tiles to subchannels is performed in the following manner:

- Divide the 420 tiles into six groups, containing 70 adjacent tiles each. Tiles are considered adjacent if they have successive physical indices.

- Choose six tiles per subchannel using Equation (5).

where,

- n	is the tile index 0…5

- Pt	is the tile permutation

- s	is the subchannel number

- UL_PermBase	is an integer value in the range 0…69, which is assigned by a management entity.

After allocating the tiles for each subchannel the data subcarriers per slot are enumerated by the following process:

- After allocating the pilot carriers within each tile, indexing of the data subcarriers within theeach slot is performed starting from the first symbol at the lowest indexed subcarrier of the lowest indexed tile and continuing in an ascending manner throughout the slot's subcarriers in the same symbol, then going to the next symbol at the lowest indexed data subcarrier, and so on. Data subcarriers shall be indexed from 0 to 47.

- The mapping of data onto the subcarriers will follow Equation (6). This equation calculates the subcarrier index (as assigned in item 1) to which the data constellation point is to be mapped.

where,

- n	is a running index 0…47, indicating the data constellation point,

- s	is the subchannel number,

- Nsubcarriers	is the number of subcarriers per slot.

For example, for subchannel 1 (s = 1), the first data constellation point (n = 0) is mapped onto subcarrier (0,1) = 13, where 13 is the subcarrier with index 13. Considering the PUSC tile structure, it can be seen that this is the second indexed subcarrier on the second symbol within the slot. Similarly, for subchannel 3, the ninth data constellation point (n = 8) is mapped onto subcarrier (8, 3) = 47, this is the last indexed subcarrier of the third symbol within the slot.

## OFDMA ranging

A ranging channel is composed of one or more groups of six adjacent subchannels, using the symbol structure defined in 8.4.6.2.1, where the groups are defined starting from the first subchannel. Optionally, ranging channel can be composed of one or more groups of eight adjacent subchannels using the symbol structure defined in  or 8.4.6.3. Subchannels are considered adjacent if they have successive logical subchannel numbers. The indices of the subchannels that compose the ranging channel are specified in the UL-MAP message. Users are allowed to collide on this ranging channel. To effect a ranging transmission, each user randomly chooses one ranging code from a bank of specified binary codes. These codes are then BPSK modulated onto the subcarriers in the ranging channel, one bit per subcarrier (subcarriers used for ranging shall be modulated with the waveform specified in / and are not restricted to any time grid specified for the data subchannels).

### Initial-ranging transmissions

The initial ranging transmission shall be used by any SS that wants to synchronize to the system channel for the first time. An initial-ranging transmission shall be performed during two consecutive symbols. The same ranging code is transmitted on the ranging channel during each symbol, with no phase discontinuity between the two symbols. A time-domain illustration of the initial-ranging transmission is shown in Figure 8.

Figure 8: Initial-ranging transmission for OFDMA

The transmitted signal is according to section 1.5, Equation (1), except that

The BS can allocate two consecutive initial ranging slots; onto those the SS shall transmit the two consecutive initial ranging codes (starting code shall always be a multiple of 2), as illustrated in Figure 9.

Figure 9: Initial-ranging transmission for OFDMA, using two consecutive initial ranging codes

### Periodic-ranging and bandwidth-request transmissions

Periodic-ranging transmissions are sent periodically for system periodic ranging. Bandwidth-requests transmissions are for requesting uplink allocations from the BS.

These transmissions shall be sent only by SS that have already synchronized to the system.

To perform either a periodic-ranging or bandwidth-request transmission, the SS can send a transmission in one of the following ways:

- Modulate one ranging code on the ranging subchannel for a period of one OFDMA symbol. Ranging subchannels are dynamically allocated by the MAC layer and indicated in the UL-MAP. A time- domain illustration of the periodic-ranging or bandwidth-request transmission is shown in Figure 10.

Figure 10: Periodic-ranging or bandwidth-request transmission for OFDMA using one code

- Modulating three consecutive ranging codes (starting code shall always be a multiple of 3) on the ranging subchannel for a period of three OFDMA symbols (one code per symbol). Ranging subchannels are dynamically allocated by the MAC layer and indicated in the UL-MAP. A time- domain illustration of the periodic-ranging or bandwidth-request transmission is shown in Figure 11.

Figure 11: Periodic-ranging or bandwidth-request transmission for OFDMA using three consecutive codes

### Ranging codes

The binary codes are the pseudonoise codes produced by the PRBS described in Figure 12, which implements the polynomial generator 1+x+x4+x7+x15. The PRBS generator shall be initialized by the seed b14...b0 = 0,0,1,0,1,0,1,1,s0,s1,s2,s3,s4,s5,s6 where s6 is the MSB LSB of the PRBS seed, and s6:s0 = UL_PermBase, where s6 is the MSB of UL_IDcellPermBase.

Figure 12: PRBS for ranging code generation

The binary ranging codes are subsequences of the pseudonoise sequence appearing at its output Ck. The length of each ranging code is 144 bits. These bits are used to modulate the subcarriers in a group of six adjacent subchannels, where subchannels are considered adjacent if they have successive logical subchannel numbers. The bits are mapped to the subcarriers in increasing frequency order of the subcarriers, such that the lowest indexed bit modulates the sub-carrier with the lowest frequency index and the highest indexed bit modulates the sub-carrier with the highest frequency index. The index of the lowest numbered subchannel in the six shall be an integer multiple of six. The six subchannels are called a ranging subchannel. The ranging subchannel is referenced in the ranging and Bandwidth Request messages by the index of lowest numbered subchannel.

For example, the first 144 bit code obtained by clocking the PN generator as specified, with UL_PermBase = 0, the first code shall be .00110000010001... The next ranging code is produced by taking the output of the 145th to 288th clock of the PRBS, etc.

The number of available codes is 256, numbered 0..255. Each BS uses a subgroup of these codes, where the subgroup is defined by a number S, 0  S  255. The group of codes will be between S and ((S+N+M+L) mod 256).

- The first N codes produced are for initial-ranging. Clock the PRBS generator 144  (S mod 256) times to 144  ((S + N) mod 256) - 1 times.

- The next M codes produced are for periodic-ranging. Clock the PRBS generator 144  ((N + S) mod 256) times to 144  ((N + M + S) mod 256) –1 times.

- The next L codes produced are for bandwidth-requests. Clock the PRBS generator 144  ((N + M + S) mod 256) times to 144  ((N + M + L + S) mod 256) –1 times.

### Ranging and BW request opportunity size

For CDMA ranging and BW request, the ranging opportunity size is the number of symbols required to transmit the appropriate ranging/BW request code (1,2,3 or 4 symbols), and is denoted N1. N2 denotes the number of subchannels required to transmit a ranging code (6). In each ranging/BW request allocation, the opportunity size (N1) is fixed and conveyed by the corresponding UL_MAP_IE that defines the allocation.

The ranging allocation is subdivided into slots of N1 OFDMA symbols by N2 subchannels, in a time first order, i.e. the first opportunity begins on the first symbol of the first subchannel of the ranging allocation, the next opportunities appear in ascending order in the same subchannel, until the end of the ranging/BW request allocation (or until there are less than N1 symbols in the current subchannel), and then the number of subchannel is incremented by N2. The ranging allocation is not required to be a whole multiple of N1 symbols, so a gap may be formed (that can be used to mitigate interference between ranging and data transmissions). Each CDMA code will be transmitted at the beginning of the corresponding slot. See Figure 13.

Figure 13: Ranging/BW request opportunities