IEEE P802.22
Wireless RANs

| HuaWei’s Efficient and Flexible Structure for CPE CSIT collection at base station for TDD OFDMA architecture |  |  |  |  |
| --- | --- | --- | --- | --- |
| Last Revised - Date: 2006-06-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Vincent K. N. Lau | HKUST | Hong Kong, China | 852-2358-7066 | eeknlau@ee.ust.hk |
| Roger S. Cheng | HKUST | Hong Kong, China | 852-2358-7072 | eecheng@ee.ust.hk |
| Ross D. Murch | HKUST | Hong Kong, China | 852-2358-7044 | eermurch@ee.ust.hk |
| Wai Ho Mow | HKUST | Hong Kong, China | 852-2358-7070 | eewhmow@ee.ust.hk |
| Khaled Ben Letaief | HKUST | Hong Kong, China | 852-2358-7064 | eekhaled@ee.ust.hk |
| Linjun Lu | Huawei Technologies | Shenzhen, China | 86-755-28973119 | lvlinjun@huawei.com |
| Soo-Young Chang | Huawei Technologies | Davis, CA, U.S. | 1-916 278 6568 | sychang@ecs.csus.edu |
| Jianwei Zhang | Huawei Technologies | Shanghai, China | 86-21-68644808 | zhangjianwei@huawei.com |
| Lai Qian | Huawei Technologies | Shenzhen, China | 86-755-28973118 | qlai@huawei.com |
| Jianhuan Wen | Huawei Technologies | Shenzhen, China | 86-755-28973121 | wenjh@huawei.com |

Abstract

This document proposes an effective and flexible structure for facilitating the collection of necessary CSIT at the BS for TDD based OFDMA architecture.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Document Policy and Procedures: The contributor is familiar with the IEEE 802 Document Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of document(s), including document applications, provided the IEEE receives assurance from the document holder or applicant with respect to documents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of document information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if documented technology (or technology under document application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Document Committee Administrator at <patcom@ieee.org>.

| Co-Author(s): |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Company | Address | Phone | email |
| Edward K. S. Au | HKUST | Hong Kong, China | 852-2358-7086 | eeedward@ee.ust.hk |
| Peter W. C. Chan | HKUST | Hong Kong, China | 852-2358-7086 | peter@ee.ust.hk |
| Ernest S. Lo | HKUST | Hong Kong, China | 852-2358-7086 | eeern@ee.ust.hk |
| Lingfan Weng | HKUST | Hong Kong, China | 852-2358-7086 | lingfan@ee.ust.hk |
| Zhou Wu | Huawei Technologies | Shenzhen, China | 86-755-28979499 | wuzhou@huawei.com |
| Jun Rong | Huawei Technologies | Shenzhen, China | 86-755-28979499 | rongjun@huawei.com |
| Jian Jiao | Huawei Technologies | Beijing, China | 86-10-82882751 | jiao_jian@huawei.com |
| Mingwei Jie | Huawei Technologies | Shenzhen, China | 86-755-28972660 | jiemingwei@hauwei.com |

Contents

1.	Effective and flexible structure for CPE CSIT collection at base station for TDD OFDMA architecture	4

1.1.	Introduction	4

1.2.	Motivation	4

1.3.	Main Features	4

1.4.	Details of the Polling and Feedback Format	5

1.4.1.	For those selected CPEs that are active	5

1.4.2.	For those selected CPEs that are inactive	7

1.5.	Conclusion	8

# Effective and flexible structure for CPE CSIT collection at base station for TDD OFDMA architecture

## Introduction

Channel state information at the transmitter (CSIT) is crucial for efficient centralized resource allocation and scheduling at the base station (BS). Note here that we refer to the channel gain only because the phase is not important for resource allocation. For TDD system, channel reciprocity of the uplink (UL) and downlink (DL) channels enables the CSIT acquisition at the BS for the excited subchannels of those currently UL-active CPEs. However, the CSIT of the temporarily UL-inactive CPEs and the un-excited subchannels of those UL-active CPEs cannot be obtained using the channel reciprocity because there is no UL traffic. Therefore, it is very important to design a good CSIT collection mechanism such that efficient resource allocation and scheduling can be properly carried out at the BS. In this section, we propose an effective and flexible structure for facilitating the collection of necessary CSIT at the BS for TDD based OFDMA architecture.

## Motivation

For the DL transmission, BS has the full knowledge of the QoS requirements and the queueing states of all the CPEs. Therefore, BS is able to determine which CPEs have higher priority and are more urgent. As a result, the BS can postpone the collection of the channel state information (CSI) of a CPE until its traffic becomes more urgent. We propose to use a polling (request) based feedback mechanism, where both the polling (request) and feedback format are defined. Note that, due to the fact that the maximum Doppler frequency of the WRAN system is very small and the variation of Doppler frequency among CPEs is limited, the CSIT polling of each CPE can be performed rather infrequently and the polling overhead is limited. Hence, we shall adopt a polling based CSIT feedback mechanism in the collection of CSIT.

## Main Features

There are four main features that are detailed as follows.

    - Centralized polling at the BS. BS decides which CPEs to poll based on QoS requirements, queueing states, etc. It also decides for each selected CPE which subchannel in which subband to estimate based on the detected incumbents, history, etc.

    - Placement of the polling information. For currently active CPEs, the polling information is contained in the UL-MAP, whereas for currently inactive CPEs, the polling information is contained in some broadcast channels.

    - Flexibility. BS may request CSIT of any subchannel in any subband. There is a default constraint on the number of subchannels N and the number of OFDM symbols M that a CPE should use to convey the feedback to the BS, yet we still provide the option that the BS has the freedom to allocate less or more subchannels depending on the QoS requirement or urgency of the DL traffic. Note that the selected subchannels (that are used to convey CSIT) of the first M OFDM symbols of the UL subframe will be used to convey the CSIT feedback. Additionally, aside from a default CSIT quantization level that is known to both BS and CPEs, the BS has the option to control the precision of the feedback.

    - CPEs decide which subchannel CSIT to feedback based on the channel condition. Since the modulation and coding scheme is predefined, the CPE knows the number of bits Q it can use to do feedback given the number of subchannels N and the number of OFDM symbols M. Assume there are 64 subchannels in a subband, which means 6 bits is needed to represent the subchannel index, and x bits is used to represent the subchannel gain as can be known from the quantization level. The CPE knows it can feedback the DL CSI of c=Q/6+x subchannels. The CPE should choose c subchannels that are UL un-excited with the largest gains. The feedback format is shown in Table 1.

Table 1 CSIT_Feedback_Format

|  | Size (bits) | Remarks |
| --- | --- | --- |
| CSIT_Feedback_Format() { |  |  |
| for i = 1:c { |  | If Q-bit feedback is allowed, then c=Q/(6+x) |
| Subchannel Index | 6 |  |
| Subchannel Gain | x |  |
| } |  |  |
| } |  |  |

## Details of the Polling and Feedback Format

### For those selected CPEs that are active

The polling information is contained in UL-MAP specified by the CSIT_Collection_Request function. The polled CPEs should feedback their DL CSI of the subband they are currently using if Subband_change_flag = 0 through the allocated uplink subchannels specified by UL-MAP using predefined modulation and coding scheme. If the Subband_change_flag = 1, it means that the BS tell the CPE to estimate the DL CSI of another specified subband in the next frame. The BS will inform the CPE of the allocated UL subchannels in the UL-MAP of that specified subband in the next frame, whereas the CPE will use the DL pilot of that subband to estimate the DL CSI and do feedback on the allocated UL subchannels specified by the UL-MAP.

Table 2 CSIT_Collection_Request for active CPEs

| Syntax | Size (bits) | Remarks |
| --- | --- | --- |
| CSIT_Collection_Request() { |  |  |
| N_CID | 8 | N_CID is the number of selected downlink-active-only CPEs and both-downlink-and-uplink-active CPEs that are in this sub-band |
| for i = 1: N_CID { |  |  |
| CID | 16 |  |
| Feedback_Control( ) | Variable |  |
| } |  |  |
| } |  |  |

Table 3 Feedback_Control Message

| Syntax | Size (bits) | Remarks |
| --- | --- | --- |
| Feedback_Control() { |  |  |
| Subband_change_flag | 1 | 0: estimate the downlink CSI of this subband 1: in the next frame estimate the downlink CSI of the ub-b subband specified by Subband Index |
| If{Subband_ change_flag==1}{ |  |  |
| Subband Index } | 8 | At most 256 6MHz subband |
| Else{ |  |  |
| Quantization_level_flag | 1 | 0: use default quantization level, L=a 1: use specified quantization level |
| If{ Quantization_level_flag ==1}{ |  |  |
| Quantization level, L=b } | 2 | Assume there are at most 4 additional quantization precision levels |
| Feedback_ch_constraint_flag | 1 | 0: use default number of subchannels, N=c 1: use specified number of subchannels |
| If{ Feedback_ch_constraint_flag==1}{ |  |  |
| Number of subchannels, N=d } | 6 | Assume 64 subchannels in a sub-band |
| Feedback_symb_constraint_flag | 1 | 0: use default number of OFDM symbols, M=e 1: use specified number of OFDM symbols |
| If{Feedback_symb_constraint_flag==1}{ |  |  |
| Number of OFDM symbols, M=f } | 2 | Assume at most 4 OFDM symbols can be feedback |
| for j=1:N{ |  |  |
| Subchannel Index } | 6 |  |
| } |  |  |
| } |  |  |

Subband_change_flag

Indicate whether to estimate the same subband in this frame or another subband in the next frame.

Quantization_level_flag

Indicate whether to use default or specified quantization level.

Feedback_ch_constraint_flag

Indicate whether to use default or specified number of subchannels to do feedback.

Feedback_symb_constraint_flag

Indicate whether to use default or specified number of OFDM symbols to do feedback.

### For those selected CPEs that are inactive

The polling information is contained in some broadcast channel and the CPEs should scan through all the broadcast channels to find whether it has been polled or not.

Table 4 CSIT_Collection_Request for inactive CPEs

| Syntax | Size | Remarks |
| --- | --- | --- |
| CSIT_Collection_Request() { |  |  |
| N_CID | 8 bits | N_CID is the number of selected inactive CPEs |
| for i = 1:N_CID{ |  |  |
| CID | 16 bits |  |
| Subband Index | 8 bits | At most 256 6MHz subbands |
| Quantization_level_flag | 1 bit | 0: use default quantization level, L=a 1: use specified quantization level |
| If{ Quantization_level_flag ==1}{ |  |  |
| Quantization level, L=b | 2 bits | Assume there are at most 4 additional quantization precision levels |
| } |  |  |
| Feedback_ch_constraint_flag | 1 bit | 0: use default number of subchannels, N=c 1: use specified number of subchannels |
| If{ Feedback_ch_constraint_flag==1}{ |  |  |
| Number of subchannels, N=d | 6 bits | Assume 64 subchannels in a subband |
| } |  |  |
| Feedback_symb_constraint_flag | 1 bit | 0: use default number of OFDM symbols, M=e 1: use specified number of OFDM symbols |
| If{Feedback_symb_constraint_flag==1}{ |  |  |
| Number of OFDM symbols, M=f | 2 bits | Assume at most 4 OFDM symbols used for feedback |
| } |  |  |
| for j=1:N{ |  |  |
| Subchannel Index | 6 bits |  |
| } |  |  |
| } |  |  |
| } |  |  |

## Conclusion

In this proposal, an effective and flexible structure for facilitating the collection of necessary CSIT at the BS is proposed for TDD based OFDMA architecture. In particular, we use a polling based mechanism where the BS decides which CPEs to be polled based on the QoS requirements, queueing states, etc. There are two specific polling signal formats corresponding to whether the selected CPE is active or not. Default constraint on the number of subchannels and the number of OFDM symbols that a CPE should use to do feedback is known to both the BS and the CPEs, yet we provide the option that the BS has the freedom to allocate less or more subchannels depend on the QoS requirement or urgency of the downlink traffic. In addition, the BS also has the option to adjust the precision of the feedback CSIT.