| IEEE P802.22 Wireless RANs Deployment Scenarios with reuse 1 (utilizing FUSC and PUSC concept) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-06-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eli Sofer | Runcom | 2 Hachoma St. Rishon Le zion, Israel | +972-54-4997 996 | elisofer@runcom.co.il |
| Peretz Shkalim | Runcom | 2 Hachoma St. Rishon Le zion, Israel | +972- 543108 448 | peretzs@runcom.co.il |
| Moti Goldstein | Runcom | 2 Hachoma St. Rishon Le zion, Israel | +972-545 728 729 | motig@runcom.co.il |
| Itzik Kitroser | Runcom | 2 Hachoma St. Rishon Le zion, Israel | +972-3 952 8829 | itzikk@runcom.co.il |
| Zion Hadad | Runcom | 2 Hachoma St. Rishon Le zion, Israel | +972-54-4560 650 | zionh@runcom.co.il |
| Yossi Segal | Runcom | 2 Hachoma St. Rishon Le zion, Israel | +972 –3 942 8894 | yossis@runcom.co.il |

Abstract

Different deployment scenarios with reuse 1 are presented utilizing the FUSC and PUSC sub-channelization concept. Simulation results describing system performance using ITU-R M 1.522 channel model are presented.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Table of Contents

- Introduction									  2

- Sub-Channel allocation (FUSC DL, PUSC DL and UL) 			  2

## WRAN system deployment configurations using FUSC

## and PUSC schemes								  6

4.  Simulation results 								10

### 1. Introduction

### IEEE 802.16 has developed the IEEE standard 802.16-2004 for broadband wireless access systems, primarily for fixed/nomadic applications.  The OFDMA PHY layer defining FUSC (Fully Used Sub-carrier) and PUSC (Partially Used Sub-Carrier) were developed within the 16-2004 standard and was adopted as the Base Line for 16e. Basically, the 16e uses the same OFDMA sub-channelization structure and its extension to address mobility has retained the OFDMA concept for FUSC and PUSC. This contribution addresses WRAN deployment configurations using only one TV channel by several adjacent WRAN Base Stations. Use of FUSC, mainly in the DownLink, and PUSC in both DL and UL as defined in 16d or 16e with Reuse 1 will endow WRAN system the needed capability and flexibility to configure the WRAN in different scenarios with manageable interference among the Base Stations where different Service Providers may deploy the system in the same region using the same free TV channel.

2  Sub-Channel allocation (FUSC DL, PUSC DL and UL)

Active (data and pilot) sub-carriers are grouped into subsets of sub-carriers (sub-channels). The minimum frequency-time unit of sub-channelization for both the uplink and downlink, is one slot, which is equal to 48 data sub-carriers. There are two types of sub-carrier permutations for sub-channelization: Diversity and contiguous. The diversity permutation draws sub-carriers pseudo-randomly to form a sub-channel. It provides frequency diversity and inter-cell interference averaging. The diversity permutations include DL FUSC, DL PUSC and UL PUSC and additional optional permutations.

DL PUSC

With DL PUSC,  for each pair of OFDM symbols, the available or usable sub-carriers are grouped into clusters containing 14 contiguous sub-carriers per symbol, with pilot and data allocations in each cluster in the even and odd symbols.

A re-arranging scheme is used to form groups of clusters such that each group is made up of clusters that are distributed throughout the sub-carrier space. A sub-channel in a group contains two clusters and is comprised of 48 data sub-carriers and eight (8) pilot sub-carriers.

UL PUSC

Analogous to the cluster structure for DL, a tile structure is defined for the UL PUSC. The available sub-carrier space is split into tiles and six (6) tiles, chosen from across the entire spectrum by means of a re-arranging/permutation scheme, are grouped together to form a slot. The slot is comprised of 48 data sub-carriers and 24 pilots sub-carriers in three OFDM symbols.

The contiguous permutation groups a block of contiguous sub-carriers to form a sub-channel. The contiguous permutations include DL AMC and UL AMC, and have the same structure.

In general, diversity sub-carrier permuntations perform well in mobile applications while contiguous sub-carrier permutations are well suited for fixed, portable, or low mobility environments.

2.1 Sub-channel structure for FUSC and PUSC

|  | FUSC DL | PUSC DL |
| --- | --- | --- |
| Number of DC subcarriers | 1 | 1 |
| Number of Guard subcarriers, Left | 173 | 184 |
| Number of Guard subcarriers, Right | 172 | 183 |
| #### Number of used subcarriers | 1703 | 1681 |
| Number of Sub-channels | 32 | 60 |
| Number of subcarriers per subchannel | 54 | 28 |
| #### Number of pilots / (per subchannel) | 167/ (5 pilots per subchannel) | 240/ (4 pilots per subchannel) |
| Number of data subcarriers in each symbol per subchannel | 48 | 24 |

### PUSC UpLink- OFDMA Uplink subcarriers allocations

| Parameter | Value |
| --- | --- |
| Number of dc subcarriers | 1 (Index 1024, counting from 0) |
| Guard subcarriers: Left, Right | 184, 183 |
| TilePermutation | 6, 48, 58, 57, 50, 1, 13, 26, 46, 44, 30, 3, 27, 53, 22, 18, 61, 7, 55, 36, 45, 37, 52, 15, 40, 2, 20, 4, 34, 31, 10, 5, 41, 9, 69, 63, 21, 11, 12, 19, 68, 56, 43, 23, 25, 39, 66, 42, 16, 47, 51, 8, 62, 14, 33, 24, 32, 17, 54, 29, 67, 49, 65, 35, 38, 59, 64, 28, 60, 0 |
| N subchannels | 70 |
| N subcarriers | 48 |
| N tiles | 420 |
| Tiles per subchannel | 6 |

Figure 1   :  PUSC UpLink  Sub-Channel structure

Figure 2:  PUSC DownLink Cluster Structure

## 3. WRAN system deployment configurations using FUSC and PUSC schemes

The following is description of WRAN system deployment configurations with reuse factor 1 utilizing FUSC and PUSC schemes. Usually, OFDMA system can be deployed with a reuse factor of 1, by allocating different Sub-Channels to different sectors. Although this lowers the throughput per sector, the coexistence, DFS and load balancing of the sectors becomes much simpler. Such a scheme is shown in the next figure:

### 3.1 Reuse of 1 scenario (FUSC in DL, PUSC in UL)

In order to satisfy requirement of reliability, coverage, capacity, spectral efficiency,   the WRAN system can be configured to work in a reuse of 1, which means the same RF frequency is allocated to all sectors in the cell.  In this case a new scheme of work must be introduced in order to achieve the needed performance.

Figure 3:Reuse of 1 configuration, 3 sectors per cell

There are three options of operation in the reuse of 1 scenario:

- Asynchronous configuration- in this configuration every base-station uses its own permutation, the frame lengths and starting times are  not synchronized among the base-stations. Therefore orthogonality is kept within the base-station but not between base-stations. In this scenario the base-stations could be synchronized or not to the same reference clock. This mode will introduce interference between base-station (sub-carriers from different sub-channels collide in a controlled way, determined by the different permutations). This configuration could be expected with WRAN systems being deployed by several Service Providers in the same geographical area without seeking coordination.

- Synchronous configuration - in this configuration all base-stations use the same reference clock (for example, by using GPS), the frame durations and starting times are also synchronized among the base-stations but still each base station uses different permutations. Therefore the time/frequency orthogonality is kept between and within the base-stations operation but still interference between the same sub-channels of different base-station occurs. This configuration could be used as an independent base-stations deployment with a controlled interference level.

- Coordinated Synchronous configuration - in this configuration all base-stations work in the synchronous mode but use also the same permutations. An upper layer is responsible for the handling of sub-channels allocations within the sectors of the base-station, making sure that better handling of the bandwidth is achieved and the system could handle and balance load between the sectors and within the system.  The bandwidth allocated to each sector is only a portion of the overall bandwidth, but when using the load balancing additional system gain is achieved. This configuration could be used as a full scale system deployment, with a common backbone (as an example).

The preferred scenario is of course the Coordinated Synchronous mode (when using this configuration with different permutations per base-station we get the synchronous mode, and if we do not use a synchronized clock between the base stations as well we end up with the asynchronous mode of operation); the configuration of the base-station sectors are presented in the following figure

Figure 4 Reuse of 1 configuration using sub-channalization, 3 sectors per cell

3.2 Combined use of FUSC and PUSC in the Down Link (Deployment Scenario 1)

Split of the OFDMA symbol between FUSC and PUSC in the same cell to maximize throughput.  FUSC can be used for area close to the Bas Station  whereas PUSC can be allocated for areas  near  the edge of the cell. PUSC is more robust and pilot density is higher compared to FUSC sub-channel.  Inter Base Station interference is kept to minimum by using Sub-channels segmentation and use of different sub-channels in adjacent sectors of the Base Stations.

###### PUSC

BS1

FUSC

BS3

Seg. 0

Seg. 0

Seg. 0

Seg. 1

Seg. 1

Seg. 1

Seg. 2

Seg. 2

Seg. 2

BS1

BS2

FUSC

Figure 5: Combined use of FUSC and PUSC in the same cell

Figure 6-  Deployment Scenario 2, FUSC DL and PUSC UL

##### 4. Simulation results

The Channel model defined for 802.16d and 16e is based on ITU-R Recommendation M1.225, (A additional  channel model (COST 231) has been selected recently for the 802.16e). Simulation results are shown for ITU-R Recommendation M1.225.

Dynamic channels are usually defined with the following parameters:

- Number of taps (6 taps in ITU-B as it seen in table 5 taken from ITU-R Recommendation M1.225)

- The time delay relative to the first tap.

- The average power relative to the strongest tap.

- The Doppler spectrum of each tap.

Parameters example used for the simulations

P = [-2.5 0 -12.8 -10 -25.2 -16]; % power in each tap in dB
        K = [0 0 0 0 0 0] ; %Ricean K-Facor in linear scale
        tau = [0 0.3 8.9 12.9 17.1 20]*1e-6; % tap delay in usec
        Doppler = [2 2 2 2 2 2]*120; % doppler maximal frequency parameter in Hz - 120Kph for 10MHz
        Antenna_correllation = 0.3; %antenna correlation (envelope correlation coefficient)
        Fnorm = -1.5113; %gain normalization factor in dB

.

Figure 7:  DL ITU-B channel Model

Figure 8; UL Simulation using Channel Model ITU-B

## Figure 9: Example  using AWGN for Low Mobility