IEEE P802.22
Wireless RANs

| Channel Sensing Based on Self-Signal Suppression (SSS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Baowei Ji | Samsung Telecom America | USA | +1-972-761-7167 | Baowei.ji@samsung.com |
| Yinong Ding | Samsung Telecom America | USA | +1-972-761-7975 | yding@sta.samsung.com |
| David Mazzarese | Samsung Electronics Co. Ltd. | Korea | +82 10 3279 5210 | d.mazzarese@samsung.com |

Abstract

A novel channel sensing scheme is proposed based on self-signal suppression/cancellation, which allows in-band channel sensing without the requirements for quiet periods. The feasibility and advantages of the scheme are analyzed, together with some important enhancements. The optimized tradeoff could be achieved between spectral utilization and the reliability of in-band channel sensing by combining SSS-based channel sensing with on-demand scheduling of quiet periods or dynamic channel hopping.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Motivation

A novel channel sensing method is proposed in this document, which can sense the channel currently being used by the same cell (i.e., in-band channel sensing) without interrupting its own transmission for creating quiet period or hopping to another channel for sensing the previous channel.

This new approach is based on self-signal suppression (SSS). The SSS channel sensing is done by detecting and suppressing self-signal which occupies the channel to be sensed. The SSS channel sensing consists of three steps. First, the signals transmitted from its own cell (the self-signal) is detected (i.e., demodulated and decoded) and verified via CRC check or other data integrity verification methods by the receiver. Second, the self-signal is reconstructed from the detected signal and the estimated channel which the self-signal passed through; and is removed/cancelled from the original received signals. Third, the channel sensing is performed over the remaining signal after the self-signal suppression/cancellation. The remaining signal includes the suppression residual signal and other signals (e.g., incumbent-user signal or interference signals from other resources) if existing. Refer to Fig.1 for further illustration.

Fig. 1: Self-Signal Suppression.

# Operation Procedure

Note that self signal means different things for BS and CPE. To the BS of a cell, the uplink (UL) bursts are the self signal of the cell. To a CPE associated with the BS, the self signal consists of the preamble, other broadcasting signals (FCH, DL-MAP, UL-MAP, DCD, UCD), its own downlink (DL) bursts, and, probably, the DL bursts for other CPEs. Given this clarification, the operation procedure can be presented as follows for both the BS and CPEs.

SSS-based channel sensing approach operates as follows:

- A unit (BS or CPE) tries to detect the signal intentionally transmitted to it. This signal is referred as the self-signal. If the self-signal is correctly detected, proceed to the next step. Otherwise,

  - If the self-signal cannot be detected correctly the first time, the receiver and the transmitter should try to resolve the issue through conventional retransmission protocols (e.g., HARQ process and/or applicable power control procedures) until either the self-signal is detected correctly or a pre-determined maximum number of trials is reached. In which case the system should interrupt the self transmission in that channel, thus creating quiet period for further sensing the channel. Note that, a CPE does not request the retransmission of other CPEs’ signal even though the detection of other CPEs’ signal fails.

- The detected self-signal is reconstructed, and then cancelled from the original signal.

- Explicit channel sensing is performed over the remaining signal. Various methods can be used for channel sensing operations on the remaining signal. For example, energy detection can be used to determine whether the channel is being occupied by other systems based on the comparison result of the residue signal energy against the normal noise level. Signal feature detectors can be used to further identify which type of system is occupying the channel by analyzing the signal characteristics of the residue signal.

The operation is further described in Fig. 2.

Fig. 2: SSS-based channel sensing scheme.

# Feasibility/Complexity for Implementation

Theoretically, the self signal can be cancelled completely with ideal channel estimation and correct signal detection. Furthermore, as illustrated in Fig.3, the channel estimation accuracy can be enhanced by accumulating and averaging the channel estimation results from a number of preambles transmitted within the coherence time of the channel. Actually, the fixed point-to-multipoint topology of WRAN inherently has long coherence time as desired. Note that, a well designed preamble can be detected reliably even with some interference due to the processing gain typically available to the preambles. All these desired characteristics assist positively to the sufficient suppression of the self signal.

Another important point is that most of the functions shown in Fig.2 are either the regular signal receiving procedures (signal detection, signal integrity check, and data retransmission), or the necessary regular channel sensing procedures. Even the iterative channel estimation procedure shown in Fig. 3 is normal operation independent of SSS. Only two extra procedures (signal reconstruction and signal cancellation) are unique to SSS.

To measure the effect of the self-signal suppression, we define a quantity called self-signal suppression ratio (SSSR) as

SSSR = 10log10Power of the Self Signal (P1)Power of the Suppression Residual Signal(P2)

See Fig. 4 for the illustration of the power levels of P1 and P2.

Fig. 3: Channel estimation over multiple frames within the channel coherence time.

Fig. 4: Comparison of before and after Self-Signal Suppression.

# Some Enhancements of SSS Channel Sensing

# The BS can do channel sensing over all the UL bursts.

# A CPE can do channel sensing over other CPE’s DL bursts, besides its own, because any CPE has the capacity to use the whole channel. However, the CPE won’t require for retransmission of other CPEs’ data if signal detection fails.

# Both the BS and CPEs can do long-time channel sensing over the remaining signals of multiple frames in order to increase the reliability and accuracy of channel sensing.

# Optimal Channel Sensing Scheme

SSS can operate alternatively with the other two approaches (quiet-time and DFH). The optimal method could be that channel sensing should be based on SSS most time, and the other two approaches (either one) can be used on demand when the self-signal cannot be detected correctly and further channel sensing is required.

Fig. 5: Optimal channel sensing scheme.

Advantages

No quiet period and channel moving is needed for inband channel sensing in the novel scheme, which removes the cost incurred by periodic quiet period or periodic channel hopping. Compared to quiet-period based approach, while a moderate complexity is incurred in the receiver side for self-signal reconstruction and self-signal suppression, the preamble and pilot tones can be transmitted without interruption which otherwise could reduce the channel estimation accuracy and reliability. The uninterrupted transmission in the cell could result in better QoS.

Meanwhile, SSS can operate alternatively with the other two approaches (quiet-time and DFH). The optimal method could be that channel sensing should be based on SSS most time, and the other two approaches (either one) can be used on demand when the self-signal cannot be detected correctly and further channel sensing is required.

Appendix

Incumbent Detection Threshold in 802.22 WRAN System [1]

Table 15.1.2: DFS-like parameters for sensing and vacating channels used by incumbent services in the US

| DFS Parameter | Value for Part 74 Devices | Value for TV Broadcasting |
| --- | --- | --- |
| Channel Availability Check Time | 30 sec (recommended) | 30 sec (recommended) |
| Non-Occupancy Period (minimum) | 10 minutes (recommended) | 10 minutes (recommended |
| Channel Detection Time | 500 msec – 2 sec to >=90% PD | 500 msec – 2 sec to >=90% PD |
| Channel Setup Time | 2 sec | 2 sec |
| Channel Opening Transmission Time (Aggregate transmission time) | 100 msec | 100 msec |
| Channel Move Time (In-service monitoring) | 2 sec | 2 sec |
| Channel Closing Transmission Time (Aggregate transmission time) | 100 msec | 100 msec |
| Incumbent Detection Threshold | -107 dBm (200kHz BW) | -116 dBm (6 MHz BW) for DTV |

# Reference:

[1] Functional Requirements for IEEE 802.22 WRAN Standard, 802.22/05-0007r46, September 2005.