IEEE P802.22
Wireless RANs

| Explicit Out-band Signaling (EOS) for Hidden Incumbent System Detection (with supplements) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tae-In Hyon | Samsung Advanced Institute of Technology | Korea | +82-31-280-9503 | taein.hyon@samsung.com |
| Sangjo Yoo | Inha University | Korea | +82-32-860-8304 | sjyoo@inha.ac.kr |

Abstract

Explicit out-band signaling is used to overcome the situation of there being a hidden incumbent system that could cause the CPE’s in the WRAN cell edge areas to be out of synchronization with the BS, thus causing the overall system to incur interference to the incumbent system. This  (July) edition edition supplements the document “SAIT-MAC(0222).doc” written in March, 2006, by adding the reinforcement of the hidden incumbent reporting scheme with acknowledgements, improving the probability of coexistence among multiple WRAN BSs for outband signaling, and efficient channel usage of the explicit outband signaling by utilizing  the fractional bandwidth usage method.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

16.4 Explicit Out-band signaling for hidden incumbent system detection

16.4.1 Hidden incumbent scenarios

In the WRAN system, the WRAN BS may not be able to detect or be informed of the existence of an incumbent signal. This situation may arise when the incumbent system signal does not reach the WRAN BS so that the BS cannot sense it. And CPEs that are inside the coverage areas of both the WRAN BS and incumbent system cannot report this overlapping to the WRAN BS because they are not able to decode the WRAN DS signal due to the strong interference. This very serious case is called the “hidden incumbent system case”.

Figure 1 shows the typical scenario for a hidden incumbent system. For WRAN service, assume WRAN BS had sensed some channels and it recognized channel F was available (it may use a geographical channel usage database information for that area). When an incumbent system begins service with the same channel F, the BS does not know the existence of incumbent system because the BS is located outside radio transmission range of incumbent system. Therefore, as shown in Figure 1, WRAN CPEs that are located in overlapped area of WRAN and incumbent system are not able to decode the WRAN DS signal because of strong incumbent system interference. In this case, BS will keep its service on channel F and it will cause strong interference to incumbent system users possibly for a long time.

Figure 1 - Example of a hidden incumbent system

This hidden incumbent system case may occur not only during the WRAN BS initialization period (service start time) but also during any service period. Incumbent systems can start their service at any time without any notification or may initiate their service disregarding the service pattern as stored in WRAN database. Therefore, WRAN BS can mistakenly change its service channel to the channel that that is already being used by an incumbent system because of its unreliable sensing results.

The hidden incumbent system case causes the following problems:

- harmful interference to the incumbent system users

- some WRAN CPEs cannot communicate with WRAN BS

An implicit incumbent system detection method as described in Section 21 can be used to resolve this hidden incumbent system case. The implicit detection method is based on the periodic sensing report exchanges between BS and CPEs after normal association procedure. But if some CPEs suddenly turn off their power without any notifications, then WRAN BS is not able to distinguish between the hidden incumbent system interfering case and power off case. And sometimes, CPEs in the overlapped area may not have associated with the WRAN BS yet, in which case the BS cannot be expected to recognize any problems associated with an incumbent system being detected.

In this section, to resolve the hidden incumbent system case, an explicit out-band signaling method is used. Through some extra channels that are not currently used, a short message is broadcast. The implicit method of Section 21 can be used together with the explicit method as a supplementary algorithm.

16.4.2 Explicit Out-band signaling for hidden incumbent case detection

To address the hidden incumbent system case, BS periodically broadcasts out-band signal that includes some information about WRAN BS’s current channel through some unoccupied channels (e.g., candidate channels) that are not used for BS service channel or incumbent system. This broadcast out-band signal follows the same PHY and MAC frame architecture (to not necessitate additional protocol or PHY modulation). In addition, the SCH includes a flag (see Table 1) to indicate whether the current MAC frame is for regular service or out-band broadcast signal. Explicit out-band signal DS-Burst includes service channel information, such as current BS’s service channels and candidate channels. When some CPEs are not able to decode the BS’s current service channel, they will try to sense other channels to locate the BS signal. If CPEs receive the explicit out-band broadcast signal, the CPEs will know what the current service channel ID is. If the current channel was already sensed and was determined not to be decodable at the CPEs, then the CPE will send a report to the BS using the out-band upstream resource. After receiving the report, the BS changes its service channel to another one that is unoccupied because BS notices the existence of the hidden incumbent in the current band.

Figure 2 shows the explicit out-band signaling for hidden incumbent case detection. Channel F is used by the BS for its service. To solve the problem that is caused by the hidden incumbent system, the BS broadcasts out-band signal periodically by in the available channels B, C and D. If the CPE can not decode the signals of channel F, the CPE starts sensing other channels for detecting out-band signal from the BS. When the CPE detects the out-band broadcast signal of the BS in channel C, it transmits hidden incumbent system report message (see Table 5) to BS with upstream resource that exists in out-band broadcast signal.

Figure 2 - Example of explicit out-band signaling

16.4.2.1 BS Out-band signal broadcasting

The BS sets the EOS field value in the SCH periodically and broadcasts the out-band signal MAC frame in channel B, channel C and channel D which is selected among candidate channel list as shown in Figure 3. The EOS field indicates whether the transmitting MAC frame is a general MAC frame or out-band signal MAC frame. The CPEs can detect the signal of BS using to the defined explicit out-band signaling period because the BS broadcasts out-band signal in other available channels, none of which are currently being used. The explicit out-band signaling period is decided by the BS. If a BS receives Hidden Incumbent System Report messages from CPE during the predefined period, it will recognize that current service channel F can not be decoded by the CPE(s). So, the BS must move the service channel to another channel. This process is illustrated in Figure 5-1.

Figure 3 - Explicit out-band signaling

Although the basic scheme is that the out-band signal is broadcast in candidate channel B, channel C and channel D at the same time, the BS may choose to broadcast out-band signal sequentially in time for simplification of system organization, as shown Figure 4.

Broadcasting out-band signals at the same time like Figure 3 requires multiple PHY transceivers at the BS. On the other hand, the sequential out-band signal transmission as described in Figure 4 only requires one transceiver for out-band signaling. But, this sequential method has a drawback that the probability witha which the CPEs can detect the out-band signal decreases.

Figure 4 - Sequential explicit out-band signaling

Basically, out-band signal does not include the CPE list that should respond to the out-band signal with Hidden Incumbent System Report because the BS does not know which CPEs are located in the overlapped area in advance. But, optionally BS may choose to include a CPE list to poll specific CPEs. However, in this case the polled CPE should respond even if the current service channel is available to the CPE.

16.4.2.2 CPE hidden incumbent reporting

If the CPE can not decode the signal of the BS, it will detect out-band signal after sensing other channel as shown in Figure 5-2. If a signal in the current service channel is not decodable by the CPE because of incumbent system’s interference, then the CPE sends “Hidden Incumbent System Report” (see Table 5) to BS using US-Burst in the out-band. Hidden Incumbent System Report can indicate either “the current service channel F is not decodable” or “the current service channel F is used by an incumbent system” if the CPE can recognize incumbent signal. Also, the report can include sensing results for some other channels.

| Figure 5-1 BS | Figure 5-2 CPE |
| --- | --- |

To reduce reporting overhead and possible report collisions, only CPEs that are not able to decode the current service channel because of strong interference from other system (e.g., incumbent system) respond with “Hidden Incumbent System Report,” except when the CPE is explicitly polled by the BS.

Because the BS does not know which CPEs will send the report in advance, BS generally cannot allocate US resource to each CPE that will send the report. Therefore collisions of reports may be occurred. To increase the probability of successful report delivery to the BS, during one explicit outband signaling time k (2 ~ 3) number of MAC (including DS and US) frames are transmitted as shown in Figure 6. In SCH of DS the information for k should be included. For the first US1 all CPEs that detected hidden incumbent systems will try to send reports. As in Figure 7, in DS-2 and DS-3 BS gives acknowledgements for the previous reports to indicate successful report receptions. So that during the US-2 and US-3, only CPEs that did not receive acknowledgements for the previous reports will try sending reports again.

Figure 6 – Explicit outband signaling time.

Figure 7 – Report and Acknowledgment (k=3 case)

16.4.2.3 CPE upstream bandwidth allocation for hidden incumbent report

For the CPE Hidden Incumbent System Report message, there needs to be a process or method for allocation of upstream resource to CPEs. To reduce this overhead, the BS can allocate upstream resource for CPE’s Hidden Incumbent System Report (see Table 5) by using one of the following two methods.

i) First method is that the BS divides US resource into US-Burst slots for all unknown CPEs. When the BS transmits Out-band signal frame, it divides upstream resource according to maximum hidden incumbent report size and allocates each upstream burst. Also, it transmits US-MAP including each upstream burst's allocation information. When a CPE sends the Hidden Incumbent System Report, the CPE randomly selects a US-Burst, and then the CPE sends the Hidden Incumbent Report with the selected US-Burst. Figure 8 shows out-band signal MAC frame of slotted method.

Figure 8 – Out-band signal MAC frame of slotted method

ii) The second method is to use the CDMA code. The PHY supports the usage of a CDMA mechanism for the purpose of hidden incumbent system report.

As specified in the PHY spec, the PHY has a subset of the available CDMA codes that shall be used for contention-based CDMA. The CPE, upon needing to transmit a hidden incumbent system report, shall select a CDMA code with equal probability from the code subset allocated for hidden incumbent notification. The CDMA code subchannel shall be selected by the PHY amongst the ones reserved by the MAC for hidden incumbent report.

When the BS transmits out-band signal, it allocates resource to transmit the CDMA code in upstream burst. A CPE transmits the selected random CDMA code in order to be assigned upstream resource to transmit the Hidden Incumbent System Report message. The BS shall provide upstream allocation for the CPE. But instead of indicating a basic CID, the broadcast CID shall be sent in combination with a CDMA_Allocation_IE, which specifies the transmit region and code that were used by the CPE. This allows a CPE to determine whether it has been given an allocation by matching these parameters with the parameters it used. The CPE shall use the allocation to transmit the Hidden Incumbent System Report. In addition, if allowed by the BS, data could also be transmitted in this allocation. Figure 9 shows Out-band signal MAC frame of CDMA method.

If the BS does not issue the CDMA_Allocation_IE, the CPE shall assume that the Code transmission resulted in a collision and try Random Backoff.

Figure 9 - Out-band signal MAC frame of CDMA method

16.4.2.4 Co-existence between WRAN BSs for Out-band signaling

Transmitting multiple out-band signals has a possibility to collide with out-band signals of other co-located WRAN BSs. If communication between BSs is possible, the BSs can coexist by sending out-band signal in candidate channels at different time after exchanging broadcasting time schedule with each other. But if communication between BSs for exchanging their time schedule is not available, to avoid out-band signal collision, BSs randomly select their out-band signal broadcasting time within the pre-defined explicit out-band signaling period as shown in Figure 10 and Figure 11. Figure 10 illustrates the case when BS broadcasts outband signaling messages at multiple channels simultaneously. And Figure 11 is for the case that BS broadcasts outband signaling message sequentially in time at the different channels.

To guarantee that CPE decodes the outband signal within a short time interval without sensing time discordance, CPEs should try to find the signal at one channel at least during one explicit outband signaling period.

Figure 10 – BS coexistence for outband signaling.

Figure 11 – BS coexistence for outband signaling (sequential case)

16.4.2.5 Efficient channel use for Explicit Outband Signaling with Fractional Bandwidth Usage method

Explicit Outband Signaling does not need a full bandwidth of 6 MHz channel. Broadcasting some channel information through an explicit out-band signal message requires only small bandwidth as shown figure 12. WRAN PHY defines “OFDMA with Fractional Bandwidth Usage” (see 8.1.2.3.4) that allows WRAN BSs to use a fractional bandwidth of full 6MHz. WRAN BS broadcasts Explicit Outband Signals with only BWEOS bandwidth at the beginning part of 6MHz. If a BS that is broadcasting Explicit Outband Signal detects that other BS uses the remained part, then it will try to change the channel for Explicit Outband Signaling. Figure 13-1 and figure 13-2 illustrate how explicit outband signaling BS and other CR WRAN BS for data transmission operate.

Figures 12 – Efficient channel use for Explicit outband signaling

with Fractional bandwidth usage.

| Figure 13-1 BS | Figure 13-2 other WRAN BS |
| --- | --- |

Table 1 – Superframe control header format

| Syntax | Size | Notes |
| --- | --- | --- |
| Superframe_Control_Header_Format() { |  | 1 OFDM symbol long and transmitted with well-known modulation/coding (e.g., QPSK rate ½) |
| ST = 0 | 7 bits | System Type Indicates the type of the system using this band. |
| CT | 1 bits | Content Type Indicates the type of the content following the transmission of the SCH. Superframe = 0 CBP Beacon = 1 |
| FS | 8 bits | Frames per Superframe Indicates the number of frames within a superframe. Frames within a superframe have a fixed size which preferably does not change across superframes. |
| FDC | 8 bits | Frame Duration Code |
| TTQP | 16 bits | Time To Quiet Period The amount of time it will take for the next scheduled quiet period. This serves to synchronize the quiet period of overlapping BSs. the TTQP is divided into two subfields: Time Scale and Time. The Time subfield consists of a 15 bit unsigned integer number. |
| DQP | 16 bits | Duration of Quiet Period The estimated duration of the next scheduled quiet period. This is specified similar to the TTQP field. |
| PP | 1 bit | Preamble Present Indicates whether the preamble of the following frame is present or not. For example, a frame preamble may not be needed when the cell is operating in a single physical (i.e., TV) channel |
| EOS | 1bit | Indicates whether the current MAC frame is for regular service or for out-band broadcast signal. |
| Tx ID | 48 bits | Address that uniquely identifies the transmitter of the SCH (CPE or BS) |
| CN | 8 bits | Channel Number Indicates the starting physical (i.e., TV) channel number used by the BS. |
| NC | 5 bits | Number of Channels In case channel bonding is used, this field indicates the number of additional consecutive physical (i.e., TV) channels used by the BS. In the basic mode, NC = 3 (i.e., three TV channels). |
| Reserved | 1 bit |  |
| GIF | 1 bit | Guard Interval Factor Specifies the GIF used by the PHY in the frame transmissions of this superframe. Pre-determined values are: 0 = O-QAM 4 = Default mode used for superframe transmission |
| Length | 8 bits | The length of the information following the SCH |
| IEs | Variable | Optional Information Elements which would be transmitted after the SCH. They are: - MAC version - Current transmit power - Part 74 acknowledgement - Location configuration information |
| HCS | 8 bits | Header Check Sequence |
| } |  |  |

Table 2 – Bulk Measurement Report (BLM-REP) Message format

| Syntax | Size | Notes |
| --- | --- | --- |
| BLM-REP_Message_Format() { |  |  |
| Management Message Type = 41 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Number of Single Measurement Reports | 8 bits | The number of single measure ment reports contained in this message |
| Single Measurement Reports | Variable | Table 3 |
| } |  |  |

Table 3 – Single Measurement Report Message format

| Syntax | Size | Notes |
| --- | --- | --- |
| Single_Measurement_Report_Format() { |  |  |
| Element ID | 8 bits | Table 4 |
| Length | 8 bits |  |
| Transaction ID | 16 bits |  |
| Report Information Element | Variable | Table 4 |
| } |  |  |

Table 4 – Report Information Elements

| Element ID (1 byte) | Length (bytes) | Description |
| --- | --- | --- |
| 129 | Variable | TV Measurement Report |
| 130 | Variable | Part 74 Measurement Report |
| 131 | Variable | Beacon (802.22 Related) Measurement Report |
| 132 | Variable | CPE Statistics Measurement Report |
| 133 | Variable | Location Configuration Measurement Report |
| 134 | Variable | Beacon (Part 74) Measurement Report |
| 133 | Variable | Consolidated Spectrum Occupancy Measurement Report |
| 132 | Variable | Channel Feedback Measurement Report |
| # (ex. 135) | Variable | Hidden Incumbent System Report – Table 5 |
| 131-255 |  | Reserved |

Table 5 – Hidden Incumbent System Report Message format

| Syntax | Size | Notes |
| --- | --- | --- |
| Hidden_Incumbent_System_Report_Format() { |  |  |
| Report Mode | 4 bits | 6 |
| Start Frame | 8 bits | Frame number in which the channel measurement start ed |
| Duration | 16 bits | The actual duration of the measurement |
| Channel Number | 8 bits |  |
| Number of Channels | 8 bits |  |
| for (i = 1; i  Number of Channels; i++) { |  |  |
| Channel State | 3 bits | Table 7 |
| } |  |  |
| } |  |  |

Table 6 – Report mode

| Syntax | Size | Notes |
| --- | --- | --- |
| Report_Mode_Format() { |  |  |
| Late | 1 bit | Indicates whether this CPE is unable to carry out a measurement request because it received the request after the requested measurement time. The Late bit shall be set equal to 1 to indicate the request was too late. The Late bit shall be set to 0 to indicate the request was received in time for the measurement to be executed, or if no start time was specified. |
| Incapable | 1 bit | Indicates whether this CPE is incapable of generating this report requested by the BS. The Incapable bit shall be set to 1 to indicate the CPE is incapable. The Incapable bit shall be set to 0 to indicate the CPE is capable or the report is autonomous. |
| Refused | 1 bit | Indicates whether this CPE is refusing to generate this report requested by the BS. The Refused bit shall be set to 1 to indicate the CPE is refusing. The Refused bit shall be set to 0 to indicate the CPE is not refusing or the report is autonomous. |
| Unmeasured | 1 bit | CPE did not measure the channel |
| } |  |  |

Table 7 – Channel state information

| Value | State |
| --- | --- |
| 000 | Unmeasured |
| 001 | Vacant |
| 010 | Occupied |
| 011 | Occupied by Incumbent |
| 100 | Occupied by 802.22 |
| 101-111 | Reserved |